�����������������������������������������������������������������������ͻ

�  CI2FAX - Faxmodem properties                                         �

�����������������������������������������������������������������������ͼ

The list below describes the critical properties of faxmodems that have

been tested by TurboPower Software or an Async Professional 2.0 user.

The far left column contains a stack of modem manufacturer information.

The TPS column indicates whether the modem was tested by TurboPower

Software directly. Class indicates which classes the modem supports: 1

for Class 1, 2 for Class 2, 2.0 for Class 2.0 and C for CAS. AutoSWF and

AutoHWF indicate whether or not the modem enables software or hardware

flow control upon entering fax mode: Y means yes, N means no and is

usually followed by the command to enable flow control. The Notes column

includes any notes of interest for that modem. A question mark in any

fields means we're not certain of the value for that faxmodem. Anywhere

a question mark appears we are not certain how the modem behaves. For

example "?,2" under Class means we don't know if it supports Class 1 but

it does support Class 2.

Updated 12/93

===========================================================================

Manufacturer

Modem Name

Chip

Model

Revision                  Class     TPS  AutoSWF   AutoHWF  Notes

============================================================================

AT&T                       1         N   N AT\Q1   N AT\Q3

DataPort 14.4

NA

NA

NA

----------------------------------------------------------------------------

April                      ?,2       N   N AT\Q1   N AT\Q5

????

EXAR

9648,2,E,290X

08/12/92 MICC14400-PK-V5.05J-M

----------------------------------------------------------------------------

CompletePC                 ?,2       N   N AT\Q1   N AT\Q5

TurboModem Plus

Rockwell

144DP

FM-144VBIS/R V1.5

----------------------------------------------------------------------------

Hayes                      1         N   ?         ?

Optima 144 and

Optima V.fc 288

NA

NA

NA

----------------------------------------------------------------------------

IBM                        ?,2       N   ?         Y

ThinkPad 700

Rockwell

RC32ACL

V1.291 IBM Corp. / (C)P.N.B 1993 - 024

----------------------------------------------------------------------------

Intel                      1,C       Y   ?         ?        CAS modem, can

SatisFAXtion 300                                            be used as

Rockwell                                                    Class 1

V.32AC

U10,U11-27C256-6.00

----------------------------------------------------------------------------

Intel                      C         N   NA        NA       CAS faxmodem, must

SatisFAXtion 400                                            be used with

?                                                           CASMODEM.EXE

?

?

----------------------------------------------------------------------------

Multitech                  1,?       N   ?         ?        Requires 10

MT224BAF                                                    msec delay between

?                                                           command chars:

?                                                           InterCharDelay:=10

?

----------------------------------------------------------------------------

Multitech                  1,?       N   ?         ?        Requires 10

MT1432BL                                                    msec delay between

?                                                           command chars:

?                                                           InterCharDelay:=10

?

-----------------------------------------------------------------------------

(noname clone)             2         Y   Y         Y

9624

Sierra, *****

SSX196

1.4 122091, SSC

----------------------------------------------------------------------------

Practical Peripherals      1,2       Y   Y         Y        Ignores switch

PPI 14400PKT                                                from C1 to C2,

PPI, 1992                                                   must first switch

PM14400FXPPM [BA4]                                          C0, then new class

2.16

----------------------------------------------------------------------------

Supra Corp.                1,2       Y   N AT&K5   N AT&K3

SupraFax 144

Rockwell

V.32AC

V1.71-02 TR14-Jxxx-001

----------------------------------------------------------------------------

TwinCom                    ?,2       N   ?         ?

144000 DFi

Rockwell

V.32AC

V1.270 TR14-Jxxx-001

----------------------------------------------------------------------------

U.S. Robotics              1         Y   Y         Y

Sportster 14400

NA

NA

NA

----------------------------------------------------------------------------

U.S. Robotics              1         Y   ?         ?

16800

NA

NA

NA

----------------------------------------------------------------------------

VIVA                       1,2       Y   Y         Y

2442ef

Rockwell

RC9624AC

TROO-J260-001 100

----------------------------------------------------------------------------

Zyxel                      ?,2       N   ?         ?

1496B

ZyXEL

U1496B V 6.10a M

U1496B V 6.10a M

----------------------------------------------------------------------------

Zyxel                      ?,2       N   ?         ?

1496E

ZyXEL

U1496E V 5.04f M

U1496E V 5.04f M

�����������������������������������������������������������������������ͻ

�  CI2FAX - Error code descriptions                                     �

�����������������������������������������������������������������������ͼ

{Error types}

etFatal          = 0;          {Fatal errors}

etNonFatal       = 1;          {Non-fatal I/O errors}

etWarning        = 2;          {Warning messages (currently not used)}

etMessage        = 3;          {Status information (generally should

not be acted on by an error handler)}

{Error prefixes}

epFatal          = etFatal * 10000;

epNonFatal       = etNonFatal * 10000;

epWarning        = etWarning * 10000;

epMessage        = etMessage * 10000;

{Error/status code constants}

ecOk                     = 0;     {Reset value for AsyncStatus}

{+++General error codes (0-999)+++}

{DOS errors}

ecFileNotFound           = 0002;  {File not found}

ecPathNotFound           = 0003;  {Path not found}

ecTooManyFiles           = 0004;  {Too many open files}

ecAccessDenied           = 0005;  {File access denied}

ecInvalidHandle          = 0006;  {Invalid file handle}

ecOutOfMemory            = 0008;  {Insufficient memory}

ecInvalidDrive           = 0015;  {Invalid drive}

ecNoMoreFiles            = 0018;  {No more files}

{Turbo Pascal I/O errors}

ecDiskRead               = 0100;  {Attempt to read beyond end of file}

ecDiskFull               = 0101;  {Disk is full}

ecNotAssigned            = 0102;  {File not Assign-ed}

ecNotOpen                = 0103;  {File not open}

ecNotOpenInput           = 0104;  {File not open for input}

ecNotOpenOutput          = 0105;  {File not open for output}

ecInvalidFormat          = 0106;  {Invalid format for packed window}

{DOS critical errors}

ecWriteProtected         = 0150;  {Disk is write-protected}

ecUnknownUnit            = 0151;  {Unknown disk unit}

ecDriveNotReady          = 0152;  {Drive is not ready}

ecUnknownCommand         = 0153;  {Unknown command}

ecCrcError               = 0154;  {Data error}

ecBadStructLen           = 0155;  {Bad request structure length}

ecSeekError              = 0156;  {Seek error}

ecUnknownMedia           = 0157;  {Unknown media type}

ecSectorNotFound         = 0158;  {Disk sector not found}

ecOutOfPaper             = 0159;  {Printer is out of paper}

ecDeviceWrite            = 0160;  {Device write error}

ecDeviceRead             = 0161;  {Device read error}

ecHardwareFailure        = 0162;  {General failure}

{+++Capacity or environmental errors (2900-2999)+++}

{APUART port errors}

ecNoMorePorts            = 2900;  {Can't open port, no slots available}

ecOverrunError           = 2901;  {UART receiver overrun}

ecParityError            = 2902;  {UART receiver parity error}

ecFramingError           = 2903;  {UART receiver framing error}

{APINT14 port errors}

ecTransmitFailed         = 2910;  {Int14 transmit failed}

ecUartError              = 2911;  {Int14 receive failed}

{APCOM/OOCOM errors/status}

ecBlockIncomplete        = 2920;  {Block shorter than requested}

ecBufferIsFull           = 2921;  {No room for new char in buffer}

ecBufferIsEmpty          = 2922;  {No characters to get}

ecTimeout                = 2923;  {Timed out waiting for something}

ecStringIncomplete       = 2924;  {String shorter than requested}

ecStringOverrun          = 2925;  {String longer than 255}

ecUserAbort              = 2926;  {User aborted during "wait"}

{APMODEM/OOMODEM errors}

ecTableFull              = 2930;  {No room in table to add new entry}

ecNullCommand            = 2931;  {Modem - no command registered}

{Tracing/EventFile file errors}

ecEventFileError         = 2940;  {Failed to open or write to the event file}

ecTraceFileError         = 2941;  {Failed to open or write to the trace file}

{Other device layer errors}

ecNoFossil               = 2950;  {No fossil installed}

ecDigiFailure            = 2960;  {Generic Digiboard failure code}

{+++Warnings or user errors (7900-7999)+++}

{This category not current used by Async Professional}

{+++Programmer errors (8900-8999)+++}

{APCOM/OOCOM port errors}

ecBadPortNumber          = 8900;  {Out-of-range port number}

ecOutofRange             = 8901;  {General out-of-range error}

ecPortNotOpen            = 8902;  {Port not open}

ecInvalidBaudRate        = 8903;  {Bad baud rate for this device}

ecInvalidArgument        = 8904;  {General programming error}

ecNoDevice               = 8905;  {No device layer installed}

ecNotaUart               = 8906;  {Couldn't find a uart at this address}

ecInvalidParity          = 8907;  {Bad parity option for this device}

ecNotBuffered            = 8910;  {Operation only allowed on buffered ports}

ecNotSupported           = 8911;  {Function not supported by device-layer}

{+++Status codes (9800-9999)+++}

{APFAX error codes}

ecFaxVoiceCall           = 9800;  {Call is VOICE}

ecFaxDataCall            = 9801;  {Call is DATA}

ecFaxBusy                = 9802;  {called modem is busy}

ecFaxNoFontFile          = 9803;  {could not find font file}

ecFaxNoCASManager        = 9804;  {CASMGR TSR not installed}

ecFaxInitError           = 9805;  {unexpected response in init}

ecFaxTrainError          = 9806;  {Failed to train with remote modem}

ecFaxSessionError        = 9807;  {Error during session}

ecFaxNoConnect           = 9808;  {No connection after dial}

ecFaxPageError           = 9809;  {Failed to send page after retries}

{APABSPCL/OOABSPCL status codes}

ecInitFail               = 9900;  {Xmodem init failed}

ecInitCancel             = 9901;  {Xmodem init was canceled on request}

ecCancelRequested        = 9902;  {Cancel requested}

ecDuplicateBlock         = 9903;  {Duplicate block received}

ecSequenceError          = 9904;  {Wrong block number received}

ecDirNotFound            = 9905;  {Directory not found in protocol transmit}

ecNoMatchingFiles        = 9906;  {No matching files in protocol transmit}

ecLongPacket             = 9907;  {Long packet received during protocol}

ecEndFile                = 9908;  {End of transmitted file}

ecHandshakeInProgress    = 9909;  {Initial protocol handshake in progress}

ecFileRenamed            = 9910;  {Incoming file was renamed}

ecFileAlreadyExists      = 9911;  {Incoming file already exists}

ecInvalidFilesize        = 9912;  {Ymodem header has bad file size}

ecInvalidDateTime        = 9913;  {Ymodem header has bad date/time}

ecUnexpectedChar         = 9914;  {Unexpected char during protocol}

ecBlockCheckError        = 9915;  {Incorrect CRC or checksum received}

ecNoSearchMask           = 9916;  {No search mask specified for transmit}

ecNoFilename             = 9917;  {No filename specifed in xmodem download}

ecAsciiReceiveInProgress = 9918;  {Ascii receive in progress}

ecFileRejected           = 9919;  {Receiver rejected file}

ecTooManyErrors          = 9920;  {Too many errors received during protocol}

ecBadFileList            = 9921;  {No end of list marker found in file list}

{APZMODEM/OOZMODEM status codes}

ecGotCrcE                = 9925;  {Zmodem - got CrcE DataSubpacket}

ecGotCrcW                = 9926;  {Zmodem - got CrcW DataSubpacket}

ecGotCrcQ                = 9927;  {Zmodem - got CrcQ DataSubpacket}

ecGotCrcG                = 9928;  {Zmodem - got CrcG DataSubpacket}

ecGarbage                = 9929;  {Zmodem - got garbage from remote}

ecSkipFile               = 9930;  {Zmodem - skip file}

ecBadPosition            = 9931;  {Zmodem - bad file position}

ecFileDoesntExist        = 9932;  {Zmodem - specified file doesn't exist}

ecCantWriteFile          = 9933;  {Zmodem - not allowed to overwrite file}

ecFailedToHandshake      = 9934;  {Zmodem - never got proper handshake}

ecNoFilesToReceive       = 9935;  {Zmodem - no files to receive}

ecBuffersTooSmall        = 9936;  {ZModem - port buffers too small}

ecGotHeader              = 9937;  {Zmodem - got a complete header}

ecNoHeader               = 9938;  {Zmodem - (internal) no header yet}

{APMODEM/OOMODEM status codes}

ecUnknownModemResult     = 9940;  {Unexpected char in modem result string}

ecConnect                = 9941;  {Modem response - CONNECT}

ecRing                   = 9942;  {Modem response - RING}

ecNoCarrier              = 9943;  {Modem response - NO CARRIER}

ecNoDialTone             = 9944;  {Modem response - NO DIALTONE}

ecBusy                   = 9945;  {Modem response - BUSY}

ecNoAnswer               = 9947;  {Modem response - NO ANSWER}

ecRinging                = 9948;  {Modem response - RINGING}

ecVoice                  = 9949;  {Modem response - VOICE}

ecError                  = 9950;  {Modem response - ERROR}

{APKERMIT/OOKERMIT status codes}

ecGotData                = 9954;  {Kermit - got packet}

ecNoData                 = 9955;  {Kermit - no data yet}

{Archive status messages}

ecUnknownMethod          = 9960;  {Unknown compression method}

ecFileEncrypted          = 9961;  {Cannot extract--file is encrypted}

ecBadFileCRC             = 9962;  {Bad CRC--file is probably corrupted}

ecCannotCreate           = 9963;  {Unable to create output file}

ecBadFileFormat          = 9964;  {Bad archive file format}

ecNotAnLzhFile           = 9965;  {Not an LZH file}

ecNotAZipFile            = 9966;  {Not a ZIP file}

ecEmptyFileMaskList      = 9967;  {File mask list is empty}

{FAX conversion error/warnings}

ecFaxBadFormat           = 9970;  {Not a valid APRO fax file}

ecFontNotSupported       = 9971;  {Bad font format or unsupported feature}

ecHPFontCvt              = 9972;  {Converting font}

ecBadGraphicsFormat      = 9973;  {Bad tiff/pcx format or unsupported feature}

ecBadEscapeSeq           = 9974;  {Bad escape sequence in PCL file}

{CaptureTerminalWindow error codes}

ecScrollBackTooBig       = 9980;  {Scroll back buffer > 64k}

{APBPLUS/OOBPLUS status codes}

ecResync                 = 9985;  {Resyncing with host}

ecWaitACK                = 9986;  {Waiting for ACK}

ecDropout                = 9987;  {Dropout}

ecHostCan                = 9988;  {Host cancel}

ecTryResume              = 9989;  {Attempting resume}

ecHostResume             = 9990;  {Host resuming}

ecResumeOK               = 9991;  {Resumed OK}

ecResumeBad              = 9992;  {Failed to resume}

ecUnPacket               = 9993;  {Invalid packet type}

�����������������������������������������������������������������������ͻ

�  END OF DOCUMENT                                                      �

�����������������������������������������������������������������������ͼ