-----------------------------------------------------------------------------

WSDIAG - Copyright (C) 1992-1995 ACS - ALL RIGHTS RESERVED

Program and source code are protected in whole or part under Canadian / Int'l

copyright laws as an original work.Portions Copyright(C) Novell,Inc. Provo UT

-----------------------------------------------------------------------------

WSDIAG   Version 3.40  (Unregistered)              Copyright (C) 1992-1995 ACS

==============================================================================

Usage: WSDIAG <Station# | ? | 0> SCROLL [ > FILE.OUT ]

or: WSDIAG DUMP <FILENAME>

Station#   -  Valid connection ID for a workstation to diagnose

?          -  This help screen

0          -  Diagnose Local Workstation

SCROLL     -  May be abbreviated S, don't pause for any keystrokes.

> FILENAME -  [optional] Program output will be redirected '>' to the

requested file. Highly recommend using in conjunction with

the SCROLL option.

NOTE: IPX/SPX diagnostic initialization may fail if the your system is

running more than one COMMAND.COM image (ie. A DOS shell). Please ensure that

WSDIAG is running from the master shell (type EXIT at the DOS prompt to check)

Using the DUMP command line will cause a complete dump of all connected

workstation diagnostic tables to a comma delimited file as specified on the

command line. DO NOT use the DOS redirection symbol '>' when using DUMP.

This program is *NOT* free software. It is being distributed under the

shareware "try before you buy" concept.  To register this software and

to obtain the latest version, send cheque or money-order for $75US/copy to:

AccuSys Computer Services

P.O. Box 1215

Fonthill, ON

CANADA   L0S 1E0

Please remember to include your full return mailing address. If you

require additional information, call: 905-734-1992. Specify disk size

(3.5" or 5.25" media) and the name of the program to be registered.

*** Support the ShareWare concept - Send your registration today! ***

*** Compuserve users GO SWREG to register this program ONLINE! ***

Program usage: Type WSDIAG ? for usage help.

New Features:

- 250 user connection limit removed

- Full paged output for connection list

- Connection list now accepts longer user names

- New scroll options, select scroll and STILL use connection pick list

(Command line: "WSDIAG S > FILENAME.EXT") Output will go to file

connection list goes to screen...

- Status messages sent to screen while output scrolling to disk

Future considerations:

- Full diagnostic support for print servers, routers, bridges, etc...

- Full diagnostic support for Netware 2.x servers

- TCP/IP diagnostics/statistics

- SNMP support

Output from the WSDIAG program (!! 7 PAGES of diagnostic info !!)

=================================================================

WSDIAG   Version 3.40  (Unregistered)                Copyright (C) 1992 - ACS

=============================================================================

Basic Information       ID: 000000E1:00003800356C - TOTHDA           Page: 01

Connection Number     : 58

Internetwork Address  : 000000E1:00003800356C

Socket Number         : 4003

Object Name           : TOTHDA

Object Type           : User

Object ID             : 76000001

Login Date            : 03/23/93 (Tuesday)

Login Time            : 14:37:05

Default File Server   : ACS

Server Connection ID  : 1

Netware Revision      : NetWare v3.11 (100 user)       2/20/91

Serial Number         : (deleted from sample output)

Application Number    : (deleted from sample output)

Attempting to open diagnostic session ... successful

WSDIAG   Version 3.40  (Unregistered)                Copyright (C) 1992 - ACS

=============================================================================

Version and Shell Confi ID: 000000E1:00003800356C - TOTHDA           Page: 02

IPX Version           : 3.30

SPX Version           : 3.30

Shell Driver Version  : 1.39

LAN Hardware ID       : 240

Selected Configuration: Novell NE2000 Ethernet

IRQ A, Port 0300

WSDIAG   Version 3.40  (Unregistered)                Copyright (C) 1992 - ACS

=============================================================================

IPX Statistics Info     ID: 000000E1:00003800356C - TOTHDA           Page: 03

Send Packet Count     : 3217

Malformed Packet Count: 0

Get ECB Request Count : 3228

Get ECB Failure Count : 10

AES Event Count       : 26

Postponed AES Events  : 0

Max Configured Sockets: 20

Max Open Sockets Count: 9

Open Socket Failures  : 0

Listen ECB Count      : 3230

ECB Cancel Failures   : 0

Find Route Failures   : 0

WSDIAG   Version 3.40  (Unregistered)                Copyright (C) 1992 - ACS

=============================================================================

SPX Statistics Info     ID: 000000E1:00003800356C - TOTHDA           Page: 04

Max Connections Count : 15

Max Used Connections  : 2

Establish Connect Req.: 1

Establish Connect Fail: 0

Listen Connection Req.: 1

Listen Connection Fail: 0

Send Packet Count     : 6

Window Choke Count    : 2

Bad Send Packet Count : 0

Send Failure Count    : 0

Abort Connection Count: 0

Listen Packet Count   : 10

Bad Listen Packets    : 0

Incoming Packet Count : 15

Bad Incoming Packets  : 0

Suppressed Packets    : 0

NoSession Listen ECB  : 0

Watchdog Destroy Count: 0

WSDIAG   Version 3.40  (Unregistered)                Copyright (C) 1992 - ACS

=============================================================================

Shell Diagnostic Info   ID: 000000E1:00003800356C - TOTHDA           Page: 05

Total Shell Requests  : 131087

Operator Aborts       : 1

Operator Retrys       : 0

Timeouts              : 1

Write Errors          : 0

Invalid Reply Headers : 6

Invalid Slots         : 0

Invalid Sequence Num. : 2

Errors Receiving      : 0

No Router Found       : 0

Being processed       : 0

Unknown Error         : 0

Invalid Server Slot   : 10

Network Vanished      : 0

Alloc can't find route: 15

Alloc no slot avail.  : 0

Alloc server is down  : 0

WSDIAG   Version 3.40  (Unregistered)                Copyright (C) 1992 - ACS

=============================================================================

Shell Driver Statistics ID: 000000E1:00003800356C - TOTHDA           Page: 06

Statistics Version    : 1.0

Total TX Packets      : 3205

Total RX Packets      : 3207

No ECB Available      : 11

TX Packet - Too Big   : N/A

TX Packet - Too Small : N/A

RX Packet - Too Big   : 0

RX Packet - Too Small : N/A

RX Packet - Overflow  : 0

Misc. TX Error        : 0

Misc. RX Error        : 0

Retried TX Count      : 0

Checksum Errors       : 0

Hardware RX Mismatch  : 0

WSDIAG   Version 3.40  (Unregistered)                Copyright (C) 1992 - ACS

=============================================================================

Driver Custom Statistic ID: 000000E1:00003800356C - TOTHDA           Page: 07

TxAbortTimeOut                     : 0

DMAWouldNotComplete                : 0

ReinitializeNIC                    : 0

RxBufferError                      : 0