December 1994

This README File contains some additional information to

supplement the Software Driver Installation Manual/Owners

Guide.

NOTES ON DRIVER INSTALLATION

1. INDICATE THE KIND OF SCSI HOST ADAPTER INSTALLED

Use QUICK INSTALL and make sure you indicate the correct

SCSI host adapter (interface card) you have installed in

your computer:  FUTURE DOMAIN. DD-C5002, IBM or ASPI

Compatible.  The installer will then place the proper driver

into the CONFIG.SYS and the proper files into the AUTOEXEC.BAT.

One of the driver listed below will be installed for the specific

SCSI Host Adapter you indicate:

* DRD600.SYS is the device driver installed for FUTURE

DOMAIN SCSI host adapter (8bit, 16bit and microchannel)  or

the compatiblse Pioneer DD-C5002.

* DRD60PS.SYS is the device driver for IBM PS/2 SCSI Adapter/ A.

* DRD60ASP.SYS is a device driver for ASPI compatible SCSI

host adapters.  It will work with any host adapter that

supports the ASPI protocol and multiple LUNs (Logical Unit

Numbers).  See Point 2 below.  Please check the Pioneer

Device Driver line in the CONFIG.SYS file and make sure the

correct driver has been installed for your SCSI Host

Adapter. See Point 3, below.

2. ABOUT ASPI COMPATIBLE SCSI HOST ADAPTERS

If you are using an ASPI compatible SCSI host adapter, you

must install the ASPI MANAGER for your specific card BEFORE

you install the Pioneer device drivers.  The ASPI MANAGER is

available from the Dealer who sold the card or from the card

manufacturer.

Many ASPI MANAGERS require the "/L" or "/ET parameter to

support Logical Units.  Without Logical Unit support, only

the top disc in the Pioneer CD-ROM Changer can be accessed.

3. ABOUT THE CONFIG.SYS AND THE AUTOEXEC.BAT

CONFIG.SYS

If you are using a FUTURE DOMAIN or the PIONEER DD-C5002

SCSI host adapter, the device statement should load first in

the CONFIG.SYS and look like this:

"DEVICE=C:\PIONEER\DRD600.SYS /DMSCD000"

If you are using the IBM PS/2 SCSI Host adapter, the device

statement should load first and look like this:

"DEVICE=C:\PIONEER\DRD60PS.SYS /DMSCD000"

If you are using an ASPI Compatible SCSI host adapter, the

device statement should load into the CONFIG.SYS after the

ASPI Manager and look like this:

"DEVICE=C:\PIONEER\DRD60ASP.SYS /DMSCD000"

AUTOEXEC.BAT

Make sure the MSCDEX.EXE line is loaded BEFORE WINDOWS or

any MENU program.  It should look like this:

"C:\PIONEER\MSCDEX.EXE /D:MSCD000"

NOTE:  The "/D" parameter in the CONFIG.SYS and the

AUTOEXEC.Bat do not have to read "MSCD000", but they must

match.  You can use something else, such as "MSCD001" for

example, if another driver is using "MSCD000".

4. SCSI ID AUTO SEARCH

The Pioneer QUICK INSTALL program has a feature that will

automatically search the SCSI bus and find the SCSI ID

number of the Pioneer CD-ROM Changer(s) attached.

5. INSTALLING TWO OR THREE PIONEER CD-ROM CHANGERS

If you are installing more than one Pioneer CD-ROM Changer,

on the same SCSI chain, assign each one its own unique SCSI

ID number.  The SCSI ID Auto Search feature will automatically

find all Changers connected on the SCSI bus.

In the Custom Install menu, do not indicate the SCSI ID number for

the Changers.  The SCSI ID Auto Search will find the changers

as long as their IDs have been set to separate distinct numbers.

NOTE:  There are a total of 24 letters available to identify

drives and discs within the system.  Since each Changer

holds up to six discs, if all six discs are accessed in each

Changer, only three Changers can be installed on one SCSI

chain.  To install more than three changers, add a /N:

parameter switch in the CONFIG.SYS and use XDISC.EXE.

As described below in Point 6.

NOTE:  Make sure the Changers in the middle of the SCSI

chain are not terminated, and that the last changer on the

chain is terminated.

6. INSTALLING UP TO SEVEN PIONEER CD-ROM CHANGERS

You can assign one drive letter for each Pioneer CD-ROM

Changer installed on a single SCSI chain by editing the

Device= line in the CONFIG.SYS and using the XDISC utility

included on  the Pioneer Driver Diskette.  First, edit the

CONFIG.SYS Pioneer Device Driver line so it includes a

a /N: switch, set to 0 for each drive to be installed.

See example below:

DEVICE = C:\DEV\DRD600.SYS /D:MSCD000/N:0,0,0,0,0,0/C:CDP

Then, launch XDISC by typing C:\PIONEER\XDISC.  After XDISC

is launched, follow instructions on the screen for accessing

discs in each drive.  For example, type:

XDISC_E_3   (NOTE:   _  is the symbol for pressing the spacebar.)

This will allow access to the third disc in the changer

assigned drive letter 'E'.

NOTE:   Do not set 0 and other numbers together as /N:

settings. To use XDISC, the /N; switch must be set to all 0s.

7. ABOUT MULTIPLE HOST ADAPTERS

If you have more than one SCSI host adapter supported by

ASPI, the AUTO SEARCH function will find the drive. However,

with multiple adapters, the search takes longer. You may

specify the adapter number in CUSTOM INSTALL menus. Again

this can also be indicated in the "/S: parameter. Counting

starts at 0 and as 0 is the default, it does not need to be

specified. To support a drive at SCSI ID 3 on host adapter

1, and a drive at SCSI ID 2 on host adapter 0, the

CONFIG.SYS "/S? switch would look like this:

/S: 3; 1,2

8. FOR THOSE USING IBM SCSI ADAPTER /A  INTERFACE BOARDS

Please Power ON the Changer before you power ON the computer.

The IBM SCSI Adapter /A interface board checks the SCSI

devices when the computer is booted up.  If the SCSI device

doesn't answer correctly, the device will not be available once

the computer has booted up.

Pioneer New Media Technologies, Inc.

2265 E. 220th. Street

Long Beach, CA  90810

Technical Support 800-872-4159

BBS 310-835-7980

LaserFacts 201-327-9273