Observer 2.0

(c) 1994 Network Instruments, LLC

P.O. Box 581156

Minneapolis, MN  55458-1156

(612) 822-2025

FAX (612) 825-5647

BBS (612) 822-0902

TABLE OF CONTENTS

TABLE OF CONTENTS					1

INTRODUCTION TO OBSERVER				7

Functional Overview					8

Observer						8

Netware Discoverer					10

Who Should use Observer and Netware Discoverer		12

Limited Warranty					15

Copyright and License					15

System Requirements					16

Technical Support					17

INSTALLATION						19

Licensing						19

Quick overview for experts				21

Putting the files on your PC:				21

Configuring the DOS component of Observer:		22

Configuring the DOS component of Netware Discoverer:	23

Running Observer and Netware Discoverer:		24

Step by Step Installation:				24

Differences between ODI, NDIS and the Packet Driver	25

Step 1: Configuring ODI, NDIS or a Packet Driver.	27

Procedure 1a: Configuring ODI				28

Procedure 1b: Using NDIS				31

Procedure 1c: Using a Packet Driver			31

Step 2: Copy the Observer files onto the PC		34

USING OBSERVER						41

Overview and quick start				41

Filters							43

Saving a Filter						45

Adding and Editing Hardware Addresses			46

Setting hardware address Filters			47

Choosing a Protocol AS A sub-filter			49

Discovery Mode						50

Monitoring your LAN: Network Statistics			52

Bandwidth Utilization mode				52

General Statistics Information				55

Received Packets Statistics				56

Size Distribution Statistics				58

Protocol Statistics					60

Printing Statistics screens				61

Capturing packets on your LAN:  Packet capture mode	61

Viewing Captured Packets				63

Printing captured packets				67

Saving Captured Packets to a File			69

Settings Dialog						70

Program Flags						74

OBSERVER.EXE						74

VPACK and VPACKODI					75

USING NETWARE DISCOVERER				81

Overview and quick start				81

The Main MDI Window					84

File Menu						85

Options Menu						87

The Preferences dialog					88

Discoverer Discovery Mode				92

Server exclusion List					93

Login to Server						94

The Window Menu						96

The MDI Window Button Bar				96

The List of Nodes View					98

Information Windows (general)				99

Server Information Windows				103

Charts							103

Number Connections (server)				104

Open Files (server)					104

IPX Packet Statistics (server)				104

SPX Packet Statistics (server)				105

Total Packet Statistics (server)			105

Bridge Statistics (server)				106

Volume Usage (server)					109

Options							109

Lists							110

IPX Packet Statistics List (server)			110

SPX Packet Statistics List (server)			113

Total Packet Statistics (server)			117

Bridge Statistics List (server)				119

Volume Information List (server)			121

Open Files List (server)				122

Info:							122

Server Version (server)					122

IPX-SPX Versions (server)				122

Bridge Driver Configuration (server)			123

Node Information Windows				124

Charts							125

IPX Packet Statistics chart (node)			125

SPX Packet Statistics chart (node)			126

Total Node Packets chart (node)				126

Options							127

Lists							127

IPX Packet Statistics List (node)			128

SPX Packet Statistics List (node)			130

Shell Driver Statistics List (node)			134

Shell Statistics List (node)				137

Info							139

OS Version Information (node)				140

IPX-SPX Versions (node)					140

Shell Driver Configuration (node)			140

User Volume Information (node)				142

REFERENCE						145

A Short Overview of TCP/IP for troubleshooting		145

Monitoring TCP/IP Sessions with Observer		148

Ethernet Frame Header					148

Token Ring Frame Header					150

IEEE 802.2 and IEEE 802.2_SNAP subheaders		151

Internet Addresses					152

Address Resolution Protocol (ARP)			153

Internet Protocol (IP)					155

Transport Control Protocol (TCP)			158

User Datagram Protocol (UDP)				162

Internet Control Message Protocol (ICMP)		163

Short Overview of Novells IPX and SPX protocols	166

Internetwork Packet Exchange (IPX) packet header	167

Sequential Packet Exchange (SPX) packet header		168

NetWare Core Protocol (NCP).				171

Service Advertising Protocol (SAP).			176

Routing Information Protocol (RIP).			177

Serialization packets.					179

WatchDog Packets					179

APPENDIX						181

Program Descriptions					181

Troubleshooting						183

Observer Troubleshooting				185

Discoverer Troubleshooting				189

Sample Configuration Files:				190

ODI:							190

NDIS:							191

Packet Driver:						192

Packet Driver Installation Information			192

INDEX							199

INTRODUCTION TO OBSERVER

As Local Area Networks (LANs) become part of

our every day lives, the need to manage and

troubleshoot these LANs becomes increasingly

important.  Most network operating systems have

some ability to diagnose problems but these

abilities usually fall short as truly useful LAN analysis

tools.

Observer is a LAN analyzer (packet viewer and

decoder), network load monitor, Novell network

statistics gathering tool, and much more.

Observer was designed to be useful to both the

average LAN user and the computer professional.

Observer can be used by a system administrator,

network user, programmer, network application

developer, network product tester - basically

anyone who needs to know exactly what

happens on a network, and/or how to make a

network run smoothly.

After using Observer, network administrators will

not be able to imagine how it was possible to

work without it.   Observer allows you to truly "see"

your LAN working.

Observer allows you to keep track of your network

conditions, warns you about a network overload,

and captures packets from a single computer (or

multiple computers) on the network.  Observers

Netware Discoverer collects Novell specific

statistics and displays or graphs them over time.

FUNCTIONAL OVERVIEW

Observer contains two separate programs

designed to complement each other:

Observer is the network analyzer that captures

packets into a buffer, and decodes that capture

buffer.  Observer also includes a number of

statistical collection processes that display

different types of LAN activity over time.  By

design, Observer only "observes" it only listens to

the network, it does not generate any traffic.

Netware Discoverer is a Netware specific

statistical collection tool.  Unlike Observer,

Discoverer continually polls both Novell Servers

and nodes to collect statistics and information.

Because Discoverer polls each station, it is quite

active on your LAN, and (using Observer if you

like) you will see that Discoverer generates its own

small amount of LAN traffic.

Observer

Observer has four main modes.  Each mode gives

you insight into different functions of your LAN in

real time.  Most modes are affected by the filters

you set.

DISCOVERY mode allows you to capture all of the

network addresses on your LAN, store them in the

filters table, and alias them as names or IP

addresses.

PACKET CAPTURE mode displays a graph that

represents your current packet capture state.  This

graph shows you, in real time, how many and

what kind of packets are being captured and

whether packets are being dropped.  Once

packets have been captured, they can be

viewed using the packet captures view mode.

BANDWIDTH UTILIZATION has two modes:  Standard

Bandwidth Utilization allows you to graphically

monitor the amount of your total network

bandwidth that is currently being used - showing

the data in a sliding 8-16 minute graph.  Long

Term Bandwidth Utilization (LTBU) shows

graphically high, low, and average utilization over

a tunable time frame (up to 7 days), and allows

the data to exported to a comma delimited

spreadsheet format for further analysis and

charting.

NETWORK STATISTICS includes Received Packet

Statistics, Size Distribution Statistics, and Protocol

Statistics.  These statistical modes let you see who

is using your LAN and who or what is generating

traffic.  Each modes sheds different light on the

workings of your LAN.

In each of the four modes, there are many filters

and sub-filters which can be used.  You can

choose to trace all protocols on your network, or

choose only a subset of protocols to filter on.

Packet capturing and filtering can help you

localize problems.  Translation of packets into

function and type gives a record and an

explanation of the data exchange on a LAN.  If

you need to thoroughly study packet content

byte by byte, raw mode allows the packets to be

viewed without any translation.  Once packets

are captured, you can filter and print captured

packets to a printer or to a file.

Netware Discoverer

Netware Discoverer provides detailed information

and statistics collected from Novell servers,

stations, bridges and routers.  Discoverer also

provides a Discovery Mode search similar to

Observer's Discovery Mode.  Discoverers

Discovery Mode supplements Observer's

Discovery Mode auto aliasing of IP address to the

hard network address - in Netware Discoverer's

Discovery Mode, you can auto alias a users Novell

login name to the hard network address.

Netware Discoverer supplies information about a

specific Novell Server, and all of the Novell Servers

reachable on the network (both LAN and WAN).

For servers, some examples of this kind information

are: the number of users logged in on a server,

the nodes that are currently down, free/used disk

space on all volumes, number of open files - the

total number and the number used by each user,

and much more.  For users, it gives information

about number of files, directories and disk space

used.

Statistical information from the Server's Bridge

drivers provides information about the general

condition of the network and routing activity

between networks.  IPX and SPX statistics provide

information about a particular node or server,

number of packets sent and received, number of

good and bad packets, ECB availability and

failures.

Lists provide miscellaneous version information

about servers, stations and users, which may be

useful for a network administrator during daily

operation, and particularly for keeping a network

up to date.

Important information that changes over time

can be monitored as it develops in a graphical

format (charts).  These include IPX and SPX traffic

statistics, number of users attached to a server,

volume usage, and bridge statistics.  All of these

graphs are auto scrolling, showing your particular

information graphically, as it develops over time.

Finally, all of this information is provided in a MDI

(Multiple Document Interface), where a system

administrator can simultaneously monitor the

general condition of a network server, and a

number of servers or users in particular - displaying

similar information, or completely different

information, all at the same time in each MDI sub-

window (known as an information window).

Information windows come in two classes; 1)

NODE information windows that display station

(node) information and charts, and 2) SERVER

information windows that display information that

the server provides.

For example, the initial list on the left of the

Discoverer main MDI window (The List of Nodes)

shows a list of servers.  Each Netware server can

be double clicked on (using the LEFT mouse

button) to display a list of users currently

connected to that server (print and other servers

may or may not have users associated with

them).  Clicking on the RIGHT mouse button on a

server or node icon opens the SERVER or NODE

information window (on the right of the MDI

window).

On the right part of the MDI window there can be

a server graphic chart showing network traffic on

a server, and volume information of another

server, a number of NODE windows showing

statistics and charts for different users attached to

different servers.  In essence, it is like a Windows

within Windows, where SERVER or NODE

information windows can be re-sized, placed in

different order, tiled, cascaded, minimized, etc.

With all of this power, an administrator can run

Discoverer and notice very little interruption of the

other functions of the Windows PC - Discoverer

takes very little resource to function.

WHO SHOULD USE OBSERVER AND NETWARE DISCOVERER

Any LAN administrator, network programmer, or

LAN user will find Observer and Netware

Discoverer helpful.  Observers utilities were

designed to meet the needs of the most technical

network professional, as well as to provide

information for an average LAN user.

Observer will be most useful for a network LAN

administrator who  knows his or her LAN workings

from a functional view, but wants to see what is

really going on "on the wire" for troubleshooting

an existing problem or pre-solving a network

condition that could spell disaster in the future.

Observer, like all network analyzers "sees" all

traffic on your LAN segment.  To see and analyze

traffic on a different segment, you will need to

load and run Observer from a PC on that

segment.  If you find this limitation inconvenient or

impractical (your segments are located on a

WAN), you may want to consider upgrading (or

purchasing - if you do not own Observer) to

Network Instruments Analyst/Probe.

Analyst/Probe provides complete Observer

functionality over a multi-segment LAN or WAN.  It

allows you to "see" and analyze up to 253

segments of a LAN or WAN from an Analyst PC.

Probes are software modules that are loaded on

a Windows PC on each segment that the Analyst

PC communicates with via TCP/IP.  Contact your

dealer or Network Instruments for more

information on Analyst/Probe.

Discoverer will be most useful for a Novell Netware

administrator that manages Netware LANs that

are either physically far apart or have elusive non-

reproducible problems that happen over the

course of time and need to be isolated.

Note:  Netware Discoverer is only useful if you

have a Novell LAN.  If you do not have a Novell

LAN, do not choose to install Netware Discoverer

during the installation process.

To set up and use Observer and Netware

Discoverer, you will need to be familiar with both

DOS and Windows, including editing files,

changing  configurations, and have a basic

knowledge of your LAN setup.

Observer contains both DOS and Windows

programs.  Observer must be installed on at least

one PC on each network segment to analyze

network traffic.  For Observer installation, a

network card is required for each PC.  No

additional hardware is required on the Observer

PC.

Netware Discoverer requires LSL.COM,

IPXODI.COM and the VLMs (VLM.EXE and any

other components your VLM is configured to use)

to be installed and running.  Additionally, you will

need to have Novell's Windows Network support

drivers and DLLs properly (and completely)

installed on your PC (NETWARE.DRV, NWCALLS.DLL

and NWIPXSPX.DLL and all of the DLLs, DRVs and

VxDs and *.386s that they call need to be

present in your Windows SYSTEM subdirectory).  If

you are using Microsoft Windows for Workgroups

(WFW), you will need to make certain you are

using WFW over ODI, and make sure that you

have set up Windows Networking to recognize the

Netware Shells.

The Netware Windows drivers and DLL's are

proprietary programs of Novell Inc., and are either

included with your Novell system or you can

download the latest versions of the Drivers and

DLL's from Internet (ftp.novell.com), from

Compuserve, or ask your Novell dealer for the

latest VLMUPx, WINDRx and NWDLLx (where "x" is

the version number).

LIMITED WARRANTY

Network Instruments, LLC will replace defective

media or documentation for 90 days after the

shipment of the product from Network

Instruments, LLC.  Should Network Instruments, LLC

release a new version of Observer within 60 days

of shipment of the product, Network Instruments,

LLC will update the copy of Observer at no

charge.  This update may consist of disks or a

manual or both, at the discretion of Network

Instruments, LLC.

The Observer license does not guarantee the

usability of Observer, expressed or implied.  In

cases of loss, destruction, or corruption of data,

Network Instruments, LLC shall not be liable.

Network Instruments, LLC makes no other

warranty.

COPYRIGHT AND LICENSE

Observer (including Netware Discoverer) is neither

shareware nor freeware.  Observer is a

commercial software package that is subject to

international copyright laws.

Upon registration of the 1-25 Node LAN version of

the Observer product you are licensed to use

Observer at one site on one LAN, if and only if that

LAN has less than 25 nodes connected in total.  If

you have a LAN with more than 25 nodes, you will

need the 25+ Node LAN version of the Observer

product.  The 25+ Node LAN Observer product is

also licensed for one site and one LAN.

If you have multiple LANs (LANs which are not

connected to each other) or multiple physical

sites (different locations or buildings), you will

need to purchase additional copies of Observer.

Multiple site licenses and multiple LAN licenses are

available at a reduced license fee.  Contact

Network Instruments for more information.

Observer is the property of Network Instruments,

LLC and may not be copied for purposes other

than backup.

Depending on how you purchase Observer you

will either have a RTU (Right To Use) license, or

upon registration of the DEMO you will receive a

RTU.  In either case the RTU is your proof of

purchase.  You will need to produce this

document for upgrades and you will need to

provide the RTU activation numbers to receive

technical support.

This software is licensed as stated above.  The

license does not constitute ownership of the

software, only the right to use it.

SYSTEM REQUIREMENTS

Minimum: 486SX/25 w/4MB RAM, Windows 3.1, a

supported network adapter, and a VGA color

monitor.  Note:  If you are using Windows for

Workgroups, the minimum RAM required is 8MB.

Recommended: 486DX/25 (or faster) w/16MB

RAM, Windows 3.1 (or 3.11), a 16 bit network

adapter, a fast 16 bit VGA adapter, and a 15"

VGA color monitor running in at least 800x600

resolution.

Observer may run on systems with less power and

RAM than those listed above, but Network

Instruments does not support configurations with

less than the minimum stated requirements.

TECHNICAL SUPPORT

Network Instruments provides technical support

from 9:00 AM to 5:00 PM Central Standard Time.

Technical support is provided on the phone at

(612) 822-2025, by fax at (612) 825-5647, or by

email at support@netinst.com.

Before calling technical support please read the

troubleshooting section at the end of this manual,

and the README.WRI on your distribution disk to

see if your problem has not already been

answered.

INSTALLATION

LICENSING

Observer is always sold in a DEMO version.  The

DEMO version can be licensed to become a fully

functioning product.

Observer will run in DEMO mode until it is licensed.

The DEMO mode will simulate LAN traffic and

does not require a LAN or LAN hardware to be

present.  If you choose to install Observers

Netware Discoverer, it will run in DEMO mode

"unlicensed" for 5 minutes, and then it will disable

itself (Note: Netware Discoverer DOES require a

network adapter and ODI to be loaded even in

DEMO mode).

The DEMO mode is provided so that a potential

Observer user can get a feel for the package

without having to purchase it.  You can turn a

DEMO version of Observer into a licensed version

with a key number as described below.

This key number is unique and can only be used

for one copy of Observer and will only work for

one site.  The key number will either enable

Observer for use on a 1-25 node LAN or a 25+

node LAN, depending on the license fee paid.  A

1-25 node license key number will not work on a

25+ node LAN.

To turn the DEMO version of Observer into a full

functioning licensed version, you will need to fill

out the license dialog with your name and your

company name to generate your unique

customer registration number.  To bring up the

license dialog, choose "File" -> "License" from the

main Observer (or Discoverer) screen.

Once the licensing information has been filled out,

you will need to fax your company registration

number (and payment - depending on how you

purchased Observer) to Network Instruments to

receive your key number.  The key number will

turn your DEMO copy of Observer into a fully

functioning copy of Observer.  Network

Instruments FAX number is (612) 825-5647.

Depending on where and how you purchased

Observer, you may have a "Right to Use" (RTU)

certificate.  If you have purchased such a

certificate, follow the instructions on the Right to

Use document to license Observer.  If a Right to

use certificate came with your copy of Observer,

follow the instructions on that Right to Use

document to license Observer.

Once you have licensed Observer, Observer and

Observers Netware Discoverer will automatically

be licensed.

QUICK OVERVIEW FOR EXPERTS

Putting the files on your PC:

Run the Observer installation program from

Windows by putting the Observer disk in your

floppy drive and running the program

A:\INSTALL.EXE from the Windows "File" -> "Run"

menu.

Install will ask you which directory you want

Observer to be installed into.  Unless you have a

compelling reason to install Observer elsewhere,

we suggest that you install Observer in the default

destination.

INSTALL lets you choose between DEMO mode

(check the DEMO box) or licensed mode (un-

check the DEMO box).  As mentioned above,

DEMO mode will simulate network traffic so that

you can see Observer functioning as if it were on

a real LAN.

When installing in DEMO mode, you will not be

given the chance to install Netware Discoverer

(see note below for more information on

DEMOing Netware Discoverer).

Once the product is licensed, and the key

number is provided, Observer will capture data

from your LAN, and work without any of the DEMO

limitations.

If you install in licensed mode, Observers

installation program will offer to make the required

changes in your AUTOEXEC.BAT (and other

required files).  Observers install will also ask you if

you would like to install Netware Discoverer -

make the appropriate choice.

Note:  If you do not have a Novell LAN, do not

install Netware Discoverer.  If you are not using

ODI, you cannot use Netware Discoverer.  See

note below.

Make sure to check the README.WRI for any late

breaking installation information.

[NOTE:  Because Netware Discoverer requires ODI,

including LSL, IPXODI, VLM support and ALL of the

Netware Windows DRVs, DLLs, 386s and VxDs, it

is not automatically installed during DEMO

installation.  If you would like to DEMO Netware

Discoverer, and are confident that you have your

Windows PC properly configured as described

above, uncheck the DEMO box and install

Observer in licensed mode, including Netware

Discoverer.  Both Observer and Netware

Discoverer will run in DEMO mode if not licensed.

For Observer this means it will play simulated data,

for Netware Discoverer this means that it will only

function for 5 minutes, but it will see real data from

your LAN.)

Configuring the DOS component of Observer:

If you are installing in licensed mode, Observers

Windows programs require a DOS TSR that

interfaces with your network driver and makes

packets available to the various Windows DLLs

and programs.  This TSR can take less than 5K of

RAM.

Observers installation program will ask if you

would like to use ODI, NDIS, or the Packet Driver.

If you choose ODI, INSTALL will install

VPACKODI.EXE in your AUTOEXEC.BAT for you.

If you choose NDIS, INSTALL will install

DISPKT10.DOS, modify your PROTOCOL.INI,

SYSTEM.INI and install VPACK.EXE into your

AUTOEXEC.BAT.

If you choose the Packet Driver, INSTALL will install

VPACK.EXE in your AUTOEXEC.BAT for you.

ODI, NDIS, and Packet Driver installation methods

expect that your network is up and running and

configured correctly for a protocol analyzer to

function.  If you run into troubles, see the step by

step installation for your particular network setup

for help.

(For ODI, see step 1a below for an example of a

NET.CFG to use with VPACKODI.EXE.]

Configuring the DOS component of Netware Discoverer:

If you choose to install Netware Discoverer, it

requires: ODI and the VLMs to be installed and

running.  Additionally, you will need to have

Novell's Windows Network support DRVs, VxDs,

386s, and DLLs properly (and completely)

installed on your PC.  For more information on

exactly what is required see the  "If you are using

Netware Discoverer" section under procedure 1a.

Note: If you are using Microsoft Windows for

Workgroups (WFW), you will need to make certain

you are using WFW over ODI.

If installing in licensed mode, install will then ask to

reboot your PC.

Running Observer and Netware Discoverer:

Once you have rebooted and VPACK.EXE (or

VPACKODI.EXE) has been loaded, you can start

Observer by double clicking on the Observer icon

in the Observer group.  Start Netware Discoverer

by double clicking the Netware Discoverer icon.

If Observer fails to load with an error indicating

that you do not have enough DOS memory to

allocate VPACK (or VPACKODIs) buffer space,

see the section on Program Flags later in this

manual, or the troubleshooting section in the

Appendix.  This describes the way to lock VPACK

(or VPACKODIs) buffer space prior to going into

Windows before other applications can steal it

away.

STEP BY STEP INSTALLATION:

The installation of Observer will involve the

following 4 steps:

1. Decide which network stack you are

using/going to use.  Prepare that stack if

necessary.

2. Run INSTALL.EXE from Windows.

3. Decide if you will install Netware Discoverer.

4. Reboot your PC.

If you are installing Observer in DEMO mode, you

will not need to go through Step 1, you can go

directly to Step 2.

This installation procedure expects that you have

your network adapter installed and configured

properly.  If you are able to connect to your

network and run your network software, you

probably have the network adapter installed and

configured properly.

If you are using ODI to interface with your network

adapter, you may need to modify your NET.CFG

ODI configuration file.  If you are using NDIS, make

sure that your NDIS network is up and running.  If

you plan to use the Packet Driver, you will need to

know what kind of network adapter you will be

using and the various hardware settings of your

network adapter for step 1.

Note: If you plan on using Netware Discoverer,

you must use ODI.

Differences between ODI, NDIS and the Packet Driver

If you are using Novell, we recommend that you

use ODI as your network access method.  This

includes Windows for Workgroups with Novell.  If

you plan on using Netware Discoverer you MUST

use ODI and set up the VLMs (if you do not know

what the VLMs are, see you Novell supplier, or

contact Network Instruments support

department).

If you are using Windows for Workgroups without

Novell, we recommend that you use NDIS for your

network access.

If you are not using either ODI or NDIS we

recommend that you use the Packet Driver for

your network access.

ODI is Novells method for accessing the network

from a DOS or Windows PC.  ODI was designed

with network monitors in mind and provides a

complete interface for Observer while allowing

other network traffic to flow.  Because of this, ODI

is the preferred method for running Observer.

If you use ODI for your network interface,

VPACKODI.EXE will work in conjunction with

LSL.COM so that you can concurrently use both

Observer and your Novell (or any other) network

software.

NDIS also allows you to use most network shells

and Observer at the same time.  Some network

software (specifically some TCP/IPs that use

blocking calls may not run concurrently with NDIS

and Observer).  If you experience trouble with

Observer and your other network, try not loading

your other network software while running

Observer.  Note:  When using NDIS, Observer will

suspend all packet receives when you switch to a

full screen DOS prompt.

Packet Drivers are small, efficient network card

drivers that virtualize the network adapter from

the network software.  This allows network

software such as Observer to access the network

without specific knowledge about the network

adapter itself.  While it is possible to use a Packet

Driver with Novell, we suggest that you use ODI if

you have a Novell network.  It is not possible to

use a Packet Driver with Windows for Workgroups

if you are using Windows for Workgroups

networking.  You can not use Observer with Token

Ring and a Packet Driver.

If you use the Packet Driver interface (assuming

you do not have Novell or Windows for

Workgroups), VPACK.EXE will take all incoming

packets.  VPACK.EXE can not be run with any

other network software shells.  This means that you

will not be able to concurrently use Observer with

any other network software (i.e. IPX.COM, and

NETX.EXE) at the Observer workstation.  This will not

affect the use of network software on any other

workstations.

For all three methods (ODI, NDIS, and Packet

Driver) it not recommended to run a full screen

DOS prompt while Observer is trying to capture

information.

Step 1: Configuring ODI, NDIS or a Packet Driver.

Note:  If you are installing Observer in DEMO

mode, go directly to Step 2.

This step is potentially the most complicated part

of setting up Observer.  Your procedure will

depend on what kind of a network you have.

If you have ODI, you may need to modify your

NET.CFG file.  This is explained in Procedure 1a.

If you are using NDIS you will only need to verify

that your LAN is up and running.  Go to Procedure

1b

If you decide to use a Packet Driver, you will need

to select and configure the Packet Driver.  This is

described in Procedure 1c.  Procedure 1c is for

people who are using a Novell network pre-3.x/4.x

(Netware 2.x) or for those of you using a TCP/IP

product that uses the Packet Driver.

Procedure 1a: Configuring ODI

This procedure assumes that you have a Novell

LAN and a pre-existing ODI setup.  This procedure

describes what VPACKODI.EXE (Observers ODI

interface) needs to have configured in your

NET.CFG file.  Note:  If you do not have a NET.CFG

file, you will need to create one.  This file is the

configuration file for LSL.COM and is usually in the

same directory as LSL.COM.

If you are not sure whether you have an existing

ODI configuration or not, check your

AUTOEXEC.BAT or your STARTNET.BAT file for the

following files:

LSL.COM

IPXODI.COM

NETX.EXE (or VLM.EXE)

If these files are being loaded on your system, you

are using ODI.

ODI uses the configuration file NET.CFG to give its

programs and drivers information about the

network protocol settings and network card driver

settings.  You may need to add to your NET.CFG

FRAME designations for each of the frame types

that you will want to capture and examine.  Your

NET.CFG may have one, some, or all of these

types already.  For Ethernet add the ones that are

not in your NET.CFG but are listed below (for

Token Ring, check the Appendix under "Sample

Files").

A sample NET.CFG file for a NE2000 network

adapter might look like this (yours may be

different):

LINK DRIVER NE2000

INT 5

PORT 300

FRAME ETHERNET_802.3

FRAME ETHERNET_II

FRAME ETHERNET_802.2

FRAME ETHERNET_SNAP

LINK SUPPORT

BUFFERS 15 4028

MEMPOOL 4096

Observers VPACKODI.EXE (this will automatically

be installed in step 2) will capture and make

available to Observer all protocols for each

FRAME line in your NET.CFG.  VPACKODI.EXE will

not see any FRAME types that are missing from

your NET.CFG.  We recommend adding all frame

types unless you specifically know that you will not

be using a particular one.  Note: If you are

planning to monitor TCP/IP, you must have the

ETHERNET_II frame type listed in your NET.CFG.

[See the Appendix for a sample NET.CFG for

Token Ring.)

If you are using Netware Discoverer

For Discoverer to work properly it requires:

LSL.COM, IPXODI.COM and VLM.EXE to be

installed and running (this includes any *.VLMs

that VLM.EXE may require).  Additionally, you will

need to have Novell's Windows Network support

drivers (*.DRVs,  DLLs, VxDs and 386s) properly

(and completely) installed on your PC

(NETWARE.DRV, NWCALLS.DLL and NWIPXSPX.DLL

and all of the drivers that these files call need to

be present in your Windows SYSTEM subdirectory).

You must also configure Windows to recognize

the Netware shells.  This is done in Setup for

Windows 3.1, and in the Network dialog in WFW

(3.11).

The Netware Windows drivers and DLL's are

proprietary programs of Novell Inc., and are either

included with your Novell system or you can

download the latest versions of the Drivers and

DLL's from Internet (ftp.novell.com), from

Compuserve, or ask your Novell dealer for the

latest VLMUPx, WINDRx and NWDLLx (where "x" is

the version number).

Once you have modified your NET.CFG file (and

verified the complete VLM installation if you are

using Discoverer), you can now proceed to Step

2.

Procedure 1b: Using NDIS

Since Observers installation program sets up all of

the necessary NDIS configuration files for you, the

only thing you need to do at this point is to verify

that your NDIS LAN is up, and functioning.

You can verify that your NDIS LAN is running by

starting Windows, and logging into your LAN.

Make sure that you can send and receive

information.

Once you have verified that your LAN is up and

running, you can now proceed to step 2.

Procedure 1c: Using a Packet Driver

This procedure is for installations with any kind of

network EXCEPT Novell (using ODI), Novell with

ODI and Windows for Workgroups, Windows for

Workgroups with NDIS, or any Token Ring.  In this

procedure you will use a program called a Packet

Driver to provide VPACK.EXE with an interface to

your network adapter.

Packet Drivers are small, fast programs that

separate your network adapter from your network

software.  This allows one version of Observer to

work with all kinds of network adapters.  Packet

Drivers are freeware and a collection of them are

included with Observer.  You will find both the

Packet Driver collection and a detailed installation

guide (INSTALL.DOC) for Packet Drivers in the

\DRIVERS directory of the Observer distribution

disk.  A subset of the INSTALL.DOC is located in the

Appendix of this manual.

Find the Packet Driver for your network adapter in

the DRIVERS directory on the Observer distribution

disk.  Packet Drivers are named in a manner that

should allow you to determine which  driver to use

with which card.  For example, the packet driver

NE2000.COM is for use with a Novell NE2000

network card (or an NE2000 clone).  For a

complete list of all the Packet Drivers, see

INSTALL.DOC located in the \DRIVERS directory.

In this example, we will use the NE2000 Packet

Driver.  This Packet Driver requires three pieces of

information about your network card: 1) The

software interrupt, 2) The IRQ (interrupt), and 3)

The I/O address (also known as base memory).  A

typical Packet Driver will have the following

invocation format:

NE2000  [packet_int_no]  (irq)  (i/o)

(other_paramiters)

Where:

packet_int_no: (Packet Interrupt Number) Is the

software vector that the first network program will

use to connect to the Packet Driver.  This can

range from 60-79, but it will usually be 61.  Note

that this flag is represented in hex notation, so 61

would be represented by 0x61.

irq: (IRQ) Stands for "Interrupt ReQuest" and is

often just called interrupt.  This must be unique for

each card in your PC and can not, under any

circumstances, be shared with any other adapter.

Some good choices are 5, 10, 11, or 12.  Some

very bad choices are 2, 3,4,7, or any number

above 13.  Keep in mind the "right" choice is

whatever is set on your network adapter card,

and not used by any other adapter.  To determine

the IRQ, you may need to open up your PC and

look at the jumper setting on the adapter itself, or

run the cards diagnostics to find out what the IRQ

is set to in the cards setup memory.  This value is

not represented in hex so an IRQ of 5 would be

represented by 5.

i/o (I/O) Is the I/O or Port that the card uses.  This is

also set on the card and can not be used by any

other adapter in your PC.  The default for the

NE2000 card is 300 and thus would be represented

in hex notation by 0x300.

other parameters:  Some network adapters

(notably the SMC/WD family) use shared memory.

Check your Packet Driver for any additional

parameters required and look in the INSTALL.DOC

in the appendix of this manual for your specific

cards requirements.  [Another way identify what

your specific Packet Drivers requirements is to run

the Packet Driver at the DOS prompt without any

parameters.  This will show you a copyright and a

help screen that lists all of the required parameters

for that specific Packet Driver.]

For our example, the NE2000 command line might

be (this may not match your cards set up exactly):

NE2000 0x61 5 0x300

When executed, this is what you might see:

Packet driver for NE2000, version 11.4.3

Packet driver skeleton copyright 1988-93, Crynwr

Software.

This program is freely copyable; source must be

available.

System: 80[345]86 processor, ISA bus, Two 8259s

Packet driver software interrupt is 0x61 (97)

Interrupt number 0x5 (5)

I/O port 0x12C (300)

My Ethernet address is 08:00:00:12:07:50

If you get any errors, (i.e. an Ethernet address of

all FFs or all 0s, or a usage screen), the Packet

Driver did not load correctly.  Check your settings

and try again.  If you still have troubles, see the

troubleshooting section in the Appendix of this

manual.

Once the Packet Driver has been loaded, you

can now proceed to step 2.

Step 2: Copy the Observer files onto the PC

Run Windows and click on "File"->"Run".  In the

"Run" dialog box, fill in the path to the executable

A:\INSTALL.EXE.

You can now choose to install Observer in DEMO

mode or in licensed mode.  To install in DEMO

mode, check the DEMO box at the bottom of the

install dialog.  To install in licensed

mode, uncheck the DEMO box.

The install dialog box will then ask if you want

Observer installed in the C:\OBSERVER directory.

Unless you have a compelling reason to install

Observer elsewhere, we suggest that you install

Observer in this default destination.

If you chose to install in DEMO mode, you will not

be asked about network interfaces or to install

Netware Discoverer.  You can now go to the

Using section.

If you are installing in licensed mode, you will be

asked if you are sure you want to install in licensed

mode, if so, answer YES.

You will then be queried if you would like to install

Netware Discoverer.  If you do not have a Novell

LAN do not install Netware Discoverer (uncheck

the box at the bottom of the dialog).  If you do

have a Novell LAN read the dialog carefully, and

decide if you want to install Discoverer or not, if so

leave the "Install Netware Discoverer" checkbox

checked.

Install will now copy the Observer files.  Once this is

complete, install will ask you whether you want to use

ODI, NDIS or the Packet Driver Choose whichever method

you would like to use.

All choices (ODI, NDIS, or Packet Driver) expect

that you have completed Step 1 successfully.

If you choose ODI, Observers installation program

will offer to make the necessary changes in your

AUTOEXEC.BAT file.  If you tell INSTALL not to

modify your AUTOEXEC.BAT file, you will need to

run VPACKODI.EXE or add it to your

AUTOEXEC.BAT for Observer to function. Sample

files are given in the Appendix of this manual.  If

your PC uses DOS 6.xs multiple boot setup,

Observers installation program will create a file

called AUTOEXEC.INS in your Observer directory

with the required load lines to run Observer.  You

should copy these load lines into the appropriate

section of your AUTOEXEC.BAT.

If you choose NDIS, Observers installation

program will offer to make the necessary changes

in your AUTOEXEC.BAT, PROTOCOL.INI, and

SYSTEM.INI files.  If you tell INSTALL not to modify

your files, you will need to run VPACK.EXE or add it

to your AUTOEXEC.BAT for Observer to function.

You will also need to modify your PROTOCOL.INI

and SYSTEM.INI files.  Sample files are given in the

Appendix of this manual.  If your PC uses DOS 6.xs

multiple boot setup, Observers installation

program will create a file called AUTOEXEC.INS in

your Observer directory with the required load

lines to run Observer.  You should copy these load

lines into the appropriate section of your

AUTOEXEC.BAT.

If you choose to use the Packet Driver, INSTALL will

offer to make the necessary changes to your

AUTOEXEC.BAT file.  If you tell install not to modify

your AUTOEXEC.BAT file, you will need to run

VPACK.EXE or add it to your AUTOEXEC.BAT for

Observer to function.  If your PC uses DOS 6.xs

multiple boot setup, Observers installation

program will create a file called AUTOEXEC.INS in

your Observer directory with the required load

lines to run Observer.  You should copy these load

lines into the appropriate section of your

AUTOEXEC.BAT.

Once the Observer files have been installed on

your PC, and network access method

configuration files modified, INSTALL will ask to

reboot your system.  Once you reboot your

system, you will be able to start Windows and run

Observer by double clicking on the Observer icon

in the Observer group.

If Observer fails to load with an error indicating

that you do not have enough DOS memory to

allocate VPACK (or VPACKODIs) buffer space,

see the section on Program Flags later in this

manual, or the troubleshooting section in the

Appendix.  This describes the way to lock VPACK

(or VPACKODIs) buffer space prior to going into

Windows before other applications can steal it

away.

USING OBSERVER

OVERVIEW AND QUICK START

Observer has four modes of operation:

1) DISCOVERY Mode allows you to capture all of

the network addresses on your LAN, store them

in the filters table, and alias them as names or

automatically alias them as IP addresses.

2) PACKET CAPTURE mode allows you to capture

packets.  Once  packets have been captured,

you can view the capture buffer contents or

filter and print the results.  The packets

captured will depend on the filters you set in

the "Filters" dialog.

3) BANDWIDTH UTILIZATION has two modes:  Standard

Bandwidth Utilization allows you to graphically

monitor the amount of your total network

bandwidth that is currently being used -

showing the data in a sliding 8-16 minute

graph.  Long Term Bandwidth Utilization (LTBU)

shows graphically high, low, and average

utilization over a tunable time frame (up to 7

days), and allows the data to be exported in

comma delimited spreadsheet format for

further analysis and charting.

4) NETWORK STATISTICS includes Received Packet

Statistics, Size Distribution Statistics, and

Protocol Statistics.  These statistical modes let

you see who is using your LAN and who or

what is generating traffic.  Each mode sheds

different light on the workings of your LAN.

Packet Capture mode and Network Statistics

mode allow filtering by address or protocol or

both.  You can set filters from the "Filters" dialog

from any Observer screen.  Bandwidth Utilization

mode does not use any filters - it reads and

records all packets regardless of type or address.

[Note: if using ODI, Bandwidth Utilization will only

see registered FRAME types as described by your

NET.CFG file.]

When using Bandwidth Utilization mode, you must

set the maximum theoretical network bandwidth

of your network in the "Modes" -> "Settings"

dialog box (default is set 10MB/s - the speed of an

Ethernet network).  For more information about

the "Modes" -> "Settings" dialog, see the Settings

section of this manual.

Note:  Windows provides a multitasking-tasking

environment for applications - sort of.  Windows

3.x multitasking-tasking is non-preemptive single

threaded multitasking-tasking.  This means that

you can choose to run other applications while

Observer is running, but depending on the speed

of the PC, the amount of filtering you are doing,

and the load that the other applications put on

the PC, Windows may not give Observer the

processor time it requires and thus Observer may

drop packets.  This is most likely to be a problem

on a slow PC, in a heavily trafficked LAN with a

very large capture taking place.  If you believe

that you are not seeing all of the packets that you

should, or if Observer is reporting dropped

packets, try running Observer without any other

Windows applications active.

The Network Statistics modes give you an alternative way

to discover (this is different from Discovery Mode)

network addresses on your LAN and to copy an address to

the clipboard for use in the filter and alias dialog

boxes.  You can do this by double clicking on any

network address - this will copy it to the Windows

clipboard. FILTERS Before you use Observer you will need

to familiarize yourself with the Filters dialog box.

The setting and use of filters is central to the

functioning of Observer.  If you do not set a filter,

Observer will capture all packets and all protocols.  In

other words, setting no filter is just like setting an

ANY_ADDRESS to/from ANY_ADDRESS address filter with an

"All Protocols" sub-filter.

Since LAN traffic can generate a tremendous

amount of data, we suggest that you filter out as

much as possible to make the interpretation of

the captured data easier.

Filtering allows you to choose an address or set of

addresses (an "address filter") and a protocol or

set of protocols ("a protocol sub-filter") to filter the

capture with, and to display packets with.  These

filters and sub-filters will sort packets and capture

only those that you wish to capture.

The "Filters" -> "Current Filter" dialog box allows

you to select one to five specific computers (or

hosts) by address, for monitoring network traffic or

statistics.  See "Saving a Filter" later in this section.

You can select "ANY_ADDRESS" and one

computers address to get information from that

computer to any address or from any address to

that computer.

You can also select "ANY_ADDRESS" to and from

() "ANY_ADDRESS" to activate a "filter" that

sees all addresses, and thus all LAN activity.

(Within the parameters of your protocol sub-filter

and, if using ODI, your FRAME specifications.)

Filtering can be done by address or protocol or both.

You can select a specific protocol that you want to

monitor or choose to monitor all protocols present on

your network.  Observer currently supports specific

translation of the IP and IPX/SPX suite of protocols.

Other protocols are viewed in raw mode.

After you make changes in the Filters dialog box,

you can either click on the "O.K." button to save

and enable the changes or "Cancel" to discard

all changes and preserve previous filter settings.

Filters are stored in the file OBSERVER.FLT in

Observers installation directory.  This file is a text

file and can be edited with any text editor.

Saving a Filter Once you have set a filter, it can be

saved as a named filter that can then be selected at a

later time using the "Filters" -> "Select Filter"

dialog. You can save and name up to 5 different filters

to be recalled at a later time.

Filter names can be edited by clicking in the

name section of the dialog.

Changes are kept in a text file in your Observer

installation directory called OBSERVER.FLT. Adding and

Editing Hardware Addresses The hardware address listbox

shows the addresses (and aliases) you may want to

monitor.  You can create as many entries as you are

licensed for, however you can only set a filter to

monitor up to 5 addresses at a time.  See the section on

Discovery mode for more information about adding network

addresses

The format of an address entry is either the six

numbers of the Ethernet address separated by

colons or dots, or the Token Ring address.  This

may be followed by a space then an alias up to

17 letters long.  An alias is a name that Observer

will substitute for an address when showing the

headers of incoming packets (if you tell Observer

to use aliases).  This can make packets easier to

recognize and analyze.

For example:

00:02:8A:49:B2:48    David Jones

To manually add a hardware address, click on the

edit line at the top of the Filter dialog box and

enter the hardware address and press <ENTER> or

click on a down arrow at the end of the edit line.

The entry will be added to the list.  To edit an

existing entry, select that entry using your mouse

and click on the "Edit" button to the right from the

hardware addresses list or simply double-click on

the line you want to edit.  The address will appear

in the edit line and you can make all necessary

changes and save it as described above.  You

cannot edit or change the "ANY_ADDRESS" entry.

To remove an address entry, highlight the address

by a clicking on it and click on the "Remove"

button.

Setting hardware address Filters

Observer filters allow you to capture packets

coming from one hardware address to another -

in one or both directions:  All incoming packets to

a particular address from any source, all outgoing

packets from a particular address to any

destination, or all the traffic on the network -

subject to the protocol sub-filter.

To set up a filter, select one of the five lines in the

three boxes below the hardware address with a mouse

click.  Then click on an address you want to monitor in

the hardware address box and click on the "Add" button

over right or left filter sub-box.  Repeat the above

procedure for the second address.

Once you have added both addresses, you can

choose the direction to capture.  Choose "->" to

only see packets from the left address the right

address on the filter line.  Choose "" to only see

packets from the right address to the left address

on the filter line.  Choose the "" to see packets

from or to the right and left addresses on the filter

line.

If you want to capture all packets in one or both

directions - select the line "ANY_ADDRESS" in the

hardware addresses box and add it to the filter

line.

If you want your current filters to be loaded on

startup of Observer, check the "Enable current

filters on startup" box, otherwise Observer will start

up with no filter and thus default to a capture all

packets state.

Choosing a Protocol AS A sub-filter

Observer allows you to select a protocol that you

want to monitor as a sub-filter.  The choices in this

list will depend on whether you are using Ethernet

or Token Ring.  For Ethernet you have 5 choices of

protocol sub-filters:

All protocols (no filtering by protocol)

TCP/IP set of protocols (TCP,UDP,ICMP,....)

IPX-SPX ETHERNET II (Novell)

IPX - IEEE 802.3 (Novell)

IPX - ETHERNET II + IEEE 802.3 (Novell)

For Token Ring, the protocol sub-filter list will show:

All protocols (no filtering by protocol)

TOKEN-RING_SNAP - all sub-protocols

TOKEN-RING_SNAP - TCP/IP set of protocols

TOKEN-RING_SNAP - IPX/SPX set of protocols

TOKEN-RING - all sub-protocols

TOKEN-RING - IPX/SPX set of protocols

MAC frames only

To choose a protocol, click the arrow on the right

of the "Protocol:" line at the bottom of the

"Filters" dialog box.  Select the option that best

filters the situation you wish to monitor.  It is best to

do as much filtering as possible when trying to

isolate network problems.

Note: Observer can analyze many more protocols

than it allows you to filter.  If your particular

protocol is not present in the specific list of

protocols, just choose the "All Protocols" (no

filtering by protocol) line.

DISCOVERING HARDWARE ADDRESSES ON YOUR LAN:

DISCOVERY MODE

Discovery mode allows you to capture all of the

network addresses on your LAN, store them in the

filters table, and alias them as names or IP

addresses.

Observer allows you to add addresses manually or

to "discover" addresses on your LAN with the

Discovery mode dialog.  To start Discovery mode

choose "Modes" -> "Discovery Mode" from the

menu.

Once you are in Discovery mode, you can have

Observer "listen" to your LAN and record all of the

active network addresses.

By pressing the start button on the right of the

screen, Observer will begin to collect all of the

active addresses on your LAN segment.

Addresses will be added immediately as each

station accesses the LAN (up to the number of

addresses you are licensed for).

Once you have collected the addresses you are

interested in saving, you can save them by

clicking on "Save All" or highlight just a few

addresses using your mouse, and save only those.

The addresses will be saved in the OBSERVER.FLT

file.  They will then be available for use as an

address in the Filters dialog.

You can choose to alias addresses at this point, or

you can wait until you need to use the address

and alias it in the Filters dialog.

Clicking on the "Load list of addresses at startup"

loads all of the addresses you have stored in your

OBSERVER.FLT file when going into the Discovery

mode dialog.  If you start Discovery mode with

"Load a list of addresses", then clicking on the

"Highlight New" button will display all of the

addresses that are newly discovered in this

Discovery mode session.

If you are using TCP/IP, Observers Discovery Mode

allows you to automatically fill the alias of an

address with the IP address of that station.  To

activate this feature check the "Fill new addresss

alias with IP address" checkbox at the bottom of

the Discovery Mode dialog.

MONITORING YOUR LAN: NETWORK STATISTICS

Observer has four modes which allow you to

monitor and observe different aspects of your

LANs movement of data across the wire:

Bandwidth Utilization (short and long term),

Received Packets Statistics, Size Distribution

Statistics, and Protocol Statistics.

Bandwidth Utilization mode

Observer provides two different bandwidth

utilization modes.

1. Bandwidth Utilization - this displays a graph

that is an instantaneous "window" of your

bandwidth utilization.  Information is real time,

but the graph will only display up to 16 minutes

of information.  Sampling is once a second.

2. Long Term Bandwidth Utilization (LTBU) - this

displays (and allows for export) longer term

information about your bandwidth utilization.

The graph shows high, low and average over

time - up to 7 days.  Sampling is still once a

second, but the display can be configured to

report at various time intervals.

The Bandwidth Utilization screen can be

activated from the main window by selecting

"Modes" -> "Bandwidth Utilization".

Once you are in the Bandwidth Utilization screen,

the graph shows the current utilization.  A

Maximum Utilization value is kept at the top of the

graph.  This value is reset each time you start and

stop Bandwidth Utilization.

This graph shows the amount of data on your

network segment graphed against the maximum

theoretical bandwidth for your network topology,

as set in the "Modes" -> "Settings" dialog box.  For

Ethernet you should choose "10 Mbps", for Token

Ring choose either "4 Mbps" or "16 Mbps"

depending on your LAN.

Note: This graph is not subject to any filters.

The Long Term Bandwidth Utilization screen can be

activated from the main window by selecting "Modes" ->

"Long Term Bandwidth Utilization".

Once the LTBU graph pops up, it automatically

begins capturing data.  Three lines will be shown,

red - maximum, blue - average, and green for

minimum.  Although data points are only shown

for the time period set in "Modes" -> "Settings",

data is collected and processed every second.

The scroll bar at the bottom of the graph will allow

you to see historical utilization data.  The LTBU

graph can collect 10,922 data points.  So, for

example, if you choose a 1 minute averaging

time, this calculates out to 7.59 days before the

LTBU graph will reach its data limit.

See the Settings section for more information on

Averaging Time and Time scales.

Note: Long Term Bandwidth Utilization is not

subject to any filters.

General Statistics Information

General Statistics screen information (for all

Network Statistics screens):

To start the statistics capture, click on the "Start"

button.  To stop the capture, click on the "Stop"

button.

Once you are in a network statistics screen, you

have a number of choices to make.

The network statistics screen has three views:

Received Packets, Size Distribution, Protocol

Statistics.  You can view LAN traffic by address

and percentage of captured bandwidth, by

address and packet size distribution as a percent

of captured bandwidth, or by protocol.  You can

choose the view you would like to see from the

"Modes" menu.  Each of the network statistics

windows are shown and explained below.

Some screens have additional options that are

explained in each of the dialogs individual

sections that follow.

By default, each statistics view will show all traffic

(subject to your current filter selections) by the

"Destination Address".  You can press the

"Direction" button to switch from a "Destination

Address" listing to "Source Address" listing.

You can press the "-Aliases" button to switch from

a listing of hard network address to one of aliased

addresses that you have set up in the "Filters"

dialog box.  (For more information on how to set

aliases, see the Filter section.)  When the button

shows "-Aliases", aliasing is off.  When it shows

"+Aliases" aliasing is on.

The "Clear" button clears the current screen,

clears the capture buffer and stops the capture.

Press the "Start" button to begin a new capture.

Received Packets Statistics

The Received Packets window can be activated

from the main window by selecting "Modes" ->

"Received Packets Statistics".

To begin a capture press the "Start" button.

The Destination/Source Address field will show

either the destination or source address.  This can

be either a hard network address or an alias

(depending on the state of the alias button).

The "%" field shows the percent of bandwidth

utilization for that destination/source address.

Note that this is the % of filtered bandwidth.  If you

would like to see the percent of total bandwidth

that that particular address is using, you will need

to set up an ANY_ADDRESS to and from

ANY_ADDRESS filter.

The "Packets" field shows the number of packets

to (or from) the destination/source address,

subject to the current filter set.

The "Bytes" field shows the bytes to (or from) the

destination/source address, subject to the current

filter set.

The "Sort" button activates the sort dialog:

The sort dialog allows you to choose the direction

of the display (with respect to source or

destination) and the sort order of the window.

Sorting is useful for determining which stations are

sending the most data, and which are less active.

Once the capture begins, you can double click

on any address to copy the hard address into the

Windows clipboard.  This can provide a

convenient way to begin building an alias table.

For example, if you double click on an address

and then switch to the "Filter" dialog box, you can

then paste that address into the "Hardware

Address" edit box using Shift-Ins.  To automatically

build a complete hard address table, see the

section on Discovery mode.

Size Distribution Statistics

The Destination/Source Address field will show

either the destination or source address.  This can

be either a hard network address or an alias

(depending on the state of the alias button).

The "%" field shows the percent of  bandwidth

utilization for that destination/source address.

Note that this is the % of filtered packets.  If you

would like to see the percent of total packets that

that particular address is using, you will need to

set up an ANY_ADDRESS to and from

ANY_ADDRESS filter.

The "Packets" field shows the number of packets

to (or from) the destination/source address,

subject to the current filter set.

The rest of the screen shows the packet size

distribution divided by packet size in bytes.  This is

shown as a percentage of the total for that

address.  This is a good way to determine if a

particular station is being as efficient as it could

be by sending mostly large or mostly small

packets.

Protocol Statistics

The Protocol Statistics screen shows Ethernet LAN

traffic divided by network protocol and sub-protocol

(the Token Ring display will be slightly different).

Protocols are displayed as a percentage of the total

filtered traffic.  Sub- protocols are displayed by the

number of packets received of that type. Printing

Statistics screens Any of the statistics screens can be

printed by selecting "File"->"Print Protocol Statistics"

from the Network Statistics screen. CAPTURING PACKETS ON

YOUR LAN:  PACKET CAPTURE MODE Observers Packet Capture

screen  consists of a menu bar, monitor graph,

Start(/Stop), View and Clear buttons.

Note: To capture packets, you should first select a

filter set.  This can be done using the "Filters" menu

choice from Packet Capture (or any Observer)

screen to bring up the "Filters" dialog box.  See

the section on "Filters" for more information.

To begin capturing packets, press the "Start"

button.  You will see three different lines on the

capture graph.  The red line shows the non-

captured traffic, the blue line shows  the captured

traffic, and the black line shows the dropped

packets.

Note the red blinking "Capturing" in the upper

right hand corner of the graph.  This is your

"Capture is on" indicator.  Observer will also tell

you the percent of your capture buffer that is full,

and will automatically stop capturing when the

buffer becomes full.  For more information on the

capture buffer size, see the Observer "Settings"

dialog section in this manual.

Once you have captured the packets you can

view the packets with the "View" button, or save

the capture buffer to a file to view later.  Pressing

the "View" button will turn capturing off unless you

disable the "Stop capturing if switched from the

main window" checkbox from the "Modes" ->

"Settings" dialog box.

For information on viewing captured packets see

the "Viewing Captured Packets" section.

To save a capture buffer to a file, capture the

information you want to save and select "Save

Packet Buffer" from the "File" menu on the

Capture Mode graph or from the View screen.  To

load a previously saved capture buffer, choose

"Load Packet Buffer" from the "File" menu on the

Capture Mode graph or from the View screen.

To stop capturing packets, press the "Stop"

button.

To clear the capture buffer, and stop the capture,

press the "Clear" button.

Viewing Captured Packets

To view the packets in the capture buffer, click on

the "View" button from the "Receive Packets"

screen.

Once you are in the view screen, you can click on

a particular packet (with your LEFT mouse button)

in the top window to display the packet decoded

information in the bottom window.  The two

windows are fully sizable by dragging the middle

border up or down.  Observer decodes all Novell

packets and all IP packets.  Other packets are

shown in raw mode.

You can click on the "Show aliases" box at the

bottom of the screen to enable aliasing of

network addresses.

Clicking on the "Raw Packets" box at the bottom

of the screen will switch from a decoded packet

view to a raw packet view.

The Settings menu choice has two option:

Select Frame Colors - This brings up the colors

dialog.  You can also access this dialog by single

clicking your RIGHT mouse button on any packet

line on the top part of the View Packets screen.  In

either case, this allows you to choose the color of

the packet line you would like to associate with

the selected frame type.  For example, you could

set all IP packet types to show with a white

background and a green foreground, while

displaying all IEEE 802.3 packet types (Netware

3.xs default) as white foreground with a red

background.  This can help you visually pick out a

particular packet type, if you are capturing

multiple types.

View Configuration - This brings up the View

Configuration dialog where you can control

Observers post-capture filtering:

Probably one of the most powerful features of

Observers packet capture/decode this allows you

to filter the packet capture after you have

captured a buffer.  The post- filters will filter out

any unwanted data both in the packet decode

screen and on any print outs of the data capture.

The view configuration dialog options include:

This dialog allows you to "post-filter" all or some of

the IP header information.

This dialog allows you to "post-filter" all or some of

the TCP header information.

Show Differential Packet Times - choosing this

option (check the checkbox) will allow Observers

View Packets windows to display the differential

packet times.  Differential packet times can show

difference in time of arrival (and thus time sent)

for the current packet and the previous packet.

This is called the packet delta.

You can also search the data segment of a

packet capture buffer by using the "File"-> "Find

Packet" dialog:

Multiple instances of the Find Packet dialog can

be active at one time.  To activate the multiple

instance search, start one search and choose

"File"->""Find Packet" again without closing your

first search - both will remain active.

Printing captured packets

You can print a packet capture buffer (the "View

Packets" screen content) on a printer attached to

your computer or (if you are using ODI or NDIS) to

any printer available on your network.

To set up a printer for Observer select

"Print"->"Print Setup" from  the View Packets

dialog box and select either the default printer or

choose a different printer from the "Specific

Printer" dialog.

To print out a packet capture, click on the "File"

-> "Print" to bring up the printer dialog sub-menu.

You will be given options for the set of packets to

print.  The default is set to print all captured

packets.

You can choose to print commented or raw

packets, or both (which can be most useful for a

programmer analyzing packet details in depth).

You can have Observer print out either Ethernet

addresses or aliases as the printed headers

Lastly, you can choose whether Observer will print

packets continuously or print each packet on a

single page. (Providing that length of a packet

allows it, every new packet will always start

printing on a new page).

Once you have made your print options

selections, click on the "Print" button.

To exit without saving, press "Cancel".

Saving Captured Packets to a File

To save a packet capture to a file select

"Print"->"Save to a File" menu in the View Packets

dialog box.

You will be given a choice of packets numbers to

print.  The default is set for all captured packets.

However, if after reviewing a packets contents in

the "View Packets" dialog you are interested in

some particular section of the capture you can

specify only that section.

Next you can choose to save commented or raw

packets, or both (which can be most useful for a

programmer analyzing packet details in depth).

You can let Observer save either Ethernet

addresses or aliases as the printed headers.

When you have made your options choices, click

on the  "Save" button.  You will be given a choice

of file name and directory to save the capture.

To exit without saving, press "Cancel".

SETTINGS DIALOG

The settings dialog box allows you to

set the global variable information in Observer.

Windows buffer size is the amount of Windows

memory that Observer will set aside to store

captured packets.  Values are in kilobytes, so a

setting of 132 would represent a .132 megabyte

(MB) buffer.  A 2048 KB buffer would represent a

2.048 MB buffer.

You should choose the smallest buffer that will

capture the event you are interested in looking at.

Observer will show the buffer percentage full and

you will get an idea of what is the best buffer for a

particular situation.  Keep in mind that a full 4 MB

buffer is a lot of data to sort through.  You will

want to capture an event in as little time with as

little buffer space as possible.

It is not recommended (although it is allowed) to

use a buffer larger than the amount of physical

RAM in your PC.

On a 16 MB system the maximum recommended

buffer is 10 MB (a setting of 10,000).  On a 8 MB

system, the maximum recommended buffer is 4

MB (a setting of 4,000).  Again, use the smallest

buffer possible to capture the event of interest.

Lock Windows buffer from paging will tell Windows

to not page out the Observer packet buffer.  This

will make Observer more efficient, since its buffers

will always stay in RAM (as opposed to Windows

swapping out the buffer).

This is the recommended mode for operation of

Observer, but be careful.  If you set your buffer

size too high and lock the buffer from paging,

Windows may swap out its own programs and

cause Windows to crash.

Stop capturing if switched from main window - this

will turn capturing off when you switch focus from

the capture graph window.  Switching focus is

done when you start another Windows

application, when you choose to view packets, or

when you move the Windows focus to another

window.

Observer can capture packets while Windows is

doing something else.  However, under heavy

traffic conditions, (or with a slow PC) Observer

may drop packets if this option is not checked.

Ask confirmation to clear packet buffers - brings

up a conformation (yes/no) dialog when the

"Clear" button pressed.

Network Bandwidth - select the network

bandwidth of your LAN.  This will be used to

calculate the % of Bandwidth Utilization when in

Bandwidth utilization mode.  Options are:

4 Mbps	- Used for 4 MB Token Ring

10 Mbps	- Used for all Ethernets

16 Mbps	- Used for 16 MB Token Ring

Long Term Bandwidth Utilization configuration - this

is where you configure the averaging time and

stretch scale:

Averaging time - LTBU graph can collect 10,922

data points.  If you choose a 1 minute averaging

time, this calculates out to 7.59 days before the

LTBU graph will reach its data limit.

So, for example, you could choose 30 seconds for

and averaging time and Observers buffer would

capture and display for  10,922 (points) / 120

(points per hour) / 24 (hours per day) = 3.79 days.

Stretch time scale - By default, the LTBU graph will

plot each data point 10 pixels apart (10x) and fill

in the distance between with a solid line.  Setting

this to 1x will plot data points one pixel apart.

LTBU will plot times on the graph in a manner

which insures that no time stamps will overwrite

the previous or the next.  All export files contain all

time stamps and all data points as described in

the averaging time above.

Program Flags

This section discusses the optional flags for all

programs that use flags.  Flags shown in brackets

[] are optional.  Flags are not case sensitive.

OBSERVER.EXE

OBSEVER.EXE has the following optional flags:

OBSERVER [-i=xx] [-DEMO] [-TOKEN]

where:

-i=xx

The software interrupt you specified in the VPACK

(or VPACKODI) command line.  This is only

required if you have specified a flag with VPACK

or VPACKODI.  Values (xx) are in hex and can be

from 60-6F.  6F is the default.

This flag can be used in conjunction with any

other flag.

Example:

OBSERVER -i=63

-DEMO

This starts Observer in DEMO mode.  In this mode,

Observer will play fake data, and does not require

a LAN or LAN adapter to function.

Example:

OBSERVER -DEMO

-TOKEN

Only used with the -DEMO flag, this tells the

Observer DEMO that your would like to look at

Token Ring data.

When using Observer in licensed mode, it will

automatically determine whether you are using

Token Ring or Ethernet.

Example:

OBSERVER  -TOKEN  -DEMO

VPACK and VPACKODI

VPACK and VPACKODI have the following option

flags.

VPACK or VPACKODI  [-i=xx]  [-m=y]

where:

-i=xx

The software interrupt.  You may want to specify

this if the default (6F) is conflicting with some other

program.  If you have specified a software

interrupt different that the default, you must also

specify the same interrupt on the OBSERVER.EXE

command line.  Values (xx) are in hex and can be

from 60-6F.

-m=y

where y can be:

6	for a 60K buffer

5	for a 50K buffer

4	for a 40K buffer

3	for a 30K buffer

2	for a 20K buffer

A sample command would look like:

VPACKODI  -m=3

This flag allows you to lock a capture buffer when

loading VPACK or VPACKODI.  This makes sure

that a buffer is allocated no matter what is using

lower DOS memory in Windows.

Since the allocation of the lower DOS memory

under Windows is dynamic and often very

inefficiently laid out, locking VPACK or

VPACKODIs memory will usually not result in any

loss of functionality.  In other words, programs

often take as much RAM as is available, and if

they get less, it will not hurt them - WFW

networking is an example of this.

If you see an error indicating that you do not have

enough contiguous DOS memory to allocate

VPACK or VPACKODIs buffer, you will need to use

the "-m" flag.

This error is VPACK or VPACKODI telling you that

other applications have taken all of the DOS

memory available and have not left enough

room for VPACK or VPACKODI to allocate a

buffer.

First off, lets explain how Windows handles DOS

drivers and what happens to your lower DOS RAM

when you start Windows.

People often think that because when they open

a DOS window in Windows they may have 500+ K

of RAM free, that this is how much RAM they

actually have available in DOS.  This is not true.

Each time Windows opens a new DOS window it

allocates an additional 640K of RAM from the

RAM above 1 MEG.  Your original (lower) DOS

memory (the memory that is below 1 MEG) is used

by Windows and Windows programs for all sorts of

things.  For example, Windows for Workgroups

drivers can use up to 450K of your original DOS

memory to load.  If it cannot allocate this Ram

Windows will not load WFW networking.  (Try and

load WFW with only 420K free in DOS - you will see

what we mean).

VPACK and VPACKODI were designed to take

only 3K of DOS memory while Observer is not

running and auto allocate up to 60K of DOS buffer

when you run Observer (the memory is returned

when you terminate Observer).  VPACK and

VPACKODI try to allocate 60K if they can find the

60K contiguous.  If they can not find 60K, they try

50K, 40K, 30K then 20K.  After that they give up

and you see the error message.

This can be solved in two ways:

1. Unload some of the offending programs.

Sound blasters, screen savers, clock utilities,

and unused network protocols are the usual

suspects.  And try running VPACK or

VPACKODI again.

2. Run VPACK or VPACKODI with a locked

memory buffer - explained above.

USING NETWARE DISCOVERER

OVERVIEW AND QUICK START

To start Discoverer double click on the Discoverer

icon.

Note:  As mentioned in the install, Discoverer

REQUIRES Novell Netware, ODI, the VLMs and all

of the Netware Windows DRVs, DLLs and VxDs

installed and running.  If you experience problems

running Discoverer, see the installation section

under ODI (section 1a) for things to check.

Additionally, at this time to receive Netware

statistics you must be using Novells 802.3 for your

IPX/SPX FRAME type.  This is the Netware default.

This is a Novell limitation.

Discoverer's main window consists

of a MDI window with a menu on top and a set of

often used commands in a button bar.  On the

left there is a graphical list of servers and nodes

(Called the "List of Nodes"), the right side is

separated by a split window bar and is the area

reserved for SERVER and NODE information

windows.

All the buttons on the button bars have a short

informational description for the button that can

be invoked by placing the cursor over a particular

button and pressing (and holding down) the

RIGHT mouse button.

To display a list of nodes attached to a particular

server, double click on the server icon on the left

side of the MDI window (known as the "List of

Nodes") with the LEFT mouse button.  To close the

graphic node list of a particular server double

click on the server icon again (using the LEFT

mouse button).

To display a SERVER or NODE informational

window click on the RIGHT mouse button on a

server or node icon in the graphical list of servers

on the left of the MDI interface, and an

information window will appear in the area to the

right.

From the SERVER or NODE informational window

you may select the information you would like to

display.  This information includes:

The Charts menu - this shows graphical

information in real time about the network,

The Lists menu - this shows tables of information

that are updateable with time,

The Info menu - this shows version information and

statistics that do not change considerably in short

period of time and do not need to be updated as

often.

Depending which selection you made you may

use different entries in the Options menu to

customize the display to fit your needs and

preferences.  You can set update periods and

colors separately for every type of chart.  The

Color Setup dialog also can be invoked by click

of the RIGHT mouse button anywhere on a chart.

The vertical scale of a chart can be changed

from Scale menu item under Options in the

specific NODE or SERVER information window.

The NODE list's update period is settable under

"Options" -> "Timing" from any NODE information

window.  This sets the polling simultaneously for all

NODE lists.  Setting this from a SERVER information

window, sets it for all SERVER lists.  Timing set while

displaying a chart sets timing for that chart only.

Note:  At certain moments - when Discoverer polls

the network or collects information - new

commands that could interfere with current

activity will be ignored.  Do not be alarmed - this

is a normal way Discoverer operates.

THE MAIN MDI WINDOW

Discoverer's initial main screen (the MDI window)

consists of a menu bar, button bar, List of Nodes

listbox (on the left) and information window area

(on the right) divided by vertical splitbar.

The List of Nodes is a two level graphical

representation of servers and nodes on the LAN.

Discoverer fills the List of Nodes with icons of

servers it finds on your LAN in the first few seconds

after startup.

Note:  The first polling's success is as an indication

of the Novell support VLMs, drivers and DLLs

installation.  If you do not get the initial listing of

servers, some piece of the Novell driver set was

not properly installed.  An error message should

appear telling you which file(s) needs to checked

or added.

After listing of servers appears, you can display a

list of nodes attached to any server by double

clicking on the LEFT mouse button.  This "opens"

the server list.  Double clicking again on the LEFT

mouse button closes the list.

Click on RIGHT mouse button over any server or

node icon opens a SERVER or NODE information

window on the right side of the MDI window.

File Menu

Discoverers "File" menu includes the following

items:

License - This choice only appears if Observer and

Discoverer are not licensed.  You can license

either program from either license dialog.

Store List of Nodes - stores the current list of nodes

for Discoverer to recall or reuse at a later point in

time.  (Duplicates the functionality of the button in

the Main MDI window button bar).

This is an important concept.  The "List of Nodes"

should be saved and added to from time to time

as you add station to your LAN.  Think of this as

your template from which you begin each

Discoverer session.  When your template is

accurate - listing all nodes and servers - you can

see which nodes or servers are on, running and

active.  But most importantly, you will also be able

to see any inactive nodes or server.  If you do not

save and update the List of Nodes, Discoverer will

only see active stations each time it starts up, and

thus will not show any stations or servers that may

be down for valid or non valid reasons.

Load List of Nodes - loads saved list of nodes

stored with "Store List of Nodes" and forces

Discoverer to displays the saved list.

Save List of Nodes to a Text File - saves the

complete (current) list of nodes to a text file

(*.SAV).

Print List of Nodes - prints out the (currently

displayed) list of nodes (prints all servers and all

attached nodes).

Save Node Information to a File - stores List or Info

tables to a file.  If the active information window

(window displaying server or node information ) is

currently displaying a chart, this menu item is

replaced by:

Save Chart in Spreadsheet Format - which

allows to save the charted information in

coma delimited text format compatible

with Microsoft Excel and other spreadsheet

applications.

Print Node Statistics - prints the List or Info table.  If

the active information window (window displaying

server or node information) is currently displaying

a chart, this menu item is replaced by:

Copy Chart to Clipboard - which copies the

current chart to Windows clipboard to be

processed by any image editor including

Windows Paintbrush.

Print Setup - lets you select and configure a printer

from the currently installed Windows printer list.

Exit - exits Discoverer.

About Discoverer - displays version and copyright

information.

Options Menu

Discoverers "Options" menu consists of three

entries:

Preferences - the dialog to set the general

operational parameters for Discoverer (a more

detailed explanation of this dialog is located in

the Preferences section later in this manual).

Observer Discovery Alias List - Discoverers

Discovery Alias List is designed to collect hardware

addresses used on the network, and in the same

way as Observer aliases them with Internet

Addresses, it aliases hardware addresses with

Novell user names.  When saved, these aliases will

be used by Observer for Packet Viewing.

Server exclusion List - the dialog that allows you to

exclude/include various servers from the list of

nodes.

Login to Server - Login to Server lets the

administrator login to a highlighted (on the list of

nodes) server.  To receive a list of users from the

server, a Discoverer operator MUST HAVE

SUPERVISOR privileges.  If he or she logged in

earlier with different name, this option first logs you

out, then asks you to login as supervisor (or

equivalent).  If desired, Discoverer can autologin

on some or all of the servers (this option must be

enabled in Preferences dialog).

The Preferences dialog

Ask Confirmation on Exit - if selected, Discoverer

displays a warning message when a user chooses

either the Exit button or chooses "Exit" from the

"File" menu.  It is intended to prevent accidental

termination of Discoverer and loss of collected

data.

Ask confirmation to save List of Nodes - if selected

this prevents accidental overwriting of the List of

nodes by popping up a conformation dialog.

Show Tips - this option if selected enables

displaying a short list on the most essential tips on

Discoverer's startup.

Statistics display mode - this gives the choice of

the three ways information windows can be

displayed:

Single maximized window node - in this

mode only one maximized information

window can be displayed at one time.

When a user starts an information window

for different SERVER or NODE, the current

information window terminates.

Multiple maximized window nodes - in this

modes an information window is maximized

as in the previous option, but when starting

a new window, the previous one does not

terminate.  The new window takes place of

the previous one.  A user can switch

between the windows using the CTRL-TAB

key combination.  The windows can then

be tiled or cascaded with the appropriate

buttons from the menu bar, or menu

choice.

Multiple normal window nodes - in this

mode all of the information windows are

displayed non-maximized.  They can be

positioned, tiled and cascaded to enable

monitoring of multiple servers or

workstations simultaneously.

Load saved List of Nodes on startup -

automatically loads the current saved list of

nodes.  Otherwise the List of Nodes starts blank

and is gradually filled with active servers and

nodes (as they respond to polling).  Note: this

must be checked to use an exclusion list.  In other

words, if this is not checked, the list of nodes fills

with whatever servers answer and no exclusions

are checked, or used.

Allow autologin to selected servers - When this

option is selected, Discoverer automatically logs in

on the servers that the Discoverer's PC was not

attached to prior to the start of Discoverer.  If the

login entry was not saved, a user will be asked to

login.  If the user selects to save current login

(from the login dialog), Discoverer will save and

then use this login and password in future.

Note: To receive information about users

attached, and consequently other information for

these users from a server, you must be logged in

as SUPERVISOR, or a user with equivalent

privileges.  If you were not logged in as

SUPERVISOR prior to starting Discoverer, you may

find that all nodes are dark (if Load Saved List of

Nodes on startup option was selected), or there

are no nodes attached to the server.  In this case

LOGIN TO THE SERVER by pressing on the similarly

named line in the Options menu, and on the next

polling session (every 20 seconds by default)

Discoverer will autologin to the server, or you will

be asked to enter your name and password.

Display polling cursor when minimized - this option

toggles off and on the display of the Discover

hourglass cursor when Discoverer itself is minimized

but still polling servers or nodes.  This special cursor

will help you determine whether a system delay is

caused by Discoverer polling, or by another

program.  When Discoverer is the active window,

it always displays the Discoverer hourglass when

polling.

Discoverer server polling period - This option

allows you to adjust the polling period Discoverer

uses to update the "List of Nodes" table on the left

of the main MDI Discoverer Window.  Two values

are required:

Window - this is the number of seconds that

Discoverer waits to poll all servers/nodes

active in the "List of Nodes" WHILE

DISCOVERER IS ACTIVE (NOT MINIMIZED).

Minimized - this is the number of seconds

that Discoverer waits to poll all

servers/nodes active in the "List of Nodes"

while Discoverer is iconified.  You may want

to adjust this quite high if you are collecting

statistics on a node or server.  Doing so will

minimize the disruption of your work while

Discoverer is running.

Addition Polling Time - this is a tuning parameter

that should be used on large or busy LANs when

the time required for the server to respond to a

poll is so long that Discoverer believes that the

server is down.  Typically this should be left at 0.  If

your servers seem to go dark (i.e. Discoverer thinks

they are down) every so often, introduce a 1 or 2

second additional polling time, this should solve

the problem.

Discoverer Discovery Mode

Discovery mode allows you to capture all of the

hard network addresses on your LAN, store them

in Observer's the filters table, and alias them as

names or Novell Login names.

Press "Add New" to begin, and the discovery

mode table will begin polling servers and

capturing addresses.  Once captured you can:

Clear - clear the table

Add/Edit Alias - selecting one address and

pressing this button will pop up the edit box to

add an alias.  Make sure you click on the down

arrow at the right when complete.

Highlight New - shows the new addresses.  These

would be addresses that were discovered, but are

not yet on the address list in Observer's filter

section.

Save Highlighted - saves the highlighted address

to Observer's filter table.

Save All - Saves all of the addresses/aliases

(overwrighting all others).

Check the box at the bottom of the dialog to

automatically "Fill Addresses with Novell Name"

Or check the "Load List of addresses at startup" to

load Observer's filter table at startup to try to

discover Novell login names for addresses without

aliases.

Server exclusion List

Discoverer includes the ability to "exclude"

certain servers from the list of nodes.  You may

want to do this if you have many servers that

cannot provide useful information in your list of

nodes.  To exclude a server, choose "Options" ->

"Server exclude List" and highlight the server you

want to exclude in the "Display Servers" list and

pressing the "Exclude" button.

For example, if you have many print servers,

CDROM servers, dead servers (servers that your

active servers can see across a wide area

network, but are not available for your

inspection), or servers that you are simply not

interested in - you can exclude these, and as long

as you have "Load saved List of Nodes on

Startup" checked in the Preferences dialog, they

will not appear in your list of nodes.

You can restore any server simply by going into

the "Server exclude list" highlighting the server to

be restored in the "Excluded Severs" box, and

pressing the "Restore" button.

Login to Server

To be able to receive information about users

attached, and consequently other information for

these users, you have to be logged in as

SUPERVISOR, or a user with equivalent privileges.  If

you were not logged in as SUPERVISOR prior to

starting Discoverer, you may find that all nodes

are dark (if load saved List of Nodes on startup

option was selected), or there are no nodes

seemingly attached to a server.  In this case Login

to Server by choosing this from the Options menu,

and on the next polling session (which happens

every 20 seconds by default) Discoverer will

autologin to the server (if you have saved the

servers autologin information and have Allow

Autologin to Servers checked in preferences), or

you will be asked to enter your name and

password.

The Login to Server dialog has the following

options:

Login name - this should be SUPERVISOR or a user

with SUPERVISOR equivalencies.

Password - the users password.

Save current login - this saves the login

information  for autologin (only used if you have

"Allow Autologin to Selected Servers" enables in

"Options" -> "Preferences").  This information is

kept encrypted in the DISCOVERER.INI file.

Do not login (print server only) - this deactivates

the autologin feature for the highlighted server.

Use this option only if it is a server that does not

require a login (i.e. a print server).  If you do not

want to login to a standard server, use the Server

exclude/include option.

The Window Menu

The Window Menu contains the following items:

Tile - tiles all information windows (both SERVER

and NODE).

Cascade - arranges information windows in a

standard Windows cascade.

Arrange Icons - if there are minimized information

windows, this option lets you neatly arrange them.

Close All - closes all information windows.

In addition, if there is one or more information

window, a list of them is appended to this menu,

from which you can choose a window to make it

the active window.

THE MDI WINDOW BUTTON BAR

The Main MDI Window button bar is provided for a

quick access to some of the often used, or

potentially often used functions of Discoverer.

Short help information for the purpose of each

button can be seen by pressing (and holding

down) the RIGHT mouse button on the button in

question.

Here is the list of the buttons and their functions in

order from left to right:

List of Nodes Display Options - allows you to

configure the display of the contents of the server

and node text in the List of Nodes.  It can be a

node name only, node name + address, or node

name + address + attach time.

Save List of Nodes - this is used to save the current

list of nodes (and servers).  You might use this to

save a list of all servers and workstations.  At a

later date you could "Load List of Nodes" (see

below), and Discoverer would display not only the

active stations discovered, but also all stations in

the saved list of nodes.  This would then display all

active stations and all non-active stations in the

saved list.  This is useful to get a complete listing of

all station to determine when the network

configuration changes, or to display which

stations are up or down.

It is recommended that you keep "Ask

Confirmation to Save List of Nodes" checkbox

selected (in "Options" -> "Preferences") to

prevent inadvertent overwrighting of the current

list.

Load Saved List of Nodes - this loads the saved list

of nodes.  You may want to do this prior to saving

the list of nodes.  This way you can get all stations,

both active and non-active in your list of nodes.

Delete entry - this deletes a highlighted entry on

the List of Nodes.  You might want to use this to

remove a station or server that has been stored

on the list of nodes, but has since been removed

from your site.

Stop Polling for Statistics - is used to completely

stop polling.  You can start polling again by

pressing the button again.  Stopping the polling

will  prevent updating the List of Nodes.  Stop

Polling can be also used to save the current state

of a chart or list, when it is necessary to prevent it

from updating or changing position.

Tile Node Windows and Cascade Node Windows

can be used to arrange Information Windows in

the view area.

Exit - closes Discoverer.

THE LIST OF NODES VIEW

The "List of Nodes" View is the actual list of SERVES

and NODES on the left hand side of the main MDI

window.  This is where you will see a graphical

depiction of all of the NODES, SERVER, ROUTERS,

BRIDGES, etc. on your LAN.

You can expand any server by double clicking on

the LEFT mouse button on the server icon.  You

can display a SERVER or NODE information

window on any SERVER or NODE by single clicking

on icon with your RIGHT mouse button.

INFORMATION WINDOWS (GENERAL)

The following information applies to both the

NODE Information window and the SERVER

information window.

An information window Options button menu

displays the following choices:

2D - 2D charting offers choices of Lines, Columns

and Pie charts.  Sample charts:

3D - charting offers Separate and Alternate

Columns.  A user can choose the chart that is

most useful for their needs.  Some menu items

may be grayed out depending on the

applicability to the active window. (charts only)

Colors - The Colors dialog appears when the

Colors item is selected from the Options menu, or

when user clicks on the RIGHT mouse button

anywhere on a chart.  It allows you to select the

background and foreground colors, hatch, axis,

coordinate lines, appearance, and color or a

specific attribute.  This is done separately for every

type of chart (as appears in Charts menu), and

separately for servers and nodes. (charts only)

Scale - The Scale submenu allows to expand or

contract the vertical scale of a chart to

accommodate the charts scale to the wide

range of amplitudes of data.  This value is also

saved separately for every type of chart.

Timing - The Timing dialog is applied to all

updateable information window display modes

(included in Charts and Lists menus for SERVERS

and NODES).  It is where you set the polling period

for the chart or list.  Reading (list refresh) period is

the interval of time between polling a server or a

node by Discoverer.  Values are from 10 seconds

to 1 hour.

SERVER INFORMATION WINDOWS

A SERVER information window is displayed in the

information area (on the left of the MDI main

window) after a user clicks on the RIGHT mouse

button on a server icon in the List of Nodes.

SERVER information windows (which are very

similar to NODE information windows) consists of a

button bar with Charts, Lists, Info, and an Options

popdown menu buttons and an Exit button.

Button bar buttons can be selected using mouse

clicks, and in most cases by pressing a letter on

the keyboard that corresponds to one displayed

in red on the button. (Note: due to the nature of

the MDI interface you DO NOT use ALT when

accessing the information window buttons.  Use

ALT-KEY combination to popdown menus on the

Main MDI  window).

Context sensitive help is available from any

window by pressing F1.

Charts

The SERVER information window Charts menu

contain the following choices (sample charts are

located in the "Information Windows (general)

section):

Number Connections (server)

Number of Connections chart displays the

number of active connections to a specific Novell

server.  This is also called the number of logged in

users.  This statistic is received directly from the

Novell server.

This chart can be saved to a file in spreadsheet

format ("File"-> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

Open Files (server)

The Open Files chart shows number of files

currently open on a specific Novell server. This

statistic is received directly from the Novell server.

This chart can be saved to a file in spreadsheet

format ("File"-> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

IPX Packet Statistics (server)

The IPX statistics chart displays the number of sent

and received IPX packets as a function of time.  It

shows the server's IPX packet flow activity.

Malformed packets for send and Get ECB (Event

Control Block) for receive are a sum of several

failure conditions, and can indicate, if significant,

improper functioning or overload of the server.

More detailed information can be seen in IPX

Statistics List.

This chart can be saved to a file in spreadsheet

format ("File" -> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

SPX Packet Statistics (server)

The SPX statistics chart displays the number of sent

and received SPX packets as a function of time.  It

shows the server's IPX packet flow activity.

Bad Send Packets for send and Get Bad Receive

Packets receive are a sum of several failure

conditions, and can indicate, if significant,

improper functioning or overload of the server.

More detailed information can be seen in SPX

Statistics List.

This chart can be saved to a file in spreadsheet

format ("File" -> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

Total Packet Statistics (server)

The Total Packet Statistics display number of total

(all protocols) sent and received packets by a

server as a function of time.  It is an indication of

the server's total network activity.

"Bad Send Packets" for send and "Get Bad

Receive Packets" for receive are a sum of several

failure conditions.

Bad Send Packets combines sum of: Send Packet

Too Big, Send Packet Too Small and Send

Miscellaneous Errors, and can indicate improper

functioning of a server, or malfunctioning one of

the NLMs.

Get Bad Receive Packets is the sum of a set of

error conditions, and if significant, indicate an

overload of the server.  More detailed information

can be can be seen in Bridge Driver Statistics List.

This chart can be saved to a file in spreadsheet

format ("File" -> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

Bridge Statistics (server)

The Bridge Statistics displays the number of

packets routed through the bridge as a function

of time.  Novell considers each network card

driver in your server as a "Bridge".  Novell

considers each instance of the driver loaded

(usually for each frame supported) as a "board".

So, each "Bridge" (card in your server) may have

one or more logical "boards" loaded in it.  Each

board is displayed and each statistic is kept for

each board.  You can only "see" the Bridge in the

server that you are connected to (this is a

Netware limitation).

It includes the number of packets serviced and

packets routed, and some the most common

error conditions:

Total Packets Serviced - the total number of

packets the bridge serviced since the bridge was

initialized.  This is the number of packets that

arrived on the server, but not necessary were

routed.

Total Packets Routed - the total number of

packets the router actually routed.

Too Many Hops - this is an indication that the

packet (that reached the current server) has

"hopped" too many times before reaching its

destination.  It is the number of times (since the

bridge was initialized) the bridge received

packets on the fifteenth hop across an

Internetwork bridge.  These packets are

discarded.

Hop is the transition (or hop) of a packet on its

way to a destination server or station from another

server or station.  This error may indicate that the

routing scheme used on the network is

inadequate, or possibly the server that was

servicing a more direct connection path is

currently down or out of order.  The situation may

require the network administrators intervention.

This error can occur only on larger sites with many

nets and bridges, or on sites experiencing an

"infinite loop" between routers or server.

Unknown Network - these are packets that

reached the server and can not be routed

because the destination network is unknown to

the server.

This error may appear if a router that was

transferring packets to the known destination is

down or was shut down.  In this case the router

sends "going down" broadcasts from which all

routers update their routing tables to remove any

routes that are no longer available.

No Space for the Service - this is the number of

times (since the bridge was initialized) the bridge

has received Internetwork packets it could not

accommodate because the router did not have

enough space in its DGroup area to copy the

packets.  These packets are lost.

This condition, can be an indication of network

overload, or the server may be too slow to

process the amount of data on a growing

network.  Monitoring the network bandwidth

utilization using Observer can help clarify this

question.  In either case, if the condition persists

and has an adverse effect on the network

performance, there may be a need add a server

and split the network load between them,

upgrade the server system with a faster CPU or

more RAM, or reconfigure stations and servers via

segmentation of the wire to provide a more even

load on the bridges and servers.

No Receive Buffers - this is the number of times

(since the bridge was initialized) the bridge could

not receive inbound packets because of

inadequate buffer space.  These packets are lost.

This condition is essentially similar to the previous

one, with exception that the error occurs on the

earlier stage of packet processing and indicates

either too few receive buffers, or more directly the

network (as opposed to server or bridge) is

overloaded.  Again a more definite answer can

be obtained monitoring long and short term

bandwidth utilization using Observer.

This chart can be saved to a file in spreadsheet

format ("File" -> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

Volume Usage (server)

The Volume Usage chart displays used and free

volume space (separately by each available

volume) on a server.  If the number of volumes is

larger than can be fit in the information window

area, use the scroll bar to move along the volume

listing.  This is useful for checking remotely how

much free disk space remains on a server volume,

and take appropriate actions when any of the

volumes reach critical usage.

This chart can be saved to a file in spreadsheet

format ("File" -> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

Options

A user can customize the way charts are

displayed by selecting the Options button in the

SERVER information window and then choosing 2D

or 3D, Colors, Timing and Scale options from the

Charts menu.

Under 2D or 3D the choices are - Lines, Columns

and Pie for 2D charts, Separate columns and

Alternate columns for 3D charts.

The Colors dialog lets customize colors for every

chart separately.  You can for example, have a

different chart type and set of colors for Open File

than you have for IPX Packet Statistics, and

Volume usage, and so on.

The Timing dialog sets timing periods for each

chart separately, and one common timing value

for all Lists.

The Scale dialog lets change the vertical scale of

a chart to fit best your system parameters.  The

best way to understand this dialog is to

experiment.

Lists

Lists are updateable tables of information about

servers. The update period is set in "Options"->

"Timing" menu.

The SERVER information window List menu has the

following choices:

IPX Packet Statistics List (server)

The IPX Packet Statistics List shows a summary of

IPX traffic since the Novell server was booted.  It

includes:

Send Packet Count - the total number of IPX

packets sent.

Malformed Packets - the number of times (since

IPX was loaded) applications have passed

malformed packets to IPX.  A packet is malformed

if the value in its ECB's fragment count field is 0 or

if the value of the size field within ECB's first

fragment descriptor is less than 30 bytes.  This can

be due to malfunctioning of one of the NLMs.

Get ECB Request Count - the number of times

(since IPX was loaded) IPX has supplied Receive

ECBs for incoming packets.  Event Control Blocks

(ECB) should be provided for every incoming

packet in order to process it.  This count does not

indicate an error condition.

Get ECB Failure Count - the number of packets

the driver has received, for which there was no

available ECB.

In network or server overload condition there can

be no ECBs available, and packets may be lost.

AES Event Count - the number of times (since IPX

was loaded) IPX has used the AES to schedule an

event.

Asynchronous Event Service (AES) is called when

the stack is not able not keep up with processing

of network traffic and must schedule a send or

receive event to happen in later time (usually a

few milliseconds later).

Postponed AES Events - the number of times (since

IPX was loaded) IPX has been unable to service

an AES event on time.  This can be due to a MLID

being in a critical section.  For example - IPX

cannot send an outgoing packet to an ECB busy

with another packet.  In this case the AES must be

postponed (rescheduled) to happen at later time.

If number of Postponed AES Events is high, it can

be indication of high network or processing load

on the server, and may indicate that it is time to

upgrade all or parts of the server (i.e. CPU,

memory, disk access time, etc.).

Max Configured Sockets - the maximum number

of open sockets possible on the target server.

Max Opened Sockets - the maximum number of

sockets open simultaneously since IPX was

loaded.  This should be 75% or lower than the Max

Configured Sockets.

Open Socket Failures - the number of times (since

IPX was loaded) applications have unsuccessfully

called IPXOpenSocket.  IPX cannot open a socket

if the socket table is full or if the socket is already

open.  If this number is greater than 0, you will

need to tune your server to accommodate more

sockets.

Listen ECB Count - the number of times (since IPX

was loaded) applications have given IPX a Listen

ECB.

A Listen ECB is given when a Netware application

expects to receive a packet associated with a

specific socket (usually a reply to a "just sent

command" or reques).  Neither a low or high

value is an indication of a failure condition.

ECB Cancel Failures - the number of times (since

IPX was loaded) IPX has been unable to cancel

an ECB.  For example, IPX cannot cancel an ECB

if the driver and the ECB have entered a critical

section just prior to sending a packet.  In this case,

the cancellation is too late, and would be

flagged as an ECB Cancel Failure.

Find Route Failures - the number of times (since

IPX was loaded) IPX has been unable to find a

route to a requested network address.

This error may appear if a router that was

transferring packets to the known destination was

shut down.  In this case the router sends "going

down" broadcasts from which all routers update

their routing tables and remove any routes not

longer available.

SPX Packet Statistics List (server)

The SPX Packet Statistics List displays a summary of

the SPX traffic since the Novell server was booted.

It includes:

Max Connections - the maximum number of SPX

connections possible on the target server.

Max Used Connections - the maximum number of

SPX connections open simultaneously since IPX

was loaded.

Establish Connection Requests - the number of

times (since SPX was loaded) applications called

SPXEstablishConnection.  This call attempts to

contact a partner to establish a session.

Establish Connection Failure - the number of times

(since SPX was loaded) SPXEstablishConnection

calls failed because the SPX Packet Header was

too small, the SPX Connection Table was full, no

router was found to the target network, or the

target node was down.

Listen Connections Requests - the number of times

(since SPX was loaded) applications called

SPXListenForConnection.  This call is issued by

Netware application to instruct SPX to watch for a

connection request from a peer system.

Listen Connection Failure - the number of times

(since SPX was loaded) SPXListenForConnection

calls failed because the SPX Connection Table

was full. This is an indication that the total number

of available SPX connections on the server is not

adequate for current network activity.

Send Packet Count - the number of times (since

SPX was loaded) applications tried to send a SPX

packet.  This number is a sum of all send packet

attempts, successful and not successful.

Window Choke Count - the number of times (since

SPX was loaded) SPX failed to send a packet

because the target station had not allocated a

receive buffer.  This is an indication of high load

on the target station.  This statistic does not show

the station responsible; if necessary the station

can be determined using Observer in Packet

Capture mode or Protocol Statistics mode.

Bad Send Packets Count - the number of times

(since SPX was loaded)

SPXSendSequencedPacket was incorrectly called

by passing an invalid connection ID or by passing

the address of an ECB indicating a packet header

size of less than 42 bytes.  This can indicate a

malfunctioning SPX application.  In this case the

connection to the target station is aborted.

Send Failure Count - the number of times (since

SPX was loaded) SPX was unable to send a

packet across a connection and receive

acknowledgment.

This can happen when a remote peer closes a

connection without acknowledging the SPX

packet.  In this case there is no guarantee that

the packet was received.  Other causes can be a

connection closed abnormally (frozen or a

rebooted station).

In such a case, SPX aborts the connection and

informs the calling application.

Abort Connection Count - the number of times

(since SPX was loaded) SPXAbortConnection was

called.

This event is initialized when some catastrophic

condition (from the point of view of application

that is using this connection) occurs.  This could be

an indication of malfunctioning NLM application.

In this case the connection is terminated

unilaterally by the current application without

notifying the other machine.  The partner

discovers the broken connection when it tries to

send a packet to this application (getting Send

Failure error, see above), or when the Watchdog

checks the connection after the inactivity time

expires.

Listen Packet Count - the number of times (since

SPX was loaded) applications gave listen ECBs to

SPX.  Listen ECB is given when a Netware

application expects to receive a packet

associated with a specific socket (usually a reply

to a just sent command or reques).  Neither a high

or low value is any indication of an error

condition.  There is no limitation on the number of

Listen ECBs, which should be provided by a

Netware application.

Bad Listen Packet Count - the number of times

(since SPX was loaded) applications passed SPX a

packet whose associated ECB is malformed.  An

ECB is malformed if the value in Fragment Count is

0, if the value in its first Fragment Descriptor Size is

less than 42, or if the listen socket is not open.

Incoming Packets - the number of times (since

SPX was loaded) the node's driver gave an

incoming packet to SPX.

Bad Incoming Packets - the number of times

(since SPX was loaded) SPX received and

discarded a packet because the connection ID in

the packet was wrong.

Suppressed Packet Count - the number of times

(since SPX was loaded) SPX discarded inbound

packets because they were duplicates of

previously received packets.  This condition

indicates the sending station has not received an

acknowledgment for a sent packet (due to delay

or collision).  It may indirectly indicate high

network utilization.  Observer will help determine if

this is the case.

No Session Listen ECB Count - the number of times

(since SPX was loaded) SPX was forced to discard

an inbound SPXEstablishConnection packet

because SPX lacked a corresponding SPX Listen

For Connection ECB.

Watchdog Destroy Sessions - the number of times

(since SPX was loaded) WatchDog destroyed a

connection because the connection was no

longer valid.  WatchDog packets are sent

periodically if no packets are received from a

peer within 5 minutes.  If there is no WatchDog

reply, the station repeats WatchDog request ten

times in one minute intervals, after which the

connection is considered invalid.

Total Packet Statistics (server)

The Total Packet Statistics List includes the

following entries:

Driver Version, LAN driver version for the specified

LAN board.

Statistics Version - the major and minor version

number of the Driver Diagnostic Table.

Total Sent Packets - the number of packets the

driver has successfully transmitted since the last

driver reset or initialization.

Total Received Packets - the number of packets

the driver has successfully received and passed

into the system since the last reset or initialization.

No ECB Available Count - the number of packets

the driver has received (since the last reset or

initialization) for which there was no listening ECB.

In a server overload condition, there may be no

ECBs available, and packets may be lost.

Packet Too Big for Send - the number of times

(since the last reset or initialization) applications

have asked the driver to send a packet over the

maximum legal size.  This can indicate improper

functioning or configuration of an application.

Packet Too Small for Send - the number of times

(since the last reset or initialization) applications

have asked the driver to send a packet under the

minimum legal size.  This can indicate improper

functioning or configuration of an application.

Receive Packet Overflow - the number of times

(since the last reset or initialization) the driver has

received a packet larger than the buffer space

allocated for the packet.  When a packet is

received, the protocol stack that accepted the

packet provides a set of buffers into which the

packet should be dispersed.  If there is not

enough buffer space or for any reason the

protocol stack allocated a smaller buffer, this error

occurs.

Received Packet Too Big - the number of times

(since the last reset or initialization) the driver has

received a packet over the maximum legal size.

It may indicate an error in the packet header due

to a collision, or malfunctioning of the sending

application.

Received Packet Too Small - the number of times

(since the last reset or initialization) the driver has

received a packet under the maximum legal size.

This may indicate an error in the packet header

due to a collision, or malfunctioning of the

sending application.

Transmit Misc. Errors - the number of

miscellaneous errors preventing the driver from

transmitting a packet.

Receive Misc. Errors - the number of

miscellaneous errors preventing the driver from

receiving a packet.

Send Retry Count - the number of times (since the

last reset or initialization) the driver resent a

packet.  For  example, when the driver detects a

collision or does not receive a confirmation on a

sent packet, the driver re-sends a packet.  If

significant, this number can indicate high network

bandwidth utilization.  Use Observer to check this

assumption.

Checksum Error Count - the number of checksum

errors occurring while receiving packets (since the

last reset or initialization).  High checksum errors

can indicate bad cabling or a bad hub.

Receive Hardware Mismatch - the number of

times (since the last reset or initialization) the

hardware has received more or fewer bytes than

expected.

Bridge Statistics List (server)

Novell refers to the "Bridge" as the physical LAN

board in your server.

Bridge Statistics List includes the following values:

Total Packets Routed - the total number of

packets the router actually routed.

Too Many Hops - this is too many hops for the

packet (that reached the current server) to reach

it's destination.  It is the number of times (since the

bridge was initialized) the bridge received

packets on the fifteenth hop across an

Internetwork bridge.  These packets are

discarded.

Hop is the transition (or hop) of a packet on its

way to a destination server or station from another

server or station.  This error may indicate that the

routing scheme used on the network is

inadequate, or possibly the server that was

servicing a more direct connection path is

currently down or out of order.  The situation may

require the network administrators intervention.

This error can occur only on larger sites with many

nets and bridges, or on sites experiencing an

"infinite loop" between routers or server.

Unknown Network - these are packets that

reached the server can not be routed because

the destination network is unknown to the server.

This error may appear if a router that was

transferring packets to the known destination was

shut down.  In this case the router sends "going

down" broadcasts upon which all routers update

their routing tables and remove the no longer

available routes.

No Space for the Service - the number of times

(since the bridge was initialized) the bridge has

received Internetwork packets it could not

accommodate because the router did not have

enough space in its DGroup area to copy the

packets.  These packets are lost.

This condition, can be an indication of network

overload, or the server may be too slow to

process amount of data on a growing network.

Monitoring the network bandwidth utilization using

Observer can help clarify this question.  In either

case, if the condition persists and has an adverse

effect on the network performance, there may be

a need add a server and split network load

between them, upgrade the server system with a

faster CPU or more RAM, or reconfigure stations

and servers via segmentation of the wire to

provide a more even load on the bridges and

servers.

No Receive Buffers - the number of times (since

the bridge was initialized) the bridge could not

receive inbound packets because of inadequate

buffer space.  These packets are lost.

This condition is essentially similar to the previous

one, with exception that the error occurs on the

earlier stage of packet processing and indicates

either too small number of buffers, or more

directly network (as opposed to server or bridge)

overload.  Again more definite answer can be

obtained monitoring long and short term network

utilization using Observer.

Not My Network - the number of incoming

packets with a destination other than LAN A.  This

statistic shows all the packets that are routed

through the current LAN.

NetBIOS Propagate - the number of times the

bridge received NetBIOS broadcasts since it was

initialized.

Volume Information List (server)

The Volume Information List provides information

about available disk space on the server volumes.

It includes total and free disk space, and total and

free directory entries for each volume.  It can be

used to check remotely how much free disk

space remains on a server volume, and take

appropriate actions when any of the volumes

reaches critical usage.

Open Files List (server)

The Open Files List displays the number of currently

open files.  This list shows the total number of files

open on a server, and then number of files

opened by each user attached to the server.

Info:

Info menus consist of version and other constant

information that does not need updating on the

server.  The Info menu consists of:

Server Version (server)

The Server version provides an extensive list of

versions of the Novell server and miscellaneous

utilities and services, as well as maximum number

of connections, current connections and

maximum number of available volumes.

IPX-SPX Versions (server)

The IPX-SPX version display contains the specific

IPX and SPX version numbers of the server's

IPX/SPX drivers (not of the node or operating

system).

Bridge Driver Configuration (server)

This displays the logical LAN board driver in your

server.  This is how Novell describes the actual

loadable card driver for each FRAME.  With

respect to the Bridge Driver, each value is:

Max Data Size - the maximum size of a packet's

data portion for the target driver.  The data

portion's maximum size is always 64 bytes less than

the packet size advertised with the LAN board.

(The packet header and control information

require 64 bytes).

Transport times (ticks) - the value indicating the

speed of the LAN associated with the target driver

and board.  Speed is measured by the amount of

time it takes a 576-byte packet to travel from one

node on the LAN to another node.  Time is

measured in units of 1/18 second and rounded to

the next highest 1/18.

Shell Version - the major and minor versions of the

driver release; this does not identify the installed

NetWare version.

Description - the LAN hardware currently

connected to the driver.

I/O Address - the I/O address block to be

decoded by the LAN board the driver is

associated with.  If the board uses a second I/O

addresses, this field will be also present in the list (if

it uses I/O).

RAM Address - the memory address block to be

decoded by the LAN board (if it uses shared

memory).

DMA Line - the DMA line used by the LAN board (if

it uses DMA),

Selected Line - the value indicating which

hardware configuration in the Hardware

Configuration Table the driver is using,

LAN hardware ID - the value that is hard-coded

into the Master Configuration Table.  This uniquely

identifies the LAN hardware.  (i.e. the OEM/Driver

Support Group Manager that Novell assigns this

ID)

Note: Not all fields listed here may be available

from different versions of Novell servers and

drivers.

NODE INFORMATION WINDOWS

A NODE information window is displayed in the

information area after a user clicks on the RIGHT

mouse button on a node icon in the List of Nodes.

NODE information windows (which are very similar

to SERVER information windows) consists of a

button bar with Charts, Lists, Info, and an Options

popdown menu buttons and an Exit button.

Button bar buttons can be selected using mouse

clicks, and in most cases pressing a letter on the

keyboard that corresponds to one displayed in

red on the button. (Note: due to the nature of the

MDI interface you DO NOT use ALT when

accessing the information window buttons.  Use

ALT-KEY combination to popdown menus on the

Main MDI  window).

Context sensitive help is available from any

window by pressing F1.

Charts

The NODE information window Charts menu has

the following choices (sample charts are

displayed in the "Information Windows (general)

section):

IPX Packet Statistics chart (node)

The IPX statistics chart displays the number of sent

and received IPX packets as a function of time.  It

shows the node's IPX packet flow activity.

Malformed packets for send and Get ECB (Event

Control Block) for receive are a sum of several

failure conditions, and can indicate, if significant,

improper functioning or overload of the server or

node.  More detailed information can be seen in

the IPX Statistics List.

This chart can be saved to a file in spreadsheet

format ("File" -> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

SPX Packet Statistics chart (node)

The SPX statistics chart displays the number of sent

and received SPX packets as a function of time.  It

shows the node's IPX packet flow activity.

Bad Send Packets for send and Get Bad Receive

Packets receive are a sum of several failure

conditions, and can indicate, if significant,

improper functioning or overload of the server or

node.  More detailed information can be seen in

the SPX Statistics List.

This chart can be saved to a file in spreadsheet

format ("File" -> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

Total Node Packets chart (node)

The Total Packet Statistics chart displays the total

number of packets (all protocols) sent and

received by a node as a function of time.  It

allows you to see node's total network activity.

"Bad Send Packets" for send and "Get Bad

Receive Packets" receive are a sum of several

failure conditions, and can indicate, if significant,

improper functioning or overload of the node

drivers.  More detailed information can be seen in

Bridge Driver Statistics List.

This chart can be saved to a file in spreadsheet

format ("File" -> "Save Chart in Spreadsheet

Format"), or the current view of the chart can be

copied to the clipboard to be used by an image

editor (Paintbrush, for example).

Options

A user can customize the way charts are

displayed by selecting the Options button in the

NODE information window and then choosing 2D

or 3D , Colors, Timing and Scale options from the

Charts menu.

Under 2D or 3D there are choices of the type of

chart - Lines, Columns and Pie for 2D charts,

Separate columns and Alternate columns for 3D

charts.

The Colors dialog lets customize colors for every

chart separately (for example, Open Files can

have a different chart type and set of colors than

IPX Packet Statistics, and Volume usage, and so

on.

The Timing dialog sets timing periods for each

chart separately, and one common for all Lists.

The Scale dialog lets you change the vertical

scale of a chart to best fit your system

parameters.  The best way to understand this

dialog is to  experiment.

Lists

Lists are updateable tables of information about

nodes or servers. The update period is set in

"Options" -> "Timing" menu.  The NODE

information window List menu has the following

choices:

IPX Packet Statistics List (node)

The IPX Packet Statistics List shows a summary of

IPX traffic since the node's IPX/SPX shell was

loaded.  It includes:

Send Packet Count - the total number of sent IPX

packets.

Malformed Packets - the number of times (since

IPX was loaded) applications have passed

malformed packets to IPX.  A packet is malformed

if the value in its ECB's fragment count field is 0 or

if the value of the size field within ECB's first

fragment descriptor is less than 30 bytes.  This can

be due to malfunctioning of one of the NLMs.

Get ECB Request Count - the number of times

(since IPX was loaded) IPX has supplied Receive

ECBs for incoming packets.  Event Control Block

(ECB) should be provided for every incoming

packet in order to process the packet.  This count

does not indicate an error condition.

Get ECB Failure Count - the number of packets

the driver has received for which there was no

available ECB.

If the nodes shell does not provide enough ECBs,

(usually in high transfer rate conditions) no ECBs

available will result in a failure to receive or send

packets.

Many things can cause this count to rise, including

the condition of a slow network card in a fast

system, or a network card with I/O or memory

conflicts.

AES Event Count - the number of times (since IPX

was loaded) IPX has used the AES to schedule an

event.

Asynchronous Event Service is called when the

stack is not able not keep up with processing of

network traffic and must schedule a send or

receive event to occur at later time (usually within

a few milliseconds).

Postponed AES Events - the number of times (since

IPX was loaded) IPX has been unable to service

an AES event on time.  This can be due to MLID

being in a critical section.  For example - IPX

cannot send an outgoing packet to a ECB busy

with another packet.  In this case the AES must be

postponed (rescheduled) to happen in later time.

If number of Postponed AES Events is high, it can

be indication of to high of network or processing

load on the station.

Max Configured Sockets - the maximum number

of open sockets possible on the target station.

Max Opened Sockets - the maximum number of

sockets open simultaneously since IPX was

loaded.

Open Socket Failures - the number of times (since

IPX was loaded) applications have unsuccessfully

called IPXOpenSocket.  IPX cannot open a socket

if the socket table is full or if the specific socket is

already open.

Listen ECB Count - the number of times (since IPX

was loaded) applications have given IPX a Listen

ECB.

Listen ECB is given when a Netware application

expects to receive a packet associated with a

specific socket (usually a reply to a "just sent

command" or reques).  Neither a low or high

value is an indication of a failure condition.

ECB Cancel Failures - the number of times (since

IPX was loaded) IPX has been unable to cancel

an ECB.  For example, IPX cannot cancel an ECB

if the driver and the ECB have entered a critical

section just prior to sending a packet.  In this case,

the cancellation is too late, and the packet is lost.

Find Route Failures - the number of times (since

IPX was loaded) IPX has been unable to find a

route to a requested network address.

This error may appear if a router that was

transferring packets to the known destination was

shut down.  In this case the router sends "going

down" broadcasts upon which all routers update

their routing tables and remove any routes that

are no longer available.

SPX Packet Statistics List (node)

The SPX Packet Statistics List shows a summary of

SPX traffic since the node loaded IPX/SPX.  It

includes:

Max Connections - the maximum number of SPX

connections possible on the target node.

Max Used Connections - the maximum number of

SPX connections open simultaneously since IPX

was loaded.

Establish Connection Requests - the number of

times (since SPX was loaded) applications called

SPXEstablishConnection.  This call attempts to

contact a partner to establish a communication

session.

Establish Connection Failure - the number of times

(since SPX was loaded) SPXEstablishConnection

calls failed because the SPX Packet Header was

too small, the SPX Connection Table was full, or no

router was found to the target network, or the

target node is down.

Listen Connections Requests - the number of times

(since SPX was loaded) applications called

SPXListenForConnection.  This call is issued by a

Netware application to instruct SPX to watch for a

connection request from a peer system.

Listen Connection Failure - the number of times

(since SPX was loaded) SPXListenForConnection

calls failed because the SPX Connection Table

was full.

This is an indication that the total number of

available SPX connections on the node is not

adequate for current network activity.  This value

is set in the NET.CFG file.

Send Packet Count - the number of times (since

SPX was loaded) applications tried to send SPX

packet.  This number is a sum of all send packet

attempts, successful and not successful.

Window Choke Count - the number of times (since

SPX was loaded) SPX failed to send a packet

because the target station had not allocated a

receive buffer.  This is an indication of high load

on the target station.  This statistics does not show

the station responsible; if necessary, the station

can be determined using Observer in Packet

Capture mode or Protocol Statistics mode.

Bad Send Packets Count - the number of times

(since SPX was loaded)

SPXSendSequencedPacket was incorrectly called

by passing an invalid connection ID or bypassing

the address of an ECB indicating a packet header

size of less than 42 bytes.  This can indicate a

malfunctioning SPX application.  In this case the

connection to the target station is aborted.

Send Failure Count - the number of times (since

SPX was loaded) SPX was unable to send a

packet across an SPX connection and receive

acknowledgment.  This happens when a remote

peer closes the connection without

acknowledging the packet.  In this case there is

no guarantee that the packet was received.  A

connection that closed abnormally may be

caused by a frozen or rebooted station.

In such a case, SPX aborts the connection and

informs the calling application.

Abort Connection Count - the number of times

(since SPX was loaded) SPXAbortConnection was

called.

This event is initialized when some catastrophic

condition (from the point of view of application

that is using this connection) occurs.  This can be

an indication of malfunctioning NLM application.

In this case connection is terminated unilaterally

by the current application without notifying the

other machine.  The partner discovers the broken

connection when it tries to send a packet to this

application (getting Send Failure error, see

above),  or when the Watchdog checks the

connection after the inactivity time expires.

Listen Packet Count - the number of times (since

SPX was loaded) applications gave listen ECBs to

SPX.  Listen ECB is given when a Netware

application expects to receive a packet

associated with a specific socket (usually a reply

to just sent command or reques).

This value is a count and does not indicate an

error condition.  There is no limitation on the

number of Listen ECBs, which should be provided

by a Netware application.

Bad Listen Packet Count - the number of times

(since SPX was loaded) applications passed SPX a

packet whose associated ECB is malformed.  An

ECB is malformed if the value in Fragment Count is

0, if the value in its first Fragment Descriptor Size is

less than 42, or if the listen socket is not open.

Incoming Packets - the number of times (since

SPX was loaded) the node's driver gave an

incoming packet to SPX.  This value is a count and

does not indicate an error condition.

Bad Incoming Packets - the number of times

(since SPX was loaded) SPX received and

discarded a packet because the connection ID in

the packet was wrong.

Suppressed Packet Count - the number of times

(since SPX was loaded) SPX discarded inbound

packets because they were duplicates of

previously received packets.  This condition

indicates the sending station has not received an

acknowledgment for a sent packet (due to delay

or collision).  It may indirectly indicate high

network utilization.  Observer will help determine if

this is the case.

No Session Listen ECB Count - the number of times

(since SPX was loaded) SPX was forced to discard

an inbound SPXEstablishConnection packet

because SPX lacked a corresponding

SPXListenForConnection ECB.

WatchDog Destroy Sessions - the number of times

(since SPX was loaded) WatchDog destroyed a

connection because the connection was no

longer valid.  A WatchDog packet is sent

periodically if no packets are received from a

peer in 5 minutes.  If there is no WatchDog reply,

the station repeats the WatchDog request ten

times in one minute intervals, after which the

connection is considered invalid.

Shell Driver Statistics List (node)

The Shell Driver Statistics List includes the following

entries specific to the node:

Driver Version, LAN driver version for the specified

LAN board.

Statistics Version - the major and minor version

number of the Driver Diagnostic Table.

Total Sent Packets - the number of packets the

driver has successfully transmitted since the last

driver reset or initialization.

Total Received Packets - the number of packets

the driver has successfully received and passed to

the system since the last reset or initialization.

No ECB Available Count - the number of packets

the driver has received (since the last reset or

initialization) for which there was no listening ECB.

In an CPU overload condition there can be no

ECB available, and packets may be lost.

Packet Too Big for Send - the number of times

(since the last reset or initialization) applications

have asked the driver to send a packet over the

maximum legal size.  This can indicate improper

functioning or configuration of an application.

Packet Too Small for Send - the number of times

(since the last reset or initialization) applications

have asked the driver to send a packet under the

minimum legal size.  This can indicate improper

functioning or configuration of an application.

Receive Packet Overflow - the number of times

(since the last reset or initialization) the driver has

received a packet larger than the buffer space

allocated for the packet.  When a packet is

received, the protocol stack that accepted it

provides a set of buffers into which the packet

should be dispersed.  If there is not enough buffer

space or for any reason, or the protocol stack

allocated a smaller buffer, this error occurs.

Received Packet Too Big - the number of times

(since the last reset or initialization) the driver has

received a packet over the maximum legal size.

It may indicate an error in the packet header due

to a collision, or a malfunctioning sending

application.

Received Packet Too Small - the number of times

(since the last reset or initialization) the driver has

received a packet under the maximum legal size.

This may indicate an error in the packet header

due to a collision, or a malfunctioning sending

application.

Transmit Misc. Errors - the number of

miscellaneous errors preventing the driver from

transmitting a packet.

Receive Misc. Errors - the number of

miscellaneous errors preventing the driver from

receiving a packet.

Send Retry Count - the number of times (since the

last reset or initialization) the driver resent a

packet.  For  example, when the driver detects a

collision or does not receive a confirmation on a

sent packet, the driver re-sends a packet.  If

significant, this number can indicate high network

bandwidth utilization.  Use Observer to check this

assumption.

Checksum Error Count - the number of checksum

errors occurring while receiving packets (since the

last reset or initialization).  If this value is high, this

can be an indication of bad cabling or a bad

hub port.

Receive Hardware Mismatch - the number of

times (since the last reset or initialization) the

hardware has received more or fewer bytes than

expected.

Shell Statistics List (node)

Shell Statistics List includes the following values:

Shell Requests - the number of times (since the

shell was activated) the shell has made requests

to a file server.

Operator Aborts - the number of times (since the

shell was activated) the user has aborted the

shell-server connection by entering "A" in reply to

a "Network Error" message.

Operator Retries - the number of times (since the

shell was activated) the user has instructed the

shell to retry an operation.

Time-outs - the number of times (since the shell

was activated) the shell sent a request to a server

and then timed out without receiving a reply.

(Normally, the shell sends another request

packet).

Write Errors - the number of times (since the shell

was activated) the driver has been unable to

send a request to a file server (after repeated

retries).  In this case, the shell displays the message

"Error writing to network" on the workstation

screen.  The shell does not increment this counter

if, after repeated retries, the driver is able to send

the request.

Invalid Reply Headers - the number of times (since

the shell was activated) the shell has received a

reply packet header whose Checksum was -1 or

whose PacketType indicated the packet was not

a file server reply.

Invalid Slot Count - the number of times (since the

shell was activated) the shell has received a file

server reply packet specifying an incorrect

connection ID.

Invalid Sequence Number - the number of times

(since the shell was activated) the shell received a

file server reply packet specifying an incorrect

sequence number.  It usually indicates the reply

was unnecessary.

Errors Receiving - the number of times (since the

shell was activated) IPX has indicated an error

even though a packet was received on the

socket.  It usually indicates an "Overrun" error.

No Router Found - the number of times (since the

shell was activated) the shell tried and failed to

find a route to a destination node.  The shell

attempts to reroute a packet when a connection

seems to fail and the user requests a "Retry.".

Being Processed - the number of times (since the

shell was activated) the shell received a "being

processed" reply from a file server.  A file server

sends this reply to a shell when the server, while

processing the shell's request, receives duplicate

requests from the shell for the same service.

Unknown Errors - the number of times (since the

shell was activated) the shell received a packet

containing an undefined error value.

Invalid Server Slot - the number of times the shell

attempted to communicate on a particular client

connection number, and the server indicated the

connection number is invalid.

Network Gone Count - the number of times (since

the shell was activated) the shell received a

packet from a file server indicating the target

network has gone away.  Only a 68000 file server

can generate this kind of packet.

Allocate Can Not Find Route - the number of times

(since the shell was activated) the shell, asked by

an application to establish a connection with a

file server, could not find a route to the

destination.

Allocate No Slots Available - the number of times

(since the shell was activated) the shell, asked by

an application to establish a connection with a

file server, could not establish the connection

because the file server's connection table was

full.

Allocate Server Is Down - the number of times

(since the shell was activated) the shell, asked by

an application to establish a connection with a

file server, could not establish the connection

because the target file server was down.

Info

Info menus consist of version and other constant

information that does not need updating over

time on a node.  The Info menu consists of:

OS Version Information (node)

OS Version Information of a node includes:

Machine type - type of the computer on which

the shell is running

Operating System - for example MS DOS 6.22

Shell Version - The IPX/SPX shell version number

IPX-SPX Versions (node)

The IPX/SPX versions list shows the IPX and SPX

version numbers of the nodes IPX/SPX driver (not

the shell version number).

Shell Driver Configuration (node)

Information regarding the board driver on the

node (i.e. the ODI or IPX driver):

Max Data Size - the maximum size of a packet's

data portion for the target driver.  The data

portion's maximum size is always 64 bytes less than

the packet size advertised with the LAN board.

(The packet header and control information

require 64 bytes.).

Transport times (ticks) - the value indicating the

speed of the LAN associated with the target driver

and board.  Speed is measured by the amount of

time it takes a 576-byte packet to travel from one

node on the LAN to another node.  Time is

measured in units of 1/18 second and rounded to

the next highest 1/18.

Shell Version - the major and minor versions of the

driver release; they do not identify the installed

Netware version.

Description - displays the LAN hardware currently

connected to the IPX/SPX driver.

I/O Address - the I/O address block to be

decoded by the LAN board the driver.

RAM Address - the memory address to be

decoded by the LAN board (if it uses shared

memory).

DMA Line - the value of the DMA line used by the

LAN board (if it uses DMA).

Interrupt Lines - value of the interrupt used by the

LAN board.

Selected Configuration - the value indicating

which hardware configuration in the Hardware

Configuration Table the driver is using.  This value

ranges from 0 to n or -1,  where n is the maximum

number of configurations supported by the driver,

and -1 means no value (and is not displayed by

Discoverer).

Note: Not all fields listed here may be available

from different versions of Novell servers and

drivers.

User Volume Information (node)

The User Volume Information list provides

information about space restrictions for a specific

user, numbers of used directories, files, and bytes.

Note:  Because server has to check all volumes

and file systems to collect this information,

obtaining this information can take considerable

period of time (up to a few minutes).

REFERENCE

A SHORT OVERVIEW OF TCP/IP FOR TROUBLESHOOTING

TCP/IP is a family of protocols designed to perform

a wide set of network activities.

This overview will give a short description of TCP/IP

from prospective of packet transfer and

monitoring.

While TCP/IP is the name by which we refer to this

protocol, it is really a family of protocols based

around the IP method of addressing.  TCP is one

of the family of protocols included in IP.

The main components of TCP/IP are (there are

numerous less used components):

IP - Internet Protocol,

ICMP - Internet Control Message Protocol,

TCP - Transmission Control Protocol,

UDP - User Datagram Protocol,

ARP - Address Resolution Protocol.

The descriptions here are based on a model that

uses the term "lower" to describe the starting

point of a packet, and higher to describe deeper

and deeper information within a packet.  The

vegetable analogy would be an onion - it has a

skin (the lowest level) and many possible layers

within (each one a higher layer to the last).

TCP/IP datagrams are transferred on top of

(inside) a network specific frame - or packet (the

most common are Ethernet and Token Ring), and

are used in turn to transfer a higher level

specialized protocols that actually define what

kind of activity is performed on the network.

Some of these are: Telnet, FTP, NFS, SMNP, remote

printing, and many others.  In other words, TCP/IP

packets are really packets within packets, that is a

TCP/IP packet is wholly contained in a Ethernet

frame (i.e. packet or wrapper).

In order to transfer information (for example, a

file) through the network, it must be divided in

shorter data units which contain information

about the source from which data originates,

destination, length, position of the particular data

unit in a data stream (of many units), different

kinds of management information, routing in the

case the datagrams have to cross one ore more

bridges, and checksums to insure that data was

delivered without loss or alteration due to network

conditions like packet collisions or noise.  These

units are called datagrams or frames.  They can

be considered as a sequence of bytes ordered

according to certain rules.

To make the process of managing packets

controllable, protocol and datagrams produced

by TCP/IP are built in a layered fashion.  On the

lowest level (and arranged in stream of data at

the first position) is a network specific frame

header.  It can be different for different types of

networks, but in general it contains a destination

and source hardware address, and size of the

packet and  frame type.  This layer, for example,

may define the packet as either Token Ring or

Ethernet.

A network card adds a network specific preamble

and checksum to a packet and converts the

frame into a stream of radio frequency signals

that are transmitted through the network wire.  On

the other end the receiving network card

converts the radio signal back into a stream of

characters, determines if it is to the addressee,

and if so, transfers it to an application that

processes it further.

According to the layered structure, everything

that is placed after the frame header can be

considered as a data portion of the frame.  In the

same fashion each higher level portion can be

considered as a data portion of the underlying

one, making assembling before sending and then

disassembling and extracting transferred data or

a command portion of transmission as easy and

as logical as possible.

In TCP/IP, the frame header that follows the IP

header, is either TCP, UDP, ICMP, or another

higher protocol header, followed by the data

portion of the packet, or a still higher level

protocol header - RPC (NFS) on top of UDP

header, for example.

So, for example, when you look at a packet, you

might first see the MAC portion of the packet,

followed by the IP portion, followed by either the

TCP, UDP, etc. portion (depending on why the

packet was formed) and then the data portion.

Knowledge of basic structure of TCP/IP protocols

will help to determine the possible causes of

problems on your network and help choose the

appropriate actions to correct them.

IMPORTANT POINT! -> Another approach to

figuring out a problem that occurs on one of the

stations is to try to find a good example of how

the network communication should function.  For

example, it may be helpful to capture a TCP

connection or a file transfer that seems to work

properly, and then compare it to one received

from a malfunctioning station or host.

A detailed description of TCP/IP protocols is a

subject for many volumes, and there are many

books available.  Although it could be edifying for

somebody to learn everything available on the

subject, many problems can be detected and

solved with basic understanding of the main

TCP/IP protocols.  The next section gives a short

overview of the most important packet structures

as they are viewed by Observer and a concise

explanation of each of the labels (in View

Packets) and their function.

MONITORING TCP/IP SESSIONS WITH OBSERVER

The descriptions here decode each type of

packet, starting at the beginning of the packet

structure and moving down, layer by layer.

Ethernet Frame Header

An Ethernet frame contains a preamble and a

frame checksum, which are added and removed

by the Ethernet network card, and are not passed

to the higher levels.  TCP/IP on an Ethernet

Network uses an Ethernet II type of frame.  This

consists of a destination and source hardware

address and frame type field.  The hardware

address field is six bytes long and uniquely

identifies a network card on the network.

In Observers View Packets window the Ethernet

Frame header is shown in two places.  First in the

List of Packets (in the top portion of the screen),

where all packet are displayed in order of arrival

from the network.  Hardware addresses use the

following notation (example): 80:00:00:21:D5:98,

where every number separated by a colon is a

byte (number from 0 to 255) represented in a

hexadecimal form.

The next field in the List of Packets is the frame

type. For example, ARP (0806),  IP (0800), or

Netware Ethernet II (8137).

There is a bit of inconsistency in the type field.  If

the frame type is smaller than 1518, this field

actually means the length of the packet - the

frame type for all the packets below 1518 is called

IEEE 802.3 (real 802.3 as opposed to Novell 802.3).

Using Observers List of Packets options.

In addition to displaying hardware addresses and

frame types, the List of Packets can show

difference in time of arrival (and thus time sent)

for the current packet and the previous packet.

This is called the packet delta.  To enable this

option select differential times option in "Settings"

-> "View Configuration", and this entry will appear

in the List of Packets.

All hardware addresses can be aliased to the

names of users or any desirable text from the list of

hardware addresses in Filters dialog.

Token Ring Frame Header

The Token Ring frame header is very close to an

Ethernet header.  It consists of start and end

delimiters (performing a similar functions as a

preamble in Ethernet header).  There are two

additional bit fields: Access Control field (AC) and

Frame Control field (FC), followed by hardware

destination and source address, which may be

followed by a routing information field.  Because

there is no frame type field, a Token Ring frame

header is always followed by IEEE 802.2_SNAP, or

IEEE 802.2 subheader specifying the higher level

protocol used.  TCP/IP packets are formed on top

of the 802.2_SNAP header.

An Access Control field has the following bit fields:

Priority Bits - states the priority of a frame or a

token.  The value can be from 0 to 7.

Token Indicator - if this field is not set the packet is

a token, not a frame.  These packets are not

displayed by Observer.

Monitor Count Bit - this allows the Active Monitor

to determine if the packet was not removed from

the ring by the responsible station, and does so if

necessary.

Priority Reservation Bit - this sets the priority of the

packet.  Higher priority packets will be processed

before lower priority packets.  Values are from 0-7.

The Frame Control field determines whether the

frame is a MAC or data frame.  It consists of the

following fields:

Frame Type - this determines whether the frame is

MAC or data,

Control Field - which is valid only for MAC frames,

This specifies the MAC control message or mode

of processing the following data fields.

Hardware destination and source addresses are

similar to Ethernet headers, except that the

highest bit in the first byte of the source address

serves as a routing information indication.  When

this bit is set, the routing information field

immediately follows the source address.  The

routing field has a few subfields which specify if

the frame is a broadcast, and if it is, what type of

broadcast, maximum size of the frame, direction

of the routing path (either originator or

respondent), and the path itself.

IEEE 802.2 and IEEE 802.2_SNAP subheaders

These subheaders can be used as extension of an

Ethernet frame header (IEEE 802.2_SNAP) header,

and is always used as an extension for the Token

Ring frame header to specify the overlying

protocol.

The first two fields in both subheaders are

Destination Service Access Point (DSAP) and

Source Service Access Point (SSAP), which denote

the destination and the packets (which is the

source) upper layer protocol.

For example:

If DSAP and SSAP are 0xE0, the subheader is IEEE

802.2 (Novells Ethernet 802.2), and the protocol is

IPX.

If DSAP and SSAP equal 0xAA, the subheader is

IEEE 802.2_SNAP.

Both subheaders have a control field (Netware

protocols do not rely on this field).  This field closes

the IEEE 802.2 subheader.  The first field in IEEE

802.2 specifies the protocol; and because the

same field in IEEE 802.2_SNAP specifies that this

subheader is IEEE 802.2_SNAP, there is one more

field of this subheader which specifies the

protocol.  Protocol numbers in 802.2_SNAP

protocol field are the same as used in an Ethernet

II frame header.

Internet Addresses

When systems to communicate with each other

on Ethernet and Token Ring networks they use 48

bit (6 byte) hardware addresses.  Hardware

addresses are usually assigned by manufacturer

of a network card.  Different manufacturers have

their assigned major card numbers, and create

unique minor card numbers for every card.  On

most networks there are network cards of different

makes, which creates quite wide range of

hardware addresses.

To make internetworking routability easier and be

able to assign some systemized set of addresses

for the station on a local net and an Internet as a

whole, Internet addresses use 32 bit (4 byte)

numbers instead of 6 byte hardware addresses.

The Internet addresses are uniquely assigned by

an Internet authority for every network (This can

be you if you are not connected to the Internet,

or THE Internet authority if you are).

IP addresses are usually represented by four

numbers in range from 0 to 255 each separated

from the next by a ".".

For example:  200.200.200.18

Internet addresses cane be either class A, B, C or

D - each determines how many machines can be

on the local network.  A class A addresses can use

3 lower bytes as a local address (9.27.6 for an

address of 9.27.6.100, for example), B - two bytes,

C - one byte, and D are multicast addresses.  This

allows, for example, a class A addresses to have

up to 65,536 machines on the network, and for

class C only 256.  To make even finer subdivisions

subnet address masks can be used to assign only

a part of an address to a network address space.

Address Resolution Protocol (ARP)

Because the IP layer uses a four byte Internet

Addresses, and frame headers contain actual

hardware addresses of the network cards

receiving and sending the datagrams, it is

necessary to find out what hardware address

corresponds to an IP address (and store this

information in the ARP table for later use).

Address Resolution Protocol (ARP) serves this

purpose.

If a station needs to establish a TCP connection to

a server or peer, there are two possible scenarios:

1. If a connection was already established to the

destination station, the hardware -to- IP

address mapping may already be in the local

ARP table.  In this case the station will directly

proceed with establishing connection using a

hardware address from the table.

2. Otherwise an ARP broadcast packet is sent to

the IP address that the connection needs to

be established with, and the station waits for a

reply from the station in question.  This ARP

packet contains the hard network address and

IP address of the sending station.  The

requested station, if it is present on the

network, sends an ARP reply with its hardware

address as one of the reply fields.  The

requesting station places the IP -to- hardware

correspondence in the local ARP table, and is

ready to establish connection.

An ARP message contains the following fields:

Type of Operation - this either indicates an ARP

request or ARP reply.

Source Host address - the IP address of the

requesting host (or replying, if the ARP message is

a reply).

Destination Host address - the IP address of the

requested host (or the host that send the request,

if this is a reply).

Source Hardware address and Destination

Hardware address - the hardware addresses of

sending and receiving station.  If it is an ARP

request, the destination hardware address is set to

zeroes; an ARP reply always contains both

addresses.

Internet Protocol (IP)

The Internet Protocol Datagram is the second

level of encapsulation after the frame level.

Encapsulation allows IP to be completely ignorant

concerning the type of network packets that are

sent through.  IP headers look the same for all

networks and frame header+subheader

combinations.  While Internet Protocol can be

considered as a data portion of the frame, it in

turn has as its own data portion - TCP, UDP, ICMP

or another higher level protocol(s).

A station can only begin to send IP packets if it

has resolved the IP to hardware address mapping

(see ARP section).

If the destination station is not on the local

network, and the Datagram is transferred through

a bridge or gateway, the layered structure allows

the bridge to remove the frame header of the

incoming frame, replace it with its own and send it

in the next network destination, (which can be a

different type of network) until it reaches the

destination host.

The main function of IP is packet delivery (and

routing packets through Internet bridges and

routers).  IP itself does not contain any provision to

make delivery of the packet reliable.  It delivers

packets by "best effort attempt" through the

network to their destination, but IP has no way of

knowing if the packet was lost, underwent a

collision, could not be received by the destination

machine due to network congestion or

destination machine overload.  Responsibility for

taking care of  these kinds of problems is done by

a higher level protocol (TCP, or by an

application).

An overview of the IP header structure will show

the means by which IP tries to meet its goal.  The

IP header has the following fields:

Source host - the Internet address of the sending

station.

Destination host - the Internet address of the

destination station.

Version - the version of IP protocol that was used

to create the Datagram.  It is used to see if the IP

header format is the same on sending and

receiving stations.  The current IP protocol version

is 4.

IP header length - this specifies the length of the IP

header.  Most of the fields in the IP header are

fixed, except IP options, which can be absent, or

be of variable length.  If IP options are not

present, IP header length is 20 bytes (or 5 four

byte words).

Service type - this field is split into four bit fields:

Precedence - ranging from 0 (normal

precedence) to 7 (network control).

Currently most hosts and gateways ignore

precedence.

The next three fields are the mode of transport the

Datagram requested:

Delay - normal or low,

Throughput - normal or high,

Reliability - normal or high.

Total IP length - the length of IP Datagram in

bytes, including the IP header.

ID - the identification field is a unique integer that

identifies the Datagram.  It is used to reassemble

fragmented packets.

Fragments - this can be "Yes" or "No".  There are

a few bit fields that give more detailed

information about the Datagram fragmentation,

but for practical purposes it is only important to

know if the packet is fragmented or not.  More

detailed information about fragmentation is rarely

needed, and can be obtained from viewing the

IP header in raw mode.

Time to live - this specifies how many hops the IP

Datagram is allowed to perform on the Internet

system.

Protocol - this specifies what higher level protocol

is encapsulated as a IP data.

Header checksum - the checksum that ensures

that the IP header is not corrupted during

transmission.

The IP header contains many fields.  If you are

interested only a few of them, use Observers

post-filtering in the "Settings" -> "View

Configuration" dialog.  This lets you select which

fields will be displayed.  For example, you may be

interested only in source and destination address

and protocol type.

Transport Control Protocol (TCP)

The same way as IP can be considered a data

portion of network frame, the TCP portion of the

Datagram can be looked at as IP data.  TCP adds

one more level of complexity and functionality to

the TCP/IP protocol set.

TCP is a data stream and connection oriented

protocol.  In order to communicate two machines

must first establish a connection.  A unique port

number is assigned to each connection on both

machines.  There can be more than one

connection established between two machines -

using different port numbers gives the means to

distinguish between them.

While IP provides "best effort" in delivering a

packet, it can not guarantee the packet delivery

or reception.  TCP protocol provides mechanisms

to make the overall delivery service reliable.  This is

achieved by means of positive acknowledgment

(ACK) and retransmition.  In TCP, the recipient of

data must send back to the sending station

acknowledgment about received data.  If the

period in which the sender expects to receive an

acknowledgment expires,  the sender will re-send

the packet.  Sometimes for efficiencies sake, a

station may "piggyback" an ACK along with the

next bit of data - this is well within the TCP/IP

specification.

When analyzing a TCP data exchange it is

important to understand  the concept of a sliding

receiving data window.  Let go briefly go through

this.  If a sender was waiting for the

acknowledgment on every packet sent, the data

exchange rate would be quite slow.  Using a

sliding window mechanism, the two machines

can exchange information about how much data

each machine can send and receive.  This is

called the window size.  When sending a data

stream, the sender sends packets without waiting

for acknowledgment until it sends enough data to

fill the known receiving station window size.  The

other side sends back acknowledgments as it

receives data, and the sending machine

continuously updates the amount of data it still

can send.  Ideally, if no packets are lost, this

mechanism allows transmission of large amounts

of data completely without idle periods while

waiting for acknowledgment.  This also handles

the common situation with a multi-tasking

operating system where the receiving machine

may be to busy doing something else to

immediately send an acknowledgment.

From network traffic analysis point of view this

means that one can not expect that every

packet is going to be immediately

acknowledged.  In addition, checking the stream

of data sequence and acknowledgment number

patterns can help in discovering a malfunctioning

TCP interaction.  These problems are much more

common than most people expect.  Different TCP

implementations react differently in certain "soft

error" conditions.  Just because Hewlett-Packard

or IBM wrote the host TCP software, this does not

make that TCP infallible.  It is the job of all TCP/IPs

(and specifically host based TCP/IPs where size,

speed and memory limitations are few) to handle

and try and overcome error conditions.

Observer can help to find the causes of problems

that manifest themselves as slow transport rate or

stopping and going during terminal sessions.  This

knowledge can be used to tune either the host or

station TCP software, or to request from the

software provider more specific information about

the solution to the displayed problem.

The TCP packet header includes the following

fields:

The TCP label itself, which can contain, if it is

present in the TCP header, a Code field which

indicates the purpose of the packet.  For

example, it shows whether  the packet is an

acknowledgment to a received data stream.

Source port and Destination port  identify the port

assigned to an application on both sides of the

connection.  If the port number is one of well

known origin, Observer displays the name of

service the port provides (i.e. telnet for port 23).

Sequence number - this identifies the position

where the current packet should be attached in

the senders stream of data.  If the packet

contains data, the next sequence number will be

increased by the size of the transferred data

(provided the data was acknowledged, or the

next packet fits into the window size).

Acknowledgment number - this indicates until

which position the data in the receivers current

packet stream has been received.  Or, in other

words, starting from which position in the stream it

expects data to be accepted.

TCP header length - this specifies the length of the

TCP header.  Most of the fields in the TCP header

are fixed, except TCP options, which can be

absent, or be of variable length.  If the TCP option

field is not present, the TCP header length is 20

bytes (or 5 four byte words).

Window - or window size - the amount of data in

bytes that the sender of the packet has

advertised that it is able to receive.  Usually it is

equal the size of the buffer assigned by the TCP

implementation to the current connection.

TCP data length - the length of data that follows

TCP header.  In most cases it is raw TCP data, for

example, the case of a file transfer or terminal

emulation.  It can also be a still higher level

header with its own data, according to the

general encapsulation model.

Checksum - the TCP checksum is calculated in a

similar way as the IP header checksum.  In the

case of TCP segments it includes in the

calculation both the header and the data.

Applications use the TCP checksum to ensure that

the TCP segment (and thus the packet) arrived

intact.

Sequence number + data length - this is not

exactly a part of the TCP header.  It is provided by

Observer to enable a user to conveniently

compare sequence and acknowledgment

numbers in the stream of data without constantly

adding what can be quite long sequence and

data length numbers.  Using this value one can

immediately know what sequence number to

expect in the next packet if the packet was

accepted (ACKed) successfully.

TCP options - there are three TCP options. They

are:

End of Option List, No Operation, and

Maximum Segment Size (MSS).  The only

one of importance is the Maximum

Segment Size option.  The sender of the

packet with this option informs the other

side about the maximum size of the TCP/IP

data buffer of the receiving system.

Usually, the Maximum Segment Size TCP

option appears only once during the initial

negotiation between systems - although

there is no restriction for this to be used

more than once.

User Datagram Protocol (UDP)

UDP like TCP is also a higher encapsulation level

protocol of IP protocol.

UDP uses IP to transport messages from one

machine to another.  Unlike TCP, UDP does not

provide a reliable mechanism for data transfer.  It

does not require acknowledgment of received

data and does not provide means to control data

position in transmitted datagrams.  Datagrams

can be lost, or arrive out of order.  However the

same way as TCP, UDP also provides multiple port

destinations that can support multiple

connections from one station to another system.

Applications using UDP as a transport must

accept a full responsibility for making UDP a

reliable communication process.  This includes

managing potential loss of packets, delays,

receiving duplicates of packets, etc.

Although it may sound as if UDP is an inferior

protocol to TCP, it has its own advantages (speed,

block transfers, etc.), and there are many

protocols and applications using UDP.  A couple

of good examples are NFS and SNMP.

UDP headers include the following fields:

Source port and Destination port identify the port

assigned to an application on both sides of the

connection.  If a port is one of the well known

ports, Observer displays the name of service this

port provides.

UDP length - the length of the UDP Datagram,

including the length of the header.

Checksum - this is checksum calculated on the

whole UDP Datagram (including UDP header).

This value is optional, and a zero is placed in the

UDP header checksum if it was not calculated.

However, using UDP checksum is the only way to

ensure that a Datagram arrived intact, and it is

usually calculated.

Internet Control Message Protocol (ICMP)

On an IP network, each bridge or gateway

operates autonomously.  Sometimes datagrams

can not be delivered to their destination - either

because the destination machine is down or out

of order, or the network the system is on cannot

be reached (permanently or temporarily), or

because a bridge or router is down or

disconnected, or because of delays in the

gateways and the datagrams time to live expires,

or a bridge network is overloaded and can not

deliver incoming datagrams.  In any of these

cases some mechanism is necessary for delivery

of the error messages.  This mechanism is called

Internet Control Message Protocol.  ICMP is

designed to report error conditions to the original

source of data transmission - the host or station

that sent the failed Datagram.

ICMP messages are transferred through the

network as data portion of IP Datagram.  This

means that ICMP messages themselves can be

potentially lost.  In this case to avoid generation of

error messages about error messages, new error

messages about ICMP errors are not generated.

ICMP contain the following messages:

Echo reply - returns exact copy of data sent by

Echo request.  This message has Identifier and

Sequence number fields used to match  replies

with requests.  It can be used to test destination

reachability and status (this is what PING uses).

Destination unreachable - this reports about an

unreachable destination with a description of

possible reasons for problem.

Source quench - this is sent when a Datagram

can not be accepted by a gateway or

destination station because of a network

congestion or station overload.

Redirect - if gateway detects that host is not using

an optimal route to route a Datagram through

the network, a Redirect ICMP message is sent to

the host.  The original Datagram is still delivered to

the destination.

Time exceeded - sometimes a routing path has

too many hops, or an error in the routing table

produces a circular routing cycle.  In this case

when time to live (hop count) reaches zero, the

Datagram is discarded by the gateway on which

it happened.  This gateway sends ICMP Time

exceeded message to the source host notifying it

about this condition.

This message is also sent when time-out for

reassembling of a fragmented Datagram was

reached.

Parameter problem - this ICMP message is sent to

the source host when an error occurs in a

Datagram, for example a gateway receives a

Datagram with incorrect Datagram header.  A

pointer field indicates which parameter in the

original Datagram is considered to be incorrect.

Timestamp request and Timestamp reply are used

to synchronize clocks between machines.

Address mask request  - this is used by a machine

that needs to receive a subnet mask for current

net.  If the machine knows the address of a

requested host, it can send it directly to the host,

otherwise the message can be broadcasted.

Address mask reply serves to return a subnet

mask to the requesting machine.

SHORT OVERVIEW OF NOVELLS IPX AND SPX PROTOCOLS

Netware uses two network communication

protocols: IPX and SPX.  SPX has two flavors: SPX

and SPX II which is an extended version of SPX.

In the general scheme of things IPX and SPX can

be considered as a data portion of the network

packet frame, the same way as TCP/IP protocols

are in relation to a packet frame.  The discussion

of Ethernet and Token Ring packet frames are

also applicable.  (For more information see

sections about the packet frame headers in the

TCP/IP protocols overview).

IPX and SPX provide different types of services for

the applications using them.  IPX employs

connectionless services, like TCP/IPs UDP, while

SPX uses a connection oriented model somewhat

like the TCP protocol.

This means that IPX does not provide reliable data

delivery.  Data can be lost, or be received out of

order, and the application or protocol employing

IPX must handle these conditions.  Being a

connectionless protocol, IPX supports broadcast

of packets.

On the other hand SPX and SPX II are connection

oriented protocols.  They do not support

broadcasts, and for two stations to send and

receive data they must establish connection first.

When a connection is established, all received

data packets are acknowledged by the receiving

station.  The next sections will give short overview

of IPX and SPX protocols, packet headers and

some higher level protocols used by IPX from the

prospective of network monitoring and analysis.

Internetwork Packet Exchange (IPX) packet header

Let consider the structure of an IPX packet.  This

will help show the ways IPX meets its goals.

IPX packet header has 10 fields:

Checksum - this field is the first field of the IPX

packet and is not usually used by Novell Netware.

This field is always set to 0xFFFF in Netware before

4.0, but starting from Netware 4.01 it can be

optionally used.

Packet length - this is the length if IPX packet

including IPX header and data.

Transport control - this field shows the number of

routers a packet passed.  This number is set to 0

by the source station, and then is increased by

one by every bridge passed.  When the count

reaches 16, the packet is discarded, which limits

reachability of IPX packets to 15 routers.

Protocol type - this defines the protocol a packet

will use.  IPX uses 0 or 4, SPX - 5, NCP - 17.

Destination Address comprises two IPX header

fields: destination network address which is the

four byte address of the network to which the

node belongs, and destination node address

which is a six byte hardware address of the

network card in the node.  If the destination node

is a NetWare server, hardware address is replaced

by address 00-00-00-00-00-01;  the network frame

address will still contain the network card

hardware address.  The destination address can

be a broadcast address (the largest of all possible

addresses, namely FF-FF_FF-FF-FF-FF).

Destination Socket - this defines the destination

process the packet is addressed to.  Some of the

main processes will be discussed later, and

include:

NetWare Core Protocol (NCP),

Service Advertising Protocol (SAP),

Routing Information Protocol (RIP),

NetBIOS,

Diagnostics,

Serialization.

Stations use a range of socket numbers from 4000

to 8000 hexadecimal to specify the remote or

local process entry point numbers of the process.

Source Address - this also comprises two IPX

header fields: source network address, and

source node address, which are similar to

destination addresses, and define the sending

nodes coordinates.

Source Socket - this field is similar to destination

socket number.

Sequential Packet Exchange (SPX) packet header

The SPX headers first ten fields are exactly the

same as an IPX packet headers first 10 fields.

For comparison purposes, if it was defined this

way, the SPX part of the header could be

considered as a higher level protocol to IPX, just

as UDP is for IP.  However, the SPX header not only

has the same 10 fields as IPX, but it also has its

own additional fields.  Lets look at the following

seven fields additional to SPX, plus one more

additional field of SPX II:

Connection flag - this is a one byte bit field that

can have the following values:

System - designates a system packet (used

internally by the server).

ACK - indicates that data was sent and

acknowledgment is required.

End of Message - indicates that the sending

side wants to close connection.

Datastream type - this is used for the end of

connection request and acknowledgment.

Source connection ID - this specifies the

connection number for the source SPX.  It is used

to determine the difference for potential multiple

connections from the same node and to the

same source socket.

Destination connection ID - this is similar to Source

connections ID except that during the initial

establishment of the connection the Destination

connection ID can be set to -1 (FFFF

hexadecimal), since the destination is unknown at

this time.

Sequence number - this is the number of packets

transmitted from the node.  It is incremented by

one after reception of acknowledgment from the

destination node for the packet sent.

Acknowledgment number - this is the value of the

sequence number expected to be received.  If

packet is lost, and sequence number does not

reach the acknowledgment number after the

time-out period, a sending node assumes that the

packet was lost.

Allocation number - this is the number of available

receive buffers on the station.  The actual number

of buffers is equal to the allocation number plus

one.

A value of 0, for example, means that one buffer

is available.  If the station receives packets and

can not keep with the packet processing, the

allocation number decreases, as packets are

processed, the allocation number is increased

with each released buffer.

SPX II is an enhancement of SPX supporting

packet sizes up to 1518 bytes (SPX can send and

receive maximum 576 bytes packets).

Additionally, while SPX requires acknowledgment

for every packet,  SPX II employs a window size

mechanism close to that of a TCP sliding window.

To accommodate these additions, SPX II packets

contain one more field, and in addition, new

definitions for connection control and Datastream

fields.  It also uses an enhanced packet

sequencing mechanism.

The additional values in the Connection flags field

are:

Size - this allows SPX II to negotiate the size

of the send and received packets.

SPX II packet type - this field indicates that

the packets are in fact SPX II.

Datastream type also contains two additional

values:

Orderly release request, and

Orderly release acknowledgment.

The additional SPX II field is called Extended

Acknowledgment.   This is for the situation where

one station uses SPX and the other an SPX II

connection.  In this case, the SPX II station

automatically switches to SPX during initial

connection establishment.

NetWare Core Protocol (NCP).

While IPX and SPX are the means of transfer of the

packets, NCP is the main communication

protocol used by Novell NetWare - as is implied by

the word "Core".  NCP if a higher level

encapsulation protocol to IPX and SPX.  NCP

controls the establishment of connections, opens,

transfer and closes files, establishment of service

queues, and performs many other tasks.  Overall

there are over three hundred NCP functions.

The NCP protocol is comprised of a series of

requests and replies.  A station sends a NCP

request with a number of a function specified in

the NCP header, and waits for a reply - usually

confirmation of the execution.

NCP headers are slightly different for request and

for replies.

REQUEST HEADER contains the following fields:

Request type - this field is displayed by Observer

next to the NCP label itself.  It specifies the type of

NCP message, and can be one of the following:

1111	Create service connection.

2222	Service request.

5555	Destroy service connection.

7777	Burst mode transfer.

When a station connects to a server it sends a

Create service connection request, and when

disconnecting it sends Destroy service

connection.  The service request is an example of

the most common field encountered in NCP.

The fields following the NCP header specify

precisely what kind of request the NCP packet

carries.  These are:

Sequence - this field is a one byte field that keeps

track of the sequence of commands between the

station and server.  It is increased by one with

every packet until it get to 255 and then wraps

around to 0 at which time it begins again.

Connection - this is a service connection number

that the server assigns to the station during the

initial connection.

Task - this is the number of the task that is making

a request.  It also allows the server to de-allocate

specific resources when a task terminates.

Connection high - this is an additional connection

number that can appear only on a 1000 user

versions of Netware to keep track of the higher

number connections.

These fields can be followed in the NCP request

by the following fields:

Function code - a major number of function being

requested.

Length - this is the length of NCP data, if present.

Sub-function code - the sub-function number of

the major function.

For example, the function (58h) has the sub-

function codes:

Get Volume Audit Statistics (01), or

Add Audit Property (02), etc.

REPLY HEADER contains the following fields:

Reply type - this field is displayed by Observer next

to the NCP label.  It specifies the type of NCP

reply message, and can be one of the following:

3333	Service reply.

7777	Burst mode connection.

9999	Request being processed.

Service reply - this  is the most common reply and

used as an answer to most requests.

During the transferring of files in burst mode, the

server uses a Burst mode connection reply.

Request being processed appears when a server

accepts a packet and queues it for execution,

but is not able to process it immediately.  If the

station time-out expires and it repeats the request,

the server may reply again with Request being

processed.

Sequence, Connection, Task, and Connection

high  fields are similar to NCP request fields and

are followed by:

Completion code - which indicates whether the

client request was successful.

Connection status - this indicates if the server is

going down.  This field is usually set to 0, and when

the server is going down , this value in NCP replies

is set to 1.

The BURST MODE HEADER is different from the NCP

request and reply header and contains the

following fields:

Request type field, which is displayed by Observer

next to NCP label, has the value of 7777

hexadecimal.

Flags - this is a bit field that specifies the mode or

condition of the connection.

Stream type - this currently has the only value of 2,

which means burst.

Source ID - this is a unique (random) number

created to identify a current burst connection by

a sender.

Destination ID - this is a unique number similar to

Source ID, but created by the receiver.

Packet sequence - this number is similar to a NCP

non-burst sequence number and is used to track

transmitted packets.

Send delay time - this specifies the delay time

between packets in burst in units of 100

microseconds.

Burst sequence number - this is the sequence

number of the burst.  Each burst is considered as

one unit by this count.

Acknowledgment sequence number - this is the

sequence number the node expects to receive

next.  This indicates whether the last burst

transmission was received successfully.

Total burst length - this specifies the length of the

whole burst transaction.

Burst offset  - this is the number of the packet in a

burst.

Burst length - this is the length of a current burst.

Fragment list - this defines the number of elements

that are missing from the transaction.

Function - this specifies if the current burst is a read

or write.

Service Advertising Protocol (SAP).

Service Advertising Protocol (SAP) provides service

information on all known servers throughout the

internetwork.

SAPs are transmitted in two situations.

1. There are periodic SAPs broadcasted by a

server on a LAN.  This periodic broadcast

occurs at 1 minute intervals.  If a server does

not broadcast SAP information in more than

three minutes, all  routers and servers assume

that the server is not available, and remove it

from their binderies.

2. SAP packets are sent by the server (in this case

a SAP reply) when a station loads a redirector

(NETX or VLM).  NETX (or VLM) sends a SAP

request to locate the nearest server.  This

request is called the "Get Nearest Server"

request.

SAP messages are transferred in IPX packets with

the Socket number set to 452h (SAP).

SAP REQUESTs contains the following fields:

Packet type - this currently has only one value:

nearest service request.

Server Type - this specifies the type of server

requested.

The most common are:

File server - (04)

Job server - (05)

Print server - (07)

Job queue - (0A)

etc.

SAP RESPONSE contain the following fields:

Packet type and Server Type similar to those fields

in SAP request, followed by the list of servers that

are specified in the following fields:

Server name - the name of the server.

Network address - the 4 byte network address.

Node address - a 6 byte server node address

(contains 00-00-00-00-00-01 for all servers).

Socket - this is the socket number that receives

the service requests.

The fields above will be repeated as many times

as are the number of servers that are in the list.

Routing Information Protocol (RIP).

Routing Information Protocol passes the routing

information between servers and routers.  RIP is

sent for the following reasons:

1. When router initializes, to inform other routers

that it is available.

2. When router first loads it requires initial routing

information from other routers, and thus

requests this information via RIP.

3. For periodical update of the routing tables the

RIPs are broadcast at one minute intervals.

4. When router changes any routing information

in its tables it sends a RIP broadcast to notify

other routers about the change.

5. When a router is going down, it notifies any

other routers.

RIP messages are transferred by IPX packets with

value of 453h in Socket field.

Routing information packet have the following

format:

Packet Type - a request or reply, followed by three

fields which are repeated as many times as there

are routes:

Network address - this is the address of the

network.  In a RIP request, this field can be set to

broadcast and receive information for all

available networks.

Hops - this specifies distance to a network in

number of bridges that has to be crossed.

Ticks - this specifies amount of time necessary to

reach the network. One tick is equal 1/18 of a

second.

Serialization packets.

Serialization packets are sent periodically to

ensure that a server with the same serial number is

not installed twice on the same network.  Every

NetWare server is sold with the unique serial

number.  Loading more than one server with the

same distribution copy is not allowed, and

serialization packets check and notify everyone if

a license is being broken.

Serialization packets are transferred as broadcasts

in IPX packets with the Socket number of 457h.

WatchDog Packets

The WatchDog protocol is used to determine if a

station that does not transmit packets for a long

period of time is still up.  If the station is inactive for

5 minutes (a default, which can be changed), a

server sends a WatchDog packet to the station.  If

the station is alive, it sends a WatchDog reply.

If the server does not receive a Watchdog reply, it

continues to send Watchdog packets every

minute for ten minutes (both values are defaults

and can be set differently).  If the server does not

receive the Watchdog reply after this period, the

server considers the connection closed and clears

connection from the connection table.

APPENDIX

PROGRAM DESCRIPTIONS

Observer is a Microsoft Windows based program

that captures and analyzes packets as they are

broadcast onto your LAN.  Observer has both DOS

and Windows components.  Discoverer is also a

Microsoft Windows based program, but it does

not require any DOS based programs for function

(from Network Instruments - it does require ODI

and the VLM shells to be loaded at the

workstation).

Observers main components are:

OBSERVER.EXE is the main program for viewing

and capturing LAN information.

OBSERV.DLL is the Windows Dynamic Link Library

for  OBSERVER.EXE.

VPACK.EXE is the Packet Driver interface that

makes network packets available to the Windows

programs.  VPACK.EXE will only work with NDIS or a

Packet Driver.  For more information on the

Packet Drivers, see the Installation section or this

Appendix.

VPACKODI.EXE is the ODI interface that makes

network packets available to the Windows

programs.  VPACKODI.EXE will only work with

Novells ODI.  For more information on the ODI,

see the Installation section or this Appendix.

OBSERVER.HLP is the Windows help file for

OBSERVER.EXE.

OBSERVER.INI (stored in the Windows directory) is

where Observer stores Windows information.

OBSERVER.ADR (stored in the OBSERVER

installation directory) stores all of the hard address

and alias information.

OBSERVER.FLT (stored in the OBSERVER installation

directory) stores the saved filter information.

DEMOETH.LST the DEMO data for Ethernet.

DEMOTOK.LST the DEMO data for Token Ring.

INSTALL.EXE the installation program.

INSTALL.DAT installs data file.

OBS.DLL used by Observer and Discoverer.

DISCOVER.NIC stores information about the

charting and display for Discoverer - this is not a

text file.

DISCOVER.NOD stores information about the

NODES Discoverer finds.  This is not a text file.

DISCOVER.SRV stores information about servers.

This is not a text file.

DISCOVER.HLP the help file for Discoverer.

DISCOVER.EXE the main Discoverer program.

NICHART.DLL the charting DLL used by Discoverer.

TROUBLESHOOTING

Although most installations of Observer should

proceed without any trouble, due to the vast

number of network configurations and PC

hardware/software options that Observer

supports, sometimes trouble arises.

If you experience trouble in setting up Observer,

keep a number of things in mind.

First and foremost, try to simplify your setup in any

way possible.  This means if you have a screen

saver loaded, disable it.  If you are running some

fancy network add on peer-to-peer jet engine

turbo stimulator, remove it.  This does not mean

that you will not be able to use Observer with your

other products, but if you can determine where

the problem is, you can focus on that piece of the

puzzle and you may be well on your way to

solving the problem.

Second, dont trust anyone or anything.  The only

way to really know what your hardware settings

are is to have the card or device in one hand and

the manual in the other.  Programs that try and

discover interrupts and other settings only function

properly when everything is working correctly -

exactly when you dont need them.  Don't trust

other network drivers - they may or may not be

reporting the correct information.

Third, you can not, under any circumstances,

share interrupts, i/o ports, or memory addresses

between adapters.  No matter what worked

before, or what might work in the future, sharing

interrupts or memory settings is not a valid

configuration.

Troubleshooting checklist:

1. Does your network work without any Observer

programs or drivers loaded?  If not, check your

network installation instructions.  Once your

network appears to be running correctly, try

and install Observer again.

2. Does Observers VPACK or VPACKODI driver

load and report back that all is well?  If not,

check the underlying network driver and verify

that you have loaded the specific driver

according to the Observer installation

directions.

3. Try installing Observer on a different PC and

see if you experience the same problem.  This

does not mean that you will not be able to use

Observer on the desired PC.  It may give you

some insight into the problem you are having.

Observer Troubleshooting

Packet Driver:

Getting all 00s or all FFs in the Packet Driver load

screen?

Check your card settings and verify you are giving

the Packet Driver the correct information.

Double check that the card is not set to an

interrupt that is being used by another adapter.

[IRQ 3 is used by COMM 2 in most computers - this

is a very bad choice for a network adapter if you

have 2 COMM ports in your PC]

Packet Driver giving a usage statement (or Invalid

PROM contents)?

Verify that you are using the correct notation in

your Packet Driver load line.  Typically the

following values are represented using the

following conventions:

IRQ	number	5

I/O	hex		0x300

mem	hex		0xCC00  (note: only 4 digits

after the "x")

NDIS:

NET START reporting an incomplete binding?

Check the [PKTDRV] section of your

PROTOCOL.INI file and verify that the BINDINGS

line matches your network card sections

heading.

ODI:

Observer is not "seeing" the packet types you are

interested in?

Check to make sure you have included the

correct FRAME line in your NET.CFG.

Observer not accepting packets on your LAN?

Are you licensed?  See the "Licensing" section in

this manual for information on turning a DEMO

version of Observer into a licensed product.  You

can tell the state of your license by clicking on

"File" -> "About Observer" - a license description is

in the dialog box.

Do you have the correct filter(s) set?  Check the

"Filter" dialog to verify the active filter set.

Note: When setting up a to/from filter you should

set the address you are interested in the left side

of the filter box, and ANY_ADDRESS in the right

section.  Then select "".  You should not put the

desired address in both sides of the filter.

Observer can not allocate DOS packet buffer.

Not enough contiguous DOS memory to allocate

a DOS packet buffer.

This error is VPACK or VPACKODI telling you that

other applications have taken all of the DOS

memory available and have not left enough

room for VPACK or VPACKODI to allocate a

buffer.

VPACK and VPACKODI were designed to take

only 3K of DOS memory while Observer is not

running and auto allocate up to 60K of DOS buffer

when you run Observer (the memory is returned

when you terminate Observer).  VPACK and

VPACKODI try to allocate 60K if they can find the

60K contiguous.  If they can not find 60K, they try

50K, 40K, 30K then 20K.  After that they give up

and you see the error message.

This can be solved in two ways:

1. Unload some of the offending programs.

Sound blasters, screen savers, clock utilities,

and unused network protocols are the usual

suspects.  And try running VPACK or

VPACKODI again.

2. Run VPACK or VPACKODI with a locked

memory buffer - explained below.

VPACK and VPACKODI have a flag that allow you

to lock a capture buffer when loading.  This

makes sure that they get their buffer no matter

what starts in Windows.  And, since the allocation

of the lower DOS memory under Windows is

dynamic and often very inefficiently laid out,

locking VPACK or VPACKODIs memory will not

usually result in any loss of functionality.  In other

words, programs often take as much RAM as is

available, and if they get less, it will not hurt them -

WFW networking is an example of this.

The flags for VPACK and VPACKODI are:

-m=y

where y is either:

6 for a 60K buffer

5 for a 50K buffer

4 for a 40K buffer

3 for a 30K buffer

2 for a 20K buffer

A sample command would look like:

VPACKODI  -m=3

See the Program Flags section on VPACK and

VPACKODI for a further explanation on why this

error occurs and what Windows is doing with the

lower DOS memory.

Can not register MLID receive monitor, or MLID

version to old.

This is indicating that your ODI driver for your

network card does not support the receive

monitor function.

Most ODI drivers developed before early 1993 do

not support receive monitor.

Check with your card manufacturer and

download or get a more recent driver.

Discoverer Troubleshooting

Cannot find or load NWIPXSPX.DLL or

NWCALLS.DLL.

This is an indication that you have not updated

your Windows PC to the latest Novell shell.

See the ODI section (1a) of the step by step

installation instructions for a detailed description of

what Discoverer requires.

Discoverer hangs with a "Polling Server" message

in the status bar.

This may be an indication that you are not using

Novells 802.3 frame type for IPX/SPX.  At this time

Novell requires you to use 802.3 as your frame to

collect statistics.

Cannot get information about a particular node or

set of nodes.

If this condition is for some, but not all nodes,

check to see what the nodes that do not respond

have in common.

Maybe they are all using the same (faulty) driver?

Try updating the ODI drivers on the stations in

question.

SAMPLE CONFIGURATION FILES:

Observer specific lines are indicated in bold.

ODI:

The following example is for a system using an

Eagle NE2000 Ethernet adapter.

AUTOEXEC.BAT:

PROMPT $P$G

C:\DOS\SMARTDRV.EXE

C:\DOS\DOSKEY

SET MOUSE=C:\MSMOUSE

C:\MSMOUSE\MOUSE

PATH=C:\;C:\DOS;C:\WINDOWS

SET TEMP=C:\TEMP

CD ODI

LSL

NE2000

IPXODI

VLM

C:\OBSERVER\VPACKODI

WIN

A sample NET.CFG for use with Token Ring might

look like:

LINK DRIVER TOKEN

INT 5

MEM CC000

FRAME TOKEN-RING

FRAME TOKEN-RING_SNAP

LINK SUPPORT

BUFFERS 15 4028

MEMPOOL 4096

NDIS:

AUTOEXEC.BAT:

NET INIT

NET START

PROMPT $P$G

C:\DOS\SMARTDRV.EXE

C:\DOS\DOSKEY

SET MOUSE=C:\MSMOUSE

C:\MSMOUSE\MOUSE

PATH=C:\;C:\DOS;C:\WINDOWS

SET TEMP=C:\TEMP

C:\OBSERVER\VPACK

WIN

SYSTEM.INI:

[Network Drivers]

transport=DISPKT10.DOS

PROTOCOL.INI:

[PKTDRV]

DRIVERNAME=PKTDRV$

BINDINGS=MS$NE2000		(yours may be

different)

INTVEC=0X61

Packet Driver:

The following example is for a system using a

SMC/WD adapter.

AUTOEXEC.BAT

PROMPT $P$G

C:\DOS\SMARTDRV.EXE

C:\DOS\DOSKEY

SET MOUSE=C:\MSMOUSE

C:\MSMOUSE\MOUSE

PATH=C:\;C:\DOS;C:\WINDOWS

SET TEMP=C:\TEMP

C:\OBSERVER\WD_SMC 0x61 5 0x300 0xCC00

C:\OBSERVER\VPACK

WIN

PACKET DRIVER INSTALLATION INFORMATION

Note:  This is an edited version of the file INSTALL.DOC

located in the Observer \DRIVERS directory.  If you do not

find your network adapter listed here, check the

INSTALL.DOC file.

Installation Instructions for the Crynwr Packet Driver

Collection

Document conventions

All numbers in this document are given in C-style

representation.  Decimal is expressed as 11, hexadecimal is

expressed as 0x0B, octal is expressed as 013.  All reference to

network hardware addresses (source, destination and

multicast) and demultiplexing information for the packet

headers assumes they are represented as they would be in a

MAC-level packet header being passed to the send_pkt()

function.

Using the packet drivers

The packet driver must be installed prior to use.  Since each

packet driver takes only a few thousand bytes, this is best

done in your AUTOEXEC.BAT.  Since the Ethernet boards

typically have jumpers on board, the packet driver must be

informed of the values of these jumpers (auto-configure is

possible, but can disturb other boards).  The first parameter is

the entry point used to communicate with the packet driver.

And again, because each board is different, the rest of the

parameters will be different.

All parameters must be specified in C-style representation.

The same number is expressed in decimal as 11,

hexadecimal is expressed as 0x0B, octal is expressed as 013.

Any numbers that the packet driver prints will be in the same

notation.

Before installing the packet driver, you must choose an entry

point (software interrupt) number in the range between 0x60

and 0x7e inclusive.  Some people have reported trouble with

dBase when using interrupts in the low 60's.  These problems

go away when they switch to an interrupt in the high 70's

(e.g. 0x7e).

Interrupt 0x67 is unavailable because it's used by the EMS

interface interrupt.  Interrupts 0x70 through 0x77 are

unavailable because the second interrupt controller uses

them for IRQ 8 through IRQ 15.  Interrupts 0x7f and 0x80 are

unavailable because at least one package, when locating a

packet driver, stops searching before 0x7f.

Running a packet driver with no specifications will give a

usage message.  The parameters for each packet driver are

documented below.

Options

-d -- Most drivers can also be used in a PROM boot

environment, see PROMBOOT.NOT for how to use -d and -n

options for that purpose.  This switch delays the adapter's

initialization until the first time the packet driver is accessed.

-n -- NetWare can use two different framing types on

Ethernet, "IEEE 802.3" and Ethernet II.  The BYU packet driver

shell requires Ethernet II.  However, the Crynwr packet drivers

can convert Ethernet II into Novell's version of IEEE 802.3 (and

back) when the -n switch is used.

-p -- A certain small level of security can be achieved by

disabling promiscuous mode with the -p switch.  Do not

mistake this for real security, however.

-i -- A switch used with client software that expects to find an

IEEE 802.3 packet driver.  Many Crynwr Ethernet packet

drivers implement both IEEE 802.3 (class 11) and Ethernet II

aka Bluebook (class 1) framing.  The packet driver

specification only allows a driver to report one class. The

default is to report Ethernet II.  Using -i switches the reported

class to IEEE 802.3.

3Com 3C501

usage: 3C501 [options] packet_int_no [hardware_irq

[io_addr]]

The 3c501 driver requires two additional parameters -- the

hardware interrupt number and the I/O address.  The

defaults are 3 and 0x300.

3Com 3C503

usage: 3C503 [options] packet_int_no [int_level(2-5)[io_addr

[cable_type]]]

The 3c503 driver requires up to three additional parameters --

the hardware interrupt number, the I/O address, and the

cable type.  The 3c503 can be attached to thick or thin

Ethernet cables, and the selection is made in software.  The

cable type is automatically determined at start-up, but may

be forced to external transceiver (AUI/Thickwire) by

specifying zero or internal transceiver

(Thinwire/10BaseT/10Base2) or one for thin. The defaults are 2,

0x300, and 65535 (automatic).  The 3c503 can use shared

memory, but the driver automatically determines that

parameter from the hardware.

3Com 3c505

usage: 3c505 [options] packet_int_no [hardware_irq [io_addr

[base_addr]]]

The 3c505 driver requires three additional parameters -- the

hardware interrupt number, the I/O address, and the

memory base address.  The defaults are 2 and 0x300 and

0xd000.

3Com 3c507

usage: 3c507 [options] packet_int_no io_addr

The 3c507 will determine its parameters by reading the

board.  The only time you would need to specify the

parameters is when you have multiple 3c507s in the same

machine.

The 3c507 driver will use three additional parameters -- the

hardware interrupt number, the I/O address, and the

memory base address.

3Com 3c509

usage: 3c509 [options] packet_int_no  (id_port] | [io_port] |

[board_num]

The 3c509 will determine its parameters by reading the

board.  The only time you would need to specify the

parameters is when you have multiple 3c509s in the same

machine, or if you have an I/O conflict with the default

id_port (0x110).

The 3c509 driver will use three additional parameters -- the id

port, or the I/O port, or the board number.  If the number is

between 0 and 0xff, it is the board number.  If between

0x100 and 0x1ff, it is an ID port.  Otherwise it is an I/O port

number.

3Com 3c523

usage: 3c523 [options] packet_int_no [hardware_irq [io_addr

[base_addr]]]

The 3c523 driver requires no additional parameters.  It gets

the board's parameters out of the Microchannel POS

registers.

HP Ethertwist

usage: hppclan [options] packet_int_no [hardware_irq

[io_addr]]

The hppclan driver requires two additional parameters -the

hardware interrupt number and the I/O address.  The

defaults are 3 and 0x300.

Intel EtherExpress

usage: exp16 [options] <packet_int_no> [<io_addr>]

The Intel EtherExpress packet driver has one optional

parameter.  The <io_addr> is only needed if there is more

than one EtherExpress card in your system.  Otherwise, the

driver will search for adapter and get its parameters from it.

Novell ne/2

usage: ne2 [options] <packet_int_no>

The ne/2 driver requires no additional parameters.

Novell ne1000

usage: ne1000 [options] packet_int_no [hardware_irq

[io_addr]]

The ne1000 driver requires two additional parameters -

the hardware interrupt number and the I/O address.  The

defaults are 3 and 0x300.

Novell ne2000

usage: ne2000 [options] packet_int_no [hardware_irq

[io_addr]]

The ne2000 driver requires two additional parameters -the

hardware interrupt number and the I/O address.  The

defaults are 2 and 0x300.

Novell ne2100 and ne1500

usage: ne2100 [options] packet_int_no [hardware_irq

[io_addr [dma_no]]]

The ne2100 Ethernet card is software compatible with the

ne1500 card. The ne2100 driver requires three additional

parameters -- the hardware interrupt number, the I/O

address, and the DMA channel number.  The defaults are 3,

0x300, and 5.

SMC (formerly Western Digital) (also IBM) SMCWD

usage: smc_wd [options] packet_int_no [-o] [int_level

[io_addr [mem_base]]]

The SMC_WD driver runs the SMC (formerly Western Digital)E,

EBT, EB, ET/A, and E/A Ethernet cards (but not the Ultra), and

also on the IBM Microchannel Ethernet cards with POS ID's

0xEFE5, 0xEFD4 and 0xEFD5. The ISA SMC_WD requires three

additional parameters -- the hardware interrupt number, the

I/O address, and the memory base address. The ISA defaults

are 3 and 0x280 and 0xd000.  The MCA SMC_WD picks up its

default parameters from the POS registers, so you only need

specify them it you have multiple adapters. The smc_wd

cards do not enable their memory until configuration time.

Some 386 memory mappers will map memory into the area

that the card intends to use. You should be able to configure

your software to leave this area of memory alone.  Also the

driver will refuse to map memory into occupied memory.

The occupied memory test fails on some machines, so the

optional switch -o allows you to disable the check for

occupied memory.

If you get the error "PROM ADDRESS Invalid", use EZSETUP to

set all the parameters again (to the same values).

Occasionally wayward programs will write to locations that

don't belong to them.  This can corrupt the EEPROM

checksum on the card. EZSETUP will restore the correct

checksum.

INDEX

A

Acknowledgment number, 154

address

format of entry, 44

remove, 45

alias, 8, 41, 44, 48, 49, 54, 56, 176

Aliases, 53

Alternate Columns Chart

sample, 97

Analyst/Probe, 13

ANY_ADDRESS, 41, 46, 55, 57, 180

ARP, 147

AUTOEXEC.BAT

adding VPACK, 36

adding VPACKODI, 35

Averaging time, 70

B

bandwidth utilization, 9, 39, 50, 70

long term, 9, 50

Bridge

Driver Configuration (server),

118

Statistics (server), 102

Statistics List (server), 115

buffer

DOS memory, 73

button bar, 78, 92

C

captured packets

printing, 65

saving to a file, 66

capturing

stop from when switching, 69

charting

2D sample, 95

3D sample, 97

checksum, 141

Colors

dialog, 97

specify frame color, 62

Configuration

sample files, 183

confirmation

clearing packet buffer, 70

copyright, 15

Current Filter, 42

D

default

direcetory, 21

DEMO

install dialog, 34

mode, installation, 27

to licence, 19

version, 19

DEMOETH.LST, 176

DEMOTOK.LST, 176

direction

button, 53

DISCOVER.EXE, 176

DISCOVER.HLP, 176

DISCOVER.NIC, 176

DISCOVER.NOD, 176

DISCOVER.SRV, 176

Discoverer, 8, 10, 77

Discovery Mode, 88

File menu, 81

install dialog, 34

loggin in as supervisor, 91

Options menu, 83

polling, 79

preferences dialog, 84

required files and drivers, 29

discovery mode, 8, 39, 44, 48, 56

DOS

full screen in Windows, 27

not enough memory, 24

packet buffer, 180

TSR, 22

DRIVERS

packet driver, 31

DSAP, 145

E

Ethernet

choosing maximum bandwidth,

51

frame header, 142

protocol sub-filters, 47

sample NET.CFG for ODI, 28, 184

F

FAX number

Network Instruments, 20

Filter

address, 42

post, 63

saving, 43

setting a filter, 46

using, 41

flags

-DEMO, 72

-i, 71

-m, 73

-TOKEN, 72

Fragments, 151

FRAME

referenced in NET.CFG, 28, 43,

179, 184

H

hardware address

manually adding, 45

hardware addresses

adding, 44

I

i/o, 32

ICMP, 157

information window

NODE, 120

SERVER, 99

SERVER and NODE, 11

Information Windows

general, 95

Info:, 118

Lists, 106

Options, 105

INSTALL.DAT, 176

INSTALL.DOC

packet driver, 185

INSTALL.EXE, 21, 33, 176

installation

step by step, 24

Internet addresses, 146

interrupts

shared, 177

IP, 149

automatically fill in address, 49

protocol sub-filter, 47

IPX, 160

statistics chart (node), 121

statistics chart (server), 100

statistics List (node), 123

statistics List (server), 106

IPX/SPX

overview, 159

protocol sub-filter, 47

IPX/SPX Versions

node, 135

server, 118

irq, 32

K

key

number, 19

L

LANs

multiple, 15

license, 15

Licensing, 19

Line chart

sample, 96

List of Nodes, 80, 94

display options, 93

loading, 93

saving, 93

Login to Server, 90

LSL.COM, 26

LTBU, 51

number of data points, 52

M

Maximum Utilization, 51

MDI window, 77

main, 80

MLID

can not register, 182

Monitor Modes, 50

MSS, 156

multitasking, 40

N

NCP, 165

NDIS, 26

choosing, 35

install overview, 22

sample configuration, 184

troubleshooting, 179

using, 30

NET.CFG, 23, 24, 28, 29, 179

Ethernet sample, 28

Network Bandwidth, 70

network statistics, 9, 39, 53

NICHART.DLL, 176

Nodes

List of, 11

Novell

ODI, 25

Number Connections

server, 99

NWCALLS, 182

NWIPXSPX, 182

O

OBS.DLL, 176

OBSERV.DLL, 175

Observer

running, 23

using, 39

OBSERVER.ADR, 176

OBSERVER.EXE, 175

OBSERVER.FLT, 44, 49, 176

OBSERVER.HLP, 176

OBSERVER.INI, 176

ODI, 22

choosing, 35

configuring, 27

sample configuration, 183

troubleshooting, 179

Open Files

chart (server), 100

list (server), 118

OS Version Information

node, 135

P

packet

delta time stamp, 64

find, 65

serialization, 172

WatchDog, 173

packet capture, 8, 59

overview, 39

Packet Driver, 23, 26

configuring, 30

Installation Information, 185

sample configuration, 185

troubleshooting, 178

w/ Token Ring, 26

packet_int_no, 31

packets

capturing, 60

displaying raw, 62

dropped, 40, 60

viewing captured, 60, 61

Pie chart

sample, 96

polling

cursor, 86

starting/stopping, 94

polling period, 87

post-filter

IP header, 64

TCP header, 64

print captured packets

options, 65

Program Flags, 71

protocol

no fitlering, 47

sub-filter, 47

R

Received Packets

network statistics screen, 54

Right To Use, 20

license, 16

RIP, 171

S

SAP, 169

Scale, 98

search, 64

Separate Columns Chart

sample, 97

Sequence number, 154

Server exclusion List, 89

Server Version, 118

Settings

dialog, 68

Shell Driver

configuration (node), 136

list (node), 130

Shell Statistics List

node, 132

Sort, 55

SPX, 162

statistics (server), 101

statistics chart (node), 121

statistics List (node), 126

statistics list (server), 109

SPX II, 160

SSAP, 145

statistics

by protocol, 58

general screen information, 53

printing screens, 59

size distrabution, 56

Stretch time scale, 70

SUPERVISOR, 86

System Requirements, 16

T

TCP

protocol overview, 152

TCP/IP

datagrams, 140

frame header, 141

main components of, 139

overview, 139

Timing, 98

tips, 85

Token Ring, 29

choosing maximum bandwidth,

51

Frame Header, 144

sub-filter, 47

Total Node Packets chart

node, 122

Total Packet Statistics

server, 101, 113

Troubleshooting

checklist, 178

Discoverer, 182

U

UDP, 156

V

View Configuration dialog, 62

VLM

support, 22

Volume

Information List (server), 117

Usage (server), 105

User Information (node), 137

VPACK, 23, 175

troubleshooting, 178

VPACKODI, 22, 175

troubleshooting, 178

W

WAN, 10

Warranty, 14

Window Menu, 92

Windows

buffer size, 68

lock buffer from paging, 69

network support, 23

Windows for Workgroups

w/ Novell, 25

(c) 1994 Network Instruments, LLC