The Windows 95 Driver (DC690.PDR) only supports one DC-690x in a system.

Other DC-6x0 cards could be used as a secondary controller.

<< Windows 95 Driver Installation >>

1. Boot Windows 95 and click on "Start"

2. Select "RUN"

3. Type "A:\win95\install.exe"

4. Follow the instructions step by step to install the driver

5. Click on "Next"

6. Select "No" and Click on "Next"

7. Select "Hard disk controller"

8. Click on "Next"

9. Click on "Have disk"

10. Type "a:\win95"

11. Select "Tekram PCI IDE Cache Controller (DC690-CD)"

12. Click on "Next"

13. Click on "Next"

14. Click on "Finish"

15. The message  "To finish setting up your new device driver you must

restart your computer. do you want to restart your computer now"

will be on the screen. Click on "YES" to restart the computer.

[Note1]

After Windows 95 reboots, it will detect the DC-690x as a new Primary PCI

IDE controller and require you to re-boot. Select "NO" to proceed. The

system will go on detecting the Seconary IDE controller and ask

if you want to re-boot again.

Press "YES".

<< Windows 95 Driver Uninstallation >>

1. Click on "Start"

2. Select "RUN"

3. Type "A:\win95\uninstal.exe"

4. The message  "To finish un-installation you must restart your computer,

do you want to restart your computer now" will be on the screen.

Click on "YES" to restart the computer.