Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

With Multi-Satellite Tracking

(Enhanced Version for 386/486/586 Computers)

(C) Copyright David H. Ransom, Jr., 1989-1995

All rights reserved.

Version  9537

September, 1995

David H. Ransom, Jr.

7130 Avenida Altisima

Rancho Palos Verdes, California, 90275  USA

Bulletin Board System

---------------------

RPV ASTRONOMY BBS

(310) 541-7299 @ 2400-14400 Baud

Program STSORBIT PLUS Satellite Orbit Simulation                  Page ii

TABLE OF CONTENTS

INTRODUCTION ......................................................1

HARDWARE AND SOFTWARE REQUIREMENTS ................................6

PROGRAM DESCRIPTION ...............................................8

STSORBIT PLUS FILES ...............................................10

STSPLUS MAP PROJECTIONS AND DATABASES .............................13

PROGRAM SETUP AND USAGE NOTES .....................................15

DOS Memory Managers and Performance Tips ........................15

DOS CONFIG.SYS Setup ............................................16

Copying Files for STSORBIT PLUS .................................16

Slow Computers and 80x87 Math Coprocessor Chips .................17

Starting Program STSORBIT PLUS and Command Line Options .........20

Predicting Visible Satellite Passes .............................22

Predicting Satellite Passes with STSPLUS ......................23

Predicting Satellite Passes with TRAKSTAR .....................25

Printing Graphics Screens .......................................28

Known STSPLUS Problems and Bugs .................................28

Satellite Name Cross-Reference using STSPLUS.XRF ..................31

Preparing 2-Line Elements using VEC2TLE by Ken Ernandes ...........32

PROGRAM OPERATION .................................................34

Satellite Icon Selections .......................................34

Map Database File Selections ....................................35

STSORBIT PLUS Title Screen ......................................35

STSORBIT PLUS SATELLITE TRACKING FEATURES .........................37

Orthographic Projection Maps ....................................37

Rectangular Projection World Maps ...............................38

Rectangular Projection Quadrant Maps ............................38

Rectangular Projection Zoom Maps ................................39

Location Maps with Isocontours ..................................40

Tracking Station Maps with Isocontours ..........................41

Location and Features Labels ....................................42

Big Clock Options ...............................................43

Satellite Motion Maps ...........................................44

Satellite Position and Orbit Projections ........................44

Satellite Visibility ............................................45

User's Circle of Visibility .....................................46

Spacecraft Circle of Visibility .................................47

SUN and Solar Features ..........................................47

TDRS and Real Time Satellite Features ...........................49

Ground Tracking Stations and .TRK Files..........................51

Event Timers and Audible Alarms .................................53

Pausing the Ground Track Display (F6 Key) .......................56

Switching between MET and T+Epoch ...............................57

Using FAST Time (F4 Key) ........................................57

On-line Help (F1 Key) ...........................................58

Launch Simulation for the Space Shuttle ...........................59

Launch Simulation Setup .........................................62

Satellite Communications and Amateur Radio ........................64

STSPLUS Doppler Shift Mode ......................................65

Satellite Phase (Mean Anomaly) ..................................67

Program STSORBIT PLUS Satellite Orbit Simulation                  Page ii

Satellite Communications Technique and Cautions .................68

Preparing File STSPLUS.FRQ for Amateur Radio Use ................70

ACTIVE KEYS DURING GROUND TRACK DISPLAY ...........................74

STSORBIT PLUS MAIN MENU ...........................................78

F1  Convert Keplerian Data to 2-Line Format .....................79

Example Data Input and Output ...............................81

Received Keplerian Orbital Data Form ........................83

F2  Read/Update NASA/NORAD 2-Line Elements ......................84

Select/Update Preset Frequency Selections ...................84

Update Current TDRS and Real Time Satellites ................85

Read NASA/NORAD 2-Line Elements from a File .................86

F3  Data Output and Pass Prediction Selections ..................90

Setting up Position and State Vector Data Output ............92

Setting up Tabular Pass Predictions .........................95

Data Mode 1: Azimuth/Elevation Data Format ..................103

Data Mode 2: Latitude/Longitude Data Format .................104

Data Mode 3: Topocentric RA/DEC Data Format .................105

Data Mode 4: Ascending Node X-Y-Z State Vector ..............106

Data Mode 5: X-Y-Z Cartesian State Vector, 2 Data Lines .....108

Data Mode 6: X-Y-Z Cartesian State Vector, Comma Delim ......110

Data Mode 7: X-Y-Z Cartesian State Vector, Labeled Data .....112

Data Mode 8: Doppler Shift Predictions ......................114

Data Mode 9: Pass Predictions ...............................115

F4  Calculate Satellite Positions with TRAKSTAR .................117

F5  Set Launch Time and Date ....................................117

Using File STSPLUS.LTD for Launch Date & Time ...............119

F6  Set/Read/Save TDRS and Real Time Satellites .................120

F1  Display/Modify Satellite Assignments ....................121

F2  Save SCF Satellite Configuration File ...................124

F3  Read SCF Satellite Configuration File ...................125

F4  Select New PRIMARY Satellite ............................126

F5  Select New TARGET Satellite .............................127

F6  Clear Static and Real Time Satellites ...................128

F7  Set FILENAMES and PATHS .....................................128

F8  Set Program TIME and DATE....................................129

F1  Restore System Date and Time ............................131

F2  Set DOS System Clock ....................................131

F3  Set Simulated Date and Time using Calendar Method .......132

F4  Set Simulated Date and Time using MET ...................133

F9  Display Current RIGHTIME Corrections ....................133

F10 Set UTC OFFSET, DAYLIGHT Flag, and DOS Time Zone.........134

F9  DOS Shell ...................................................135

F10 Set STSORBIT PLUS Program Options and Features ..............136

ENTER  Resume Mission ...........................................136

ESC    Quit STSORBIT PLUS and Save Current Mission ..............136

PROGRAM OPTIONS AND FEATURES MENU .................................137

F1  Program STSORBIT PLUS Information ...........................137

F2  Set New Local Coordinates ...................................137

F1/F2 Search CITYFILE for location ..........................138

F3/F4 Enter New Coordinates for Location ....................139

F5  Clear (disable) Secondary Location ......................140

F6  Select TRACKING STN for PRIMARY Location ................140

F3  Set Display Features (see separate section below) ...........141

F4  Set Satellite Coordinates ...................................142

F5  Show Ascending & Descending Node Data .......................142

Program STSORBIT PLUS Satellite Orbit Simulation                 Page iii

F6  Set Map Projection and Size .................................143

F7  Enable/Disable EVENT TIMERS .................................143

F8  Enable/Disable Audible ALARMS ...............................143

F9  Set User-Definable Map Colors ...............................144

F10 Enable/Disable Printer Logging ..............................145

SET DISPLAY FEATURES ..............................................146

F1  Select Satellite Icon Size: 15/21 pixels ....................146

F2  Display TDRS and Real Time Satellites .......................147

F3  Display Additional Map Grid Lines ...........................147

F4  Display Tracking Stations ...................................147

F5  Display Ground Track: DOTS/LINE .............................147

F6  Display Local/Satellite Circles of Visibility ...............148

F7  Display South Atlantic Anomaly Zone .........................149

F8  Display Terminator, Sun and Spacecraft Lighting .............149

F9  Display Map Locations and Features ..........................149

F10 Display Lakes and Rivers ....................................149

STSORBIT PLUS's Orbital Model .....................................151

Accurate Time and the Personal Computer ...........................153

Methods for Setting DOS Time ....................................154

Maintaining Accurate DOS Time ...................................155

Programs TIMESET and RIGHTIME ...................................157

Program PRECISION TIME ..........................................161

Computer Bulletin Board Systems and the Internet ..................163

Bibliography of Astronomy and Space Subjects ......................166

STSORBIT PLUS Revision History ....................................168

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 1

INTRODUCTION

------------

Program STSORBIT PLUS is an enhanced version of STSORBIT, my original

orbital tracking and display program. As a general rule, a 386 or better

(IBM compatible) computer is required. A math coprocessor chip is STRONGLY

RECOMMENDED and will dramatically improve performance; the math coprocessor

chip is REQUIRED for acceptable performance when using the orthographic

projection. (Some users have reported acceptable performance using a

286/287 system but some operations are quite slow. Other users report

"usable" performance on an XT-class machine with a math coprocessor

(8088/8087) but the performance is marginal at best.) See the section

HARDWARE REQUIREMENTS for additional information and discussion. The

program is intended for use during Space Shuttle missions and for general

satellite tracking using NASA/NORAD 2-Line Orbital Elements. One primary

satellite and up to 32 additional "static" or "real time" satellites may be

simultaneously tracked in real time on most computers. Both orthographic

and rectangular map projections are available, displaying the Earth as a

globe or the more traditional "flat map". Tabular line-of-sight satellite

pass predictions may be made from within STSORBIT PLUS and passes of

interest may be easily displayed.

STSORBIT PLUS is very accurate when used with current (and accurate)

2-line elements. The program has been qualified by the USAF and meets the

requirement of placing a satellite within 0.5 km over a 24-hour time span

from the epoch time of the 2-line elements. It is used daily at the USAF

Central Computer Complex at Cape Canaveral, by the DOD C-Band Radar Network

at their worldwide radar sites, and at half a dozen NASA and aerosapce

control centers around the country. The program is made available to schools

through the NASA Spacelink BBS and the NASA Teacher Resource Centers.

Numerous schools and amateur radio enthusiasts have made contact with the

space shuttle via SAREX amateur radio while using STSORBIT PLUS.

STSORBIT PLUS is copyrighted software; you are hereby granted a non-

exclusive license for non-commercial or educational use only. Agencies of

the U.S. Government are also hereby granted a non-exclusive license for

internal use. Use STSORBIT PLUS if you like it, discard it if you don't.

There are no warranties of any kind. If you wish to use STSORBIT PLUS

commercially, write for license information. The only request I make of

users is that they take the time to complete and return the confidential

questionnaire in file README. The questionnaire gives users a chance to

offer comments and suggestions, and lets me know that people use and

appreciate STSORBIT PLUS. Registration of STSORBIT PLUS is inexpensive and

optional -- but will be appreciated and will encourage me to continue

supporting and enhancing the program. Additional higher detail map database

files are available to registered users by mail.

Program STSORBIT PLUS (which I will usually refer to as STSPLUS from

here on) is intended to display the position and ground track of an

orbiting satellite on a selection of maps ranging from a full map of the

world to zoom maps showing considerable detail. The program has special

features implemented at the request of NASA astronauts and others for use

during a NASA Space Shuttle mission. With the appropriate 2-line elements,

STSPLUS displays the position and ground track of a variety of satellites

such as the Space Shuttle, the Hubble Space Telescope, the Gamma Ray

Observatory, or the Russian MIR Space Station. Accurate TDRS coverage,

including times for acquisition and loss of signal, is calculated for

satellites which use that satellite network for communications. Special

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 2

Location and Tracking Station displays show concentric isocontours, circles

of equal satellite altitude; these special maps can be especially valuable

for visual or amateur radio sightings.

**********************

*  IMPORTANT NOTICE  *

**********************

After almost four years, I have discontinued support for the

simple orbital model first used in my original STSORBIT program.

The accuracy of that model is marginal at best and timely 2-line

orbital elements for space shuttle missions and other satellites

are now widely available. If you wish to use the simple orbital

model, use STSORBIT or a version of STSORBIT PLUS prior to 9240.

Users who wish to convert space shuttle or satellite state

vectors to 2-line format should see the section "Preparing 2-Line

Elements using VEC2TLE" for a description of a program by Kenneth

Ernandes written specifically for this purpose. VEC2TLE has been

validated during several space shuttle missions and is highly

recommended.

The initial premise for STSORBIT was to attempt to duplicate the wall

map in NASA's Mission Control Center in Houston, Texas. Before I started

this project, I had seen several other programs which tracked satellites

but each fell short of my map and display objectives for one reason or

another. I therefore set out to do the job myself. STSORBIT and now

STSORBIT PLUS have been the result. Since then other programs have appeared

which produce similar information, most notably Paul Traufler's excellent

TRAKSAT (which was inspired by STSORBIT). It may be, of course, that others

will judge this effort lacking for some tasks, but no one program can do

everything. One problem is that of screen size and resolution: the wall map

at Mission Control Center is some twenty feet wide with an impressive pixel

resolution, very different from the typical personal computer monitor. The

NASA wall map shows essentially the entire globe in a cylindrical

projection; STSORBIT also used a cylindrical projection and restricted the

vertical display to latitudes from +85 degrees to -85 degrees in order to

achieve reasonable proportions and vertical resolution while at the same

time showing recognizable land features. STSORBIT PLUS now presents the

Earth as a globe using an orthographic projection with zoom while still

retaining the original cylindrical (rectangular) projection. STSPLUS adds

many additional features and improved accuracy over the original STSORBIT.

Initially, and as a consequence of a lack of accurate orbital data for

Space Shuttle missions while they were in progress, I did not try to be

especially precise with respect to the orbital mathematics. Additionally,

mathematical complexity had to be held to a reasonable minimum if older

computers not equipped with a math coprocessor were to be able to maintain

the presentation in real time. My somewhat casual attitude toward

mathematical precision changed with the launch of the Hubble Space

Telescope (HST) and the regular availability of US Space Command 2-Line

Elements via modem from TS Kelso's Celestial BBS. Until HST, I had been

content to manually adjust the orbital data occasionally during the course

of a typical five day mission and live with the errors inherent in my

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 3

original simple orbital model. The accuracy of that model degrades rapidly

after five or ten orbits and, although it could be adjusted from time to

time during a mission, more accurate data are now readily available prior

to a launch and during a mission. The NASA SpaceLink BBS in Huntsville,

Alabama began posting 2-line orbital elements for the Space Shuttle in

early 1991 due in part to my persistent and continuing requests; Bill

Anderson, Jeff Ehmen, and Flint Wild, sysops of the SpaceLink BBS, are

continually upgrading the services available.

Beginning in mid-1990, therefore, STSORBIT was extensively modified to

read orbital data from these USSPACECOM 2-line elements and thereby

maintain significantly improved accuracy over long periods of time. As an

incidental benefit, the ground tracks of other satellites (such as the

Russian space station MIR) could also be displayed. At present, the orbital

model (SGP4) used with 2-line elements is accurate only for low Earth

orbits. Deep space orbits, defined as orbits having an orbital period

greater than or equal to 225 minutes, require a more complex orbital model

(SDP4) which takes into account solar and lunar perturbations for best

accuracy. STSPLUS calculates data and displays a ground track for deep

space objects but the accuracy of these data has not been validated; it is

believed to be "reasonably" accurate. I plan to add the SDP4 orbital model

to STSPLUS in a future release when time permits.

At about the same time, STSORBIT also found its way to the NASA

Johnson Space Center in Houston, Texas. Quite a few individuals from JSC

sent me comments and suggestions for further improving the program, among

them Ron Parise of the STS-35/ASTRO-1 crew. Ron suggested that I make

modifications to allow the display of Mission Elapsed Time (MET) for

shuttle missions while using the USSPACECOM 2-line elements. This would allow

both the higher accuracy of the USSPACECOM 2-line orbital data and permit

following the mission timeline using MET. Since launch time and date are

not included in the 2-line elements but are required to compute MET, these

data must be entered independently. Another suggestion from Ron and others

was to include the Sun, solar terminator (calculated at Mean Sea Level),

and spacecraft lighting conditions to determine if the spacecraft is

visible.

Not satisfied with the somewhat rough map used with STSORBIT (a

digitized EGA world map), I upgraded the maps to use a modified version of

the World Data Base II. This had the desired effect, to the point where

rivers and other landmarks could easily be recognized on the monitor and on

downlinked orbiter television. As a side effect, however, the processor

overhead increased dramatically -- some slower computers not equipped with

a math coprocessor were unable to keep up. I therefore essentially "froze"

the original STSORBIT program (except for minor updates) and created this

new program, STSORBIT PLUS, intended for the faster, more capable

processors. Since mid-1991, STSPLUS has also spread throughout the various

NASA Centers and around the world.

In addition to NASA and individual users all over the world, STSPLUS

and STSORBIT are also being used in an educational setting. As many as 1100

high schools participated in the Inspire Project, a VLF propagation test

flown on STS-45 and for which STSPLUS was one of the recommended tools. At

a middle school in Kansas, the program is projected in the school

auditorium from time to time during a mission to show the children

graphically what is happening and to give them a sense of "real time"

participation in our space program. At an Air Force training facility,

STSORBIT is one of many tools used to prepare Air Force officers for their

duties in the Air Force Space Command. The program was widely distributed

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 4

at a recent National Association of Science Teachers convention and by

radio amateurs at regional "ham fests". It is also available to educators

through the NASA Teacher Resource Centers and the NASA Spacelink BBS.

In perhaps its most prestigious installtion, STSPLUS is the software

used by the NASA/JPL Multimission Computer Control Center in Pasadena,

California, to display the ground track of Earth-orbiting satellites. The

Canadian Space Agency used STSPLUS as part of their briefing government

officials during the STS-52 mission in October, 1992. Intelsat used STSPLUS

operationally in May of 1992 at their Launch Control Center in Washington,

DC, and at five tracking stations around the world during the exciting STS-

49 mission, the maiden flight of Endeavour and the rescue/reboost of the

INTELSAT-VI satellite. Intelsat was kind enough to send me a letter saying

that STSORBIT PLUS was "critical to mission success"! Numerous other

official and semi-official installations use STSPLUS as the primary

satellite tracking software or to supplement other software.

A brief biographical note: I am a retired physicist and engineer who

spent all of his professional life in the world of electronics, data

communications and, more recently, computers. As a young man I was actively

involved in the early American space program including projects such as

Ranger, Mariner, Mercury, Gemini, and Apollo. Exciting times indeed! I

spent considerable time at the Jet Propulsion Laboragory in the early

1960's as a contractor on Ranger and Mariner; my respect and admiration for

JPL and its people has, if possible, increased over the intervening

decades. My interest in space has continued to this day. The desire to

"keep in touch" with our Space Shuttle missions was one of the incentives

in the development of this software. I continue to be astonished that a

relatively inexpensive personal computer is sufficient to perform

calculations that pushed the limits of our best mainframe computers only a

decade or so ago. If STSORBIT PLUS also serves to help spark the interest

of young people in science and technology or can be a learning tool at any

level, I will have more than achieved my goals.

No discussion of satellite tracking would be complete without thanks

to Major T. S. Kelso, USAF, who almost single handedly brought satellite

tracking within the reach of "ordinary folks". TS's Celestial BBS has been

providing unclassified 2-line orbital elements direct from US Space Command

(formerly NORAD, the North American Air Defense Command) at Cheyenne

Mountain, Colorado, since 1986 or so. For many years, Celestial BBS was the

only electronic source for orbital elements in the world. The Celestial BBS

may be reached at (513) 427-0674 and is located near Dayton, Ohio. TS has

also written a variety of satellite tracking software and his most recent

program, TRAKSTAR, may be used directly from within STSPLUS to generate

tabular data on upcoming satellite passes.

Special thanks to Paul Traufler for his friendship and encouragement.

Our regular telephone conversations have generated many a new idea and the

synergism has been beneficial to us both. Our two programs, STSORBIT and

TRAKSAT, have engaged us in a friendly rivalry which has, I think, improved

both programs many fold. I may have provided the initial spur to Paul to

write TRAKSAT (in order to improve on my "sloppy orbital math", as Paul

described it) but TRAKSAT has in turn kept my nose to the grindstone and is

recognized by many as the standard against which other satellite tracking

programs are judged. The emphasis of the two programs is slightly

different, with STSORBIT concentrating on the graphical display and TRAKSAT

on high precision analytical and predictive techniques. I highly recommend

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 5

TRAKSAT for the serious satellite tracker. My thanks as well for Paul's

help in upgrading STSORBIT to use the USSPACECOM 2-Line Elements and other

technical assistance.

Thanks also to Rob Matson and Joel Runes. Rob for offering comments

and code to help me implement several of STSPLUS's more exotic features;

Rob coined the phrase "isocontours" to describe the circle of equal

satellite altitude around a location and his fine SKYMAP program generates

high accuracy printed star maps with or without satellite tracks. And Joel

for keeping us all up to date with current elements for space shuttle

missions and his patience in testing innumerable beta versions of STSPLUS,

thereby helping to track down some of the more subtle bugs.

Finally, my thanks to all those individuals who have taken the time to

write or leave a message on my BBS with comments and suggestions. While I

haven't implemented every suggestion, many are now included and the

feedback is most welcome.

For individuals interested in our space program and who have access

to a modem, I recommend NASA's SpaceLink Bulletin Board System in

Huntsville, Alabama, (205) 895-0028, available twenty four hours per day,

300 to 2400 baud. NASA SpaceLink, located at the NASA Marshall Space Flight

Center and with 8 lines, provides a wealth of information on NASA and its

projects. 2-line orbital elements for a Space Shuttle mission are usually

available while the mission is in progress. In addition to educational

materials and software (including my programs STSORBIT PLUS, STSORBIT and

JPLCLOCK), general information on NASA programs and plans, news releases,

and images from prior spacecraft missions such as Voyager, SpaceLink also

devotes a complete section to current news and information on the Space

Shuttle. I particularly appreciate the STS Mission Press Kit, posted about

two weeks before each mission, which provides a great deal of information

on the upcoming mission, payload and crew as well as broadcast schedules on

NASA Select Television, Satellite F2-R, Transponder 13. Mission status

reports are generated daily during the course of a mission. I regularly

call SpaceLink and post files of interest on my own RPV ASTRONOMY BBS.

For current Space Shuttle orbital information (if a mission is in

progress), 2-line elements for more than 1,000 satellites, and the most

recent versions of STSORBIT PLUS, STSORBIT, TRAKSTAR, TRAKSAT, and SKYMAP,

call RPV ASTRONOMY BBS (see title page for telephone number and data rates

available). The main system has well over 1,500 more or less regular users

and is often busy, so please be patient. If you wish to receive STSORBIT

PLUS (or any of my other programs) on disk, see file README for

information.

David H. Ransom, Jr.

7130 Avenida Altisima

Rancho Palos Verdes, CA 90275

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 6

HARDWARE AND SOFTWARE REQUIREMENTS

----------------------------------

A 386SX-class computer running at 20MHz and equipped with a 387 math

coprocessor chip is the minimum system used for all program testing and

development. While other systems may give acceptable performance, this

minimum configuration assures that most features will execute as described

and in real time. Performance with 386DX/387 and 486DX or higher systems

will be considerably superior to 386SX systems. Note that NO TESTING is

performed on systems not equipped with a math coprocessor chip. The

following minimum hardware is recommended:

386 or higher IBM-compatible computer

387/487 math coprocessor chip (if not included in main processor)

VGA color display

Hard disk with up to 3MB available

RAM disk with at least 500K space

The 387 (and 487 for 486SX processors) math coprocessor chip is

STRONGLY RECOMMENDED and is REQUIRED for acceptable performance. The

calculations relating to orbital mechanics are very complex and STSPLUS

will use the coprocessor if one is equipped; performance is improved by

about an order of magnitude. Other "fast" processor and coprocessor

combinations may yield acceptable performance. Math coprocessor chips are

now reasonably inexpensive and the performance improvement is impressive

and well worth the modest cost. As an example, my vintage Zenith laptop,

equipped with an 80C88 processor and an 8087 math coprocessor, is just able

to keep up in real time (rectangular modes ONLY!) when running at a clock

speed of 8 MHz but the map drawing times are very slow. However, an 8 MHz

286 computer without a math coprocessor does NOT provide reasonable

performance; map drawing times are painfully slow.

STSPLUS is intended to be used with an EGA or VGA video adapter and a

color monitor; with these systems, the display is in color. Because of its

improved vertical resolution, the VGA is recommended over the EGA. A

monochrome display with shades of gray may also be used with the program

(with the "/M" command line option). Because of hardware limitations, CGA

and HGC systems can only present graphics in monochrome; although those

display adapters are supported to some degree in current versions of

STSPLUS, that support may NOT continue in future versions. The original

STSORBIT will continue to support CGA and HGC monitors.

A hard disk is required for performance reasons and for storage of the

program, map databases and orbital elements files. A RAM disk with

sufficient space to hold the various data files is also recommended for

improved performace and to reduce wear and tear on the hard disk during

periods of extended use.

STSPLUS is a large program and requires significant memory. As of

Version 9537, the program requires approximately 414KB of RAM. Systems

which have less RAM available may fail to execute the program with a

"Not enough memory" error.

Although the program may execute properly on other software operating

systems, STSPLUS has been designed and tested using standard configurations

of Microsoft DOS 3.3, 5.0, and 6.x. No optional Terminate and Stay Resident

programs (TSR's) or "shell" programs have been tested except for standard

DOS programs supplied by Microsoft (like MOUSE and DOSKEY) and Tom Becker's

RIGHTIME. Third party memory management programs and Digital Research DRDOS

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 7

may experience problems although some users report that the latest release

of DRDOS 6.0 works correctly.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 8

PROGRAM DESCRIPTION

-------------------

A typical Space Shuttle orbit is nearly, but almost never exactly,

circular with an altitude of approximately 160 nautical miles to a maximum

of approximately 300 nautical miles and an inclination of about 28 degrees

through about 57 degrees. Occasional missions, especially military

missions, fly at higher altitudes and/or inclinations and often use more

elliptical orbits. Prior to 1990, little of this information was known to

very good accuracy by the casual listener. Initially, therefore, the

interested would-be mission observer may have only the time and date of

launch and intended orbital altitude and inclination to initialize a

tracking program. Given the geographical coordinates of the Kennedy Space

Center and assuming a circular orbit, the data is sufficient to calculate

at least a rough idea of the Shuttle's position for the first several

orbits. After that, additional information was required if the position was

going to be very close. This was the method used in my original STSORBIT

program when 2-line orbital elements were not available.

Estimated 2-line elements are usually available prior to a space

shuttle launch and I usually post "adjusted" 2-line elements within two

hours of a launch. "Real" 2-line elements from NASA or US Space Command are

usually available 8 to 12 hours after launch. 2-line elements yield a more

accurate position over longer time periods (provided no orbital maneuvers

are performed). Using 2-line elements for any satellite is quite simple; no

adjustment of orbital parameters is necessary. An abbreviated version of

the 2-line element file available at the time of this release of STSPLUS is

included in the distribution files; this abbreviated file contains

approximately 150 satellites while the "full" file as posted on my BBS

typically has more than 700 satellites. The data for each satellite is

referenced to a specific date and time, the "Epoch" of the data. As a

general rule, orbital calculations will be relatively accurate for 10 to 20

days after the Epoch date; the lower the orbit, the greater the effect of

factors such as atmospheric drag and the less accurate the calculations

will be as time passes.

STSPLUS displays a portion of the Earth using either an orthographic

porjection (the Earth seen as a globe) or cylindrical projection (similar

to the Mercator projection commonly used). The maps show most of the

Earth's land boundaries and continental areas. Major oceans, seas, and

rivers are easily recognizable. Considerable detail is shown at higher zoom

factors. Automatic map generation ensures that the satellite is always

displayed. The display shows the selected satellite as a small symbol or

icon, the projected orbital ground track for the next three hours and the

the past one and a half hours, and many other features including circles of

visibility, TDRS coverage, and the solar terminator.

Data is displayed which gives the current ground track position of the

satellite, known as the "sub-satellite point", antenna or viewing angles,

spacecraft lighting, TDRS communications coverage (when applicable), and a

variety of other information. A selection of map modes and display features

allow users to configure the program to meet their requirements.

STSPLUS has been validated by the USAF and is approved for use on the

Eastern and Western Ranges. It is sufficiently accurate (given current and

accurate 2-line orbital elements, of course!) that the program is being

used operationally by NASA, USAF, and Intelsat. Among the current users

are:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 9

NASA/JPL Multimission Computer Control Center, Pasadena, CA

NASA/Lewis User Operations Facility, Cleveland, OH

Rockwell Mission Control & Support, Downey, CA

Aerojet Emergency Control Center, Sacramento, CA

USAF Central Computer Complex, Cape Canaveral, FL

DOD C-Band Radar Network, worldwide radar sites

In addition to these official or semi-official users, thousands of amateur

radio operators and "just plain folks" all over the world use STSPLUS to

track the space shuttle and other satellites.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 10

STSORBIT PLUS FILES

-------------------

STSORBIT PLUS is normally distributed via bulletin board systems in

archived form using the ZIP format by PKWare. Note that all files (except

map databases) for STSORBIT PLUS are called "STSPLUS" in order to conform

to DOS filename requirements and to avoid confusion with the similarly

named files for the original STSORBIT. The following files are usually

included:

STSPLUS.EXE        Main STSPLUS Program     (required)

STSPLUS.DOC        Documentation            (not required)

STSPLUS.FRQ        Preset Frequencies       (optional)

STSPLUS.ICO        Icon for WINDOWS 3.x     (optional)

STSPLUS.KEY        STSPLUS Active Keys      (optional)

STSPLUS.LOC        Map Locations & Features (optional)

STSPLUS.LTD        Satellite Launch Dates   (optional)

STSPLUS.SCF        Satellite Config File    (optional)

STSPLUS.TRK        NASA Tracking Stations   (optional)

STSPLUS.CTY        City Coordinates         (optional)

STSPLUS.XRF        Sat Name Cross Reference (optional)

STSPLUS.INI        Initialization data      (see below)

EARTH4.MCX         Level 4 Map Index        (required)

EARTH4.MCP         Level 4 Rect Map Data    (required)

EARTH4.XYZ         Level 4 Ortho Map Data   (required)

EARTH3.MCX         Level 3 Map Index        (optional)

EARTH3.MCP         Level 3 Rect Map Data    (optional)

EARTH3.XYZ         Level 3 Ortho Map Data   (optional)

EARTH2.MCX         Level 2 Map Index        (optional) *

EARTH2.MCP         Level 2 Rect Map Data    (optional) *

EARTH2.XYZ         Level 2 Ortho Map Data   (optional) *

EARTH1.MCX         Level 1 Map Index        (optional) *

EARTH1.MCP         Level 1 Rect Map Data    (optional) *

EARTH1.XYZ         Level 1 Ortho Map Data   (optional) *

MSHERC.COM         Hercules driver          (required for HGC)

STSICON.EXE        Satellite Icon Program   (not required) **

STSMIR.ICN         Shuttle/Mir Icon         (not required)

STSMIR.NOT         Notes for STSMIR.ICN     (not required)

TLEnnn.TXT         2-Line Elements          (required)

NASA.TRK           NASA Tracking Stations   (not required)

CIS.TRK            Russian Tracking Stations(not required)

INTELSAT.TRK       INTELSAT Tracking Stns   (not required)

SPACENTR.TRK       Other Tracking Stations  (not required)

STSLNDG.TRK        Orbiter Landing Sites    (not required)

README             STSPLUS Registration and Questionnaire

SETUP.DOC          Quick Setup Instructions

SOP9311.ICO        Alternate STSPLUS Icon   (not required)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 11

*    The EARTH3 files are available separately for download from the

RPV Astronomy BBS and are also included with registration. The

EARTH2 and EARTH1 are available by mail only to registered users

of STSORBIT PLUS for an additional donation.

**   Available by mail only and included with with registration of

STSORBIT PLUS. Provides the capability of creating and modifying

satellite icons.

Files noted as "(required)" must be in the current default directory (or in

a specified directory in some cases) for program operation. Files noted as

"(optional)" are not required when STSPLUS is operated but provide

additional features or information if present. In order to minimize the

disk space required, all.EXE files been compressed with PKWare's PKLITE

Professional; these files require a small amount of additional time to

begin execution since they are decompressed "on the fly" at load time.

***  IMPORTANT NOTE ***

File STSPLUS.INI contains initialization data from previous runs

of the program. If file STSPLUS.INI is not present it will be

created. Note that if STSPLUS.INI was written by a version prior

to 9332, all data will be ignored and the program must be

initialized as if being run for the first time.

STSPLUS can use map databases with different degrees of map detail.

Level 4, required for operation, contains the minimum detail and Level 1

contains the maximum detail. As noted in the list above, three files are

used for each level of map detail: MCX files contain an index of the map

data; MCP files contain map coordinates for rectangular projection; and

XYZ files contain map coordinates for orthographic projection. STSPLUS

checks for the levels that are present and uses the level appropriate for

the zoom factor in effect or, if that level is not present, the maximum

level that is present. Level 1 is checked first, then Level 2, etc. Level 4

files MUST be present or an error message is displayed and the program

aborts.

***  IMPORTANT NOTE ***

STSPLUS assumes that if a particular level of map database is

found, ALL lower levels of map database are present. Missing

levels of map database will cause a program error.

File STSPLUS.FRQ contains a list of preset frequencies for amateur

radio satellites and is not required except when STSPLUS is operated in the

Doppler Shift Mode. See the section "Satellite Communications and Amateur

Radio" for additional information.

File STSPLUS.KEY is a quick reference list of the keys that are

active while the map is displayed and includes a brief description of the

function of each key. It has been extracted from this documentation.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 12

File STSPLUS.LTD contains the launch date and time for selected

satellites. The file may be updated when new satellite launch date and

time data is entered via the program.

Files with filetype .TRK are tracking station locations or other

locations of interest which may be plotted on the map. These files may be

created or edited with a standard ASCII editor. If you wish to use a

different TRK file, use Function Key F7 from the Main Menu to select

the desired file.

File STSPLUS.SCF is a sample multiple satellite configuration file

which may be used to display multiple TDRS and real time satellites. Up to

16 additional satellites may be tracked using this feature.

File STSPLUS.XRF is a satellite name cross-reference file (NORAD

Number to Satellite Name) which may be used to substitute a preferred name

for that found in a TLE file. STSPLUS.XRF must be in the same directory as

the main program. See the section Satellite Cross-Reference for additional

information.

File TLEnnn.TXT (where "nnn" will be a number such as "153") is a set

of USSPACECOM 2-line elements as of the date of the file. Note that the 2-

line elements should only be used for ten to twenty days after the epoch

date for each satellite if reasonable accuracy is to be maintained. Current

orbital elements are regularly posted on my bulletin board system and

elsewhere. Other files with 2-line elements are also available; they

typically have names like GSFCnnn.TXT or N2L-nnn.TXT for general

satellites, and STSmmNnn.TXT for Space Shuttle missions. Space Shuttle

orbital elements are usually posted at least daily during missions; because

of orbital maneuvers, Space Shuttle elements more than 24 hours old may

yield inaccurate positions.

File STSPLUS.LOC contains geographic coordinates and labels for

selected locations and major oceans and seas. These labels may optionally

be displayed on the maps. The file may be edited with a standard ASCII

editor to add or delete locations and features. See the section "Location

and Features Labels" for additional information.

Other files, such as 2-line elements for an upcoming Space Shuttle

mission or a mission in progress, may be included from time to time. Files

with 2-line orbital elements normally have filetype ".TXT" or ".TLE". Some

common satellite name abbreviations are:

STS       Space Shuttle missions

HST       Hubble Space Telescope

GRO       Compton Gamma Ray Observatory

UARS      Upper Atmosphere Research Satellite

TOPEX     Topex/Poseidon Earth Resources Satellite

ROSAT     Roentgen Satellite Observatory

MIR       Russian Space Station

There are many other satellites for which data is available. US Space

Command (formerly NORAD) currently tracks some 7000+ objects, of which data

for more than 700 is usually included in the full TLEnnn.TXT files posted

on my bulletin board system.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 13

STSPLUS MAP PROJECTIONS AND DATABASES

-------------------------------------

Cartographers and navigators have long wrestled with the problem of

map projections, the process of transferring location information from a

sphere to a flat surface or map. The U.S. Geological Survey publishes "An

Album of Map Projections", Professional Paper 1453, that contains some 90

basic projections in over 130 different modifications and aspects. Each

projection or modification was developed to serve some specific need or to

optimize certain parameters. The primary concern with any map projection is

distortion. For STSPLUS, this means the accuracy with which the selected

portion of the Earth's surface is displayed. Naturally, the viewer desires

accuracy in area, shape, and distance; unfortunately, you can't have all

three simultaneously with a single map projection.

The cylindrical or rectangular projection, used here and with the

original STSORBIT program, is a good compromise where drawing speed is to

be minimized. The map coordinate data translate exactly to screen pixels

with a minimum of computer processing. However, this projection is unable

to diplay the polar regions well; the distortion increases non-linearly as

the latitude increases. I have chosen to limit the latitude to plus and

minus 85 degrees to avoid some of the severe problems which occur very near

the poles. As the magnification is increased, the distortion inherent in

this projection is reduced for the area shown but different latitudes are

displayed at different scales for a given magnification. A satellite

appears to move more rapidly in high latitudes than at mid or equatorial

latitudes.

The orthographic projection views the world as a sphere and thus only

a single hemisphere can be seen at any given time. However, since the map

center may be placed at any desired point, the "hemisphere" may include a

pole. The projection is calculated as if the viewer were at a great

distance from the Earth and therefore can see a complete hemisphere. This

makes the projection ideal for viewing high inclination satellite orbits.

Perspective is not included in the projection calculations for simplicity.

The orthographic projection has the advantage that ANY area of the Earth

may be viewed, including the poles, and the scale remains the same for any

given magnification and is independent of the area viewed. At the center of

the map, circles of visibility appear as true circles; however, area

distortion increases toward the edge of the screen and is especially

noticeable when the full hemisphere is displayed.

The map databases used for STSPLUS are an adaptation of the Micro

World Database II ("WDB"), generously placed in the public domain by Peter

Pospeschil and Antonio Riveria, and were produced in about 1986. The

original data were from the U. S. Central Intelligence Agency (CIA) as

distributed by the National Technical Information Service (NTIS). There are

several known errors in the database: one island in the South Pacific is

missing its northwestern portion, and several islands near the North Pole

are classified as "lakes" instead of "islands" and are therefore rendered

in the wrong map color. There are also numerous gaps in coastlines which

make color fill very difficult over the range of magnifications used here.

The original map data are identified by "level of detail" with Level 1

having the greatest detail and Level 5 having the least detail. I have

elected to use Levels 1 through 4 for STSPLUS; Level 5 is so coarse as to

be almost useless. I have also written several utility programs to extract

the map coordinate data from the original WDB files by level of detail and

to build an index file for each level to speed access to the data. For the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 14

orthographic projection, the data are also converted from latitude and

longitude to geocentric Cartesian coordinates to minimize subsequent

processing time. Although the resulting map database files are

substantially smaller than the original data files, they are still quite

large for the higher levels of detail; for example, the Level 1 files

require approximately 1.5MB.

STSPLUS attempts to select the map database files appropriate to a

given magnification and map projection. If the selected level is not

present, the program tries the next lower level until the selection process

reaches Level 4, the lowest level of detail. The Level 4 map database files

are required for operation of the program and are included in the standard

program distribution. The Level 3 map database files are included with

program registration and are available separately on my BBS. The Level 2

and Level 1 map database files are available separately to registered

users. See file README for registration information.

Because of the size of the map database files, and because certain

third party memory managers have had problems with the dynamic memory

allocation (as implemented by Microsoft BASIC) used in STSPLUS prior to

Version 9240, map data are ALWAYS read directly from disk. This means that

map drawing times, even for the rectangular projection, are somewhat slower

than with previous versions of STSPLUS. It also means that the program uses

the disk files every time the map is redrawn. Users who plan on running

STSPLUS for long periods of time may wish to place the map database files

on a RAM disk to minimize wear and tear on their disk drive. Use Function

Keys F7+F3 from the Main Menu to set up the correct map database path.

The structure of the map database files is the same for all levels of

detail and the index file (filetype .MCX) is the same size in each case.

STSPLUS cannot distinguish between levels except by file names. Thus, if

you are using a high speed computer such as a 486DX, you may rename the

files to force STSPLUS to use a higher level of detail for a given zoom

or magnification factor. All three files (.MCX, .MCP, and .XYZ) for a

particular level must be kept together with the same filename or chaos will

result! Naturally, drawing times will be increased as the price of the

improved detail.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 15

PROGRAM SETUP AND USAGE NOTES

-----------------------------

The following notes may prove helpful in setting up STSPLUS to operate

most efficiently on your system or to provide hints in ways that some of

the system and program features may be used to advantage.

DOS Memory Managers and Performance Tips

----------------------------------------

The use of memory managers such as EMM386, QEMM386, and 386MAX (but

not NETROOM3 by Helix Software, see below) with 386 and higher systems will

cause the processor to operate in the Virtual 8086 Mode, a "feature" which

is not well documented by Microsoft. Users should be aware that this may

cause considerable additional processor overhead, especially with graphics.

For example, my 486DX/33 typically draws the orthographic map in almost

half the time when a memory manager is NOT present but the improvement is

less significant with a 386SX/20:

Pentium 120 Drawing Time WITHOUT EMM386:    0.88 seconds

Pentium 120 Drawing Time WITH EMM386:       2.09 seconds

Pentium 90 Drawing Time WITHOUT EMM386:     1.05 seconds

Pentium 90 Drawing Time WITH EMM386:        2.80 seconds

486DX/33 Drawing Time WITHOUT 386MAX:       3.68 seconds

486DX/33 Drawing Time WITH 386MAX:          6.70 seconds

386SX/20 Drawing Time WITHOUT QEMM386:     17.91 seconds

386SX/20 Drawing Time WITH QEMM386:        21.97 seconds

Times will vary as a function of the satellite position and other program

features. As the example times above illustrate, the time saving is

proportionally higher with a faster computer. Note, however, that a memory

manager may be required for the use of extended or expanded memory and for

certain applications. The use of different configuration files, selected by

a batch file or other methods, can optimize performance for various

applications. DOS 6.2 and higher includes a multiple boot configuration

facility. Another method is to boot the computer from a floppy disk

(suitably formatted and configured) when the "simple" CONFIG.SYS is to be

used.

A notable exception to the performance penalty associated with memory

managers is NETROOM3 by Helix Software. NETROOM3 uses "cloaking", a memory

technique becoming more popular and which may soon be used in other memory

managers. Initial tests of NETROOM3 indicates little or no performance

penalty as compared to a "vanilla" system with times essentially the same

as those listed as "WITHOUT EMM386" above. NOTE: NETROOM3 does not appear

to operate with Adaptec SCSI-2 cards!

Users should also be aware that typical Local Area Network (LAN)

software, such as NetWare or Lantastic, can impose a performance penalty on

programs such as STSPLUS. Map drawing times with Lantastic LAN operating

showed approximately a 20% increase over the same system without the LAN.

The actual performance will vary as a function of the LAN software used and

whether the system is configured as a server or as a workstation.

HIMEM.SYS can still be used to load DOS in high memory even when

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 16

EMM386 or a similar memory manager is not used by just including the

following lines in CONFIG.SYS:

DEVICE=C:\DOS\HIMEM.SYS

DOS=HIGH

Change the "C:\DOS\" portion of the first line to correspond to the actual

drive and directory where HIMEM.SYS is located in your system.

Most systems now provide disk caching which stores recently read disk

data in high speed Random Access Memory (RAM). While I avoid write caching

(because it increases the risk of disk corruption in the event of power

interruption), read caching using a utility such as Microsoft's SMARTDRV

can all but eliminate disk accesses when STSPLUS is operating. This is

generally a more effective technique than the older RAM disk approach and

is usually automatically included in the computer setup.

DOS CONFIG.SYS Setup

--------------------

Users with DOS 5.0 and higher, especially those who take advantage of

the high memory management capabilities and those who use add-on memory

managers, should include the following line in their CONFIG.SYS file:

STACKS=9,256

This command causes DOS to allocate more memory for the internal stacks

used by DOS and some applications programs. DOS 5.0 seems more sensitive to

the amount of allocated stack space than were prior DOS versions and

programs which executed with no problems on DOS 3.3 may fail on DOS 5.0. A

common symptom of insufficient internal stack space is that STSPLUS

"freezes" the computer and a reboot is required. Other unpredictable errors

can also result from stack errors. "9,256" creates 9 stacks of 256 bytes

each; the "256" may be replaced by "384" or "512", and the "9" may be

replaced by "15" if the basic STACKS command improves but does not cure the

problems. See your DOS manual for additional information.

Copying Files for STSORBIT PLUS

-------------------------------

STSPLUS is intended to be used on systems with a hard disk. If all

files are present, up to 3MB may be required. While it is possible to

operate the program from a high density floppy disk (with some files

omitted), map drawing times are painfully and unacceptably slow.

I recommend that a separate directory called STSPLUS be created for

all of the required and optional files. If you received STSPLUS on disk

with unpacked files, simply copy all files from the floppy disk(s) to your

hard disk using the command:

COPY /B A:*.* C:\STSPLUS

where the floppy drive is assumed to be A:, the hard disk is assumed to be

C:, and the subdirectory STSPLUS already exists. Use the commands

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 17

C:

CD \

MD STSPLUS

to create the subdirectory STSPLUS if it does not already exist.

If you received the program in compressed, self-extracting .EXE format

(with a filename like SOP9537A.EXE and/or SOP9537B.EXE), create the STSPLUS

directory as above then log into the STSPLUS directory and unpack the files

with the commands:

C:

CD \STSPLUS

A:SOP9537A

and repeat the last command for each .EXE file received, changing the

command to reflect the correct filename.

If you received the program in compressed format (.ZIP), copy the .ZIP

file(s) to the STSPLUS directory and then enter the command:

PKUNZIP <filename>

where <filename> is the name of the .ZIP file to unpack. After all files

have been unpacked, you may delete the .ZIP files (but keep a backup copy

just in case!). You MUST use PKUNZIP Version 2.04G or later to successfully

unpack .ZIP files received from my BBS or from NASA SpaceLink BBS!

********************

*  IMPORTANT NOTE  *

********************

If you are upgrading from a prior version of STSPLUS and

experience problems, delete the file STSPLUS.INI to force the

program to recreate its initialization parameters! This will

resolve most path and filename problems. Use Function Keys F7 and

F10+F3 from the Main Menu to set all program paths, filenames,

options, and display features.

Slow Computers and 80x87 Math Coprocessor Chips

-----------------------------------------------

STSPLUS has been designed for 80386/80387 or better computers equipped

with an EGA or VGA color display. While the program can be executed on some

older 8088/8087 (XT-class) or 80286/80287 (AT-class) computers, performance

is seriously degraded. But it would seem that warnings and suggestions can

NEVER convince people that their old clunker is past its prime or that the

calculations required for orbital mechanics and graphics are very complex

and tax even a powerful computer. The best mainframe computers we had a

decade or more ago had trouble doing what I now take for granted on a

486DX personal computer!

Not all personal computers are created equal. Further, the 80x87 math

coprocessor chip can do many of the calculations ten or twenty times faster

than the main processor. For 80386 computers, this makes a tremendous

difference AND for a very modest cost, often well under $100. Some users

report problems with coprocessor chips manufactured by IIT and ULSI; while

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 18

the problems may be related to the computer rather than the coprocessor

chip, I recommend avoiding coprocessor chips by those manufacturers.

Finally, I really don't want to hear from users how slow this program runs

on older machines; I recommend the original STSORBIT if you want the best

performance from any computer not equipped with a math coprocessor chip.

STSPLUS always runs as fast as the processor will permit and, since

most features are available for all computer configurations, it is the

responsibility of the user to select program features and options

consistent with the desired performance. For example, the solar terminator

requires considerable time to perform the required calculations and to

update the display and, if this feature is not required, performace will be

enhanced if it is disabled.

As features have been added to STSPLUS, it has become increasingly

difficult for very slow computers or computers not equipped with a math

coprocessor to perform the necessary calculations in a reasonable time.

STSPLUS tests to determine whether or not a math coprocessor chip is

present and will use it if so. The following table illustrates the

difference the processor and a math coprocessor chip make:

Processor   Speed   Coprocessor   Time (secs)

---------------------------------------------

Pentium     90 MHz      YES               2.5

486DX       33 MHz      YES               6.2

386DX       20 MHz      YES              12.8

386SX       20 MHZ      YES              16.8

386SX       20 MHz       NO              66.0

286          8 MHz      YES              30.0

286         12 MHz       NO              86.6

8088         8 MHz      YES              65.0

8088         8 MHz       NO             426.4

All systems used MS-DOS in the "typical" configuration for the machine

under test; a memory manager was present in 386 and higher systems. The

tests were performed with STSPLUS by measuring the time required from the

Main Menu until the satellite appeared on the world map display using the

rectangular map projection. (Not all tests were performed with the same

version of STSPLUS!) All data were resident in memory (no disk operations

required). All tests were made using the same display options (most were

enabled). Slightly better performance can be achieved from the slower

computers if some options, such as Sun terminator, are disabled. Clearly,

the 8088 without the math coprocessor chip is not acceptable, and NONE of

the computers without the math coprocessor chip performs very well.

When STSPLUS begins, it checks to determine the type of processor and

math coprocessor. The Title Screen displays the processor type, coprocessor

type (if present, otherwise "(not installed)") and display type. Since

STSPLUS cannot distinguish between 486DX and Pentium processors, it

displays "486DX+" for those processors. If no math coprocessor chip is

detected, the following caution message is displayed. Press ENTER (or any

key) to continue.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 19

** CAUTION **

STSORBIT PLUS has detected NO MATH COPROCESSOR CHIP!

CPU Type = 80386DX or 80386SX

NDP Type = (not installed)

STSPLUS requires a Math Coprocessor Chip for acceptable performance.

Calculation and map drawing times will be VERY SLOW. There is no

remedy except to add a Math Coprocessor Chip to your computer.

Users without a coprocessor should consider using

STSORBIT

which is my original, simplified tracking program.

If, in spite of everything, you insist on using STSPLUS on your old

clunker, here are a few cautions and reminders.

1.   Especially at startup and when drawing the maps, long time delays can

be expected with no math coprocessor -- on the order of minutes in

some cases. Status messages are presented on the screen during some

(but not all) of these delays. Note also that STSPLUS.EXE is

compressed to save disk space and is decompressed at load time; this

may cause a noticeable delay on some systems.

2.   Avoid using the orthographic projection with slow computers; the map

drawing times are much longer than the rectangular projections. Also

avoid high zoom factors since it is possible that the satellite may

move during the drawing process to the point where as soon as the

display is completed, it's time to redraw the map again. This process

will repeat endlessly and a reboot may be required. Restart the

program without the "/R" command line option and use F10+F6 from the

Main Menu to select WORLD or QUAD maps.

3.   If you have a monochrome monitor, experiment with the "/M" command

line option to force monochrome operation. On some monochrome systems

the program may otherwise fail with or without an error message. On

other monochrome systems, certain colors may not be visible when

simulated using shades of gray.

4.   The original CGA Color Graphics Adapter, even when equipped with a

color monitor, can display reasonable graphics (640x320) ONLY in

monochrome. Your color CGA monitor buys you nothing for graphics that

are quite inferior to the EGA and VGA. Support for CGA and Hercules

monitors may be discontinued for future versions of STSPLUS.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 20

Starting Program STSORBIT PLUS and Command Line Options

-------------------------------------------------------

STSPLUS uses file STSPLUS.INI to save various information required for

operation. Since the format of that file may change from time to time,

STSPLUS ignores the file unless it was written by a recent program version.

If the file does not exist, STSPLUS will automatically create it.

To start program STSPLUS, use a batch file similar to "S.BAT" in the

preceeding section or enter one of the following commands:

STSPLUS            Automatic monitor, CGA/HGC/EGA/VGA

STSPLUS /EGA       Force EGA (or lower) monitor

STSPLUS /CLK       Use 43 or 60 lines for graphics display of

data and large clock characters. NOTE: This

feature available with EGA and VGA displays

ONLY! It is ignored for CGA/HGC monitors.

STSPLUS /CGA       Force CGA monitor

STSPLUS /M         Force monochrome operation, EGA/VGA

STSPLUS /21        Use 21x21 pixel internal satellite icons

rather than the standard 15x15 pixel icons.

STSPLUS /R         Resume last mission automatically

STSPLUS /R/SS      Force "screen saver" mode

Only one display option (/EGA or /CGA) may be used. Other options may

be combined and entered in any order. For example, using the following

command will resume the prior mission and force EGA mode:

STSPLUS /R/EGA

If you are using a Hercules Graphics Card, run the program MSHERC

prior to running STSPLUS. This Microsoft program works with compiled BASIC

programs to enable use of the Hercules Graphics Card. Orthographic

projections are NOT supported for Hercules Graphics Cards. One user

reported that setting the HGC to FULL and selecting Page Zero (using

software supplied with the HGC) was sufficient for proper operation. At

least one HGC "clone" user reported that STSPLUS could not be run under any

circumstances.

If you have already run STSPLUS and simply wish to resume the prior

mission, use the /R (resume) command line option:

STSPLUS /R

STSPLUS will sense the monitor type, enable color for EGA and VGA systems,

then proceed directly to plotting the mission. The data from the last run,

as saved in file STSPLUS.INI, is used to initialize the program. Once

started in this manner, pressing the ENTER key after the map is displayed

will return to the Main Menu.

The special command line option "/R/SS", used after you have already

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 21

run STSPLUS, forces the "screen saver" mode:

STSPLUS /R/SS

In this mode, available only with EGA and VGA monitors, the program

displays the orthographic globe with ground track and other selected

display features but NO DATA. Using the orthographic globe, the screen

changes frequently EXCEPT that the satellite icon remains always centered

on the map and the circle outlining the globe is always drawn (which may

eventually cause phosphor burns in those areas). To return to DOS, press

ENTER or ESC.

Several additional command line options are available to control

certain map display features. These may be included in addition to the

command line options above.

+L   Include Location and Feature Labels

-L   Exculde Location and Feature Labels

+R   Include Rivers and Lakes

-R   Exclude Rivers and Lakes

+T   Include Tracking Stations

-T   Exclude Tracking Stations

+V   Include Local Circle of Visibility

-V   Exclude Local Circle of Visibility

These command line options are useful in the "screen saver" mode, with

batch files, and for operations when no operator is present. When a feature

is enabled or disabled with these command line options, the new status of

the feature is saved for subsequent uses. For example, the following

command line could be used in "screen saver" mode for a "minimum" display:

STSPLUS /R/SS-L-R-V

and normal use would be invoked with:

STSPLUS /R+L+R+V

The following sequence of operations is recommended when STSPLUS is run for

the first time:

1.   Make sure all required STSPLUS files are in subdirectory \STSPLUS (or

whatever you named it). In addition to the program files, check that

2-line elements are also available; the file TLEnnn.TXT (where "nnn"

is a number like "440" is usually supplied with STSPLUS) and similar

files are available via BBS or the Internet.

2.   STSPLUS will first request that the user enter the time offset from

Universal Coordinated Time ("UTC") and whether or not Daylight Savings

Time is in effect. Enter the appropriate responses. Examples are shown

on the screen.

3.   After a short delay, the Main Menu will be displayed. Press Function

Key F7 and set all paths and filenames. If all files have been copied

to the subdirectory from which the program was executed, the default

values shown should be correct. Press ENTER to return to the Main Menu

when all changes have been completed.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 22

4.   Press Function Key F10 followed by Function Key F2 to set your local

coordinates. STSPLUS defaults to Palos Verdes, CA, near Los Angeles

and some program functions require your coordinates for accuracy.

Press Function Key F1 to set the primary location and follow the

instructions. If your location is not included in STSPLUS file of city

names (STSPLUS.CTY), select a large nearby city. If you know your

local coordinates, you may enter them manually using Function Key F3

and append them to file STSPLUS.CTY. When your local coordinates have

been set, press ENTER twice to return to the Main Menu.

5.   Press Function Key F2 to select the primary satellite you wish to

track with STSPLUS. You will be asked to select a filename (the

available files whose filetype is ".TXT" or ".TLE" are displayed and

may be selected using the arrow keys). Type the filename or press

ENTER after selecting a filename. You will then be asked to select a

satellite by name or NORAD number. If you do not know a specific

satellite name, enter "HST" for the Hubble Space Telescope. (Caution:

some TLE files also include "HST Array"; press the SPACE BAR to

continue searching for HST.) The data for the satellite is displayed;

press ENTER to accept the data. STSPLUS will immediately begin drawing

the map.

6.   While the map is displayed you may press ENTER to return to the Main

Menu. Press ESC at the Main Menu to return to the DOS prompt.

7.   STSPLUS is a complex program. Only the most basic and essential

features have been covered in this brief startup list. Please take

time to read the manual with your favorite editor or print the text as

reference material.

Predicting Visible Satellite Passes

-----------------------------------

One of the most popular uses for a satellite tracking program is to

show when a satellite of interest will be visible from a specified

location. Using STSPLUS, my wife and I have spotted four different Space

Shuttle missions, the Hubble Space Telescope, the Russian MIR Space

Station, and many others with the naked eye. The trick, of course, is

knowing when and where to look for the satellite.

During normal operation, STSPLUS displays two timers in the form of

countdown clocks (Minutes:Seconds), "AOS" and "LOS", for the user's

location (as well as for a number of other events).  AOS, Acquisition of

Signal, is the time remaining until the satellite is next within the circle

of visibility. LOS, Loss of Signal, is the time remaining until the

satellite next passes outside the circle of visibility. STSPLUS looks ahead

four hours (240 minutes) from the time the map is drawn to determine the

next AOS and LOS event; the clocks are blank if the time is greater than

240 minutes. A quick inspection of these clocks can therefore determine if

a potentially visible pass is upcoming within four hours.

Using its internal pass prediction feature, STSPLUS can look ahead

in 48 hour blocks and list the passes when the selected satellite will

be within the local circle of visibility (line of sight, no lighting

constraints applied).

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 23

For more extensive tabular data on upcoming visible passes, external

software must be used. There are a number of satellite tracking programs,

most notably Paul Traufler's TRAKSAT and TS Kelso's TRAKSTAR, which can

generate tabular data for a given satellite or group of satellites listing

when the satellite(s) will be visible subject to various visibility

constraints.

Predicting Satellite Passes with STSPLUS

----------------------------------------

STSPLUS has a pass prediction capability to make tabular visible and

line-of-sight pass predictions for the current satellite quick and easy.

Once a satellite has been selected (with F2 from the Main Menu), pressing

F3 from the Main Menu, and selecting Data Format 9 (Pass Predictions) will

quickly produce a listing of the passes for that satellite over the next 48

hour period. Visible passes during that time period, if any, are specially

flagged. See the section "Pass Predictions and Data Output" for additional

information. Here are some comments and suggestions for satellite viewing

and tracking with STSPLUS.

1.   The pass predictions are listed in blocks of 48 hours beginning with

the current real or simulated time, and are given an arbitrary pass

number from 1 to 99. The data include the "AOS" or Acquisition of

Signal when the satellite rises above the local horizon, "MAX

VISIBILITY" or the time at which the satellite reaches the highest

altitude above the local horizon, "LOS" or Loss of Signal when the

satellite sets below the local horizon, and "Duration" or the total

time the satellite is above the local horizon. Passes for which the

satellite may be visible for some or all of the pass are colored in

WHITE and the "V" (for Visible) column at right is flagged with a

flashing asterisk. Note that "local horizon" means the true horizon

rather than local terrain. For either visual or electronic tracking,

the altitude above the true local horizon when the satellite may

actually be seen or tracked is usually at least five degrees even

under optimum conditions.

CAUTION: Users with slow computers, especially those without a math

coprocessor, will find that pass predictions may require considerable

time -- up to tens of minutes using a slow 286 without a 287!

2.   Be sure to use current orbital elements for the satellite. My RPV

Astronomy BBS has current 2-line orbital elements for more than 1600

satellites and these data are also available from other electronic

sources. For Low Earth Orbit satellites like the Space Shuttle or MIR,

orbital elements should be no more than about ten days old, more

recent if the satellite may be maneuvering; for higher orbit

satellites, orbital elements remain accurate for longer periods, up to

about 30 days. Satellite maneuvers can radically change the orbital

elements. Pass predictions can be no more accurate than the orbital

elements used!

3.   Examine the "Alt" (maximum altitude or elevation in degrees above the

true horizon) given under the "MAX VISIBILITY" columns for each pass

and select a suitable pass. As a general rule, the higher the maximum

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 24

altitude, the better the visibility. If you wish to see a particular

pass as a ground track display, enter the pass number and STSPLUS will

set simulated time to about 30 seconds prior to the time of maximum

visibility and prepare the display. Press "L" while the ground track

is displayed to use the Location Map centered on your location with

the "bulls-eye" concentric circles of equal altitude. You may

stop/pause the display by pressing Function Key F6, then move the

satellite forward or backward in time using the "+" and "-" keys and

adjust the time step (1, 10, or 60 seconds) with Function Key F4.

Press ENTER to resume normal (simulated time) operation.

4.   STSPLUS lists all Line-of-Sight passes, those passes where the

satellite rises above the local true horizon, and flags all passes

which may be visible, those passes for which the satellite is in

sunlight and the user's location is in darkness. Visible passes are

marked in WHITE and flagged in the far right column with a flashing

asterisk. A satellite may be visible for only a portion of a pass, in

which case not all items may be marked in white. In this context, a

"visible pass" means "visible to the naked eye or with binoculars".

Although there are exceptions in unusual situations, this generally

restricts the times for visible passes to the several hours prior to

local dawn and the several hours after local sunset. (Note, however,

that "visible" to a ham radio operator or radar tracking station

simply means above the horizon!) In most cases, the Space Shuttle and

satellites such as MIR Space Station and Hubble Space Telescope are

visible with the naked eye given favorable lighting and weather

conditions. Satellites in higher altitude orbits will be visible

sooner before dawn and longer after sunset. Satellites in very high

orbits, no matter how large the satellite, are seldom visible without

high power binoculars or a telescope. Dates and times may be given in

either Coordinated Universal Time (UTC) or local time.

5.   The geometry of the pass and the attitude and geometry of the

spacecraft are also important in determining visibility. The relative

angles between the Sun, the satellite, and the viewer determine how

light is reflected from the surfaces of the spacecraft to you, the

viewer. A spacecraft passing between you and the Sun may not reflect

much light to you and may therefore not be visible even at higher

altitudes. On the other hand, a spacecraft nearer the horizon but on

the other side of you from the Sun may appear brilliantly lighted. The

kinds of surfaces on the spacecraft are important too; solar panels

and flat surfaces can reflect enough light to appear the most

brilliant objects in the sky while larger but rounded spacecraft may

seem all but invisible.

6.   Given otherwise good conditions and favorable weather, the single most

important factor is spacecraft apparent altitude ("Alt") during a

pass. This is the spacecraft's apparent elevation above your local

horizion. Depending upon local conditions, an altitude of at least 5

to 10 degrees will generally be necessary before a spacecraft can be

seen even under the best lighting conditions. In the Los Angeles area,

at least 20 or 30 degrees is a better number to use because of smog,

haze and light polution (except when looking out over the Pacific

Ocean).

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 25

7.   The azimuth ("Azm") is the angle in degrees which describes the

direction from the viewer to the spacecraft at any moment. This is

given in the sense NESW, North to East to South to West; North is 0

degrees, East is 90 degrees, etc. For a good visible pass after

sunset, for example, an azimuth ranging from 60 to 150 degrees would

indicate a pass moving generally from the Northeast to the Southeast,

ideal lighting conditions with the Sun in the West.

8.   Remember that STSPLUS automatically sets SIMULATED TIME when using the

pass prediction feature to display passes. If simulated time is

already in effect, pass predictions start from the current simulated

time and a new simulated time is automatically set for a selected

pass. Once set, simulated time remains in effect until changed by

selecting another pass or until reset with F8 from the Main Menu. To

return to "real time", press F8+F1 from the Main Menu. The Main Menu

displays the current time with the time mode in effect labeled as

"Current Time" or "Simulated Time".

Predicting Satellite Passes with TRAKSTAR

-----------------------------------------

In order to generate detailed tabular predictions for satellite

passes, an external program is required. I recommend two programs for this

purpose: Paul Traufler's TRAKSAT and TS Kelso's TRAKSTAR. Each programs is

copyrighted by the respective author and is readily available. They are

both fine programs and set a standard against which other satellite

tracking programs are measured for performance and accuracy.

I have selected TRAKSTAR as the default external program used with

STSPLUS for two reasons: first, the program quickly produces very accurate

tabular data without graphics; and second, TRAKSTAR requires minimum memory

and is easily configured for seamless operation with STSPLUS. TRAKSTAR is

available on my RPV Astronomy BBS as file TRAKSTR2.ZIP and on the Internet

via FTP at "archive.afit.af.mil".

TRAKSAT is a very large program with many features, and most computers

will not have sufficient memory to execute TRAKSAT when STSPLUS "shells" to

DOS. In order to run TRAKSAT, users must first exit STSPLUS (press "ESC"

from the Main Menu).

For additional information on these programs, contact the authors:

TRAKSTAR:      Dr. T. S. Kelso

Adjunct Professor of Space Operations

Air Force Institute of Technology

1653 London Town Lane

Montgomery, AL  36117-1755

BBS: Celestial BBS

(334) 409-9280

300/1200/2400/4800/9600 baud

TRAKSAT:       Paul Traufler

111 Emerald Drive

Harvest, AL  35749  USA

BBS: RPV Astronomy BBS

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 26

(310) 541-7299

2400 - 28800 baud

As a courtesy to the author, I suggest enclosing a stamped, self-addressed

envelope if you write and request a reply. The current version of each

program is always posted on the indicated BBS and messages may be left

there for the author.

There is no "standard" filetype used for 2-line elements files;

typical filetypes in regular use are ".TXT", ".TLE", ".N2L", and ".ELE";

some files also include comment lines, multi-line commentary, or additional

data, some or all of which must be removed prior to use with most satellite

tracking programs. As released by Dr. Kelso, TRAKSTAR expects a filetype of

".TLE" and cannot accept any other filetype without being re-compiled.

STSPLUS defaults to both filetype ".TXT" and ".TLE". TRAKSAT defaults to

filetype ".TXT" only. However, 2-line elements files, even on Dr. Kelso's

Celestial BBS, may use either ".TLE" or ".TXT", depending upon the file.

For the past four or five years, Paul Traufler and I have been jointly

releasing file TLEnnn.TXT (usually as file TLEnnn.ZIP, where "nnn" is a

number like "143"). Until February, 1993, the file was named NASAnnn.TXT.

This file is a sorted concantenation of files TLE.TXT and GROUP000.TLE from

Celestial BBS and currently includes 2-line elements for some 700+

satellites.

STSPLUS solves the filetype problem with TRAKSTAR by dynamically

creating a file called STSPLUS.TLE which contains the 2-line elements for

the currently selected satellite. Since STSPLUS can select 2-line elements

from a file of any size, calling TRAKSTAR through STSPLUS also circumvents

a minor problem in the current version of TRAKSTAR which limits the number

of satellites in a 2-line elements file to 250 element sets. As a

convenience, STSPLUS also dynamically creates the files STSPLUS.OBS (which

contains the name, coordinates, and elevation of the current user location)

and TRAKSTAR.CFG (which contains the drive and path information for

TRAKSTAR).

An accurate elevation (above mean sea level) for the user location is

required for accurate calculations in any satellite tracking program,

including STSPLUS, TRAKSTAR, and TRAKSAT. Note that the elevations of most

locations in file STSPLUS.CTY are not readily available and have been set

to zero. The elevation is the last parameter on each line in file

STSPLUS.CTY and is given in integer meters; 1 meter equals 3.28083 feet.

STSPLUS is coded to operate with TRAKSTAR Version 2.15. It may or may

not operate correctly with other versions. To set up program TRAKSTAR

either for independent use or for use with STSPLUS, follow the following

steps:

1.   Copy the TRAKSTAR files to your hard disk. Unpack the files if they

are contained in an archive file such as TRAKSTR2.ZIP. I recommend

using a separate directory called "TRAKSTAR". The complete TRAKSTAR

package includes documentation, Pascal source, example and test files,

and the program itself. The following two files are required for

operation with STSPLUS:

TRAKSTAR.EXE        Main Program

TRAKSTAR.HDR        Header File

Additional files are required for independent operation. Read the

TRAKSTAR documentation for details.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 27

2.   Run STSPLUS and enter the drive and path information for TRAKSTAR by

pressing F7+F5 (Set FILENAMES and PATHS) from the Main Menu. STSPLUS

will automatically default to its own drive and directory if you omit

this step.

3.   Select the desired 2-line elements file and satellite by pressing F2

from the Main Menu.

4.   When the map is on the screen and you have verified that the correct

satellite is being tracked, press ENTER to return to the Main Menu.

5.   Now press F4 from the Main Menu to run TRAKSTAR. You should

immediately see TRAKSTAR's opening screen and the first selection.

Make the various selections by using the up and down arrow keys to

move between selections and press ENTER when you have the correct

selection. Don't forget to press SPACE to select the satellite!

*************

*  CAUTION  *

*************

TRAKSTAR Version 2.15 uses the DOS clock to determine the default

start and stop times for its calculations. Not mentioned in the

TRAKSTAR documentation, however, is the fact that TRAKSTAR assumes

that the computer is set to Coordinated Universal Time (UTC). Be sure

to take that difference, including the date, into account when

entering start and stop times!

6.   TRAKSTAR will now make its calculations and write the results to a

file. The file is written in the TRAKSTAR directory and will overwrite

an existing file of the same name. The time required will be a

function of the time span and time interval requested as well as the

calculation speed of your computer. Read the TRAKSTAR documentation

carefully so that you will recognize the name of the file that

TRAKSTAR writes. For example, requesting visible passes for the Hubble

Space Telescope (NORAD #20830) will result in a filename of "VOB20830"

and a filetype which is the last three digits of the element set

number (such as ".866").

7.   As soon as TRAKSTAR finishes, you will return to STSPLUS's Main Menu

and may continue normal operations.

8.   To examine the tabular data produced by TRAKSTAR, you must either

"shell to DOS" using F9 from the Main Menu or exit STSPLUS by pressing

ESC at the Main Menu. Use an ASCII editor to view the file or send it

to your printer for hard copy.

If STSPLUS cannot find TRAKSTAR.EXE (or TRAKSTAR.BAT, see below) in the

selected directory, an error message will be displayed. Press ENTER to

return to the Main Menu.

An alternative method is to create a file TRAKSTAR.BAT in which you

place all commands required to run TRAKSTAR or the satellite tracking

program of your choice. STSPLUS.TLE and TRAKSTAR.CFG will still be written

to the selected directory but they need not be used. This method is only

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 28

recommended for individuals who understand the use and operation of DOS

batch files.

Printing Graphics Screens

-------------------------

Many users have requested that I add a "print" function to STSPLUS.

Given the number and variety of printers available for use with these

systems and the fact that I only have a couple of printers I can use for

testing, this is not practical within STSPLUS. However, DOS includes the

GRAPHICS command which may be used with many computers to enable printing

of graphics images. Check your DOS and printer manuals for details. As an

example, I use the following command on systems equipped with a Hewlett-

Packard LaserJet II or III:

GRAPHICS LASERJETII

There are also quite a number of screen capture and print screen

programs, both commercial and shareware, which can perform this task. Note,

however, that all these programs are TSR's (Terminate and Stay Resident)

and some could interfere with STSPLUS's operation.

Some print screen programs, inculding DOS's built-in GRAPHICS command,

do not render colors very well. Certain colors may not be visible on the

printed copy at all. For such programs, use the "/M" command line option to

force monochrome operation when you wish to print graphics images from the

screen.

Known STSPLUS Problems and Bugs

-------------------------------

STSPLUS is being used on thousands of computers around the world

without any significant problems. However, like almost any computer program

and in spite of my best efforts, there are several known problems or "bugs"

with STSPLUS. Some are the result of slow computers, others are in the

program itself. Hopefully, some or all of these problems, those I can

reproduce at least, will be repaired in due course. Some problems are

caused by other software interfering with the program's operation. Still

other problems are the result of incompatible "IBM-compatible" computers

for which there is no remedy.

One "problem" which I occasionally still hear about is that a user's

CGA color monitor only displays STSPLUS in monochrome. THIS IS NOT A BUG!

CGA systems display "high resolution" 640 x 200 graphics in monochrome

ONLY. Compared to the EGA or VGA, that resolution is barely acceptable. The

color graphics mode for the CGA is 320 x 200 which is inadequate for

STSPLUS. Because of the poor performance of the CGA display, some program

features are NOT available. Support for the CGA and HGC display may be

discontinued completely in the future.

1.   STSORBIT PLUS has been run extensively on systems using Microsoft DOS

3.3 through 6.2 and there are no known problems with those operating

systems EXCEPT the STACKS problem with DOS 5.0 (see the section

Program Setup and Usage Notes above). Because of the many bugs

reported, I do NOT recommend use of DOS 4.xx under any circumstances;

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 29

upgrade ASAP to DOS 6.2! Many users report good performance using IBM

OS/2, Version 2.1; earlier versions of OS/2 are NOT recommended. Users

report memory allocation problems with some early versions of Digital

Research DRDOS 6.0 and certain third party memory allocation programs.

A typical symptom of this kind of problem is that when you attempt to

return to DOS, you receive an error message or the computer freezes.

2.   Some math coprocessor chips fail to execute STSPLUS and similar

programs correctly. In at least two reported cases, early 387SX math

coprocessor chips from IIT generated random errors; when notified of

the problem, ITT replaced the suspect chips and the problems

disappeared. Reports indicate that USLI 387SX chips do not operate

correctly with STSPLUS or TRAKSAT; Intel or Cyrix 387SX chips have

always cured the problems. No problems have been reported for Intel or

Cyrix chips.

3.   Note that not all computers (especially older CGA systems) will

display the extended graphics characters used for the large clock

characters (selected with F2 when the map is displayed). The symptom

of this problem is that the lower left portion of the data block is

mostly blank after pressing F2. If you have this problem and your

computer is running DOS 3.x or higher, enter the command "GRAFTABL"

at the DOS prompt before running STSPLUS or include the line

"GRAFTABL" in your AUTOEXEC.BAT file; this sets the "code page" to

enable the computer to display the extended graphics characters. [The

program GRAFTABL.COM is usually included as part of DOS.]

4.   Some users report problems with certain Terminate and Stay Resident

(TSR) programs for which the only remedy is to remove the offending

TSR. This usually requires a "trial and error" approach to pinpoint

the TSR causing the problem. The best method is to remove ALL such

programs from your AUTOEXEC.BAT file, including "DOSSHELL", to make

sure the program will work with your computer in the simplest possible

configuration. Similarly, delete all special memory and device drivers

from your CONFIG.SYS file. For DOS 5.0 and higher, include the line

"STACKS=9,256" to your CONFIG.SYS file. In some cases it may be

necessary to increase the number "256" to "384" or "512". One user

reported a problem on an IBM PS/2 when a mouse driver was used (but I

regularly execute STSPLUS with my mouse active!).

5.   LCD VGA displays, such as are found on newer laptops, have 480 fixed

scan line positions (unlike the variable scan modes available with CRT

displays). This means that the EGA emulation used in the STSPLUS

Motion Maps may use only 350 of the available 480 scan lines and the

image will be compressed vertically. An alternate solution to EGA

emulation is to repeat (double) some scan lines; this can result in

peculiar display artifacts such as double-thick horizontal lines at

certain screen locations. The solution to this dilemma varies by

laptop manufacturer.

6.   Even with CRT monitors, all VGA adapter cards are not equal -- just in

case any of you had some illusions left. In at least one case, the

aspect ratio of the CRT display is incorrect when the display is

operated in the EGA simulation mode. The vertical scale is compressed

by about 20% as compared to either a true EGA display or other

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 30

(correct) VGA adapter cards. So far as I know, there is no remedy.

Some early VGA cards (the 449 card from Zenith is an example) are not

always recognized as VGA; the card may not be register-compatible with

the IBM standard and is recognized only as EGA instead.

7.   There appears to be a subtle problem when changing to or from Daylight

Savings Time (which recently happened and "announced" the bug). The

display appears to get caught in a loop, endlessly redrawing the

screen or shows the wrong time and/or time zone. To avoid the problem,

change the Daylight Flag then EXIT THE PROGRAM AND RESTART.

8.   Several users have reported "bugs" with the local Circle of Visibility

and the ZOE (Zone of Exclusion) when displaying satellites with

relatively high eccentricity. This is NOT a bug, rather it's a problem

with the orbit. STSPLUS determines the local coverage and ZOE based

upon the satellite altitude at the instant the map is drawn and these

map features are updated only when the map is redrawn. For a satellite

with a high eccentricity (such as RS-10, NORAD #18129, a popular

amateur radio satellite), the diameter of the local Circle of

Visibility can change considerably over relatively short time periods

especially when the satellite is near perigee. Likewise, TDRS coverage

will be continuous near apogee and hence no ZOE. When the map is

redrawn nearer perigee, coverage will not be continuous and a ZOE will

be shown. To minimize this effect, press ENTER twice as the satellite

approaches a point or time of interest; this will cause the map to be

redrawn with current data. As an aside, the TDRS system is NOT used

with such satellites!

9.   The map database used with STSPLUS originated with the CIA quite some

years ago (early 1980's). Certain islands in Northern Canada are shown

instead as lakes, and several islands in the South Pacific are

misdrawn or are missing entirely; Fiji and Western Samoa are examples

respectively. Most very small islands were intentionally omitted. I

have a more recent version of the map database but the current files

are large and distribution is so widespread that changing the database

presents major problems. For the present, I plan to continue with the

current database files, errors notwithstanding.

10.  Finally, as noted elsewhere, all computers are NOT equal. There are a

few (usually older) computers which will not execute STSPLUS under any

circumstances. Tandy is the most common offender followed by Leading

Edge. Some models from these and other manufacturers have BIOS

problems or errors which prevent programs compiled with the Microsoft

BASIC compilers from operating (sometimes only in graphics modes).

There is no remedy. Other computers, Ergo for example, exhibit

"strange" behavior in some graphics and text modes. There may be an

update or workaround available for these problems; check with the

computer manufacturer.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 31

Satellite Name Cross-Reference using STSPLUS.XRF

------------------------------------------------

STSPLUS can perform satellite name cross-references using file

STSPLUS.XRF. Each time TLEs are read and accepted, STSPLUS checks for the

cross-reference file and, if the file is present in the current directory,

checks for the NORAD Number of the satellite and a cross-reference name. If

found, the new name is substituted for that in the TLE file. The XRF file

is standard ASCII and may be created and/or edited with a standard ASCII

editor. Word processor users must use the "non-document" mode. Each entry

consists of a SPACE, the five-digit NORAD Number, a SPACE, and then the

satellite name. A sample file:

22920 HST Solar Array

22076 Topex/Poseidon

21225 Gamma Ray Observ

20638 Rosat Observatory

20580 Hubble Telescope

16609 MIR Space Station

NOTE: The leading space shown above is required for file compatibility with

XRF files used with my program ORBITEL (but is optional for STSPLUS). The

NORAD Number MUST be 5 digits; pad with leading zeroes if necessary.

Only one entry is read per satellite; the search stops with the first

match of NORAD Numbers. There is no limit on the number of entries, but

larger files with many entries may introduce a slight delay as the file is

read. If the satellite name is longer than 19 characters, only the first 19

characters of the name will be used because of space restrictions on the

display. Since the name substitution is unconditional, care should be taken

to enter the correct NORAD Number for the named satellite; if in doubt, use

F2 from the Main Menu to request orbital data by the NORAD Number (enter

"#nnnnn" for the satellite name) to inspect the data and to verify that it

is the correct satellite.

This feature may be used to substitute a preferred name for that given

in a TLE file (which may not always be consistent from source to source) or

to specify the name of a payload piggy-backed on another satellite (as is

frequently the case with amateur radio transponders). In the example above,

"Hubble Telescope" is substituted for "HST", the satellite name usually

present in TLE files for NORAD Number 20580. A number of XRF files are

posted on my RPV Astronomy BBS; such files must be renamed STSPLUS.XRF

before use with STSPLUS.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 32

Preparing 2-Line Elements using VEC2TLE by Ken Ernandes

-------------------------------------------------------

Especially for Space Shuttle missions, Earth-Centered Inertial ("ECI")

cartesian state vectors may be the only orbital information available in

near real time. Such a state vector, consisting of position and velocity

data at a specified time, is sufficient to determine the instantaneous

orbit of a satellite. NASA and other agencies may provide state vectors

referenced to the mean equator and equinox of the Besselian year 1950

("M50", "Mean of 1950", or "B1950") with units of measure in feet and feet

per second. Data may also be available for the true equator and equinox of

date (such as the state vectors generated by STSPLUS), the mean equator and

equinox of the Julian year 2000 ("J2000"), or in the time-independent

Earth-Fixed Greenwich ("EFG") coordinate systems. Kilometers or nautical

miles or variations may also be used as the units of measure.

However, STSPLUS and most other satellite tracking programs require

orbital data in the "2-Line Elements" or "TLE" format and state vectors

must be converted to that format before the data may be used. The 2-Line

format originated as 2-Card Elements back in the days of IBM punched cards

at NORAD (North American Aerospace Defense Command, now US Space Command),

and has become the de facto standard format for orbital data used with

satellite tracking software.

Mr. Kenneth J. Ernandes has written program VEC2TLE, Vector to Two

Line Elements, specifically to convert state vectors to the 2-line format.

VEC2TLE is copyrighted software distributed as shareware, and registration

is required prior to regular use. Mr. Ernandes has extensive experience in

orbital mechanics with US Space Command and in industry, and has used his

expertise to create a precision conversion program. For additional

information and registration details, write:

Mr. Kenneth J. Ernandes

16 Freshman Lane

Stony Brook, NY  11790-2712

CompuServe:  70511,3107

Internet:    70511.3107@cis.com

When writing Mr. Ernandes for information, I suggest including a stamped

self-addressed envelope as a courtesy. The current version of VEC2TLE is

usually posted on the RPV ASTRONNOMY BBS and on the CompuServe Astronomy

and Space Forum.

Note that although 2-line elements can be generated using only the

data in an ECI state vector and these elements will yield an accurate

position at the specified time, the "epoch" of the data, additional data

(in particular, Drag and B-Star parameters) are required to generate 2-line

elements which propagate accurately over time. Certain additional

parameters, such as element set number and orbit number, do NOT affect the

accuracy of the propagated position; these data may be obtained from other

sources or default values may be used. US Space Command assigns a Catalog

Number, often referred to as the "NORAD Number", some time after launch;

pre-launch elements for Space Shuttle missions may have a temporary Catalog

Number (corresponding to the mission number) until the actual Catalog

Number is assigned. The International Designation is assigned by COSPAS at

the time a launch is registered by the launching country and may be blank.

Element set ("ElSet") numbers are assigned arbitrarily by the originating

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 33

individual or agency and have no effect on the orbital data. Orbit (or

revolution or "REV") numbers are incremented on each revolution at the

ascending node, the point at which a orbit crosses the equator heading

North. Note that US Space Command does not usually use the same reference

for orbit numbers as does NASA; NASA defines the first partial orbit as

"Rev 1" whereas US Space Command may call that "Rev 0" or some other

arbitrary number. At least for Space Shuttle missions, it is common

practice to adjust USSPACECOM orbit numbers to conform to the NASA

convention.

VEC2TLE accepts all data required to form a complete 2-line orbital

element set, either as manually entered data or from a properly formatted

vector input file, performs limited error checking on these data, then

displays and writes the generated 2-line elements file. VEC2TLE supports

STSPLUS Data Mode 5 through 7 state vector formats. The program also

supports a variety of coordinate systems, units of measure, and time

formats as well as offering many other useful features.

Care must be taken when using VEC2TLE that the proper units of measure

(kilometers, feet, or nautical miles), coordinate system (ECI or EFG), and

epoch (True of Date, Mean of 1950, etc.) are used. See the VEC2TLE

documentation for additional information on the available options. STSPLUS

generates ECI X-Y-Z state vectors for the true equator and equinox of date

and may use any of three units of measure. NASA, on the other hand, usually

generates their state vectors for the mean equinox and equator of 1950

("M50") and uses feet and feet/second units of measure. Obviously, using

the wrong units of measure or coordinate system will yield invalid results!

VEC2TLE has been extensively validated and tested in conjunction with

STSPLUS using NASA ECI state vectors (provided courtesy Willie Musty,

Mission Support, Rockwell International, Downey, CA) heginning with Space

Shuttle missions STS-56 and STS-55 in early 1993. The resulting 2-line

elements yielded orbiter positions which corresponded exactly with those

shown live on NASA Select TV, and the 2-line elements were in close

agreement with 2-line elements subsequently released by US Space Command

for a comparable epoch. In fact, during mission STS-56, Rockwell used 2-

line elements at their Mission Control Center which I generated using

VEC2TLE when USSPACECOM and NASA 2-line elements were not forthcoming in a

timely manner.

In addition to simply converting state vectors to 2-line elements,

the primary purpose of the program, VEC2TLE may be used in conjunction with

STSPLUS or other sources of state vectors to model orbit adjust burns and

similar maneuvers. STSPLUS is used to generate a state vector at the

midpoint of the burn, the appropriate delta velocities (obtained

independently) are added to the state vector quantities Xdot, Ydot, and

Zdot, then new, post-burn 2-line elements are generated with VEC2TLE. More

complex maneuvers may also be calculated or modeled although these more

complicated exercises are not recommended for the novice. VEC2TLE may also

be used to precess a set of 2-line elements to the next Ascending Node,

required by some software.

My thanks to Ken Ernandes for writing VEC2TLE, for making it available

to the satellite tracking community, and for his assistance in validating

the precision state vector output data from STSPLUS. Thanks also to Willie

Musty (for providing state vectors) and to Joel Runes (for validation and

testing). As with any complex program, considerable effort has been

expended in writing, testing, and documenting the program. If you use

VEC2TLE, please register your copy so as to encourage Ken and others to

continue writing such useful software.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 34

PROGRAM OPERATION

-----------------

STSPLUS automatically checks for the presence of a VGA or EGA and will

execute in color if one is found UNLESS the /M command line option is used

to force monochrome operation. However, if you wish to operate STSPLUS in

the EGA mode when you have a VGA monitor, you must use the /EGA option. In

cases where a monochrome monitor is connected to an adapter which simulates

color with gray scale, the /M command line option may be omitted but the

various portions of the display may or may not be visible. The EGA mode is

generally not correctly emulated with VGA LCD displays and the display may

appear compressed vertically or have multiple scan lines repeated.

STSPLUS depends upon the Microsoft VBDOS BASIC Compiler to determine

whether or not a particular monitor type is available. Some video adapter

boards which claim to be VGA are not recognized as such by BASIC and

therefore cannot be used in the higher resolution VGA display mode.

Similarly, "clone" Hercules Graphics Cards do not always perform correctly.

The /M option is not required for HGC and CGA graphics operation,

since those adapters always render their "high resolution" graphics in

monochrome. Although color CGA systems do have a 3-color mode, the limited

number of colors and coarse resolution of 320x200 is not suitable for

STSPLUS. Naturally, the appearance of the program is enhanced by the use of

color. The vertical resolution is also adjusted depending upon the type of

adapter which has been detected. Microsoft does not support SVGA adapter

cards in their VBDOS BASIC Complier, unfortunately. Once STSPLUS has

started, the display type may not be changed without exiting the program at

the Main Menu with the ESC key, then restarting the program with the

desired command line options.

The program checks for the presence of a math coprocessor and will use

it if found. Since the calculations required to determine orbital data are

very complex, the use of a math coprocessor will improve the speed of

operation by a very substantial amount. Users without a math coprocessor

are warned at program startup that performance may be reduced.

SATELLITE ICON SELECTIONS

-------------------------

STSPLUS selects an internal icon or symbol used to graphically

represent the satellite based upon the satellite name or NORAD Number.

Three internal icons are available in two sizes each: for satellite names

which start with the letters "STS" STSPLUS will use an icon resembling a

plan view of the Space Shuttle; for the Mir Space Station (NORAD Number

#16609) STSPLUS will use a special icon resembling that space station; and,

for all other missions STSPLUS will use an icon resembling the Hubble Space

Telescope. The standard internal icons are 15x15 pixels and are suitable

for most applications. Including the "/21" command line option will select

alternate 21x21 pixel icons which may be more appropriate for television

and projection display formats. STSPLUS also checks for a file with the

name "#nnnnn.ICN", where "nnnnn" is a 5-digit NORAD Number, and if present

uses that icon file for the specified satellite; for example, the icon file

"#16609.ICN" will be used in place of the internal icons for the Mir Space

Station. Icon files may be created and modified using the program STSICON,

available with program registration only.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 35

MAP DATABASE FILE SELECTIONS

----------------------------

STSPLUS reads the map coordinates from the appropriate map database

files. Map database files are selected automatically according to the ZOOM

or MAG factor currently in use. Lowest detail maps (world or hemisphere

views) ALWAYS use EARTH4 map database files; higher detail maps will select

higher detail map database files (EARTH3 through EARTH1) automatically if

present. The EARTH3 Map Database Files are included with program

registration; EARTH2 and EARTH1 Map Database Files are available by mail

only for an additional donation. For most applications, the EARTH4 and

EARTH3 Map Database Files will be sufficient.

The map coordinates are automatically converted to screen coordinates

for the type of monitor detected or specified by command line option, the

current projection method (rectangular or orthographic), and the current

magnification or zoom factor. The use of high magnification maps with slow

computers may result in constant redrawing of screen maps for fast moving

low Earth orbit satellites.

STSORBIT PLUS Title Screen

--------------------------

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Current time:  19:01:32 PDT   02:01:32 UTC

Current date:   15 APR 1995    16 APR 1995

Last Mission = Mir

2-Line Elements File = C:\STSPLUS\TLE431.TXT

CPU Type = 80486DX+ or 80486SX

NDP Type = 80486DX+ or 80487SX

Display = VGA Color

(C) Copyright David H. Ransom, Jr., 1989-1995

All rights reserved.

Once the initial "housekeeping" chores have been performed, the Title

Screen is displayed for 15 seconds for normal program operation or for 3

seconds if the "/R" command line option has been used. The Main Menu,

described in a subsequent section, is then displayed. Press ENTER to

proceed to the Main Menu immediately.

The Title Screen displays the program version, current time and date,

last mission, and 2-line elements path and filename. It also displays the

type of Central Processing Unit (CPU) and Numeric Data Processor (NDP, or

math coprocessor), and the type of display. The program detects 8088,

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 36

80286, 80386, and 80486 processors and the associated math coprocessor; if

the math coprocessor is not present or fails a simple test, it will show as

"(not installed)". The program detects CGA, HGC, EGA and VGA display

systems; unless the "/M" command line option is used to force monochrome

operation, EGA and VGA systems will always indicate "Color".

STSPLUS is "aware" of program RighTime by Tom Becker. If RighTime is

active, the current version number will be displayed; if RighTime is not

active or is not detected, no message will be displayed. STSPLUS is

configured to use RighTime Version 2.5+; performance with prior versions

may be unpredictable and audible alarms should NOT be enabled in that case.

See the section "Accurate Time and the Personal Computer" for a further

discussion of RighTime and other aspects of maintaining accurate DOS time.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 37

STSORBIT PLUS SATELLITE TRACKING FEATURES

-----------------------------------------

The principal objective of STSPLUS is to graphically display the

position of the space shuttle or satellite relative to a map of the world

or some relevant portion of the world along with relevant time and

numerical data. Two map projections and six different map displays are

available: Orthographic, World, Quadrant, Zoom, Location, Tracking Station,

and Satellite Motion. Varying magnifications or zoom factors are available

in most map modes. Each is discussed below.

In addition to the map itself, a number of other items of interest are

displayed. Some features are available only with higher resolution displays

(EGA and VGA) in order to avoid cluttering the display screen. Other

features may be enabled or disabled according to the user's preference. The

sections which follow the map types discuss these various features.

Orthographic Projection Maps

----------------------------

The orthographic projection views the Earth as a sphere as if from a

great distance (perspective is not included) and is the latest addition to

the map projections available in STSPLUS. This projection has the advantage

that the map may be centered at any point on the Earth and may include a

pole, especially helpful for high inclination satellite orbits. Unlike the

world map shown with rectangular projection, only one hemisphere can be

seen at a given time and therefore automatic map redrawing is always

enabled. Because of the more complex calculations required to generate a

map, users with slower computers may find that drawing times in the

orthographic modes are unacceptably long. (A math coprocessor will improve

map drawing times by almost a factor of ten!)

Orbital ground tracks, especially for high inclination orbits, and the

solar terminator are more readily understood using this projection. The

orthographic projection displays circles of visibility as true circles near

the center of the map.

The default magnification for orthographic maps is 100% which displays

the entire globe as a hemisphere. Using the PgUp and PgDn keys, the

magnification may be selected from 100% to 4000% (2000% if Level 1 maps are

not present). Each time the map is drawn, the center of the map is selected

so that the satellite will remain on the map for the longest time

practical. When high magnification factors are selected and the computer is

not equipped with a math coprocessor, it is possible that the map drawing

time will exceed the time the satellite is in view; this will cause the map

to be immediately redrawn. The current map database file and map drawing

time are shown near the bottom of the data block (e.g. "EARTH4   10.91").

Because only one hemisphere is shown (or a portion of a hemisphere

when magnification factors greater than 100% are used), automatic map

generation is ALWAYS enabled in orthographic modes. In addition, even if

the satellite never leaves the current map (as is the case with

geosynchronous satellites), the map will be redrawn every 2.5 hours. Users

without a math coprocessor may find that map drawing times in orthographic

modes are painfully slow. The only remedy is to purchase a new, more

powerful computer or to add a math coprocessor chip; this will improve

performance by about a factor of ten and the math coprocessor chips are now

relatively inexpensive, often under $100.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 38

The orthographic version of the Satellite Motion Map, enabled with the

"M" key when the map is displayed on the screen (EGA and VGA systems ONLY)

centers the satellite on the map and "moves" the map beneath the satellite

using EGA graphics. The next map is started in offscreen memory as soon as

a map is completed and displayed, then that map is displayed when completed

and the process is repeated. Especially for slower computers, this map mode

may be preferred since a map is always on the screen (once the first map is

drawn) regardless of the drawing time required.

As a footnote, the orthographic version of the Satellite Motion Map

can duplicate many of the views presented on NASA Select TV during a

mission. NASA frequently uses the equivalent of MAG=150 or MAG=200 for

their display. STSPLUS, however, can display far greater map detail than

can the NASA program, especially when the Level 3 and Level 2 map database

files are present.

Rectangular Projection World Maps

---------------------------------

The STSPLUS rectangular projection (similar to Mercator projection)

ground track display defaults to a map of the world centered on the Prime

Meridian (0 degrees) and extending from approximately +85 degrees North

latitude to -85 degrees South latitude using a linear cylindrical

projection. Omitting the two 5 degree bands at the poles permits better

detail in the mid latitudes where all space shuttle orbits and many other

satellite orbits are concentrated and avoids the extreme distortion

inherent in the rectangular projection near the poles. Ground track details

very near the poles are therefore sacrificed for a better display in the

main portion of typical orbits. The vertical resolution of the display is

automatically adjusted for the type of display system in use from 200 lines

(CGA) to 480 lines (VGA). Two World Maps are available: one centered on the

Prime Meridian at Greenwich, England (0 degrees longitude); and, one

centered on the International Date Line (180 degrees longitude).

All screen maps drawn by STSPLUS use a vector database derived from

the Micro World Database II. The full map of the world as used here

can include up to approximately 470,000 sets of vector coordinates

describing the world's coastlines, islands, lakes, and major rivers when

used with the highest detail (Level 1) map database.

Pressing the "W" or "0" key will switch the display to the World map.

If automatic map generation is enabled, STSPLUS will select the map which

most nearly centers the satellite on the map. If automatic map generation

is disabled, pressing "W" or "0" will toggle between the two maps. When

automatic map generation is enabled, the letter "A" appears in the upper

right of the screen.

Rectangular Projection Quadrant Maps

------------------------------------

The original STSORBIT program used a digitized pixel map of the world

derived from an EGA display. As a consequence of the EGA source, boundaries

were sometimes discontinuous on VGA displays and the display on a CGA was

sometimes difficult to read. STSPLUS uses a vector map drawing method which

automatically adjusts to the display type and the scale of the map. The full

world map (above) is quite similar in appearance to the original display.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 39

However, some geographic details are still difficult to distinguish,

even on a VGA display. STSPLUS includes twelve quadrant maps, each overing

1/4 of the Earth's surface and using rectangular projection. These are

numbered 1 to 12 and are centered on the world map roughly according to the

following illustrations:

0                                180

+---------------------------+     +---------------------------+

|             |             |     |             |             |

|    1        4        7    |     |    7        10       1    |

|             |             |     |             |             |

|             |             |     |             |             |

|             |             |     |             |             |

|    2        5        8    |     |    8        11       2    |

|             |             |     |             |             |

|             |             |     |             |             |

|             |             |     |             |             |

|    3        6        9    |     |    9        12       3    |

|             |             |     |             |             |

+---------------------------+     +---------------------------+

The center vertical quadrants in the left illustration, 4 through 6, are

centered on the Prime Meridian at zero degrees longitude (Greenwich,

England). The center vertical quadrants in the right illustration, 10

through 12, are centered on the International Date Line at 180/-180

degrees. The center horizontal quadrants, 2, 5, 8, and 11, are centered on

the Equator.

Each individual quadrant map may be selected by pressing the

corresponding number key, "1" through "9"; use keys "!", "@", and "#" to

select quadrants 10, 11, and 12 respectively. Pressing any of these keys

for individual quadrant maps will disable automatic map generation if it is

enabled (indicated by the letter "A" at the upper right of the screen).

Pressing "Q" will allow STSPLUS to select the quadrant most appropriate for

the satellite's current position.

Rectangular Projection Zoom Maps

--------------------------------

Although I was pleased with the enhanced maps using the Quadrant Mode,

the map data base files contain far more information than can be

effectively displayed in that mode. The next obvious step was to add the

ZOOM feature, maps which yielded greater detail and which spanned as little

as 30 degrees across the screen, six times better than the 180 degree

quadrant maps. This approaches the practical limit for the map database

files. Because of the smaller area covered, a different approach was used

for map selection. There would simply be too many different possibilities

for manual selection so a fully automatic Zoom Mode was implemented which

calculates the optimum map center point based upon the current position of

the satellite.

Press the "Z" key to enable Zoom Mode. The initial map width is 75

degrees; use PgUp to widen the map width (up to 180 degrees) or PgDn to

narrow the map width (down to 30 degrees. The Home key will always select

75 degrees width and the End key will return to the prior field of view.

The width of the map is shown at the upper left of the map display. Press

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 40

the TAB key to enable or disable automatic map generation (the map will

always be redrawn). When automatic map generation is enabled, the letter

"A" appears in the upper right corner of the screen next to the map width

or field of view. Zoom field of view selections are 30, 45, 60, 75, 90,

120, and 180 degrees.

Location Maps with Isocontours

------------------------------

By popular request, especially from the amateur radio community, I

have added the Location Map with Isocontours. (Isocontours is a term coined

by Rob Matson for his SkyMap program and for which he generously supplied

sample code which I adapted for STSPLUS.) Press the "L" key when the map

display is present to select this display. The map will be drawn with the

current magnification/zoom factor and centered on the user's location. The

usual circle of visibility will be drawn and within that "circle" are seven

isocontours representing viewing angles of 10 through 70 degrees in ten

degree increments (five degree increments at maximum magnification

factors). The projection, orthographic or rectangular, used for the

Location Map is the projection in use when the "L" key is pressed. The

balance of the Location Map includes the usual features.

If you have entered a second location (using F10+F2 from the Main

Menu), pressing the "L" key when the Location Map is already displayed will

toggle between your primary location and the second location. The data

related to your location (Location, Altitude, Azimuth, etc.) is calculated

with respect to the indicated location. If no second location has been

entered, pressing the "L" key while the Location Map is displayed will have

no effect.

The principal advantage of the Location Map is, of course, the

isocontours -- lines of equal viewing altitude (line of sight not taking

into account any refraction near the horizon) from the user's own location.

The user can immediately tell by inspection whether current or upcoming

passes will be "good" and what approximate maximum satellite viewing

altitude can be expected. Amateur radio buffs who need to know if a pass

will appear above some altitude threshold, say 20 degrees, now have that

information available visually.

Since this map mode uses the ZOOM map algorithms, the usual zoom map

features (PgUp, PgDn, Home, End) are active.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 41

Tracking Station Maps with Isocontours

--------------------------------------

The Tracking Station Maps with Isocontours are similar to the Location

Maps except that they use the current TRACKING STATION file locations

rather than the user's location(s). This feature was implemented at the

request of folks working on the STS-49 Intelsat Reboost Mission. STSPLUS

was used operationally by INTELSAT during this mission at their Launch

Control Center near Washington, DC, and at their five ground tracking

stations around the world. I subsequently received a letter thanking me for

the use of the program and saying that it was "critical to mission

success".

Pressing the "T" key will select this map mode. STSPLUS calculates

which of the available tracking stations is nearest to the current

satellite position and centers that tracking station on the screen. This is

calculated by determining the angular difference between the sub-satellite

point and each tracking station. However, this means that depending upon

the Zoom factor in effect, the satellite may or may not be visible on the

screen. For example, if the sub-satellite point is in South America and the

only tracking station in the Western Hemisphere is in the United States,

the satellite cannot be seen at narrower fields of view.

The "tracking stations" may be any locations the user chooses and

includes in the current TRACKING STATION file. Several different tracking

station files accompany the normal STSPLUS distribution as described in the

section "NASA Ground Tracking Stations" below. Use Function Key F7 from the

Main Menu to select the desired file. Tracking Station files are identified

by the filetype ".TRK" and may contain 50 locations maximum; if the current

Tracking Station file contains more than 50 entries, only the first 50 will

be used. STSPLUS contains an internal list of NASA tracking stations which

will be used if the current tracking station file cannot be found. The file

is standard ASCII "comma-delimited" data with place names and abbreviations

delimited with double quotation marks. The following format is used for all

Tracking Station (".TRK") files:

"Maui, Hawaii",-156.7,20.9,396,"HAW"

------+-----  ---+-- --+- -+-  -+-

|          |     |   |    |

|          |     |   |    +---  Three-letter Abbreviation

|          |     |   |

|          |     |   +--------  Elevation in meters

|          |     |

|          |     +------------  Latitude in degrees

|          |

|          +------------------  Longitude in degrees

|

+-----------------------------  Station Name

Note that Station Names are limited to 20 characters and that Station

Abbreviations are limited to 3 characters. Names or abbreviations longer

than the maximum will be truncated.

Maps in this mode are displayed using the current map projection.

Since this map mode uses the ZOOM map algorithms, the usual zoom map

features (PgUp, PgDn, Home, End) may be used to adjust the field of view

(zoom or magnification).

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 42

Location and Features Labels

----------------------------

Beginning with Version 9240, STSPLUS can add labels for locations and

features to all maps. Enable or disable location and features labels using

Function Keys F10+F3+F9 from the Main Menu. By default, STSPLUS expects the

geographic location and features data to be in file STSPLUS.LOC. If you

have created your own .LOC file or the file is not in the current

directory, use F7 from the Main Menu to specify an alternate file and/or

path. The supplied file includes 350+ locations (cities) and features

(oceans and seas), their coordinates, and certain information required by

the program. The file is standard ASCII "comma-delimited" data with place

names delimited by double quotation marks. The following is a typical data

line in the file:

"London",-.1167,51.5,7,100

---+--  ---+-- --+- + -+-

|       |     |  |  |

|       |     |  |  +---  Minimum magnification to display

|       |     |  |

|       |     |  +------  Color to display label (1 to 15)

|       |     |

|       |     +---------  Latitude in degrees

|       |

|       +---------------  Longitude in degrees

|

+-----------------------  Location or feature name

The label will be displayed if the current magnification is equal to or

greater than the minimum magnification value specified for that label. All

label names are converted to upper case for display; avoid the use of

punctuation other than the period ("."), dash ("-"), or comma (",").

The minimum magnification factors in file STSPLUS.LOC have been

carefully selected so that the display is not too "cluttered" at a given

magnification and so that location or feature names near to each other do

not usually overlap each other. Some care is required when adding new

locations to avoid this problem! If you wish to add your own locations

and/or features, use the supplied STSPLUS.LOC file as a guide and template

for your changes. Use a standard ASCII editor; word processor users must

use the "non-document" mode.

Magnifications are calculated automatically and ranges from 50% to

4000%, depending upon the map mode in effect. In orthographic map mode the

magnification factor is displayed as "MAG". In rectangular map modes, the

displayed ZOOM factor is the approximate field of view and may be converted

to magnification according to the following table:

ZOOM    MAG

-----------

360     50    World maps

180    100    Quadrant and Zoom maps

120    150    )

90    200    )

75    240    ) Zoom maps

60    300    )

45    400    )

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 43

30    600    )

The "color" is a number from 1 to 15 according to the following table:

Black   = 0       Gray      = 8

Blue    = 1       LtBlue    = 9

Green   = 2       LtGreen   = 10

Cyan    = 3       LtCyan    = 11

Red     = 4       LtRed     = 12

Magenta = 5       LtMagenta = 13

Brown   = 6       Yellow    = 14

White   = 7       LtWhite   = 15

The color Black is ignored and is only shown for completeness. If the color

is a NEGATIVE number, the small circle marking the location of the city or

feature will NOT be displayed and the label will be displayed centered on

the coordinates specified. Thus, if the a mountain range, lake, or river is

to be labeled, you may wish to set the color negative.

The latitude and longitude are expressed in degrees and decimal

fractions of a degree; West longitudes and South latitudes must be

negative. Note that many atlases show a number such as "24.45" which is

actually 24 degrees and 45 minutes (NOT 24.45 degrees!) and should be

entered as "24.75" (24 + 45/60). When using multi-line feature labels (see

file STSPLUS.LOC for examples), labels should be spaced approximately one

degree apart in latitude for display at a magnification of 100%, less for

higher magnifications. All labels are automatically centered with respect

to longitude.

IMPORTANT NOTE: The program performs no error checking on the contents of

the location and features file. Avoid the use of punctuation other than the

period or comma. Be sure to use a simple ASCII editor or use your word

processor in the "Non-Document" mode when editing or creating a location

and features file. Illegal characters will be replaced by a SPACE. Very

strange results can appear if the wrong data are present or the wrong

number of items is in a data line!

Big Clock Options

-----------------

In rectangular map projections, STSPLUS defaults to a standard display

with a text block shown on the lower five lines of the display; three

different times are shown at the lower left of this display: Launch/Epoch

date and time, UTC date and time, and local date and time. Pressing

Function Key F2 while the map is displayed switches between this default

mode and three Big Clock modes: UTC date and time, local date and time, and

MET/T+Epoch. Because of display space limitations, big clock options are

NOT available in orthographic projections.

For EGA and VGA users, an additional command line option, "/CLK", is

available which changes the number of lines per screen to 43 and 60 lines

respectively for rectangular projections. The big clocks are then placed

below the standard data instead of replacing a portion of the regular data

area.

In orthographic modes, VGA monitors ONLY, F2 will display the time at

the bottom of the data block at the right of the screen. Note that for all

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 44

magnification factors above 100%, the time is shortened to hours and

minutes because of display space limitations.

Satellite Motion Maps

---------------------

It is sometimes instructive and interesting to see the ground track

from the satellite point of view. The Satellite Motion Map, available

ONLY with EGA and VGA displays, centers the satellite in the display and

draws the map accordingly. Unlike all other map displays, this mode takes

advantage of the dual-page capability of the EGA display and the VGA

display (operating in EGA emulation mode); the current map is always

displayed and the new map is drawn "off screen" and updated as frequently

as the capability of the processor will permit. For VGA users, the vertical

map resolution in this mode is reduced from 400 lines to 280 lines

(rectangular projections) and from 480 lines to 350 lines (orthographic

projection).

Users with slow computers may prefer this display because, once the

map has been drawn the first time (however long that may take), a complete

map is always displayed. This is especially true at higher zoom or

magnification factors where the map is redrawn more frequently.

The Satellite Motion Map is enabled by pressing the "M" key when the

map is displayed. The Motion Map will be displayed using the same

projection as is presently in effect, rectangular (automatically switches

to zoom) or orthographic. When switching to this map mode, the message

Switching to EGA Dual-Page Mode ...

is displayed on the screen while the initial map is being drawn offscreen.

Thereafter, the map is drawn off-screen and will require the "usual" time

during which the screen will be unchanged. (My 386DX systems update every

10 seconds but my 286 systems can only manage every 20 or 30 seconds -- and

they all have math coprocessors!) The following keys are active when the

Satellite Motion Map is displayed:

Home      Zoom=75 (rect) or Mag=100 (ortho)

End       Return to last zoom/mag

PgDn      Decrease field of view (zoom in)

PgUp      Increase field of view (zoom out)

M         Return to normal map mode (rect or ortho)

ENTER     Return to Main Menu

Satellite Position and Orbit Projections

----------------------------------------

The focus of the display, and the reason for program STSPLUS, is to

show the position of the spacecraft or satellite. For the space shuttle

(and provided the mission name begins with the letters "STS"), a symbol has

been chosen which resembles that spacecraft. For all other satellites, a

symbol has been chosen which resembles the Hubble Space Telescope. In

either case, the symbol is shown in the following colors (EGA and VGA

displays only):

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 45

Satellite is sunlit                Bright White

Satellite is in penumbra           Yellow

Satellite in refracted sunlight    Red

Satellite is in umbra              Dim White

In addition, an asterisk ("*") is shown to the right of "Orbit #" when the

satellite is sunlit or in penumbra; this will assist users of monochrome

monitors where colors cannot be distinguished. For better visibility, the

satellite symbol will normally "blink" on CGA monitors; the symbol may be

made to blink on any system if desired by pressing the letter "B" while the

ground track is displayed. Note however that for very slow computers, the

blink may appear erratic if most of the time is spent performing

calculations.

Just seeing the spacecraft or satellite on the map display yields

information as to its present position. However, for satellite viewing and

planning purposes, STSPLUS calculates the predicted ground track for

approximately three hours in the future and the past ground track for

approximately one and a half hours in the past. The ground track may be

selected to display as a series of light green dots (future track) or light

red dots (past track), as a solid light green line, or as a solid light

green line with yellow dots(future track) and light red dots (past track).

The dots are plotted at one minute intervals. To select the desired ground

track display, press Function Key F10 and then press Function Key F5 until

the desired mode is displayed; the available selections are: OFF, DOTS,

LINE, and BOTH.

Note that for satellites in high Earth orbits, the ground track may

appear as a solid line if the dots are very close together. Unlike prior

versions of STSPLUS, the ground track is NOT updated (except to re-color

dots for past track). With automatic map generation off, the map will be

automatically redrawn every 2.5 hours.

Satellite Visibility

--------------------

Satellite visibility, or the ability to see a satellite with the naked

eye (or binoculars for the truly dedicated), attracts the novice and expert

alike. It can be a great thrill to point out the Space Shuttle or MIR Space

Station to a child or a friend as it streaks across the sky where and when

predicted. Unfortunately, predicting that visibility is more complicated

than "simple" orbital mechanics and trigonometry; spacecraft altitude,

position, physical geometry, reflectivity, and attitude with respect to

both the viewer and the Sun, as well as local atmospheric clarity not to

mention weather, all contribute to whether or not a satellite may actually

be seen. Some of these factors are beyond the capabilities of a program

like STSPLUS. All that can be done is to indicate when conditions are such

that the satellite MIGHT be sighted visually. STSPLUS estimates viewer

visibility by calculating that:

a)   The satellite is at least partially lighted by the Sun;

b)   The Sun is 1.7 degrees or more below the viewer's horizon so that

the viewer is in at least partial darkness; and,

c)   The satellite is within the viewer's local circle of visibility

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 46

and is four degrees or more above the horizon.

If all these conditions are met, STSPLUS displays the word "VIS" in the

data block next to the orbit inclination. To these calculations must be

added the uncertainties described above. In my experience, the best

satellite sightings have usually occured when I am between the satellite

and the Sun, enabling flat surfaces such as solar panels to reflect the

sunlight back to me.

Although STSPLUS makes the visibility calculations for any satellite,

as a general rule only satellites in low Earth orbit, say under 1000 km

(600 miles) altitude, are likely to be seen with the naked eye. Binoculars

can extend that range somewhat. Satellites in very high or geosynchronous

orbits can be seen only with precision optical or radar equipment. Last but

not least, current 2-line elements must be used for reliable predictions.

**********

*  NOTE  *

**********

SATELLITE VISIBILITY IS ONLY ACTIVE WHEN THE SOLAR

FEATURES ARE ENABLED WITH F10+F3+F8.

User's Circle of Visibility

---------------------------

Centered around the user's geographic location, and marked with a

small circle on EGA and VGA systems, is a magenta circle of the approximate

line of sight visibility for the mission in progress. For rectangular

projections, the "circle" appears on the display as a circle near the

Equator and as a distorted circle at higher latitudes. In near-polar

regions, the circle takes on a very strange shape. The shape is entirely an

artifact of the map projection; when displayed using the orthographic

projection, it appear as a true circle near the center of the map. The

radius of this circle of visibility is calculated for each satellite based

upon its altitude at the instant the map is first drawn as well as the

user's elevation above mean sea level and corresponds to "line of sight"

visibility for that satellite. When a second location has been selected

using F10+F2, a second circle of visibility will also be drawn for that

location.

When audible alarms are enabled (F10+F8 from the Main Menu), tones

sound 2 minutes before the satellite enters the circle of visibility and

again 30 seconds before the satellite leaves the circle of visibility. Both

the primary and secondary locations are monitored, with slightly different

tones for each. And "up-down" sequence of tones is used prior to entry into

the circles and a series of tones prior to leaving the circles.

When a satellite is within the circle, direct visual, radio or radar

communications with the satellite should be practical. Actual visibility,

of course, depends upon more than simply whether or not the satellite is

above the viewer's horizon. Most important is the sun to satellite to

viewer geometry; the satellite must be in sunlight and the viewer in

darkness for reasonable visibility. Almost as important is the size and

geometry of the satellite itself; a large, bright-metal satellite with huge

solar arrays reflects far more sunlight than a small dark satellite. In

addition to the satellites themselves, many booster rockets and other

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 47

"spare parts" are orbiting the Earth. Since they are not attitude

stabilized, booster rockets often are tumbling and may therefore appear

to flash on and off as they pass over.

For the Space Shuttle as well as most other satellites with near

circular orbits, the circle of visibility calculations are reasonably

accurate; however, the position of the sun and the effect of atmospheric

refraction are not taken into account, only whether or not the satellite is

in line of sight view from the observing location. For highly eliptical

orbits, however, the accuracy is substantially degraded since the radius of

the circle of visibility changes dramatically depending upon whether the

satellite is nearer apogee or perigee at the time the calculation is made,

and the period of the orbit. In the course of a single orbit, the altitude

of such a satellite may change by thousands of miles. Satellite "DE 1",

usually included in the TLEnnn.TXT 2-line elements file, is in a highly

elliptical orbit with long period and illustrates the problem.

Spacecraft Circle of Visibility

-------------------------------

The spacecraft circle of visibility is calculated dynamically using

the same alrorithm as for the user's circle of visibility. Like the user's

circle, the spacecraft circle may appear on rectangular projections as an

odd shape because of the scaling factors used by the map projection. The

difference is that the spacecraft circle moves with the spacecraft and

illustrates the approximate area visible from the spacecraft at any given

moment. The circle is updated every ten seconds (or as frequently as the

processor will permit). Comparisons using a VGA display system during the

STS-35/ASTRO-1 mission in December, 1990 confirmed that the circle shown is

quite close to that shown by one of the special graphics displays

occasionally seen on NASA Select Television as well as the actual horizon

view seen from the payload bay television cameras.

SUN and Solar Features

----------------------

Many types of observations, especially Earth observations, often

require that the target or terrain be in sunlight. The performance of solar

panels and certain other instruments on a spacecraft is dependent upon

whether or not the Sun is in view. Also, it is usually impossible to

visually see a spacecraft which is not in sunlight. The solar terminator is

a series of yellow points on the display which represent the line at which

the center of the Sun is at an observer's horizon for Mean Sea Level.

Although a quick glance at the clock should suffice to determine which side

of the terminator line is in sunlight and which in darkness, EGA and VGA

systems also display the Sun as a small yellow circle.

The terminator as displayed by STSPLUS is sometimes confused with a

line denoting sunrise and sunset. Two factors make the terminator only an

approximation: first, the terminator is based upon the center of the Sun,

while actual sunrise and sunset are calculated using the upper limb of the

Sun; and second, the terminator is calculated for Mean Sea Level rather

than a specific local elevation. These two factors combined can result in a

difference of up to ten minutes when the times are compared against

published values or those calculated by my program ASTROCLK, sunrise being

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 48

earlier and sunset being later.

The Sun and solar features are enabled and disabled using F10+F3+F8

from the Main Menu. Since these features -- especially the terminator --

require some calculation and drawing time, users with slower computers may

wish to disable these features for faster screen updates. The following

features are included:

Sun:           A yellow circle is plotted at the sub-solar point, the

geographic coordinates directly beneath the current position

of the Sun. The position of the Sun is recalculated every 10

seconds and the display is updated every 60 seconds. (Not

shown on CGA and HGC displays.)

Terminator:    A dotted yellow line is plotted for the solar terminator,

that point on the Earth at which the center of the Sun is at

the horizon. The terminator is partially updated every 10

seconds and is fully updated every 60 seconds.The terminator

is shown for Mean Sea Level and does NOT take into account

the non-spherical shape of the Earth.

Lighting:      The current spacecraft lighting is shown using color for the

satellite icon, and in the data block (to the right of

"Orbit #:") and displayed using the following symbols and

colors:

*    Bright White   Full sunlight

+    Yellow         Partial sunlight (penumbra)

-    Light Red      Refracted sunlight

White          Full shadow (umbra), no symbol

Note that Yellow and Light Red will display as Bright White

or shades of gray on monochrome monitors.

Solar lighting conditions are updated every second or as

rapidly as the speed of the processor will permit. In order

to minimize calculation delays during ground track plotting,

the event times for orbital sunrise and sunset are

approximated. The dynamic lighting calculations, used to

plot the color of the spacecraft icon, are more precise.

Typical errors due to the simplified algorithm, are on the

order of 10 seconds.

SUN Timer:     When Event Timers are enabled, the orbital sunrise (AOS) and

sunset (LOS) times are shown. A blank AOS or LOS timer

indicates the event will not occur within the next four

hours. An asterisk ("*") to the left of "SUN" indicates

sunlight is on the satellite:

AOS    LOS

*SUN  73:20  37:40

In this example, the satellite is in sunlight. Orbital

sunset will occur in 37:40 and the next orbital sunrise will

occur in 73:20.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 49

TDRS and Real Time Satellite Features

-------------------------------------

TDRS and Real Time Satellite features are available only on EGA and

VGA monitors. These features permit up to thirty two additional satellites

to be tracked in real time. Satellites to be tracked are designated by the

user as "static" (geosynchronous or geostationary) and "real time"

(satellites whose sub-satellite point changes substantially with time).

Static satellites are only plotted when the map is redrawn; real time

satellites are updated every second with 386/387 or better computers, every

ten seconds for older computers, or as often as the processor can complete

the required calculations. When using X10 or X60, the update occurs at the

display rate (10 or 60 seconds).

When TDRS and Real Time Satellite coverage is enabled (F10+F3+F2), all

active TDRS satellites (Tracking and Data Relay Satellites), used for most

communications to and from the Space Shuttle, the Hubble Space Telescope,

and other active spacecraft, are shown as a dot inside a small circle near

the Equator (provided they have been included in the current TDRS and Real

Time Satellite configuration). Use F6 from the Main Menu to display and/or

modify that configuration. See the text section on Function Key F6 for a

complete description of Static and Real Time satellites.

As of August, 1993, there are five TDRS satellites in geosynchronous

orbit. The primary satellites consist of TDRS East ("TDRS 3") at

approximately 41 degrees West longitude and the TDRS West Cluster

consisting of two satellites, TDRS West ("TDRS 4") and TDRS Spare ("TDRS

1") at approximately 174 and 170 degrees West longitude respectively. The

TDRS Spare satellite has partially failed but is used occasionally as a

backup; this satellite is also low on propellant and is allowed to drift

considerably with an inclination of about 7 degrees. "TDRS 2" has also

partially failed and is currently parked at approximately 62 degrees West

longitude and is presently dedicated to downlinking data from GRO, the

Gamma Ray Observatory, whose tape recorders have failed. "TDRS 5" is the

most recent satellite launched, is fully operational, and is parked at

about 138 degrees West longitude as an on-orbit spare. Users should check

the current positions of the TDRS satellites since they are periodically

moved or reassigned.

Each TDRS location provides communications coverage for almost half

of the Earth for low Earth orbits and essentially full time coverage for

higher orbits. However, since the NASA Ground Terminals are located at

White Sands, New Mexico, the coverage has been slightly overlaped to

provide good ground communications at White Sands. This, in turn, means

that there is a narrow band, known by NASA as the Zone of Exclusion (marked

"ZOE" on the screen), off the East coast of Africa which is not covered by

either primary TDRS for low Earth orbits. Two red "circles" on the display

show the limits of coverage for each primary TDRS satellite. Each circle,

whose shape may be quite distorted when using rectangular projection,

encloses an area where the TDRS satellite is out of range of the primary

satellite being tracked and is centered on the opposite side of the Earth

from the TDRS satellite's position.

While the ground track is being calculated, STSPLUS also calculates

the times for acquisition of signal (AOS) and loss of signal (LOS). When

TDRS coverage is enabled, these times are displayed for TDRS East and TDRS

West. The method used for the calculation of the TDRS coverage is usually

accurate to about 10 seconds (assuming accurate 2-line elements for the

satellite and for the TDRS). However, spacecraft attitude can cause loss of

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 50

TDRS communications at unexpected times. When Event Timers are enabled,

STSPLUS calculates AOS and LOS for four hours (240 minutes) from the time

the map is drawn. If a time is beyond that limit (or if the condition does

not occur), the time is left blank.

Each time is presented in the form of a countdown clock, minutes and

seconds, until the next occurence of the condition:

*TDRE AOS/LOS  72:42  50:42     (for rectangular projections)

*TDRW AOS/LOS  45:42  22:42

AOS    LOS

*TDRE:  72:42  50:42     (for orthographic projections)

*TDRW:  45:42  22:42

An asterisk ("*") is shown to the left of the satellite name if AOS is in

effect. For rectangular projections, these data are displayed in the lower

right portion of the map. For EGA and VGA users, however, the data may be

displayed in the lower section of the data block by pressing F2 while the

map is displayed until MET/T+E is displayed. For orthographic projections,

the data are part of the standard data block at the right of the screen.

The clocks for each TDRS are color coded to indicate the current

status: GREEN when the satellite is in communication, and RED when the

satellite is out of range of the TDRS. Two minutes prior to a change in

status, the appropriate clock color changes to YELLOW. Users with

monochrome monitors must observe the presence or absence of the asterisk to

determine the status.

STSPLUS can sound an audible alarm (three beeps) 30 seconds prior to

TDRS AOS or LOS. Use F10+F8 to enable or disable the audible alarms.

Most satellites which utilize the TDRS system for communications are

in low Earth orbits (generally below 1500 km). However, other satellites

may also use the TDRS system for regular or backup communications. The

NAVSTAR Global Positioning Satellites (GPS), with orbital altitudes of

about 11,000 nautical miles (20,000 kilometers), are an example. For such

high orbits, the coverage by each TDRS satellite is nearly continuous.

As noted above, there are currently five TDRS satellites in orbit as

of mid 1993, TDRS 1 through TDRS 5. (These numbers are those currently used

by US Space Command in their 2-line elements. NASA sometimes uses different

numbers corresponding to the original launch letters as shown in the

following chart. TDRS "B", which should have become "TDRS 2", was lost in

the Challenger accident.) As of July, 1993, the TDRS assignments are:

TDRS#  *   NORAD#     Long    Description

-----------------------------------------------------------------

TDRS 1 (A)   13969    -170W    TDRS West Spare, used occasionally

TDRS 2 (C)   19548     -62W    Dedicated to Gamma Ray Observatory

TDRS 3 (D)   19883     -41W    TDRS East (STSPLUS default)

TDRS 4 (E)   21639    -174W    TDRS West (STSPLUS default)

TDRS 5 (F)   22314    -138W    On-orbit spare

* Original NASA letter designation at launch

STSPLUS will use TDRS 3 and TDRS 4 by default and approximate positions as

of July, 1993 are automatically saved in file STSPLUS.INI. Note that some

2-line element files (including TLEnnn.TXT) often refer to the TDRS

satellites using numbers 1 through 5 or letters A through D or E. Use the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 51

NORAD numbers to be certain that you display the correct satellite.

TDRS satellites do change position and/or assignment periodically for

one reason or another. Users who wish the most accurate TDRS positions and

AOS/LOS data should periodically update the default positions by using the

automatic update feature with F2 from the Main Menu to read and update the

2-line elements.

Ground Tracking Stations and .TRK files

---------------------------------------

NASA maintains a number of ground tracking stations around the world.

Some of these tracking stations are essential for the ascent or landing

phases of a space shuttle flight; others are used for in-flight

communications. File STSPLUS.TRK contains the information for these ground

tracking stations. Other files with filetype .TRK contain the information

for other launch and/or tracking stations.

Each ground tracking station is shown as a small symbol surrounded by

a brown or light yellow "circle of visibility" which gives the approximate

area of antenna coverage and shows how small a proportion of each orbit can

be monitored without the TDRS system. When for some reason the TDRS system

is down (as has occurred during infrequent computer failures at White

Sands, New Mexico, the TDRS Ground Station), these ground tracking stations

become the only means of communication with the Space Shuttle. (Another

unexpected method was demonstrated during a 1992 space shuttle mission when

the SAREX, Shuttle Amateur Radio EXperiment, was used during a complete

loss of normal communications!) Because of budget constraints, many of

these ground tracking stations may be (or have already been) shut down.

Some, such as MIL and BDA (see list below) will be retained because they

are required for the ascent phase of a space shuttle mission. Others, such

as HAW, CTS and GWM, are operated jointly with, or independently by, the

U.S. Air Force. NASA is not always consistent as to the abbreviations used

for these tracking stations; on NASA Select TV, Gwan, Hawaii, and

Vandenbert are usually shown as GTS, HTS, and VTS respectively.

When using rectangular map projections, the shape of the antenna range

"circle of visibility" varies as a function of the latitude and is an

artifact of the map projection; when projected on a sphere, as is the case

with orthographic projection, they are true circles. In order to avoid

cluttering the display with countless meaningless lines, tracking station

circles of visibility are shown only if that circle has an angular diameter

of 90 degrees or less.

The following table lists the internal ground tracking stations as of

early 1989 which are shown along with their abbreviations and approximate

map coordinates (longitude, latitude):

MIL     -81,28         Merritt Island, FL

BDA     -64,32         Bermuda

DKR     -17,14         Dakar, Senegal

ACN     -14,-8         Ascension Island

MAD     -5,41          Madrid, Spain

IOS      56,-5         Indian Ocean

HAW     -156,20        Hawaii

GWM      143.33,14     Guam

VAN     -120.57,34.73  Vandenberg, CA

YAR      115,-29       Yarragadee, Australia

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 52

CAN      149,-36       Canberra, Australia

GDS     -116.88,35.93  Goldstone, CA

CTS     -105,38        Colorado Springs, CO

AGO     -71,-34        Santiago, Chile

NGT     -106,33        White Sands, NM

STSPLUS first checks for the presence of file STSPLUS.TRK for its

ground tracking station information. This is the default TRACKING STATION

filename used if no other selection has been made using F7 from the Main

Menu. If that file is present, its data is used instead of the internal

data above. The supplied STSPLUS.TRK has the following format:

"Maui, Hawaii",-156.7,20.9,0,"HAW"

"Vandenberg, CA",-120.5667,34.7333,112,"VAN"

"White Sands, NM",-106,33,0,"NGT"

"Colorado Springs, CO",-105,38,0,"CTS"

"Merritt Island, FL",-81,28,0,"MIL"

"Santiago, Chile",-71,-34,0,"AGO"

"Bermuda",-64,32,0,"BDA"

"Dakar, Senegal",-17,14,0,"DKR"

"Ascension Island",-14,-8,0,"ACN"

"Madrid, Spain",-5,41,0,"MAD"

"Indian Ocean Stn",56,-5,0,"IOS"

"Yarragadee, Australia",115,-29,0,"YAR"

"Guam",143.3333,14,0,"GWM"

"Canberra, Australia",149,-36,0,"CAN"

These files are ASCII and may be prepared or edited with any standard

ASCII editor; if using a word processor, select the "non-document" mode.

Five items are required for each location; the longitude and latatude are

expressed in degrees and fraction of a degree, elevations above Mean Sea

Level are expressed in meters, and names or abbreviations are included in

double quotation marks. The following example illustrates the .TRK file

format:

"Merritt Island, FL",-81.0,28.0,0,"MIL"

--------+----------- --+-- --+- + --+--

|              |     |  |   |

|              |     |  |   +--- 3-Letter Abbreviation

|              |     |  +------- Elevation (meters)

|              |     +---------- Latitude (degrees)

|              +---------------- Longitude (degrees)

+------------------------------- Location Name

For those interested in the Russian space program, a list of Russian

ground tracking stations is provided in file CIS.TRK (data courtesy

Ellwood Marshall). With the breakup of the Soviet Union, some of these

installations may no longer be active or the name may have changed.

"Tyuratam Cosmodrome",63.3392,45.9235,0,"TYR"

"Kaliningrad Cntrl Ctr",37.816,55.916,0,"KAL"

"Plesetsk Cosmodrome",40.7,62.75,0,"PLS"

"Petropavlovsk Russia",158.933,53.216,0,"PTR"

"Tbilisi Georgia",44.75,41.66,0,"TBL"

"Ulan Ude Russia",107.683,51.983,0,"ULN"

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 53

"Ussuriysk Russia",132.15,43.8,0,"USS"

"Yevpatoria Ukraine",33.3666,45.2166,0,"YEV"

Other nations also have facilities for satellite launches. As of early

1993, file SPACENTR.TRK includes the following locations:

"Alcantara LC Brazil",-44.3999,-2.3999,0,"ALC"

"Esrange,Kiruna Sweden",21.067,67.883,0,"ESR"

"Jiuquan Space LC China",100.033,40.83,0,"JIU"

"Kagoshima Center Japan",131.083,31.25,0,"KAG"

"Kourou Space Ctr Fr.Gu",-52.7669,5.23,0,"KOU"

"San Marco Platform",40.2,-2.9329,0,"SMP"

"Sriharikota Ctr India",80.25,13.78,0,"SRI"

"Tanegashima SC Japan",130.967,30.4,0,"TAN"

"Xichang Space LC China",102.217,27.967,0,"XUC"

The first mission of Endeavour, STS-49, was in May, 1992. This

dramatic and exciting mission captured the INTELSAT VI (F3) satellite,

stranded in a useless orbit by its booster rocket failure since January,

1990, and attached a new booster rocket which placed the satellite in its

proper orbit. STSPLUS was used operationally during the mission by

Intelsat, another "first" for the program. Intelsat used its own ground

tracking stations for communications with INTELSAT VI (F3); the ground

stations which participated in the mission are listed in file INTELSAT.TRK

(information courtesy Dee Smith):

"Paumalu, Hawaii",-158.0342,21.6711,157.86,"PAU"

"Tangua, Brazil",-42.7845,-22.7442,35.38,"TAN"

"Jatiluhur, Indonesia",107,-6.5213,161.49,"JAT"

"Perth, Australia",115.25,-31.8,0,"PER"

"Gandoul, Senegal",-17.4745,14.43,0,"GAN"

These TRK files are standard ASCII files and may be edited with any

editor; word processor users be sure to use the ASCII or non-document mode.

The files use a standard comma-delimited format as shown; positions are

given in longitude (degrees) and latitude (degrees), rounded to the nearest

degree. A maximum of 25 ground stations is permitted. The use of TRK files

is not restricted to tracking stations, of course. So long as the correct

data format is observed, any desired location may be included in the

tracking station file up to the maximum of 25 locations.

Event Timers and Audible Alarms

-------------------------------

STSPLUS is often operated for long periods of time with minimum

operator attention or intervention. Users may perform other tasks while the

satellite display is active and while awaiting some subsequent event of

interest. Event timers are displayed for the selected events if they will

occur within approximately four hours of the time that the map is drawn; if

an event will not occur within that time, the event is blank. Audible

alarms serve to alert the user to upcoming selected events. The event

timers are enabled with F10+F7 from the Main Menu. Audible alarms are

enabled with F10+F8 from the Main Menu and require also that the event

timers be enabled. All events are termed "AOS" (Acquisition of Signal or

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 54

Source) or "LOS" (Loss of Signal or Source) and are generally calculated

for line of sight conditions.

Each phenomena which may be timed has an associated AOS and LOS timer

which displays the hours and minutes ("HHH:MM") remaining until the next

event if that event will occur within the next four hours (240 minutes),

one hour past the last time for which the ground track is plotted. The

current status of the signal or source is indicated by the color of the

timer digits and the presence or absence of an asterisk ("*") to the left

of the event name: GREEN indicates signal or source acquisition, and RED

indicates signal or source loss. Two minutes prior to an event, the timer

for that event will change from the signal status color (GREEN or RED) to

YELLOW to visually alert the user. Users with monochrome monitors will be

unable to distinguish these color changes, of course, but can determine the

current signal status using the asterisk indicator.

All calculations for upcoming events are made each time the ground

track is drawn on the display and will affect the time required to prepare

the display, especially on slower computers. In order to minimize these

calculation delays, the event calculations for orbital sunrise and sunset

use a simplified algorithm which does not take into account the non-

spherical shape of the earth (unlike the dynamically calculated spacecraft

lighting features which are more accurate). Orbital sunrise and sunset are

the times that the spacecraft transitions between refracted sunlight

(sunlight refracted through the Earth's atmosphere) to partial sunlight

(illumination from only a portion of the solar disk); this corresponds to

the transitions between RED and YELLOW color on the satellite icon and

illumination symbols respectively. The errors resulting from the simplified

algorithm are usually less than plus or minus 10 seconds; because of the

more oblique angles and geometry involved, higher errors are usually

associated with higher inclination orbits.

The default mode for event timers is countdown clocks in minutes and

seconds (maximum of about 240 minutes, the time period that the program

searches forward for events). Function Key F12 may be used to cycle through

four different event time units: countdown clocks; UTC (GMT) in hours and

minutes; local time in hours and minutes; and, MET in hours and minutes. If

no launch time has been set, MET AOS and LOS times will NOT be available.

The appropriate time units label will be displayed for the last three

modes; no label is displayed for the countdown clock mode. A blank item

indicates that the event does not occur within the next four hours.

When no secondary location is selected, the headings "AOS" and "LOS"

will appear in orthographic modes; the headings do not appear in

rectangular modes or in orthographic modes when a secondary location is

selected in order to make room for the additional line of data. Typical

Event Timers are shown in the following examples.

For orthographic projections using the countdown clock mode:

*STN   95:15   6:21           AOS now in effect

AOS will next occur in 95:15

LOS will occur in 6:21

STN   23:47  45:18           LOS now in effect

AOS will occur in 23:47

LOS will next occur in 45:18

and similarly for rectangular projections:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 55

*TDRW AOS/LOS  85:14  33:43   AOS now in effect

AOS will next occur in 85:14

LOS will occur in 33:43

TDRW AOS/LOS  14:21  57:32   LOS now in effect

AOS will occur in 14:21

LOS will next occur in 57:32

except the primary location AOS/LOS, which is unlabeled in rectangular

projections and appears at the upper right of the data block (immediately

to the right of MET/T+E):

* 89:39   1:27                AOS now in effect

AOS will next occur in 89:39

LOS will occur in 1:27

70:15  76:38                LOS now in effect

AOS will occur in 70:15

LOS will next occur in 76:38

Note the use of the asterisk ("*") to mark AOS in the examples above.

The following events may be timed and will cause an audible alarm if

audible alarms are enabled and the appropriate events are enabled:

Local Visibility    For the primary location ("STN" or "STN1"): three

sets of up/down tones two minutes prior to AOS and

five tones thirty seconds prior to LOS. For the

secondary location ("STN2" if enabled): two sets

of up/down tones two minutes prior to AOS and four

tones thirty seconds prior to LOS. Refers to the

times the satellite enters or leaves the local

circle of visibility.

TDRS Acquisition    ("TDRE" or "TDRW" if enabled) Three short tones

thirty seconds prior to AOS or LOS. Refers to the

times the satellite acquires or loses the ability

to communicate with either of the programmed

Tracking and Data Relay Satellites.

Orbital Sunrise/set ("SUN" if enabled) Two tones thirty seconds prior

to approximate orbital sunrise or sunset. Refers

to actual line of sight solar contact; refracted

sunlight is not included.

The characteristics of the audible tones have been selected to allow the

user to uniquely identify the AOS or LOS event that is about to happen.

STSPLUS is now "aware" of program RighTime by Tom Becker and its use

is recommended for accurate timekeeping. Audible alarms in prior versions

would perform unpredictably when RighTime was active because they use the

hardware clock's timer functions (which RighTime also uses). STSPLUS now

detects RighTime and temporarily disables RighTime while an audible alarm

is being generated and then re-enables RighTime after the alarm has

completed, restoring precise timekeeping. With RighTime active, alarms are

generated in foreground, which may cause a slight delay in screen updating.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 56

*************

*  CAUTION  *

*************

STSPLUS expects RighTime Version 2.5+; performance with prior

versions of RighTime may yield unpredictable results. If using a

prior version of RighTime, do NOT enable audible alarms!

If RighTime is not present or is not detected, the audible alarms are

generated in background as in prior versions. This usually causes the loss

of several clock ticks in the DOS software clock for each audible alarm.

Although the time loss per audible alarm is very small, the cumulative

error may become significant over extended time periods.

Pausing the Ground Track Display

--------------------------------

Pressing F6 will cause the ground track display to "freeze" at the

current time. This is called PAUSE mode. This permits closer examination of

the data and/or display at any given time and to "move" the display forward

and backward in time. Note that the pause takes effect AFTER the next

second tick on the system clock; thus, if you wish to pause at 01:00 (one

minute exactly on one of the clocks), press F5 when the display reads

00:59.

After pressing F6, the following message will appear near the lower

right of the screen:

PAUSE...Press ENTER

This reminds the user that PAUSE is in effect and to press ENTER to

resume normal operation. When normal operation is resumed, the time

continues from its present value, as if you had set SIMULATED TIME using

F8+F3 or F8+F4 from the Main Menu. To return to REAL TIME, use F8+F1 from

the Main Menu.

While PAUSE is in effect, the "+" and "-" keys may be used to advance

or retard the current time by the amount of the current time step. You may

also use the "=" key instead of the "+" key to avoid pressing the SHIFT

key. The only other key active in PAUSE mode is F4, which may be used

to adjust the "time step" by pressing the key until the desired time

multiplier is displayed at the upper right of the screen. Time multipliers

of "X1" (no message displayed), "X10", and "X60" are selected in

succession. The default time step is 1 second.

Note that the automatic map generation feature is also used in PAUSE

mode; automatic map generation may be enabled or disabled in the

rectangular projections (use the TAB key) and is ALWAYS enabled in

orthographic projections. Thus, when the satellite is moved near the edge

of the display, the map may be redrawn if the appropriate point is reached.

If you wish to synchronize the time used by STSPLUS to some other

source (such as the slightly delayed orthographic displays presented from

time to time on NASA Select TV), you may also use PAUSE mode for this

purpose. Simply pause the display, use the "+" or "-" keys to adjust the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 57

time slightly ahead of the time to which you wish to synchronize. Then

press ENTER when the times agree. Time can only be synchronized in this

manner to plus or minus one second. If you need higher precision, set

SIMULATED TIME using F8+F3 or F8+F4 from the Main Menu.

Switching between MET and T+Epoch

---------------------------------

STSPLUS by default displays the time elapsed since the epoch date of

the elements in the upper right portion of the data block unless the launch

date and time are included in file STSPLUS.LTD, in which case Mission

Elapsed Time (MET) is the default. This marked on the display as "T+Epoch"

or "T+E" and "MET" respectively. While T+Epoch is not of particular value

for satellite viewing purposes, it does indicate the relative age of the

orbital data. As a general rule, especially for lower Earth orbits, the

effects of orbit decay make position predictions less accurate as time

passes. Data which are more than 10 or 20 days old may produce less

accurate positions.

For a space shuttle mission, however, all mission events are scheduled

against the mission timeline and are reckoned in Mission Elapsed Time

(MET), the time elapsed since launch. It is therefore useful to be able to

display MET during the course of a mission or to review the flight post-

mission. Unfortunately, the standard NASA/NORAD 2-line element format does

not include the launch time and launch date and therefore this information

must be secured independently and manually entered into STSPLUS. Once

entered, STSPLUS saves the information in file STPLUS.LTD.

Once the ground track map is displayed, the F5 command may be used to

switch the display between Time Since Epoch ("T+Epoch" or "T+E") and

Mission Elapsed Time ("MET").

The F5 command checks that you have already entered the launch time

and launch date or that it has been read from file STSPLUS.LTD. If no

launch time and date are present, the command will have no effect. Press

ENTER to return to the Main Menu and press F5 to enter launch time and

launch date.

Using FAST Time

---------------

Press F4 while the map is displayed to use FAST time. FAST TIME is a

variation of SIMULATED TIME which automatically advances the time displayed

by 10 or 60 seconds, as indicated at the upper left of the screen. Press F4

again to change the time step; when "(X10)" or "(X60)" is NOT displayed,

the time step is one second. The actual time increment is a function of the

computer's speed. For fast computers, the time increment will usually be 10

or 60 seconds but may vary by a second occasionally; for slower computers,

the time increment may be somewhat longer. FAST time is disabled when PAUSE

mode is in effect and for the Satellite Motion Map.

FAST time may be used to advance the display to a future time and for

demonstration purposes. Once the desired time has been reached, simply

press F4 until no time step is displayed at the upper left of the screen

and time will advance normally. Note that FAST time sets the program to

SIMULATED TIME; to restore "real time", press F8+F1 from the Main Menu. The

Main Menu also indicates when SIMULATED TIME is in effect: the words

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 58

"Current Time" will appear at the left of the times at the top of the

screen when real time (the time your DOS clock is using) is in effect; the

words "Simulated Time" appear when a simulated time is in effect.

On-line Help

------------

An on-line Help Screen is available during the ground track display

to remind the user of the more commonly used functions and which keys to

press to enable those functions. When the ground track display is active,

press Function Key F1 to display the Help Screen. The following help screen

will appear in rectangular modes in the lower portion of the screen:

F1=Resume Data   F6=Pause (+,-)    TAB=Auto Maps On/Off  L=Location Maps

F2=Select Clocks F7=Circle of Vis    W=World Maps        T=Tracking Maps

F3=Printer Log                       Q=Quadrant Maps     M=Motion Map

F4=Time Step     F9=Dist:km/nm/sm    Z=Zoom Maps:        O=Orthographic

F5=MET/T+Epoch  F10=Coords/Target      Home,PgUp,PgDn    STSPLUS Ver 9537

The help screen in orthographic modes is similar but in a vertical format

at the right side of the screen:

F1=Resume Data

F2=Select Clocks

F3=Printer Log

F4=Time Step

F5=MET/T+Epoch

F6=Pause (+,-)

F7=Circle of Vis

F8=FRQ : AOS/LOS

F9=Dist:km/nm/sm

F10=Coords/Target

W=World Maps

L=Location Maps

T=Tracking Maps

M=Motion Map

Change Zoom:

Home,End,PgUp,PgDn

ENTER = Main Menu

Note that "F8=FRQ : AOS/LOS" (for Doppler shift frequency data) is not

available in rectangular projections, and the TAB key is not used in the

orthographic projections. Not shown because of space limitarions in the

orthographic help screen are "Q=Quadrant Maps" and "Z=Zoom Maps". See the

section ACTIVE KEYS DURING GROUND TRACK DISPLAY for additional information

on keys which are active and available when the ground track map is active.

The ground track display will continue to be updated in real time

while the help screen is displayed. If only the graphical display of the

ground track is of interest, the help screen may be kept on the display

continuously. Press Function Key F1 again to return to the normal data

display in the lower portion of the screen.

NOTE: The Help Screen is DISABLED when the Motion Map is displayed.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 59

Launch Simulation for the Space Shuttle

---------------------------------------

STSPLUS can generate launch simulation for Space Shuttle launches from

Cape Canaveral, Florida. Prior to Version 9509, STSPLUS used only 2-Line

Elements ("TLEs") and assumed that whatever satellite was being displayed

"had always been in orbit" and that the current position could be

accurately calculated from the TLEs, not quite in line with the typical

situation for a Space Shuttle launch! Many STSPLUS viewers were confused by

the fact that prior to launch, the orbiter appeared to be off the coast of

Baja California rather than sitting quietly on Pad 39A or 39B at Cape

Canaveral. The STSPLUS simulation did not catch up with the true orbit

until approximately the OMS-2 burn (Orbital Maneuvering System, Burn #2)

which typically occurs about 35 to 45 minutes after launch. The OMS-1 burn

is not required for a nominal ascent trajectory but its place and number

are reserved in the STS Timeline regardless of whether or not it takes

place. I am always relieved by the CapCom call "OMS-1 NOT REQUIRED" because

that means that the Solid Rocket Boosters (SRBs) and the Main Engines

(SSMEs) did their job correctly.

In order for the STSPLUS launch simulation to operate correctly, the

following data must be available:

Accurate Estimated Pre-Launch 2-Line Elements (TLEs)

Planned Launch Time corresponding to the TLEs

In addition, the launch must take place from Cape Canaveral, Florida, not a

concern since the Vandenberg Space Shuttle launch complex was abandoned

some years ago. The Launch Simulation must be enabled when entering the

Launch Time and Date using F5 from the Main Menu (described below).

*** IMPORTANT NOTE ***

STSPLUS' Launch Simulation will ONLY execute correctly

for Space Shuttle launches from Cape Canaveral, Florida.

Attempting to "trick" the program with other spacecraft

or invalid TLEs may produce unpredictable results or

program crashes!

Estimated pre-launch TLEs are usually prepared well in advance of a

flight by Gil Carman or others at the NASA Johnson Space Center, Flight

Dynamics, in Houston, Texas. These TLEs represent the planned orbit for the

Space Shuttle for a nominal launch and are calculated for a time (the

"Epoch" of the TLEs) following the OMS-2 burn. It is the OMS-2 burn which

circularizes the orbit at the intended altitude; prior to OMS-2, the

orbiter is in a highly elliptical orbit whose perigee (low point,

technically the minimum orbital distance from the center of the Earth) is

some tens of miles above the surface and whose apogee (high point) is at or

near the planned altitude. The OMS-2 burn occurs at apogee and in effect

raises the perigee to the planned altitude. With apogee and perigee both at

approximately the same altitude, a nearly circular orbit is the result.

Without the OMS-2 burn the orbit is not sustainable, dipping too far into

the Earth's atmosphere, and the orbiter would have to make a landing

immediately.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 60

Launch simulation is divided into two phases: Pre-Launch Phase with

the orbiter on the launch pad; and, Ascent Phase which begins with liftoff

from the pad and continues until approximately OMS-2 burn. During the Pre-

Launch Phase, the Space Shuttle icon simply remains on the selected launch

pad at Cape Canaveral as the clock counts down. No ground track or circle

of visibility is drawn for the orbiter and the usual on orbit data'" are

neither calculated nor displayed. Since many of the parameters displayed

are referenced to inertial coordinate systems (Earth Centered Inertial

[XYZ] or Equatorial [RA/DEC], etc.) which take into account the rotation of

the Earth, these data will change with time even though the orbiter is

stationary! At approximately five minutes pior to liftoff, STSPLUS

calculates and plots the ground track for the ascent phase and the next

several orbits.

For those who may be interested, the geodetic coordinates and

elevation above Mean Sea Level (Inertial Measurement Unit position in the

orbiter) for the two Space Shuttle launch pads at Cape Canaveral are:

Pad 39A

PadLat =  28.6080 N

PadLon = -80.6041 W

PadElv =  62.484  meters

Pad 39B

PadLat =  28.6272 N

PadLon = -80.6208 W

PadElv =  64.244  meters

If the launch pad is unknown, select either one since the difference in

orbit trajectory is essentially negligible. The launch pads are plotted on

orthographic projections (only) with MAG=2000 or greater. They are

unlabeled because of their close proximity to each other and to Cape

Canaveral.

At T=0, STSPLUS begins using its internal Ascent Phase logic to

simulate liftoff, the powered portion of ascent, and the coasting portion

of ascent to apogee and the OMS-2 burn. While based upon actual orbital

ascent data, the Ascent Phase is only a very rough approximation for any

given flight. The internal ascent model, adjusted for the planned altitude

and orbital inclination, is used exclusively until after MECO (Main Engine

Cutoff, which occurs about 8 minutes after launch). From MECO until OMS-2,

STSPLUS begins a gradual transition from its internal model to the standard

NORAD SGP4 orbital model which uses the estimated pre-launch TLEs. From

OMS-2 on, only the NORAD SGP4 model and the TLEs are used. During the

Ascent Phase, all the usual orbital data are calculated and displayed.

All users are cautioned that the internal model is very approximate

and is intended only to provide a reasonably realistic transition from the

launch pad to orbit without trying to exactly duplicate all of the data for

a specific ascent trajectory. The displayed data are representative but NOT

exact. Actual ascent trajectories vary considerably as a function of

payload, orbital altitude, orbital inclination, and to a lesser extent

other factors such as winds aloft. The internal ascent model from my

original STSORBIT program, supplemented by numeric ascent data supplied by

Willie Musty of Rockwell Mission Support, has been adapted for this

purpose. My thanks to Willie for his assistance in developing the current

ascent model!

In order to use the STSPLUS launch simulation, accurate estimated pre-

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 61

launch TLEs and launch time are REQUIRED. Estimated pre-launch TLEs are

valid for a specific launch time ONLY; a launch delay requires new

estimated TLEs. (For most but not all missions, new TLEs may be calculated

from the original TLEs and launch time using my program ADJ2LINE.

Rendezvous missions are an example of missions where ADJ2LINE cannot be

used to recalculate estimated TLEs.) The following is a sample for the STS-

67 ASTRO-2 Mission in early March of 1995:

----------------------------------------------------------------------

Date: Mon, 13 Feb 1995 14:27:46 CST

From: Gilbert Carman <GCARMAN@jscdm.jsc.nasa.gov>

To: ELEMENTS@jscdm.jsc.nasa.gov

Subject: STS-67 prelaunch elements

STS-67

1 99967U          95061.32643015  .00011689  00000-0  86071-4 0    43

2 99967  28.4675  88.9985 0004519 287.7960  72.2100 15.71297841    28

Satellite: STS-67

Catalog number: 99967

Epoch time:      95061.32643015   =    (02 MAR 95   07:50:03.56 UTC)

Element set:     004

Inclination:       28.4675 deg

RA of node:        88.9985 deg            Space Shuttle Flight STS-67

Eccentricity:     .0004519               Prelaunch element set JSC-004

Arg of perigee:   287.7960 deg           Launch:  02 MAR 95  06:37 UTC

Mean anomaly:      72.2100 deg

Mean motion:   15.71297841 rev/day                Gil Carman

Decay rate:     1.1689e-04 rev/day^2       NASA Johnson Space Center

Epoch rev:               2

Checksum:              331

The deorbit burn will be Friday, March 17, at 19:09 UTC on orbit 245.

Gil Carman

NASA Johnson Space Center

----------------------------------------------------------------------

The Launch date and time are specifically identified and must not be

confused with the Epoch Time (which is the instant for which the orbital

data have been calculated). Note that the orbiter has been assigned a dummy

NORAD (Catalog) number (99967) and no International Designation because the

launch has not yet actually taken place. The data in the example are given

as 2-Line Elements (TLEs) and in a "plain English" format to satisfy the

needs of various user; STSPLUS uses only the TLE format. The estimated pre-

launch data are usually available several weeks prior to the launch on the

NASA Spacelink BBS and my own RPV Astronomy BBS as well as various other

sources.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 62

Launch Simulation Setup

-----------------------

In order to generate a Space Shuttle Launch Simulation, STSPLUS must

be set up correctly. Note that the Launch Simulation model is only

available for Space Shuttle launches from Cape Canaveral, Florida; use of

Launch Simulation under any other circumstances may yield unpredictable

results! The following procedure will guide the user through the proper

setup process.

1.   Obtain estimated pre-launch estimated 2-line elements (TLEs) for the

mission. Using an ordinary ASCII editor if necessary, make sure that

the first line of data in the file is the mission name ("STS-67" in

the example above. This step is to ensure that the TLEs can be read by

STSPLUS. (The TLEs may also be available separately, already formatted

for use by STSPLUS and similar tracking programs.) The following

example illustrates the (edited) TLE format for the estimated orbit

which consists of the three lines between the dashed lines:

---------------------------------------------------------------------

STS-67

1 99967U          95061.32643015  .00011689  00000-0  86071-4 0    43

2 99967  28.4675  88.9985 0004519 287.7960  72.2100 15.71297841    28

---------------------------------------------------------------------

The length of the first line is variable, usually less than 20

characters, and identifies the mission. The second and third lines

contain the orbital data and must be exactly 69 characters long. There

must be no spurious characters or lines, and each line should be

terminated by the CR and LF characters. Provided the TLE data are the

first three lines in a file, additional lines of text or other data

may follow; such a file will be read correctly by STSPLUS.

Following launch, the actual NORAD number and International

Designation are assigned and real orbital data are given, illustrated

by TLEs from another mission:

---------------------------------------------------------------------

STS-66

1 23340U 94073A   94317.54483356  .00013187  72758-9  36062-4 0   199

2 23340  56.9893 152.3330 0012220 307.6197 288.0410 15.93309153  1587

---------------------------------------------------------------------

Note the NORAD Number ("23340") and International Designation

("94073A", which means Year 1994, Satellite 73, First Object assigned.

2.   Place the file in the 2-line elements directory specified for STSPLUS.

Check the directory using F7 from the STSPLUS Main Menu if necessary.

3.   Note the planned launch time ("02 MAR 95  06:37 UTC" in the example).

4.   Start STSPLUS with the DOS command "STSPLUS". Do NOT use the "/R"

command line parameter at this time.

5.   From the STSPLUS Main Menu, press F2 to select 2-line elements. When

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 63

prompted enter the filename for the estimated pre-launch 2-line

elements, followed by "STS" to select the Space Shuttle as the

satellite. The orbital data will be displayed on the screen. Press

ENTER to accept the data. The ground track map will be drawn.

6.   When map drawing has been completed, press ENTER to return to the Main

Menu.

7.   From the Main Menu, press F5 to select Launch Time and Date. You will

be prompted to enter the time ("06:37U" for the example and be sure to

include the trailing "U" in this example to signify that the time is

UTC = Coordinated Universal Time, formerly known as Greenwhich Mean

Time). You will next be prompted to enter the date ("03/02/1995" [US

date style] or "02.03.1995" [European date style] for the example).

Up to this point, the procedure is identical to setting the launch time and

date for any satellite so as to be able to display Mission Elapsed Time

(MET). The following step allows you to select Launch Simulation or normal

operation.

8.   Finally, you will be prompted to select the Launch Site. Enter "1"

to select Launch Simulation from Pad 39A at Cape Canaveral, or "2" to

select Launch Pad 39B. (Entering "0" will disable launch simulation

and should be used for all other satellites and missions.)

9.   Verify the time, date, and launch site information, then press ENTER

to accept the data or SPACE to repeat. To save these data for

subsequent use, press "Y" when asked if you wish to append this data

to file STSPLUS.LTD. The program will return to the Main Menu.

10.  From this point on, selecting this mission (by name or NORAD number)

will use Launch Simulation until after the OMS-2 burn.

Some time after launch (usually well after the OMS-2 burn), the mission

will be assigned a NORAD number and an International Designation. The

actual launch time may vary from the planned launch time! TLEs released

after that time will carry the correct NORAD number and STSPLUS must be

reconfigured for that information by REPEATING STEPS 1 THROUGH 10 to re-

enter the launch time, date, and launch site and cause them to be assigned

to the actual NORAD number. Once the mission has passed the OMS-2 burn,

Launch Simulation may be disabled.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 64

Satellite Communications and Amateur Radio

------------------------------------------

By now, everyone is familiar with communications satellites. They

provide almost instant communications, particularly television, around the

globe from their assigned geostationary "parking slots" some 22,300 miles

above the surface of the Earth. The concept of the geostationary

communications satellite was originated by the science fiction writer

Arthur C, Clarke some thirty-odd years ago. Novel and revolutionary at the

time, they have become an accepted part of global communications, all but

taken for granted by the millions of people who see the images they

transmit over vast distances. Glossing over some of the "minor

technological details" that make these miracles possible, the

communications satellite is relatively easy to use. Because of its

geostationary orbit (which matches its orbital velocity with the Earth's

rate of rotation), it appears to remain at the same point in the sky. Once

properly located, ground terminals may be more or less permanently aimed

and that's that. Reliable communications are routine except during the

semi-annual Sun blockage periods when the Sun, satellite, and ground

terminal are in a direct line with each other and the Sun's powerful

radiation overwhelms the signals from the satellite.

However, geostationary communications satellites are but one example

of the uses for satellite communications. Except for a relatively few

passive satellites, each satellite has on board radio transmitters and

receivers so that its ground control centers may send commands and receive

data; these commands and data provide for the operational control and

orbital position and stability of the satellite. Unlike the geostationary

communications satellites, these satellites are in orbits which cause them

to appear to move rapidly across the sky when viewed from the ground. The

typical effective ground speed of the space shuttle, for example, is some

17,500 miles per hour; other satellites in higher orbits move more slowly.

Viewed from afar, both the ground station and the satellite are moving

rapidly, sometimes toward each other and sometimes away, as a result of the

rotation of the Earth and orbital direction/velocity respectively.

Since the typical satellite's receiver(s) and transmitter(s) are

usually set for fixed frequencies, these high relative velocities cause a

problem on the ground known as Doppler Shift. Almost everyone has heard a

train whistle as it speeds past; the whistle's pitch is high when first

heard, then drops steadily as the train passes. The "true" pitch of the

whistle is heard when the train is opposite the listener. While the train

is approaching the pitch is shifted to a higher frequency, and as the train

recedes the pitch is shifted to a lower frequency. For satellite

communications, this effect is increased by the much higher relative

velocities involved and it is usually necessary to adjust the transmit and

receive frequencies on the ground to compensate for the shift.

Stated simply, the ground station must adjust its transmitting

frequency such that the shifted frequency as received by the satellite is

the exact frequency for which the satellite receiver is set. Similarly, the

ground station must adjust its receiver frequency to the shifted frequency

at which the satellite's signal will be received. Like the train whistle,

the ground station's transmit and receive frequencies are constantly

changing as the satellite approaches and then departs from the ground

station. As a general rule, no two satellite passes over a ground station

have exactly the same geometry and therefore these frequency shift

adjustments must be calculated dynamically for each pass. In the special

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 65

case of Frequency Modulation (FM) transmissions using a receiver with

Automatic Frequency Control (AFC) and a sufficiently wide receiver AFC

bandwidth, no adjustments may be necessary.

Given a satellite's orbital paramaters and the appropriate computer

software, these data can be calculated in advance of an upcoming pass as

well as in real time. Most of the required data are already calculated in

satellite tracking software such as STSPLUS. Ken Ernandes, N2WWD, offered

his expertise and amateur radio equipment to assist in the implementation

and test of Doppler Shift calculations in STSPLUS. The necessary changes

and additions to the software were implemented in mid-March 1994 and Ken

made a preliminary test using Radio Sputnik 10 (RS-10, a piggybacked

transponder on the Russian COSMOS 1861 satellite, NORAD #18129). To our

considerable surprise and delight, the very first test was a complete

success; although the satellite only reached a maximum of 7 degrees above

the ground station's horizon, the transponder signal was heard (rather weak

and noisy) on the predicted frequency. Although we both were confident in

our mathematical solution to the Doppler shift problem, it is seldom that

such calculations turn out to be correct on the first try! Testing and

validation continue.

STSPLUS Doppler Shift Mode

--------------------------

STSPLUS' Doppler Shift Mode of operation may be used for real time

communications with any satellite, not just amateur radio transponders, for

which orbital data ("2-line elements" or "TLE") are available. For each

satellite, the user prepares a preset frequency list in file STSPLUS.FRQ

which includes the satellite's NORAD Number, the transmit (XMIT) and

receive (RECV) center frequencies, and a special code which is used to

select NORMAL or INVERTED satellite transmitter transponders (see below).

For satellites with fixed transmit and receive frequencies, that is

all that is required; for satellites which receive and transmit over a band

of frequencies, such as the passbands of the typical amateur radio repeater

transponder, the receive and transmit frequencies may be quickly "tuned" in

tandem by fine increments of 100 Hz or coarse increments of 1 KHz over the

entire passband.

For those who may be interested, the solution of the Doppler shift

computations required that the ground station position vector and the

satellite position and velocity vectors be calculated using standard

transformation algorithms (and the SGP4 Orbital Model for determining the

satellite data), then converted to Earth-Fixed Greenwich ("EFG")

coordinates, a geocentric intertial coordinate system using the WGS-72

Geodetic Model. From these data the relative velocity and frequency shift

ratios are next calculated. These ratios are then applied to the preset

transmit and receive center frequencies to yield the shifted frequencies,

all of which are then displayed to the user. Provided the computer is

equipped with a math coprocessor chip, all data are updated each second.

The following is an example of the frequency data displayed as a satellite

(RS-10 in the example) approaches the ground station:

UpLink:   145.8900    Uplink frequency received by satellite

XMIT:   145.8880    TRANSMIT frequency at ground station

DnLink:    29.3900    Downlink frequency xmitted by satellite

RECV:    29.3904    RECEIVE frequency at ground station

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 66

The shifted transmit frequency (XMIT) and receive frequency (RECV) are

also color coded to indicate the signal status:

RED       The satellite is below the receiver horizon; communications

are normally not possible.

YELLOW    The satellite is from zero to five degrees above the

receiver horizon; transmissons MAY be possible.

GREEN     The satellite is five degrees or more above the receiver

horizon; transmissions should be practical if the receiver

horizon is clear in the direction of the satellite.

The altitude (or elevation) of the satellite above the receiver

horizon is usually a good indicator of communications capability. However,

transmitter power, receiver sensitivity, antenna structure and orientation,

and atmospheric conditions all play a role in making reliable full duplex

communications practical. For example, the large antennas used by the DOD

C-Band Radar Network, used to track the orbiter and other satellites during

ascent and critical maneuvers, typically acquire signal lock when the

satellite is between 3 and 4 degrees above the local horizon. A low power

amateur radio rig may require that the satellite be from 5 to 8 degrees

above the local horizon for reliable communications. To illustrate the role

atmospheric conditions may play, the space-based geostationary TDRS

(Tracking and Data Relay Satellite) typically acquires signal lock with a

target satellite at or near Earth limb (what passes for the "horizon" at

the satellite). In addition to the frequency data, the ground station

("STN") times for Acquisition of Signal ("AOS") and Loss of Signal ("LOS"),

calculated for the true ground station horizon, are displayed so that the

user may quickly determine how soon a pass will begin or how much time

remains in a current pass.

For ground station to satellite communications, operation is

straightforward. The user simply adjusts his transmit ("XMIT") and receive

("RECV") frequencies to those shown by STSPLUS as the satellite passes his

location. Since the frequencies required at the satellite are known and do

not change, there are no "fine tuning" adjustments required.

Compared to a satellite with fixed receive and transmit frequencies,

the typical amateur radio satellite transponder (also referred to as a

"crossband repeater") presents a slightly more complex situation. The

transponder receives signals across a passband of frequencies (20 to 80 KHz

are typical bandwidths), then retransmits the received signals across a

passband of the same width but centered at a different frequency. The

center frequency of the receive and transmit passbands are known in advance

but may change from time to time depending upon the transponder mode (CW,

voice, digital packet, etc.). The transmit side of the transponder may also

operate in either NORMAL or INVERTED mode. That is, for NORMAL mode the

transmitted signal is the same frequency above or below the center

frequency as is the received signal; for INVERTED mode, the transmitted

signal is the same frequency above (below) the transmit center frequency as

the received signal is below (above) the receive center frequency.

STSPLUS addresses this situation in two ways. First, once the user has

received a signal of interest, he uses the PgUp, PgDn, UP, and DOWN keys to

"fine tune" the downlink frequency shown by STSPLUS ("RECV") until it

matches the actual received frequency. PgUp and PgDn perform "coarse

tuning" in increments of 1 KHz, and UP and DOWN perform "fine tuning" in

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 67

increments of 100 Hz. The response is quite rapid and the fine tuning may

usually be performed in no more than several seconds. STSPLUS then makes

the necessary calculations to show the required uplink frequency ("XMIT")

to permit full duplex communications. The second part of the problem is the

transponder mode; STSPLUS selects NORMAL or INVERTED transponder mode based

upon the mode parameter supplied by the user in file STSPLUS.FRQ: 1 =

NORMAL, -1 = INVERTED.

Satellite Phase (Mean Anomaly)

------------------------------

Some amateur satellite transponders (an example is AO-13) change their

mode of operation based upon a parameter called "phase". Data is published

by AmSat and/or others for these transponders giving the mode of operation

for specific ranges of Phase. Phase is the Mean Anomaly, one of the six

parameters of orbital data, normalized to the range from 0 to 255. Phase =

0 (Mean Anomaly = 0 degrees) corresponds to perigee, the point at which the

satellite's orbit makes its closest approach to the center of the Earth

(NOT the surface!); Phase = 128 (Mean Anomaly = 180 degrees) corresponds to

apogee, the point in the orbit at which the satellite is most distant from

the center of the Earth.

When the Doppler Shift Mode is enabled in orthographic projection (by

pressing F8 while the map is displayed), the Phase is displayed PROVIDED

that the Eccentricity is greater than or equal to 0.005:

Phase:       123.9

When the conditions for the display of Phase data are met, it replaces the

orbital period data ("Per'd") in the Doppler Shift Mode; if not, the

orbital period data will be displayed. Technically, Phase is defined as an

integer in the range 0 to 255. However, in order to give the user some

advance warning when the Phase will step from one number to the next,

STSPLUS also displays the first decimal digit. Phase is NOT displayed in

rectangular projections due to insufficient display space.

TECHNICAL NOTES REGARDING PHASE:

1.   The Mean Anomaly is dynamically calculated by the SGP4 algorithm

within STSPLUS. This value (in radians) is divided by (2 * PI) and the

result multiplied by 256 to obtain Phase. The Phase is then displayed

as a number ranging from 0.0 to 255.9. The technical definition of

Phase is as an integer ranging from 0 to 255 (so as to fit in a single

8-bit byte of computer data).

2.   Phase is calculated based upon geocentric (center of the Earth) data

rather than geodetic (surface of the Earth taking into account the

shape of the Earth) data. What may appear as an apogee or perigee

based upon "Elv", the distance above Mean Sea Level, may NOT be the

actual apogee or perigee when calculated using geocentric data.

Because of the shape of the Earth, these distances may vary up to 21

kilometers from the Equator to a pole.

3.   Mean Anomaly (and therefore Phase) is undefined for a perfectly

circular orbit; there is no perigee or apogee in this situation. In

addition, some uncertainties are also introduced by gravitational

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 68

variations which can make accurate dynamic determination of apogee or

perigee difficult for low Earth orbits. These variations and

inaccuracies become significant for orbits with an Eccentricity of

0.005 or less, a situation which is typical for most low Earth orbits.

For this and other reasons, amateur satellites in low Earth orbits do

not use Phase to determine operating modes.

4.   The following conditions are required for the Phase data to be

displayed:

a)   The orthographic projection has been selected by pressing "O"

while the map is displayed.

b)   The Doppler Shift Mode has been selected using F8 while the map

is displayed.

c)   The Eccentricity of the satellite's orbit is 0.005 or greater.

If any of these three conditions is not satisfied, the orbital period

data ("Per'd") will be displayed.

Satellite Communications Technique and Cautions

-----------------------------------------------

Once full duplex communications have been established, remaining "in

lock" throughout a pass requires that both parties continually adjust their

transmit and receive frequencies to the values displayed by STSPLUS to the

extent practical and consistent with the bandwith capabilities of their

receivers. Although this may seem a bit daunting at first, the actual rate

of change of the frequencies is sufficiently slow that it can easily be

managed by the relative novice with a little practice.

Equally important in the relatively new area of communications with or

via manned and unmanned satellites is that all participants recognize that

the available bandwidth and communications opportunities are a "scarce

resource" that must be used with care and shared. This is in keeping with

long standing amateur radio tradition. The problem can be particularly

difficult with the manned spacecraft, MIR and the Space Shuttle. Careful

attention to doppler shift can be of considerable help in completing the

call by using the correct frequency or frequencies.

********************

*  IMPORTANT NOTE  *

********************

Experience with communications via amateur ratio satellites such as

RS-10 has shown that careful test and calibration of the receiver and

transmitter are essential to successful communications. For example, an

error or bias of 2 or 3 KHz on the receiver frequency can make the

difference between a successful call and a failure. If the receiver or

transmitter has a consistent bias, it may be possible to temporarily adjust

the values of the center frequencies to compensate for the problem but the

best solution, of course, is to have the equipment calibrated and operating

correctly.

Equally important, the computer clock must be accurately set. Radio

time signals such as those broadcast by the National Institute of Standards

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 69

and Technology (NIST) on WWV are sufficiently accurate for this purpose.

The program TIMESET by Peter Petrakis is highly recommended to

automatically set the computer clock via the telephone time services of

NIST or the U.S. Naval Observatory (USNO).

Finally, the frequencies calculated by STSPLUS are no more accurate

than the orbital data used. For the typical amateur radio satellite, the

orbital data should be no more than a week old for reasonable results. If

the satellite is performing orbital maneuvers (as MIR does from time to

time), only the most current elements will yield satisfactory results.

Sources such as Internet, Celestial BBS, NASA Spacelink BBS, NASA GSFC

RBBS, and my own RPV Astronomy BBS offer up-to-date 2-line elements for all

or most of the common amateur radio satellites. See the section "Computer

Bulletin Board Systems" near the end of this document for current BBS

telephone numbers and related information. Since the amateur radio

transponders are often "piggybacked" on a primary satellite, the name of

the satellite used by these sources may be different from the amateur radio

designation. Use the example file STSPLUS.FRQ to check for the NORAD

numbers of common amateur radio satellites and use the NORAD number rather

than a satellite name or designation when searching for TLEs.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 70

Preparing File STSPLUS.FRQ for Amateur Radio Use

------------------------------------------------

File STSPLUS.FRQ contains the parameters required for STSPLUS to

operate in the Doppler Shift Mode. Each entry (line) in the file includes

the satellite NORAD Number, UPLINK and DOWNLINK center freqencies, and the

transponder mode, specified in in that order, separated by commas and

without any leading or trailing spaces. The following format is used for

each entry:

00000,100,100,1           (Default values if sat not included)

18129,145.8900,29.3900,1  (Parameters for NORAD #18129)

--+-- ----+--- ---+--- +

|       |       |    |

|       |       |    +-- Transponder Mode:  1 = NORMAL

|       |       |                          -1 = INVERTED

|       |       |

|       |       +------- DownLink Center Frequency (MHz)

|       |

|       +--------------- UpLink Center Frequency (MHz)

|

+----------------------- Satellite NORAD Number

The first sample line shows the "00000" entry which determines the default

values if the satellite is NOT included in file STSPLUS.FRQ. This should be

the FIRST LINE in file STSPLUS.FRQ. The second line gives real parameters

for a specific satellite; the frequencies shown select the Mode A voice

passband for Radio Sputnik 10 (RS-10, piggybacked on COSMOS 1861, NORAD

#18129). Preset frequencies may range from 1.0000 MHz to 99000.0000 MHz.

Neither the uplink nor downlink frequency should exceed approximately

99000.0000 MHz to avoid an overflow condition on the display. Although the

center frequencies are shown above in MHz, any desired units may be used

since STSPLUS simply calculates a ratio and displays the results with four

digits to the right of the decimal point.

File STSPLUS.FRQ may be created or edited with any ASCII editor; word

processor users, use the "non-document" mode. Note that only minimum error

checking is performed and the user must observe the required format exactly

for each line in the file. Up to ten entries may be included for a given

satellite (using the same NORAD Number) in order of preference. If more

than one entry is present for the current satellite, the user is presented

with a list and asked to make a choice.

Ken Ernandes, N2WWD, in conjunction with his tests of STSPLUS' Doppler

Shift Mode, has prepared a preliminary STSPLUS.FRQ file with the current

(as of March, 1994) center frequencies of fourteen amateur radio

satellites. Note that several satellites have more than one entry,

corresponding to different modes of operation:

00000,100,100,1                    (Default values)

14129,435.1025,145.9025,-1         (AO-10)

16609,145.5500,145.5500,1          (MIR)

18129,145.8850,29.3800,1           (RS-10)

18129,21.1800,145.8800,1

18129,21.1800,29.3800,1

19216,435.4950,145.9000,-1         (AO-13)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 71

19216,144.4500,435.9650,-1

19216,1269.4750,435.8600,-1

19216,435.6190,2400.7290,-1

20437,145.9750,435.0700,1          (UO-14)

20439,145.9000,437.02625,1         (PACSAT)

20441,1265.0000,437.0751,1         (WO-18)

20441,1265.0000,437.1258,1

20442,145.8400,437.15355,1         (LO-19)

20480,145.9550,435.8500,-1         (FO-20)

21087,435.0160,145.9870,1          (AO-21)

21087,435.0620,145.8920,-1

21087,435.0830,145.9060,-1

21089,21.2400,29.4400,1            (RS-12/13)

21089,21.2400,145.9400,1

21089,145.4400,29.4400,1

21575,145.9000,435.1200,1          (UO-22)

22077,145.9000,435.1670,1          (KO-23)

22077,145.9000,435.1200,1

22825,145.8500,436.8000,1          (AO-27)

NOTE: The center frequencies listed above are preliminary, based upon

available information. For example, the uplink frequencies for the

first entry for RS-10 and the entry for FO-20 have been adjusted up by

5KHz to compensate for an apparent transponder bias. These data will

be coordinated by Ken Ernandes, N2WWD. Ken may be contacted through my

RPV Astronomy BBS at (310) 541-7299 or on CompuServe at 70511,3107.

Users who have carefully calibrated their receivers and transmitters

and who have updated information are encouraged to contact Ken.

SAREX, the Shuttle Amateur Radio EXperiment, is frequently assigned to

Space Shuggle flights. The following uplink and downlink frequencies have

been assigned for that flight (data as of April 1994):

UPLINK       DOWNLINK         NOTES

-------------------------------------------

VOICE:    144.91 MHz     145.55 MHz     EXCEPT EUROPE

144.93

144.95

144.97

144.99

144.70 MHz     144.55 MHz     EUROPE ONLY

144.75

144.80

PACKET:   144.49 MHz     145.55 MHz     WORLDWIDE

NASA adds the following note with respect to the voice uplink: "The

astronauts will not favor any one of the above frequencies. Therefore, the

ability to talk with an astronaut depends on selecting one of the above

frequencies chosen by the astronaut." (Information courtesy NASA Spacelink

BBS as of April, 1994) Thus, for a station in North America, the user may

add the following entries to file STSPLUS.FRQ:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 72

00059,144.91,145.55,1

00059,144.93,145.55,1

00059,144.95,145.55,1

00059,144.97,145.55,1

00059,144.99,145.55,1

00059,144.49,145.55,1

where "00059" must be replaced by the actual NORAD number assigned to the

flight. A temporary NORAD number corresponding to the flight number,

illustrated by "00059" in the example above, is usually used until the

permanent NORAD number is assigned (although some sources use a different

method to assign a temporary NORAD number -- check the TLEs). However, if a

temporary NORAD number is used during the initial portion of the flight,

the data in file STSPLUS.FRQ must later be updated to the permanent NORAD

number when that number is used in the TLEs in order for STSPLUS to

recognize the flight's frequency list. A typical set of TLEs for a Space

Shuttle are shown with the NORAD number and Eccentricity noted:

STS-65

1 23173U 94039A   94203.67836573  .00310904  40043-6  28999-3 0   371

2 23173  28.4640 263.1143 0006121  89.0653 314.1440 16.10872029  2244

--+--                   ---+---

|                        |

|                        +----- Eccentricity (=0.0006121)

|

+------------------------------ NORAD Number

Because different sources of 2-line elements (TLEs) may use different

names for the same satellite, numerous satellites may have similar names,

and some payloads (especially amateur radio transponders) are "piggybacked"

on a primary satellite with a different name, always use the NORAD Number

if possible when searching a file for the TLEs. The MIR Space Station,

NORAD Number #16609, can be mistaken for numerous MIR debris objects ("DEB"

or "D" appears in the first line of the TLEs). For AO-13, for example,

press F2, select the desired TLE filename, then enter "#19216" (without the

quotation marks but WITH the pound sign) as the satellite name. This method

will ALWAYS find the data if they are present in the file. Once the data

are found, STSPLUS displays them as usual. If there is only one entry in

file STSPLUS.FRQ for the satellite, STSPLUS will immediately draw the map

after ENTER is pressed to approve the satellite and its data. However, if

more than one entry for the satellite is present, and STSPLUS is currently

in the Doppler Shift Mode, STSPLUS will display a list of the available

preset frequencies and request the user to select one:

Preset Frequency Selections for 19216

#      UpLink       DnLink  Mode

---------------------------------

1    435.4950     145.9000    -1

2    144.4500     435.9650    -1

3   1269.4750     435.8600    -1

4    435.6190    2400.7290    -1

Enter Desired Preset Frequency Selection Number [1]:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 73

Enter the desired preset frequency selection number followed by ENTER. If

you wish selection #1, you may simply press ENTER. Entering a number less

than 1 or greater than the highest selection number will also pick

selection #1.

Note that when STSPLUS is NOT in the Doppler Shift Mode, no list of

preset frequency selections is displayed and STSPLUS automatically picks

selection #1 (to avoid bothering folks who are not interested in the

Doppler Shift Mode).

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 74

ACTIVE KEYS DURING GROUND TRACK DISPLAY

---------------------------------------

The following table lists the various keys which are active when the

ground track display is shown on the screen. Some of these features are

more fully described elsewhere.

ENTER  Return to Main Menu (cancel the simulation).

F1  On-line HELP. Press F1 to display a help screen in the lower

portion of the screen. Press F1 again to resume normal data

display.

F2  Selects the Big Clock mode. These modes are selected in the

following order:

0    No clock displayed

1    UTC date and time

2    Local date and time

3    STN/TDRS AOS/LOS and MET or T+Epoch (select with F5)

Note that not all computers (especially older CGA systems) will

display the extended graphics characters used for the large clock

characters. The symptom of this problem is that the lower left

portion of the data block is mostly blank after pressing F2. If

you have this problem and your computer is running DOS 3.x or DOS

5.0, enter the command "GRAFTABL" at the DOS prompt before

running STSPLUS or include the line "GRAFTABL" in your

AUTOEXEC.BAT file; this sets the "code page" to enable the

computer to display the extended graphics characters. [The

program GRAFTABL.COM is included as part of DOS in most cases.]

F3  Enable or disable printer logging. If logging is enabled, the

word LOG appears in the lower right of the screen. Be sure the

printer is turned on BEFORE using the L command. The "L" command

automatically enables the display of ascending and descending

node information.

F4  Toggle FAST mode from x1 to x10 to x60 to x1, etc. When either of

the fast modes is enabled, "(x10)" or "(x60)" will appear at the

upper right of the data block in red. This feature operates in

both the normal (real or simulated time) and PAUSE modes. When

x10 or x60 fast modes are used, automatic map generation is

disabled in rectangular projections; use the TAB key to restore

automatic map generation.

F5  Switches the elapsed time between "T+Epoch" and "MET". If no

launch time and date have been entered, this command will have no

effect.

F6  Enable PAUSE mode. The plot is frozen at the current position and

the "+" and "-" keys are enabled. The "=" key may be used instead

of the "+" key to avoid use of the SHIFT KEY. Press ENTER to

resume normal operation using the current simulated time. (NOTE:

To return to REAL TIME after the PAUSE mode, press ENTER after

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 75

leaving PAUSE to return to the Main Menu, then press F8+F1.)

F7  Enable or disable the spacecraft circle of visibility.

F8  Switches program operation between NORMAL and DOPPLER SHIFT

modes. (Orthographic projection only; see text for details.)

F9  Change units of distance between kilometers (km), nautical miles

(nm), and statute miles (sm).

F10  Change satellite coordinates between Altitude/Elevation and

Azimuth (Topocentric), Right Ascension and Declination

(Equatorial), and X-Y-Z (Geocentric Rectangular, also known as

ECI or Earth-Centered Inertial) coordinate systems. If a Target

Satellite has been selected, F10 may also be used to display

Relative Range and Velocity between the Primary and Target

Satellites.

F12  Select units of display for AOS and LOS. Cycles through countdown

clocks, UTC time, local time, and MET. If launch time has not

been set, MET AOS/LOS times will NOT be available.

+/=  During PAUSE mode only, "+" moves the satellite to the NEXT

calculated position based upon the FAST mode then in effect:

simulated time is advanced 1, 10, or 60 seconds. The "=" key may

be used instead of "+" to avoid use of the SHIFT KEY.

-  During PAUSE mode only, "-" moves the satellite to the PREVIOUS

calculated position based upon the FAST mode then in effect:

simulated time is backed up 1, 10, or 60 seconds.

TAB  In rectangular map modes only, enable or disable automatic map

generation. Automatic map generation is ALWAYS enabled in

orthographic modes. When automatic map generation is enabled in

rectangular map modes, the letter "A" will appear in the upper

right of the display screen. Pressing the TAB key will always

cause the map to be redrawn.

PgUp  NORMAL OPERATION: When in one of the zoom modes, increases the

field of view up to a maximum of 180 degrees. Press rapidly to

execute multiple zoom steps without redrawing the map for each

keypress.

DOPPLER SHIFT MODE: Increases the "RECV" frequency by 1 KHz. Hold

down or press rapidly for large frequency changes.

PgDn  NORMAL OPERATION: When in one of the zoom modes, decreases the

field of view down to a minimum of 30 degrees. Press rapidly to

execute multiple zoom steps without redrawing the map for each

keypress.

DOPPLER SHIFT MODE: Decreases the "RECV" frequency by 1 KHz. Hold

down or press rapidly for large frequency changes.

Home  NORMAL OPERATION: When in one of the zoom modes, returns the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 76

field of view to 75 degrees (rectangular projections) or the full

globe (orthographic projections).

DOPPLER SHIFT MODE: Restores the UpLink and DnLink center

frequencies to the original values read from file STSPLUS.FRQ.

End  NORMAL OPERATION: When in one of the zoom modes, returns the

field of view to the last zoom factor used prior to pressing the

HOME key.

DOPPLER SHIFT MODE: (not used and inactive)

UP  NORMAL OPERATION: (not used and inactive)

DOPPLER SHIFT MODE: Increases the "RECV" frequency by 100 Hz.

Hold down or press rapidly for larger frequency changes.

DOWN  NORMAL OPERATION: (not used and inactive)

DOPPLER SHIFT MODE: Decreases the "RECV" frequency by 100 Hz.

Hold down or press rapidly for larger frequency changes.

B  Toggle the BLINK mode of the satellite symbol between blinking

and steady. NOTE: On most systems, the satellite symbol will

appear to blink very briefly as it is erased and redrawn even

when BLINK is OFF.

O  Select Orthographic Projection (the LETTER "O" not the digit

zero). PgUp, PgDn, Home, and End are active to select the

magnification.

W,0  Select World Map display, showing the full world from +85 degrees

North latitude to -85 degrees South latitude using rectangular

projection. If automatic map generation is disabled, pressing "W"

or "0" will toggle between the two world map displays.

Q  Select Quadrant Map display, showing 180 degrees field of view

(rectangular projection) and selected so as to approximately

center the satellite.

1-9 Select the indicated Quadrant Map. Automatic map generation is

!@# disabled when a specific quadrant map is selected. See the chart

in the section Quadrant Maps for the map numbers.

Z  Select Zoom Map display, showing from 180 to 30 degrees field of

view (rectangular projection) and selected so as to approximately

center the satellite. The default is 75 degrees.

L  Select Location Map display, showing concentric isocontours for

your location. If a second location has been enabled, press "L"

again for that location.

T  Tracking Station Map display, showing concentric isocontours for

the tracking station closest to the current ground track position

of the satellite using the current projection. Uses the data in

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 77

the current TRACKING STATION file to select the tracking

station(s); if the file is not found, STSPLUS defaults to an

internal set of tracking stations. Use F7 from the Main Menu to

select the TRACKING STATION filename.

M  Satellite Motion Map display, available on EGA and VGA systems

only. Displays a map with the satellite centered using the map

projection in effect when the key is pressed. In rectangular

modes, the map is shown in zoom. Maps are drawn "off screen" and

a complete map is always displayed. The map is updated every 10

seconds or as rapidly as the computer processor will permit.

While the Satellite Motion Map is displayed, the following keys

are active: ENTER, "M", "Home", "End", "PgUp", and "PgDn"; these

keys perform the same functions as during the normal display

except that the "M" key cancels the Satellite Motion Map and

returns to normal display.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 78

STSORBIT PLUS MAIN MENU

-----------------------

Once the map coordinates have been stored internally, STSORBIT PLUS

presents its Main Menu:

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Current time:  19:01:57 PDT   02:01:57 UTC

Current date:   15 APR 1995    16 APR 1995

F1     Convert Keplerian Data to 2-Line Format

F2     Read NASA/NORAD 2-Line Elements       (.TXT/.TLE Files)

F3     Pass Predictions and Data Output

F4     Tabular Satellite Positions      (TRAKSTAR by TS Kelso)

F5     Set Launch Time and/or Launch Date

F6     Set/Read/Save TDRS and Real Time Satellites

F7     Set FILENAMES and Paths

F8     Set program TIME and/or DATE

F9     DOS Shell            (CAUTION: DOS 3.x or higher ONLY!)

F10    Set STSORBIT PLUS Program Options and Features

ENTER  Resume Mission                                    (Mir)

ESC    Quit STSORBIT PLUS and Save Current Mission

Select desired function:

WHILE MAP IS DISPLAYED:  F1 = HELP  ENTER = Main Menu

During operation of STSPLUS, data are displayed by STSPLUS in several

standard formats:

26 JUL 1994       Date in day/month/year format

02:01:57 UTC   Coordinated Universal Time in hours:mins:secs

19:01:57 PDT   Local Time in hours:mins:secs (abbr. may vary)

3/09:23:15       MET in days/hours:minutes:seconds

320.50 nm    Distance in nautical miles

551.37 km    Distance in kilometers

-69.34       Angles in degrees; WEST longitude and SOUTH

latitude are negative.

Note that latitudes and longitudes also include "N" and "E" for positive

values respectively, and "S" and "W" for negative values respectively. This

convention, which may seem redundant, has been used to avoid possible

confusion; there are a number of representations for latitude and longitude

in common use which use different sign conventions. Azimuth (heading) is

given in the sense North-East-South-West where North is 0 degrees, East is

90 degrees, and so forth. The degree symbol is shown on the display for

all angles but has been omitted from this documentation because it may not

print correctly on all printers.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 79

F1     Convert Keplerian Data to 2-Line Format

----------------------------------------------

This function provides a means to individuals without modems to

receive the so-called "Keplerian Orbital Elements" by voice or other means

and reliably convert those data to the "2-Line Element" format as required

by STSPLUS and other satellite tracking programs. However, more data is

included in the 2-line orbital element set than is usually distributed as

the Keplerian orbital elements; this means that the missing data must

either be obtained from other sources or be set to a specified value or

zero. These instructions and the example and sample form which follows will

help the unskilled user to do these tasks.

This conversion function has but a single purpose: to prepare 2-line

orbital elements from Keplerian orbital elements by means of user keyboard

input. Since the program is designed only to transcribe valid data from one

format to another, no error checking is performed and the program makes no

tests of the "reasonableness" of the various data and parameters. The user

is therefore cautioned to check his data before using this program, or to

use the resulting data with caution until it has been verified.

The information shown below is usually included in the Keplerian

orbital elements as received via modem or voice (amateur radio or

telephone). This sample is an actual file for Space Shuttle Flight STS-55

launched in early 1993 as received via modem direct from the NASA Johnson

Space Center prior to launch:

Satellite: STS-55

Catalog number: 00055

Epoch time:      93073.67556033 =====> (14 MAR 93   16:12:48.41 UTC)

Element set:     JSC-003

Inclination:       28.4697 deg

RA of node:       228.7025 deg          Space Shuttle Flight STS-55

Eccentricity:     .0003812             Prelaunch Keplerian Elements

Arg of perigee:   314.2100 deg         Launch:  14 MAR 93  15:00 UTC

Mean anomaly:      45.8202 deg

Mean motion:   15.90487610 rev/day              G. L. Carman

Decay rate:     1.2020e-03 rev/day~2     NASA  Johnson Space Center

Epoch rev:               2

If you compare the information required by STSPLUS (as shown in the

example on the following page) with that above, several items are missing

or may be in a slightly different format. Here are some suggestions.

1.   A sample form is included in this documentation which will assist you

when receiving Keplerian orbital information by voice or by amateur

radio link. Note that the form includes lines for "Int'l Designation"

and "BSTAR Drag"; these items are frequently omitted and must be

determined independently (see below).

2.   The Catalog Number (often referred to as the NORAD Number) is assigned

by US Space Command at Cheyenne Mountain after a satellite has been

successfully launched. Initial or pre-launch Keplerian elements may

use the mission number or some other convenient number for this item.

Most tracking programs will accept any number here. STSPLUS uses the

Catalog number to keep track of launch date and time; if the Catalog

number used for a mission changes, the launch date and time must be

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 80

entered again.

3.   The Epoch time is the instant for which the Keplerial elements have

been calculated and is NOT the same as the launch date and time.

Launch date and time is shown separately, is NOT part of the 2-line

elements, and must be entered in STSPLUS using F5 from the Main Menu.

4.   The International Designation is often not assigned until well after a

flight is in progress; press ENTER to use all spaces for this item.

5.   For the 2-line orbital element set, the element set number (shown as

"JSC-003" in the example above) MUST be numeric. For this example,

enter "3" or "003". Most tracking programs will accept any number for

this item, up to a maximum of FOUR DIGITS.

6.   The "Epoch Rev/Orbit" is the orbit number at the epoch (the instant

when the data is taken). Up to FOUR DIGITS may be entered here but

except for the orbit number on displays, this number has no effect on

the propagated orbit. NASA assigns Orbit/Rev 1 to the first partial

orbit following launch; US Space Command usually (but not always)

assigns Orbit/Rev 0 to the first partial orbit and the orbit number

for space shuttle missions must usually be adjusted to conform to NASA

convention.

7.   When the program asks for "NDOT2 Drag/Decay", the information being

requested is the "Decay rate" in the Keplerian elements. It may be

entered in the form shown in the example or as a decimal fraction such

as ".002102" (which is identical to the "2.10200e-03" in scientific

notation as used in the example).

8.   When the program asks for "NDDOT6 Drag", press ENTER to insert a value

of zero. This should be satisfactory for most satellites and most

tracking programs.

9.   When the program asks for "BSTAR", you may press ENTER to insert a

value of zero. This will be satisfactory for times very close to the

Epoch Time for the elements. However, as time progresses the satellite

may lag behind the propagated (projected) ground track if no value is

avaialble for BSTAR. For space shuttle missions, NASA often assigns a

default value of "25599-3" or ".00025599". A value from a prior set of

2-line elements may also be used.

10.  After all data have been entered, STSPLUS will request the filename to

which the 2-line orbital data is to be written. Enter the full

filename and filetype, such as "TESTSAT.TXT". The file will be written

to the drive and directory set using F7+F3 from the Main Menu. DO NOT

INCLUDE A DRIVE OR DIRECTORY; THAT IS ADDED AUTOMATICALLY! An existing

file with the same name will be overwritten.

11.  If you make a mistake during the data entry process, you may use the

BACKSPACE key until ENTER has been pressed. If you wish to cancel the

program without writing the data to a file, press ENTER when asked for

the filename and the data will be discarded.

As noted elsewhere, orbital elements have a limited lifetime. How long

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 81

that lifetime may be depends primarily upon the orbital altitude. Low Earth

orbit (LEO) satellites, especially those such as the space shuttle and

Russian space station, are frequently maneuvered to maintain the desired

orbit. Orbital elements for these satellites may be valid for only days or

sometimes even hours. Orbital elements for higher orbital altitudes, say

above 1,500 km, tend to be valid for much longer -- up to weeks. Satellites

in very high or geosynchronous orbits exhibit usually long term orbital

stability. The Bottom Line: The accuracy with which any tracking program

can determine a satellite's position is primarily dependent upon having

accurate orbital elements!

Finally, a word about the NDOT2 and BSTAR parameters. A satellite's

orbit is completely and accurately determined at the epoch time without

NDOT2 and BSTAR. These two parameters determine various adjustments as the

orbit is propagated in time and are a function of such things as orbital

altitude, spacecraft attitude and cross-sectional area, atmospheric density

changes due to sunspot activity, and so on. Both US Space Command and

Johnson Space Center have been known to "tweak" these values for a variety

of reasons, usually because the orbit is in a state of change due to

maneuvers and/or excessive drag.

NOTE: A positive exponent for BSTAR, which indicates high acceleration

and is somewhat unusual (and often an indication of "tweaking"), is NOT

handled correctly and must be manually edited. The resulting elements may

not propagate accurately for more than a few hours.

EXAMPLE DATA INPUT

------------------

Enter 2-LINE ELEMENTS

Enter the required data as prompted. Most essential data is included in the

Kelplerian Orbital Data available to amateur radio enthusiasts. Other data

may be inserted if available or may be omitted if not. Accuracy may be

affected, see documentation.

Enter Satellite Name (15 chars max):      sts-55

Enter NORAD Number (NNNNN):               55

Enter Int'l Designation (8 chars max):

Enter Epoch Time (YYDDD.TTTTTTTT):        93073.67556033

Enter Element Set Number (NNN):           3

Enter Inclination (DDD.DDDD):             28.4697

Enter RA of Ascend Node (DDD.DDDD):       228.7025

Enter Eccentricity (.NNNNNNN):            .0003812

Enter Arg of Perigee (DDD.DDDD):          314.21

Enter Mean Anomaly (DDD.DDDD):            45.8202

Enter Mean Motion (NN.NNNNNNNN):          15.9048761

Enter Epoch Rev/Orbit (NNN):              2

Enter NDOT2 Drag/Decay (.NNNNNNNN):       .001202

Enter NDDOT6 Drag (NNNNN-N):              00000-0

Enter BSTAR (NNNNN-N):                    36300-3

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 82

EXAMPLE DATA OUTPUT

-------------------

Satellite Data has been entered as:

Satellite Name (15 chars max):            Sts-55

NORAD Number (NNNNN):                     00055

Int'l Designation (8 chars max):

Epoch Time (YYDDD.TTTTTTTT):              93073.67556033

Element Set Number (NNN):                  3

Inclination (DDD.DDDD):                    28.4697

RA of Ascend Node (DDD.DDDD):             228.7025

Eccentricity (.NNNNNNN):                  .0003812

Arg of Perigee (DDD.DDDD):                314.2100

Mean Anomaly (DDD.DDDD):                   45.8202

Mean Motion (NN.NNNNNNNN):                 15.90487610

Epoch Rev/Orbit (NNN):                     2

NDOT2 Drag/Decay (.NNNNNNNN):             .00120200

NDDOT6 Drag (NNNNN-N):                    00000-0

BSTAR (NNNNN-N):                          36300-3

Sts-55

1 00055U          93073.67556033  .00120200  00000-0  36300-3 0    33

2 00055  28.4697 228.7025 0003812 314.2100  45.8202 15.90487610    23

Enter FILENAME for 2-Line Elements:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 83

RECEIVED KEPLERIAN ORBITAL DATA FORM

------------------------------------

This form is provided as a convenience to users receiving Keplerian

orbital data via voice link (amateur radio or telephone). Each set of

underline characters indicates an expected character; the decimal point is

shown where expected (if appropriate for the item). Make multiple copies of

this form prior to a mission.

Satellite:          __ __ __ __ __ __ __ __ __ __ __ __ __ __ __

Catalog Number:     __ __ __ __ __

Int'l Designation:  __ __ __ __ __ __ __ __

Epoch Time:         __ __ __ __ __ . __ __ __ __ __ __ __ __

Element Set:        __ __ __ __

Inclination:        __ __ __ . __ __ __ __

RA of Node:         __ __ __ . __ __ __ __

Eccentricity:       . __ __ __ __ __ __ __

Arg of Perigee:     __ __ __ . __ __ __ __

Mean Anomaly:       __ __ __ . __ __ __ __

Mean Motion:        __ __ . __ __ __ __ __ __ __ __

Decay Rate:         . __ __ __ __ __ __ __ __

BSTAR Drag:         __ __ __ __ __ - __

Epoch Rev:          __ __ __ __

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 84

F2     Read/Update NASA/NORAD 2-Line Elements from *.TXT/*.TLE Files

--------------------------------------------------------------------

In order to read or update the NASA/NORAD 2-line elements, you must

have a file with data for the appropriate satellites. A current file is

included in the standard distribution of STSPLUS. These files have names

such as "TLE360.TXT" where the "360" corresponds to the particular US Space

Command Prediction Bulletin number from T. S. Kelso's Celestial BBS and are

usually updated several times per week. This and other TLE files are

available from my BBS and from various other sources including CompuServe

and INTERNET.

Users are cautioned that some sources, including NASA Spacelink, use

UNIX systems to prepare and/or format the TLE data. UNIX uses the CR

character (Carriage Return) alone as a line terminator; DOS (and STSPLUS)

use CR+LF (Carriage Return plus Line Feed) as a line terminator. Using a

UNIX-formatted TLE file with STSPLUS will usually generate the error "Data

problems with <filename>, check data format!". Some ASCII editors, such as

the DOS shareware program QEDIT by SemWare Corp., will automatically

reformat UNIX files to standard DOS format simply by reading and then

saving the file.

Select/Update Preset Frequency Selections

-----------------------------------------

STSPLUS reads file STSPLUS.FRQ for preset frequency selections for the

current satellite each time the program is started. STSPLUS will re-read

file STSPLUS.FRQ each time F2 is used to select or update the current

primary satellite. When the Doppler Shift Mode is active (selected using F8

while the map is displayed) and more than one preset frequency selection is

available for the satellite, STSPLUS will present a list of available

selections and the user may choose the desired selection. The following is

a sample list of preset frequency selections for AO-13, NORAD #19216:

Preset Frequency Selections for 19216

#      UpLink       DnLink  Mode

---------------------------------

1    435.5325     145.9375     1

2    144.4625     435.9275     1

3   1269.5475     435.9325     1

4    435.6280    2400.7380     1

Enter Desired Preset Frequency Selection Number [1]:

Enter the desired preset frequency selection number followed by ENTER. If

you wish selection #1, you may simply press ENTER. Entering a number less

than 1 or greater than the highest selection number will also pick

selection #1. If you wish to change your preset frequency selection, you

must be in the Doppler Shift Mode -- press F8 while the map is displayed.

Return to F2 on the Main Menu, re-select the desired satellite, then choose

the desired preset frequency selection. See the section "Satellite

Communications and Amateur Radio" for additional information on the Doppler

Shift Mode.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 85

NOTE: When STSPLUS is NOT in the Doppler Shift Mode, no list of preset

frequency selections is displayed and STSPLUS automatically picks selection

#1 to avoid bothering folks who are not interested in the Doppler Shift

Mode.

Update Current TDRS and Real Time Satellites

--------------------------------------------

Once Secondary Satelliets (TDRS, Static, and Real Time) have been

selected (using F6+F1 from the Main Menu), it is necessary to periodically

update the 2-line elements ("TLEs") for these satellites so that the

calculated positions are accurate. Naturally, this is also true for the

primary satellite being tracked. TLEs have a limited lifetime. For higher

altitude satellites such as geosynchronous satellites, 4 to 8 weeks is

probably sufficient unless the satellite is being maneuvered (although I

usually do this task at least weekly since I have the data). For low Earth

orbit satellites, even when the satellite is not being maneuvered, I

recommend a maximum interval of about two weeks; if the satellite is being

maneuvered, such as is usually the case with the Space Shuttle, daily

updates may be required. The Secondary Satellite display (F6+F1) flags real

time satellites whose elements are more than 10 days old, and static

satellites whose elements are more than 60 days old.

To update 2-line elements for the primary satellite and all Secondary

Satellites, press F2 from the Main Menu and select the desired file as

described in the following section. Then enter the satellite name as "&" to

request TLE update mode. The TLE file will be scanned and all satellite

TLEs will be updated if their Epoch Time is later than those presently

stored. The entire process takes only a few seconds. The following prompt

illustrates the "Update" selection:

Select NASA/NORAD 2-Line Elements File

Enter 2-Line Filename  [GSFC315.TXT]:     GSFC315.TXT

Enter Satellite Name/#nnnnn  [#20638]:    &

(Enter '*' to match any satellite name, '&' for AUTO UPDATE)

As STSPLUS updates TLEs, a list of the satellites for which new TLEs have

been loaded is displayed:

Automatic TLE updates for:

Satellite 8:  Norad# 22920 HST Solar Array @ 93355.87870989

Satellite 10: Norad# 22076 TOPEX           @ 93357.21248411

Satellite 12: Norad# 21987 EUVE            @ 93356.85536486

Satellite 7:  Norad# 21701 UARS            @ 93353.94360770

Satellite 11: Norad# 21225 GRO             @ 93356.46954065

Satellite 9:  Norad# 20638 ROSAT           @ 93353.77650216

Satellite 13: Norad# 20580 HST             @ 93357.18124168

Satellite 6:  Norad# 16609 MIR             @ 93356.89342327

Press any key to continue ...

The satellite number indicates the "slot" in the Secondary Satellite

configuration, or "P" for the Primary Satellite. In the example shown, the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 86

TLEs for eight satellites were updated to the Epoch Times (YYDDD.DDDDDDDD)

indicated. The satellites are listed in the order found in the TLE file.

The updated TLEs will be saved in file STSPLUS.INI. Press ENTER to return

to the Main Menu.

Read NASA/NORAD 2-Line Elements from a File

-------------------------------------------

For both "Read" and "Update" functions, pressing F2 will display a

list of all available files with default filetypes ".TXT" and ".TLE". The

following example has been edited to show only 8 files:

Select NASA/NORAD 2-Line Elements File

Enter 2-Line Filename  [TLE147.TXT]:

Use ARROW KEYS, press ENTER to use the current default file

shown in square brackets [...], or press ESC to CANCEL.

8 matching files in directory F:\TLE

GPS.TXT

GROUP000.TLE

GSFC198.TXT

GSFC199.TXT

MIR.TLE

TLE141.TXT

TLE146.TXT

TLE147.TXT

The list of files is sorted in alphabetical order by filename then

displayed using up to five columns. The display mode is adjusted for the

maximum number of lines permissible for the active monitor type: 25 lines

for CGA and HGC, 43 lines for EGA, and 50 lines for VGA. The maximum number

of files which may be displayed for each monitor type is shown in the

following table:

Screen     File      Max

Monitor    Lines    Lines    Files

----------------------------------

VGA           50       42      210

EGA           43       35      175

CGA/HGC       25       17       85

To accept the default file shown in the square brackets, TLE147.TXT in

the example, press ENTER. To manually enter a filename, type the name (the

filetype .TXT will be appended if no filetype is typed) and press ENTER. To

select one of the displayed files, use the ARROW KEYS (UP, DOWN, LEFT,

RIGHT), Home, End, PgUp, or PgDn to move through the list until the desired

file is highlighted and shown in the square brackets, then press ENTER. To

cancel the function and return to the Main Menu, press ESC.

STSPLUS defaults the drive and directory to the current directory, the

one from which STSPLUS is being executed. However, some users prefer to use

a separate directory for 2-line elements files. To specify a different

drive and/or directory, enter the drive (followed by a colon) and the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 87

desired directory (followed by a trailing backslash, "\"). The specified

drive, directory, and filename are saved in file STSPLUS.INI and will be

used the next time STSPLUS is executed. The following examples illustrate

this method:

D:\                 Use the root directory on drive D:

\ELEMENTS\          Use the current drive and directory

\ELEMENTS\

C:\TLE\             Use drive C: and directory \TLE\

Failing to include the trailing backslash will cause STSPLUS to interpret

what you intended as a directory to be a filename! The complete path with

filename and filetype mask may also be entered:

C:\TLE\*.TXT        Use .TXT files on Drive C: and directory TLE

D:\TLE\*.*          Display all files on drive D: and directory

\TLE\

F:\TLE\TLE*.TXT     Use drive F:, directory \TLE\ and all files

matching "TLE*.TXT"

Although STSPLUS defaults to filetypes "*.TXT" and "*.TLE", you may use

this command to temporarily specify a different filename and filetype mask

if desired.

If no files with filetype .TXT or .TLE (or files corresponding to the

current filename and filetype mask) are found in the specified directory,

the following error message will be displayed:

No matching files found in specified drive/directory: E:\JUNK

Press any key to continue ...

To specify NO fileltype, enter the filename followed by a period, i.e.

"ELEMENTS.". Any desired filetype may be used, but the program will always

default to ".TXT" and ".TLE" each time F2 is used.

If you include a drive (such as "D:") and/or directory (the directory

MUST be followed by a trailing backslash, "\"), and the drive or directory

cannot be found, the following error message will be displayed:

Drive or path error: E:\JUNK

Press any key to continue ...

Once the file has been selected, a default satellite name will appear

in the next prompt:

Enter 2-Line Filename  [STS50N38.TXT]:    STS50N38.TXT

Enter Satellite Name/#nnnnn  [STS...]:

(Enter '*' to match any satellite name, '&' for AUTO UPDATE)

STSPLUS will normally display the first three characters of a

satellite name or the full NORAD number, enclosed in square brackets, as

the default choice. If no prior satellite has been selected, the satellite

name will default to "STS..." for space shuttle missions (provided the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 88

filename begins with "STS") and to "HST..." for all other satellites;

otherwise, it will be the first three letters or full five digit NORAD

number of the currently selected satellite. NORAD numbers are always

prefixed with the "#" character. If you wish to change the information (or

if no default is shown), enter the required information followed by ENTER.

For the satellite name, only sufficient letters to unambiguously identify

the desired satellite, upper or lower case, are required. For example,

"Alou" would select "Alouette 1". However, note that entering "MIR" could

select "MIRANDA" or "MIR deb" (Mir debris) before it finds "MIR" depending

upon the ordering of the 2-line elements within the file. Alternatively,

you may enter the NORAD number for the desired satellite by entering "#"

followed by the number; leading zeroes may be omitted.

Once the information has been entered, STSPLUS will attempt to locate

the data for the requested satellite. If a satellite matching the requested

name or NORAD number is found, the data for that satellite are displayed.

Certain non-essential data are not always included in the 2-line elements

and may be replaced by spaces, indicated by "(n/a)".

Satellite Name:              Mir

Satellite NORAD Number:      #16609

Elements File:               TLE669.TXT

Elements File Record#:       465

Element Set Number:          221

Elements Epoch:              95247.20219116

04 SEP 1995 @ 04:51:09.316 UTC

Orbit # at Epoch:            54524

Int'l Designation:           1986 017A

Inclination:                 51.6459

RA of Ascend Node:           122.0823

Eccentricity:                .0003414

Arg of Perigee:              62.1935

Mean Anomaly:                297.9402

Mean Motion:                 15.57354527

Acceleration/Drag:           .00002788

2nd Deriv Mean Motn:         0

BSTAR Drag:                  .000044079

Press ENTER to ACCEPT this satellite, OR

Press any other key to REJECT and continue searching:

(*) This line is normally blank. However, one of the following

messages will appear here if a checksum error is detected in the

element set:

BAD CHECKSUM in line 1 ignored!

BAD CHECKSUM in line 2 ignored!

BAD CHECKSUM in both lines ignored!

In all three cases, STSPLUS will accept the data and attempt to use

it. Be advised, however, that the checksums are included to help

detect data errors that might otherwise yield an incorrect position!

Serious errors may even cause STSPLUS to abort with an error message.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 89

For convenience, the Elements Epoch (the instant at which these

orbital elements were calculated) is shown in two formats: the first format

is that used in the 2-line elements, YYDDD.DDDDDDDD; and the second format

is the same time converted into conventional date and time notation. You

may thus see immediately how old the elements are and take this into

account when evaluating the satellite's projected position.

If this is the satellite you wish, press ENTER and the data will be

entered into STSPLUS. If, on the other hand, a different satellite is

desired, press any other key (such as the SPACE BAR) and STSPLUS will

search for another name matching the requested satellite. For example,

there are a number of NAVSTAR Global Positioning Satellites usually

included in the file with official names such as "GPS-0001", "GPS-0002",

"GPS BII-01" and so forth; requesting "GPS" will allow you to cycle through

all the available choices. The file TLEnnn.TXT is an ASCII file; it may be

helpful to view or print the file to see the available satellite names.

Once the satellite has been selected, STSPLUS will require a brief

time to calculate certain required orbital parameters, then will proceed

directly to the display of the ground track. However, if the current

calculated altitude of the satellite is less than 75 nautical miles, the

satellite has probably decayed. STSPLUS will display the following message

before returning to the Main Menu:

Satellite MIR DEB (#22209) indicates a current altitude less than

75 nautical miles and has probably decayed. STSORBIT PLUS can NOT

process the orbital data for this satellite!

Use Function Key F2 from the Main Menu to select another

satellite and verify the satellite NAME and NORAD NUMBER.

Press any key to return to the Main Menu ...

As a point of interest, the 2-line elements for the Space Shuttle

Mission STS-50 used in the example above are as follows:

STS-50

1 22000U 92 34  A 92187.57342677 -.00032668  00000-0 -97874-4 0   380

2 22000  28.4670 275.0700 0007237 340.7929  19.1530 15.91359642  1596

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 90

F3     Data Output and Pass Prediction Selections

-------------------------------------------------

By popular request, STSPLUS has been enhanced to send selected data

for the current satellite to other equipment via a serial port (COM1 or

COM2), to a file (STSPLUS.LOG), or to the printer (LPT1). Validation of the

serial port output has been accomplished using two computers and a "Null

Modem" cable. Three classes of data may be selected for output: current

position data in three formats, precision Earth-centered inertial ("ECI")

state vectors in four formats, and tabular Line-of-Sight pass predictions

(which are also displayed on the screen).

The precision state vector outputs have been carefully coordinated

with Ken Ernandes so that they may be used as input to his program VEC2TLE,

Version 9331 or later. Using these state vectors and VEC2TLE, the user may

generate 2-line elements at any desired time (including just after the

ascending node) for use with STSPLUS or other satellite tracking programs.

With these programs, the user has a very powerful set of tools which can

be used for a variety of analytical and display purposes. Cross validation

of the two programs during Space Shuttle missions STS-56 and STS-55, as

well as comparison with US Space Command data of comparable epoch,

demonstrated high accuracy and excellent correlation.

Each Data Output function is assigned a "Data Mode" number:

1 = Azimuth, Elevation, Range

2 = Latitude, Longitude, Orbit Altitude

3 = Right Ascension, Declination

4 = Ascending Node Data with State Vector

5 = Precision X-Y-Z State Vector (2-Line Data)

6 = Precision X-Y-Z State Vector (Comma Delimited)

7 = Precision X-Y-X State Vector (Labeled Data)

8 = Doppler Shift Frequency Predictions

9 = Tabular Line-of-Sight Predictions

The current position data and precision state vectors are generated while

the ground track map is displayed; for all Data Modes EXCEPT #4, data

output may be logged continuously, for a specified time (UTC/GMT or local

time), or for a specified time span (UTC/GMT or local time). Data Mode 4

records data ONLY at the Ascending Node, e.g. when the Northbound equator

crossing is detected. The predicted pass data is calculated "off-line"

using UTC/GMT or local time, and is displayed on the screen as well as

being sent to the selected output destination.

Current position data include the UTC date/time and are generated for

local horizon coordinates (altitude and azimuth), geographic coordinates

(geocentric latitude, longitude, and orbital altitude), and topocentric

equatorial coordinates (right ascension and declination calculated for the

user's location). Precision X-Y-Z Earth-centered inertial state vectors

(ECI position and velocity components) are generated as two numeric data

lines, single line comma delimited, and multi-line labeled data. The

details for each data output format are given in following sections.

The precision ECI X-Y-Z state vectors, generated by STSPLUS for the

true equator and equinox of date, have been extensively tested and

validated in conjunction with Ken Ernandes' program VEC2TLE during Space

Shuttle missions STS-56 and STS-55 in early 1993. For example, the

combination of the two programs, STSPLUS and VEC2TLE, may be used to

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 91

convert data between 2-line and ECI formats with very high accuracy. State

vectors from STSPLUS may be read by VEC2TLE and converted into 2-line

elements, then in turn read again by STSPLUS with essentially exact

conversions. VEC2TLE has also been used during STS-56 and STS-55 to convert

real time state vectors ("M50" or Mean of 1950) supplied courtesy Willie

Musty of Mission Support, Rockwell International, into 2-line elements

equal in accuracy to those generated by US Space Command (and made

available four to eight hours sooner!). See the separate text section

describing VEC2TLE.

Note that the timing accuracy for Data Mode 4 is a fixed at 0.01

seconds, regardless of the time step (X1, X10, or X60) then in effect.

STSPLUS detects the Ascending Node data when the latitude switches from

negative to positive on the Northbound crossing of the Equator. An

iterative process is then used to refine the time to the nearest 0.01

seconds and the data at that time are recorded.

Potential applications for the position data include automatic antenna

pointing systems, off-line high precision plotting, and widespread

distribution of the data within a large facility or via modem. ECI state

vectors may be used in real time to create 2-line elements for a specified

epoch to full precision. Users who develop applications to utilize these

data are invited to contribute their programs and documentation for general

use. Since these are new features for STSPLUS, comments and suggestions are

welcome.

Although STSPLUS retains the capability of performing off-line pass

predictions with TS Kelso's TRAKSTAR or other satellite tracking software,

many users have requested that a similar feature be incorporated directly

into STSPLUS. Pass predictions may only be calculated for satellites having

a mean motion greater than 1.5; this eliminates satellites in near

geosynchronous or higher orbits, but since such satellites don't move much

that does not represent a significant constraint. The satellite's orbit is

examined for 48 hours, starting at the current real or simulated time, with

a sampling interval which ranges from 10 to 60 seconds depending upon the

orbit. Because of this "granularity" in the search algorithm, it is

possible to skip passes whose duration is less than the sampling interval.

Since those brief passes would barely peek above the user's horizon, they

are thus not significant. Pass predictions may be continued in 48 hour

segments until 99 passes have been displayed, at which point the assigned

pass number is reset for subsequent pass predictions. Since a typical

satellite may have from about two to seven passes in a 24 hour period, the

passes may be examined for a considerable time into the future.

Predicted pass data are calculated using the current satellite for

Line-of-Sight visibility; that is, when the satellite is in line of sight

to the user's location and without consideration of lighting effects.

Passes which are visible, the satellite is lighted by the Sun and the

user's location is in darkness, are flagged and displayed in WHITE; these

passes normally occur near local sunrise or sunset. Since STSPLUS users are

about equally divided between those who track satellites visually and those

who use electronic equipment such as amateur radio, this method provides

data for all users. Dates and times may be displayed in Coordinated

Universal Time (UTC/GMT) or in local time. Note that the date for each pass

is given only for AOS (Acquisition of Signal); it is possible for the pass

to span 00:00:00 hours (midnight) for the time scale in use with a

consequent date change during the pass for MAX VISIBILITY and/or LOS (Loss

of Signal).

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 92

Prediction calculations may require some time; calculation delays are

noted with the message "calculating ...". Using a processor equipped with a

math coprocessor chip, each 48 hour block may require from less than 10

seconds to a minute or more. However, users without math coprocessor chips

will experience significant delays -- minutes or even tens of minutes! The

following table lists typical calculation times for various processors (all

with math coprocessors!) using the Russian MIR Space Station:

286/287        386SX/387SX         486DX

8 MHz          20 MHz              33 MHz

---------------------------------------------------

MIR       60 sec         30 sec              6 sec

The data output feature MUST be enabled with F3 each time STSPLUS is

run; it is NOT automatically restarted when the "/R" (RESUME) command line

option is used.

**********

*  NOTE  *

**********

Users are reminded that when data output is sent to the file

STSPLUS.LOG, a considerable volume of data may be accumulated

over long periods of time. It is possible to completely fill a

disk with this data! The file should be periodically copied to

other media or deleted to avoid this problem.

Setting Up Position and State Vector Data Output

------------------------------------------------

Data output of position and state vectors ONLY occurs while the

ground track is displayed; no data are generated until the ground track is

actually in progress! The appropriate data are sent to the destination

device at the selected data interval (continuous), at a specified time, or

at the selected data interval over a specified time span covering no more

than 24 hours.

************************

*  IMPORTANT REMINDER  *

************************

STSPLUS generates Earth-Centered Inertial ("ECI") state vectors

for the true equator and equinox of date. Other software and

various agencies may use different coordinate systems. In

particular, NASA uses the mean equator and equinox of the

Besselian year 1950 ("Mean of 1950", "M50" or "B1950").

Astronomers and other agencies may use the mean equator and

equinox of the Julian year 2000 ("Mean of 2000" or "J2000").

Other agencies, such as the DOD C-Band Radar Network, use a time-

independent coordinate system ("Earth-fixed Greenwich" or "EFG")

for predicted state vectors prior to a launch. Users must take

care that the appropriate coordinate system is used for each

application and/or perform the required conversions.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 93

STSPLUS sets up certain initial default parameters for data output and

displays the current parameters each time F3 is pressed, as shown in the

following example:

STSORBIT PLUS Data Output Parameters:

Data Output:                  STSPLUS.LOG

Data Format:                  7 = Precision X-Y-Z State Vector

Data Interval:                60 seconds (continuous)

Data Units:                   Kilometers, Multi-Line Labeled

Accept Parameters [Y,n,x]:

For Data Modes 8 and 9, the final prompt includes the option to display the

data on the screen:

Accept Parameters [Y,n,s,x]:

To cancel data output and return to the Main Menu, press "X". If the

current parameters are correct, press "Y" (or ENTER) to accept them. If the

parameters are to be changed or if a specified time or time span is

desired, press "N" to be prompted for new parameters. In each case, the

default value which will be used if ENTER is pressed will be shown in

square brackets; if more than one choice is shown, separated by commas, the

first choice will be used if ENTER is pressed. The user must first select

the data output device or destination by pressing the indicated key:

Select Output [F,p,1,2]:

F = File STSPLUS.LOG

P = Printer LPT1:

1 = Serial Port COM1:

2 = Serial Port COM2:

Pressing ENTER or the letter "F" (upper or lower case) will select the FILE

output and the data will be sent to the file STSPLUS.LOG. If the file does

not exist, it will be created; if the file already exists, the data will be

appended to the existing data. Press the letter "P" to direct the data to

the printer on LPT1. Press "1" or "2" to direct the data to one of the two

serial ports.

When a serial port (COM1: or COM2:) is selected, the user next selects

the data rate to be used for communications with the external equipment.

Only the four data rates shown below the prompt are supported. Use the

first character of the desired rate to select it, or press ENTER to use the

data rate shown in the square brackets:

Select DATA RATE [9600]:

(300, 1200, 2400, 9600)

STSPLUS automatically sets the communications parameters to "8,N,1"; these

are fixed and may not be altered. These communications parameters select 8

data bits per transmitted byte, NO parity, and 1 stop bit. Most external

equipment will operate satisfactorily with these parameters.

STSPLUS requires several additional items of information before it can

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 94

send data to the external equipment, file or printer. The first is the data

format to be used. Eight different data formats are available. The next

prompt lists the formats and shows the current default in square brackets:

Select Data to Output [7]:

1 = Azimuth, Elevation, Range

2 = Latitude, Longitude, Orbit Altitude

3 = Right Ascension, Declination

4 = Ascending Node X-Y-Z State Vector

5 = Precision X-Y-Z State Vector (2-Line Data)

6 = Precision X-Y-Z State Vector (Comma Delimited)

7 = Precision X-Y-X State Vector (Labeled Data)

8 = Doppler Shift Frequency Predictions

9 = Tabular Line-of-Sight Predictions

Press the number key corresponding to the desired Data Mode or press ENTER

to select the choice shown in square brackets. See the Data Mode Formats in

the following sections for specific details on the data included in each

data mode.

For current position and state vector formats except Data Mode 4, the

desired time interval between successive sets of data must be also

selected. (Data Mode 4 records data immediately after the Ascending Node

and does not use the time interval parameter.) Any interval between 1 and

900 seconds may be selected (that is, up to 15 minutes maximum). Add the

letter "T" or "t" if you wish the data to be logged for a specific time or

time span. Note also that this is the DESIRED time interval; if your

computer is too slow to complete its calculations in that time, the

interval will be longer. In other words, STSPLUS will generate the

requested data no more frequently than the interval requested but may take

longer, depending upon what has to be done each time. In response to the

prompt, press ENTER to accept the default value shown in square brackets or

type the desired numerical value (in seconds) followed by ENTER:

Data Interval (secs) [60]:

(Min = 1 sec, Max = 900 secs; Add 'T' for timer)

If a value less than 1 second is entered, 1 second will be substituted; if

a value greater than 900 seconds is entered, 900 will be substituted. If

"T" is entered by itself, the default value shown in square brackets will

be used for the Data Interval.

If "T" is appended to the desired interval or is entered by itself,

STSPLUS requests the Start Time for logging:

Start Time (HH:MM:SS):

(Add 'U' or 'G' for UTC/GMT)

Enter the desired LOCAL Start Time or add the letter "U" or "G" for UTC/GMT

time. STSPLUS will reformat the entered time and add the appropriate time

zone designation, then prompt for the Stop Time:

Start Time (HH:MM:SS):        08:45:00 PDT

Stop Time (HH:MM:SS):

(Press ENTER for Stop Time = Start Time)

Enter the desired Stop Time using the SAME time scale used for Start Time,

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 95

or press ENTER to use the Start Time. STSPLUS will reformat the entered

time and add the appropriate time zone designation.

Stop Time (HH:MM:SS):         08:50:00 PDT

When Start Time equals Stop Time, only one set of data will be recorded.

Reasonable care is required when setting up these times. If the current

time (real or simulated) is past the Start Time, data will be recorded

immediately. Times may be set to less than 24 hours into the future.

When state vectors are requested (Data Modes 5 through 7), STSPLUS

must also know the desired units of measure, kilometers ("km" or "KM"),

feet ("ft" or "FT"), or nautical miles ("nm" or "NM"):

Data Units [KM,ft,nm]:

(Press 1st letter to select)

Press the first letter of the desired units of measure or press ENTER for

the current default units of measure (shown in capital letters in the

prompt, "KM" in the example above).

STSPLUS now displays the new parameters for approval:

STSORBIT PLUS Data Output Parameters:

Data Output:                  STSPLUS.LOG

Data Format:                  7 = Precision X-Y-Z State Vector

Data Interval:                60 seconds, 08:45:00-08:50:00 PDT

Data Units:                   Kilometers, Multi-Line Labeled

Accept Parameters [Y,n,x]:

If no Start and Stop Times have been entered, "(continuous)" will appear in

place of the Start and Stop Times.

As before, press "Y" (or ENTER) to accept the parameters and enable

data output, "N" to re-enter the parameters, or "X" to cancel data output

and return to the Main Menu.

Setting Up Tabular Pass Predictions

-----------------------------------

STSPLUS provides two Data Modes, 8 and 9, to generate tabular pass

prediction data. These two Data Modes may also be used simply to display

passes of interest (with the generated data not used or discarded).

8 = Doppler Shift Frequency Predictions

---------------------------------------

Data Mode 8, Doppler Shift Frequency Predictions, displays

upcoming satellite passes over the user's primary location for 48-hour

periods. When a pass is selected and displayed, the map is drawn with

simulated time set to just before the beginning of the pass. As the

pass proceeds, the Doppler Shift information is calculated and sent to

the selected output device.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 96

9 = Tabular Visible and Line-of-Sight Predictions

-------------------------------------------------

Data Mode 9, Tabular Visible and Line-of-Sight Predictions,

displays (and optionally sends to an output device) data for upcoming

satellite passes over the user's primary location during 48-hour

periods. The user may select a particular pass and cause simulated

time to be set to the middle of the pass. The map is drawn and the

user may view the pass to determine any additional information of

interest.

The appropriate data are displayed and sent to the destination device at

the selected data interval (continuous), at a specified time, or over a

specified time span covering no more than 24 hours. The user may simply

view the tabular pass information or he may select a specific pass to view

on the map.

Data Mode 8, the Doppler Shift Frequency Predictions, can be

particularly helpful in preparing for a satellite amateur radio contact by

printing the resulting frequency predictions for use during the contact. In

this data mode, the tabular data are generated as a selected pass is

displayed; the data include date and time, satellite geodetic coordinates

(latitude and longitude), horizon coordinates (elevation and azimuth), and

the frequency differences from the specified uplink and downlink center

frequencies. See the format description for Data Mode 8 below for

additional details. The pass selection list is NOT sent to the output

device in this Data Mode.

STSPLUS sets up certain initial default parameters for tabular pass

predictions and displays the current parameters each time F3 is pressed, as

shown in the following example:

STSORBIT PLUS Data Output Parameters:

Data Output:                  STSPLUS.LOG

Data Format:                  9 = Tabular Line-of-Sight Predictions

(Using PDT for times)

Accept Parameters [Y,n,s,x]:

To cancel data output and generate pass predictions on the screen

ONLY, press "X". If the current parameters are correct, press "Y" (or

ENTER) to accept them or press "S" if output to the screen only is desired.

If the parameters are to be changed or if a specified time is desired,

press "N" to be prompted for new parameters. In each case, the default

value which will be used if ENTER is pressed will be shown in square

brackets; if more than one choice is shown, separated by commas, the first

choice will be used if ENTER is pressed. The user must first select the

data output device or destination by pressing the indicated key:

Select Output [F,p,1,2]:

F = File STSPLUS.LOG

P = Printer LPT1:

1 = Serial Port COM1:

2 = Serial Port COM2:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 97

Pressing ENTER or the letter "F" (upper or lower case) will select the FILE

output and the data will be sent to the file STSPLUS.LOG. If the file does

not exist, it will be created; if the file already exists, the data will be

appended to the existing data. Press the letter "P" to direct the data to

the printer on LPT1. Press "1" or "2" to direct the data to one of the two

serial ports.

When a serial port (COM1: or COM2:) is selected, the user next selects

the data rate to be used for communications with the external equipment.

Only the four data rates shown below the prompt are supported. Use the

first character of the desired rate to select it, or press ENTER to use the

data rate shown in the square brackets:

Select DATA RATE [9600]:

(300, 1200, 2400, 9600)

STSPLUS automatically sets the communications parameters to "8,N,1"; these

are fixed and may not be altered. These communications parameters select 8

data bits per transmitted byte, NO parity, and 1 stop bit. Most external

equipment will operate satisfactorily with these parameters.

STSPLUS requires several additional items of information before it can

send data to the external equipment, file or printer. The first is the data

format to be used. Five different data formats are available. The next

prompt lists the formats and shows the current default in square brackets:

Select Data to Output [7]:

1 = Azimuth, Elevation, Range

2 = Latitude, Longitude, Orbit Altitude

3 = Right Ascension, Declination

4 = Ascending Node X-Y-Z State Vector

5 = Precision X-Y-Z State Vector (2-Line Data)

6 = Precision X-Y-Z State Vector (Comma Delimited)

7 = Precision X-Y-X State Vector (Labeled Data)

8 = Doppler Shift Frequency Predictions

9 = Tabular Line-of-Sight Predictions

Press the "8" or "9" number key to select one of the pass predictions or

press ENTER to select the choice shown in square brackets. See the Data

Mode Formats in the following sections for specific details on the data

included in each data mode.

For tabular line-of-sight predictions, STSPLUS must know the time zone

for which data is to be displayed. The choices are UTC/GMT or the LOCAL

time zone:

Use UTC or PDT time [PDT]:

(Use LEFT LETTER of abbreviation to select);

Depending upon the user's choice when the UTCOffset was set, either "UTC"

or "GMT" will be displayed along with the abbreviation for the local time

zone. Use the left-most letter of the desired time zone, or press ENTER to

accept the time zone shown in the square brackets ("[PDT]" in the example).

STSPLUS now returns to the initial parameter display and again asks if

the parameters are correct. As before, press "Y" to proceed with pass

predictions AND sending the data to the specified destination device, "N"

to change parameters, or "X" return to the Main Menu and display the pass

predictions on the screen WITHOUT sending the data to a destination device.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 98

For both pass prediction modes, the data are displayed on the screen

as calculated and optionally (for Pass Predictions ONLY) sent to the

selected destination device. The following is a sample of the output data

(the screen display is similar) for Space Shuttle Mission STS-69 in

September of 1995:

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 19

Location:          Palos Verdes, CA

Prepared:          Saturday, 09 Sep 1995  17:02:42

Satellite Name:    STS-69

Catalog Number:    23667  95048A

TLE Filename:      STS69R04.TXT

-----------AOS-----------  --MAX VISIBILITY--  ------LOS------

#    PDT Date & Time    Azm  PDT Time Alt   Azm  PDT Time    Azm Duration V

1 10 SEP 95 06:24:57  196.9  06:28:03   4 159.4  06:31:10  121.7  0:06:13

2 10 SEP 95 08:00:25  228.0  08:04:43  12 171.1  08:09:02  114.1  0:08:37

3 10 SEP 95 09:37:04  243.6  09:41:31  14 183.6  09:46:00  123.5  0:08:56

4 10 SEP 95 11:14:25  244.4  11:18:17   8 195.7  11:22:08  147.1  0:07:43

5 11 SEP 95 05:21:27  178.6  05:23:25   2 155.9  05:25:23  133.0  0:03:56

6 11 SEP 95 06:55:58  219.5  07:00:00  10 167.1  07:04:06  114.3  0:08:08

7 11 SEP 95 08:32:19  239.9  08:36:47  14 179.5  08:41:17  118.8  0:08:58

8 11 SEP 95 10:09:25  245.9  10:13:35  11 191.6  10:17:46  137.5  0:08:21

9 11 SEP 95 11:47:44  231.9  11:50:13   3 202.9  11:52:40  174.2  0:04:56

10 12 SEP 95 05:51:39  209.2  05:55:20   7 162.9  05:59:01  116.8  0:07:22 *

11 12 SEP 95 07:27:39  234.5  07:32:03  13 175.1  07:36:29  115.7  0:08:50

12 12 SEP 95 09:04:30  245.5  09:08:51  13 187.6  09:13:13  129.6  0:08:43

13 12 SEP 95 10:42:12  240.2  10:45:34   5 199.2  10:48:54  158.5  0:06:42

14 13 SEP 95 04:47:37  196.0  04:50:39   4 159.3  04:53:41  122.5  0:06:04

15 13 SEP 95 06:23:02  227.6  06:27:19  12 170.8  06:31:36  114.1  0:08:34 *

16 13 SEP 95 07:59:40  243.4  08:04:07  14 183.3  08:08:34  123.4  0:08:54

17 13 SEP 95 09:37:00  244.4  09:40:52   8 195.4  09:44:43  146.8  0:07:43

When the calculations have been completed, the user is prompted:

Elapsed time = 4.6 seconds

Repeat for NEXT 48 HOURS or DISPLAY PASS [N,y,pass#]:

The final prompt for Doppler Shift Frequency Predictions is similar:

Repeat for NEXT 48 HOURS or CALCULATE DOPPLER [N,y,pass#]:

The initial information ("Location through "TLE Filename") is sent only to

file STSPLUS.LOG and is omitted in the screen version; the screen version

includes "#23667 AOS" in the column heading to identify the satellite. The

initial information is:

Location            Current primary location

Prepared            Computer date and time of preparation

Satellite Name      Satellite common name or mission name

Catalog Number      NORAD Number and International Designation

TLE Filename        2-Line file used for data calculations

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 99

The column headings indicate the data displayed. "AOS" is Acquisition of

Signal or when the satellite rises above the user's true horizon. "MAX

VISIBILITY" is the maximum altitude above the user's true horizon that the

satellite reaches during the pass. "LOS" is Loss of Signal or when the

satellite sets below the user's true horizon. "#" is an arbitrary pass

number for this set of calculations. The format for the data sent to the

destination device is slightly different from that displayed; see the Data

Mode 9 format description below.

For Pass Predictions, the user may select either UTC/GMT or LOCAL date

and times for pass predictions. If other than UTC is selected, substitute

the appropriate time zone abbreviation as required. The date is given as dd

MMM yy (where: "dd" is the day of the month, "MMM" is the alphabetic

abbreviation for the month, and "yy" is the last two digits of the year)

and only for AOS; since passes may span 00:00:00 hours for the selected

time zone, the actual date for MAX VISIBILITY and/or LOS may have to be

incremented.

Times are calculated to the nearest second, altitudes are rounded to

the nearest degree, and azimuths are rounded to the nearest tenth of a

degree. Azimuth is always calculated in the sense NESW where North = 0

degrees, East = 90 degrees, etc. Note that the degree symbol will appear on

the display for all "Azm" and "Alt" data but has been deleted in the sample

above in order to maintain compatibility with various printers; the actual

display is thus four columns wider than the example above.

STSPLUS uses color to enhance the Pass Prediction display and make

visible passes more obvious to the user. Users with a monochrome monitor

may have to examine candidate passes using the map display to obtain the

same information. The orbit of the primary satellite is first examined for

line-of-sight passes (as in previous versions). For each pass which rises

above the horizon of the user's location, four times are calculated: "AOS",

the Acquisition of Signal; "MAX", the time of maximum satellite altitude;

"LOS", the Loss of Signal; and "Duration", the total duration of the pass

from AOS to LOS. Each pass is then examined for lighting conditions at AOS,

MAX, and LOS. If the satellite is in full or partial sunlight and the

user's location is in darkness, and the satellite is at least five degrees

above the true horizon, the pass is a VISIBLE PASS and the satellite may be

visible to the naked eye or binoculars if the satellite is sufficiently

large and other geometric conditions (along with the weather) are suitable;

these visibility conditions are indicated by the color of the information:

NOT VISIBLE         VISIBLE

-------------------------------------------------------------

AOS       Bright Green        White

MAX       Bright Cyan         Bright White (Altitude >=5 deg)

Cyan                White        (Altitude < 5 deg)

LOS       Bright Red          White

These visibility conditions may appear in a number of combinations. A

satellite may be visible at AOS (or LOS) but not at MAX, the satellite may

not rise five or more degrees above the user's horizon yet still be

technically visible under essentially perfect conditions, and so forth.

Further, since the visibility conditions are only tested at three points in

the pass, it is possible that certain very low altitude visible passes may

not be marked with the appropriate "VISIBLE" color; it is unlikely that

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 100

these passes would actually be visible in any event. To aid the user (and

particularly the user with a monochrome display), passes which are visible

at MAX are marked with "*" at the far right in the columy labeled "V".

SAMPLE VISIBLE PASS PREDICTIONS FOR MIR SPACE STATION

-----------------------------------------------------

--------#16609 AOS-------  --MAX VISIBILITY--  ------LOS------

#    PDT Date & Time    Azm  PDT Time Alt   Azm  PDT Time    Azm Duration V

1 24 APR 95 20:31:54  329.2  20:34:09   2 355.1  20:36:26   21.4  0:04:32

2 24 APR 95 22:08:52  338.3  22:12:06   4  16.8  22:15:16   54.7  0:06:24

3 24 APR 95 23:44:22  325.0  23:49:18  22  35.5  23:54:11  105.0  0:09:49

4 25 APR 95 01:20:29  301.7  01:25:28  27 229.2  01:30:26  157.2  0:09:57

5 25 APR 95 14:46:12  141.7  14:47:56   1 121.8  14:49:44  101.3  0:03:32

6 25 APR 95 16:18:20  214.6  16:23:28  49 132.8  16:28:37   52.0  0:10:17

7 25 APR 95 17:55:15  266.1  17:59:50  15 328.5  18:04:28   31.2  0:09:13

8 25 APR 95 19:34:30  315.8  19:37:15   3 348.0  19:40:01   20.4  0:05:31

9 25 APR 95 21:12:39  339.7  21:15:13   2   9.6  21:17:47   39.3  0:05:08

10 25 APR 95 22:48:16  330.6  22:52:45  12  29.6  22:57:09   87.7  0:08:53 *

11 26 APR 95 00:24:01  311.2  00:29:15  72 226.2  00:34:27  138.8  0:10:26

12 26 APR 95 02:01:56  271.4  02:04:52   4 236.8  02:07:44  202.9  0:05:48

13 26 APR 95 15:22:31  195.5  15:27:15  20 129.3  15:32:02   62.4  0:09:31

14 26 APR 95 16:58:19  248.8  17:03:19  27 322.7  17:08:20   37.1  0:10:01

15 26 APR 95 18:36:58  299.5  18:40:23   5 341.0  18:43:50   22.6  0:06:52

Several types of visible pass conditions are illustrated by the sample

data for the Russian Mir Space Station, reproduced above. In this example,

all three test points (AOS, MAX, and LOS) are indicated in White for Pass 1

and Pass 8; however, since both are very low passes (2 and 3 degrees,

respectively), no special flag is shown at the right and it would be

unlikely that the pass could actually be seen. Pass 9 shows both AOS and

MAX in White but again it is a low pass so no flag is shown at the right.

Pass 10 shows AOS in White but MAX in Bright Cyan and includes the "*"

special flat at the right; this pass happens to have a 12 degree MAX

altitude but the satellite disappears into the Earth's shadow approximately

80 seconds (at an altitude of approximately 8 degrees) after it breaks the

horizon and might be visible under good conditions. The sample was

deliberately chosen to illustrate the difficulties involved. Ideally, a

"good" visible pass will be shown in Bright White at AOS, have a MAX

altitude of 25 or more degrees, and will be indicated by the special "*"

flag at the right. Clear skies are assumed for all viewing conditions.

Satellite viewers are reminded that satellite size and shape as well

as the physical geometry of the situation can affect visibility. For

example, a large satellite positioned between the viewer and the Sun may

reflect little or no sunlight towards the viewer and thus not be visible.

Conversely, a small satellite with a highly reflective surface (such as a

solar panel) which happens to be ideally positioned at a given instant, may

produce a brilliant flash on a particular night but nothing the next night.

Other satellites (and especially debris objects such as rocket bodies) may

be rotating or tumbling and appear to flash from time to time. The specific

circumstances which make one satellite visible and another not are too

complex for exact analytical solution given the data available. The best

that can be done is to estimate when the circumstances are such that naked

eye visibility is of relatively high probability for larger, low Earth

orbit satellites. MIR, GRO, HST, and the Space Shuttle are ideal targets.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 101

When Launch Simulation is enabled (a launch time and date plus launch

pad selection has been made using F5 from the Main Menu), STSPLUS adjusts

the start time for prediction calculations to MET = 0/00:40:00 or forty

minutes after launch (by which time the orbiter will have reached the

expected orbit). Pass predictions are not available during the ascent phase

of a launch. The start time used is also converted to conventional date and

time and an additional message will appear above the data for Pass 1 on

both the screen and the selected device output:

(Start time adjusted to MET = 0/00:40:00 or 08 JUN 1995 @ 15:00:00 UTC)

Therefore, the user may view pass predictions for an upcoming launch

without cycling through days or weeks of meaningless data. Note, however,

that if a launch pad selection has NOT been made, STSPLUS will use the

estimated 2-line elements to calculate prediction data.

Press ENTER while the passes are being calculated to stop the

calculations. When all calculations for the current 48 hour block have been

completed, the time elapsed for the calculations is displayed and the user

is asked if another set of calculations is desired or if a particular pass

should be displayed. Press "N" or ENTER to return to the Main Menu, or

press "Y" to perform the next 48 hour block of calculations. The

calculations will be stopped when 99 passes have been listed. Typically,

from four to sixteen passes are listed for each 48 hour block, the number

being related to the characteristics of the current satellite's orbit. If

additional passes beyond that time are desired, set simulated time (F8+F3

from the Main Menu) to the desired start time and repeat the predictions.

To display a particular pass, enter the listed pass number (the number

in the far left column marked "#"). Passes are assigned arbitrary numbers

from 1 to 99 beginning with the first pass which occurs during or

subsequent to the current real or simulated time. Any pass number may be

selected, from pass #1 to the last listed pass shown on the screen.

(Attempting to enter a pass number larger than the last one shown will

cause the computer to "beep" and the prompt will be repeated.) STSPLUS will

set simulated time to approximately 30 seconds prior to the mid-point of

the selected pass and prepare the display. The pass may then be examined

for details of lighting, ground track, and so forth. STSPLUS displays "VIS"

next to the orbit inclination if a visual sighting may be possible. While

examining the pass, use F6 to PAUSE the display, then use the "+" or "-"

keys to adjust the simulated time forward or backward. The default time

step is one second; press F4 to select a different time step: 1, 10, or 60

seconds. Press ENTER to return to normal operation from the PAUSE mode.

For Doppler Shift Frequency Predictions only, the displayed pass

begins just before AOS (Acquisition of Signal) instead of near the maximum

elevation. Doppler shift frequency data will be sent to the output device

only after AOS is reached and until LOS (Loss of Signal) is reached. The

user may use "fast time" to speed up the operation but there may be some

loss of accuracy. Press ENTER once the pass has been completed to return to

the Main Menu.

Each time predicted passes are requested, the list begins with the

first pass which occurs at or subsequent to the current real or simulated

time. Note that STSPLUS automatically sets simulated time to display a

predicted pass, and that new simulated time remains in effect until the

user returns to the Main Menu, at which point the time is restored to the

real or simulated time in effect BEFORE the pass prediction was displayed.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 102

Repeated use of pass predictions will therefore generally produce the same

list of passes; however, if sufficient time elapses between predictions

that a pass "comes and goes", new pass numbers will be displayed.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 103

Data Mode 1: Azimuth/Elevation Data Format

------------------------------------------

1         2         3         4         5

0123456789012345678901234567890123456789012345678901234

-------------------------------------------------------

02/10/1993  13:58:09  20580    -2.472   248.222    1675 [CR/LF]

-----+----  ----+---  --+--  ----+---  ----+--- ---+---

|          |       |        |         |       |

|          |       |        |         |       +--- Range

|          |       |        |         |

|          |       |        |         +----------- Aximuth

|          |       |        |

|          |       |        +--------------------- Elevation

|          |       |

|          |       +------------------------------ NORAD #

|          |

|          +-------------------------------------- UTC Time

|

+------------------------------------------------- UTC Date

UTC Date:      Current date in Universal Coordinated Time, mm/dd/yyyy

UTC Time:      Current time in Universal Coordinated Time, hh:mm:ss

NORAD #:       Satellite NORAD Number

Azimuth:       Azimuth in degrees to satellite in the sense NESW

Elevation:     Elevation to satellite in degrees above true horizon

Range:         Range from User Location to Satellite in km

[CR/LF]:       Each data line is terminated with a CR and LF in addition to

the 54 printing characters shown, for a total of 56

characters per data line.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 104

Data Mode 2: Latitude/Longitude Data Format

-------------------------------------------

1         2         3         4         5

0123456789012345678901234567890123456789012345678901234

-------------------------------------------------------

02/11/1993  13:46:40  20580    -5.182   155.667     593 [CR/LF]

-----+----  ----+---  --+--  ----+---  ----+--- ---+---

|          |       |        |         |       |

|          |       |        |         |       +--- Orbit Altitude

|          |       |        |         |

|          |       |        |         +----------- Longitude

|          |       |        |

|          |       |        +--------------------- Latitude

|          |       |

|          |       +------------------------------ NORAD #

|          |

|          +-------------------------------------- UTC Time

|

+------------------------------------------------- UTC Date

UTC Date:      Current date in Universal Coordinated Time, mm/dd/yyyy

UTC Time:      Current time in Universal Coordinated Time, hh:mm:ss

NORAD #:       Satellite NORAD Number

Latitude:      Geodetic Latitude in degrees of sub-satellite point

(satellite ground track)

Longitude:     Geodetic Longitude in degrees of sub-satellite point

(satellite ground track)

Orbit Alt:     Altitude in kilometers of the satellite above the Earth's

surface

[CR/LF]:       Each data line is terminated with a CR and LF in addition to

the 54 printing characters shown, for a total of 56

characters per data line.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 105

Data Mode 3: Topocentric RA/DEC Data Format

-------------------------------------------

1         2         3         4

01234567890123456789012345678901234567890123456

-----------------------------------------------

02/11/1993  13:47:20  20580     7.111   -25.941 [CR/LF]

-----+----  ----+---  --+--  ----+---  ----+---

|          |       |        |         |

|          |       |        |         +----------- DEC

|          |       |        |

|          |       |        +--------------------- RA

|          |       |

|          |       +------------------------------ NORAD #

|          |

|          +-------------------------------------- UTC Time

|

+------------------------------------------------- UTC Date

UTC Date:      Current date in Universal Coordinated Time, mm/dd/yyyy

UTC Time:      Current time in Universal Coordinated Time, hh:mm:ss

NORAD #:       Satellite NORAD Number

RA:            Topocentric Right Ascension in hours

DEC:           Topocentric Declination in degrees

[CR/LF]:       Each data line is terminated with a CR and LF in addition to

the 46 printing characters shown, for a total of 48

characters per data line.

NOTES:

1.   Topocentric coordinates give the right ascension and declination as

seen from the current user's location on the surface of the Earth.

2.   Coordinates are equator and equinox of date.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 106

Data Mode 4: Ascending Node X-Y-X State Vector

----------------------------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 4

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93206.71622934028

07/25/1993  17:11:22.215 UTC

ECI X:                   6083.74442210995 km

Y:                   2969.71930867257 km

Z:                      0.01043524694 km

Xdot:                  -2.09290827983 km/sec

Ydot:                   4.27922666083 km/sec

Zdot:                   6.01892329735 km/sec

Ndot/2 (Drag):              0.00056174000

Nndot/6:                    0.00000000000

B-Star:                     0.00071196000

ElSet #:                  196.00000000000

Rev @ Epoch:            42514.00433526011

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 14

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93206.71622934028

07/25/1993  17:11:22.215 UTC

ECI X:               19959752.12027331000 ft

Y:                9743154.40544174400 ft

Z:                     34.23630116129 ft

Xdot:               -6866.48346437341 ft/sec

Ydot:               14039.42991197058 ft/sec

Zdot:               19747.08480675116 ft/sec

Ndot/2 (Drag):              0.00056174000

Nndot/6:                    0.00000000000

B-Star:                     0.00071196000

ElSet #:                  196.00000000000

Rev @ Epoch:            42514.00523843931

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 24

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93206.71622934028

07/25/1993  17:11:22.215 UTC

ECI X:                   3284.95919120368 nm

Y:                   1603.52014507239 nm

Z:                      0.00563458372 nm

Xdot:                  -1.13008006471 nm/sec

Ydot:                   2.31059754904 nm/sec

Zdot:                   3.24995858388 nm/sec

Ndot/2 (Drag):              0.00056174000

Nndot/6:                    0.00000000000

B-Star:                     0.00071196000

ElSet #:                  196.00000000000

Rev @ Epoch:            42514.00325144509

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 107

NOTES:

1.   The X-Y-Z Cartesian State Vector is given as a standard Earth-centered

inertial ("ECI") cartesian 6-dimensional state vector where the X-Axis

is pointing toward the vernal equinox, the Z-Axis is pointing toward

the North Pole, and the Y-Axis is mutually orthogonal to the other

axes in a right-handed axis system. All coordinates are for true

equator and equinox of date.

2.   The units of measure for the state vector may be determined by the

tens digit of the Data Mode in the initial header line as well as

being indicated with the data:

4   Kilometers and kilometers per second

14   Feet and feet per second

24   Nautical miles and nautical miles per second

3.   One data item is given per line, labeled as shown in the examples. The

data in the first four lines (Satellite Name, Catalog Number, and two

lines of Date/Time) begin in column 25. The remaining numeric data

items begin in column 21 and use a FORTRAN-like format statement

F21.11.

4.   The Catalog Number is given first as the NORAD Number ("16609" in the

example) and then as the International Designation ("86017A"). The

International Designation is from 1 to 3 letters. Some 2-line elements

omit the International Designation, in which case that portion will be

blank.

5.   Note that the Date/Time is presented on two lines in two different

formats. The first format is the NASA Day-of-Year ("DOY") format,

YYDDD.DDDDDD, since that is the format used by NASA/JSC for X-Y-Z

state vectors for the Space Shuttle, in 2-line elements, and in

program VEC2TLE. In the DOY format, time is counted from midnight

(00:00 UTC) each day. Some calculations may require instead the Julian

Date format which counts time from noon (12:00 UTC) each day. The

Date/Time is also "decoded" and given in the more conventional

"MM/DD/YYYY  HH/MM/SS.SSS" format for clarity using Coordinated

Universal Time (UTC/GMT).

6.   The ElSet Number is specified in the 2-line elements used to generate

the ground track and is always given as an integer. Note that ElSet

Numbers may not necessarily follow in sequence and that different

sources will use different sequences of ElSet numbers.

7.   The Rev Number at Epoch (the orbit number at the time the data is

sampled) is based upon the Rev Number specified in the 2-line elements

used to generate the ground track. The fractional part is calculated

by STSPLUS geometrically from the ascending node. Note that US Space

Command uses a different orbit numbering convention than does NASA for

space shuttle missions; US Space Command usually specifies the first

(partial) orbit number as Rev 0, while NASA specifies that orbit

number as Rev 1. For satellites which have been in orbit for long

periods of time, the Rev Number may be arbitrary.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 108

Data Mode 5: Precision X-Y-Z Cartesian State Vector, 2 Data Lines

-----------------------------------------------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 5

20580  93110.043125      4920.98348         4440.02814        -2158.84295

-4.02147461570      5.78870948196      2.74131815428

20580  93110.043171      4904.85124         4463.14112        -2147.85724

-4.04461763461      5.76773962933      2.75148946765

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 15

20580  93110.045081  13656864.66720     17514322.54968     -5452252.42794

-16168.27686974290  15789.75251859515  10248.33566657315

20580  93110.045139  13575822.39276     17593013.13238     -5400930.14914

-16248.48336702945  15686.35334359047  10280.38786725583

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 25

20580  93110.046829      1817.50246         3234.50460         -631.72242

-3.02340746009      2.05326951871      1.82336359537

20580  93110.046991      1774.96638         3262.87575         -606.12299

-3.05304428709      1.99966800523      1.83360328215

NOTES:

1.   The X-Y-Z Cartesian State Vector is given as a standard Earth-centered

inertial ("ECI") cartesian 6-dimensional state vector where the X-Axis

is pointing toward the vernal equinox, the Z-Axis is pointing toward

the North Pole, and the Y-Axis is mutually orthogonal to the other

axes in a right-handed axis system. All coordinates are for true

equator and equinox of date.

2.   The units of measure for the state vector may be determined by the

tens digit of the Data Mode in the initial header line:

5   Kilometers and kilometers per second

15   Feet and feet per second

25   Nautical miles and nautical miles per second

3.   The NASA Day-of-Year format is used here for date and time since that

is the format used by JSC for X-Y-Z state vectors and also in 2-line

elements. In the DOY format, time is counted from midnight (00:00 UTC)

each day. Some calculations may require instead the Julian Date format

which counts time from noon (12:00 UTC) each day.

4.   Two successive data samples are shown for each data mode.

5.   The following FORTRAN-like format statements may be used to read the

two lines of data in this mode for all units of measure:

First Line:

-----------

Catalog #:     I5

2X

Date/Time:     F15.9

X:             F15.5

4X

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 109

Y:             F15.5

4X

Z:             F15.5

CR/LF

Second Line:

------------

23X

Xdot:          F18.11

1X

Ydot:          F18.11

1X

Zdot:          F18.11

CR/LF

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 110

Data Mode 6: Precision X-Y-Z Cartesian State Vector, Comma Delimited

--------------------------------------------------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 6

0,0,20580,93110.0476041667,2982.28779295502,6229.01725815628,-940.796339818487,

-5.85010701911522,3.3199940892324,3.46047048985284

0,0,20580,93110.0476851852,2941.25120957693,6252.07713790904,-916.54588610151,

-5.87454395677527,3.2685046500327,3.46815363454982

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 16

0,1,20580,93110.0482060185,8771281.06140276,20969911.6226162,-2491608.1429704,

-19762.0576284838,9623.02409449012,11524.6952898439

0,1,20580,93110.0482638889,8672341.7437806,21017717.5456987,-2433947.77946384,

-19813.4647395816,9499.28476766938,11539.2506632381

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 26

0,2,20580,93110.0493865741,1103.65585656622,3591.279279101,-214.476932551084,

-3.4057330597861,1.1598790254898,1.9344163566285

0,2,20580,93110.049537037,1059.27271053518,3605.99939807939,-189.308779132495,

-3.42231018081901,1.10471766926502,1.93751948609809

NOTES:

1.   The X-Y-Z Cartesian State Vector is given as a standard Earth-centered

inertial ("ECI") cartesian 6-dimensional state vector where the X-Axis

is pointing toward the vernal equinox, the Z-Axis is pointing toward

the North Pole, and the Y-Axis is mutually orthogonal to the other

axes in a right-handed axis system. All coordinates are for true

equator and equinox of date.

2.   The units of measure for the state vector may be determined by the

tens digit of the Data Mode in the initial header line as well as the

second parameter in the comma delimited data string:

Data Param

Mode    #2     Units

-----------------------------------------------------------

6     0     Kilometers and kilometers per second

16     1     Feet and feet per second

26     2     Nautical miles and nautical miles per second

3.   The comma delimited data are generated as a single line terminated by

CR/LF. The examples above have been split into two lines for printing

purposes.

4.   The data are written in a single data line in the following order,

separated by a comma between items:

Epoch Flag, always zero signifying equator and equinox of date.

Units Flag (see Note 1 above)

Catalog/NORAD number

Date (YYDDD.DDDDDDDD... format)

ECI X

ECI Y

ECI Z

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 111

ECI Xdot

ECI Ydot

ECI Zdot

5.   Line length will vary as a function of the data.

6.   Two successive data samples are shown for each data mode.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 112

Data Mode 7: Precision X-Y-Z Cartesian State Vector, Labeled Data

-----------------------------------------------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 7

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93192.11956018518

07/11/1993  02:52:10.000 UTC

ECI X:                  -3441.20195444797 km

Y:                  -3110.29870646026 km

Z:                   4920.32069520120 km

Xdot:                   2.90216455238 km/sec

Ydot:                  -6.74909064951 km/sec

Zdot:                  -2.23710677970 km/sec

Ndot/2 (Drag):              0.00008567000

Nndot/6:                    0.00000000000

B-Star:                     0.00011546000

ElSet #:                  167.00000000000

Rev @ Epoch:            42286.31052536559

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 17

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93192.12233796297

07/11/1993  02:56:10.000 UTC

ECI X:               -8617248.92526347200 ft

Y:              -15077459.08108566000 ft

Z:               13806887.09734187000 ft

Xdot:               12614.26230523560 ft/sec

Ydot:              -18214.71804776612 ft/sec

Zdot:              -12006.04438377176 ft/sec

Ndot/2 (Drag):              0.00008567000

Nndot/6:                    0.00000000000

B-Star:                     0.00011546000

ElSet #:                  167.00000000000

Rev @ Epoch:            42286.35385448637

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 27

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93192.12280092592

07/11/1993  02:56:50.000 UTC

ECI X:                  -1333.74601094830 nm

Y:                  -2598.74977950943 nm

Z:                   2190.96899536823 nm

Xdot:                   2.14691236531 nm/sec

Ydot:                  -2.86693936350 nm/sec

Zdot:                  -2.09121400862 nm/sec

Ndot/2 (Drag):              0.00008567000

Nndot/6:                    0.00000000000

B-Star:                     0.00011546000

ElSet #:                  167.00000000000

Rev @ Epoch:            42286.36107600650

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 113

NOTES:

1.   The X-Y-Z Cartesian State Vector is given as a standard Earth-centered

inertial ("ECI") cartesian 6-dimensional state vector where the X-Axis

is pointing toward the vernal equinox, the Z-Axis is pointing toward

the North Pole, and the Y-Axis is mutually orthogonal to the other

axes in a right-handed axis system. All coordinates are for true

equator and equinox of date.

2.   The units of measure for the state vector may be determined by the

tens digit of the Data Mode in the initial header line as well as

being indicated with the data:

4   Kilometers and kilometers per second

14   Feet and feet per second

24   Nautical miles and nautical miles per second

3.   One data item is given per line, labeled as shown in the examples. The

data in the first four lines (Satellite Name, Catalog Number, and two

lines of Date/Time) begin in column 25. The remaining numeric data

items begin in column 21 and use a FORTRAN-like format statement

F21.11.

4.   The Catalog Number is given first as the NORAD Number ("16609" in the

example) and then as the International Designation ("86017A"). The

International Designation is from 1 to 3 letters. Some 2-line elements

omit the International Designation, in which case that portion will be

blank.

5.   Note that the Date/Time is presented on two lines in two different

formats. The first format is the NASA Day-of-Year ("DOY") format,

YYDDD.DDDDDD, since that is the format used by NASA/JSC for X-Y-Z

state vectors for the Space Shuttle, in 2-line elements, and in

program VEC2TLE. In the DOY format, time is counted from midnight

(00:00 UTC) each day. Some calculations may require instead the Julian

Date format which counts time from noon (12:00 UTC) each day. The

Date/Time is also "decoded" and given in the more conventional

"MM/DD/YYYY  HH/MM/SS.SSS" format for clarity using Coordinated

Universal Time (UTC/GMT).

6.   The ElSet Number is specified in the 2-line elements used to generate

the ground track and is always given as an integer. Note that ElSet

Numbers may not necessarily follow in sequence and that different

sources will use different sequences of ElSet numbers.

7.   The Rev Number at Epoch (the orbit number at the time the data is

sampled) is based upon the Rev Number specified in the 2-line elements

used to generate the ground track. The fractional part is calculated

by STSPLUS geometrically from the ascending node. Note that US Space

Command uses a different orbit numbering convention than does NASA for

space shuttle missions; US Space Command usually specifies the first

(partial) orbit number as Rev 0, while NASA specifies that orbit

number as Rev 1. For satellites which have been in orbit for long

periods of time, the Rev Number may be arbitrary.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 114

Data Mode 8: Doppler Shift Predictions

--------------------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 8

Satellite Name:         AO-27

Catalog Number:         22825  93061C

Uplink Center Frequency:    145.85000 MHz

Dnlink Center Frequency:    436.80000 MHz

Freq Diff in Hz

MM/DD     TIME     Sat Lat   Sat Lon  Sat Elv  Sat Azm   Uplink   DnLink

------------------------------------------------------------------------

07/31 18:21:00 UTC  56.992  -102.805    1.748   19.947    -3171     9497

07/31 18:22:00 UTC  53.549  -104.703    5.777   22.118    -3135     9388

07/31 18:23:00 UTC  50.085  -106.353   10.523   24.984    -3062     9170

07/31 18:24:00 UTC  46.604  -107.814   16.339   29.016    -2926     8764

07/31 18:25:00 UTC  43.109  -109.129   23.770   35.186    -2673     8005

07/31 18:26:00 UTC  39.603  -110.330   33.417   45.735    -2189     6555

07/31 18:27:00 UTC  36.088  -111.439   44.547   65.985    -1291     3865

07/31 18:28:00 UTC  32.565  -112.475   50.407  101.881       63     -188

07/31 18:29:00 UTC  29.035  -113.452   43.796  136.739     1389    -4161

07/31 18:30:00 UTC  25.501  -114.381   32.707  155.928     2247    -6730

07/31 18:31:00 UTC  21.962  -115.271   23.276  165.962     2707    -8108

07/31 18:32:00 UTC  18.419  -116.131   16.011  171.851     2949    -8832

07/31 18:33:00 UTC  14.873  -116.966   10.302  175.688     3080    -9223

07/31 18:34:00 UTC  11.326  -117.782    5.625  178.391     3150    -9435

07/31 18:35:00 UTC   7.777  -118.584    1.638  180.406     3186    -9543

NOTES:

1.   The header information gives the satellite name and catalog number,

the Epoch Date/Time of the TLEs, and the center frequencies for the

Uplink and the Downlink (usually in units of MHz) as read from file

STSPLUS.FRQ (if present) or the default value of 100 MHz.

2.   One data line is generated for each time step. For the date, only the

month and day ("MM/DD") are given in order to accommodate line length

restrictions. The time is given in Coordinated Universal Time (UTC).

3.   The satellite geodetic coordinates, latitude and longitude, are given

in degrees. Negative latitude is South, negative longitude is West.

4.   The satellite horizon coordinates, elevation and azimuth, are given in

degrees and the sense is NESW, North = 0, East = 90, etc.

5.   The doppler shift calculations are shown as the frequency difference

(in Hz) from the corresponding center frequency (in MHz, shown in the

header) at 60 second intervals throughout the pass.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 115

Data Mode 9: Pass Predictions

-----------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 19

Location:          Palos Verdes, CA

Prepared:          Saturday, 09 Sep 1995  17:02:42

Satellite Name:    STS-69

Catalog Number:    23667  95048A

TLE Filename:      STS69R04.TXT

-----------AOS-----------  --MAX VISIBILITY--  ------LOS------

#    PDT Date & Time    Azm  PDT Time Alt   Azm  PDT Time    Azm Duration V

1 10 SEP 95 06:24:57  196.9  06:28:03   4 159.4  06:31:10  121.7  0:06:13

2 10 SEP 95 08:00:25  228.0  08:04:43  12 171.1  08:09:02  114.1  0:08:37

3 10 SEP 95 09:37:04  243.6  09:41:31  14 183.6  09:46:00  123.5  0:08:56

4 10 SEP 95 11:14:25  244.4  11:18:17   8 195.7  11:22:08  147.1  0:07:43

5 11 SEP 95 05:21:27  178.6  05:23:25   2 155.9  05:25:23  133.0  0:03:56

6 11 SEP 95 06:55:58  219.5  07:00:00  10 167.1  07:04:06  114.3  0:08:08

7 11 SEP 95 08:32:19  239.9  08:36:47  14 179.5  08:41:17  118.8  0:08:58

8 11 SEP 95 10:09:25  245.9  10:13:35  11 191.6  10:17:46  137.5  0:08:21

9 11 SEP 95 11:47:44  231.9  11:50:13   3 202.9  11:52:40  174.2  0:04:56

10 12 SEP 95 05:51:39  209.2  05:55:20   7 162.9  05:59:01  116.8  0:07:22 *

-+ ---------+--------  --+--  ----+---  -+ --+--  ----+---  --+-- ----+--- +

|          |            |        |      |   |        |       |       |    |

Pass #      |            |        |      |   |        |       |       |    |

|            |        |      |   |        |       |       |    |

AOS Date/Time            |        |      |   |        |       |       |    |

(dd MMM yy hh:mm:ss)     |        |      |   |        |       |       |    |

|        |      |   |        |       |       |    |

AOS Azimuth (degrees) ---+        |      |   |        |       |       |    |

|      |   |        |       |       |    |

MAX Time (hh:mm:ss) --------------+      |   |        |       |       |    |

|   |        |       |       |    |

MAX Altitude (degrees) ------------------+   |        |       |       |    |

|        |       |       |    |

MAX Azimuth (degrees) -----------------------+        |       |       |    |

|       |       |    |

LOS Azimuth (degrees) --------------------------------+       |       |    |

|       |    |

LOS Time (hh:mm:ss) ------------------------------------------+       |    |

|    |

Total Pass Duration (hh:mm:ss) ---------------------------------------+    |

|

Visible Pass Flag (blank or "*") ------------------------------------------+

NOTES:

1.   When the Data Output mode is set up for pass predictions, dates and

times for pass predictions may be selected for UTC/GMT or local time

and the selected time zone abbreviation and time scale will be used.

Substitute the appropriate abbreviation for "UTC" as required if other

than UTC has been selected. When UTC or GMT is selected, the Data Mode

will be given as "9"; when local time is selected, the Data Mode will

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 116

be given as "19".

2.   The Pass # is an arbitrary number assigned by STSPLUS during the pass

calculations and is a function of the real or simulated time at which

the calculations are performed. If the real or simulated time is

changed, the pass numbers may change and different passes may be

shown.

3.   The satellite NORAD number is included in the heading, "16609" in the

sample above.

4.   The Date (UTC/GMT or local) is given only for AOS. Since a pass may

span 00:00:00 hours, the date for MAX VISIBILITY and/or LOS may have

to be incremented from that shown for AOS.

5.   All azimuths ("Azm") have been rounded to the nearest 0.1 degree; the

MAX VISIBILITY altitude ("Alt") has been rounded to the nearest

degree.

6.   When viewed on the screen, all Azimuths and the Altitude will include

the degree symbol and the data is four columns wider than the sample

shown above.

7.   The Visible Pass Flag will be blank if the pass is not visible or will

display "*" if all or part of the pass is visible. A visible pass is

defined as a pass which occurs with the satellite in sunlight, the

viewing location in darkness, and the satellite elevation above the

true horizon greater than 5 degrees. These conditions typically occur

prior to sunrise and subsequent to sunset (although exceptions are

possible with higher inclination satellites during certain times of

the year).

8.   The initial header information does not appear on the screen version

of the pass predictions. The location is the current primary location;

the preparation date and time shown is that for which the computer is

set when the data are generated; the catalog number is the NORAD

Number followed by the International Designation.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 117

F4     Calculate Satellite Positions with TRAKSTAR

--------------------------------------------------

Tabular predictions for the currently selected satellite may be made

by using TRAKSTAR by Dr. TS Kelso. STSPLUS has been arranged to operate

seamlessly with TRAKSTAR by simply pressing Function Key F4 from the Main

Menu. All data required by TRAKSTAR is automatically supplied by STSPLUS.

See the section above "Predicting Satellite Passes with TRAKSTAR" and the

TRAKSTAR documentation for full information.

F5     Set Launch Time and Date

-------------------------------

Orbital data for the satellite must be loaded using the F2 command on

the Main Menu before the launch date and time may be set or changed. Once

saved in file STSPLUS.LTD, the launch date and time will be automatically

read from that file each time the satellite is selected (see below).

********************

*  IMPORTANT NOTE  *

********************

LAUNCH TIME AND DATE MUST EITHER BE INCLUDED IN FILE STSPLUS.LTD

OR BE MANUALLY ENTERED FOR EACH SATELLITE OR MISSION SINCE THAT

INFORMATION IS NOT INCLUDED IN 2-LINE ELEMENTS.

Launch date and time are most important for manned missions such as

the Space Shuttle since the mission timeline is reckoned using Mission

Elapsed Time. However, MET may be used whenever actual launch date and time

are known. The only requirement is that 2-line orbital elements must be

available for the satellite. Since launch date and time are NOT included in

the 2-line orbital elements, this means that you obtain the launch date and

launch time independently and manually enter that data.

Pressing F5 to enter launch date and time begins with the prompt:

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Current time:  19:01:32 PDT   02:01:32 UTC

Current date:   15 APR 1995    16 APR 1995

Enter LAUNCH TIME (HH:MM:SS)    [06:37:00 UTC]:

[Add 'U'or 'G' for UTC/GMT]

Enter the time in the format shown using 24-hour notation. Add the letter

"U" to signify UTC (Coordinated Universal Time) or the letter "G" to

signify GMT (Greenwich Mean Time, essentially identical to UTC for the

purposes of this program). Use no suffix for local time; it will be

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 118

internally converted to UTC/GMT. If you enter "U" or "G", the abbreviation

used in the menus will be set to "UTC" or "GMT" respectively. One or two

digit numbers may be used as required. [The comma is also acceptable as a

separator in place of the colon.] You may omit seconds [or minutes and

seconds] if desired. For example, an entry of "16" will be entered as

16:00:00 or 4:00 PM. Press ENTER to leave the entry unchanged.

Enter LAUNCH DATE (MM/DD/YYYY)   [02 MAR 1995]:

[Enter '*' to clear LAUNCH DATE]

Enter the date in the format shown. Note that if you requested UTC or GMT

when entering the time, the date is interpreted as the UTC/GMT date. The

full four digit year may be used OR two digits as in "92". Be sure to use

the SLASH "/" rather than the MINUS "-" as the separator; STSPLUS's

internal date algorithms will interpret the minus sign as just that and

some rather strange dates can result! You may also use relative dates: -1

will use the prior day, +2 will use two days hence, and so forth. Press

ENTER to leave the date unchanged. Press "*" (followed by ENTER) to clear

the launch date and time; this does NOT remove it from file STSPLUS.LTD.

Select LAUNCH SITE (0=Disabled)  [1]:

[Enter '1' for Pad 39A, Cape Canaveral, FL]

[Enter '2' for Pad 39B, Cape Canaveral, FL]

If the current mission is a Space Shuttle launch, you may optionally cause

STSPLUS to generate Launch Simulation. Launch Simulation displays the

orbiter on the selected Launch Pad up to the time of launch, then simulates

the ascent trajectory for the first 45 minutes of the mission (after which

time STSPLUS uses the estimated pre-launch 2-line elements for orbital

calculations. Launch Simulation is enabled by selecting as a LAUNCH SITE

either Pad 39A or Pad 39B at Cape Canaveral, Florida. Press "1" or "2"

respectively to select the LAUNCH SITE desired, or press "0" to disable

Launch Simulation. The current default choice is shown in square brackets

in the prompt ("[1]" in the example above); press ENTER to select the

current default.

NOTE: STSPLUS can only simulate the launch of the Space Shuttle from

Cape Canaveral, Florida. Attempting to enable launch simulation for

other satellites or missions may produce unpredictable results. Launch

Simulation REQUIRES valid pre-launch 2-line elements for the specified

launch time and date. If a launch is delayed, new estimated pre-launch

2-line elements must be prepared for the new launch time and date. For

most missions (not including rendezvous missions), my program ADJ@LINE

may be used to calculate new estimated 2-line elements when a delay is

ecountered.

When all data have been entered, the program displays the launch data

as entered and pauses for your approval. If all data are correct, press

ENTER. Press the SPACE BAR to start over.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 119

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Current time:  19:01:32 PDT   02:01:32 UTC

Current date:   15 APR 1995    16 APR 1995

Enter LAUNCH TIME (HH:MM:SS)    [06:37:00 UTC]:  22:37:00 PST   06:37:00 UTC

Enter LAUNCH DATE (MM/DD/YYYY)   [02 MAR 1995]:   01 MAR 1995    02 MAR 1995

Select LAUNCH SITE (0=Disabled)  [1]:  Pad 39A, Cape Canaveral, FL

Press ENTER to accept, SPACE BAR to repeat: _

You are next asked if you wish to save this data:

Add/Update this data in file STSPLUS.LTD [Y,n]:

Press "Y", "y" or ENTER to add or update the data in file STSPLUS.LTD (see

below). Press "N" or "n" to use the data but not add or update it in file

STSPLUS.LTD. Adding or updating the data to file STSPLUS.LTD makes sure

that the launch date and time data will be available the next time this

particular satellite is selected.

Using File STSPLUS.LTD for Launch Date & Time

---------------------------------------------

An alternative and automatic method to set launch date and time is to

use file STSPLUS.LTD. This file contains the NORAD number and launch date

and time (UTC Julian date) for selected satellites. A sample entry appears

as:

22194,2448918.21503472,0

--+-- -------+-------- +

|          |         |

|          |         +--- Launch Site for Space Shuttle launches

|          |                0 = Launch Simulation DISABLED

|          |                1 = Pad 39A, Cape Canaveral, FL

|          |                2 = Pad 39B, Cape Canaveral, FL

|          |

|          +------------- Launch Date (UTC Julian date)

|

+------------------------ NORAD Number

The Sample above shows the data for Space Shuttle mission STS-52 (NORAD

#22194) and corresponds to a launch date and time of 22 OCT 1992 @ 17:09:39

UTC. The file is in standard ASCII format and may be edited with any

standard editor; word processor users use the "non-document" mode. Use care

when manually editing the file as STSPLUS performs NO ERROR CHECKING!

For Space Shuttle launches, Launch Simulation may be enabled by

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 120

specifying the Launch Site as noted above. Only Space Shuttle launches may

use Launch Simulation and estimated pre-launch 2-line elements for the

specified launch time and date are REQUIRED. Attempting to use Launch

Simulation without accurate estimated pre-launch 2-line elements OR with

other satellites or missions may cause unpredictable results.

Estimated 2-line orbital elements are usually available prior to a

Space Shuttle launch, and actual 2-line orbital elements within about 8 to

12 hours after a launch. Note, however, that the NORAD number is not

assigned until actual launch and a "dummy" NORAD number is used for

estimated pre-launch elements; for example, "99967" for STS-67. Once the

launch has taken place, the permanent NORAD number is assigned. This change

in NORAD number will require either that file STSPLUS.LTD be edited OR that

a new entry be made. Note that there are still a few satellites around with

low NORAD numbers!

If file STSPLUS.LTD is present and if the selected satellite is found,

the launch time and date will be set and Mission Elapsed Time (MET) will be

used automatically; otherwise, T+Epoch (T+E) will be used. When MET is

displayed, it may be changed to T+E by pressing F6 from the Main Menu

(below) or F5 while the map is displayed.

File STSPLUS.LTD is read each time a new satellite is selected using

F2 from the Main Menu. If file STSPLUS.LTD is NOT present, the launch date

and time will be saved in file STSPLUS.INI and must be MANUALLY MAINTAINED

as in versions prior to 9245! If you wish to use the old method, rename or

delete file STSPLUS.LTD.

F6     Set/Read/Save TDRS and Real Time Satellites

--------------------------------------------------

STSPLUS is able to display up to 32 additional TDRS (or other

geosynchronous/geostationary) or Real Time satellites. The first menu

selection allows the user to select the desired satellite for each of the

sixteen available "slots" as well as abbreviation, mode, and icon/label

color. Note that these features are enabled and disabled using F10+F3+F2

from the Main Menu. The following menu is displayed:

TDRS and Real Time Satellite Maintenance Menu

Current Satellite Configuration File: STSPLUS.SCF

F1  Display/Modify satellite assignments

F2  Save SCF Satellite Configuration File

F3  Read SCF Satellite Configuration File

F4  Select new PRIMARY SATELLITE

F5  Select new TARGET SATELLITE

F6  Clear Static and Real Time Satellites

Press function key for desired choice or ENTER for Main Menu:

The name of the current Satellite Configuration File (SCF) is shown. Press

ENTER to return to the Main Menu, or press the desired function key to

select the indicated function.

Before the current position of each satellite can be calculated, 2-

line elements must be read or updated using F2 from the Main Menu. After

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 121

this has been performed once, the position will be calculated based upon

those 2-line elements. The user is reminded that the 2-line elements should

be updated periodically so that the calculated position accurately reflects

the actual position.

NOTE:     Users of prior versions of STSPLUS will note that Function Key F6

has been reassigned for its present function. Switching between

MET and T+Epoch may now be accomplished ONLY by pressing F5 while

the ground track is displayed.

F1  Display/Modify Satellite Assignments

----------------------------------------

This menu selection displays the current TDRS and Real Time Satellite

assignments and option selections as a table:

--------------------------------------------------------------------------

TDRS and Real Time Satellite Setup, file STANDARD.SCF

Sat#  NORAD#   Abbr        Mode  Color Size  Label  Vis        Color Chart

1   19883   TDRE      Static   10 o    3     ON

2   21639   TDRW      Static   10 o    3     ON               1 = XXXXX

3   22314   TDR5      Static    2 o    3     ON  OFF          2 = XXXXX

4   19548   TDR2      Static    2 o    3     ON  OFF          3 = XXXXX

5   13969   TDR1      Static    2 o    3     ON  OFF          4 = XXXXX

6   16609   MIR      Primary   12 o          ON   ON          5 = XXXXX

7   21701   UARS   Real Time   13 o          ON  OFF          6 = XXXXX

8   20580   HST    Real Time   14 o          ON  OFF          7 = XXXXX

9   20638   ROSAT  Real Time   13 o          ON  OFF          8 = XXXXX

10   22076   TOPEX        OFF   13 o          ON  OFF          9 = XXXXX

11       0         (not used)                                 10 = XXXXX

12       0         (not used)                                 11 = XXXXX

13       0         (not used)                                 12 = XXXXX

14       0         (not used)                                 13 = XXXXX

15       0         (not used)                                 14 = XXXXX

16       0         (not used)                                 15 = XXXXX

Enter Satellite # to edit or press ENTER when done:

(The maximum Satellite # available is 32)

--------------------------------------------------------------------------

NOTE:     The first two satellite slots, #1 and #2, are reserved for

geosynchronous communications satellites. These are usually the

standard TDRS East and TDRS West satellites, currently NORAD

Numbers 19833 and 21639. Instead of conventional circles of

visibility, these two satellites generate commumications coverage

circles. Assigning satellites which are not geossynchronous may

produce unpredictable results.

The Satellite Setup table headings indicate:

Sat#      Satellite Assignment Number, 1 to 32. If an asterisk ("*")

appears to the right of the number, the epoch (date) of the

associated orbital elements is more than 10 days (Real Time)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 122

or 60 days (Static) old as in this example:

8*  22920   ARRAY  Real Time   14 o          ON  OFF

and the following caution note will appear at the bottom of

the display:

*  2-Line Elements more than 10 (Real Time) or 60 (Static)

days old!

The caution note has no effect on orbital caculations and is

simply a reminder that the orbital elements may be out of

date and may or may not be valid. Use F2 from the Main Menu

with "&" as the satellite name to update TLEs from a current

data file.

NORAD#    The NORAD Number of the assigned satellite, or "0" if no

satellite is assigned to this slot.

Abbr      The five character abbreviation assigned to this satellite.

Use only letters and numbers; the common satellite name or

NORAD Number are the recommended choices. This abbreviation

will be used as a label on the map if the Label is ON. May

be left blank.

Mode      Five mode classifications are available:

Primary        Primary satellite selected for full

tracking, selected using F2 or F6+F4

(below). The primary satellite may not

be present in the satellite list.

Static         Geosynchronous or geostationary

satellites (including TDRS). Plotted

when map is drawn and NOT updated.

Real Time      Satellites tracked dynamically; updated

every second (386/387 or higher), every

ten seconds (286 or no coprocessor).

OFF            The satellite is included in the data

but is not plotted. Used to temporarily

disable satellite slots.

(not used)     Unassigned satellite slot(s).

Color     A number selected from the Color Chart at the right which

indicates the color to be used to display the satellite icon

and label (if enabled). Each "XXXXX" in the Color Chart

appears on color monitors as a bar of the corresponding

color. The "o" character to the right of the number is

displayed as a small diamond in the selected color.

Size      For Static satellites only, selects the relative size of the

icon used to represent the satellite. Values may range from

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 123

3 to 6. This column is blank for Real Time satellites since

the icon size is fixed for these satellites.

Label     May be set to "ON" or "OFF" to indicate if the satellite

abbreviation is to be displayed. The label color will be the

same as that selected for the satellite.

Vis       May be set to "ON" or "OFF" to indicate if the satellite

circle of visibility is to be drawn. The circle of

visibility is drawn only for Static satellite on normal maps

but is drawn for all satellites on Motion maps.

Color     The color chart displays the 15 available icon and label

Chart     colors (BLACK is omitted). When a monochrone display is used

and the "/M" command line option is specified, two levels of

gray will normally be displayed instead of colors. Each

color is assigned a standard color number which is used for

all data entries.

As indicated by the prompt, simply press ENTER if all data in the

Satellite Setup is correct. You will return to the F6 menu above. If you

wish to add a satellite or change the selections for an existing satellite,

enter the satellite slot number (Sat# in the left column of the table). The

data for the selected slot will flash in white and the following prompts

will be displayed in turn:

Enter NORAD Number:

(Enter 0 to clear the satellite entry)

STSPLUS expects the NORAD number for the desired satellite. This number

will be used to "capture" the 2-line elements when this satellite is

subsequently displayed.

Enter Satellite Abbreviation:

Enter any desired abbreviation, 5 characters or less, or the NORAD number.

Use only letters, numbers, and the period ("."), dash ("-"), and comma

(",") with no leading spaces. The Satellite Abbreviation is used as the

satellite label on the map display (see below) and may be left blank if

no label is desired.

Enter Mode (0=STATIC, 1=REAL TIME, 2=OFF):

Enter "0", "1", or "2". STATIC satellites are plotted each time the map is

redrawn on the screen. REAL TIME satellites are updated every 1 or 10

seconds (depending upon the processor/coprocessor present) or as often as

the processor can complete the required calculations. Satellites set to OFF

are not processed but their data is retained so that they may be returned

to STATIC or REAL TIME subsequently.

Enter satellite color (1 to 15):

Enter the number for the desired color, as shown on the Color Chart at the

right of the screen. An illegal color number will default to YELLOW.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 124

Display satellite Label (0=NO, 1=YES):

Enter "0" or "1". The label will be displayed directly below the satellite

icon in the same color as that selected for the icon in the preceeding

step. For Real Time satellites, the label is only drawn when the map is

redrawn and remains stationary EXCEPT when using the Motion Map (in which

case it follows with the icon).

Display Cir of Visibility (0=NO, 1=YES):

Enter "0" or "1". The circle of visibility is drawn (when enabled) for all

Static satellites, and also for all Real Time satellites when using the

Motion Map.

When a new satellite is entered, a double asterisk ("**") will appear

at the right for that satellite slot. This is to remind the user to update

the 2-line elements or to display the ground track for that satellite so

that 2-line elements may be stored in the TLE memory associated with that

slot. Failure to update the TLEs may cause a program error! The following

message will also appear below the chart:

**  Update TLEs (F2+'&') or display ground track for this satellite!

When STSPLUS adds a new Secondary Satellite, a skeletal and INVALID 2-line

element set (TLE) is saved which includes only the NORAD number so as to

identify the satellite. Before the satellite can be tracked, real TLEs must

be present; the skeletal TLEs will be skipped. This may be accomplished by

updating the TLEs using F2 from the Main Menu and selecting AUTO UPDATE

(satellite name entered is "&"). Alternatively, the satellite ground track

may be displayed; to display the ground track, press F2 from the Main Menu,

select the file of 2-line elements which includes the satellite in

question, and enter the NORAD Number ("#nnnnn", where "nnnnn" is the NORAD

Number). In either case, the 2-line elements will be saved for future use.

** IMPORTANT NOTE **

Although 2-line elements (TLEs) are saved for all satellites, the user

is reminded that these data have a limited lifetime. That lifetime varies

considerably from satellite to satellite. For geosynchronous satellites, 4

to 8 weeks is probably reasonable; for other active satellites, 1 to 2

weeks should be an upper limit before new elements are used. TLEs for

satellites which maneuver may become inaccurate in a matter of days.

Allowing TLEs to become too old may yield an inaccurate position and, in

the worst case, may cause a program error! Use F2 from the Main Menu and

enter the satellite name as "&" to scan through the 2-line elements file

and update all elements which are more recent than those now saved.

F2  Save SCF Satellite Configuration File

-----------------------------------------

Users may wish to switch among several sets of TDRS and Real Time

satellites from time to time. Since it can be tedious to re-enter all the

information repeatedly, STSPLUS can save and read Satellite Configuration

Files, identified by the filetype .SCF. All parameters are saved and

restored, exactly as if entered at the time. The current satellite

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 125

configuration is automatically saved in file STSPLUS.INI each time the

program is exited and is restored when the program is restarted. The

following prompt is displayed:

Save SCF Satellite Configuration File [STSPLUS.SCF]

Enter SCF FILENAME, ENTER for STSPLUS.SCF, ESC to quit:

Current .SCF files are:

STANDARD.SCF

STSPLUS.SCF

STSPLUS displays the name of the current SCF file and a list of available

SCF files, up to a maximum of 90 files. Press ENTER to accept the default

filename shown, or ESC to cancel the operation. Otherwise, type the desired

filename with or without the .SCF filetype; STSPLUS automatically appends

the .SCF filetype and will ignore any other filetype which is included.

Filenames may be no longer than 8 characters.

If the file already exists, STSPLUS will display a warning message:

File STANDARD.SCF already exists! Overwrite [y/N]:

where "STANDARD.SCF" will be the requested file including the .SCF

filetype. Press "Y" or "y" to allow the existing file to be overwritten;

press any other key to cancel the write operation, and then press ENTER to

resume program operation. If the file does not exist, it will be created.

Do not use filename STSPLUS since a sample file by that name is included

with the distribution package (unless you wish to overwrite the supplied

file).

F3  Read SCF Satellite Configuration File

-----------------------------------------

See the comments above for saving SCF files. The following prompt is

displayed:

Read SCF Satellite Configuration File [STSPLUS.SCF]

Enter SCF FILENAME, ENTER for STSPLUS.SCF, ESC to quit:

Current .SCF files are:

STANDARD.SCF

STSPLUS.SCF

STSPLUS displays the name of the current SCF file and a list of available

SCF files, up to a maximum of 90 files. Press ENTER to accept the default

filename shown, or ESC to cancel the operation. Otherwise, type the desired

filename with or without the .SCF filetype; STSPLUS automatically appends

the .SCF filetype and will ignore any other filetype which is included.

Filenames may be no longer than 8 characters. If the file does not exist,

an error message will be displayed.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 126

F4  Select New PRIMARY Satellite

--------------------------------

This function allows the user to select a new Primary Satellite, the

satellite for which the data block is calculated, from the list of

Secondary Satellites. The current list of Secondary Satellites is first

displayed:

-------------------------------------------------------

TDRS and Real Time Satellite Setup, file STSPLUS.SCF

Sat#  NORAD#   Abbr        Mode  Color Size  Label  Vis

1   19883   TDRE      Static   10 o    3     ON

2   21639   TDRW      Static   10 o    3     ON

3   22314   TDR5      Static    2 o    3     ON  OFF

4   19548   TDR2      Static    2 o    3     ON  OFF

5   13969   TDR1      Static    2 o    3     ON  OFF

6   16609   MIR    Real Time   12 o          ON   ON

7   21701   UARS     Primary   13 o          ON   ON

8   22920   ARRAY  Real Time   14 o          ON  OFF

9   20638   ROSAT  Real Time   13 o          ON   ON

10   22076   TOPEX  Real Time   13 o          ON   ON

11   21225   GRO    Real Time   13 o          ON   ON

12   21987   EUVE   Real Time   13 o          ON   ON

13   20580   HST    Real Time   14 o          ON  OFF

14       0         (not used)

15       0         (not used)

16       0         (not used)

Enter New PRIMARY Satellite # or press ENTER to accept:

-------------------------------------------------------

Select the satellite which is to be the new Primary Satellite by entering

its Sat# at the prompt, or press ENTER to cancel. Do NOT select a satellite

which displays "**" at the right; valid 2-Line Elements are not available

for that satellite! If "6" is entered for the example shown, the current 2-

Line Elements for MIR will be displayed for approval:

2-Line Elements for new PRIMARY satellite:

Mir

1 16609U 86017A   95247.20219116  .00002788  00000-0  44079-4 0  2214

2 16609  51.6459 122.0823 0003414  62.1935 297.9402 15.57354527545247

Accept this Primary Satellite [Y/n]:

Review the 2-Line Elements to be sure that valid elements are present and

that they are for the desired satellite. Press "Y" (or ENTER) to accept the

data shown as the new Primary Satellite, or press any other key to cancel.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 127

F5  Select New TARGET Satellite

-------------------------------

This function allows the user to select a new Target Satellite, the

satellite for which the Relative Range and Velocity may be calculated, from

the list of Secondary Satellites. The current list of Secondary Satellites

is first displayed:

----------------------------------------------------------------------

TDRS and Real Time Satellite Setup

Sat#  NORAD#   Abbr        Mode  Color Size  Label  Vis

1   19883   TDRE      Static   10 o    3     ON

2   21639   TDRW      Static   10 o    3     ON

3   22314   TDR5      Static    2 o    3     ON  OFF

4   19548   TDR2      Static    2 o    3     ON  OFF

5   13969   TDR1      Static    2 o    3     ON  OFF

6   16609   MIR    Real Time   12 o          ON   ON

7   21701   UARS   Real Time   13 o          ON   ON

8   22920   ARRAY  Real Time   14 o          ON  OFF

9   20638   ROSAT  Real Time   13 o          ON   ON

10   22076   TOPEX  Real Time   13 o          ON   ON

11   21225   GRO    Real Time   13 o          ON   ON

12   21987   EUVE   Real Time   13 o          ON   ON

13   20580   HST       Target   14 o          ON  OFF

14   22917   STS61    Primary   14 o          ON  OFF

15       0         (not used)

16       0         (not used)

Enter New TARGET Satellite #, '0' to cancel, or press ENTER to accept:

----------------------------------------------------------------------

Select the satellite which is to be the new Target Satellite by entering

its Sat# at the prompt, enter the digit zero ("0") to cancel the current

Target Satellite, or press ENTER to cancel and leave the current selection

unchanged. Do NOT select a satellite which displays "**" at the right;

valid 2-Line Elements are not available for that satellite!

If "6" is entered for the example shown, the Target Satellite will be

changed from #20580 (HST) to #16609 (MIR) and the current 2-Line Elements

for MIR will be displayed for approval:

2-Line Elements for new TARGET satellite:

Mir

1 16609U 86017A   95247.20219116  .00002788  00000-0  44079-4 0  2214

2 16609  51.6459 122.0823 0003414  62.1935 297.9402 15.57354527545247

Accept this TARGET Satellite [Y/n]:

Review the 2-Line Elements to be sure that valid elements are present and

that they are for the desired satellite. Press "Y" (or ENTER) to accept the

new Target Satellite, or press any other key to cancel. If MIR is

accepted, it will then be used as the Target Satellite and Relative Range

and Relative Velocity with respect to the Primary Satellite (STS-61 in the

example) may be displayed using F10+F4 from the Main Menu OR by pressing

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 128

F10 while the map is displayed.

F6  Clear Static and Real Time Satellites

-----------------------------------------

Use this function to clear selected static and/or real time

satellites with slot numbers 3 through 32. STSPLUS asks the user to enter

the first and last slot to clear, and to confirm the operation to avoid

accidental clearing.

Clear Static and Real Time Satellites

NOTE: Slots 1 & 2, usually assigned to TDRS East and TDRS West,

must be cleared MANUALLY using F1.

The last slot currently in use is 29

Enter first slot to clear (ENTER to cancel): 26

Enter last slot to clear (ENTER to cancel):  29

Slots 26 through 29 will be cleared!

Press 'Y' to clear, any other key to cancel:

As the sample prompts above illustrate, the user must enter the first

and the last slot to be cleared, then confirm by pressing the letter "Y"

(upper or lower case).

NOTE: Slots 1 and 2 are reserved for geosynchronous communications

satellites and should ALWAYS contain TDRS or similar geosynchronous

satellites. The program may perform unpredictably if these slots are

cleared or use non-geosynchronous satellites.

F7     Set FILENAMES and PATHS

------------------------------

Function Key F7 allows the user to select the paths and/or filenames

for the various files that STSPLUS uses to select satellites, tracking

stations, and other features:

Select path or filename to set, press ENTER when done:

F1  Set 2-LINE ELEMENTS path:      [I:\TLE\]

F2  Set TRACKING STATION filename: [STSPLUS.TRK]

F3  Set MAP DATABASE FILES path:   [D:\MAPDATA\]

F4  Set FEATURES LABEL filename:   [STSPLUS.LOC]

F5  Set TRAKSTAR path:             [D:\STSPLUS\]

F6  Set CITYFILE filename:         [STSPLUS.CTY]

Enter selection or ENTER:

Press the indicated function key for the item you wish to change. The

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 129

current path or filename is shown in square brackets for each selection.

Press ENTER to leave a path or filename unchanged. The following is a

typical prompt for filename:

Enter TRACKING STATION filename: _

(Press ENTER to leave unchanged)

For filenames, enter the full filename including filetype. A drive and

directory may also be included if desired. If no filetype is entered,

STSPLUS will automatically supply ".TRK" for tracking station files, and

".LOC" for features label files. If the desired file has no filetype,

include the period in the filename entered (e.g. "STATION.") to prevent the

automatic addition of a filetype.

For the path selections, enter the desired drive and subdirectory. The

trailing backslash will automatically be added if it is omitted. If the

path cannot be found, an error message will be displayed and the path will

default to the current drive and directory. For best performance, use a RAM

disk for Map Database Files; see the section "Using a RAM Disk" for further

information.

After each entry, the Path and Filenames Menu is again displayed with

the current selections. Press ENTER when done to return to the STSPLUS Main

Menu.

F8     Set Program TIME and DATE

--------------------------------

This menu provides a number of time and date functions for use

with STSPLUS. The program clock may be set to real or simulated time

using several methods, current clock corrections applied by program

RIGHTIME may be displayed, and the UTC OFFSET and DAYLIGHT Flag may be

adjusted. The user may also select either LOCAL or UTC time for his

computer.

It is often convenient to set the TIME and DATE within STSPLUS to

something other than the current system time and date, or to return to the

current system time and date if the program time and date have been

changed. Press F8 to go to the TIME and DATE Menu:

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Current time:  19:01:32 PDT   02:01:32 UTC

Current date:   15 APR 1995    16 APR 1995

ACTUAL SYSTEM DATE AND TIME SHOWN ABOVE

F1     Restore SYSTEM date and time (use "real time")

F2     Set DOS SYSTEM CLOCK using calendar date and time

F3     Set SIMULATED date and time using calendar date and time

F4     Set SIMULATED date and time using Mission Elapsed Time

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 130

F9     Display Current RIGHTIME Corrections

F10    Set UTC OFFSET, DAYLIGHT Flag, and DOS Time Zone

ENTER  Return to MAIN MENU

Select desired function:

The Date and Time Menu, shown above, displays the available time

setting functions along with the actual system date and time as determined

by the DOS software clock in your computer (even if simulated time is in

effect). If program RIGHTIME Version 2.5+ is currently enabled, the message

"(Assisted by RIGHTIME)" will also appear. Both your local date and time,

"PDT" or Pacific Daylight Time in the example, and "UTC" (Coordinated

Universal Time) date and time are displayed. If times have been set using

the letter "G", the abbreviation at the right will be "GMT" (Greenwich Mean

Time).

Press ENTER to return to the Main Menu with the date and time as

displayed on the screen (Current or Simulated).

If you wish to execute STSPLUS in "real time", cancelling any

simulated time that may be in effect, use the F1 command. This will restore

the time and date used for the tracking display to that shown at the top of

the menu. If the actual system date or time displayed is incorrect, use

program TIMESET (if available) or the F2 command to correctly set your

system clock.

Some organizations, NASA for example, continue to use the wording

"Greenwich Mean Time" or "GMT" for what is now usually referred to as

"Coordinated Universal Time" or "UTC" (and sometimes, depending upon the

application, as "UT", "UT1" or "UT2"). STSPLUS uses Coordinated Universal

Time or "UTC", the time used for civil timekeeping and broadcast by radio

stations such as WWV and the BBC. Although technically these different time

standards are not exactly the same, the difference is only a maximum of 0.9

seconds and the program treats them all as identical. STSPLUS defaults to

the abbreviation "UTC" but if you prefer to use "GMT", enter any simulated

time using F3 and include the letter "G" (upper or lower case) at the end.

The time abbreviation at the top of the screen will change from "UTC" to

"GMT" and will continue using that abbreviation until a time is entered

suffixed with "U".

Times are always entered as "HH:MM:SS" where HH is HOURS, MM is

MINUTES, and SS is SECONDS. The time entry format is very flexible. Leading

zeroes are not required. The comma (",") may be used in place of the colon

(":") as a separator if desired. SECONDS or MINUTES and SECONDS may be

omitted if desired. Time entries are assumed to be local time; to enter UTC

or GMT times, add the letter "U" or "G" (upper or lower case) respectively

following the entry. For example, the following are valid time entries:

Entry          Interpreted as

------         --------------------------

12             12:00:00 (LOCAL TIME ZONE)

13,1           13:01:00 (LOCAL TIME ZONE)

4:1:15         04:01:15 (LOCAL TIME ZONE)

1,1,1          01:01:01 (LOCAL TIME ZONE)

13,45U         13:45:00 UTC

1:20g          01:20:00 GMT

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 131

Dates may be entered as "MM/DD/YYYY" or "MM/DD/YY" (US style) or as

"DD.MM.YYYY" or "DD.MM.YY" (European style) where MM is MONTHS (as a number

from 1 to 12), DD is DAYS, and YYYY is the full four-digit YEAR or YY is

the last two digits of the YEAR. Except for the two digit year option, the

full date must always be entered; leading zeroes are not required. The

delimiter used ("/" or ".") determines the method of interpretation:

Entry          Interpreted as

---------      --------------

12/8/94        12 AUG 1994

12/8/1994      12 AUG 1994

8.12.94        12 AUG 1994

8.12.1994      12 AUG 1994

The date entered is assumed to be for the same time zone as the time

entered. If local time is entered, the date will be treated as the local

date; if UTC (or GMT) time is entered, the date will be treated as the

UTC/GMT date.

After a time or date entry has been accepted (after you press the

ENTER key), STSPLUS reformats the entry to its standard format, clears the

characters you entered, and replaces them by the standard format in both

local and UTC/GMT time zones. This provides a double check that the program

has interpreted your entry as you wished.

F1     Restore System Date and Time

-----------------------------------

Press F1 to restore the program date and time to the system date and

time. This command reads the DOS clock and restores the program to "real

time" operation. If the program date and time have not been changed with

the F3 or F4 commands, this command will have no effect.

F2     Set DOS System Clock

---------------------------

Press F2 to set the DOS system clock. Use this command if you wish to

change the actual date and time on your system. Note that on many systems

using DOS 3.3 or higher, this command will set BOTH the software clock AND

the hardware clock.

*************

*  CAUTION  *

*************

This function should NOT be used when program RIGHTIME is

regulating the DOS clocks UNLESS no other method is available.

Use program TIMESET to set the DOS clocks accurately instead!

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Current time:  19:01:57 PDT   02:01:57 UTC

Current date:   15 APR 1995    16 APR 1995

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 132

CAUTION: This function will change the computer's SYSTEM CLOCK!

Press ENTER to leave an item unchanged

Enter TIME [16:34:20 PDT]:            16:34:20 PDT

Enter DATE  [20 AUG 1994]:             20 AUG 1994

Use US Style 'MM/DD/YYYY' or European Style 'DD.MM.YYYY'

Press ENTER to accept, SPACE BAR to repeat: _

The time shown with the "Enter TIME" prompt is the current system time when

the prompt was generated and is shown to remind the user of the required

format. Press ENTER to accept the current system time without change; the

response at the right will be a later time which represents the delay

between the time the prompt was generated and the user pressed ENTER. If a

new time is entered, the system's clock is set at the instant ENTER is

pressed at the end of the time entry; therefore, to set the clock

accurately, enter a time some seconds in the future and press ENTER when

the time entered is reached.

The sample above shows the screen after the time and date entries have

been completed. The current ACTUAL system date and time are continuously

displayed for approval. Press ENTER to accept the time and date displayed,

or press the SPACE BAR to repeat the entries.

F3     Set Simulated Date and Time using Calendar Method

--------------------------------------------------------

Press F3 to set a simulated date and time. The date and time may be

either in the past or in the future. This command does NOT affect the DOS

clock in your system! Use the F1 command above to restore the date and time

to "real time".

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Current time:  19:01:57 PDT   02:01:57 UTC

Current date:   15 APR 1995    16 APR 1995

Press ENTER to leave an item unchanged

Enter SIMULATED TIME  [12:05:06]:     13:00:00 PDT   20:00:00 UTC

Enter SIMULATED DATE  [07 JUL 1994]:   08 AUG 1994    08 AUG 1994

Use US Style 'MM/DD/YYYY' or European Style 'DD.MM.YYYY'

Press ENTER to accept, SPACE BAR to repeat: _

The sample above shows the screen after the time and date entries have been

completed. The new SIMULATED date and time are displayed (and counting) for

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 133

approval. Press ENTER to accept the time and date displayed, or press the

SPACE BAR to repeat the entries.

F4     Set Simulated Date and Time using MET

--------------------------------------------

Press F4 to set a simulated date and time using MET (Mission Elapsed

Time). Note that this command will appear ONLY if the mission name begins

with the letters "STS", signifying a Space Transportation System (Space

Shuttle) mission AND if a launch time and date have previously been

entered.

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Simulated time:  00:17:18 PST   08:17:18 UTC

Simulated date:    10/09/1994     10/09/1994

Enter desired Mission Elapsed Time (MET)

Enter MET DAY (NN):             3 day(s)

Enter MET TIME (HH:MM:SS):      04:30:00 MET

Press ENTER to accept, SPACE BAR to repeat: _

The sample above shows the screen after the day and time entries have been

completed. The Mission Elapsed Time is immediately converted to actual date

and time and the current SIMULATED date and time, based upon the MET just

entered, are then displayed (and counting) for approval. Press ENTER to

accept the time and date displayed, or press the SPACE BAR to repeat the

entries.

F9     Display Current RIGHTIME Corrections

-------------------------------------------

If program RIGHTIME has been detected, the "F9" menu item will be

displayed and you may press F9 to display the time since the last TIMESET,

the current WARM correction, and the current COOL correction:

RighTime Version 2.53 detected!

Time Since Last TIMESET:   0 days 08:58:20

Current WARM Correction:   -0.01 seconds

Current COOL Correction:   -0.35 seconds

Press any key to continue ... _

The version of RIGHTIME is displayed. The time since the last TIMESET is

saved by program RIGHTIME to the nearest 200 seconds and will therefore not

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 134

change until that increment is reached. The time is shown as days followed

by hours:minutes:seconds. If more than 7 days has elapsed since the last

TIMESET, an additional message "(TIMESET suggested!)" will also appear. The

WARM and COOL corrections are shown and are updated when the system time is

set using Function Key F1 from this menu, by program TIMESET, or by other

means. Press any key, such as ENTER, to return to the Time and Date Menu.

F10    Set UTC OFFSET, DAYLIGHT Flag, and DOS Time Zone

-------------------------------------------------------

STSPLUS uses UTC or Coordinated Universal Time, an adjusted version

of Universal Time (which STSPLUS considers the same as GMT or Greenwich

Mean Time), for certain functions such as launch time. The difference

between UT, UT1, UT2 and UTC is never more than 0.9 seconds. UTC is used

because it is the standard for civil timekeeping and agrees with standard

atomic time, TDB or Terrestrial Barycentric Time, used by astronomers.

However, NASA continues to use the GMT designation, a holdover from earlier

days before the introduction of UTC. Using UTC permits critical data to be

used across many time zones without conversion. However, it also means that

STSORBIT must know what number of hours to add to UTC in order to obtain

your local time, and whether or not you are currently using daylight

savings time (summer time in the UK).

When prompted, enter the time offset in hours from your local time to

Coordinated Universal Time. Examples are shown for most time zones in North

America. STSPLUS then asks if you are using daylight savings time; enter

"0" if not, and "1" if so. The sum of these two values is shown on the Main

Menu; for example, if the computer is set to Pacific Daylight Time (UTC

offset is -8.00 hours and Daylight Flag = 1), the sum will be -7.00. For

most time zones in North America, the correct zone abbreviation will be

shown on the ground track display for Local date and time. When you change

your computer from/to daylight savings time, use this command to update

STSPLUS. The following shows the display when using the F9 command:

Set UTC TIME ZONE OFFSET and DAYLIGHT FLAG

STSPLUS must know the difference between your local time zone and Universal

Coordinated Time (UTC), also sometimes known as Greenwich Mean Time (GMT).

With this information, STSPLUS can automatically adjust launch times and

dates for your local time zone. In addition, STSORBIT must know if you are

now on STANDARD or DAYLIGHT (SUMMER) time and if your DOS clock is set to

UTC.

First, enter the difference between your STANDARD time zone and UTC in hours.

Do NOT include the hour for daylight time if you are now on DAYLIGHT time;

it will be entered separately. For most time zones in the United States and

Canada, the entries required are:

Eastern Standard Time     EST    -5.00

Central Standard Time     CST    -6.00

Mountain Standard Time    MST    -7.00

Pacific Standard Time     PST    -8.00

Enter UTC Offset (hours) [ -8.00]:

Enter Daylight Flag (0=OFF, 1=ON)  [1]:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 135

Enter DOS time zone (0=LOCAL, 1=UTC)  [0]:

Once this information has been entered, it will be saved in file

STSPLUS.INI and will not be requested again. If you change from Standard to

Daylight Time or vice versa, use the F10+F9 command to update the Daylight

Flag.

Some users prefer to operate their computer using UTC rather than

local time. STSPLUS may be operated in either time zone mode and makes all

necessary conversions. NOTE: Even if your DOS clock is set to UTC, the UTC

Offset and Daylight Flag must be set correctly!

If you change the setting of the Daylight Flag, STSPLUS will ask if

you wish to adjust your DOS software clock:

You have changed the setting of the Daylight Flag.

Do you wish to adjust your DOS clock to reflect the

change [y/N]:

If you have already made the change at the DOS prompt (or using some other

software) or do not wish to change the DOS clock, press ENTER (or type "N"

followed by ENTER). If you wish to adjust the DOS clock to correspond to

the new setting of the Daylight Flag, press "Y" followed by ENTER. When

STSPLUS changes the DOS clock, it synchronizes the time change to the

nearest second but there may be a small error introduced; only if your

computer is precisely set would the error be detectable.

*************

*  CAUTION  *

*************

For computers equipped with 80286 or higher processors AND using DOS 3.2 or

higher, changing the DOS clock will ALSO change the hardware clock. 8088-

based computers may or may not have a hardware clock installed and, even if

a hardware clock is present, it may or may not be compatible with the DOS

time setting commands.

F9     DOS Shell (CAUTION: DOS 3.2 or higher ONLY!)

--------------------------------------------------

If a system function is desired at the Main Menu, press F9 to execute

BASIC's DOS SHELL function. This will return you to a DOS prompt and most

DOS commands may be executed immediately. When the Shell is executed,

STSPLUS remains in memory and the map data will not be re-read when you

return. All files used by STSPLUS are closed. However, this means that a

substantial amount of memory is in use and not available to DOS during the

shell operations. As of Version 9537, STSPLUS requires approximately

414KB of memory. This means that many programs will have insufficient

memory to execute under the DOS Shell.

Enter "EXIT" (without the quotation marks and followed by ENTER) at

the DOS prompt when you wish to return to STSORBIT.

CAUTION: The BASIC SHELL function is only reliable for versions of DOS

of 3.2 or higher! Systems with less than 640K memory may fail to

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 136

execute the shell and applications requiring large amounts of memory

may also fail.

CAUTION: The BASIC SHELL function does NOT perform reliably when

STSPLUS is executed under Windows and a BASIC ERROR or other

unpredictable results may follow! Instead, close STSPLUS and use the

MS-DOS PROMPT function from the FILE MANAGER.

F10    Set STSORBIT PLUS Program Options and Features

-----------------------------------------------------

A number of program features and display options are set using the F10

Program Options Menu. These selections are further described in the section

"Program Options Menu" below.

ENTER  Resume Mission

---------------------

Pressing ENTER resumes the current mission shown in parentheses to the

right of the command on the Main Menu. The 2-line elements file from which

the data was read is shown in square brackets.

ENTER  Resume Mission                  (STS-41 [STS41F])

Any manually entered data is retained. "ENTER" means the key marked ENTER,

RETURN, or with a left pointing arrow -- but not the backspace or cursor

position keys which may also be marked with arrows! (I am afflicted with

too long a memory; once upon a time this function was known as Carriage

Return and was often shortened to RETURN or even CR. With the advent of

electronic typewriters, video terminals, dot matrix printers and all the

rest, "carriages" have long since disappeared but old habits die hard! Most

PC keyboards are now marked with "ENTER".)

Use ENTER to resume plotting a mission in progress after returning to

the Main Menu to perform some change (such as enabling the node display,

enabling the NASA tracking stations, or adjusting the time or date).

ESC    Quit STSORBIT PLUS and Save Current Mission

--------------------------------------------------

Press ESC (the key marked "ESC" or "Esc", not the letters E+S+C) to

quit program STSORBIT PLUS. If you press ESC to quit the program and have

manually entered orbital data, STSPLUS will save all required mission data

in file STSPLUS.INI prior to terminating. This will be the data available

with the ENTER key the next time you execute the program. The demonstration

data will not be saved, preserving any previously saved mission data.

When you have finished with STSPLUS, press ESC at the Main Menu to

return to DOS. The data (and any adjustments you have made) for the current

mission are saved in file STSPLUS.INI, but the map data is lost and will be

re-read when you next use program STSPLUS.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 137

PROGRAM OPTIONS AND FEATURES MENU

---------------------------------

A number of program features and display options are set using the F10

Program Options Menu. When used with CGA displays, the features shown below

as selected by function keys F3 and F4 are not available because of the low

resolution of the CGA display. The following Options Menu is displayed when

the F10 command is entered from the Main Menu:

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Current time:  19:01:57 PDT   02:01:57 UTC

Current date:   15 APR 1995    16 APR 1995

F1             Program STSORBIT PLUS Information

F2             Set New Local Coordinates (Palos Verdes, CA)

F3             Select Display Features

F4     TGT     Set Satellite Coordinates: Ra/Dec, Elv/Azm, XYZ, TARGET

F5     OFF     Show Ascending & Descending Node Data

F6     ORTHO   Set Map Projection and Size

F7     ON      Enable/Disable EVENT TIMERS

F8     OFF     Enable/Disable Audible ALARMS

F9    -8.00    Set UTC Time Offset and Daylight Flag

F10    OFF     Enable/Disable Printer Logging

ENTER          Return to MAIN MENU

Select desired function:

F1     Program STSORBIT PLUS Information

----------------------------------------

Function Key F1 displays information about program STSORBIT PLUS

including the copyright notice, version number, my name and address, and

the telephone number of my RPV ASTRONOMY BBS (Bulletin Board System). The

current version of STSORBIT PLUS is always posted on the BBS. The BBS has a

power controller; if it hasn't answered after the THIRD RING, hang up and

call back in two minutes. The BBS is available 24 hours per day at 9600,

2400 and 1200 baud.

F2     Set New Local Coordinates

--------------------------------

In order to perform the calculations related to satellite visibility

and altitude/azimuth, STSPLUS must know the geographic (geodetic)

coordinates for the user's location. The names of the current PRIMARY and

SECONDARY (if present) locations are shown. When STSPLUS is first started,

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 138

the default coordinates for the PRIMARY location are set to Palos Verdes,

California, near Los Angeles, and the SECONDARY location is disabled. The

current PRIMARY location is indicated by the name in parentheses on the

Options Menu. The program provides two methods for setting your own

coordinates: reading a file of city or place names and coordinates

(STSPLUS.CTY is the default filename); or manually entering the

information.

Pressing F2 at the Main Menu will display the Local Coordinates Menu:

Current PRIMARY Location:      Palos Verdes, CA

Current active CITYFILE:       STSPLUS.CTY

F1  Search CITYFILE for PRIMARY location

F2  Search CITYFILE for SECONDARY location

F3  Enter coordinates for PRIMARY location

F4  Enter coordinates for SECONDARY location

F6  Select TRACKING STN for PRIMARY location

ENTER  Return to Program Options & Features Menu

Enter desired selection:

Or, if a SECONDARY location is currently enabled, the following Local

Coordinates Menu will be displayed:

Current PRIMARY Location:      Palos Verdes, CA

Current SECONDARY Location:    Washington (USNO), DC

Current active CITYFILE:       STSPLUS.CTY

F1  Search CITYFILE for PRIMARY location

F2  Search CITYFILE for SECONDARY location

F3  Enter coordinates for PRIMARY location

F4  Enter coordinates for SECONDARY location

F5  Clear (disable) SECONDARY location

F6  Select TRACKING STN for PRIMARY location

ENTER  Return to Program Options & Features Menu

Enter desired selection:

Press the indicated function key to perform the desired function, or press

ENTER to return to the Program Options & Features Menu. The F5 selection

will only appear if a SECONDARY Location is currently enabled. NOTE: To

change the current active CITYFILE, use F7 from the Main Menu.

NOTE: If the current active CITYFILE is not the one you wish to use,

press ENTER twice to return to the Main Menu and then use F7 to set a

new CITYFILE path and/or name.

F1/F2 Search CITYFILE for Location

----------------------------------

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 139

Pressing F1 or F2 will search the current CITYFILE for the full or

partial name (the "CITYNAME") you enter. The following prompt appears:

Enter partial NAME to match:

In other words, when you enter a name or partial name, STSPLUS will attempt

to match that group of characters anywhere in the names which appear in the

current city file. For example, 'SAN' matches 'SAN diego' as well as 'SANta

ana" and 'thouSANd oaks'. To get 'SAN FRANCISCO' on the first try, enter

'SAN F' with a space between the 'N' and 'F'. Case is not significant;

upper and lower case letters are treated identically. If you change your

mind and wish to cancel the operation, simply press ENTER by itself. Use

BACKSPACE to make corrections.

To begin the search, enter the desired name after the prompt. In the

example which follows, F2 was pressed and the name 'santa' was entered for

the search:

Processing record  1616

Location:   Santa Ana, CA

Latitude:     33.7633

Longitude:  -117.8650

Elevation:    20 meters

Press ENTER to ACCEPT this city as your SECONDARY location, OR

Press ESC to cancel, SPACE to search for next location:

If the city displayed is the one you wish to use as your PRIMARY or

SECONDARY location (depending upon which function key you selected, F1 or

F2), press ENTER. The information will be used by STSPLUS and subsequently

saved in file STSPLUS.INI. If you wish to search further in the file, press

the SPACE BAR. If you wish to cancel the search, press ESC to return to the

Local Coordinates Menu.

NOTE: Some of the elevations contained in file STSPLUS.CTY are zero

because the elevation (in meters) above mean sea level was unknown

when the file entry was prepared. If you know the correct elevation

for your location, edit the file using any ASCII text editor and

change the last number on the line. STSPLUS.CTY contains over 2000

cities. If users send me their correct elevations (or additional

cities they wish added), I will incorporate that data into subsequent

releases of file STSPLUS.CTY.

F3/F4 Enter New Coordinates for Location

----------------------------------------

To enter location data manually, press F3 (for the PRIMARY location)

or F2 (for the SECONDARY location). You will be prompted for the city name,

latitude, longitude, and elevation. Latitude and longitude may be entered

using three different formats for convenience (note the use of comma and

decimal point):

DDD.DDDDD           Degrees and decimal fraction

DD,MM.MMM           Degrees, minutes and decimal fraction

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 140

DD,MM,SS.SS         Degrees, minutes, seconds and fraction

The decimal point and decimal fraction are not required in any of the

formats. For example, to enter 33 degrees and 17 minutes, type "33,17".

Note that SOUTH latitude must be entered as a NEGATIVE number as measured

south of the Equator. Longitudes may be entered as EAST Longitude, the

number of detrees East of the Prime Meridian at Greenwich (0 to 360

degrees) OR as WEST longitude( 0 to -180 degrees), the NEGATIVE number of

degrees West of the Prime Meridian at Greenwich; regardless of the manner

entered, the longitude will be automatically converted to the range of -180

degrees through 180 degrees. The default unit for elevations is meters

above mean sea level; add "F" (upper or lower case without the quotation

marks) if you wish to use feet.

After the elevation has been entered, the data will be displayed for

approval. All data are converted to degrees and decimal fraction or integer

meters as appropriate, regardless of the format or units used on input.

City Name:  Rancho Palos Verdes CA

Latitude:     33.7675

Longitude:  -118.4033

Elevation:   186 meters

Press ENTER to ACCEPT this city as your PRIMARY location, OR

Press ESC or SPACE to cancel this data:

If you are entering data for the secondary location, the word "SECONDARY"

will appear instead of "PRIMARY" in the prompt. Press ENTER to accept the

data as shown or press ESC or the SPACE BAR to cancel the data and return

to the Local Coordinates Menu. If the data is accepted, STSPLUS will ask if

you wish to append (add) this city/location to the existing file

STSORBIT.CTY so that it will be automatically available thereafter.

Do you with to append this data to file STSPLUS.CTY (Y/n):  _

Press "Y" or ENTER to append the data to the current city file shown in the

prompt, OR press "N" to use the data but not modify the current city file.

F5 Clear (disable) Secondary Location

-------------------------------------

If a SECONDARY Location is displayed on the Local Coordinates Menu and

you wish to cancel (disable) that feature, press F5. If the secondary

location is already disabled, the F5 menu item will not be displayed.

F6 Select TRACKING STN for PRIMARY Location

-------------------------------------------

You may select one of the Tracking Stations included in the current

Tracking Station file (default is file STSPLUS.TRK) as the Primary Location

by pressing F6. The screen will display the first tracking station in the

file:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 141

Select TRACKING STATION:

Station Name:    Maui, Hawaii

Abbreviation:  HAW

Latitude:       20.9

Longitude:     -156.7

Elevation:      396

Press ENTER to ACCEPT as Primary Location, or

press SPACE to view next Tracking Station:

To select the displayed tracking station, press ENTER. To skip to the next

tracking station entry in the file, press the SPACE BAR.

F3     Set Display Features

---------------------------

A number of display features may be enabled or disabled using a

separate sub-menu. See the section SET DISPLAY FEATURES below for a full

description.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 142

F4     Set Satellite Coordinates

--------------------------------

The F4 command may be used to select the units used to display the

current coordinates for the satellite. The choices are:

Ra/Dec    Right Ascension and Declination (Equator and Equinox of

Date).

Elv/Azm   Elevation and Azimuth. Elevation (altitude) is the elevation

above the horizon (assuming mean sea level), and azimuth is

the direction in the sense NESW (North to East to South to

West).

XYZ       Geocentric Cartesion Coordinates. The X-Axis and Y-Axis are

aligned with the Equator with the X-Axis pointing in the

direction of the Vernal Equinox. The Z-Axis points toward

the North Pole.

TARGET    Relative Range and Velocity for a Target Satellite selected

from among the current Secondary Satellites. Use F6+F5 from

the Main Menu to select the Target Satellite BEFORE

selecting this mode. When enabled, the data are displayed im

metric or English units:

TARGET:   20580          Target NORAD Number

Rng:    1661.69 km       Relative Range, kilometers

Vel:      -1.85 m/s      Relative Velocity, meters/second

or:

TARGET:   20580          Target NORAD Number

Rng:     902.55 nm       Relative Range, nautical miles

Vel:      -6.4 ft/s      Relative Velocity, feet/second

NOTE: This mode cannot be enabled if a Target Satellite has

not been selected. Relative Range and Velocity is displayed

ONLY if the Relative Range is less than 10,000 kilometers.

Since this is a "line of sight" calculation, data for large

ranges may be misleading (cutting through the Earth)!

For all modes, select the desired units of measure using F9 while the

map is displayed. The desired satellite coordinates may also be selected

using F10 while the map is displayed.

F5     Show Ascending & Descending Node Data

--------------------------------------------

The nodes of an Earth orbit are the points on the ground track where

the path crosses the Equator. The ascending node crosses from South to

North and the descending node crosses from North to South. Orbit numbers

normally increment at the ascending node. This command adds two additional

lines of data at the lower left of the screen giving the time (MET or time

since epoch) and longitude of the most recent ascending and descending

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 143

nodes. This information is useful when comparing STSPLUS's data against

other sources such as the wall map in Mission Control.

F6     Set Map Projection and Size

----------------------------------

The F6 command selects the size and field of view of the displayed

map. By default, the map is displayed using the orthographic projection,

"ORTHO", shows one complete hemisphere, and is centered so that the

selected satellite is visible. This corresponds to a magnification factor

of 100%.

Pressing F6 will select between WORLD, QUAD, ZOOM, and ORTHO maps.

Selecting WORLD will display the full world using rectangular projection

centered on the Prime Meridian at Greenwich, England at 0 degrees

longitude or at the International Date Line at 180 degrees longitude.

Selecting QUAD will select one of twelve Quadrant Maps showing a field of

view (horizontal size) of 180 degrees using rectangular projection.

Selecting ZOOM will select a Zoom Map with field of view adjustable from 30

degrees to 180 degrees; the default field of view is 75 degrees; the Zoom

Map is approximately centered on the current ground track position of the

satellite. Selecting ORTHO will select the orthographic projection. See the

sections ORTHOGRAPHIC MAPS, QUADRANT MAPS, ZOOM MAPS, and AUTOMATIC MAP

GENERATION for additional information.

F7     Enable/Disable EVENT TIMERS

----------------------------------

Press F7 to enable or disable all event timers. Event timers are

enabled by default if file STSPLUS.INI is present. Especially while the map

is being drawn, the calculations associated with the event times require

appreciable time. If the event timers are disabled, the audible alarms will

also be disabled. See the section "Event Timers and Audible Alarms" for a

full discussion of the event timers.

F8     Enable/Disable Audible ALARMS

------------------------------------

Provided event timers are enabled (above), you may press F8 to enable

or disable audible alarms. Many users allow their computer to run STSPLUS

while performing other tasks and the audible alarm will alert them to an

imminent AOS (Acquisition of Signal) or LOS (Loss of Signal) event

associated with either their local circle of visibility or the TDRS

communications satellites.

For the local circle of visibility, an "up/down" tone sounds six times

two minutes prior to AOS and five tones sound thirty seconds prior to LOS.

Provided TDRS coverage is enabled (F10+F3+F2 from the Main Menu), three

brief tones sound thirty seconds prior to AOS or LOS. Provided Sun features

are enabled (F10+F3+F8), two tones will sound approximately thirty seconds

before orbital sunrise and sunset. The characteristics of the audible tones

will thus allow the user to identify what kind of AOS or LOS event is about

to happen.

Depending upon the computer and the version of DOS being used, "music"

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 144

such as these audible alarms may cause the DOS clock to lose a small amount

of time each time an alarm sounds. The amount of time loss is quite small

but may accumulate over long periods of time. (The DOS clock may also run

either fast or slow and effectively mask the time loss due to sound

effects.)

F9     Set User-Definable Map Colors

------------------------------------

STSPLUS allows the user to set the colors for certain map features.

These features are the Local Station circle of visibility, the concentric

isocontours used in the Location and Tracking Modes, and the Tracking

Station circle of visibility color. Pressing Function Key F9 displays the

current color assignments (and a color chart on the right of the screen):

Current User-Definable Map Colors

----- 13 Local Station Color

----- 14 Isocontour Color

----- 12 Tracking Station Color

Press ENTER to ACCEPT, SPACE to CHANGE:

The "-----" in the example is a solid line illustrating the map color. The

number to its right is the number used to represent that color. The colors

shown in the example are the default colors. If the colors shown are

acceptable, press ENTER. To change the color assignments, press the SPACE

BAR and the color assignments will be prompted in turn:

Enter color for Local Station: 5

Enter color for Isocontours:   6

Enter color for Tracking Stns: 4

Use the color chart at the right of the screen to select new colors and

enter the corresponding number. Press ENTER to leave a color unchanged. As

each color is assigned, the display sample will change accordingly. When

the last color has been assigned, the initial display will be repeated:

Current User-Definable Map Colors

-----  5 Local Station Color

-----  6 Isocontour Color

-----  4 Tracking Station Color

Press ENTER to ACCEPT, SPACE to CHANGE:

Press ENTER if the new colors are acceptable, or press SPACE BAR again to

change again.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 145

F10    Enable/Disable Printer Logging

-------------------------------------

I have found it interesting to log the orbital data and the ascending

and descending node information on my printer when analyzing the mission

data over long periods of time. The F10 command toggles the printer logging

function on and off. The first page of the log includes the current orbital

data and subsequent pages contain only node information. In addition to the

information presented on the display, the printer log also calculates the

current orbital time, the time from one ascending (descending) node to the

next, for the third and subsequent nodes. A typical log is shown below.

IMPORTANT:     BE SURE THE PRINTER IS TURNED ON PRIOR TO ENTERING THE

F10 COMMAND.

STSORBIT: Space Shuttle Tracking Program, Version 9537          Page 1

ORBITAL DATA for STS-31 Discovery/HST

NORAD Number:            20580

Launch Date:             04/24/1990

Launch Time:             05:33:52

Orbit Inclination:       28.4695

Orbit Altitude:         329.50 nm

UT DATE   UT TIME ORBIT    LONG           MET     TIME

Ascend Node: 04/28/1990  20:32:52    70  -69.95    4/14:58:07

Dscend Node: 04/28/1990  21:20:52    70   97.64    4/15:46:35

Ascend Node: 04/28/1990  22:09:52    71  -94.77    4/16:35:02  1:36:55

When printer logging is enabled and the ground track is displayed, the

word "LOG" will appear in red at the right of the text area. Enabling

printer logging also automatically enagles the display of ascending and

descending node information.

The Launch Date and Launch Time entries are given if that information

has been entered indepentently. The Epoch Date and Epoch Time are always

shown. Note also that the orbit altitude shown is the altitude at the time

the log was started and will not be correct for subsequent entries,

especially if the satellite has an elliptical orbit (high eccentricity).

A printer log may be prepared in advance of a mission by enabling

printer logging from the Set Options Menu (with the F10+F10 command),

setting the desired simulation time (F8+F3 command), then starting the

ground track display with ENTER; once the ground track has appeared on the

screen, pressing the F key twice to set STSPLUS in the X60 fast time mode

will generate the date relatively quickly (although the UT TIME printed may

be off by as much as one minute in the X60 mode). Allow the simulation to

run for the desired length of time, then press ENTER to return to the Main

Menu. While the ground track is active Function Key F3 performs the same

function as the F10+F10 command to enable or disable printer logging.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 146

SET DISPLAY FEATURES

--------------------

Depending upon the satellite and personal preferences, a variety of

display features may be enabled or disabled. Not all features are available

with monochrome or CGA monitors. Pressing F3 on the Set Program Options and

Features Menu will display the following menu:

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9537

Current time:  19:01:57 PDT   02:01:57 UTC

Current date:   15 APR 1995    16 APR 1995

F1     15x15   Select Satellite Icon Size: 15/21 pixels

F2     TDRS    Display TDRS and Real Time Satellites

F3     ON      Display Additional Map Grid Lines

F4     OFF     Display Tracking Stations

F5     BOTH    Display Ground Track: DOTS/LINE

F6     BOTH    Display Local/Satellite Circles of Visibility

F7     OFF     Display South Atlantic Anomaly Zone

F8     ON      Display Terminator, SUN, and Spacecraft Lighting

F9     ON      Display Map Locations and Features

F10    ON      Display Lakes and Rivers

ENTER          Return to MAIN MENU

Select desired function:

F1     Select Satellite Icon Size: 15/21 pixels

-----------------------------------------------

This command allows the user to select the size of the internal

satellite icons used to display the ground track position of the primary

satellite. The choices are: 15x15 pixels, which should be suitable for most

applications; and, 21x21 pixels for television and display or projection

formats where higher visibility of the satellite icon is desired. This

selection has no effect when external icon files (files with names

"#nnnnn.ICN") are used; each external icon file includes its own icon

size parameter which is independent of the internal icon size selection.

NOTE: For compatibility with earlier versions, the 21x21 pixel size

may also be enabled by the "/21" command line option. However, unlike those

earlier versions, the current icon size is now saved in file STSPLUS.INI

and the smaller size is NOT selected by default when the program is

executed. Once the 21x21 icon size is selected (by either method), the

small 15x15 icon size must be re-selected using F10+F3+F1.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 147

F2     Display TDRS and Real Time Satellites

--------------------------------------------

This command allows the user to display TDRS and/or selected real time

and static satellites. The command cycles through "OFF", "TDRS", "SATS",

and "BOTH". When "TDRS" or "BOTH" is selected,the communications coverage

for the Tracking and Data Relay Satellites (TDRS) or other geosynchronous

satellites is also shown. The TDRS coverage boundaries overlap between the

East and West TDRS satellites and Mission Control may select either

satellite during the overlap period. STSPLUS displays the areas covered by

each satellite and the times for acquisition and loss of signal (AOS and

LOS). See the section "TDRS and Real Time Satellite Features" above for a

full discussion of the TDRS coverage features.

F3     Display Additional Map Grid Lines

----------------------------------------

This command is not available for CGA systems. The basic world map

includes the Equator and the meridians at 0 degrees (Prime Meridian) and

180 degrees (International Date Line) shown in bright blue on color

monitors. Turning on the map grid adds additional lines of longitude and

latitude. Displaying the additional grid lines on some monochrome monitors

may make the screen too "busy".

In the Orthographic, Quadrant and Zoom Map modes, the spacing of the

additional grid lines is adjusted for the map field of view. In all

rectangular map modes and for orthographic map modes with MAG > 500 except

for polar and near polar views, each latitude grid line is labeled at the

left and each longitude grid line at the top or bottom of the display

screen.

F4     Display Tracking Stations

--------------------------------

The F4 command enables/disables the display of the tracking stations

included in file STSPLUS.TRK or the current TRACKING STATION filename as

set by Function Key F7 on the Main Menu. If that file is not found,

internal data are used for NASA's 14 original ground tracking stations plus

the NASA Ground Terminal at White Sands, NM. Each tracking station is

located with a small red circle. The circle of visibility is also shown if

that circle has an angular diameter of 90 degrees or less. The circles of

visibility are calculated at the instant the map is drawn and may not be

accurate over long periods of time for satellites with highly eccentric

orbits. For all map modes EXCEPT the World Maps, each tracking station is

also labeled with its 3-character abbreviation. This command is not

available for CGA monitors.

F5     Display Ground Track: DOTS/LINE

--------------------------------------

STSPLUS calculates the ground track for the satellite for a period

from one and one half hours in the past to three hours in the future. Press

F5 to change from one mode to the next. Depending upon the user's

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 148

preferences, this function may be used to set the displayed ground track to

any of the following modes:

NONE      The ground track is not displayed.

DOTS      The ground track is displayed using RED dots for the past

ground track and GREEN dots for the future ground track. The

dots are spaced at one minute intervals. As time passes, the

GREEN dots will change to RED.

LINE      The ground grack is displayed using a GREEN line.

BOTH      The ground track is displayed using a GREEN line with RED

dots for past ground track minute marks and YELLOW dots for

future ground track minute marks. As time passes, the YELLOW

dots will change to RED.

F5     Display Local/Satellite Circles of Visibility

----------------------------------------------------

Function Key F6 enables and disables the local and satellite circles

of visibility, cycling through OFF, LCL, SAT, and BOTH with successive key

presses. These abbreviations have the following meaning:

OFF       Both the local and satelitte circles of visibility are

disabled.

LCL       The local circle(s) of visibility are enabled; the satellite

circle of visibility is disabled.

SAT       The satellite circle of visibility is enabled; the local

circle(s) of visibility are disabled.

The local circle of visibility is centered on your location and a

second location (if enabled), and shows the approximate area within which

direct line of sight communication with the satellite is possible. The

local circle of visibility is calculated at the instant the map is drawn

and may not be accurate over long periods of time for satellites with

highly eccentric orbits. In some situations (geosynchronous satellites, for

example), these circles of visibility cover so large an area that they

simply confuse and clutter the display; use this command to disable the

local circles in this case.

The satellite circle of visibility shows the approximate circle of

visibility from the spacecraft, the area of the Earth's surface which is

visible from the cockpit windows and television cameras or, for unmanned

spacecraft, the direct line of sight visibility from the ground. Note that

the shape of the "circle" varies depending upon the magnification or zoom

factor and map projection being used. With rectangular projection, the

shape is approximately a circle near the Equator and more like a rounded

triangle at higher latitudes; near the poles, the "circle" spreads out

across the map. This is an artifact of the rectangular map projection.

When enabled, the satellite circle of visibility is recalculated every

10 seconds based upon the spacecraft's current altitude. This means that

orbits with a high eccentricity (that is, a highly elliptical orbit whose

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 149

apogee and perigee are very different) will exhibit a constantly changing

circle of visibility.

F7     Display South Atlantic Anomoly Zone

------------------------------------------

The South Atlantic Anamoly (SAA) is an area in the southern hemisphere

lying between southern tip of Africa and South America which can cause

severe electromagnetic disturbances on spacecraft. For example, the

semiconductor memory on the Hubble Space Telescope (which regularly passes

through the SAA) was being changed by this phenomenon until a patch was

uplinked to work around the problem. The area is shown on the ground track

as an ellipse for simplicity; its actual outline is more nearly shaped like

a kidney bean. Using NASA Mission Charts for various Space Shuttle

missions as a reference, the SAA is adjusted for spacecraft altitudes from

160 nm to 350 nm (although it extends out to geosynchronous orbits).

SAA coverage is disabled in orthographic modes pending better data and

the development of a mathematical model for use in those modes.

F8     Display Terminator, Sun, and SpaceCraft Lighting

-------------------------------------------------------

This function enables and disables the Sun and related solar features.

See the section "Sun and Solar Features" for a full discussion. STSPLUS

calculates whether the spacecraft is in full sun, penumbra (partial shadow)

or refracted sunlight, or umbra (full shadow) and adjusts the color of the

spacecraft icon accordingly: bright white, yellow, and dim white

respectively. This feature is not available on CGA and HGC monitors. The

current spacecraft solar lighting is indicated in the data block (next to

"Orbit #:") by the following symbols:

*    Full sunlight

+    Partial sunlight (penumbra)

-    Refracted sunlight

Full shadow (umbra)

F9     Display Map Locations and Features

-----------------------------------------

This feature enables or disables the display of the map locations and

features contained in file STSPLUS.LOC if present. See the section

"Location and Features Labels" above for a full discussion.

F10    Display Lakes and Rivers

-------------------------------

This feature enables or disables the display of lakes and rivers on

the map. Removing the lakes and rivers will lessen the time required to

draw a map and can improve screen legibility especially for CGA systems.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 150

(The lakes and rivers are always disabled on the rectangular world map to

avoid cluttering an already busy display!)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 151

STSORBIT PLUS's Orbital Model

-----------------------------

The original version of STSORBIT was first prepared without reference

materials of any kind and the simplest possible orbital model was therefore

selected. The primary objective was to duplicate the NASA wall map at

the Mission Control Center in Houston, Texas. This "simple" model assumed

that the orbit was perfectly circular at a specified altitude and

inclination which never degraded due to other factors such as drag or

perturbation. Some simplifying assumptions were incorporated to handle the

initial ascent portion of a mission and the launch site was hard coded to

Cape Canaveral, Florida. With only minor modifications, the program was

essentially unchanged for the next year.

The launch of STS-31 and the Hubble Space Telescope highlighted the

need for improved accuracy because of public interest and the length of the

mission. The orbital calculations were modified (STSORBIT Version 9015) to

include the J2 factor, the perturbation of low Earth orbits due to

variations in the gravitational field related to the non-spherical shape of

the Earth (among other factors); omission of the J2 factor caused errors in

longitude of approximately -5 to -7 degrees per day. That is, the orbital

track drifted Westward from its true position by that amount. More accurate

models of low orbits also include the J3 and J4 perturbation factors,

atmospheric drag, and a host of other less significant items.

Although reasonably accurate for the first day or so of a space

shuttle mission, the "simple" model is by no means ideal. In calculating

the current orbital longitude, for example, the "simple" model assumes a

circular orbit. For orbits with low inclinations, as is typical for

launches from Kennedy Space Center, the errors are not particularly

significant and are probably overshadowed by the fundamental uncertainties

in orbital parameters and by the limitations imposed by display resolution.

Orbits with higher inclinations, as would be the case if near-polar

launches from Vandenburg AFB in California are ever initiated, would have

much larger periodic errors which would be both noticeable and

objectionable. More important for longer missions and for general satellite

tracking is the fact that due to the method used, errors in the orbital

calculations tend to be cumulative. After a day or two, the errors become

unacceptably large.

The real problem with the simple method, of course, is that the Earth

is not a perfect sphere and actual satellite orbits are never perfectly

circular. Satellite orbits are significantly perturbed by the non-spherical

gravitational field of the Earth, by the Sun and Moon, atmospheric drag,

and other factors. Accurate satellite tracking over longer periods of time

therefore demands more accurate data and a more rigorous treatment of

satellite orbits. The only practical alternative is to use the NASA/NORAD

2-line orbital element sets. Not only are these data readily available

publicly, but they are relatively accurate and are updated regularly.

Therefore, STSORBIT PLUS relies on the NORAD SGP4 prediction model and the

2-line orbital element sets for orbit predictions. 2-line element sets for

non-military space shuttle missions are typically available on the same day

as the launch. Amateur astronomers and satellite tracking experts often

generate "unofficial" 2-line element sets even for military missions.

Six quantities are required by classical gravitational theory to

completely characterize the orbit of one body about another in time and

space, the "Two Body Problem". These six quantities, often referred to as

Keplerian orbital elements, are included in the NASA/NORAD 2-line element

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 152

sets along with other numerical and statistical data. The U.S Space

Command, (formerly NORAD, the North American Air Defense Command)

headquartered in Cheyenne Mountain, Colorado, developed the SGP4 and SDP4

orbital models and the 2-line element format many years ago as part of

their satellite tracking efforts and NASA subsequently adopted the same

format -- more or less. NASA and NORAD do not always use the same

definition for revolution (orbit) numbers; NASA frequently gives a number

one (or two) greater than NORAD, calling the first partial orbit number one

while NORAD calls that same partial orbit number zero. Except for short

duration missions, such as the Space Shuttle, revolution numbers are of no

practical importance.

Having timely and accurate orbital data is of little help without a

computer model or program which can use those data. NORAD has rather

arbitrarily divided satellite orbits into two categories: near Earth orbits

and deep space orbits. Near Earth orbits are defined as those with orbital

periods of 225 minutes or less and deep space orbits are all others.

Computer models are described in the literature for each category. STSORBIT

PLUS employs the SGP4 Near Earth Model only, using a composite of code of

my own combined with translated Fortran and Basic source supplied by Paul

Traufler and C source by Paul Hirose. I plan to add the SDP4 deep space

model in due course. Not only are the near Earth orbits generally of more

interest to observers, but the errors associated with deep space orbits

processed with the SGP4 model (rather than the correct SDP4 model) are not

particularly significant for the purposes of a program such as STSORBIT

PLUS. Further, watching a geostationary satellite orbit on the screen is

not unlike watching grass grow and is about as exciting.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 153

Accurate Time and the Personal Computer

---------------------------------------

For a program like STSPLUS, accuracy and precision of the timekeeping

functions are essential. There is a tendency these days to accept whatever

a computer says as the absolute truth without regard for whether or not the

information is even "reasonable". For something as basic as time, even an

experienced computer user may assume that it is correct. This discussion

attempts to compare reality with that expectation.

Given the clock drift and accuracy problems inherent in the design of

the typical IBM-compatible personal computer, frequent time setting and/or

adjustments are required. Accurate time setting would not be practical

for most people without the various time services provided by the National

Institute of Standards and Technology (NIST) and the U.S. Naval Observatory

(USNO). The NIST radio stations WWV and WWVH provide an inexpensive and

convenient means for "ordinary folks" to synchronize clocks and other

equipment. The NIST and USNO Telephone Time Services offer a high precision

standard time calibration source when such accuracy is required. Similar

radio and telephone services are available in Canada and Europe.

Once a computer clock has been set with reasonable accuracy, the

accuracy of the computer's clock will indeed be sufficient for many

applications; if you are using a word processing or spreadsheet program,

knowing the time to within a minute or two is probably adequate. For

programs such as STSPLUS and other time-dependent applications, however,

this level of accuracy simply will not suffice; when used for satellite

tracking, the time should be accurate to within a second. Unless steps are

taken to both set the clock and to maintain its accuracy, this will not be

the case. No matter how accurately the clock on a typical personal computer

is set, it will only be a matter of hours before the time will have drifted

by some seconds. Measured over a number of days, the accumulated errors can

easily amount to a minute or more.

The timekeeping operations of an IBM-compatible computer are actually

performed by two separate and independent functions: a clock-calendar

CMOS integrated circuit and lithium battery combination which maintains the

current time and date in hardware; and, a section of the DOS operating

system software which maintains the current time and date in software. When

computer power is off, the hardware chip continues to operate using its

battery; when the computer is started ("booted"), the operating system

software reads the hardware clock and sets its internal software clock.

Absent special software, the DOS time thereafter relies entirely on the

software clock until the next time the computer is restarted.

Unfortunately, neither of these clocks was designed for accuracy; early

versions of the PC did not even include the hardware/battery arrangement.

Even the typical electric clock, which uses the power line frequency for

its timekeeping reference, is usually far more accurate.

The accuracy of the DOS time at any instant is the result of the

accumulated errors in both clocks. The hardware clock will drift as a

function of time, temperature, voltage, and crystal aging; the software

clock will gain or lose time depending upon the skill with which its

software was written and how well that software "cooperates" with the

balance of the computer's hardware and software. Some software, especially

network and high speed communications software, can prevent the DOS clock

software from incrementing when it should, usually resulting in the DOS

clock losing time. The problem was compounded with the release of DOS

Version 3.3; beginning with that version, the DOS TIME and DATE commands

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 154

adjust BOTH the hardware and software clocks and thereby potentially

eliminate the hardware clock as even a modestly reliable reference.

Methods for Setting DOS Time

----------------------------

Bearing these considerations in mind, there are a number of approaches

to the DOS time question. The most obvious approach, used by the vast

majority of computer users, is to either ignore the computer clock entirely

or to say "It's close enough". Regardless of the application, I strongly

recommend that the DOS clock be REGULARLY set to the correct time if only

to assure that files are more or less correctly date and time stamped. If

the accuracy of DOS time is important, the computer clock may be set or

synchronized in a number of ways, some of which are described below. In

this context, "ACCURACY" means the accuracy of the time setting operation

and NOT the longer term accuracy and stability of the DOS time.

1.   TELEPHONE: Many local telephone companies offer a telephone time

service, usually with a message such as "When you hear the signal the

time will be ... (beep)". I am not aware of any hardware or software

which uses this signal for time setting purposes.

ACCURACY: Generally plus or minus 5 seconds. With the advent of

digital voice response equipment in recent years, the accuracy has

improved to perhaps plus or minus 1 second.

2.   COMMERCIAL RADIO: Hourly time signals broadcast on commercial radio

may be used to manually set the time. My experience suggests that the

CBS network time signal is usually reliable.

ACCURACY: Usually within plus or minus 2 seconds, depending upon the

source. Satellite distribution of network feeds add a time delay of

approximately 0.25 seconds per "hop" but some stations, including some

network stations, generate time signals locally.

3.   SHORTWAVE RADIO: Time signals are broadcast on shortwave radio

stations WWV and WWVH by the National Institute of Standards and

Technology. These time signals may be used to manually set the time.

WWV and WWVH broadcast on several frequencies: 2.5MHz, 5MHz, 10MHz,

15MHz, and 20 MHz (WWV only). Reception will vary according to your

distance from the transmitter, time of day, and atmospheric

conditions. These time signals are very precise; the only major

variable is the propagation delay, the time it takes the radio signal

to travel from the transmitter to your receiver. The typical

propagation delay is approximately 5 microseconds per mile. Outside

North America, other national radio services such as the British

Broadcasting Company's BBC World Service offer accurate hourly

shortwave time signals.

ACCURACY: Time setting using WWV or WWVH can usually be performed to

within about plus or minus 250 milliseconds, of which up to 25

milliseconds is transmission time and the balance is user response

time. With practice, plus or minus about 100 milliseconds is

practical.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 155

4.   HEATH GC-1000 MOST ACCURATE CLOCK: The GC-1000 is a combination

digital clock and scanning shortwave radio receiver which may be

equipped with an RS-232 communications port for use with computers and

other electronic equipment. Operation with DC power is available to

maintain accurate time during periods of AC power loss. This is the

only method which provides more or less continuous accurate time

information without telephone toll charges.

ACCURACY: When properly configured for your location, equipped with an

external antenna, used with appropriate computer software, and when

the receiver is locked to one of the WWV (or WWVH) time signals, the

GC-1000 can provide time information and a standard calibration

frequency to an accuracy of plus or minus 10 milliseconds. When signal

lock is lost, the receiver scans the 5MHz, 10MHZ, and 15MHz broadcasts

to reacquire signal and lock. Even after signal lock is lost, the

receiver maintains an accuracy of plus or minus 100 milliseconds for

some hours.

5.   NIST/USNO TELEPHONE TIME SERVICE: When real precision and accuracy are

required, the computer clock may be set remotely using the telephone

time service of either the National Institute of Standards and

Technology (NIST, formerly the National Bureau of Standards or NBS) in

Boulder, Colorado, or the U.S. Naval Observatory (USNO) in Washington,

D.C. This method requires a modem connected to a telephone line and is

available for systems using DOS version 3.3 or higher AND equipped

with 80286 processor or higher; some 8088-equipped systems may also

use this method depending upon the type of clock hardware installed

and the version of DOS being used. The recommended method uses the

programs TIMESET and RIGHTIME (see below) although other commercial

and shareware programs may be available.

ACCURACY: This is the most accurate method available for setting and

maintaining the DOS clocks. Depending upon which service is used, NIST

or USNO, whether or not line delay compensation ("lag") is employed,

and the frequency of time setting, the DOS time can be set to within

plus or minus 2 milliseconds. However, since the "time ticks" of the

DOS software clock occur every 55 milliseconds, or 18.2 times per

second, this "granularity" may limit the accuracy of reading the DOS

clocks. See the documentation for programs TIMESET and PRECISION TIME

(below) for additional discussion.

Choose one of the methods suggested or a suitable alternative based

upon your precision and accuracy requirements. Other methods of maintaining

an accurate time standard such as atomic clocks, Global Positioning

Satellite (GPS) time receivers, and NIST time code equipment, are also

available -- for a price. Those methods are beyond the scope of this

documentation.

Maintaining Accurate DOS Time

-----------------------------

Just in case you missed the point earlier, accurately setting DOS time

is only half the battle. Even if the DOS time is set very precisely as

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 156

discussed above, all that assures is that the time is correct to the

required accuracy at that instant. The problem then becomes one of knowing

how the DOS clocks change or drift with time and how to compensate for

those changes or, alternatively, checking the DOS time frequently enough

that any drift on the part of the DOS clocks is acceptable for the intended

application.

Of the two clocks in a typical personal computer, the hardware clock

is considerably more consistent and reliable. I have checked perhaps a

dozen PC hardware clocks in recent years, and almost all kept reasonably

good time over a period of several days; as expected, none kept "perfect"

time. Typical drift rates ranged from about 3 seconds per day to near zero

seconds per day, with the magnitude and direction of the drift more or less

constant over the period of measurement. The hardware clock is typically

sensitive to both voltage and temperature, both of which undergo

significant change when the computer is turned on or off. Complete

calibration of the hardware clock requires knowledge of its performance

under both circumstances. Once a hardware clock has been calibrated,

its performance may be predicted with reasonable accuracy over periods of

some weeks or more. Crystal aging rates suggest that calibration should be

performed at least monthly.

The hardware clock is normally interrogated only when the computer is

first started or rebooted. The correct time can therefore be predicted at

that moment for a calibrated hardware clock, given the last time that clock

was synchronized with an appropriate time standard. Microsoft provides no

standard software tools for interrogating the hardware clock at other

times except for low level interrupt services. Quite the contrary;

beginning with DOS Version 3.3, using the DOS TIME and DATE commands to set

the DOS software clock will also set the hardware clock and effectively

destroy its usefulness as a calibrated time reference. I am at a complete

loss to understand the reasoning behind this change in DOS; I presume that

users were being "confused" by differences between the hardware and

software clocks; instead of either explaining or fixing the problem,

Microsoft elected to "legislate" the problem away -- a process any

politician would recognize instantly. The only mitigating consideration is

that any really effective solution would probably require hardware as well

as software changes. Blame IBM, I guess.

The software clock provides the only time information readily

accessible to DOS using standard software. Since this clock is maintained

entirely in software, with no reference to the hardware clock except at

bootup, it is at the mercy of other software which may execute from time to

time. The software clock increments its time using "interrupts", a

technique which stops a software process in progress just long enough to do

the required tasks and then resumes the interrupted process. These

interrupts occur every 55 milliseconds. So long as none is missed, the

software clock should keep accurate time -- if the software is written

correctly and if the computer's crystal controlled oscillator is in turn

accurate. It may be that neither of these conditions is true; certainly the

crystal controlled oscillator (quite similar to the one which runs the

hardware clock) was not designed for accuracy or stability. It's original

purpose was solely to generate the necessary timing signals for the

operation of the computer. Cost, not accurate time, was the primary

consideration in its design.

Other software designers have contributed to the problem by writing

software which, deliberately or inadvertently, prevents the software clock

from being updated. Off-brand BIOS firmware can present occasional

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 157

problems. Local Area Network (LAN) and high speed communications software

are also frequent culprits in this respect. For example, a casual check of

the clock while using a high speed computer-to-computer file transfer

program indicated that the clock was effectively suspended when data

transfers were in progress. In one relatively brief test, the DOS clock

lost about 30 seconds.

As a result of all of these factors, the accuracy of the DOS software

clock can vary wildly from one computer to the next and from one situation

to another. One inexpensive "clone" computer that I'd rather forget

couldn't manage to keep time to better than about 30 seconds per HOUR!

Before planning to use a particular computer as a time reference with

programs like STSPLUS, check the computer hardware and software you intend

to use very carefully.

Programs TIMESET and RIGHTIME

-----------------------------

Two fine shareware programs, TIMESET by Peter Petrakis and RIGHTIME by

Tom Becker, provide all the features required to accurately set and

maintain the computer's hardware and software clocks. Development efforts

on these programs have been carefully coordinated so that they cooperate

with each other. Both programs are copyrighted commercial software

distributed as "shareware" and require registration after an initial

evaluation period. I highly recommend these programs and encourage users to

support the authors and their work. So far as I know, there are no other

comparable programs available at any price!

TIMESET, Version 7.10 or higher, uses the telephone time services of NIST,

USNO, and three European services to precisely set the computer clocks. The

standard distribution also includes several additional time-related utility

programs. It is available on many computer bulletin board systems or direct

from:

Peter Petrakis

Life Sciences Software

8925 271st N.W., Suite 112

Box 1560

Stanwood, Washington  98292  USA

Telephone: (206) 387-9788

RIGHTIME, Version 2.5+, is a program to compensate for the various drift

factors in a computer's hardware and software clocks. When used in

conjunction with TIMESET and properly calibrated, RIGHTIME "learns" the

warm and cool drift factors for a specific computer. As a result, the

clock's can be maintained with an accuracy of a fraction of a second over

long periods of time. It is available on many computer bulletin board

systems or direct from:

Tom Becker

Air System Technologies, Inc.

14232 Marsh Lane, Suite 339

Dallas, Texas  75234  USA

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 158

Telephone: (214) 402-9660

Tom Becker and Peter Petrakis may be also contacted directly on the Air

Systems Technologies computer bulletin board system in Dallas, Texas. The

BBS always has the latest versions of TIMESET and RIGHTIME available for

download:

Air Systems Technologies BBS

(214) 869-2780

STSPLUS is now "aware" of program RighTime and its use is recommended

for accurate timekeeping. Audible alarms in prior versions would perform

unpredictably when RighTime was active because they use the hardware

clock's timer functions (which RighTime also uses). STSPLUS now detects

RighTime and temporarily disables RighTime while an audible alarm is being

generated and then re-enables RighTime after the alarm has completed,

restoring precise timekeeping. With RighTime active, alarms are generated

in foreground, which may cause a slight delay in screen updating.

*************

*  CAUTION  *

*************

STSPLUS expects RighTime Version 2.5+; performance with prior

versions of RighTime may yield unpredictable results. If using a

prior version of RighTime, do NOT enable audible alarms!

If RighTime is not present or is not detected, the audible alarms are

generated in background as in prior versions. This usually causes the loss

of several clock ticks in the DOS software clock for each audible alarm.

Although the time loss per audible alarm is very small, the cumulative

error may become significant over extended time periods.

The following descriptive text is extracted with permission from the

documentation for the current versions of TIMESET and RIGHTIME; please

consult the documentation for each program for full details. Although

future versions of both programs are expected to remain compatible with

STSPLUS, they should be tested carefully before regular use.

FEATURES OF TIMESET 7.10

------------------------

TimeSet has been evolving steadily ever since the first version was

released in the summer of 1987.  That version and several subsequent ones

could only set a computer's clock from the U.S. Naval Observatory (USNO) in

Washington, D.C.  Version 6.00, released in 1990, added ability to use

telephone time signals from the National Institute of Standards and

Technology (NIST) in Boulder, Colorado, making it the first program of its

kind able to address more than one atomic time service.  This made it

possible for computer users in the eastern and western United States to

keep down long distance bills by choosing the time service closest to them.

Version 7.10 continues that evolution with a number of new features

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 159

and supporting utilities:

o    TimeSet can now access five atomic clock-based telephone time services

on two continents: the USNO and the NIST in the United States, as

before, and atomic time services in Sweden (National Time and

Frequency Laboratory), Austria (Technical University of Graz), and

Italy (National Electrotechnical Institute).  People in European

countries who want to set their computers to an atomic clock no longer

need to make a trans-Atlantic phone call.

o    TimeSet 7.10 is designed to interact closely with version 2.5+ of

RighTime (tm), the excellent memory-resident regulator for computer

clocks developed by Tom Becker of Air System Technologies, Inc.,

Dallas.  RighTime learns the drift rate in the computer's clock and

continuously applies a correction to compensate for it, and it refines

the correction each time the computer clock is set.  A computer with

RighTime installed and trained can maintain system clock accuracy

within a second for at least a week. Furthermore, version 2.46

provides true 0.01-sec resolution in the DOS clock, in contrast to the

normal 0.055-sec resolution.  This allows greater accuracy in

timesetting than ever before, indeed the maximum accuracy that can be

obtained with a computer clock.  Life Sciences Software and Air System

Technologies cooperated closely during the development of TimeSet 7.10

and RighTime 2.46, with the result that TimeSet can access several

RighTime functions directly.

"TIMESET" is a trademark of Life Sciences Software (TM)

The following is a screen dump of the data displayed by TIMESET:

+------------------- From NIST.  Connect time: 11.97 sec. --------------------+

| DATA FOR TIME CALCULATIONS (all data pertain to Coordinated Universal Time) |

|    Time data string:  49051 93-03-05 14:07:20 81 0 -.1 051.1 UTC(NIST)      |

|                Date:  03-05-1993                                            |

|         Julian date:  2449051                                               |

|         Day of year:  064                                                   |

|                Hour:  14    Minute:  07    Second:  20                      |

|              The United States mainland is on standard time.                |

|        U.S. daylight time begins on 04-04-1993 at 02:00:00 local time.      |

+-----------------------------------------------------------------------------+

System clock set: 06:07:20.01

+-----------------------------------------------------------------------------+

|                 ACTION SUMMARY (at instant of timesetting)                  |

|    Internal delay adjustment:  .01 sec. (added to set time)                 |

|        Line delay adjustment:  .0511 sec. (precompensated by time service)  |

|                                                                             |

|   Universal Time Coordinated:  14:07:20.01           (time at 0x longitude) |

|                     UTC Date:  03-05-1993, Friday    (date at 0x longitude) |

|                                                                             |

|      Local computer time was:  06:07:20.01 (RighTime-assisted)              |

|                       Set to:  06:07:20.01 Pacific Standard                 |

|      Local computer date was:  03-05-1993                                   |

|                       Set to:  03-05-1993, Friday, Day 064 of 1993          |

+-----------------------------------------------------------------------------+

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 160

FEATURES OF RIGHTIME 2.5+

-------------------------

RighTime brings exceptional system time of day clock performance to

the DOS-based AT-class-compatible PC computer with no additional hardware.

With RighTime installed, the standard real time clock system becomes an

Adaptive Mathematically Compensated Crystal-controlled Oscillator based

clock.  Under stable conditions, RighTime can produce a system clock that

keeps time within one half second per week or better (some testers have

reported accuracy of 0.07 second per week); this is about 0.8 parts per

million error, or more than 100 times better than an unconditioned crystal

time base alone, or 30 times better than a moderately conditioned one like

a modern watch of quality.

o    True one hundredth second DOS clock resolution: the 55-millisecond

barrier is broken!  The standard DOS clock resolves to only about 1/18

second; under RighTime v2, the new high resolution DOS clock resolves

to, and increments in, hundredths while the Int 08h and 1Ch tick rate

remains standard.

o    RighTime intrinsically sets the hardware clock and solves the midnight

rollover date bug that exists in some DOS versions; this eliminates

the need for other utility programs or drivers that perform these

functions.  Unlike DOS alone, the hardware clock seconds transition

will be properly set by RighTime and the time will be set to

hundredths of a second resolution, and these qualities will survive

through rebooting.

o    Each time you set the time, RighTime will improve the accuracy of the

clock error corrections and will subsequently improve the accuracy of

the clocks.  It should be easy to achieve a worst-case error of less

than 0.5 second per day and under good conditions, less than 0.5

second per week; typical results are much better.  Command line

options are provided that allow fine tuning the correction process to

your system.  A trimming option provides for offset adjustments in

hundredths of a second.

The following is a screen dump of the data displayed by RIGHTIME:

RighTime: Indicated DOS clock date and time is 1993/03/05 06:04:45.66.

RighTime: Warm correction rate is +2.83 seconds per day.

RighTime: Cool correction rate is +4.27 seconds per day.

RighTime: Current applied DOS-CMOS RTC offset is +0.46 second.

RighTime: Last CMOS RTC adjustment was 0.00 hours ago.

RighTime: Last timeset was 23.33 hours ago.

RighTime: System has been warm 17% of the time since the last timeset.

RighTime: Stack A headroom is 92 bytes; Stack space used is 68 bytes.

Stack D headroom is 100 bytes; Stack space used is 60 bytes.

RighTime: /?=Help; Version 2.53

RighTime: Copyright 1991-93 GTBecker, Dallas 214/402-9660. All Rights

Reserved.

RighTime: Resident and enabled.

RighTime: Selftest passed.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 161

Program PRECISION TIME

----------------------

Program PRECISION TIME, a commercial product by Crystalogic, Inc.,

provides in a single program the time setting and time maintenance

functions to keep a PC clock running accurately. An AT-class (80286 or

higher) computer is required and, as with all such programs, the computer's

hardware clock must be "reasonably" stable and predictable with respect to

cold and warm drift rates in order to maintain long term accuracy.

PRECISION TIME is suitable for applications which require an absolute

accuracy of approximately +/-1 second referenced to NIST or USNO;

applications requiring higher accuracy should consider alternative

solutions. The one second granularity of the program's time adjustments may

also cause difficulties for certain applications.

PRECISION TIME ("PTIME") is an "all-in-one" program which calls a

telephone time service (NIST or USNO), sets the computer's hardware and

software clocks, then executes as a TSR (Terminate and Stay Resident)

program to maintain the accuracy of the hardware clock to within one

second. Calling the telephone time service may be performed manually on

demand or at predetermined intervals (1 to 999 days). PTIME "guards"

against inadvertent setting of the time or date EXCEPT by PTIME using a

telephone time service. PTIME may be configured to handle the changes

between Standard Time and Daylight Savings Time automatically. All program

activity may be logged for future reference in a log file, PTIME.LOG.

PTIME is well documented with a printed manual and installs easily and

quickly. Although PTIME does not adjust the hardware clock as frequently

nor to comparable precision as do programs such as RIGHTIME (above), this

may make its internal interface somewhat more robust and reliable in

complex hardware or software environments. During the entire time that

PTIME has been operating on the RPV ASTRONOMY BBS there have been no

failures or system crashes. As the following (edited) excerpt from

PTIME.LOG illustrates, the system clock is updated periodically and is kept

within approximately one second of the correct time as received from NIST:

06/21/1994  07:24:11.00  PTIME - Time adjusted one second backwards.

06/21/1994  10:32:16.01  PTIME - Time adjusted one second backwards.

06/21/1994  13:40:18.01  PTIME - Time adjusted one second backwards.

06/21/1994  16:48:24.03  PTIME - Time adjusted one second backwards.

06/21/1994  19:56:28.03  PTIME - Time adjusted one second backwards.

06/21/1994  23:04:34.00  PTIME - Time adjusted one second backwards.

06/22/1994  02:00:41.04  Time was changed by: Calling an Atomic Clock

Drift amount was: 00:00:00.98

06/22/1994  05:08:42.01  PTIME - Time adjusted one second backwards.

06/22/1994  08:18:49.00  PTIME - Time adjusted one second backwards.

In fact, the error of 0.98 seconds was the largest seen in two months of

operation and, after an initial calibration period, NIST is called only

every five days. (The RPV ASTRONOMY BBS primary computer is always turned

on except for very brief maintenance periods.)

The user may view the current status of PTIME and its adjustments at

any time. The following is an example screen:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 162

----------------- Time Adjustments ------------------

Calibration Driver Version:    1.56

Last Sleep Time:               06/22/1994 06:55:28.05

Last Wake Time:                06/22/1994 06:57:37.39

Last Wakeup Adjustment:        +0 seconds

Current Time:                  17:40:58.10

Seconds Between Adjustments:      11282

Hundredths/Sec to Adjust:          -100

Seconds Until Next Adjustment:     7771

Adjustments Handled:                262

Error Count:                          0

Last Adjustment                06/24/1994 16:42:29.03

Sleep Time Since Last Call:         129 (0%)

Awake Time Since Last Call:      229151 (100%)

PRECISION TIME is available from:

Crystalogic, Inc.

2525 Perimeter Place Drive, Suite 121

Nashville, TN  37214

Voice: (615) 391-9100

FAX:   (615) 391-9997

BBS:   (615) 391-8065

As of 1 May 1994, the list price for PRECISION TIME was $79.95 plus

shipping and handling. Contact Crystalogic for current information.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 163

Computer Bulletin Board Systems and the Internet

------------------------------------------------

RPV Astronomy BBS        (310) 541-7299

CELESTIAL BBS            (334) 409-9280  <-- Note new number 1/01/95

NASA GSFC OIG RBBS       (301) 262-6784  <-- Note new number 7/24/93

NASA Spacelink BBS       (205) 895-0028

CSS BBS                  (905) 458-5907  <-- Note new number 3/15/94

JPL PAO BBS              (818) 354-1333

JSC ARC BBS              (713) 244-5625

Timely 2-line orbital elements are essential for accurate satellite

tracking. In addition to my own bulletin board systems, six other bulletin

board systems provide authoritatve data for the general public. Some or all

files on these systems are compressed to reduce download time and must be

decompressed before use; compressed files may be recognized by file types

such as ".ZIP", ".ARJ", ".ARC", ".PAK", ".Z", etc.

******************

*  CAUTION NOTE  *

******************

Until recently, cross-platform compatibility problems were

all but non-existent. PC files stayed with PCs, MAC files with

MACs, and so forth. UNIX files were relatively rare in the PC

world. With the increasing use of the Internet (which is usually

hosted by a UNIX system), files often migrate from one platform

to another and this process may not be obvious to the user.

The use of the Internet for data distribution, therefore, has

meant that UNIX and MAC files often find their way to a PC. TLE

files from NASA Spacelink are a case in point. Text and ASCII

data files which have been prepared or processed by a UNIX system

terminate each text line with LF (Line Feed) alone rather than

with CR+LF (Carriage Return plus Line Feed) which is the DOS

standard. MACs use CR only as a line terminator. STSPLUS and most

other DOS programs will not correctly read these files. Some DOS

editors (such as QEDIT by SemWare Corp.) will correctly interpret

UNIX text files and will save the files in the standard DOS

format. The process is usually completely transparent to the

user; simply load the file into the editor and then resave it.

To compound the problem, most printers DO print UNIX files

correctly. Printing a suspect file as a test may NOT reveal the

problem. Use the DOS "TYPE" command to view a suspect file. If

text lines appear scattered all over the screen, it is UNIX-

formatted and must be corrected before use in most DOS programs.

I have maintained my RPV Astronomy BBS (computer bulletin board

system) as a public service since 1987, (310) 541-7299, 2400-28800 baud

with 2 lines. The system has some 2000+ files, 270MB, available for

download. During Space Shuttle missions, I post current 2-line elements at

least daily (provided I'm in town!). The system usually also has current 2-

line elements for a large number of satellites in addition to other files

of interest to space and astronomy enthusiasts. The system is very popular

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 164

and, therefore, is frequently busy! The RPV Astronomy BBS is open to all

callers but first time callers are granted more limited access and time

than registered users.

Since the mid-1980s, Lt. Col. T. S. Kelso, USAF, has been making the US

Space Command (formerly NORAD) orbital data available as a public service

on his Celestial BBS at (334) 409-9280, 1 line at 1200 to 9600 baud. The

2-line element sets are prepared by Kelso using data received directly from

U.S. Space Command (formerly NORAD) by special arrangement. I regularly

post a concantenated and sorted version of the current element sets on my

own RPV ASTRONOMY BBS as file TLEnnn.ZIP, where "nnn" is a number such as

"530", the current Prediction Bulletin number. Kelso provides data for

several categories of satellites:  Amateur Radio, Earth Resources, Manned

Spacecraft, Navigation, Weather, and NASA's 30 Day Specials (which contain

objects launched within the last 30 days and are often easy to spot

visually). More specifically, these include the following satellites or

satellite series: OSCAR, Radio Sputnik, UOSAT, Cosmos, LandSat, SeaSat 1,

SPOT, Mir, Salyut 7, Soyuz, Space Shuttle, NAVSTAR (GPS), GOES, Meteor, and

NOAA.

The Canadian Space Society BBS, (905) 458-5907, 1 line at 1200 and

2400 baud, also regularly posts NORAD 2-line elements. Much of the orbital

data is obtained from Celestial BBS but additional data is generated by Ted

Molczan and his worldwide team of observers. Note that the CSS format is

slightly non-standard, having additional information on the first (title)

line for each satellite, and may have to be edited for use with some

tracking programs. The CSS files also have considerable additional text

material (including current satellite news) before and after the actual 2-

line elements data; this text may have to be edited out before use.

The U.S. Space Command sends all unclassified 2-line elements to the

Orbital Information Group at Goddard Space Flight Center. These elements

are available on the GSFC OIG RBBS, (301) 262-6784, 4 lines @ 1200 to 9600

baud. The OIG database contains elements for some 7000+ satellites and is

updated every weekday morning except holidays. Elements for popular

satellites are posted in a file called GROUPTLE.ZIP which contains seven

".TLE" files for some 750+ popular satellites. The data for the entire

database of satellites are available on a query basis. Access time is

limited to 20 minutes per caller per day. I regularly post a combined and

sorted set of the .TLE data as file GSFCnnn.ZIP on my own RPV ASTRONOMY BBS

where "nnn" is a number like "490". Individuals who wish access to the RBBS

must write (include full name and address):

NASA Goddard Space Flight Center

Project Operations Branch/513

Attn: Orbital Information Group

Greenbelt, MD  20771  USA

The OIG RBBS began operation in September, 1991. Until that time, the only

method for obtaining the OIG data was by mail. OIG discontinued mail

service in August of 1994 and the data is now available only via the RBBS.

The NASA SpaceLink BBS in Huntsville, Alabama, (205) 895-0028, 8 lines

@ 300-9600 baud, provides mission information for all space shuttle

missions and (usually) 2-line orbital elements both pre-mission and while a

mission is in progress. 2-line elements for selected satellites are also

available. In addition, SpaceLink has a wealth of other NASA information,

computer programs, teaching materials, and image files.

In January of 1993 the Public Affairs Office at the NASA Jet

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 165

Propulsion Laboratory began BBS service related to JPL-supported missions.

Mission status reports and high quality GIF images are the principal files

currently available. The number is (818) 354-1333, two lines at 1200 to

9600 baud.

The Amateur Radio Club (ARC) at Johnson Space Center operates a BBS

with one line, (713) 244-5625 @ 1200-9600 baud. Current elements for the

Space Shuttle and other files of interest to amateur radio are posted.

A great (one might even say overwhelming) variety of information,

including TLEs, is now available on the Internet. Instruction on the use

of the Inernet is beyond the scope of this documentation ... there are too

many variations that are site and computer and software dependent. However,

the following selected sites include 2-line elements (TLEs), other useful

information, and/or images:

NASA Spacelink           spacelink.msfc.nasa.gov

NASA GSFC OIG            telnet oig1.gsfc.nasa.gov

Air Force Institute      archive.afit.af.mil/pub/space

Jet Propulsion Lab       kilroy.jpl.nasa.gov/pub/space/elements

The above sites are all available for public access. For NASA Spacelink,

use VT-100 emulation and follow the on-screen prompts to log on. The NASA

GSFC OIG (Orbital Information Group) site requires registration which can

be done on-line; use for login "oig" and for password "goddard1" (without

the quotation marks in both cases), then follow the on-screen prompts.

TLEs for current Space Shuttle missions are also posted on the

ASTROFORUM on CompuServe by Ken Ernandes and others.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 166

Bibliography of Astronomy and Space Subjects

--------------------------------------------

The following principal sources have been consulted during the

preparation and/or testing of my programs ASTROCLK, STSORBIT, and STSPLUS

as well as this documentation. Initially prepared for ASTROCLK and

therefore somewhat slanted towards astronomy, this bibliography contains

the fundamental references for celestial mechanics and related topics.

------, THE ASTRONOMICAL ALMANAC. U.S. Government Printing Office,

Washington, DC. Annual volumes for 1983 through 1995.

------, THE NAUTICAL ALMANAC 1989. U.S. Government Printing Office,

Washington, DC, 1987.

------, NBS TIME & FREQUENCY DISSEMINATION SERVICES, NBS Special

Publication 432. U.S. Government Printing Office, Washington, DC,

1979.

------, THE TIMES ATLAS OF THE WORLD, Comprehensive Edition, 6th ed.

Times Books, New York, NY, 1980

Acker, Agnes and Jaschek, Carlos, ASTRONOMICAL METHODS AND

CALCULATIONS. John Wiley & Sons, New York, NY, 1986.

[First published in French in 1981.]

Bretagnon, Pierre and Simon, Jean-Louis, PLANETARY TABLES AND PROGRAMS

FROM -4000 TO +2800. Willmann-Bell, Inc., Richmond, VA, 1986.

Burgess, Eric, CELESTIAL BASIC. Sybex Inc., Berkeley, CA, 1982

Carroll, Tim S., THE FLOPPY ALMANAC USER'S GUIDE, 2nd Edition.

Nautical Almanac Office, United States Naval Observatory, Washington,

DC, 1988.

Danby, J. M. A., FUNDAMENTALS OF CELESTIAL MECHANICS, 2nd Edition.

Willmann-Bell, Inc., Richmond, VA, 1988.

Doggett, LeRoy E. et al, ALMANAC FOR COMPUTERS 1988. Nautical Almanac

Office, United States Naval Observatory, Washington, DC, 1988.

Duffett-Smith, Peter, ASTRONOMY WITH YOUR PERSONAL COMPUTER. Cambridge

University Press, New York, NY, Reprinted (with corrections) 1986.

[NOTE: The disk available from Cambridge University Press,

containing the programs from this text, does NOT include the 1986

corrections (as of mid-1988). In particular, subroutine PELEMENT,

Page 141, contains errors in the DATA statements for Mercury and

Mars, lines 3725 and 3800; see text for corrections.]

Duffett-Smith, Peter, PRACTICAL ASTRONOMY WITH YOUR CALCULATOR, 2nd

Edition. Cambridge University Press, New York, NY, 1981.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 167

Espenshade, Edward B., Jr., Editor, GOODE'S WORLD ATLAS, 17th Edition.

Rand McNally & Co., Chicago, IL, 1987.

Hirshfeld, Alan and Sinnot, Roger W., Editors, SKY CATALOGUE 2000.0.

Sky Publishing Corp., Cambridge, MA, 1982.

Hobbs, Richard R., MARINE NAVIGATION 2, 2nd Edition. Naval Institude

Press, Anapolis, MD, 1987.

Lawrence, J. L., BASIC ASTRONOMY WITH A PC. Willmann-Bell, Inc.,

Richmond, VA, 1989.

[NOTE: A diskette is also available with the BASIC programs for

IBM-compatible PC's.]

Meeus, Jean, ASTRONOMICAL FORMULAE FOR CALCULATORS, 4th Edition.

Willmann-Bell, Inc., Richmond, VA, 1988.

[NOTE: The 4th Edition is identical to the 3rd Edition with the

exception of an added Chapter 43 giving formulae for the

heliocentric position of Pluto.]

Meeus, Jean, ASTRONOMICAL ALGORITHMS, Willmann-Bell, Inc., Richmond,

VA, 1991.

Menzel, Donald H. and Pasachoff, Jay M., A FIELD GUIDE TO THE STARS

AND PLANETS, 2nd Edition. Houghton Mifflin Co., Boston, MA, 1983.

Taff, Laurence G., CELESTIAL MECHANICS. John Wiley & Sons, New York,

NY, 1985.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 168

STSORBIT PLUS Revision History

------------------------------

Each released version of STSPLUS uses a four digit revision code such

as 9537. The first two digits indicate the year and the second two digits

indicate the week of the year. In some cases, an additional letter suffix

is added to distinguish changes occurring within the same week or to

identify special versions. A partial week at the beginning or end of the

year is counted as a full week. Using this method, a year will typically

have 53 weeks although it is possible to have 54 weeks in a leap year (1972

is an example). The current year-week revision code is shown on the Julian

Date display, Display Mode 7, in my program ASTROCLK.

This file records the revision history of program STSPLUS through all

of the minor twists and turns that usually accompany the evolution of such

a complex program. It illustrates the tortuous process of maintaining and

refining a program as ideas and problems are reported from every quarter.

These notes may also be helpful to users who are upgrading from one version

to another to find out what has changed.

David H. Ransom, Jr.

Version 9537 -- September 1995

------------------------------

-This is a MAINTENANCE UPDATE, adding several enhancements, correcting

several relatively minor bugs, and various cosmetic changes.

-By popular request, the current Satellite Configuration File (SCF)

filename is now saved in file STSPLUS.INI. Pressing ENTER when using either

SCF Save (F6+F2) or SCF Read (F6+F3) will now select the current SCF

filename. Use ESC to quit without reading/saving the SCF file. See text

for additional details.

-NOTE: Users upgrading from a prior version (Version 9517 or older) and

using their old .INI file must save the current satellite configuration

(F6+F2) using the appropriate filename before the correct filename will be

saved; the default SCF filename is otherwise set to STSPLUS.SCF.

-Various code and cosmetic changes have been made to the Satellite

Configuration section (F6 from the Main Menu) to improve performance and

error checking. One significant change is that STSPLUS will no longer

attempt to process the skeletal TLEs generated when a new satellite is

added to the SCF list; the satellite will be skipped with no message until

real TLEs are present.

-In response to many questions, I have added STATUTE MILES for all distance

calculations. Use F9 while the map is displayed to switch between

kilometers (km), nautical miles (nm), and statute miles (sm). Prior

versions of STSPLUS only calculated distances in kilometers and nautical

miles. NASA has contributed to the "confusion" by using STATUTE MILES in

most public statements (especially for orbital altitude) while using

NAUTICAL MILES internally.

-Added additional information to file STSPLUS.LOG for Pass Predictions (F3

from the Main Menu, Data Mode = 9) so that data may be more readily

identified at a later time. The additional information is:

Location:          Palos Verdes, CA

Prepared:          Saturday, 09 Sep 1995  17:02:42

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 169

Satellite Name:    STS-69

Catalog Number:    23667  95048A

TLE Filename:      STS69R04.TXT

Where: "Location" is the current primary location; "Prepared" is the

(computer) date and time of preparation; "Satellite Name" is the common

name of the satellite; "Catalog Number" is the NORAD Number followed by the

International Designation of the satellite; and, "TLE Filename" is the name

of the 2-line elements file used.

-File STSMIR.ICN, now included with the distribution package, provides an

icon of the Space Shuttle docked on the Mir Space Station. See file

STSMIR.NOT for instructions on using the icon during upcoming rendezvous

missions. The icon was first prepared for the STS-71 mission.

-Corrected a minor bug which required the 2-line elements file to be read

when resuming a prior mission (the command line option "/R" is included).

STSPLUS now reads the INI file for the TLEs when resuming a prior mission.

This modification also eliminates occasional TLE problems when selecting a

new Primary Satellite from the list of Secondary Satellites (F6+F4 from the

Main Menu); the LTD and XRF files are now checked when a new Primary

Satellite is selected.

-Corrected a problem with the "ZOE" (Zone of Exclusion) label when other

than TDRS-East and TDRS-West are used as the communications satellites in

secondary satellite slots 1 and 2. The location of the label was hard-coded

for the TDRS satellites. The location (longitude) of the "ZOE" label is now

calculated dynamically and will appear in the correct location for other

communications satellite configurations such as the two Russian ALTAIR

satellites #20391 (Cosmos 2054/ALTAIR-1) and #23426 (Luch/ALTAIR-2) being

used for communications with the MIR spacecraft.

-Corrected a minor bug which caused only four digits of MET days to be

displayed. Older satellites, such as Vangard 1 (NORAD #00005) have MET

greater than 9999 days. Note that the large MET display in the orthographic

display always displays MET MOD 10000 because of space limitations.

-Corrected a long standing bug which sometimes caused the "BASIC Error =

76" (path error) message and returned the user to DOS when attempting to

execute TRAKSTAR (F4 from the Main Menu). The initial default path was

missing a trailing backslash ("\") unless the path was manually entered.

Thanks to James Gundrey and others for continuing to report the error until

I finally spotted the cause!

-Modified the AOS/LOS algorithm which caused the AOS/LOS times to be

slightly processor dependent. The variation among various processors (386,

486 and Pentium) was usually on the order of plus or minus five seconds.

There may still be small differences (several seconds) between very fast

and very slow processors and systems with and without a math coprocessor

due to a simpler and slightly less accurate algorithm when the coprocessor

is absent (to minimize calculation times).

-Corrected a bug which caused the local time zone abbreviation to be blank

after changing the status of the Daylight Flag (F8+F10). In prior versions

the user had to exit STSPLUS and restart the program before the local time

zone abbreviation was correct.

-Replaced the three lines labeled "Launch Year", "Launch Number", and

"Launch Piece" with the single line "Int'l Designation" in the display for

data approval when reading new TLEs from a file (F2 from Main Menu). Also,

the 2nd Derivitave of the Mean Motion (dx/dt2) has been added.

-The use of a RAM disk has been deleted from the suggested system setup

instructions. Although a RAM disk is still an effective technique

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 170

(especially with a "vanilla" setup), use of disk read caching with

Microsoft's SMARTDRV or an equivalent disk caching program is now

recommended as a more general solution applicable to many applications.

(Disk write caching is NOT recommended; loss of power with write caching

enabled can result in inadvertent data loss.)

Version 9517 -- April 1995

--------------------------

-This is a MAJOR UPGRADE, adding several important new features and

enhancements. User feedback and requests have been a key factor in the

choice of those features to add. My thanks to everyone who has taken the

time to offer suggestions and submit bug reports. This version has been

preceeded by a number of Beta Test Versions which have been of great help

in working out the bugs. Among the new features are: visible Pass

Predictions, Launch Simulation for the Space Shuttle, and new Sun and

satellite icons.

-Several sections of the documentation have been added or materially

revised to reflect the new features and changes documented below.

-Added VISIBLE Pass Predictions so that users may now see at a glance when

potential viewing opportunities are in the offing. Visibility conditions

are tested at AOS, MAX altitude, and LOS. The feature is automatic with

line-of-sight Pass Predictions (F3 from the Main Menu, Function 9, with Sun

features are ENABLED). This has been one of the most frequently asked for

enhancements to STSPLUS! This feature is still somewhat experimental, and

comments are invited!

-Changed the displayed date format in Pass Predictions from "04/24/1995" to

"24 APR 95" to avoid confusion with various date formats and to allow two

extra columns for the "V" (visibility) flag. The new format is "dd MMM yy"

where "dd" is the current day of the month, "MMM" is the alphabetic

abbreviation for the month, and "yy" is the last two digits of the year.

-Added Launch Simulation (for Space Shuttle launches ONLY), enabled when

entering launch time and date using F5 from the Main Menu. See the section

"Launch Simulation for the Space Shuttle" for details. Thanks to Willie

Musty of Rockwell Mission Support for providing ascent data, as well as to

Ken Ernandes, Paul Traufler, and Dave Simonson for their assistance.

-Changed the Sun icon to filled -- for the second time. Hopefully, this

time will prove error-free! Thanks to Russ Nelson and Don Nicholson for

keeping after me on this enhancement.

-Corrected the coordinates for Cape Canaveral (from several miles offshore

to onshore) in files STSPLUS.CTY and STSPLUS.LOC. Added coordinates for

Launch Pads 39A and 39B at Cape Canaveral to file STSPLUS.LOC and

internally; because of their close proximity to Cape Canaveral, the two

launch pads are unlabeled and appear at MAG=2000 or greater in orthographic

projection only. Launch pad coordinate data courtesy Dave Simonson, DOD C-

Band Radar Network.

-After numerous requests for special and/or different satellite icons, I

have rewritten the code to use three default icons in two different sizes.

The three icon shapes are: Space Shuttle (used whenever the satellite name

begins with "STS"); MIR Space Station (used with NORAD Number 16609 only);

and, Hubble Space Telescope (default selection). The icon size may be

selected using "Set Display Features", F10+F3+F1. The default icon size is

15x15 pixels, approximately the same size as the satellite icons in prior

versions. The larger 21x21 pixel icon size is more easily seen when the

program is converted to NTSC or PAL television format. The command line

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 171

option "/21" may also be used when the program is executed to force the

icon size to 21x21 pixels. As usual, however, there is no free lunch; these

new icons take less code but slightly more time to draw and erase.

*** SPECIAL NOTE ***

Users who register or re-register STSPLUS Version 9517 and higher will

receive a copy of STSICON, the custom icon drawing program used to

create and modify STSPLUS icons. STSICON is ONLY available with

registration, see file README.

-As a companion feature to the new satellite icon code, STSPLUS will now

search for a file "#nnnnn.ICN" (where "nnnnn" is the satellite NORAD

Number) to use as the icon data for that specific satellite in place of the

internal icons. These "#nnnnn.ICN" files are produced by program STSICON

and are limited to a maximum size of 21x21 pixels.

-Corrected BASIC ERROR = 5 in Version 9515 when satellite icon moved

offscreen in full world rectangular projection (Auto-Redraw OFF).

-Function Key F6 for Display Features (F10+F3+F6) now controls the local

and satellite circles of visibility. Successive keypresses cycle through

OFF, LCL, SAT, and BOTH. (The local circle(s) of visibility were assigned

to Function Key F1 in prior versions.)

-By request, increased the maximum number of Tracking Stations permitted in

".TRK" files from 25 to 50. For the convenience of users making their own

Tracking Station files, the format for ".TRK" files is now included in the

main documentation and below:

"Maui, Hawaii",-156.7,20.9,396,"HAW"

------+-----  ---+-- --+- -+-  -+-

|          |     |   |    |

|          |     |   |    +---  Three-letter Abbreviation

|          |     |   |

|          |     |   +--------  Elevation in meters

|          |     |

|          |     +------------  Latitude in degrees

|          |

|          +------------------  Longitude in degrees

|

+-----------------------------  Station Name

Note: Station Names are limited to 20 characters and Station Abbreviations

are limited to 3 characters.

-Users may now select as the user location any of the tracking stations in

the tracking station file (default file is STSPLUS.TRK) using F10+F2+F6

from the Main Menu. This feature has been requested by several users.

-Changed the error message when reading a TLE file to correctly indicate

the cause of an error. When TLE data is formatted incorrectly (as with TLE

data from a UNIX system with each line terminated with LF instead of

CR+LF as required by DOS), STSPLUS incorrectly reported "File <filename>

not found!". The error message now reports "Data problems with file

<filename>, check data format!". To correct UNIX formatted TLE files, use

an ASCII editor such as the shareware program QEDIT; QEDIT will reformat

each line with the CR+LF termination. I also have posted program TOMS.ZIP

on my RPV Astronomy BBS which reformats UNIX text files for DOS. TLE data

downloaded from NASA Spacelink may exhibit this data format problem.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 172

-Repaired a bug which sometimes caused STSPLUS to fail with BASIC ERROR = 5

when using the "/R" command line option. Whether or not the error occurred

appears to have been data dependent and has now been resolved. (A call to

the DOS clock was not being made with the "/R" command line option and this

sometimes resulted in an illegal function call.) Thanks to Rudy Appel for

reporting the problem!

-Repaired a bug which sometimes caused STSPLUS not to find the correct

satellite in the TLE file when the "/R" command line option was used. The

problem occurred only when the satellite was included in file STSPLUS.XRF

and the satellite name was modified. Note that the default satellite when

using F2 is now the satellite NORAD Number ("#16609" for MIR, for example)

rather than an abbreviated satellite name. This change will also reduce

problems and errors when multiple satellites have similar names like "MIR"

and "MIR DEB" (MIR debris).

-Corrected a bug in the ZOOM feature. The PgUp and PgDn keys now operate

correctly for zoom functions in rectangular projections. My apologies for

not spotting this bug sooner ... I seldom use the rectangular projections

in conjunction with Doppler data. The bug appeared when Doppler data was

added in early 1994 (Version 9406) and only then after Doppler data was

first enabled using F8 while the map was displayed in orthographic modes.

-Corrected a bug that caused Doppler data to appear on rectangular maps

if PgDn or PgUp keys were pressed when Doppler data was enabled.

-I removed the detection for Pentium chips that was present in some Beta

Test Versions. Due to a flaw in the detection algorithm, some 486DX2 and

486DX4 chips were incorrectly detected as a Pentium. STSPLUS now simply

indicates "486DX+" for 486 and higher chips.

-Modified the TARGET logic to show relative velocity betwen the Primary and

Target satellites when the relative range is less than 10,000 km. Users

should note that these are "line of sight" calculations and may be

misleading if the satellites are at large relative range. (Prior versions

limited display to low velocities and close approaches.)

-Changed date input subroutine (used with F8 from the Main Menu) to use the

current year as the default when a partial date is entered. For example,

the date may now be entered as "1/15" or "15.01" and the current year will

be used. Prior versions were hard coded to a specific year. Thanks to Russ

Nelson for the suggestion.

-Changed launch date/time input code (F5 from Main Menu) to automatically

enable MET when data is accepted.

-Corrected a problem with Pass Predictions (Altitude and Azimuth wrong!)

which crept into the Beta Test Versions. A position vector was not being

initialized. Thanks to George Sells for spotting and reporting the problem!

-Corrected a data misalignment with AOS/LOS data in rectangular mode when

"/CLK" command line option used and TDRS satellites set OFF. Thanks to Don

Nicholson for reporting the problem (several times).

-Updated current BBS information at the end of the documentation.

-By popular request, added the Bibliography of Astronomy and Space

Subjects, extracted from the documentation for my program ASTROCLK.

Versions 9509-9515 -- March 1995

--------------------------------

-These releases were Beta Test Versions, not released publicly. The notes

for the various changes made in these releases (those that survived!) have

been incorporated in the notes for Version 9517 above without regard for

which Beta Test Version they first appeared in.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 173

-My thanks to all for the help, suggestions, and bug reports from those who

exercised these versions!

Version 9445 -- November 1994

----------------------------

-Corrected a long standing problem with orbit numbering, primarily during

space shuttle missions. STSPLUS now accepts the orbit number given in the

TLEs and allows only a 60 second time offset prior to the next ascending

node. In other words, if Epoch Time is within 60 seconds of the next

ascending node, STSPLUS assumes the orbit number in the TLEs is for that

ascending node; otherwise, STSPLUS asumes the TLE orbit number is for the

current orbit. In prior releases, STSPLUS used the NEAREST ascending node

in its orbit number calculations rather than the CURRENT ascending node and

this often required adjustment to the orbit number in TLEs. Orbit numbers

will still be correctly calculated when the Epoch Time is just after the

ascending node; this change only applies to TLEs in the second half of an

orbit. Thanks to Ken Ernandes for repeatedly "reminding" me to fix this

annoying problem!

-STSPLUS may now operate with the DOS clock set to either LOCAL TIME or UTC

TIME. Use F8+F10 from the Main Menu to set the DOS time zone (along with

the UTC Offset and Daylight Flag). The default assumes the computer is set

to LOCAL time.

-In response to several requests, added the capability to show AOS/LOS in

four time units by pressing F12:

Countdown clock in minutes:seconds (as in prior versions)

UTC  UTC time in hours:minutes

LCL  Local time in hours:minutes (proper abbreviations will be used)

MET  Mission Elapsed Time in hours:minutes

All "hours:minutes" times are TRUNCATED rather than rounded to the minute,

and days are omitted; thus, an MET of 4/01:02:00 and 4/01:02:59 will both

display as "01:02". Time Since Epoch ("T+E") will display in place of MET

if the MET flag is OFF. If no launch time has been set, MET AOS and LOS

times will NOT be available. NOTE: Older computers without an F12 key

cannot use this feature.

-In the orthographic mode, the appropriate time units label is shown for

UTC, LCL and MET to the left of "AOS   LOS" when only one local station is

enabled; no label is shown for normal countdown clocks OR if two local

stations are enabled. LCL is replaced by the appropriate abbreviation if

available for the time zone.

-In rectangular modes, space for time units labels is only available when

the "/CLK" command line option is used AND MET is selected using F2. When

displayed, the labels are UTC, LCL, and MET as above.

-Added internal time zone abbreviations for Hawaii (UTC-10, HST/HDT), Yukon

(UTC-9, YST/YDT), and Atlantic (UTC-4, AST/ADT) zones.

-When entering Launch Time and Date (F5 from the Main Menu), dates are now

displayed as "dd MMM yyyy" where "MMM" is the alphabetic abbreviation for

the month. However, all dates must still be entered using the NUMERIC

formats "mm/dd/yy" or "mm/dd/yyyy" (US style), or "dd.mm.yyyy" (European

style).

-Corrected a minor bug which caused the longitude labels not to display for

EGA and Motion Map modes in some circumstances.

-Removed the "happy face" progress indicator in the Motion Map if the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 174

system reports a 386/387 or higher processor. This undocumented feature was

added for users with slow computers and computers without a math

coprocessor so there would be some "action" as those clunkers plodded

through the off-screen map drawing process.

-Various cosmetic repairs.

Version 9435A -- August 1994

----------------------------

-Version 9435A corrects a problem with certain country codes (such as those

which use the "YY-MM-DD" date format) which still caused "BASIC Error = 5"

in the orthographic mode for Version 9435. The program should now be

independent of the country code.

-Corrected a minor bug with amateur radio satellite frequencies. If the

primary satellite was changed with F6, the frequency file (STSPLUS.FRQ) was

not rescanned and the frequencies from the prior satellite remained.

STSPLUS will now always rescan file STSPLUS.FRQ if the primary satellite is

changed.

Version 9435 -- August 1994

---------------------------

-This release repairs an unfortunate bug which surfaced when I added the

NORAD-style date on the orthographic display. Only users who set DOS to

non-US-style date formats were affected. The problem occurred immediately

when drawing the orthographic map as "BASIC Error = 5" and returned the

user to DOS.

-Added satellite name cross-reference in file STSPLUS.XRF. When TLEs are

read and accepted, STSPLUS now checks for the cross-reference file and, if

the file is present in the current directory, checks for the NORAD Number

of the satellite and a cross-reference name. If found, the new name is

substituted for that found in the TLE file. The file is standard ASCII and

each entry consists of a SPACE, the five-digit NORAD Number, a SPACE, and

then the satellite name. A sample file:

22920 HST Solar Array

22076 Topex/Poseidon

21225 Gamma Ray Observ

20638 Rosat Observatory

20580 Hubble Telescope

16609 MIR Space Station

NOTE: The leading space is required for file compatibility with my program

ORBITEL (but is optional for STSPLUS). The NORAD Number MUST be 5 digits;

pad with leading zeroes if necessary. Only one entry is read per satellite.

If the satellite name is longer than 19 characters, only the left 19

characters will be used. This feature may be used to substitute any name

for that given in a TLE file (which is not always consistent from source to

source) or to specify a payload piggy-backed on another satellite (as is

frequently the case with amateur radio transponders). A number of XRF files

are posted on my RPV Astronomy BBS; such files must be renamed for use with

STSPLUS.

-Repaired a minor bug which caused the pass predictions (F3 from Main Menu

to repeat a pass indefinitely when that pass was a grazing pass with a

maximum elevation of less than one degree (0.14 degrees in the test case

reported by Ken Ernandes).

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 175

-Corrected a bug in read/save .SCF files (F6 from the Main Menu). If a

filetype is entered (".SCF" or whatever), it is now ignored and ".SCF" is

automatically appended to the filename. Thanks to Paul Becker for reporting

the problem.

-In response to many requests, the date input routine has been modified to

permit entry in the European style "dd.mm.yyyy" in addition to the US style

"mm/dd/yyyy". The choice of delimiter ("." or "/") determines the method of

interpretation. In both cases, the year may be entered as the last two

digits only, "yy", if desired; "50" through "99" will be interpreted as

"1950" through "1999" and all others as 2000+yy. Dates are displayed as "dd

MMM yyyy" where "MMM" is the English abbreviation for the month.

-All current .SCF files are now displayed for both SAVE and READ functions

(F6+F2 and F6+F3 from the Main Menu).

-Improved backspace processing and error checking when entering TLE

filenames (F2 from the Main Menu).

-Corrected "West Latitude" to "West Longitude" in the data entry section

for local coordinates (F10+F2).

-Various minor cosmetic changes.

-Version 9434 was released as a Beta Version to selected users and included

all changes in Version 9435 EXCEPT the repair to the NORAD-style date bug.

Version 9432A -- August 1994

----------------------------

-As too often happens, a bug was discovered in Version 9432 just after it

was released. The elevation ("Elv") and azimuth ("Azm") in the rectangular

projections either remained at zeroes or at the last value calculated

during orthographic projection. There was no workaround. The problem has

been repaired. My apologies to those who received the defective version.

Version 9432 -- July 1994

-------------------------

-Version 9432 adds several new features and corrects a number of program

errors and/or "bugs". Several minor cosmetic changes were also made.

-Added Data Output Mode 8 to generate tabular output for Doppler Shift

calculations for a selected pass while the tracking map is displayed. Use

F3 from the Main Menu and select Mode 8; setup features are identical to

other modes (output device, interval, etc.). See text for details.

-Look angle calculations [elevation ("Elv") and azimuth ("Azm")] during the

tracking map display are more accurate thanks to an improved algorithm

supplied by Ken Ernandes. Users should note that near culmination (maximum

elevation) for passes near the user's zenith (approaching 90 degrees

elevation) there is some uncertainty and ambituity as to azimuth although

the angular error remains very small.

-Changed "Alt" to "Elv" as the label for satellite elevation (altitude)

above mean sea level to conform to standard satellite tracking convention.

-Because STSPLUS is used worldwide, I have reformatted all displayed dates

throughout the program to avoid confusion of the day versus the month. For

orthographic displays, the display now shows the current UTC date and time

("dd MMM hh:mm:ss UTC") and then the local time ("dd mmm hh:mm:ss PDT"

where "PDT" is replaced by the local time abbreviation). For both dates,

the year is implied. The MET/T+E is moved up one line. A typical time

display will show:

15 JUL 19:57:36 UTC

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 176

15 JUL 12:57:36 PDT

7/03:14:36 MET

For the Main Menu and rectangular projections, the dates are all given as

"dd MMM yyyy". Data locations are unchanged. A typical date and time will

now be displayed as:

15 JUL 1994  19:57:36 UTC

For all cases, "MMM" is the three letter English abbreviation for the

month, "JAN" for January, "FEB" for February, etc.

IMPORTANT NOTE: Keyboard entry of dates is still in the American

numeric format, "mm/dd/yyyy" or "mm/dd/yy".

-For orthographic projections, the current time using NORAD convention

("yyddd.dddddd") is displayed immediately beneath the current TLE filename:

94196.831667

This provides an immediate method to identify the current year and day-of-

the-year, "94" and "196" respectively in the example above.

-Added an additional data line in orthographic display to show the current

orbital period ("Per'd" using the format "hh:mm:ss") for normal display OR,

for satellites with Eccentricity >= 0.005, satellite Phase, defined as Mean

Anomaly normalized to the range 0-256 ("Phase" using the format "nnn.n")

for use with the Doppler Shift mode [press F8 while the map is displayed]

and satellites which change transponder mode based upon phase. Although

Phase is technically defined as an integer value from 0 through 255,

STSPLUS displays it to one decimal place to alert users to upcoming

phase/transponder changes. Other users may also be interested in two

important Phase or Mean Anomaly angles: Phase=0.0 (Mean Anomaly is 0

degrees) corresponds to perigee and Phase=128.0 (Mean Anomaly is 180

degrees) corresponds to apogee.

NOTE: Mean Anomaly (and therefore Phase) becomes undefined for a

perfectly circular orbit. In practice, satellites with eccentricity

less than 0.005 will display some ambiguity as to Mean Anomaly and

Phase. Note also that the "Elv" is the geodetic elevation (altitude)

of the satellite above Mean Sea Level and NOT the geocentric radius of

the satellite's orbit, the paramter used to determine apogee and

perigee; the difference due to the shape of the Earth is as much as

21 km. See the section "Satellite Communications and Amateur Radio"

for additional discussion!

-Repaired the Tracking Station color feature (F10+F9) so that it operates

correctly. Versions 9415/9414 caused the entire screen to turn light red

when the Tracking Station color was changed due to a careless coding error.

-Rewrote the local coordinates code and text (F10+F2) to improve clarity

and add several features. Now uses a function key menu to select action to

perform. Press ENTER to return to prior menu. When entering new

coordinates, an illegal latitude or longitude will cause that coordinate to

be set to zero. Longitude may be entered as East Longitude (0 to +360) or

West Longitude (0 to -180); the longitude is converted if necessary and

stored in the range from -180 to 0 to +180.

-The threshold for generating the "Satellite may have decayed!" message has

been increased to a Mean Motion of 16.4 revs/day to reduce the probability

of the message appearing during low shuttle flights.

-Added code to check for synchronization and data errors in the .INI file

between the "short form" satellite position data and the 2-line elements.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 177

If the NORAD numbers do not agree, the "short form" data will be set to the

TLE NORAD number and the remaining data reset; if TLEs are not present,

dummy TLEs are inserted. Use F6 to verify and/or correct the secondary

satellites, then use F2+& to update data from TLE file(s).

-Adjusted the "trigger" for automatic redraw in orthographic modes with

higher zoom factors to compensate for high speed processors such as the

90MHz Pentium. (Under some circumstances, redraw was being triggered

continuously.)

-Repaired a minor bug that caused the TDRS and Sun AOS/LOS data to be

missing on the rectangular display when Doppler Shift mode had been

selected on the orthographic display.

-Corrected the conversion factor used to convert meters to feet. Thanks to

Steve Lenz for reporting the bug!

-Removed all references to the RPV HOTLINE BBS since that telephone line is

now in rotary with the main number, (310) 541-7299.

-Versions 9424, 9429, and 9431 were beta test versions not released

publicly and included some of the features and changes in this release. For

Version 9424, the precision of Doppler shift frequencies was increased to 5

digits (10 Hz) to the right of the decimal point but this proved confusing

and has been restored to 4 digits (100 Hz).

Version 9415 -- April 1994

--------------------------

-This release repairs a problem in rectangular projections which caused the

Alt/Az calculations to be incorrect. (The data in orthographic projections

were alright.) The problem was traced to a duplicated variable name which

was added to shared common. Only file STSPLUS.EXE is changed.

Version 9414 -- March 1994

--------------------------

-This is a MAJOR UPGRADE, adding new features for satellite communications

and amateur radio, user-definable map colors for certain map features,

improving RA/DEC coordinates, and incorporating several bug fixes.

-See also the notes below for Version 9406 (not released publicly).

ENHANCEMENTS FOR SATELLITE COMMUNICATIONS AND AMATEUR RADIO:

------------------------------------------------------------

-By popular request and with the assistance of Ken Ernandes, N2WWD, I have

added Doppler shift calculations for uplink and downlink frequencies. The

calculated uplink (XMIT) and downlink (RECV) frequencies have been tested

in full duplex with RS-10 and yield excellent results. The Doppler shift

calculations are available in orthographic projections ONLY for this

release. See the text for complete discussion.

-The satellite NORAD Number, UPLINK and DOWNLINK frequencies (referenced to

the satellite), and the transponder mode are specified in file STSPLUS.FRQ

in that order, separated by commas and without any leading or trailing

spaces. A sample file might include:

00000,100,100,1           (Default values if sat # not found)

18129,145.8900,29.3900,1  (Parameters for NORAD #18129)

--+-- ----+--- ---+--- +

|       |       |    |

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 178

|       |       |    +-- Transponder Mode:  1 = NORMAL

|       |       |                          -1 = INVERTED

|       |       |

|       |       +------- DownLink Center Frequency (MHz)

|       |

|       +--------------- UpLink Center Frequency (MHz)

|

+----------------------- Satellite NORAD Number

The first sample line shows the "00000" entry which determines the default

parameters if the satellite is NOT included in file STSPLUS.FRQ. This

should be the FIRST LINE in file STSPLUS.FRQ. The second line gives real

parameters for a specific satellite; the frequencies shown select the Mode

A voice passband for Radio Sputnik 10 (RS-10, piggybacked on COSMOS 1861,

NORAD #18129). The uplink and downlink frequencies should not exceed

99000.0000 MHz to avoid an overflow condition on the display.

-File STSPLUS.FRQ may be created or edited with any ASCII editor; word

processor users, use the "non-document" mode. Note that only minimum error

checking is performed and the user must observe the required format exactly

for each line in the file.

-The Doppler shift calculations replace the TDRS and Sun AOS/LOS data in

the data block to the right of the orthographic map. To enable display of

these frequencies, press F8 while the map is displayed; to return to the

AOS/LOS calculations, press F8 again while the map is displayed. F8 is NOT

active when in PAUSE mode. The following example illustrates the display as

a satellite approaches the ground station (using 1000 MHZ for both

frequencies to show the relative transmit and receive ratios):

UpLink:  1000.0000    Uplink frequency received by satellite

XMIT:   999.9761    TRANSMIT frequency at ground station

DnLink:  1000.0000    Downlink frequency xmitted by satellite

RECV:  1000.0239    RECEIVE frequency at ground station

The XMIT and RECV frequencies will be shown in color on EGA/VGA color

monitors:

RED     Satellite is below receiver's horizon

YELLOW  Satellite is 5 degrees or less above receiver's horizon

GREEN   Satellite is 5 degrees or more above receiver's horizon

Transmissions will not normally be possible when RED is shown.

Transmissions MAY be possible when YELLOW is shown. Transmissions should be

practical when GREEN is shown provided the ground station has a clear

horizon in the direction of the satellite.

-STSPLUS includes a "fine tuning" feature for the uplink and downlink

frequencies. While in the Doppler shift calculation mode, the following

keys have a different function from the normal map modes:

UP Arrow        Increase RECV frequency by 100 Hz

DOWN Arrow      Decrease RECV frequency by 100 Hz

PgUp            Increase RECV frequency by 1 KHz

PgDn            Decrease RECV frequency by 1 KHz

Home            Restore Uplink and DnLink frequencies to those

read in from file STSPLUS.FRQ

End             (not used)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 179

If the satellite transponder is NORMAL, the XMIT frequency will be

increased or decreased by the same amount as the RECV frequency. If the

satellite transponder is INVERTED, the amount of change in the XMIT

frequency will be the same magnitude but in the opposite direction as the

change to the RECV frequency.

OTHER SOFTWARE ENHANCEMENTS AND CHANGES IN THIS RELEASE:

--------------------------------------------------------

-In response to numerous user requests, the colors for certain map features

are now user-definable. The assignable features are:

Local Station circle of visibility

Isocontour circles in Location and Tracking Station modes

Tracking Station circles of visibility

From the Main Menu, use F10+F9 to set these colors. The new colors will be

saved in file STSPLUS.INI for future use. To those users who want to

change EVERYTHING, my response is: a) that's a non-trivial programming

exercise, and b) I've spent considerable time designing the program to have

a certain "look and feel" which I wish to retain.

- The Program Features and Options menu has been changed. Function Key F9

is now used for User-Definable Colors (above) and not for setting the UTC

Offset and Daylight Flag. Use F8+F10 from the Main Menu to set the UTC

Offset and Daylight Flag.

-Users are reminded that STSPLUS expects ground station coordinates

(latitude and longitude) in the geodetic coordinate system, as commonly

used on maps (WGS-72 System). Ground station altitude (elevation above Mean

Sea Level) is expected in METERS; if a ground station is significantly

above Mean Sea Level, accuracy will be substantially improved if an

accurate altitude is used. Many cities in file STSPLUS.CTY have ZERO given

as the altitude if no altitude was available in the source(s) used for

preparation of the file. The same comments apply to Tracking Stations in

file STSPLUS.TRK.

-The coordinates for Right Ascension and Declination were incorrect in

prior versions. The ground station's GEODETIC latitude instead of the

GEOCENTRIC latitude was used in the calculations. The error was greatest

(especially the Declination) for ground stations in mid-latitudes as a

satellite approached local zenith. Thanks to Alan Nutley of Australia for

putting me on the track of this one!

-Local horizon coordinates were also affected by the latitude error. The

typical error near maximum was one or two degrees in altitude (elevation).

-The keyboard response time has been improved; except when the map is

actually being drawn, response is immediate instead of waiting for the next

second. In the Doppler Shift Mode, the arrow keys and PgUp and PgDn may be

held down to repeat. During rapid key repeats, map and data updates may be

deferred; waiting for a second will allow the map and data to be updated.

-The primary satellite's circle of visibility did not display on the World

Map when the Motion Map (Dual-Page EGA Mode) was enabled. This has been

corrected. Thanks to Todd Sherman for reporting the bug.

-Corrected a problem which caused BASIC ERROR 6 on restart when the SHELL

TO DOS (F9 from the Main Menu) was used and the program was in orthographic

projection.

-Various minor bug fixes and cosmetic changes.

-Versions 9412 and 9413 were BETA VERSIONS released on a limited basis.

-Special thanks to Ken Ernandes, N2WWD, for his assistance and testing of

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 180

the satellite communications and amateur radio features!

Version 9406 -- February 1994

-----------------------------

-This version was for Beta Test only and was not released publicly.

-Several users have reported that file STSPLUS.INI sometimes became corrupt

and I have (finally) found and corrected the problem. An array index was

overrunning the bounds of the array and overwriting other data in SHARED

COMMON. This usually only affected the data in secondary satellites but was

potentially more dangerous. The problem also caused some dot colors on the

ground track to be incorrect.

-When no STSPLUS.INI file is present (or when the UTCOffset is set to -99),

the user is automatically asked to set filenames and paths.

-In response to quite a number of user requests, I have added the "/SS"

command line option to force STSPLUS into a "screen saver" mode. In this

mode the program displays the full orthographic globe, ground track and all

selected map features but NO DATA at the right. Use ENTER or ESC to return

to DOS.

-New command line options have been added to control certain display

features (especially from batch files). The new feature status is saved in

file STSPLUS.INI.

+L   Include Location and Feature Labels

-L   Exculde Location and Feature Labels

+R   Include Rivers and Lakes

-R   Exclude Rivers and Lakes

+T   Include Tracking Stations

-T   Exculde Tracking Stations

+V   Include Local Circle of Visibility

-V   Exclude Local Circle of Visibility

-Because of problems reported with word processors which add the 8th bit to

some characters (and have been used to edit TLE files), I have added code

to strip off the 8th bit in Line 0 of TLEs. However, this is not foolproof,

and users are cautioned to use ONLY editors which do NOT add the 8th bit

and which maintain the "standard" CR/LF at the end of each text line.

Version 9405 -- January 1994

----------------------------

-Version 9405 is a MAINTENANCE UPDATE, correcting a number of relatively

minor bugs and updating the documentation to reflect changes in Versions

9403 and 9405.

-Rewrote MET calculations to (hopefully) avoid truncation and rounding

errors which sometimes caused MET to be one second off. The problem was

dependent upon both launch and epoch times.

-Added default filenames and paths if UTCFlag is set to -99. (Setting

UTCFlag to -99 may be used to distribute STSPLUS.INI files when the

ultimate user's time zone is unknown. This procedure forces the user to set

the UTC and DAYLIGHT values, and is NOT recommended for the novice!)

Filenames and paths should ALWAYS be checked and set if necessary using F7

from the Main Menu whenever upgrading to a new version.

-Improved calculation algorithm for "Calculating Orbital Data" phase of

program initialization for satellites with mean motion less than 15 and

greater than 2. The improvement may only be apparent on slower processors

or systems without a math coprocessor.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 181

-Corrected a cosmetic bug which caused the time portion of negative MET to

appear at the left on the next line in rectangular projection modes.

(Missing semicolon!)

Version 9403 -- January 1994

----------------------------

-In response to numerous complaints about BASIC ERROR 6 ("Overflow") when

using Relative Target Tracking with rectangular map projection, I have

corrected a condition while calculating relative velocity which could

generate the overflow error. The error may be related to processor speed

and/or specific 2-line elements and has been difficult to reproduce.

-Corrected several differences between rectangular and orthographic maps so

that rectangular maps are processed as close as practical to the way that

orthographic maps are processed. This eliminated some spurious data that

was displayed momentarily as the rectangular map was first drawn and may

also help avoid the BASIC ERROR 6 problem (above).

-Corrected a cosmetic bug which caused the "s" in "ft/s" and "m/s" to

remain on the screen when switching coordinates from target data to other

coordinates using F10 while the map is displayed.

-Several minor cosmetic changes.

Version 9402 -- January 1994

----------------------------

-Corrected (I hope) a bug which caused BASIC ERROR = 6 when switching from

MET to T+E while using a Target Satellite. The problem was traced to a

variable which was not re-initialized when F5 was pressed while the map was

displayed.

Version 9353 -- December 1993

-----------------------------

-Added Relative Target Tracking (not included in Beta Test Versions 9351

and 9352) to report relative range [km or nm] and velocity [m/s or ft/s]

between the Primary Satellite and a Target Satellite selected from among

the Secondary Satellites. Relative Velocity data is presented ONLY if the

Relative Range is less than 5000 km. The Target Satellite is selected with

F6+F5 and the relative tracking data is enabled by F3+F6 from the Main Menu

or F9 while the map is displayed. See text for details.

-The Secondary Satellite code has been modified to save a skeletal TLE when

a new satellite is added. This permits the user to use F2+'&' to update

TLEs INCLUDING the new satellite(s). The warning message for satellites

without TLEs in F6 has been modified to remind users that they may either

update the TLEs or display the satellite ground track.

-I seem to have tracked down an internal timing anomaly that caused errors

in the tracking data of about +/-0.5 seconds. Worst case, this caused an

error in calculated position on the order of tens of meters (about the

length of the space shuttle!) and would not be easily detectable under most

circumstances. Thanks to Ken Ernandes for first reporting the problem

several months ago.

-Corrected a problem which caused the STSPLUS.INI file to become corrupt if

Event Timers were set to OFF. Thanks to Chuck Dean for reporting the bug!

-Corrected a problem with secondary satellites and .SCF files when one or

more secondary satellites was deleted, leaving blank satellite slots.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 182

Secondary satellites may now occupy any slot with or without intervening

blank slots.

-The Primary Satellite is now labeled "Primary" and shown in YELLOW if it

is included in the satellite configuration data (F6+F1).

-When displaying the satellite configuration data (F6+F1), the 2-line

elements are checked for epoch. If the epoch is more than 10 days (Real

Time) or 60 days (Static) prior to the current/simulated time, a flashing

RED asterisk ("*") is shown next to the satellite number and a warning is

shown at the botton of the screen. NOTE: This is a caution warning only;

the elements may or may not still be valid.

-Added an "OFF" mode for Secondary Satellites (F6+F1). The OFF mode permits

one or more satellites to be disabled (not displayed) without removing the

data from the secondary satellite data. All data are retained and the

satellite may be re-enabled at any time. Thanks to Paul Grupp for the

suggestion!

-Added grid labels in the orthographic displays (when Additional Grid Lines

are enabled with F10+F3+F3) for latitude and longitude when the MAG factor

is 500 or greater. Latitude labels are always at the left side of the map;

longitude labels are at the top or the bottom depending upon the map

center. The labels are disabled for polar and near polar maps because of

the projection.

-Changed the plot update interval for secondary satellites to EVERY SECOND

if a 386 or higher processor AND a math coprocessor are present. This

avoids a "leapfrogging" effect between the primary and secondary satellites

during close proximity operations. For other processor/coprocessor

combinations, the plot update interval remains TEN SECONDS as in prior

versions. In all cases, the ground track for secondary satellites is only

plotted every ten seconds in order to avoid cluttering the display.

-Changed the orbit number calculation after considerable debate. The code

used in all prior versions caused the TLE orbit number to be incremented if

the epoch of the TLE was in the latter half of the orbit (at or past the

descending node). The new method increments the TLE orbit number only if

the epoch is within 0.1 orbit of the ascending node. CAUTION: This change

may cause orbit numbers to be different if prior missions are replayed

using TLEs which were adjusted to compensate for STSPLUS' prior orbit

numbering conventions.

-Removed code which translated satellite names. Names beginning with "HST"

were changed to "Hubble Space Telescope"; with the jettisoning of the "HST

Solar Array", that debris was being renamed also. A similar problem occured

with MIR debris. If you wish to rename one or more satellites, use my

program ORBITEL (available free on my BBS or for a donation of US$10.00 by

mail).

-Corrected a cosmetic bug which caused stray pixels to appear when a

secondary satellite intersected the circle of visibility for the primary

satellite. Thanks to Joel Runes for spotting the problem.

-Added the International Date Line in orthographic displays as a dotted

bright blue line where it is different from the 180th meridian.

-Corrected a cosmetic bug for map drawing time in orthographic mode. If the

operation spanned midnight, the drawing time overflowed the field with a

number like "%-86300.12", wrapping around to the left side of the screen.

-Removed the function to disable solar features (Sun and Terminator) from

the map using F8 while the map is displayed. The function is now available

ONLY from the Main Menu using F10+F3+F8.

-Changed the "Additional Grid Lines" function, F10+F3+F3, to ON or OFF.

-USSPACECOM has changed the format of the 3-character "Launch Piece"

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 183

portion of the International Designator in their 2-line elements (TLE) from

right justified to left justified. This caused STSPLUS to show "(n/a)" for

the launch piece. The data displayed when the elements are presented for

user approval have been changed from numeric to the alpha designation given

in the TLE. In other words, launch piece "B" will be so shown instead of

"2" as in prior versions.

-All references to "IAU Designation" have been changed to "International

Designation" or "Int'l Designation".

-Changed the limits on UTC Offset to allow +13 hours and -13 hours to

conform to civil practice for certain areas in the South Pacific.

-Removed the "FOV" (field of view) data from the orthographic data block.

The calculation was incorrect and misleading.

-Various cosmetic changes and improvements.

Versions 9351 and 9352 were BETA TEST VERSIONS with limited distribution.

A number of minor bugs were reported and have been corrected for this

release:

-Selecting a new Primary Satellite before a map was displayed sometimes

failed.

-Selecting a new Primary Satellite did not set MET correctly. The MET was

calculated for the prior Primary Satellite. Thanks to David Cottle!

-Secondary satellites set to OFF were drawn (but not updated) on the

rectangular projection.

-Corrected a cosmetic bug which caused the thousands digit of MET DAYS not

to be erased when switching large clock times (Mag=100, VGA only) with F2.

Similarly, the colon (":") was omitted when switching from MET to/from T+E

using F2.

Version 9338 -- September 1993

------------------------------

-Repaired a bug in multi-satellite modes which caused a "trail of dots" to

be left behind if the primary satellite icon fell on top of a secondary

satellite icon (such as during close proximity operations).

-Repaired a bug when reading SCF files. In prior versions, the primary

satellite was changed if the SCF file had a different primary satellite.

The primary satellite in the SCF file is now ignored. The format of the SCF

file is unchanged to maintain compatibility with prior versions.

-Modified the World and Zoom Maps in rectangular projection so that the

PgDn key switches to Zoom Map (180 degrees) from World Map, and the PgUp

key switches to World Map from Zoom Map (180 degrees).

-Added list of available .SCF files when reading, F6+F3 from the Main Menu.

-Added a warning when writing .SCF files if the requested file already

exists. F6+F2 from the Main Menu. User must now press "Y" to overwrite an

existing file, ENTER or any other key to cancel.

-By user request, modified the small character set (used for labels) to

include the DASH ("-").

-After numerous user complaints, I've finally repaired the BLINK mode so

that for most systems the primary satellite icon will blink on and off once

per second. Press "B" while the map is displayed to toggle the BLINK mode.

-Repaired a minor bug which caused the message "Pause ... press ENTER" not

to be displayed if the map was redrawn during Pause Mode.

-Thanks to the many users who took the time to carefully document these

bugs, enabling me to find and fix them!

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 184

Version 9334 -- August 1993

---------------------------

-The maximum number of Static and Real Time satellites is increased to 32.

This permits the entire GPS constellation, currently 25 active Block I and

Block II satellites, to be tracked. The satellite setup display, F6+F1 from

the Main Menu, has been modified to two display pages of 16 satellites

each; the second page of 16 is not displayed if no active satellites are

present.

-Added "Clear Static and Real Time Satellites", F6+F4 from the Main Menu,

to speed reconfiguration of these satellites. It may be used to clear any

block of satellites NOT INCLUDING #1 AND #2 (which are assigned to TDRS

East and TDRS West and must be cleared manually). Users are cautioned that

once cleared, the satellite data for the cleared satellites is lost. If in

doubt, save the current satellite configuration to a SCF file using F2

BEFORE clearing!

-Somehow the "bullseye" was lost in the Tracking Mode for rectangular

projection several releases back. It has now returned. Thanks to Alan Pound

for reporting the problem!

-As an experiment, I changed the color of land boundaries from Light Cyan

to Cyan to make multi-satellites and other display features a little easier

to spot. Feedback welcome!

-When Extra Grid Lines are OFF (F10+F3+F3), changed the color of the grid

lines from Light Blue to Blue and added lines of latitude at +80 and -80

degrees.

-Various repairs to eliminate spurious pixels and other minor problems

associated with multi-satellites.

Version 9333 -- August 1993

---------------------------

-This is a MAJOR UPGRADE, adding multi-satellite capability. This is the

first in a series of upgrades for tracking multiple satellites and

rendezvous missions. Comments, suggestions, and bug reports are welcome!

-Special thanks to Joel Runes for his assistance during beta testing!

-OPERATIONAL HINT: With the addition of multi-satellite tracking, the

Motion Map feature now really comes into its own; press "M" when the normal

map is displayed to switch to the EGA Motion Map, press "M" or ENTER to

return to normal map or to the Main Menu. All satellites are updated as

fast as the computer can draw the map. See the notes on a "vanilla" boot

without memory managers for dedicated STSPLUS uses.

-The format of file STSPLUS.INI has been substantially modified and files

from prior versions of STSPLUS will be ignored. The configuration

information must therefore be re-entered.

-Increased to sixteen the number of additional TDRS and real time satellite

positions in file STSPLUS.INI so that all active satellites may be updated

and displayed. Sample data are shown below. See the section "TDRS and Real

Time Satellite Features" for additional information.

19883,"TDRE ", -0.04218, -41.14169,10014

21639,"TDRW ", -0.03643,-174.14074,10014

22314,"TDR5 ", -0.31155,-138.36021,10004

19548,"TDR2 ",  0.05352, -61.66467,10004

13969,"TDR1 ", -6.59117,-170.55876,10004

16609,"MIR  ",-17.45706,-178.73251,14115

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 185

-TLEs for each active satellite are now saved in the .INI file so that

accurate positions may be calculated dynamically. These TLEs should be

updated periodically.

-TLEs are may now also be saved in and read from special .SCF (Satellite

Configuration File) files so that multiple TDRS and Real Time satellite

configurations may be saved and/or selected. A sample file, STSPLUS.SCF is

included in the standard distribution. Function Key F6 from the Main Menu

is used to display, modify, save, or read these data.

-The 5-character abbreviation of each TDRS satellite may now be used to

label the satellite. The size and color of the icon used to display each

TDRS or real time satellite may now be independently controlled.

-STSPLUS now uses the calculated LATITUDE for the display of all TDRS

satellites. This will cause TDRS1, for example, to appear up to 7 degrees

North or South of the Equator (as of 8/9/93).

-The drive, path, and name of the .CTY file may now be set with F7 from the

Main Menu.

-The Motion Map is now available for the World Map in rectangular

projection. This is helpful with multiple real time satellites.

-Repaired an infrequent bug discovered by Joel Runes which causes the

orthographic map to be redrawn continuously when a) the satellite has a

high eccentricity (.73 in the test case), and b) when the map is being

drawn at approximately the time of perigee. The code has been modified to

take the Eccentricity into account in calculating the map offset time.

-Corrected a minor bug which caused MET in excess of 9999 days to overflow

the assigned format statement for the data block and appear as

"%12038/00:00:00 MET" with the "MET" wrapping around to the left side of

the screen. Obviously, there aren't many satellites with this problem, but

NORAD #00051 is one such. The STSPLUS.LTD entry for this satellite is:

00051,2437158.90208333,0

and the problem was noted because of the pre-launch "temporary" NORAD

number assignment for STS-51.

-A similar problem occured with the large clock displays. In both

projection modes, only the last four days of MET are now displayed (12038

days will display as "2038").

-Added a reminder in the text that when entering filenames using F7 from

the Main Menu, a drive and path may also be included if desired. That

capability has been present for some time but was not stated explicitly.

-Changed the precision that the "plain English" display of the elements

Epoch Time is displayed for approval (F2 from the Main Menu) from [rounded]

integer seconds to 0.001 seconds, coordinated with VEC2TLE V9331.

-Several V9332 Beta Versions were released privately for testing.

Version 9331 -- July 1993

-------------------------

-In coordination with Ken Ernandes' VEC2TLE, the data output for Format 7,

ECI State Vector (Labeled Data), has been modified. The data have been

supplemented by the addition of two lines: Element Set Number; and, Rev

Number at Epoch. The International Designation has been added to the

Catalog Number line and the Epoch Time is now given to .001 seconds. The

data output for Format 4 has been changed so that it is identical to Format

7. See text for details.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 186

-Per Ken Ernandes, the Format 4 & 7 label "Ndot/3" is changed to "Nndot/6".

-I have received NO REPORTS from anyone using Data Formats 5 and 6. Those

formats may not continue to be supported indefinitely UNLESS I am informed

of applications which use them.

CAUTION: STSPLUS Version 9331 Data Formats 4 and 7 are NOT compatible

with VEC2TLE Version 9322 when multiple state vectors are generated!

Use a release of VEC2TLE with a version number equal to or greater

than 9331. (However, manually editing the data to remove the Elset and

Rev Number lines can restore compatibility with the older version.)

NOTE: Users are urged to update to VEC2TLE Version 9331 or the current

version, which ever is later! Not only does the current version of

VEC2TLE maintain compatibility with STSPLUS but several important new

features have been added and a bug in Version 9322 which affected data

accuracy under certain circumstances has been repaired.

-The Tracking Station Mode is now active in both Orthographic and

Rectangular projections. The projection used will same as the one which is

active when the "T" key is pressed.

-Repaired a bug which caused the NORAD number displayed in the Pass

Prediction Mode to be incorrect under certain circumstances. Thanks to

Grant Pinto!

-Repaired a bug in the pass prediction logic which caused the MAX

VISIBILITY data to be missstated by a significant amount for certain

satellites, especially ones with higher eccentricities. Thanks to Grant

Pinto for spotting and reporting the bug!

-Changed the default time zone abbreviation for unrecognized (foreign) time

zones from all spaces (which confused the Pass Prediction setup!) to "LCL".

-Corrected a bug using F2 from the Main Menu which showed no 2-line

elements files if no .TXT files were found in the current directory but one

or more .TLE files were present. Thanks to Bob Krohn for reporting the bug.

If no files of either filetype are found, an error is displayed and the

user is returned to the Main Menu.

-Corrected a bug in default paths if the root directory was used. (Path

defaulted to "B:\\" instead of "B:\" for drive B:, etc.)

-Corrected a bug in the NORAD number for F2 from the Main Menu. If the

NORAD number was greater than 32767, a negative nunber was shown.

-Disabled RIGHTIME detection for 8086/8088 processors. The detection

process affected the time in certain XT-class machines.

-Corrected a minor bug which caused the "blink" feature for the satellite

icon not to function under certain circumstances.

-Corrected a minor bug which initialized the Location Flag incorrectly for

CGA systems when file STSPLUS.INI was not present. Locations and Features

were displayed even though the menu display indicated they were OFF.

-Various cosmetic changes.

-Most of the changes in this release were included in Beta Version 9329.

Version 9320 -- May 1993

------------------------

-This is a MAJOR UPGRADE, adding new improvements and features along with

high precision state vector data output for use with Ken Ernandes' program

VEC2TLE.

-By popular request, STSPLUS now estimates if a satellite may be visible to

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 187

the naked eye or binoculars, and displays "VIS" in bright white next to the

orbit inclination if a visual sighting may be possible. See the section

"Satellite Visibility" for additional discussion.

-Also by popular request, the pass predections (F3, Data Mode 9) have been

enhanced to permit dates and times to be displayed for either UTC/GMT or

LOCAL time. An additional prompt has been added for that selection.

-When displaying predicted passes (F3, Data Mode 9), the satellite is now

approximately centered in the display (instead of being well to one side).

-After considerable confusion and several user comments, I have reworked

the pass prediction logic so that when the user returns to the Main Menu

after the ground track for a predicted pass is displayed, the time is

automatically restored to the real or simulated time in effect BEFORE the

pass prediction was displayed. This means that repeated use of the pass

prediction feature will generally display the same list of numbered passes

and the user no longer needs to restore real time (or reset simulated time)

after displaying predicted passes.

-Corrected a cosmetic bug on pass predictions when an illegal pass number

(greater than the last pass number displayed) was entered at the prompt.

-Added an asterisk ("*") at the left of each event timer when the satellite

is AOS (signal/Sun is being received). This will particularly benefit users

with monochrome or shades of gray.

-Enhanced the Precision X-Y-Z State Vector Data Modes (F3, Data Modes 4

through 7) to include four output formats: multi-line Ascending Node with

state vector, 2 numeric data lines, comma delimited, and multi-line labeled

data. State vectors may now be logged continuously, for a specific time, or

for a specified time span. See text for details.

-Corrected a problem with state vectors being generated at the wrong time

(Data Modes 5 through 7) when the time was entered in UTC and local time

was a different date.

-Removed the low precision state vector data output (F3, Data Mode 4).

-Changed the angle used to calculate the Earth's partial penumbra from 1.2

degrees to 0.3 degrees to better correspond with observed lighting. The Sun

AOS and LOS penumbral calculations were also slightly adjusted; timings

during STS-56 indicated about a 15 to 20 second error before these changes.

-Updated the TDRS information in the section "TDRS Satellite Features" and

all five TDRS satellites are now displayed on the maps at their approximate

locations as of 05/05/93. Thanks to Jim Walls for reminding me!

-Corrected a bug which caused "BASIC Error = 5" when the selected satellite

had probably decayed. #22209 MIR Debris was an example in file TLE180.TXT.

STSPLUS now displays a warning message if the satellite has a current

altitude less than 75 nautical miles and then returns to the Main Menu.

-Corrected the conversion from kilometers to feet per Ken Ernandes. (My

original conversion factor was taken from a 40+ year old Handbook of

Chemistry and Physics and was very slightly in error!)

-Repaired (I think...) a truncation problem which sometimes caused MET/T+E

to run one second slow.

-For all those who refuse to read documentation, I added a reminder to the

Main Menu: "WHILE MAP IS DISPLAYED: F1 = HELP, ENTER = Main Menu".

-Special thanks to Ken Ernandes, Joes Runes, and Willie Musty for state

vectors, testing, and validation of the new features in recent versions!

-Version 9319 was released privately for beta testing.

(Intermediate update notes deleted to save space, available on request.)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 188

Beta Version 9137 -- September, 1991

------------------------------------

-Initial public beta version.

##########