~17

G. I. BUSTER Rev 8.0     ***  3.1  A U T O - K E Y   W I Z Z A R D  ***

~4f**************************** ~17

~4f                             ~17

~4f      F E A T U R E S        ~17

~4f                             ~17

~4f**************************** ~17

****   WIZZARD KEY OPERATION WITH AUTOMATIC MONTHLY UPDATE AND DISPLAY.

****   WORKS ON ALL 032 SERIES BOARDS BOTH MINI AND FULL SIZE.

****   FEATURES THE LATEST G.I. REV.3.1 ALGORITHMS.

****   PROGRAMS AUTOMATICALLY SELECTS CORRECT WORKING KEY.

****   ELIMINATES THE NEED TO PRESS THE HELP BUTTON WHEN SWITCHING

BETWEEN REGULAR AND PAY-PER-VIEW CHANNELS.

****   AUTO-UPDATE CUSTOMER CODE ENTRIES ARE ENCODED ALLOWING

SECURITY FOR THE SERIAL NUMBER.

****   OPERATES WITH THE ORIGINAL 03 TYPE U7-NO EPOXY DIGGING!

****   REQUIRES THAT THE CUSTOMER HAVE AT LEAST ONE AUTHORIZED

CHANNEL.

****   NO CUSTOMER RETURNS- IF AUTO UPDATE BECOMES INOPERATIVE,

JUST SUPPLY THEM WITH NEW WIZZARD CODES BY PHONE.

****   IF UNIT DOES NOT GET A HIT, ALLOWS OPERATION ON EASILY

ENTERED WIZZARD KEYS WHILE THE UNIT UPDATES FOR THE

NEXT MONTH.

****   THIS PRODUCT AVAILABLE FROM:

~60+--------------------------------------------------------------+~17

~60|                   ISLAND VIDEO   1-809-332-2637              |~17

~60|                   PRICE VERSION 8.0: $1,500.00 EACH          |~17

~60|                         VERSION 7.3: $1,500.00 EACH          |~17

~60|                       BOTH FOR ONLY: $2,000.00               |~17

~60|                                                              |~17

~60|       STANDARD GIBUSTER UPDATE FOR 018-019: $500.00 EACH     |~17

~60|                                                              |~17

~60|                                                              |~17

~60+--------------------------------------------------------------+~17

2              O P E R A T I O N A L    I N S T R U C T I O N S

G. I. B U S T E R   R E V 8.0   **   3.1  A U T O - K E Y  W I Z Z A R D

~4F"HELP" BUTTON ~17

This is the basic Auto-Update Key calculation and selection command.

The following typical sreen is displayed:

~70   PUSH 1-2 TO CHANGE    ~17

~70   KEY SELECTED: AUTO 1  ~17

~70    ~F0NEED KEY: 3677       ~17

~70                         ~17

~70        AUTO1:3677       ~17

~70        AUTO2:3777       ~17

~70                         ~17

~70    PUSH VIEW TO EXIT    ~17

When first pushed, the two Auto-Update "Wizzard" keys are calculated

(this month and either last month or next month, depending if the data

stream has sent next month's keys yet). The keys are stored and the

current key being used by the station is selected automatically. Note

that near changeover time, not all stations use the same key. the "NEED

KEY:" line shows the key I.D. or I.D.'s currently being used by the

station tuned in.

Either of the two Wizzard  keys may be selected by pushing 1 - 2

with the selected key shown on line 2, flashing. If known, manually

entered keys can be entered using "SETUP 1-3" described below.

THe Key selected on exit ("VIEW" or "SETUP") will remain selected.

~4F"SETUP" BUTTON:~17

Pushing "SETUP" will cause the following menu to display and the

selected functions are described below:

~70  0: DIAGNOSTICS      ~17

~70  1: INSTALLATION     ~17

~70  2: AUDIO / GRAPHICS ~17

~70  3: CHANGE RATING    ~17

~70  4: RATING PASSWORD  ~17

~70  5: CHANGE MEMORY    ~17

~70  6: DISPLAY MEMORY   ~17

~70  7: TURN OFF DECODER ~17

~70     ( ENTER 0 - 7 )  ~17

page two

3              ~4F-----SETUP 0: DIAGNOSTICS--------~17

The following are of general interest. The top line shows the

currently selected Wizzard key being used for audio decoding.

The 1st 4 digits of the 2nd line count the number of Auto-Updates

received in the data stream. The 2nd 2 digits on the bottom line

show the error rate of the audio key decodes. Anything less than

20 will give sound (Perhaps with noise if over 8 or so). The next

two letters tell the station status:

FA  Open key. Will work with any Videocipher.

SA  Subscription service, authorized.

SB  Subscription service, blocked.

SM  Subscription service, not authorized.

AO  Authorization Off.

~4FSETUP 1: INSTALLATION~17 The following selection menu is displayed:

~70     INSTALLATION:     ~17

~F0   CODE DISPLAYS:      ~17

~70   1: AUTO WIZZARD     ~17

~70   2: ALL WIZZARD      ~17

~70   3: AUTO DATE KEYS   ~17

~F0  CODE ENTRY (CAUTION) ~17

~70   4: MANUAL WIZZARD   ~17

~70   5: AUTO UPDATE KEYS ~17

~4F 1: DISPLAY AUTO UP-DATE WIZZARD CODES.~17

The two Auto Update Wizzard codes are shown in easily

read format. A flashing arrow points to the code being

used for the month. The top key or code (AUTO 1) was

calculated using the station enable codes and monthly

date codes from 6107-611A in RAM, while AUTO 2 was

calculated using data at 616A-617D.

Note that these codes can be converted to the long enter

format some chips use by putting a "0" before 0-9's and

"9"+(1-6) for A-F's  (I.E. 77 = 07 07,C  = 9 + 3)

~4F  2: DISPLAY ALL WIZZARD CODES. ~17

All the Wizzard codes, both the two auto update codes and

the 6 manually entered codes are shown on the screen. The

number flashing shows the currently selected code (the

code selected when "HELP" was exited last time).

~4F  3: DISPLAY AUTO DATE KEYS.  ~17

The 2 station enable masks and date codes that have been

sent in the data stream and stored at 6107-611A (used by

AUTO 1) and at 616A-617D (used by AUTO 2) are displayed.

A flashing arrow points to the code presently in use.

page three

~4F------4: ENTER MANUAL WIZZARD CODES-------~17

This selection will cause the following screen to display:

~70   WIZZARD KEY ENTER    ~17

~70  SELECT 3-8 TO CHANGE  ~17

~F0      NEED KEY: 3777    ~17

~70                        ~17

~70       AUTO1:3877       ~17

~70       AUTO2:3777       ~17

~70                        ~17

~70    PUSH VIEW TO EXIT   ~17

Push 3 - 8 to select which key to replace. The "NEED KEY"

number shown is the key I.D. or I.D.'s currently being

used by the station tuned in (the last two digits of any

entered code).

After selection of 3 - 8, the following screen appears:

~70   WIZZARD KEY3 ENTRY.  ~17

~70  (-->) MAKES 1-6 ^ A-F ~17

~70  (<--) CANCELS ENTRIES ~17

~70                        ~17

~70   **** **** **** ****  ~17

~70   **                   ~17

~70                        ~17

~70    PUSH VIEW TO EXIT   ~17

The top line shows the key number being replaced. Entries

will replace the *'s as they are entered. Pushing the

right-arrow will put an up-arrow on the screen and then

a 1-6 entry will replace it with an A-F. Pushing the back-

arrow will replace an up-arrow if one exists or erase the

entry replacing it with a *. After the last number is

entered, the next to last line will change to:

~F0  ** CAUTION **  'ENTER'~17

At this point, pushing "ENTER" will replace any prior entry

at the number flashing on the top line. Pushing the back-

arrow will still back up the display, allowing corrections.

The 'ENTER' on the next to last line will disappear.

page four

~4F  -------5: ENTER AUTO UP-DATE CODES.----------- ~17

This selection will cause the following screen to display:

~70-------------------------~17

~F0   **CAUTION**   AUTO    ~17

~F0    UP-DATE KEY ENTRY.   ~17

~70                         ~17

~70   0: SKIP TO DATE KEYS  ~17

~70                         ~17

~70                         ~17

~70     PUSH VIEW TO EXIT   ~17

~70-------------------------~17

~4F 0: SKIP TO DATE KEYS. ~17

Selection of "0" will skip to the entry of date keys used

by the Auto-Update function. This would normally be done

if the serial number and seed keys are good but the box

had not updated normally (I.E. it was off for a month).

Either of the other selections will then automatically

advance to the date key entry, and entry is covered below.

~4FAUTO DATE KEY ENTRY~17

The screen will now change to:

~70-------------------------------~17

~70      (-->) MAKES 1-6 ^ A-F    ~17

~70      (<--) CANCELS ENTRIES    ~17

~70        ENTER 20 DATE KEYS.    ~17

~70       (6107-1A)OR(616A-7D)    ~17

~70       **** **** **** ****     ~17

~70       **** **** **** ****     ~17

~70       **** ****               ~17

~F0           **CAUTION**         ~17

~70        PUSH VIEW TO EXIT      ~17

~70-------------------------------~17

Entry of the current month date keys should now be done.

These keys can be read in an operating unit using "SETUP

1 (INSTALLATION) 3 (AUTO DATE KEYS). See Wizzard key entry

above for entry details. After "ENTER" is pushed, the

entered date enable mask and date keys are stored at both

of the Auto Up-Date Ram locations. Thus, until a data

stream update is made, both AUTO UP-DATE keys will be the

same. The program will now automatically execute as if

power were just turned on.

Page Five

~4F SETUP 2: AUDIO / GRAPHICS ~17

This selection will cause a typical screen display of:

~70--------------------------~17

~70      AUDIO MODE IS       ~17

~70          PRIMARY         ~17

~70    (  PRESS '1' FOR  )   ~17

~70    (    ALTERNATE    )   ~17

~70                          ~17

~70    AUDIO ON GRAPHICS IS  ~17

~70          ENABLED         ~17

~70    (  PRESS '2' TO   )   ~17

~70          DISABLE         ~17

~70--------------------------~17

Entering "1" will alternate the audio mode (the same as a

stock box). Entering "2" will alternate the audio-on  graphics

(the operation of the growing "G. I. BUSTER" message every

time the audio decode keys are recomputed).

~4FSETUP 3: CHANGE RATING (MOVIE PROGRAM RATING)   ~17

~4FSETUP 4: RATING PASSWORD (MOVIE RATING PASSWORD)~17

These setup selections work exactly as a stock videocipher box.

Note that an "ENTER" with no password entered in 4 will clear

any password (as long as any prior password was known to get

to change it). Then the rating ceiling in 3 can be changed

without entering any password.

~4FSETUP 5 CHANGE MEMORY~17  This selection will cause a screen display of:

~70----------------------------~17

~70      **** ENTER START ADR. ~17

~70      (-->) MAKES 1-6 ^ A-F ~17

~70      (<--) CANCELS ENTRIES ~17

~F0       ** CHANGE MEMORY **  ~17

~70        REG.: (0000-007F)   ~17 (Means U19 Registers)

~70        RAM : (6000-67FF)   ~17 (MEANS U30 RAM)

~F0           **CAUTION**      ~17

~70        PUSH VIEW TO EXIT   ~17

~70 ---------------------------~17

The program will not allow address entry outside the range shown.

If it is attempted, the screen will blank white except for two

lines showing the allowed address range, then restore to the above

menu.  After entry of the address to be changed (see Wizzard

entry above for entry details), the display will change to:

~70 -------------------------~17

~F0   ** CHANGE MEMORY **    ~17

~70    REG.: (0000-007F)     ~17

~70    RAM : (6000-67FF)     ~17

~F0   xxxx  **CAUTION**      ~17

~70   BBBB BBBB BBBB BBBB    ~17

~70   XXXX XXXX XXXX XXXX    ~17

~70   AAAA AAAA AAAA AAAA    ~17

~70   CAN : ENTER NEW ADR    ~17

~70   VIEW: QUIT             ~17

~70 -------------------------~17

Page Six

~4F ----SETUP 5: CHANGE MEMORY, CONTINUED-------~17

The xxxx on the 4th line shows the address to be changed next.

The location to be changed will be flashing on the XXXX line.

The line above (BBBB) shows the prior 8 location's data.

The line below (AAAA) shows the next  8 location's data.

"ENTER" will advance the nibble to be changed to the right.

Back-arrow will shift the nibble to be changed to the left.

"0-9" will change nibble to 0-9 unless right-arrow was entered

prior (up-arrow shows). Then it will cause entry of A-F.

When the location being changed gets before or after the XXXX

line, the before - after data will update and keep the

data to be changed always on the XXXX line.

The line 4 address always updates to the address being changed.

The data shown is being constantly updated. If the Videocypher

program is writing to a location, the data may change right

back after it is changed.

"CANCEL" will cause the SETUP 5 screen to reappear, allowing

a new address entry.

"VIEW" or any other function button will terminate the program

and do the pushed function.

~4F SETUP 6: DISPLAY MEMORY ~17

This selection will cause a screen display of:

~70-----------------------------~17

~70    **** ENTER START ADR.    ~17

~70    (-->) MAKES 1-6 ^ A-F    ~17

~70    (<--) CANCELS ENTRIES    ~17

~70                             ~17

~70    AFTER DISPLAY:           ~17

~70    (-->)  STEP UP 40        ~17

~70    (<--)  STEP DOWN 40      ~17

~70    CAN :  ENTER NEW ADR     ~17

~70    VIEW:  QUIT              ~17

~70-----------------------------~17

After entry of the starting address of the data display area

(see Wizzard entry above for entry details), the display will

change to:

~70-----------------------------~17

~70     SSSS EEEE               ~17

~70     DDDD DDDD DDDD DDDD     ~17

~70     DDDD DDDD DDDD DDDD     ~17

~70     DDDD DDDD DDDD DDDD     ~17

~70     DDDD DDDD DDDD DDDD     ~17

~70     DDDD DDDD DDDD DDDD     ~17

~70     DDDD DDDD DDDD DDDD     ~17

~70     DDDD DDDD DDDD DDDD     ~17

~70     DDDD DDDD DDDD DDDD     ~17

~70-----------------------------~17

Page Seven

~4F ------DISPLAY MEMORY, CONTINUED------- ~17

SSSS is the starting address of 40 Hex data DDDD and EEEE is

ending address.

The data is constantly being updated.

The right-arrow will step up the data address 40 Hex.

The left-arrow will step down the data address 40 Hex.

"CANCEL" will cause the SETUP 6 screen to reappear, allowing

a new starting address entry.

"VIEW" or any other function button will terminate the program

and do the pushed function.

~4F SETUP 7: TURN OFF DECODER ~17

Selecting this function will cause the screen to display:

~70---------------------~17

~70    DESCRAMBLER WILL ~17

~70     BE LOCKED OFF.  ~17

~70---------------------~17

After a few seconds, the message will disappear and the V.C.

light will go off. On most IRD's, changing the channel will

restore operation. On some, pushing view or cycling power may

be necessary. This function is useful to receive the station's

normal subcarrier audio signal.

"VIEW" BUTTON   Display current program name and time left.

"NEXT PROGRAM" BUTTON:  Display next program name and time left.

"TEXT" BUTTON:  Display any text sent.

"MESSAGE" BUTTON:  Display any messages sent.

###########THIS PRODUCT AVAILABLE FROM:

~60+------------------------------+ ~17

~60|   ISLAND VIDEO               | ~17

~60|   1-809-332-2637             | ~17

~60|   PRICE: $1,500 EACH         | ~17

~60|                              | ~17

~60+------------------------------+ ~17

Downloaded From P-80 International Information Systems 304-744-2253