1. Eprom writer card I/O address

The I/O address of Eprom writer card are as shown below:

model: EW-901	3C0-3C7

model: EW-901A	3C0-3C7

model: EW-901B	390-397

model: EW-904	3C0-3C7

model: EW-904A	3C0-3C7

model: EW-904B	390-397

model: EW-910	3C8-3CF

2. The Eprom type and programming voltage are as shown below:

*** Advanced Micro Devices ***

Eprom type / Programming Voltage

2716		Vpp=25V

--------------------------------

2732		Vpp=25V

2732A		Vpp=21V

--------------------------------

2764		Vpp=21V

--------------------------------

2764A		Vpp=12.5V

--------------------------------

27128		Vpp=21V

--------------------------------

27128A		Vpp=12.5V

--------------------------------

27256		Vpp=12.5V

--------------------------------

27512		Vpp=12.5V

--------------------------------

*** Fujitsu ***

Eprom type / Programming Voltage

--------------------------------

2732A-35	Vpp=21V

2732A		Vpp=21V

--------------------------------

27C32A		Vpp=21V

--------------------------------

MBM2764-20	Vpp=21V

MBM2764-25	Vpp=21V

MBM2764-30	Vpp=21V

--------------------------------

MBM27C64-20	Vpp=21V

MBM27C64-25	Vpp=21V

--------------------------------

MBM27128-20	Vpp=21V

MBM27128-25	Vpp=21V

MBM27128-30	Vpp=21V

--------------------------------

MBM27256-25	Vpp=12.5V

MBM27256-30	Vpp=12.5V

--------------------------------

MBM27C256-20	Vpp=21V

MBM27C256-25	Vpp=21V

MBM27C256-30	Vpp=21V

--------------------------------

*** Hitachi ***

Eprom type / Programming Voltage

HN462716	Vpp=25V

--------------------------------

HN462732	Vpp=25V

HN462732P	Vpp=25V

--------------------------------

HN482732A	Vpp=21V

--------------------------------

HN27C32		Vpp=25V

--------------------------------

HN27C32A	Vpp=21V

--------------------------------

HN482764G-2	Vpp=21V

HN482764G	Vpp=21V

HN482764G-3	Vpp=21V

--------------------------------

HN482764P	Vpp=21V

HN482764P-3	Vpp=21V

--------------------------------

HN27C64G-25	Vpp=21V

HN27C64G-20	Vpp=21V

--------------------------------

HN4827128G-25	Vpp=21V

HN4827128G-30	Vpp=21V

HN4827128G-45	Vpp=21V

--------------------------------

HN4827128P-30	Vpp=21V

--------------------------------

HN27128AG-20	Vpp=12.5V

HN27128AG-30	Vpp=12.5V

--------------------------------

HN27256G-25	Vpp=12.5V

HN27256G-30	Vpp=12.5V

--------------------------------

HN27C256G-20	Vpp=12.5V

HN27C256G-25	Vpp=12.5V

HN27C256G-30	Vpp=12.5V

--------------------------------

HN27512G-25	Vpp=12.5V

HN27512G-30	Vpp=12.5V

--------------------------------

*** Intel ***

Eprom type / Programming Voltage

2716		Vpp=25V

--------------------------------

2732		Vpp=25V

--------------------------------

2732A		Vpp=21V

--------------------------------

2764A-2		Vpp=21V

2764A		Vpp=21V

2764A-3		Vpp=21V

--------------------------------

P2764A		Vpp=12.5V

P2764A-3	Vpp=12.5V

--------------------------------

27C64		Vpp=21V

27C64-2		Vpp=21V

--------------------------------

27128		Vpp=21V

27128-3		Vpp=21V

27128-4		Vpp=21V

--------------------------------

27128A-2	Vpp=12.5V

27128A-3	Vpp=12.5V

--------------------------------

27256		Vpp=12.5V

27256-3		Vpp=12.5V

--------------------------------

27512		Vpp=12.5V

27512-3		Vpp=12.5V

--------------------------------

*** Mitsubishi ***

Eprom type / Programming Voltage

2716		Vpp=25V

--------------------------------

2732		Vpp=25V

2732A		Vpp=21V

--------------------------------

M5L2764K-2	Vpp=21V

M562764K	Vpp=21V

M5L2764K-3	Vpp=21V

--------------------------------

M562764P-3	Vpp=21V

--------------------------------

M5L27256K	Vpp=12.5V

--------------------------------

*** National Semiconductor ***

2716		Vpp=25V

27C16		Vpp=25V

27C16H		Vpp=25V

--------------------------------

2732		Vpp=25V

27C32		Vpp=25V

27C32H		Vpp=25V

--------------------------------

2764		Vpp=21V

2764H		Vpp=21V

--------------------------------

NMC27C256	Vpp=12.5V

--------------------------------

*** Nippon Electric Company,Ltd. ***

Eprom type / Programming Voltage

2716		Vpp=25V

--------------------------------

2732		Vpp=25V

2732A		Vpp=21V

--------------------------------

D2764D-2	Vpp=21V

D2764D	Vpp=21V

D2764D-3	Vpp=21V

--------------------------------

D2764C	Vpp=21V

D2764C-3	Vpp=21V

--------------------------------

D27C64D-20	Vpp=21V

D27C64D-25	Vpp=21V

--------------------------------

D27C64C	Vpp=21V

--------------------------------

D27128D	Vpp=21V

D27128D-3	Vpp=21V

D27128D-4	Vpp=21V

--------------------------------

D27128C-3	Vpp=21V

--------------------------------

D27256D	Vpp=21V

D27256D-3	Vpp=21V

--------------------------------

*** Toshiba ***

Eprom type / Programming Voltage

2716		Vpp=25V

--------------------------------

2732		Vpp=25V

2732A		Vpp=21V

--------------------------------

TMM2764D-2	Vpp=21V

TMM2764D	Vpp=21V

--------------------------------

TMM27128D-20	Vpp=21V

TMM27128D-25	Vpp=21V

--------------------------------

TC57256D-20	Vpp=21V

TC57256D-25	Vpp=21V

--------------------------------

27C256		Vpp=12.5V

--------------------------------######################

Downloaded From P-80 International Information Systems 304-744-2253