TMS-7000 Assembler Program

Version 3.4

This program takes ASCII source code files for the Texas Instruments'

TMS-7000 series microprocessors and assembles them into executable object code

files ready to be burned into an EPROM.  The Assembler can operate with several

different types of input source files.  The length of the source file is

limited only by the amount of memory in your PC.  The maximum length of the

assembled object code file is 64K.

This assembler is fast, but achieves this speed by hogging memory.

It allocates 1 64K segment for itself and the Symbol Table, 1 64K segment for

the assembled object code file, and the remainder of memory for the source

file.  I assembled a 271K source file into a 16K object file in 16 seconds on

an 8 MHz PC-XT Clone with 640K.

The TMS-7000 Assembler has a number of features for the VCII software

experimenter.  I have included a pseudo-op for assembling packed strings for

display using the VCII U6 chip.  The "DM" pseudo-op takes all text inside

quotation marks and converts it into the equivalent U6 packed character set.

In addition, the Define Word "DW" and the Define Byte "DB" pseudo-ops can

perform limited math functions to locate addresses based on the values of

symbols.

This is release 3.4 of the assembler.  The 3.4 version of the assembler

is compatible with earlier versions of the assembler.

CHANGES:  (1) Version 3.4 of the Assembler fixes a rare bug that can

occur when address tables composed of combinations of

DB and DW values are used.  This bug caused all address

counter values to be off by a value of 5 for the remainder

of the assembly.  The resulting object code would then fail

checksum tests and/or not execute properly.

(2) The assembler now checks the amount of free RAM available

before loading the source code file.  If sufficient RAM is not

available to permit loading of the entire source code file,

an error message is printed and the program terminates.  If

you have a number of memory-resident programs (i.e., SideKick)

loaded and active in your PC, removing these programs will free

up additional memory for the assembler.  Source code files up

to about 300K are generally no problem in a 640K PC with only

SideKick resident.  Larger files or more TSR programs in the

PC may cause a memory shortage for the assembler.

(3) The assembler will now ignore all ASCII tab codes (hex 09)

that some text editors place in the source code.  Prior

releases of the assembler had been having weird problems with

those editors that place ASCII tab codes instead of spaces in

the source code file.

Send comments or suggestions to -

Bill Meeks

1918 Tap Road

Vidalia, GA  30474

NEW FEATURES OF THE ASM7000 TMS-7000 ASSEMBLER

Several new features were added with release 3.3 of the Assembler.

These are explained below.

1)  The Assembler will now take either decimal or hexadecimal values

for all number values, including the "DB" psuedo-op.  The maximum

decimal value allowed is 65535, positive integers only.  The RADIX

psuedo-op is used to set the Assembler for decimal or hexadecimal

numbers.

EXAMPLE -      RADIX        10              ;Set Radix to Decimal

MOV          %38,A

This sequence will cause the Assembler to generate

the byte sequence 22 26 in the Object Code.  The

decimal value 38 will be converted to binary.

This new feature will also work with 16-Bit Word values

such as -

MOVD         %49152,R35

Note that a new psuedo-op has been added to govern the

conversion of immediate operands for 16-Bit Word values

used in the MOVD, LDA, STA, BR, and CMPA instructions.

The new psuedo-op, "WORDS" is described below.  It

can be used to override the default RADIX value for the

argument of 16-Bit immediate operands.  Unless

specifically overridden with the "WORDS" psuedo-op,

these instructions will treat their immediate operands

as hexadecimal values.

2)  The Assembler will generate an ASCII text file of the Symbol

Table and write this file to the same disk drive that the Object

Code file was written to.  The listing of the Symbol Table will

carry the name of the Object Code file with the exception of the

extension, which will be .SYM.  The Symbol Table listing file has

the name of the Symbol in one column and its corresponding

hexadecimal address in the Object Code in the other column.  This

file may be sent to a printer using the MS-DOS "PRINT" command

or any similar print spooler.  The listing file is pure ASCII.

The primary benefit of the Symbol Table listing is for debugging

assembled object files.  The actual hexadecimal addresses of

specific Labels can readily be determined from the listing.

3)  The Assembler will now permit additional math functions based on the

values of Symbols/Labels.  The math functions are integer-based

addition, subtraction, multiplication, and division.  The arguments

may be either decimal or hexadecimal numbers, the conversion being

controlled by the RADIX psuedo-op.

EXAMPLE -  Assume the following code sequence:

RADIX 10

dBYTE1:   EQU   16

. . .

. . .

MOV %dBYTE1 + 6,A

This will cause the value of BYTE1 at assembly time

to be added to 6 and the result stored in A.  The

op-code sequence would be, 22 16 (Note that this is

16 hex which is 22 decimal).

Anywhere a Symbol is used, the addition or subtraction signs,

"+" or "-", or the multiplication or division signs, "*" or "/",

may be used as desired.

4)  The Assembler will now allow either 1 or 2 character values for

hex bytes or 1 to 4 character values for hex words.  Formally,

the Assembler required all byte values to be 2 ASCII characters

and all word values to be 4 ASCII characters.

EXAMPLE -  The Version 2.x and 1.x series required this type

of hex byte argument;

DB   >05

and this type of hex word argument;

DW   >0001

Version 3.x series will work with either the old 2

character type or a single character type;

DB   >5

or

DB   >05

DW   >1

or

DW   >0001

Note that a leading zero for a number causes the number to be

evaluated as a hexadecimal number.  This is a change from

earlier versions of the assembler.

5)  The Assembler will now allow Symbol/Label values for immediate

operands.  The assembly time value of the Symbol/Label will be

generated as the immediate operand.

Example  --  MOV   %dBYTE1,A

This will cause the Assembler to generate the

instruction sequence to load A with the immediate

value of BYTE1 at assembly time.

6)  A new psuedo-op has been added to allow the handling of ASCII

strings in the source file.  The "DT" psuedo-op causes the direct

writing into the object code of the ASCII values of the arguments

specified by the string.  The string MUST be delimited by quotation

marks.

Example  --  DT   "This is a sample string"

This will cause the ASCII values of the delimited

string to be assembled directly into the object

code.

7)  The Assembler now compares all "word" values for CALL, BR, LDA, STA,

and MOVD instructions to the ORG given at the beginning of the code.

If the "word" value is less than the origin of the object code, it

is treated as an immediate "literal" value.  If it is greater than

or equal to the origin, it is treated as a symbol name.  This occurs

only if the value is not explicitly declared as a symbol name by the

"a" or "d" designator in front of the name.  If the value is not

marked with the "a" or "d" designator, it is evaluated according to

the WORDS radix psuedo-op discussed below, however the evaluation

will proceed in hexadecimal if the value contains a leading zero.

PSEUDO-OPS ---

RADIX  --  The Radix pseudo-op establishes the default base for

numbers.  It is a required statement in all source files

and must come before any Instruction Mnemonics.

The only valid arguments for Radix are 10 and 16 for decimal

and hexadecimal bases respectively.

Example -   RADIX  10  ;sets base to 10

RADIX  16  ;sets base to hex

Note:  The hexadecimal designator ">" can be used to

override the treatment of operands according to

the RADIX argument.  A leading zero can be used

for the same purpose.

Example -   MOV    %>32,A

MOV    %032,A

Will cause the value 32 to be

treated as a hex value regardless

of the default set by RADIX.

ORG  --    The Origin pseudo-op establishes the beginning address for

the assembled object code.  Multiple ORG statements are

allowed.

Example -  ORG  >F000  ;sets address to F000

ORG  >FFD0  ;sets address to FFD0

The ">" sign denotes a hex number follows.

Without the ">" designator to force a hexadecimal value,

the argument following the ORG psuedo-op is evaluated

according to the RADIX default value.

EQU  --    The Equate pseudo-op sets a value for a symbol.  Subsequent

references to that symbol cause the value given by EQU to

be substituted.

Example -   APORT:  EQU  >0104

Subsequent references to APORT cause the assembler to return

the value 0104.

SYMBOL  -- Valid switches are ON or OFF.  The default is OFF.  This

pseudo-op causes the Assembler to treat all memory addresses

at or above C000 hex as labels/symbols.  If this type action

is desired, the command SYMBOL ON should be used at the

beginning of the source code.

Please note that when SYMBOL ON is used, MOVD instructions

may assemble differently or cause UNDEFINED SYMBOL error

messages.

Example --  Consider the following lines of code:

SYMBOL  ON

...

...

MOVD %D4E5, R35

MOVD %>D4E5, R35

The 1st MOVD instruction will attempt to

locate the symbol "D4E5" and use its address

to load into R34,35.

The 2nd MOVD instruction will force the

Assembler to generate the proper instruction

to load the hex value D4E5 into R34,35, due

to the Hex Immediate operand type flag- ">".

DB  --     The Define Byte (DB) pseudo-op converts the value following

the DB statement into a hexadecimal byte.  Special cases

allow the 2 arithmetic operators "/" and "MOD" when used

with Symbols.

Example -  DB  >4D  ;causes 4D to be stored at

;current address

DB  10   ;causes the value to be

;evaluated according to

;the RADIX flag and stored

;as a binary value at the

;current address

DB   aDPORT/256  ;causes the value

;of DPORT to be

;divided by 256 and

;the high byte saved                                                            ;at the current

;address

DB   aDPORT MOD 256  ;causes the

;value of DPORT

;to be divided

;by 256 and the

;low byte saved

;at the current

;address

DM  --    The Define Message (DM) psuedo-op generates VCII packed

strings from the ASCII string following the statement.

The text string must be delimited by quotation marks.

Example - DM    "   DIAGNOSTIC DATA  "

causes the assembler to generate the following 15 byte

sequence in the object code segment -

BA EB 83 20 01 90 45 55 88 0A E0 C0 58 0B AE

All strings must be an even multiple of 4 characters, such

as 4, 20, 160, etc.  Only valid U6 characters will be

assembled.  Invalid U6 characters will be treated as spaces.

DS  --    The Define Space (DS) psuedo-op causes the Assembler to

reserve space in the Object Code.  The empty space is set

to hex >FF.  The argument following the DS psuedo-op is

the number of bytes to reserve.  It is evaluated according

to the RADIX default.

Example --  DS  4000

Reserve 4000 bytes if RADIX is 10 or

16384 bytes if RADIX is 16.

The effect of DS is to cause the next instruction to be

assembled at an offset from the previous instruction as

specified in the argument for DS.

DT  --    The Define Text (DT) psuedo-op causes the values delimited

by quotation marks to be treated as ASCII character bytes

to be assembled directly into the object code.

Example -  DT   "This is a sample string"

DW  --    The Define Word (DW) psuedo-op causes the value following

the DW statement to be converted into a 16-Bit word value

and inserted into the object code at the current address.

Example -  DW   >CCD7   ;CCD7 is converted to a hex

;word and stored at the current

;object code address

DW   dHERE1  ;the hex value of HERE1 is

;stored at the current object

;code address

The operand designator ">" can be used to force the Assembler

to generate hexadecimal numbers whenever the SYMBOL ON psuedo

op is used in the source code.

Example -  DW  FFFF     ;if SYMBOL ON is used, the FFFF

;operand is interpreted as a

;symbol.

DW  >FFFF    ;this form forces generation of

;the hex sequence FF FF in the

;code.

NOTE:  This same technique can be applied in the

MOVD series of instructions.

Example -  MOVD %FFFF, R47  ;FFFF is symbol

MOVD %>FFFF,R47  ;FFFF is immediate

MOVD %0FFFF,R47  ;FFFF is immediate

END  -- The END psuedo-op is an optional parameter that tells the

assembler it is at the end of the source file.  This is

an optional statement however.  The TMS-7000 Assembler is

smart enough to know it is at the end of the source file.

The END psuedo-op has no arguments.

WORDS -- The WORDS psuedo-op is provided to allow selective over-

riding of the RADIX treatment of 16-Bit Word values in

certain instructions.  It is provided mainly as a convenience

to allow easy use of disassembled listings which may have

decimal numbers for registers and byte values, but hexadecimal

values for all word values.  Valid arguments for WORDS are

10 and 16 to set the 16-Bit Word radix default to either

decimal (10) or hexadecimal (16).  Upon start-up, the Assembler

sets the WORDS default to hexadecimal (16).

$  --  This token represents the location counter.  It is a reserved

symbol in ASM7000 syntax.  It may be used to represent the

current value of the object code address pointer.  It basically

represents "here".

For Example -    C005    MOVD %$+6,R47

This instruction sequence causes the

value of the location counter (C005)

to be added to 6, and the result used

as the immediate operand for the MOVD

instruction.  Therefore this source

code line would result in the following

object code sequence:

C005 ==>  88 C0 0B 47   ;MOVD C00B to R47

NOTES --  The TMS-7000 Assembler assembles the entire instruction set of the

TMS-7000 series CPU's.

Because I wrote the assembler to directly take the output of a

disassembler, I wrote it so that it can sort through the address

field and op-code field and locate the symbols and instructions.

There are a couple of catches, however.

Example --   F0E3  88 62 47 35    MOVD %>6247,R35  ;put 6247 in R35

In this line from a typical disassembler, F0E3 is the

address and "88 62 47 35" is the op-code sequence.

My assembler will skip everything until it detects a

valid instruction sequence, such as the MOVD.  This

does leave the assembler vulnerable to some loopholes

from time to time.  First, ALL instructions must have

a space between the instruction mnemonic and the operands                       for the instruction.  Second, some lines of source may

have to be modified to delete the address field if it

happens to contain an instruction mnemonic.  For example

the line below would generate an assembler error.

FADC 88 62 47 35    MOVD %>6247,R35

The error comes from the F (ADC) part of the address

field because ADC is a valid TMS-7000 instruction.

This line in the source would have to have the FADC

part deleted.  There are several other combinations

that will confuse the assembler, all involving the

address field hex numbers matching instructions, such

as FDEC (DEC), CDAC (DAC), etc.

There is one other requirement that is peculiar to this assembler.

It requires that all labels and or symbols be preceeded by either

an "a" or "d".  This is necessary so the assembler can pick labels

from the other data on the line from typical disassemblers.  The

only time the "a", "d" or "L" designator is not needed is when the

symbol is referenced in the Peripheral File instructions, such as

MOVP, ANDP, BTJZP, etc.

Labels and Symbols may end either with a space or with the ":"

character.  The maximum length for labels and symbols is 20 characters.

The assembler has a symbol table large enough for approximately 3500

symbols, which should satisfy all requirements.

USING THE ASSEMBLER --

The assembler is invoked from the DOS prompt by typing ASM7000.  The

program will then load and prompt you for the source code filename and

the object code filename.  If the object code filename you specify

already exists, a warning message is issued and you are asked whether

or not the existing file is to be overwritten.

When assembly is complete, the program returns to the DOS prompt.

You may issue the source and object code filenames from the command

invocation line by typing them after ASM7000.  The source file name

must be come first, followed by the object code filename.  If the

object code filename already exists, you will be warned as above.  The

filenames must be separated by a single space on the command line.

You may specify a full DOS pathname for locating the files, or ASM7000

follows the DOS rules by looking first in the current directory, and

then follow the path(s) specified in the DOS 3.3 APPEND command to

locate the files.

ERRORS --

The assembler does rather extensive error checking during the assembly

process.  If an error is detected, the offending line is sent to the

display, and the appropriate error message is printed.