The 8052 Real-Time Multi-Tasking Executive

(0) INTRODUCTION

An interrupt-driven multi-processing architecture has been designed for the

Intel 8051-family of processors, complete with a small library of routines to

implement its basic operations.  Interrupts are generally reserved for

process-switching and are dynamically enabled or disabled by the routines in

this package.

Initially, the only process running is "main".  When the 8051-processor

starts up from reset, a short initialization sequence will initialize the

multiprocessing architecture and then send control to "main", which it is

assumed the programmer will define (the same as in C).

New processes are created by using "Spawn", which has the effect of calling

the process, but also initializing a new stack segment for that process.  The

new process will continue until either it returns, or until it calls "Pause".

A "Pause" will stop it, and freeze its stack.  It will remain frozen until the

corresponding "Resume" is called, at which point it resumes right at where it

left off AS IF NOTHING HAPPENED IN BETWEEN.

It is assumed that each interrupt active will call its own "Resume".

This provides a way to have processes go into the background and wait for a

hardware interrupt while other processes can resume.

(1) ARCHITECTURE

Each process "Spawn"-ed will have its own segment on the 8051 stack, which it

can use as its calling stack.  Generally, the global stack will occupy registers

128 to 255 in the current version of the library, and the address 128 is set to

be the base address of the "main" process.

It is up to the programmer to define the stack segments used for each

process.  Further, there is no error-checking for overlapping stack segments,

so the programmer is responsible for assuring that large enough stack segments

are allocated to each process to prevent conflicts.

The library, though it contains code for interrupt handlers, is not

completely self-contained.  The programmer must make modifications to the

interrupt handlers pushing and popping any registers that two or more processes

make concurrent use of.  If the programmer is using more than one priority

level of interrupts, then at the very least, the registers R0, R1 should be

saved.  It is also assumed that the register bank is permanently switched to

bank 0.

The key to this multi-processing model is the Priority Stack.  Initially

empty, each time a Spawn or Resume takes effect it will save the current

stack pointer in order to allow the stack pointer for the new process to be

loaded.  When this new process either returns or Pauses, the old stack pointer

is popped off the stack and is restored as the current stack pointer.

This allows stack segments to be switched very quickly and in a way that is

consistent with the priority structure of the 8051 interrupts, so that

processes Resumed with higher priority interrupts can preempt processes

currently running.

There will generally be registers for each interrupt source used to save

stack pointers on Pauses and Resumes.  In addition, there will be more registers

to effectively enable software interrupts.  This means that every process

currently active will have its stack pointer saved either on the Priority

Stack, in a stack pointer register, or will be currently running in the

foreground with the current stack pointer.

This library provides the following registers for saving stack pointers:

SP_IE0, SP_IE1: For external interrupts 0, and 1.

SP_TF0, SP_TF1, SP_TF2: For timer 0, 1, and 2 overflow interrupts.

SP_RI, SP_TI: For serial receive and transmit interrupts.

SP_EXF2: For the timer 2 external capture interrupt.

SP_SW0, SP_SW1, SP_SW2, SP_SW3: For "software interrupts" 0, 1, 2, and 3.

The first 8 are tied to the hardware interrupts, so that for instance, when

external interrupt 0 is active, its interrupt-handler will call Resume with

register SP_IE0.

(2) INITIALIZATION

Initialization consists of the following actions:

(1) The Priority Stack is cleared.

(2) The first stack segment, for main, is set up and the calling

address for "_Die" is pushed onto it.  Thus when main returns,

it will jump to the routine _Die, which is an infinite loop.

(3) Just under this stack, the calling address Exit will be inserted.

This provides a convenient place to initialize unused stack pointer

registers.

(3) SPAWNING A NEW PROCESS

A new process is Spawned by loading its address into the DPTR register,

and the base address of its stack segment (which is up to the programmer

to provide) in register R0 and then calling Spawn.  Spawn will then perform

the following actions:

(1) Push the current stack pointer onto the Priority Stack.

(2) Set the stack pointer to R0

(3) Initialize the stack segment by pushing Exit onto it.

(4) Jump to the address indicated by DPTR to begin the process.

By pushing the routine Exit onto the bottom of the stack, this provides

a clean way for the process to finish.  For when the process returns, the

routine Exit will be called, which will reload the current stack pointer by

popping the Priority Stack and thus effectively carrying out a return that

eliminates the current process.

Spawning a new process that returns without ever calling "Pause" will

therefore be the same as just calling that process and executing a normal

return.

(4) SWITCHING PROCESSSES WITH Pause AND Resume

The routines Pause and Resume are called after loading register R0 with

the address of the stack pointer register that will be used to load or save

the stack pointer.

Pause carries out the following actions:

(1) The current stack pointer is saved in the location indicated by R0.

(2) An address is popped off the Priority Stack into the current stack

pointer.

Resume carries out the currespondingly opposite actions:

(1) The current stack pointer is pushed onto the Priority Stack.

(2) The current stack pointer is loaded with the value saved in the

location indicated by register R0.  This location is "cleared" by

setting it to the portion of the global stack just under the

"main" stack segment, where Exit is located.

A Resume will thus reactivate the process frozen by the last call to the

corresponding Pause at exactly the place this Pause was called.

Both of these routines are, what is known as, critical regions.  Under no

conditions can they be interrupted, because to do so would destroy the integrity

of the Priority Stack architecture.  Therefore, interrupts would need to be

temporarily turned off in these routines if there is than one interrupt level

in your application.

(5) EXAMPLE

Suppose we want to echo the keys pressed on a keyboard or keypad to

a screen, while simultaneously listening on the serial communications line

for new commands.  Suppose that the keypad or keyboard is flagged using the

external 0 interrupt.  We would have a process, Echo, to carry out the echoing

action and a process, Interpreter, for reading and interpreting commands

obtained from the communications line.

Then main could probably be written as follows:

main:

... INITIALIZE INTERRUPTS AND REGISTERS ...

;;; Spawn the "Echo" routine, giving it the stack segment 90 to a0 (hex).

mov R0, #90h

mov DPTR, #Echo

call Spawn

;;; Spawn the "Interpreter" routine, giving it the stack segment a0 to b0 (hex).

mov R0, #0a0h

mov DPTR, #Interpreter

call Spawn

;;; Kill the main process ... it's no longer needed.

ret

The Echo process will repeatedly obtain a key and then display it.  It

might be written as follows:

Echo:

call GetKey

call DisplayKey

jmp Echo

GetKey will actually carry out the keyboard entry routine.  It will pause

until the keyboard has been flagged, and then read the key pressed and return:

GetKey:

mov R0, #SP_IE0

call Pause        ;;; Pause on external interrupt 0.

... READ THE KEY ...

ret

The Interpreter program will read commands from the serial line and then

interpret and carry them out.  It, too, will be an infinite loop:

Interpreter:

call Receive

call Execute

ret

Receive will wait for the serial line to flag a reception and then read the

command received:

Receive:

mov R0, #SP_RI

call Pause        ;;; Pause on serial receive interrupt.

... GET THE COMMAND ...

ret

This illustrates how the programmer can write routines to carry out more

than one action at a time asynchronously without having to destroy the

integrity of each process by breaking them up and merging them together into

one large Finite State Machine as is often done in these kinds of applications.

Consequently, modularity is greatly enhanced.