T h e   8 0 5 1 F A   I n t e r r u p t   D e m o

(0) INTRODUCTION

This is a diagnostic testing and demo program I've used when preparing

8051FA applications.  It comes in real handy when you need to verify that the

circuits containing the 8051FA will generate the correct interrupts on the

microprocessor.

This software uses the IO library, which is an interface to a keypad and

LCD unit.  Either you will need compatible hardware, as described in the

IO reference, or you will need to write an emulation of the necessary IO

routines, for example, to  display results and read commands over a RS-232.

For reference, these are the routines that you would need to emulate:

* scr_init

Initializes the display unit, clears it, and sets the cursor to

row 0, column 0.

* cputc

Prints out the character contained in the accumulator A.  Moves the

cursor right one column.  If the cursor was already at the last

column it's location is undefined.

* locate

Sets the cursor to the location given by the variables Row and Col.

Row should be in the range 0..3, and Col in the range 0..19.

You will also need to allocate 1 byte of space in internal data memory each for

the variable Row, and Col if you write your own IO routines.

(1) DISPLAY

The display unit used with this software is a 4 x 20 LCD unit.  The top row

(row 0) is used to display the number of interrupts received from various

sources.  These numbers are single digits listed in the following format:

Column                Interrupt Source

0                   IE0  falling edge

1                   IE1  falling edge

2                   T2EX falling edge

3                   CEX3 falling edge

4                   CEX3 rising edge

5                   CEX4 falling edge

6                   CEX4 rising edge

7                   CEX5 falling edge

8                   CEX5 rising edge

9                   CEX6 falling edge

10                   CEX6 rising edge

11                   CEX7 falling edge

12                   CEX7 rising edge

The CEX interrupt sources can distinguish both falling edges and rising edges.

When the count for any interrupt source exceeds 10, its count is reset to 0

and a 0 is displayed for it.

Rows 1, 2, and 3 illustrate the concurrent execution of the interrupt loops

in TTY fashion: from column 0, rightward to column 19, and from row 1 down to

row 3.  When the display reaches the last row and column, it wraps around back

to row 1 and column 0.

The program starts when all the interrupt loops are "Spawned" from main.

In the followiing sequence:

main:

<a> spawn TestIE0

<b> spawn TestIE1

<c> spawn TestT2EX

<d> spawn TestCCF

<e> start the interrupts

ret

The letters a, b, c, d, and e will be displayed as the corresponding points in

the program are reached.

Each time a CCF interrupt is triggered, a number: 0, 1, 2, 3, or 4 is displayed

corresponding to the interrupt source (0 = CCF0, 1 = CCF1, 2 = CCF2, 3 = CCF3,

and 4 = CCF4).

Finally, each of the loops is marked as follows:

TestIE0:

Initialize IE0

while (1) {

<f>

wait for IE0 interrupt

<g>

}

TestIE1:

Initialize IE1

while (1) {

<h>

wait for IE1 interrupt

<i>

}

TestT2EX:

Initialize T2EX

while (1) {

<j>

wait for T2EX interrupt

<k>

}

TestCCF:

Initialize CCF0, CCF1, CCF2, CCF3, CCF4

while (1) {

<l>

wait for T2EX interrupt

<m>

}

The letters f, g, h, i, j, k, l, and m will be displayed as the corresponding

point in each loop is reached.

The display will therefore will be the sequence:

a f b h c j d i e

followed by one or more of the sequences:

g f, i h, k j, 0 m l, 1 m l, 2 m l, 3 m l, or 4 m l.