WHAT IS PROTECTED MODE

An Intro

By Adam Seychell

After my release of DOS32 V1.2 a lot of people were asking for basic help

in protected mode programming. If you already know what a selector is then

there is probably no need for you to read this file.

Ok you know all about the 8086 ( or a 386 in real mode ) architecture and

what to know about this fantastic protected mode stuff. I'll start off saying

that I think real mode on the 386 is like driving a car that is stuck in

first gear. There is the potential of a lot of power but it is not being

used. It really degrades the 386 processor and was not designed to normally

operate in this mode. Even the Intel data book states  "Real mode is required

primarily to set up the processor for Protected Mode operation".

SEGMENTATION OF THE INTEL 80x86

A segment is a block of memory that starts at a fixed base address and

has a set length.  As you should already know that *every* memory reference

by the CPU requires both a SEGMENT value and a OFFSET value to be specified.

The OFFSET value is the location relative to the base address of the segment.

The SEGMENT value contains the information for the segment. I am going to

explain very basically how this SEGMENT value is interpreted by the 80386 to

give the parameters of segments.

In protected mode this SEGMENT value is interpreted completely different

than in real mode and Virtual 86 mode. The SEGMENT values are now called

"selectors". You'll see why when finished reading this file. So whenever you

load a segment register you are loading it with a selector.

The Selector is word length and contains three different fields.

Bits  0..1       Request Privilege level        ( just set this to zero )

Bit   2          Table Indicator        0 = Global Descriptor Table

1 = Local Descriptor Table

( DOS32 does not have a Local Descriptor

Table so always put a zero in here )

Bits 3..15      The INDEX field value of the selected Descriptor table

This index value points to a descriptor in the table.

The  GLOBAL DESCRIPTOR TABLE  (GDT)

The Global Descriptor Table ( GDT ) is a table of DESCRIPTORS and it's

stored in memory. The address of this table is given in a special register

called the global descriptor table register. There always must be a GDT when

in protected mode because it is in this table where all of

the segments are defined.

Each DESCRIPTOR ( stored in the GDT ) contains the complete information

about a segment. It is a description of the segment.  Each Descriptor is 64

bits long and contains many different fields. I'll explain the fields later.

The INDEX field  ( stored in bits 3..15 of any segment register ) selects a

descriptor to use for the type of segment wanted. So the only segments the

programmer can use are the available descriptors in the GDT.

Example code:

Suppose you what to access location 012345h in your data segment and

you were told that the descriptor for your data segment is descriptor

number 6 in the Global Descriptor Table.  Assume that the Global Descriptor

Table has already been set up and built for you  ( example, as in DOS32).

Solution:

We need to load a segment register (SS,DS,FS,GS,ES)  with a value so

that it will select (or index ) descriptor number 6 of the GDT. Then

reference the address with a instruction that will use this loaded

segment register.

A segment register must be loaded with the following three fields,

Request Privilege level ( Bits 0..1 ) = 0  (always)

Table Indicator ( bit 2 )  = 0

Index  ( bits 3..15 ) = 6

mov  ax,000000000110100b      ;load DS with the selector value

mov  ds,ax

mov  byte ptr DS:[ 012345h ],0     ; Using the DS segment register

The 386 has hardware for a complete multitasking system. There are

several different types of descriptors available in the GDT for managing

multitasking. You don't need to know about all the different descriptors just

to program in protected mode. Just the info above is enough.  All you need to

know to program in protected mode is what descriptors are available to you

and what are the selector values of these descriptors.  Also the base address

of the segment may be need to be known. See the file DOS32.DOC for obtaining

the selector values.

There are two groups of descriptors

1) CODE/DATA descriptors

which are used for any code and data segments.

2) SYSTEM descriptors are used for the multitasking

system of the 386. These type of descriptors will never need to be used

for programming applications.

Format of a code and data descriptor

BITS                 description if the field

----------------------------------------------------------------

0..15                SEGMENT LIMIT 0...15

16..39               SEGMENT BASE 0..23

40                  (A)   accessed bit

41..43              (TYPE) 0 = SEE BELOW

44                  (0)  0 = code/data descriptor 1 = system descriptor

45..46              (DPL) Descriptor Privilege level

47                  (P)  Segment Present bit

48..50              SEGMENT LIMIT 16..19

51..52              (AVL)   2 bits available for the OS

53                  zero for future processors

54                  Default Operation size used by code descriptors only

55                  Granularly:  1 = segment limit = limit field *1000h

0 = segment limit = limit field

56..63              SEGMENT BASE 24..31

format of TYPE field

bit  2    Executable (E)     0 = Descriptor is data type

1    Expansion Direction (ED) 0 = Expand up

1 = Expand up

0    Writeable (W)       W = 0 data segment is read only

W = 1 data segment is R/W

bit  2    Executable (E)  1 = Descriptor is code type

1    Conforming (C)   ( I don't understand )

0    Readable (R)        R = 0 Code segment execute only

R = 1 Code segment is Readable

Anything illegal will cause a general protection exception interrupt

( int 13 decimal ).

I'd better stop here, I am confusing myself. As you can see there is

more to a segment that just it's base address.

This was only meant to be a rough introduction to the protected mode

segmentation mechanism of the 80386+. I hope I did not make this sound

too complicated so that you've been put off with the whole idea of protected

mode programming. If you want to know more then I suggest you buy a book on

the 80386. The "Intel Programmers Reference guide" is the most detailed book

around.

Please note that DOS32 does *ALL* of the setting up needed for protected

mode. This includes the descriptors in the GDT. So you don't have to know

about any stupid things like the descriptor format, selector index fields,

privilege levels ,ect , ect.  What you do need to know are the selector

values for all the descriptors that are available to your program. Then the

segment registers can then be loaded with these known selector values. DOS32

uses only 3 descriptors ( or segments ) as described in DOS32.DOC.