XD (1)                    BDS C Users' Group                    XD (1)

NAME        NAME

xd - dump file in ascii, hex byte or hex word format

SYNOPSIS        SYNOPSIS

xd                             xd [-wba] filename

DESCRIPTION        DESCRIPTION

__                                                                       Xd is a program to dump a file to the console in one or more

of three formats: ascii data, hex (2 digits/byte) or word

(standard 8080 format, assuming low byte first).

The following options are available:

w - dump in word format (hex)

b - dump in byte format (hex)

a - dump as ascii

The default option is -ba.

In ascii, the following conventions are used to print special

characters:

control chars -         ^ followed by uppercase char

high bit (0x80) on -    | followed by the character in the

low 7 bits

ctl char & 0x80 on -    $ followed by uppercase character

del (0x7f) -            ^ followed by space

0xff -                  $ followed by space

CAVEATS        CAVEATS

This program was tested with v1.50 of the BDS C compiler

under CP/M 2.2.  It should work with other versions of BDS C

and CP/M, but has not been tested.

EXAMPLES        EXAMPLES

xd sample.c       dumps file "sample.c" to the console, in both

ascii & hex format

xd -w test.dat    dumps file "test.dat" to the console in hex

word format only

-1-

#g conventions are used to print special

chara