Mouse Function Calls

Complete set of interrupt functions and subfunctions.

Most of this information is retrieved from the shareware utility "HelpPC",

by David Jurgens (CIS 71270,2422).

Collected and uploaded to CIS/MSLANG by:

Ernest Vogelsinger, CIS PPN 100015,551.

03/04/93

----------------------------------------------------------------------

INT 33 - Mouse Function Calls

For more information see the following topics:

INT 33,0   Mouse Reset/Get Mouse Installed Flag

INT 33,1   Show Mouse Cursor

INT 33,2   Hide Mouse Cursor

INT 33,3   Get Mouse Position and Button Status

INT 33,4   Set Mouse Cursor Position

INT 33,5   Get Mouse Button Press Information

INT 33,6   Get Mouse Button Release Information

INT 33,7   Set Mouse Horizontal Min/Max Position

INT 33,8   Set Mouse Vertical Min/Max Position

INT 33,9   Set Mouse Graphics Cursor

INT 33,A   Set Mouse Text Cursor

INT 33,B   Read Mouse Motion Counters

INT 33,C   Set Mouse User Defined Subroutine and Input Mask

INT 33,D   Mouse Light Pen Emulation On

INT 33,E   Mouse Light Pen Emulation Off

INT 33,F   Set Mouse Mickey Pixel Ratio

INT 33,10  Mouse Conditional OFF

INT 33,13  Set Mouse Double Speed Threshold

INT 33,14  Swap interrupt subroutines

INT 33,15  Get mouse driver state and memory requirements

INT 33,16  Save mouse driver state

INT 33,17  Restore mouse driver state

INT 33,18  Set alternate subroutine call mask and address

INT 33,19  Get user alternate interrupt address

INT 33,1A  Set mouse sensitivity

INT 33,1B  Get mouse sensitivity

INT 33,1C  Set mouse interrupt rate (InPort only)

INT 33,1D  Set mouse CRT page

INT 33,1E  Get mouse CRT page

INT 33,1F  Disable mouse driver

INT 33,20  Enable mouse driver

INT 33,21  Reset mouse software

INT 33,22  Set language for messages

INT 33,23  Get language number

INT 33,24  Get driver version, mouse type & IRQ number

- function is specified in AX

- a mickey is 1/200 inches

- for additional information see your vendor documentation

- function number occupies all of AX rather than AH

Mouse functions can be broken down into the following classes:

Mouse Driver Control / Feedback Functions

INT 33,0   Mouse Reset/Get Mouse Installed Flag

INT 33,15  Get Mouse Driver State and Memory Requirements

INT 33,16  Save Mouse Driver State

INT 33,17  Restore Mouse Driver State

INT 33,1C  Set Mouse Interrupt Rate (InPort only)

INT 33,1F  Disable Mouse Driver

INT 33,20  Enable Mouse Driver

INT 33,21  Reset Mouse Software

INT 33,24  Get Driver Version, Mouse Type & IRQ Number

Mouse Cursor Control Functions

INT 33,1   Show Mouse Cursor

INT 33,2   Hide Mouse Cursor

INT 33,4   Set Mouse Cursor Position

INT 33,7   Set Mouse Horizontal Min/Max Position

INT 33,8   Set Mouse Vertical Min/Max Position

INT 33,9   Set Mouse Graphics Cursor

INT 33,A   Set Mouse Text Cursor

INT 33,F   Set Mouse Mickey Pixel Ratio

INT 33,10  Mouse Conditional OFF

INT 33,13  Set Mouse Double Speed Threshold

INT 33,1A  Set Mouse Sensitivity

INT 33,1B  Get Mouse Sensitivity

Mouse Button and Position Feedback Functions

INT 33,3   Get Mouse Position and Button Status

INT 33,5   Get Mouse Button Press Information

INT 33,6   Get Mouse Button Release Information

INT 33,B   Read Mouse Motion Counters

Video Control and Feedback Functions

INT 33,1D  Set Mouse CRT Page

INT 33,1E  Get Mouse CRT Page

Mouse Interrupt Setup Functions

INT 33,C   Set Mouse User Defined Subroutine and Input Mask

INT 33,14  Swap Interrupt Subroutines

Alternate Mouse Interrupt Setup Functions

INT 33,18  Set Alternate Subroutine Call Mask and Address

INT 33,19  Get User Alternate Interrupt Address

Light Pen Emulation Functions

INT 33,D   Mouse Light Pen Emulation On

INT 33,E   Mouse Light Pen Emulation Off

International Language Support Functions

INT 33,22  Set Language for Messages

INT 33,23  Get Language Number

INT 33,0 - Mouse Reset/Get Mouse Installed Flag

AX = 00

on return:

AX = 0000  mouse driver not installed

FFFF  mouse driver installed

BX = number of buttons

- resets mouse to default driver values:

.  mouse is positioned to screen center

.  mouse cursor is reset and hidden

.  no interrupts are enabled (mask = 0)

.  double speed threshold set to 64 mickeys per second

.  horizontal mickey to pixel ratio (8 to 8)

.  vertical mickey to pixel ratio (16 to 8)

.  max width and height are set to maximum for video mode

INT 33,1 - Show Mouse Cursor

AX = 01

returns nothing

- increments the cursor flag;  the cursor is displayed if flag

is zero;  default flag value is -1

INT 33,2 - Hide Mouse Cursor

AX = 02

returns nothing

- decrements cursor flag; hides cursor if flag is not zero

INT 33,3 - Get Mouse Position and Button Status

AX = 03

on return:

CX = horizontal (X) position  (0..639)

DX = vertical (Y) position  (0..199)

BX = button status:

�F-8�7�6�5�4�3�2�1�0�  Button Status

�  � � � � � � � ����� left button (1 = pressed)

�  � � � � � � ������ right button (1 = pressed)

�������������������� unused

- values returned in CX, DX are the same regardless of video mode

INT 33,4 - Set Mouse Cursor Position

AX = 4

CX = horizontal position

DX = vertical position

returns nothing

- default cursor position is at the screen center

- the position must be within the range of the current video mode

- the position may be rounded to fit screen mode resolution

INT 33,5 - Get Mouse Button Press Information

AX = 5

BX = 0  left button

1  right button

on return:

BX = count of button presses (0-32767), set to zero after call

CX = horizontal position at last press

DX = vertical position at last press

AX = status:

�F-8�7�6�5�4�3�2�1�0�  Button Status

�  � � � � � � � ����� left button (1 = pressed)

�  � � � � � � ������ right button (1 = pressed)

�������������������� unused

INT 33,6 - Get Mouse Button Release Information

AX = 6

BX = 0  left button

1  right button

on return:

BX = count of button releases (0-32767), set to zero after call

CX = horizontal position at last release

DX = vertical position at last release

AX = status

�F-8�7�6�5�4�3�2�1�0�  Button status

�  � � � � � � � ����� left button (1 = pressed)

�  � � � � � � ������ right button (1 = pressed)

�������������������� unused

INT 33,7 - Set Mouse Horizontal Min/Max Position

AX = 7

CX = minimum horizontal position

DX = maximum horizontal position

returns nothing

- restricts mouse horizontal movement to window

- if min value is greater than max value they are swapped

INT 33,8 - Set Mouse Vertical Min/Max Position

AX = 8

CX = minimum vertical position

DX = maximum vertical position

returns nothing

- restricts mouse vertical movement to window

- if min value is greater than max value they are swapped

INT 33,9 - Set Mouse Graphics Cursor

AX = 9

BX = horizontal hot spot (-16 to 16)

CX = vertical hot spot (-16 to 16)

ES:DX = pointer to screen and cursor masks (16 byte bitmap)

returns nothing

- screen mask is AND'ed to screen Cursor Mask is XOR'ed

- bytes 0-7 form the screen mask bitmap

- bytes 8-F form the cursor mask bitmap

INT 33,A - Set Mouse Text Cursor

AX = 0A

BX = 00  software cursor

01  hardware cursor

CX = start of screen mask or hardware cursor scan line

DX = end of screen mask or hardware cursor scan line

returns nothing

INT 33,B - Read Mouse Motion Counters

AX = 0B

on return:

CX = horizontal mickey count (-32768 to 32767)

DX = vertical mickey count (-32768 to 32767)

- count values are 1/200 inch intervals (1/200 in. = 1 mickey)

INT 33,C - Set Mouse User Defined Subroutine and Input Mask

AX = 0C

ES:DX = far pointer to user interrupt

CX = user interrupt mask:

�F-5�4�3�2�1�0� user interrupt mask in CX

�  � � � � ���� cursor position changed

�  � � � ����� left button pressed

�  � � ������ left button released

�  � ������� right button pressed

�  �������� right button released

���������� unused

returns nothing

- routine at ES:DX is called if an event occurs and the

corresponding bit specified in user mask is set

- routine at ES:DX receives parameters in the following

registers:

AX = condition mask causing call

CX = horizontal cursor position

DX = vertical cursor position

DI = horizontal counts

SI = vertical counts

DS = mouse driver data segment

BX = button state:

�F-2�1�0�

�  � ���� left button (1 = pressed)

�  ����� right button (1 = pressed)

������� unused

- initial call mask and user routine should be restore on exit

from user program

- user program may need to set DS to its own segment

- user routine should return via RETF instead of IRET

- see   INT 33,14

INT 33,D - Mouse Light Pen Emulation On

AX = 0D

returns nothing

- turns on light pen emulation;  the light pen is considered

down when both buttons are down;   when both buttons are

up the pen is considered off screen

- see   INT 33,E

INT 33,E - Mouse Light Pen Emulation Off

AX = 0E

returns nothing

- disables light pen emulation

- see   INT 33,D

INT 33,F - Set Mouse Mickey Pixel Ratio

AX = 0F

CX = horizontal ratio (1..32767, default 8)

DX = vertical ratio (1..32767, default 16)

returns nothing

- sets the ratio between physical cursor movement (mickeys) and

screen coordinate changes

- CX and DX must be unsigned (high bit must be 0)

INT 33,10 - Mouse Conditional OFF

AX = 10h

CX = upper X screen coordinate

DX = upper Y screen coordinate

SI = lower X screen coordinate

DI = lower Y screen coordinate

returns nothing

- defines screen region for updating in which the mouse is hidden

if found these coordinates (INT 33,1 must be used to turn cursor

on again)

- see  INT 33,1

INT 33,13 - Set Mouse Double Speed Threshold

AX = 13h

DX = threshold speed (mickeys per second, default 64)

returns nothing

- cursor speed is doubled when the cursor moves across the screen

at the threshold speed

INT 33,14 - Swap Interrupt Subroutines

AX = 14h

ES:DX = far pointer to user routine

CX = user interrupt mask:

�F-8�7�6�5�4�3�2�1�0� user interrupt mask in CX

�  � � � � � � � ���� cursor position changed

�  � � � � � � ����� left button pressed

�  � � � � � ������ left button released

�  � � � � ������� right button pressed

�  � � � �������� right button released

���������������� unused

on return:

CX = previous user interrupt mask

ES:DX = far pointer to previous user interrupt

- routine at ES:DX is called if an event occurs and the

corresponding bit specified in user mask is set

- routine at ES:DX receives parameters in the following

registers:

AX = condition mask causing call

CX = horizontal cursor position

DX = vertical cursor position

DI = horizontal counts

SI = vertical counts

DS = mouse driver data segment

BX = button state:

�F-2�1�0�

�  � ���� left button (1 = pressed)

�  ����� right button (1 = pressed)

������� unused

- initial call mask and user routine should be restore on exit

from user program

- user program may need to set DS to its own segment

- see   INT 33,C

INT 33,15 - Get Mouse Driver State and Memory Requirements

AX = 15h

on return

BX = buffer size need to hold current mouse state

- used before mouse functions 16h and 17h to determine memory

needed to save mouse state before giving up control of mouse

to another program

INT 33,16 - Save Mouse Driver State

AX = 16h

ES:DX = far pointer to mouse state save buffer

returns nothing

- used to save mouse information before relinquishing control

to another programs mouse handler

- see INT 33,15   INT 33,17

INT 33,17 - Restore Mouse Driver State

AX = 17h

ES:DX = far pointer to mouse state save buffer

returns nothing

- used to restore mouse information after regaining control

from another programs mouse handler

- see INT 33,15   INT 33,16

INT 33,18 - Set alternate subroutine call mask and address

AX = 18h

DX = offset to function

CX = user interrupt mask:

�F-8�7�6�5�4�3�2�1�0� user interrupt mask in CX

�  � � � � � � � ���� alt key pressed during event

�  � � � � � � ����� ctrl key pressed during event

�  � � � � � ������ shift key pressed during event

�  � � � � ������� right button up event

�  � � � �������� right button down event

�  � � ��������� left button up event

�  � ���������� left button down event

�  ����������� cursor moved

������������� unused

- entire mask is set to zero when INT 33,0 is called

- up to three handlers may be defined with this call

- mask should be cleared before program exit

- when handler is called it receives parameters in the following

registers:

AX = condition mask causing call

CX = horizontal cursor position

DX = vertical cursor position

DI = horizontal counts

SI = vertical counts

DS = mouse driver data segment

BX = button state:

�F-2�1�0�

�  � ���� left button (1 = pressed)

�  ����� right button (1 = pressed)

������� unused

INT 33,19 - Get User Alternate Interrupt Address

AX = 19h

CX = user interrupt call mask (see below)

on return:

BX:DX = user interrupt vector

CX = user interrupt call mask or zero if not found

�F-8�7�6�5�4�3�2�1�0� user interrupt mask in CX

�  � � � � � � � ���� alt key pressed during event

�  � � � � � � ����� ctrl key pressed during event

�  � � � � � ������ shift key pressed during event

�  � � � � ������� right button up event

�  � � � �������� right button down event

�  � � ��������� left button up event

�  � ���������� left button down event

�  ����������� cursor moved

������������� unused

- returns vector to function defined by INT 33,18

- searches the event handlers defined by INT 33,18 for a routine

with a call mask matching CX

INT 33,1A - Set Mouse Sensitivity

AX = 1A

BX = horizontal coordinates per pixel  (� 100)

CX = vertical coordinates per pixel  (� 100)

DX = double speed threshold

returns nothing

- sets mouse sensitivity by setting the ratio of the mouse

coordinates per screen pixel

- provides same results as calls to both INT 33,F and INT 33,13

- these values are not reset by INT 33,0

INT 33,1B - Get Mouse Sensitivity

AX = 1B

on return:

BX = horizontal coordinates per pixel  (� 100)

CX = vertical coordinates per pixel  (� 100)

DX = double speed threshold

- returns mouse sensitivity information as the number of mouse

coordinates per screen pixel

INT 33,1C - Set Mouse Interrupt Rate  (InPort only)

AX = 1C

BX = rate code

= 0  no interrupts

= 1  30 interrupts per second

= 2  50 interrupts per second

= 3  100 interrupts per second

= 4  200 interrupts per second

- work with the InPort mouse only

- sets the rate the mouse status is polled by the mouse driver

- faster rates provide better resolution but take away CPU time

- values in BX > 4 can cause unpredicatable results

INT 33,1D - Set Mouse CRT Page

AX = 1D

BX = CRT page number

returns nothing

- sets the CRT page which the mouse cursor is displayed

- see VIDEO PAGES

INT 33,1E - Get Mouse CRT Page

AX = 1E

on return:

BX = CRT page number cursor is displayed on

- see VIDEO PAGES

INT 33,1F - Disable Mouse Driver

AX = 1F

on return:

AX = 001F if successful

FFFF if error

ES:BX = previous INT 33 vector

- restores vectors for INT 10 and INT 71 (8088/86) or INT 74 (286+)

- INT 33 interrupt vector (software) is not affected

- use of the vector returned in ES:BX to restore the previous INT 33

vector can cause problems since it contains the value of INT 33

before the driver was installed; any other hooks into INT 33 set

after driver installation will not receive service

INT 33,20 - Enable Mouse Driver

AX = 20h

returns nothing

- reinstalls the mouse drivers interrupt vectors for INT 10 and

INT 71 (8088/86) and INT 74 (286/386)

- see INT 33,1F

INT 33,21 - Reset Mouse Software

AX = 21h

on return:

AX = 0021  mouse driver not installed

FFFF  mouse driver installed

BX = 2  mouse driver installed

- similar to INT 33,0 but does not reset the mouse hardware or

display variables

- both AX and BX must be correct for successful reset

- interrupt routines set through INT 33,18 are preserved

- resets the mouse to the following defaults:

.  mouse is positioned to screen center

.  mouse cursor is reset and hidden

.  no interrupts are enabled (mask = 0)

.  double speed threshold set to 64 mickeys per second

.  horizontal mickey to pixel ratio (8 to 8)

.  vertical mickey to pixel ratio (16 to 8)

.  max width and height are set to maximum for video mode

INT 33,22 - Set Language for Messages

AX = 22h

BX = language number (with /L switch value):

= 0  English     n/a

= 1  French      F

= 2  Dutch       NL

= 3  German      D

= 4  Swedish     S

= 5  Finnish     SF

= 6  Spanish     E

= 7  Portuguese  P

= 8  Italian     I

returns nothing

- only works with international version of the mouse driver

- see   INT 33,23

INT 33,23 - Get Language Number

AX = 23h

on return:

BX = language number (with /L switch value):

= 0  English     n/a

= 1  French      F

= 2  Dutch       NL

= 3  German      D

= 4  Swedish     S

= 5  Finnish     SF

= 6  Spanish     E

= 7  Portuguese  P

= 8  Italian     I

- only works with international version of the mouse driver

- returns English (0) if not international version

- see   INT 33,22

INT 33,24 - Get Driver Version, Mouse Type & IRQ Number

AX = 24h

on return:

BH = major version (see below)

BL = minor version (see below)

CH = mouse type:

= 1  bus mouse

= 2  serial mouse

= 3  InPort mouse

= 4  PS/2  mouse

= 5  Hewlett Packard mouse

CL = IRQ number:

= 0  PS/2

= 2  IRQ 2

= 5  IRQ 5

= 7  IRQ 7

- version 6.1 would be represented as BH = 06h, BL = 10h