MicroNet PC Command Summary

(0) Introduction

A PC-based client has been provided for MicroNet.  This is located in

the files MicroNet.*.  Communications can be established over COM1, COM2,

COM3, or COM4.  The software is also RS-485 compatible, and assumed that the

OUT1 bit of the Moden Control Register is being used as the transmit enable.

(1) Command Summary

These are the commands and variables implemented by MicroNet.c:

* int GetStats(int Counter, byte *Buf, int Bytes);

Queries the counter indicated (Counter) for statistica data.

Results are stored in the buffer indicated (Buf), and Bytes should be set

to the size of the Buf.  Either a -1 is returned, if the command failed, or

the number of bytes actually received is returned, with the data itself

stored in Buf.  The buffer will never be filled with more bytes than is

indicated by Bytes.

* int GetDump(int Counter, byte *Buf, int Bytes);

Similar to GetStats command, except the information stored is the actual

set of nutation times from the counter indicated.

* int SendStatus(int Unit);

Queries the microprocessor indicated for its current status.  A '0', '1', or

'2' should be returned indicating the status of the micro as in the section

containing the command summary.  The variable Unit can be set to 0 for

microprocessor A, or 1 for B.

* int Status;

The status of the serial port.  It has the following format:

Bit 7: Bad checksum ... can occur in response to a data transfer failure

in either the STATS or DUMP commands, when checksums fail to match.

This indicates that the Physical Layer is not functioning free of

external noise.

Bit 6: Bad header ... a similar protocol error relating to the reception of

an invalid header (i.e. '#', ':', or '.').  This has the same basic

cause as a bad checksum.

Bit 5: Packet overflow ... indicates that data transmission was too long

to be accomodated by the data buffer (Buf) provided in either the

GetStats or GetDump routine.

Bit 4: Timed out ... indicates that a microprocessor failed to respond

when it was supposed to.

Bit 3: Framing Error

Bit 2: Parity Error

Bit 1: Overrun Error ... 3 low-level communications errors directly

related to failures in the low-level protocol and in the Physical

Layer.  This can be caused by external noise on the serial link, or

by two or more processors trying to transmit at once.

Bit 0: Buffer Full ... indicates that the internal data buffer of the

port indicated has become full.  This is invariably the result of

a microprocessor that refused to stop transmitting.

* void SendAbort(int Addr);

* void SendTest(int Addr);

Sends a abort/test command to the destination indicated.  The meanings of

Addr as the following:

Addr         Meaning

0         Send the command to A.

1         Send the command to B.

2         Send the command to both A and B.

The SendAbort command will also erase any prior communication errors

by clearing out the internal status flags.