T h e   I O   k e y p a d / L C D   I n t e r f a c e

(0) OVERVIEW

I often use a keypad/LCD unit with my 8051 stand-alone board to provide

an I/O interface.  This unit is configured in a specific way, and the io.lib

software assumes this specific configuration.

(1) HARDWARE INTERFACE

(a) Memory Map

The io.lib module assumes that the following registers are mapped to

EXTERNAL DATA memory (XBY memory, as referred to in the debugger) as

illustrated below:

Register                  Address

LR (LCD REGISTER)           e080

LD (LCD DATA)               e081

LC (LCD CONTROL)            e090

KD (KEYPAD DATA)            e0a0

It assumes that the keypad input has an associated strobe which will trigger the

EXTERNAL INTERRUPT 1 pin (INT1) when INT1 is set in falling edge mode.

(b) Keypad

The keypad is assumed to have 16 keys on it.  When INT1 becomes active, the

lower order 4 bits of the KD register will contain the numeric code of the key

that was pressed,

(c) Liquid Crystal Display (LCD)

The LCD is assumed to be 4 rows by 20 columns.  The registers LR and LD are

used to sending commands, and sending data, respectively.  The strobe register,

LS, is used to latch data to or from either of these registers.

To write to LC or LR, write to its memory-mapped address, and then toggle

the strobe, LS.  For example:

WriteLR:

mov DPTR, #0e080h  ;;; Write A to LR

movx @DPTR, A

mov DPTR, #0e090h  ;;; Toggle LS.

mov A, #1

movx @DPTR, A

mov A, #0

movx @DPTR, A

ret

WriteLD:

mov DPTR, #0e081h ;;; Write A to LD.

movx @DPTR, A

mov DPTR, #0e090h ;;; Toggle LS.

mov A, #1

movx @DPTR, A

mov A, #0

movx @DPTR, A

ret

ReadLC:

mov DPTR, #0e080h  ;;; Clear out lC.

movx A, @DPTR

mov DPTR, #0e090h  ;;; Set LS to enable reading.

movx @DPTR, #1

mov DPTR, #0e080h  ;;; Read LC.

movx A, @DPTR

mov DPTR, #0e090h  ;;; Clear LS to complete the reading.

movx @DPTR, #0

ret

ReadLD:

mov DPTR, #0e081h  ;;; Clear out LD.

movx A, @DPTR

mov DPTR, #0e090h  ;;; Set LS.

movx @DPTR, #1

mov DPTR, #0e081h  ;;; Read LD.

movx A, @DPTR

mov DPTR, #0e090h  ;;; Clear LS.

movx @DPTR, #0

ret

The actual command codes used for the LCD will depend largely on the unit

itself, and have mostly been "encapsulated" in the IO interface software.  You

will need to make the appropriate changes to the IO routines for your LCD unit.

(2) IO FUNCTIONS

The following is a description of the functions contained in the IO

interface.

(a) Keypad routines

key_init ... Initialize the keypad, set up external interrupt 1.

cgetc ...... Read the keypad data into A.

(b) LCD routines

scr_init ... Initialize the display, and clear it.

clear ...... Clear the display and set the cursor to row 0, column 0

home ....... Set the cursor to row 0, column 0.

set_cursor ... Set the cursor to the mode indicated in the register A.

A can be a combination of any of the following values:

DISPLAY_ON:   Turn the display on

CURSOR_ON:    Turn on the cursor.

CURSOR_BLINK: Make the cursor blink.

cursor_leftr, cursor_right ... to move the cursor

shift-left,   shift_right .... to move the display itself.

locate ...... To place the cursor at the point indicated by the

variables Row and Col.

save_cursor, restore_cursor:

So save and restore the more recent cursor

position

cputc ....... Print the character in A out to the display in TTY manner.

cputs ....... Print out an entire string, pointed to by DPTR, to the

display unit.

(c) Variables

The io.lib software uses 6 bytes of space for variables.  They have been

named as follows:

Row, Col:       The current cursor position

OldRow, OldCol: The most recent cursor position saved by the

last call to save_cursor.

Counter: A 2-byte counter.