Some notes about screen dumps

Written by Michael Day 25 July 1988

First, display screens are less accurate than printers.  The

sizes  can  vary even when the screen is specified  as  the  same

size. Further, the adjustments of the screen can affect size  and

aspect ratio.

A  standard 12 inch monitor will provide a display  area  of

around 20cm by 15cm (or 8" by 6"). This provides an aspect  ratio

of 4:3. In order for the printout to look the same as the screen,

it is important that the aspect ratio on the printer be the  same

as the screen.

Although  the  overall screen aspect  ratio  normally  stays

around  4:3, the pixel aspect ratio can change depending  on  the

mode  selected.  As  an example, consider a  CGA  color  graphics

screen  which displays 320x200 pixels. This means that there  are

40  pixels  per  inch horizontally, and  33.33  pixels  per  inch

vertically.  This  gives  the pixels an aspect  ratio  of  2.5:3.

Meaning that if we were to magnify a single pixel large enough it

would be 2.5 inches wide by 3 inches tall.

When we switch the screen to the monochrome graphics mode we

get  640x200  pixels  on the screen. This means  that  the  pixel

aspect  ratio is now 1.25:3. The pixel has shrunk in half in  the

horizontal  direction,  but  remained the same  in  the  vertical

direction.

The pixel aspect ratio is also affected when other modes  or

displays  are used, such as 640x350 for EGA, 640x480 for VGA,  or

720x348 for Hercules.

Note:  When we say that the pixel aspect ratio is 2.5:3, that  is

describing the area allocated on the screen to the pixel, and not

the actual spot size of the electron beam. Case in point, on  the

CGA display, the pixel definition is crowded so close together in

the horizontal direction that on most displays the dots  overlap,

but  in  the  vertical direction the spot size is  too  small  to

completely fill in the area between the dots. On the VGA  display

this is corrected by double scanning (scanning two display  lines

for  every CGA line). This effectively doubles the spot  size  in

the vertical direction to fill in the gaps between the CGA lines.

The  spot  size  on  a display is  usually  referred  to  as

"pitch", and the allocated display area is usually referred to as

a "pixel". The pitch size is a function of the monitor. The pixel

size is a function of the display card.

The printer:

On  the  printer  we have similar  problems.  The  printer's

aspect  ratio  is usually not the same as  the  display's  aspect

ratio, so some correction must be done if proper aspect ratio  is

to  be  maintained. Without proper aspect ratio, circles  on  the

display will come out as ovals on the printer.

The  Epson  printer  is  the most  common  printer  used  in

conjunction  with the PC. This is because it is the printer  that

IBM  selected  to provide with their computer.

In it's "normal" graphics mode the Epson printer will  print

60  dots per inch horizontally, and 72 dots per inch  vertically.

The resolution of 72 dots per inch vertically was chosen  because

character size is measured in "points" and a point is defined  as

being  1/72  inch.

The  position of the wires in the print head  determine  the

vertical resolution, and since they are fixed at 1/72 inch apart,

the vertical resolution is fixed at 1/72 inch.

It is possible to simulate a higher resolution by moving the

paper slightly and printing a second pass over the line that  was

just printed. The Epson provides for this with the ESC 3  command

which  causes the printer to advance the paper by  the  specified

n/216 inch increment when a line feed is received. However,  this

means that three passes for each line must be used to obtain this

resolution, which slows printing down by a factor of three.  Thus

it is not often used.

Another  alternative is to use a 24 pin printer  which  uses

three rows of 8 pins to achieve the same effect on a single pass.

In  the  horizontal  direction the  printer  resolution  can

change depending on the mode selected. In normal graphics mode on

the Epson this is set to 60 dots per inch. The reason behind this

is quite simple, the normal resolution of a character printed  in

10  pitch  on the Epson is 6 dots horizontally  (discounting  the

half  dot fudging Epson uses to boost apparent  resolution).  The

paper  used  is  usually eight inches wide,  and  to  fit  eighty

characters onto the line at 6 dots per character area you end  up

with 480 dots on the line, or 60 dots per inch.

Six  (five active pixels plus one separator) is the  minimum

number of pixels wide that can be used to fully represent all the

ASCII characters.

As  was  mentioned,  the Epson has a half  dot  fudge  which

allows  it  to put twice as many dots on the same line.  It  does

this by simply moving the head only half as far to print the next

dot.  By selecting double density mode you can get 120  dots  per

inch  which  gives you a resolution of 1/120 inch.  This  doesn't

come  free. In order to print twice as many dots on a  line,  the

printer has to print twice as many dots. That means that it  will

be slower.

The reason it is slower is because it takes a finite  period

of time to move the print wires in the head. Enough time must  be

allowed  for  the wires to retract in preparation  for  the  next

print.  The 'fudge' in character mode printing is that  the  dots

are  specified  in such a way that there are no  two  consecutive

half  dots  in  a row, thus giving enough time for  the  wire  to

retract  without  having to slow the printer down. But  since  in

graphics mode the printer can't know ahead of time whether  there

will be any consecutive half dots, it must assume that there will

be if you select the double density mode.

Selecting double density mode does not mean that the printer

will slow down to half print speed, there are other factors  that

come into play that control this, including head return time, and

line feed time.

In later model printers there is also an added super density

mode  which allows four dots to be printed in the space  of  one.

And  on some 24 pin printers there is even a "hex" density  mode,

allowing  for 360 dots per inch resolution. The  printer  handles

this by adding some intelligence in the printing. If the previous

dot was printed then it will not print the next one even if  told

to. That way there is enough time for the wire to be retracted in

preparation  for  the following dot. This allows the  printer  to

print  240 dots per inch resolution or more while running at  120

dots per inch speed. Since most programs try to be compatible  to

the  original  Epson  MX-80 printer, the super  density  mode  is

seldom used.

Printer aspect ratio:

As was previously mentioned, the Epson printer does not have

the  same  aspect ratio as the display, though there  are  tricks

that can be used to restore the over all image aspect ratio. This

is  performed  by printing additional dots at  certain  times  to

change  the apparent pixel aspect ratio of the printer. With  the

exception  of  the CGA display, the translation  is  usually  not

perfect.

There  are  two ways in which the display  can  be  printed.

normal  'upright'  mode, in which the display's  horizontal  axis

appears  on the printer's horizontal axis. And  'landscape'  mode

where the display is printed out sideways.

A normal upright printout will provide an image that is  the

same   size  and  orientation  as  the  display,  but   has   the

disadvantage  of  requiring a lot of fiddling with  the  dots  to

produce the printout.

The  landscape  (or sideways) printout does not  require  as

much  fiddling, but causes the picture to grow slightly  in  size

(8.9"  x  6.7" instead of 8" x 6"). A real big advantage is  with

the  CGA display mode. When printing a CGA display  sideways  all

that needs to be done is to double the display's horizontal  scan

lines  on  the  printer for 640x200 mode, or double  it  in  both

directions  in 320x200 mode. The printout will come out with  the

proper aspect ratio.

The  CGA  display could be printed upright without  any  bit

twiddling  at all, but the aspect ratio will be way off, and  the

picture  would  be smaller. By bit twiddling it  is  possible  to

restore the picture to the same size as the display (8"x6"),  but

there will be some distortion at the pixel level of the printout.

With Hercules, EGA, and VGA type displays it is normally not

possible to perform a screen dump without some distortion.

If  it is important that the picture fit the  8"x6"  display

format  then the upright display method should be used,  but  you

should  realize that there will be significant distortion at  the

pixel level to keep the aspect ratio the same.

If  minimal distortion at the pixel level is  desired,  then

the landscape method should be used. The primary disadvantage  is

that the picture size will grow slightly (8.9"x6.7") in order  to

achieve  the  reduced  distortion. Also note  that  the  Hercules

display will grow even further, expanding to 10"x7.5" to  achieve

the reduced pixel distortion.

While it is possible to achieve the same display size in the

landscape mode, there is no reason to do so since it would end up

being  even  more distorted at the pixel level than  the  upright

mode.

The amount of pixel level distortion is a trade off of speed

verses  distortion. It is possible to achieve minimal  distortion

by selecting the super resolution mode and using six print passes

per line. However that will cause the print time to be  increased

by  a  factor of six. This is seldom acceptable  given  that  the

print time in normal mode is too slow as it is.

One final note on speed. The print time for a given mode  is

based entirely on how much data there is to print. The larger the

picture, the longer it will take to print it.

Scaling distortion:

When  scaling occurs, there will be trade-offs as a  result.

And since in the normal course of events when doing a screen dump

scaling  will  occur,  you will normally  be  encountering  these

trade-offs.   The trade-offs cause distortion of the  picture  in

one form or another.

The two main forms of distortion are aspect ratio, and pixel

distortion.  When  scaling  down  there  is  also  an  additional

degradation caused by screen pixel data being lost.

Aspect  ratio distortion is where the overall picture  ratio

on  the printer does not match the ratio provided on the  screen.

Sometimes this is intentional, but usually it is not desired. For

engineering  graphs,  aspect  ratio errors greater  than  2%  are

considered unacceptable. For business charts aspect ratio  errors

greater than 5% begin to become unacceptable. In all cases aspect

ratio errors greater than 10% usually are considered unacceptable

as the picture will be noticeably distorted.

There  are  times when it might be desirable to  exceed  the

aspect ratio error factor. This might be done if the pixel  level

distortion is considered unacceptable.

Pixel  level distortion is most noticeable in the  printer's

Y  (vertical)  direction.   This   is  because  each   distortion

increment   is  1/72 inch. This is readily noticeable.  Sometimes

for  the  sake of clarity it is desirable to  forgo  the  overall

aspect ratio in order to obtain a minimal pixel level distortion.

There  is  no  easy  way to determine what  is  best,  it  is  an

individual thing.

Pixel level distortion is caused by the inability to  evenly

divide  the  screen pixels into the same number  or  multiple  of

pixels on the printer. The usual end result is that  occasionally

there  will be additional pixels added in the picture  causing  a

slight  pixel  level distortion to maintain the  overall  picture

aspect  ratio.  By  slightly changing the  aspect  ratio  of  the

printout you can sometimes eliminate the distortion, or at  least

cause  the  distortion  to  occur in  a  pattern  which  is  more

acceptable.

When scaling up (expanding) a screen picture the  distortion

can  be  noticeable,  but no data is lost, it  is  just  slightly

distorted  due to the added pixels. When scaling  down  decisions

must  be made by the program on what data to retain and  what  to

discard. This is usually handled on a priority basis. The  colors

are each assigned a priority. When two or more screen pixels must

be  placed in the position of one printer pixel then  the  screen

pixel  with the highest priority color will take precedence,  and

the rest will be discarded.

################################################################################################################################