ARCDIAG

ARCNET DIAGNOSTIC PROGRAM

USER GUIDE

Version 2.5

For use with

Novell Netware/SM/RX Type Networks

(c) Copyright 1986, 1989, 1990, 1992

by

Olympic Software Co

8424 Warner Rd

Saline, MI  48176

(313) 996-5750

All Rights Reserved Worldwide

The information in this document is subject to change without notice and

should not be construed as a commitment by Olympic Software Co.  Olympic

Software Co. assumes no responsibility for any errors that may appear in

this document.

The software described in this document is furnished under license and may

only be used or copied in accordance with the terms of such license.

COPYRIGHT NOTICE AND LICENSE

This  document; other  accompanying  written and  disk-based  notes  and

specifications; and all referenced and related program files, demonstration

code and  object modules  accompanying  this  document are  copyrighted by

Olympic Software.   The copyright owner hereby  licenses you to:  use the

software; make as many copies of the software and documentation as you wish;

give exact copies of the original to anyone; and distribute the software and

documentation in its unmodified form via electronic means. There is no charge

for any of the above.

This is not free software.  This license allows you to use this software

without charge  for a period of 30 days.  In order to include this software

as part of any product, either commercial, shareware, freeware, or public

domain, registration is required. Any such use of the Arcdiag product is in

violation of federal copyright laws and will be prosecuted.

No copy  of the  software may  be distributed  or given  away without  this

accompanying documentation; this notice must not be removed.

DISCLAIMER OF ALL WARRANTIES

There is no warranty of any kind associated with this software, and the

copyright owner is not liable for damages of any kind.  By using this

software, you agree to this.  Every effort has been made by Olympic Software

to make this product bug-free.   However, the nature of software development

is that it is impossible to guarantee bug-free software.

REGISTRATION

Upon receipt of the appropriate registration fee,  Olympic Software will send

the user a disk containing a version of Arcdiag without the shareware

registration screen,  and entitle him to full telephone support.

BULLETIN BOARDS, USERS GROUPS AND DISK VENDORS

No Bulletin Board, User Group or Disk Vendor that charges any fee for access

or disks may distribute Arcdiag without completing the application form

below.  Free, public access bulleting boards may distribute Arcdiag with no

written permission, but are encouraged to send the application form, in order

to receive the latest version of the product.

Operators of Bulletin Boards may receive an official copy of Arcdiag, its

documentation, and all related materials in archived form, by completing the

application form below, and returning it to Olympic Software. You must

provide free access to your BBS for you to receive this disk. This will also

entitle you to free upgrades of the product.

Non-Profit Users Groups may distribute unregistered copies of Arcdiag, as

long as the group charges no more than $10 per disk.  To receive 'official'

master copies of Arcdiag and automatic updates, please fill in and return

the application form below.  You should add Olympic Software to your group's

mailing list, and send a copy of your group's newsletter (if applicable).

For-Profit disk vendors may distribute unregistered copies of Arcdiag,

providing they return the application form below, pay a $10 fee for the

master disk, shipping and handling, AND THE VENDOR MUST HAVE ATTAINED

ASP-APPROVED STATUS.

Olympic Software must be placed on the vendor's mailing list, and receive

all promotional and advertising materials with the application.

UNDER NO CIRCUMSTANCES MAY ARCDIAG BE DISTRIBUTED WITHOUT THIS COMPLETE

DOCUMENTATION FILE, ALL PROGRAMS, AND OTHER MATERIALS PROVIDED WITH THE

OFFICIAL COPIES FROM OLYMPIC SOFTWARE.  ANY SUCH DISTRIBUTION OF ARCDIAG

OR ITS RELATED FILES, EXCEPT AS PROVIDED FOR IN THIS LICENSE, SHALL BE

DETERMINED TO BE IN VIOLATION OF FEDERAL COPYRIGHT LAWS AND WILL BE

PROSECUTED.

INTRODUCTION

The ARCDIAG program is designed to be a service tool for network

administrator personnel for a network that uses ARCNET type hardware (see

specifications section for compliance).   The ARCDIAG.EXE program includes

two diagnostic sections that will help during installation of an ARCNET net-

work.  The diagnostic software can be operated at any workstation during

installation of the network or at any time while running the Novell Network

Operating System.

NOTE: The ARCDIAG program is not intended to isolate

Novell Software problems except in those cases where the

physical network hardware is the prime cause of the

problem.

The program exercises the network card by testing all of its internal functions,

RAM, RIM etc., and then displays a map of active nodes and packet counts.

The latter also provides an indication of any reconfiguration that might be

occurring.

START UP

The program can be run from the DOS prompt by typing:

ARCDIAG  [-mxxxx] [-ixxxx] [-ffile descriptor] [-ttype] [-b]

Start Up Options:

-b          Option for removing the preliminary board test.

-ffile descriptor

Option for specifying the location of the logical

name file - ARCDIAG.DAT.  40 characters

maximum. (Registered version only).

-ixxxx      Option for operating ARCDIAG at an I/O address of

xxxx (hex). ARCDIAG defaults to 02E0 (hex).  Must

use all 4 digits (Registered version only).

-mxxxx      Option for operating ARCDIAG at a RAM memory

location of xxxx (hex).  ARCDIAG defaults to D000

(hex).  Must use all 4 digits. (Registered version

only).

-q          Option allows board test only operation.  Place

[ARCDIAG -q] in the AUTOEXEC.BAT file for

automatic test of ARCNET card on power up.  If

an error condition is detected, the nature of the

error is displayed and the DOS error level is set

to 1.  Otherwise, nothing is displayed, and the

DOS error level is kept at 0.

-ttype      Option for selecting different network interface cards.

ARCDIAG defaults to standard SMC type cards with

memory mapped RAM and I/O mapped RIM

controllers.  (Registered version only). Use -t2 for

memory mapped cards such as Thomas Conrad in enhanced

mode.

ArcDiag will function in the file server or any work station, providing there

is a floppy drive available.  Work stations with no local file storage that use

boot proms to load Novell can not perform boards tests.

The ARCDIAG.EXE file may be copied onto and executed from the network hard

disk, but keep in mind that board testing is disabled if the Novell Shell

is loaded.

Type ARCDIAG at the DOS prompt of the drive containing ARCDIAG,

whereupon, if the Novell Shell is not loaded, the board test screen shown in

Figure 1 will appear:

Map      Count     Names    Format     Board     Print     Reset     Quit

+------------------------------------------------------------------++---------+

| Dec   1   2   3   4   5   6   7   8   9  10  11  12  13  14  15  ||         |

|   +--+---+---+---+---+---+---+---+---+---+---+---+---+---+---+--+||         |

|  0+                                                             +||---------|

| 15+    Network Board Test                                       +|| Rcn/Min |

| 30+                                                             +||         |

| 45+    PASSED: RAM test                                         +||---------|

| 60+    No Network Boot PROM Found                               +|| Station |

| 75+    PASSED: Internal RIM Test                                +||         |

| 90+    PASSED: Physical node address is 2                       +||---------|

|105+    PASSED: POR test                                         +|| Address |

|120+    PASSED: ETS test                                         +||         |

|135+                                                             +||---------|

|150+                                                             +|| Packets |

|165+                                                             +||         |

|180+                                                             +||---------|

|195+                                                             +|| Recnfgs |

|210+                                                             +||         |

|225+                                                             +||---------|

|240+                                                             +|| ActNode |

|   +--+---+---+---+---+---+---+---+---+---+---+---+---+---+---+--+||         |

+------------------------------------------------------------------++---------+

ARCDIAG V2.4             Copyright Olympic Software  1989

Figure 1 - Board Test Screen

BOARD TESTS

Unless option [-b] is included in the startup command, the board test

function automatically begins when entering [ARCDIAG] at the DOS

prompt.

Board Types

ARCDIAG has been set to function on most Arcnet boards,

with the exception of Nestar.

By including [-ttype] in the command line as a start up option,

different board types may be selected, e.g. [-t1], [-t2], etc.  Following is

an example of the command line:

ARCDIAG [-mxxxx] [-ffile descriptor]

[-ttype]

[-t1] is the default and is used for most Arcnet cards that use memory

mapped RAM and I/O mapped RIM controllers such as SMC,

Novell, Earth, etc.

[-t2] is used for memory mapped RIM controllers, such as Thomas

Conrad, Arcmaster, etc.  When the [-t2] option is used, the [-I] option

is disabled.

[-t3] is used for SMC type PC500 and PC550.  Use the [-I] option to

configure I/O mapping if other than 2E0.

Example:  To select a Thomas Conrad 6054 16 bit card

at D000 hex, enter the following command line:

ARCDIAG -mD000 -t2

Note:  If you do not know the option codes for

your board, contact Olympic Software Co., (313)

996-5750.

The following tests are made during the board test phase:

NOTE:  WHEN ARCDIAG IS FIRST EXECUTED, BOARD TESTING IS

DISABLED IF THE USER HAS ALREADY LOADED THE NOVELL SHELL.

RAM TEST

ARCDIAG exercises the ARCNET board's RIM RAM.  All 2K of RAM are tested

for shorted and open bits.  Also, the address lines are tested.

An error message is displayed if the RAM fails the test or if the DIP

switch setting of the memory address set is incorrect.

A PASSED message is displayed if operation is verified.

NETWORK BOOT PROM TEST

This test indicates whether or not a network boot PROM is installed.

RIM TEST

The token controller IC (9026) is tested in the RIM test.  The network

microprocessor (9026) can perform its own internal diagnostics.

When the test is run, approximately 95% of the processor's operations are

exercised.  If this test fails, replacement of the network card is

recommended if it is verified that the I/O address DIP switches are set

correctly.

A PASSED message is displayed if operation is verified.

SID TEST

During the SID test, the Controller source ID is located and read.  The

returned value from this test is the station node address.  The test will

fail if it is at zero.  If failed, make sure the address switches are set

to an address greater than zero.

A PASSED message is displayed if operation is verified.

ETS TEST

The ETS test verifies the proper location of the (E)xtended (T)imeout

(S)etting jumper.  Newer ARCNET cards may not have a jumper.  Therefore,

the card needs to be replaced if an error occurs.

A PASSED message is displayed if operation is verified.

POR TEST

During this test, a power-up cycle is activated and verified.  The 9026

RIM processor is reset and the ARCNET card is placed into an operational

state.

After performing all of the above tests, the program will halt for approximately

three seconds, then cycle into the Network Tests.

The Board Tests may be continued by selecting [Board] from the Lotus-type

menu at the top of screen, or by merely pressing [B].

NETWORK TESTS

Network tests function both in and out of the Netware shell.  However, the file

server and workstations can be tested only if they are powered on.

The Network Test Grid shown in Figure 2 appears after ArcDiag has been

started and the Board Tests are completed.

Map      Count     Names    Format     Board     Print     Reset     Quit

+------------------------------------------------------------------++---------+

| Dec   1   2   3   4   5   6   7   8   9  10  11  12  13  14  15  || 6/24/90 |

|   +--+---+---+---+---+---+---+---+---+---+---+---+---+---+---+--+|| 8:30:23 |

|  0+ FS1  .  MGR  .   .   .   .   .   .   .   .   .   .   .   .  +||---------|

| 15+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +|| Rcn/Min |

| 30+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +||    0    |

| 45+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +||---------|

| 60+  .  TST  .   .   .   .   .   .   .   .   .   .   .   .   .  +|| Station |

| 75+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +||    2    |

| 90+  .   .   .   .   .   .   .   .   .  LAB  .   .   .   .   .  +||---------|

|105+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +|| Address |

|120+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +||    3    |

|135+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +||---------|

|150+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +|| Packets |

|165+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +||         |

|180+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +||---------|

|195+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +|| Recnfgs |

|210+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +||    0    |

|225+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +||---------|

|240+  .   .   .   .   .   .   .   .   .   .   .   .   .   .   .  +|| ActNode |

|   +--+---+---+---+---+---+---+---+---+---+---+---+---+---+---+--+||    4    |

+------------------------------------------------------------------++---------�

ARCDIAG V2.4             Copyright Olympic Software  1989

Figure 2 - Network Test Grid

Each dot on the grid represents a node.  As noted by the cursor rapidly moving

across the grid, the program polls all 255 nodes.  When a file server or

workstation is powered on, the node number or node name will be displayed in

the place of the dot.

INFORMATION WINDOWS

Information from the network diagnostics is provided in the windows

on the right side of the screen.  The first window contains current

date and time.  Following is a description of the information

contained in the other windows.

RCN/MIN

Indicates the number of reconfiguration per minute, relative to

(1) the number of stations being powered on or off of the

network, and (2) any type of fault condition occurring on the

network.

This information may be used as a gross indication of network

fault conditions (see "Recnfgs" below).

STATION

If the Novell shell is running, the station number on which

ARCDIAG is running is displayed in this window.  This number

may be referenced to the station number which appears on the

file server monitor.

If the shell is not loaded, the message "No Shell" is displayed.

ADDRESS

The physical address, or the dip switch setting, on the network

interface card.

PACKETS

The number of packets that have occurred on the network since

the last reset.  This information indicates relative network

activity.

RECNFGS

Reconfiguration bursts are counted accumulatively and displayed

in this window.  Reconfiguration bursts increment whenever any

of the following occur:

1.  a workstation is powered onto the network.

2.  a workstation is powered off of the network

(intermittent).

3.  a network card is faulty.

4.  the network connecting cable is faulty.

A rapidly increasing reconfiguration count indicates a faulty

network (refer to "Network Troubleshooting, Network Tests").

ACTNODE

The number of nodes that have powered onto the network.

FUNCTION SELECTION

At the top of the Network Test screen is a "Lotus" type menu for selecting

various functions, such as "Map," "Count," "Names," etc.  To select a

particular function, either move the cursor key to highlight the function

and press [Enter], or merely type the first letter of the function label,

e.g. [M] for the "Map" function.

Note that a "help" statement, corresponding to each function, appears at

the bottom of the screen whenever a function is highlighted.

MAP

Selection of this function displays all nodes on a grid, each node

being indicated by a dot.  The number range is from 1 to 255.  The

display is arranged in a matrix form.  Nodes are displayed 15 at a

time along the horizontal.  Therefore, nodes 1 through 15 are

displayed on the first line, nodes 16 through 29 on the next line.

Stations that are powered on are indicated by a node number or node

name, in the place of a dot.

COUNT

In this mode, packets from each station are monitored and displayed

on the grid.  Packet counts can be viewed as an indication of activity

from each node.  Those nodes that transfer packets most frequently

will increment the packet count much more rapidly.

Although the program is not fast enough to catch all packets, relative

packet increases are displayed.  Therefore, the nodes that are assigned

as fileservers should have packet counts increasing at a faster rate

than workstations.

NAMES

Any of the 255 nodes can be given an alpha-numeric designation

which is useful for quick recognition of node locations.

These logical names can be assigned to physical node numbers by

using the "Names" function.  For example, a file server that might be

located on node 1 may be given the logical designation "FS1".

When selecting the "Names" function, the following message will

appear at the bottom of the screen:

Enter node number (1 to 255):

At the blinking cursor, enter the number of the node to which you

want to give a logical name, then press [Enter].  The following

message will then appear:

Enter name (3 char max):

At the blinking cursor, enter a maximum of three characters and press

[Enter].  You will then be asked:

Save (y/n) <return = yes>

If your entry is correct and you want to save it to the ARCDIAG.DAT

file on the disk, press [Enter], since "Enter" or "Return" is the

default. Otherwise, press [N] to ignore the entry.

The program uses the file ARCDIAG.DAT to store logical names for

physical node numbers.

The ARCDIAG.DAT file can be located in either the default drive or

when connected to a Novell network it can be located in the Z:

(public) directory.  Optionally a command line argument (-f) can be

used to tell ARCDIAG where to locate and what name to use for the

logical name file (see "Start Up Options").

FORMAT

Physical node numbers can be displayed on the node map as either

decimal, octal  or hexidecimal numbers.  The current mode of

operation is displayed in the upper left hand corner of the display.

Pressing the [Enter] key more than once will toggle the display mode

between decimal, octal and hexidecimal.

BOARD

This function runs diagnostics on network interface cards.  In order

for the board diagnostic test to operate, however, ARCDIAG cannot

be running in conjunction with the Novell operating system.

When this test is initiated from the menu at the top of the screen, it

will continue until the space bar is pressed or until an error is

found.

PRINT

If an IBM graphics compatible printer is available, use PrtSc to

produce an exact copy of the map including graphic borders.

If an IBM graphics compatible printer is not available, or if you want

to label and date the node map, select the "Print" function on the

menu, whereupon the following statement will appear at the bottom of

the screen:

Enter map name:

Type a name for the map, then press [Enter].  A node map with time

and date stamp will be sent to the printer.  All borders are converted

to characters that can be printed on any type of printer.

The printing program does not send a form feed character at the end

of printing allowing two maps to be printed on one 8 1/2 by 11 inch

piece of paper.

RESET

Select this function to reset all counters to zero and reformat

the grid.

EXIT

Select this function to exit the program.

SPECIFICATIONS

RIM RAM Base Address

D000 (hex) segment default  - configurable at runtime

Board I/O Address

02E0(hex) default - configurable at runtime

Interface Types

8 bit cards with Standard Microsystem interface (type 1)

16 bit cards with Thomas Conrad interface (6045)	(type 2)

16 bit cards like Standard Microsystms PC500/550 (type 3)

Active Nodes Displayed

1 thru 255

RIM RAM size

2K bytes

Computer Compatibility

IBM PC/XT/AT or compatible Network Operating Systems

Novell Netware 4.61 and above.  Novell Advanced Netware 1.02

and above.  Does not perform reliably when executing a board test

on a non-dedicated fileserver running ELS ver 1.0.

NETWORK TROUBLESHOOTING

Board Tests

Symptom            Corrective Action

RAM Test Fails

Check network card memory address DIP

switches for proper settings.

RAM defective, replace card.

RIM Test Fails

Check network card I/O address DIP switches for

proper settings.

9026 RIM controller defective, replace card

SID Test

The SID address should display the node address.

Note:  Do not allow two network cards to have the

same physical address.

ETS Test Fails

Newer network card does not have a jumper for

extended timeout.  Therefore, if this test fails, the

board needs to be replaced.

POR Test Fails

Before, continuing, remove the network cable and

try again.  If the test still fails, the network card

needs to be replaced.  Disconnecting the cable

removes any type of external interference with the

test.

Network Tests

Symptom                Likely Cause

Reconfiguration

counter increments

continuously

(1) no other nodes are connected to the  local

network card.

(2) the cable network is defective from the local

card to the rest of the network.

(3) an active or passive hub port is defective.

Multiple "phantom"

nodes are

displayed

(1) two nodes have been set to the same  address.

(2) a cable connection is not making solid contact.

(3) the memory address DIP switches are set

incorrectly.

(4) -i or -m options have been set incorrectly.

When run with Novell

shell - local node is

not displayed

Nothing is wrong with the display.  The Novell

shell has not passed the proper physical node

address to ARCDIAG.  Works correctly when not

running with the shell.  Fixed in Novell Advanced

Netware 2.0 and above.

ARCDIAG shows OK, but

cannot "find fileserver"

when shell is run

Make sure the interrupt jumper is used for all

network cards used with Advanced Netware 1.02

and above.

OLYMPIC SOFTWARE LICENSE AGREEMENT

License

Olympic Software owns the enclosed software program ("Program") and all

copyrights and other rights and interest in such program.  Olympic

Software grants you a non-exclusive license to use the enclosed Program

subject to the terms and restrictions below.

Restrictions on Use

You may use the program only on a single fileserver.  You may not use

the Program on a single user computer.  You may modify the Program or

merge it into any other program but all such portions remain subject

to the restrictions of this license.

Restrictions on Copying

The Program and documentation are copyrighted and may only be copied

as permitted by this license.  You may copy the machine readable form

of the Program only for back-up or archive purposes, provided that nor

more than three(3) copies are in existence at any one time without

prior written consent from Olympic Software.  You must maintain a

record of the number and location of all such copies.  You must include

the Olympic Software Copyright notice on any copy, modification or

merged portion.   You may copy the documentation which is provided in

printed form with the program.

Restrictions on Transfer

You may physically transfer the program from one of your own computers

to another as long as the matching key card is transported also.  You

may not distribute copies of the Program to others.  You may transfer

this license together with all originals, copies, modifications, and

merged portions of the Program and documentation, provided that the

person to whom you transfer agrees to be bound by the terms of this

license and notifies Olympic Software in writing.  You may not use,

copy, print, or transfer the program or documentation, except as

explicitly allowed by this license.

Term

This license continues in effect until terminated.  You may terminate

this license only by destroying all originals, copies, modifications,

and merged portions of the Program and documentation, in any form.  It

will also terminate if you fail to comply with any term or condition of

this license.  You agree upon such termination to destroy the Program

and documentation together with all copies, modifications, and merged

portions in any form.

No Warranty of Performance

OLYMPIC SOFTWARE MAKES NO WARRANTIES, EITHER EXPRESSED OR IMPLIED, AS TO

THE QUALITY, PERFORMANCE OR MERCHANTABILITY OF THIS PROGRAM, NOR AS TO

THE FITNESS FOR ANY PARTICULAR PURPOSE OR PARTICULAR RESULTS OBTAINED

THROUGH USING THE PROGRAM.  YOU ACCEPT THIS PROGRAM "AS IS" AND ASSUME

ALL RISK OF DEFECTS AND ALL COSTS OF SERVICING, REPAIRS AND CORRECTIONS.

Limited Warranty on Diskettes

Olympic Software  warrants the original magnetic diskette on which the

Program is recorded to be free from defects in material and faulty

workmanship under normal use and service for a period of ninety (90)

days from the date you receive it.  If, during this ninety day period,

such a defect should occur, you may return the diskette to Olympic

Software or an authorized Olympic Software dealer for a replacement

diskette at no charge, provided that you have previously returned the

enclosed Registration Form or send a copy of your receipt for the

Program.  Replacement of the diskette is your sole and exclusive remedy.

Olympic Software assumes no responsibility under this limited warranty

for, and will not replace, any diskette with defects due to accident,

abuse or misapplication. Olympic Software makes this warranty to you as

the original license only, and not to anyone to whom you transfer this

license and Program.  This limited warranty gives you specific legal

rights, and you may also have other rights which vary from state to

state.

Limitation of Liability

OLYMPIC SOFTWARE WILL NOT BE LIABLE FOR ANY DIRECT, INCIDENTAL, OR

CONSEQUENTIAL DAMAGES RESULTING FROM THE USE OF THE PROGRAM, EVEN IF

OLYMPIC SOFTWARE HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

SOME STATES DO NOT ALLOW THE EXCLUSION OR LIMITATION OR DIRECT,

INCIDENTAL OR CONSEQUENTIAL DAMAGES, SO THE ABOVE LIMITATION MAY NOT

APPLY TO YOU.  THIS LICENSE IS THE COMPLETE AND EXCLUSIVE STATEMENT

OF THE AGREEMENT BETWEEN YOU AND OLYMPIC SOFTWARE.  ALL OTHER

COMMUNICATIONS AND AGREEMENTS, WRITTEN AND ORAL, ARE SUPERSEDED.