Copyright (c) 1991 COMPAQ Computer Corporation.  All rights reserved.

This readme file contains additional information about the Novell NetWare

Version 3.11 SBACKUP support for the COMPAQ Tape Drive Options.

The support is made available on the COMPAQ USER PROGRAMS Diskette

Version 7.12 in the \NW386V31\TAPE directory.  It is also available on

CompuServe NetWire.

This document is divided into the following sections:

1.0  Overview

2.0  CPQSCSIT NLM

2.1  Supported Tape Drives

2.2  Installation instructions

2.3  EISA Advantanges

2.3.1  Selecting from multiple tape drive adapters

2.3.2  Selecting from multiple tape drives

3.0 SBACKUP Information

1.0  OVERVIEW

CPQSCSIT.NLM is a NetWare Loadable Module (NLM) designed to support the

Novell NetWare SBACKUP NLM. The CPQSCSIT NLM is a device driver that

supports SBACKUP v3.11.

Refer to the NOVELL NetWare Version 3.11 Server Backup Manual for further

instruction about installing and operating SBACKUP.  Refer to the COMPAQ

320-/525-Megabyte Tape Drive Installation Guide or the COMPAQ 1.3-/2.0-

Gigabyte Digital Audio Tape Drive Installation Guide for further information

about installing the hardware.

2.1  SUPPORTED TAPE DRIVES

The CPQSCSIT NLM allows SBACKUP for NetWare v3.11 to operate on the following

COMPAQ Tape Drives:

Drive name                                Part Number	  Drive Spare Part

320-/525-Megabyte Tape Drive               116573-001	   116804-001

1.3-/2.0-Gigabyte Digital Audio Tape Drive 131162-001    131149-001

2.2  INSTALLATION INSTRUCTION

The following paragraphs describe how to install the CPQSCSIT NLM on a

NetWare v3.11 server.

*  Log in to the NetWare v3.11 server using an account that has privileges

to copy files to the SYS:\SYSTEM server directory.

*  Copy the CPQSCSIT.NLM from the COMPAQ USER PROGRAMS Diskette to the

NetWare server SYS:\SYSTEM\DIBI directory of the server. The following

sample format assumes that the F drive is mapped to SYS:\SYSTEM and

that the COMPQA USER PROGRAMS Diskette is placed in the A diskette

drive location.

COPY A:\NW386V31\TAPE\CPQSCSIT.NLM F:\SYSTEM\DIBI\CPQSCSIT.NLM

COPY A:\NW386V31\TAPE\CPQSCSIT.DOC F:\SYSTEM\DIBI\CPQSCSIT.DOC

*  If your server is an EISA based system, the following steps are required.

Running the COMPAQ SYSTEM CONFIGURATION Utility resolves resource conflicts

between adapters installed in the EISA system.  This utility also makes

it possible for the CPQSCSIT NLM to determine the configuration of the

tape drive.  You will need to edit the SBACKUP configuration file on the

NetWare v3.11 server in the SYSTEM\DIBI directory on the SYS: volume to

add the following line:

"COMPAQ SCSI Tape Driver."             CPQSCSIT

*  If your server is an ISA based system, the following steps are required.

You will need to edit the SBACKUP configuration file on the NetWare v3.11

server in the SYSTEM\DIBI directory on the SYS: volume to add the following

line:

"COMPAQ SCSI Tape Driver."             CPQSCSIT   PORT=330 DMA=7 IRQ=5

The values specified for PORT is a hexadecimal value.  The values specified

for DMA and IRQ are decimal values.

The PORT, DMA, and IRQ settings must agree with the configuration

of the COMPAQ Tape Drive Option or the COMPAQ Tape Drive COMPRESSION

Option.  Run the COMPAQ SYSTEM CONFIGURATION Utility to view the

current settings.

2.3  EISA ADVANTAGES

The CPQSCSIT NLM takes advantage of information available when the

COMPAQ Tape Drive is configured in an EISA system.  Using this information,

the driver can prompt the user for which tape drive adapter or which tape

drive to use for the backup or restore operation if multiple devices are

installed in a NetWare server.

When the EISA system has been properly configured using the COMPAQ

SYSTEM CONFIGURATION Utility, the DIBI2$DV.DAT file is not used to

determine the configuration.  This is an advantage because administrators

may forget to update the DIBI2$DV.DAT file when changing the tape

adapter configuration or adding new tape drives to the system.

2.3.1  SELECTING FROM MULTIPLE TAPE DRIVE ADAPTERS

When more than one COMPAQ Tape Drive adapter is detected, the CPQSCSIT NLM

will display at least three panels.  The first displays a panel detailing

which slots and what kind of tape drive adapters are detected.  The next

panels allow you to select the adapter to use for the backup or restore

operation.  You will be instructed to press the ENTER key to select the

tape drive adapter shown or to press the ESCAPE key to move to the next

option for selection.

2.3.2  SELECTING FROM MULTIPLE TAPE DRIVES

When the selected COMPAQ Tape Drive adapter has more than one tape drive

attached, the CPQSCSIT NLM will display at least three panels.  The first

panel will display a list of available devices.  The next panels allow you

to select the device to use for the backup or restore operation.  You will

be instructed to press the ENTER key to select the adapter shown or to

press the ESCAPE key to move to the next option for selection.

3.0 SBACKUP INFORMATION

The following section describes specific information about using the

Novell SBACKUP NLM with COMPAQ Tape Drive Options.  Refer to the

Novell NetWare Version 3.11 Server Backup manual for more SBACKUP

information.

To increase the speed of Server Backup, you can use some of the following

optional command parameters.  Adding parameters to the LOAD SBACKUP

command lets you change the number and size of buffers.  Use this

command format:

LOAD SBACKUP buffers=10 size=256

In the above example, the buffers parameter may range from 2 to 10.  The

default is 4.  The size parameter may range from 16 to 256 Kbytes.  The

default is 64 Kbytes.

For the CPQSCSIT NLM, we recommend you use 10 buffers each 256 Kbytes.