ADDENDUM TO THE

NETWARE INSTALLATION SUPPLEMENT

FOR 3COM ETHERLINK AND

ETHERLINK PLUS (REV 1.00 AND ABOVE)

You should use this addendum with the NetWare Installation Supplement for

3Com EtherLink and EtherLink Plus (revision 1.00 and above, part

#100-000181-001).  Information in this addendum applies only if you are

installing EtherLink II (series 3C503) boards in network stations on a

NetWare v2.0x network.

This addendum explains how to install and set EtherLink II boards.

EtherLink II boards can be used in workstations, file servers, and bridges;

they can be used with thin cable, thick cable, or a combination of the two.

NOTE:  Before you use this addendum, read Chapters 1 and 2 of

the NetWare Installation Supplement for 3Com EtherLink and

Etherlink Plus.  Then, follow the instructions in this addendum to

install and set EtherLink II boards.  After you set your boards,

return to the supplement and continue the installation.

SETTING ETHERLINK II BOARDS FOR NETWORK STATIONS

The following instructions explain how to set EtherLink II (series 3C503)

network interface boards.  You must install EtherLink II network interface

boards in each station (file server, workstation, or bridge) on your network.

NOTE:  The Remote Reset feature allows workstations to boot

from a network (file server) hard disk.  NetWare v2.0x does not

support this feature for the EtherLink II board.

SEPARATING WORKSTATION BOARDS FROM FILE SERVER AND BRIDGE

BOARDS

Before you make settings on EtherLink II boards, separate the boards into

two groups:  those you install in workstations and those you install in file

servers and bridges.

Determine how many EtherLink II boards go in each file server or bridge by

referring to the records of the configuration settings you chose when you

generated the operating system for each file server and bridge.  (For file

servers, you recorded this information in your NetWare installation manual.

For bridges, you recorded this information in Chapter 2 of the Bridges

Installation Supplement.)

NOTE:  If you have more than one network connected through a

bridge, there may be file servers and other bridges that do not

need EtherLink II boards.

Set aside the correct number of EtherLink II boards for each file server or

bridge.

Tag file server boards and bridge boards with a string tag or some other

means of identification.  Then, include the following information on each

tag:

o    The specific file server or bridge the board will be installed in.

o    The configuration option the board should be set to.  (Refer again

to your records of configuration settings.)

NOTE:  No two EtherLink boards installed in the same file server

or bridge can have the same configuration option number.

However, EtherLink boards installed in different file servers or

bridges can have the same configuration option number.

You should now have two groups of boards:  an unmarked group to be

installed in workstations and a marked group to be installed in file servers

and bridges.

MAKING SETTINGS ON ETHERLINK II WORKSTATION BOARDS

Set all EtherLink II boards (that you will install in workstations) to the

following configuration:

o    Base I/O address:  300h

o    Base memory address:  DISABLE

o    Interrupt line:  3

o    DMA channel:  1

The base I/O address and base memory address are set by placing jumper

plugs on jumper pins.  The interrupt line, DMA channel, and connector type

are set by the shell software.  You do not need to do anything to choose

these settings.

If you install any other board (such as a modem board or memory board) in

your workstation and that board uses the same base I/O address, DMA

channel, or interrupt line as the EtherLink II board, you must change the

settings of the additional board.

MAKING SETTINGS ON ETHERLINK II FILE SERVER AND BRIDGE BOARDS

The configuration settings for EtherLink II boards used in file servers and

bridges consist of the following:

o    Base memory address

o    Connector type (DIX or BNC)

o    DMA channel

o    Base I/O address

o    Interrupt line

The DMA channel, interrupt line, and connector type are determined by the

configuration option you chose when you generated the operating system

with GENOS.  If you have not chosen the appropriate configuration, you

must reconfigure the operating system by running GENOS and selecting a

valid option.

To set the base I/O address and base memory address on file server and

bridge boards, continue with the next section.

SETTING THE BASE I/O ADDRESS ON ETHERLINK II FILE SERVER AND

BRIDGE BOARDS

File server boards and bridge boards should be tagged with a configuration

option number.  Find the proper address setting and set the base I/O

address jumper block according to the configuration option number.

SETTING THE BASE MEMORY ADDRESS ON ETHERLINK II FILE SERVER

AND BRIDGE BOARDS

Using the same configuration option number you used to set the base I/O

address jumper, find the proper address setting for the base memory.  If the

configuration uses a DMA channel, you must disable the base memory address

by moving the shorting plug to the DISABLE jumper block.  (You can use

either a DMA channel or a base memory address, but not both.)

Set the base I/O address and base memory address for each EtherLink II

board.  When you have configured your boards, continue with the network

installation process by returning to Chapter 3 of the installation supplement

and reading the section that explains how to install the boards in

stations.

COPYING ETHERLINK II DRIVER FILES FOR NETWARE VERSION 2.0x

If you are installing a NetWare v2.0x network containing EtherLink II

boards, copy EtherLink II driver files from the driver update diskette to the

diskettes you received with your NetWare shipment.  Copy the EtherLink II

operating system driver files before you run the following NetWare v2.0x

installation utilities:

o    GENOS, used to generate the network operating system

o    GENBR, used to generate a NetWare bridge (if applicable)

Copy the EtherLink II shell driver files before you run the following utility:

o    GENSH, used to generate your workstation shells

The instructions in the following sections explain how to copy EtherLink II

driver files from the driver update diskette to the GENOS-1, GENSH-1 (or

GENSH), and GENBR-1 diskettes.

IMPORTANT:  If your NetWare installation utilities are on

5.25-inch diskettes, copy them to 3.5-inch diskettes before you

proceed with these instructions.

COPYING OPERATING SYSTEM DRIVER FILES TO THE GENOS-1 OR

GENBR-1 DISKETTE

To copy operating system driver files from the driver update diskette to the

GENOS-1 diskette, complete the following steps.

NOTE:  If you are running the GENBR utility to configure a

NetWare v2.0x bridge, copy the operating system driver files to

the GENBR-1 diskette.  The following instructions (and the driver

files) are the same for the GENOS-1 and GENBR-1 diskettes;

substitute the GENBR-1 diskette when you are prompted for the

GENOS-1 diskette.

1)   Boot a computer with DOS.

2)   Insert the GENOS-1 diskette into drive A.  If you have a second

diskette drive, insert the driver update diskette into drive B.

3)   Use the DOS MD (Make Directory) command to create a new

subdirectory named "3C503" on the GENOS-1 diskette.  This 3C503

subdirectory will eventually contain the EtherLink II operating system

driver files.

To create this subdirectory, type the following command at the DOS

prompt for drive A:

MD 3C503  <Enter>

4)   Use the DOS CD (Change Directory) command to switch to the

subdirectory you just created.  Type

CD 3C503  <Enter>

5)   Copy the EtherLink II operating system driver files from the driver

update diskette (in drive B) to the GENOS-1 diskette (in drive A).

Copy the files from the V2.0A subdirectory of the driver update

diskette, not from the root directory.

To copy the proper files, type

COPY B:\V2.0A\?COMDRV.OBJ  <Enter>

If your computer has one diskette drive, you will be prompted to switch

between the GENOS-1 diskette (the "diskette for drive A") and the

driver update diskette (the "diskette for drive B").

6)   To verify that the files were copied correctly, type

DIR  <Enter>

The directory listing on the screen should include the following four

driver files (they may appear in any order):

ACOMDRV.OBJ

BCOMDRV.OBJ

CCOMDRV.OBJ

DCOMDRV.OBJ

NOTE:  In some instances you may see an error message indicating

that the four driver files do not fit on the GENOS-1 diskette.  If

this occurs, make room for these files by removing another

subdirectory and its driver files.  Instructions for doing so are

found in your NetWare installation manual, NetWare Bridges

Installation Supplement, or NetWare Remote Workstations manual.

After you delete a subdirectory and its driver files, you should

have enough room to copy the operating system driver files to the

appropriate diskettes.  Repeat the instructions beginning with Step

1 on page 4.

COPYING SHELL DRIVER FILES TO THE GENSH-1 DISKETTE

To copy EtherLink II shell driver files from the driver update diskette to

the GENSH-1 diskette, complete the following steps.  (In some cases you may

have only one diskette for the GENSH utility.  If so, whenever these

instructions refer to the GENSH-1 diskette, use the GENSH diskette.)

1)   Boot a computer with DOS.

2)   Insert the GENSH-1 diskette into drive A.  If you have a second

diskette drive, insert the driver update diskette into drive B.

3)   Use the DOS MD (Make Directory) command to create a new

subdirectory named "3C503" on the GENSH-1 diskette.  This 3C503

subdirectory will eventually contain the EtherLink II shell driver files.

To create this subdirectory, type the following command at the DOS

prompt for drive A:

MD 3C503  <Enter>

4)   Use the DOS CD (Change Directory) command to switch to the

subdirectory you just created.  Type

CD 3C503  <Enter>

5)   Copy the shell driver file from the driver update diskette (in drive B)

to the GENSH-1 diskette (in drive A).  The driver file you copy

depends on the transceiver connector you use (BNC for a thin-cable

network or DIX for a thick-cable network).

IMPORTANT:  Because thin- and thick-cable networks require

different shell drivers, you must copy the correct driver file from

the driver update diskette.  If you copy the SHELLDRV.OBJ file

from the wrong subdirectory, your workstation will not function.

A)   BNC connector:  Copy the driver file from the V2.0A subdirectory

of the driver update diskette, not from the root directory.

To copy the proper file, type

COPY B:\V2.0A\SHELLBNC.OBJ <Enter>

Rename the SHELLBNC.OBJ file to SHELLDRV.OBJ by typing

REN SHELLBNC.OBJ SHELLDRV.OBJ

B)   DIX connector:  Copy the driver file from the V2.0A subdirectory

of the driver update diskette, not from the root directory.

To copy the proper file, type

COPY B:\V2.OA\SHELLDIX.OBJ <Enter>

Rename the SHELLDIX.OBJ file to SHELLDRV.OBJ by typing

REN SHELLDIX.OBJ SHELLDRV.OBJ

If your computer has one diskette drive, you will be prompted to

switch between the GENSH-1 diskette (the "diskette for drive A")

and the driver update diskette (the "diskette for drive B").

6)   To verify that the file has been copied correctly, type

DIR  <Enter>

The directory listing on the screen should include the driver file

(SHELLDRV.OBJ) that you copied.

WHERE TO GO FROM HERE

You are now ready to return to your main installation manual and generate

the operating system and workstation shells for your EtherLink II network.

Use the GENOS utility to generate the operating system and the GENSH

utility to generate the workstation shells.  (If you are installing a NetWare

bridge, run the GENBR utility as outlined in the NetWare Bridges Installation

Supplement or in the NetWare Remote Workstations manual.)

As you run these utilities, select the following driver option for the

EtherLink II network:

3Com EtherLink II Version 2.3 EC

Note that the version number may differ from that shown above.

After you have generated the operating system and workstation shells, go to

the NetWare Installation Supplement for 3Com EtherLink and EtherLink Plus

and read Chapters 1 and 2.

Then, return to this addendum to set your EtherLink II boards.