ODI FOR IBM LANSTREAMER ADAPTER, VERSION 1.0   MARCH 26, 1993

About This File  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  2

Supported Platforms  . . . . . . . . . . . . . . . . . . . . . . . . . .  2

IBMTRDBO LAN Driver Installation Procedure   . . . . . . . . . . . . . .  2

NetWare Server   . . . . . . . . . . . . . . . . . . . . . . . . . . .  3

Configurable IBMTRDBO LAN Driver Parameters  . . . . . . . . . . . . . .  5

Sample IBMTRDBO.CFG Configuration File for NetWare Server  . . . . . . .  7

IBMTRDBO LAN Driver Performance Tuning   . . . . . . . . . . . . . . . .  8

NetWare Server   . . . . . . . . . . . . . . . . . . . . . . . . . . .  8

IBM LANStreamer Adapter Custom Statistic Counters  . . . . . . . . . . .  9

Messages for the IBMTRDBO LAN Driver   . . . . . . . . . . . . . . . .   12

(C) Copyright IBM Corp. 1993                                                1

ABOUT THIS FILE

This file describes the use of the IBMTRDBO LAN driver with the IBM

LANStreamer Adapter.  This LAN driver supports the IBM LANStreamer Adapter in

the Novell Netware environment.

This file is an extension of the IBM LANSTREAMER INSTALLATION AND TESTING

INSTRUCTIONS that are shipped with the adapter.  This file describes the

installation and configuration of the IBM LANStreamer Open Datalink Interface

(ODI) IBMTRDBO LAN driver.  It contains information about the following

topics:

o   Supported platforms

o   Software installation procedures

o   Configurable IBMTRDBO LAN driver parameters

o   Performance tuning the IBMTRDBO LAN driver

o   IBM LANStreamer Adapter Custom Statistic Counters

o   Messages for the IBMTRDBO LAN driver

SUPPORTED PLATFORMS

The IBMTRDBO LAN driver is supported in any of the hardware environments

supported by the IBM LANStreamer Adapter.  See the IBM LANSTREAMER

INSTALLATION AND TESTING INSTRUCTIONS for information about supported

hardware.

The following operating system environments are supported:

o   Novell NetWare 3.1x

o   Novell NetWare 4.0

IBMTRDBO LAN DRIVER INSTALLATION PROCEDURE

This section describes how to install the IBMTRDBO LAN driver on the NetWare

Server platform.

See the IBM LANSTREAMER INSTALLATION AND TESTING INSTRUCTIONS that is shipped

with the adapter for instructions on how to install the IBM LANStreamer

Adapter.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  2

NetWare Server

This section describes how to install the IBMTRDBO LAN driver on a NetWare

Server.

To install the IBMTRDBO LAN driver on a NetWare Server, perform the following

steps:

1.  Choose a target directory path on the diskette or the fixed disk (for

example, C:\NW) for installation of the IBMTRDBO LAN driver and its

associated files.  All files (in step 2, below) should be copied to this

target directory.

2.  Copy the following files to the target directory:

o   PATCHMAN.NLM

o   LSLENH.NLM

o   IBMTRDBO.LAN

o   MSM31X.NLM

o   TOKENTSM.NLM

3.  Add the following lines to the STARTUP.NCF file:

Set Minimum Packet Receive Buffers = 64

Set Maximum Physical Receive Packet Size = 4500

NOTES:

1.  If running on NetWare 3.1x add,

Set Maximum Physical Receive Packet Size = 4202

2.  If the IBM LANStreamer Adapter is installed in a 16-bit

slot in your computer, add,

Set Reserved Buffers Below 16 Meg = 64

4.  Add the following line to the AUTOEXEC.NCF file:

LOAD C:\NW\PATCHMAN.NLM

LOAD C:\NW\LSLENH.NLM

LOAD C:\NW\IBMTRDBO.LAN @IBMTRDBO.CFG

NOTES:

a.  @IBMTRDBO.CFG is an optional command line parameter (see step 5,

below).

b.  IBMTRDBO.LAN automatically loads MSM.NLM (MSM31X.NLM if running

Novell NetWare 3.1x) and TOKENTSM.NLM.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  3

See NOVELL NETWARE SYSTEM ADMINISTRATION for more information about the

AUTOEXEC.NCF and STARTUP.NCF files.

5.  If you need to change any IBMTRDBO LAN driver parameter values from their

defaults, these parameters can be specified on the command line when the

IBMTRDBO LAN driver is loaded, or they can be specified by creating an

IBMTRDBO.CFG configuration file.  If the optional @IBMTRDBO.CFG command

line parameter is used to specify an IBMTRDBO.CFG configuration file,

then the configuration file must be located in the same directory as the

IBMTRDBO.LAN file.  You can use any text editor to create or edit the

IBMTRDBO.CFG file.  See "Sample IBMTRDBO.CFG Configuration File for

NetWare Server" for a sample IBMTRDBO.CFG file.

6.  If you run the MONITOR.NLM program at your NetWare server, obtain the

latest copy of the MONITOR.NLM program from Novell.  Update your NetWare

servers with the latest copy of the MONITOR.NLM program.

7.  Obtain the latest copy of the LSLENH.NLM patch for NetWare from Novell.

Load this patch in the AUTOEXEC.NCF when you bring up the NetWare server.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  4

CONFIGURABLE IBMTRDBO LAN DRIVER PARAMETERS

The following parameters can be configured at the NetWare Server by setting

keywords on the command line and in the IBMTRDBO.CFG file.  A sample

IBMTRDBO.CFG file is shown in "Sample IBMTRDBO.CFG Configuration File for

NetWare Server."

SLOT

This keyword defines the slot number of the corresponding IBM

LANStreamer Adapter in the computer.  To avoid being prompted to enter

a slot number when the IBMTRDBO LAN driver is loaded, specify this

keyword in the AUTOEXEC.NCF file.  This keyword must be within the

range of 1-8.

FRAME

This keyword defines the frame type used with the IBM LANStreamer

Adapter.  The default is TOKEN-RING.  The value for this keyword must

be one of the following ascii strings:

o   TOKEN-RING

o   TOKEN-RING_SNAP

To set a locally administered address for the adapter, type in the

local address you want to use.  The local address must contain 12

hexadecimal characters and can be specified in either MSB (Most

Significant Bit: default) or LSB (Least Significant Bit) format.  To

specify a MSB node address the letter "M" immediately follows the last

digit of the node address.  To specify a LSB node address the letter

"L" immediately follows the last digit of the node address.  Since MSB

is the default format for the node address parameter, the use of a

letter "M" at the end of a node address is for clarity.  This keyword

is specified on the command line or in the IBMTRDBO.CFG as NODE for

the NetWare Server.  See "Sample IBMTRDBO.CFG Configuration File for

NetWare Server" for a sample IBMTRDBO.CFG file.

If a node address is specified in MSB format, the characters must be

within the range 400000000000-7FFFFFFFFFFF (for example,

400000000001M).  This address specified in LSB format would be

020000000080L with an "L" appended to the end.

NOTES:

1.  Do not assign the same local address to 2 or more stations.

Severe station communication or network failures may occur when 2

or more stations are assigned the same address.

2.  If you do not assign a locally administered address, the adapter

uses the universally administered address permanently embedded on

the adapter.

RXBUFFERS

This keyword defines the number of receive buffers used by the

IBMTRDBO LAN driver.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  5

The value for this keyword must be: 4, 8, 16, 32, or 64; the default

is 32.  (Receive buffer size is defined by the server SET command.

See NOVELL NETWARE SYSTEM ADMINISTRATION for more information.)

TXBUFFERS

This keyword defines the number of transmit buffers used by the

IBMTRDBO LAN driver.

The value for this keyword must be: 1, 2, 4, 8, or 16; the default is

16.  (Receive buffer size is defined by the server SET command.  See

NOVELL NETWARE SYSTEM ADMINISTRATION for more information.)

TXINTSOFF

This keyword will reduce CPU utilization in server environments when

using the standard IPX protocol.  Reducing CPU utilization (shown on

the monitor screen) may improve performance if CPU utilization is very

high (above 80% utilized).  This option should not be used if Novell's

packet burst protocol (PBURST.NLM) is loaded, as performance on packet

burst links may be significantly degraded.  Use of this option is

recommended only for servers using the standard IPX protocol.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  6

SAMPLE IBMTRDBO.CFG CONFIGURATION FILE FOR NETWARE SERVER

; The following keywords are generic ODI keywords for IBMTRDBO:

NODE=400000000002M

SLOT=2

FRAME=TOKEN-RING

; The following keywords are custom keywords for IBMTRDBO:

TXBUFFERS=16

RXBUFFERS=32

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  7

IBMTRDBO LAN DRIVER PERFORMANCE TUNING

This section describes how to get the best performance from the IBM

LANStreamer Adapter and IBMTRDBO LAN driver.

NetWare Server

The maximum frame size that can be sent or received by the IBM LANStreamer

Adapter configured at a 4-Mbps data rate is 4494 bytes and at a 16-Mbps data

rate is 17954 bytes.  Since frame sizes are typically negotiated between the

NetWare server and workstations, it is best to configure receive buffer sizes

at the server that equal the maximum transmit buffer size at workstations.

For this reason, increasing your receive buffer size to a value that is too

big only increases the amount of memory that is used by NetWare and may not

increase the performance of the IBM LANStreamer Adapter or NetWare.

Since the IBMTRDBO.LAN driver uses NetWare receive buffers to receive frames,

the size of receive buffers used by the IBMTRDBO.LAN driver is configurable

by changing the size of the NetWare receive buffers.  Create or edit the

STARTUP.NCF file and add the following line to change the size of the receive

buffers:

Set Maximum Physical Receive Packet Size = 4202

NOTE:  If running on NetWare 3.1x, 4202 bytes is the

maximum configurable receive buffer size.

See "Configurable IBMTRDBO LAN Driver Parameters" regarding the TXINTSOFF

keyword that may improve performance of the NetWare server.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  8

IBM LANSTREAMER ADAPTER CUSTOM STATISTIC COUNTERS

The custom statistic counters give the user information that is specific to

the IBM LANStreamer Adapter and IBMTRDBO LAN driver.  There follow

descriptions of the custom statistic counters and what action, if any, should

you take based on these counters.

MAC Frame Count:

Description:  This counter indicates the number of Media Access

Control (MAC) frames sent by the IBMTRDBO driver.

These MAC frames were built and requested to be

sent by a protocol stack.

Receive Frames Discarded

Description:  This counter indicates the number of frames that were

received by the IBM LANStreamer Adapter but were

discarded by the IBMTRDBO LAN driver because they were

received with an error.

Cause/Action:  If this counter steadily increases, it may indicate a

problem with your computer or the IBM LANStreamer Adapter.

Run the adapter diagnostic tests located on the

Option Diskette that were shipped with the IBM

LANStreamer Adapter.  If there are no problems with

the adapter, reload the IBMTRDBO LAN driver.

If this problem persists, contact your network

administrator.

Transmit Frames Discarded

Description:  This counter indicates the number of frames that could

not be transmitted by the IBM LANStreamer Adapter and were

discarded by the IBMTRDBO LAN driver.

Cause/Action: Possible explanations for this problem are as follows:

1.  The token-ring is experiencing problems and,

therefore, the IBM LANStreamer Adapter could not

transmit these frames.  If this problem persists,

contact your network administrator.

2.  The IBM LANStreamer Adapter is not responding.

Run the adapter diagnostic tests located on

on the Option Diskette that was shipped with

the IBM LANStreamer Adapter.  If there are no

problems with the adapter, reload the IBMTRDBO

driver.  If a problem was found, follow the

instructions supplied with the diagnostic tests

to correct the problem.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  9

Adapter Hardware Errors

Description:  This counter indicates the number of hardware errors

reported by the IBM LANStreamer Adapter.

Cause/Action: The IBM LANStreamer Adapter and IBMTRDBO LAN driver

may recover from these errors.

If this counter continues to increase, Run the

adapter diagnostic tests located on the Option

that was shipped with the

IBM LANStreamer Adapter.  If there are no problems with

the adapter, reload the IBMTRDBO LAN driver.

If a problem was found, follow the instructions

supplied with the diagnostic tests to correct the

problem.

Adapter Detected Parity Errors

Description:   This counter is incremented when the LANStreamer Adapter

detects a parity error while data is being transferred

between the IBM LANStreamer Adapter and the computer.

Cause/Action:  If this counter continues to increase, it may indicate a

problem with your computer or the IBM LANStreamer Adapter.

Run the adapter diagnostic tests located on the Option

Diskette that was shipped with the IBM LANStreamer

Adapter.  If there are no problems with the adapter

reload the IBMTRDBO LAN driver.  If a problem was found,

follow the instructions supplied with the diagnostic tests

to correct the problem.

Adapter RAM Access Errors

Description:   This counter is incremented when the LANStreamer Adapter

reports a RAM access violation.

Cause/Action:  If this counter continues to increase, and you are

experiencing problems with the IBM LANStreamer Adapter,

run the adapter diagnostic tests located on the Option

Diskette that was shipped with the IBM LANStreamer

Adapter.  If there are no problems with the adapter,

reload the IBMTRDBO LAN driver.  If this problem still

exists, contact your network administrator or IBM

support center.

Micro Channel Errors

Description:  This counter indicates the number of Micro Channel

bus errors that were detected.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  10

Cause/Action: The IBM LANStreamer Adapter detected an error with

the Micro Channel bus while accessing or attempting

to access the bus.

If this counter continues to increase, run the

diagnostics on the reference diskette that came with

your computer to test for problems with the Micro

Channel bus.

Receive Overrun Errors

Description:  This counter is incremented when the IBM LANStreamer Adapter

receives a frame from the token-ring network

faster than it can be moved into computer memory.

Cause/Action: If this counter continues to increase, run the

diagnostics on the reference diskette that came with

your computer to test for problems with the Micro Channel

bus.  If this problem still persists, contact you

network administrator or IBM support center.

Transmit Underrun Errors

Description:  This counter is incremented when the IBM LANStreamer Adapter

attempts to transmit a frame to the token-ring network

but cannot move the frame fast enough from computer

memory to the network.

Cause/Action: If this counter continues to increase, run the

diagnostics on the reference diskette that came with

your computer to test for problems with the Micro Channel

bus.  If this problem still persists, contact your

IBM support center.

Transmit Checksum Errors

Description:  This counter is incremented when the transmission of

a frame completes and the checksum value for the frame

is incorrect.

Cause/Action: If this counter continues to increase, contact

your network administrator or IBM support center.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  11

MESSAGES FOR THE IBMTRDBO LAN DRIVER

IBMTRDBO-xx-071: The matching virtual adapter could not be found.

Cause:  An error occurred while trying to load another copy

of the IBMTRDBO LAN driver to support an additional

frame type (FRAME= parameter) on an IBM LANStreamer

Adapter.  The MSM could not find a previous copy of

the IBMTRDBO LAN driver required to load the specified

frame type.

Action: Obtain the most current copy of the MSM.NLM or

MSM31X.NLM (if running NetWare version 3.1x), and then

reload the IBMTRDBO LAN driver for the desired frame

types.  If this error persists, contact your network

administrator or IBM support center.  Refer to the

NOVELL NETWARE SYSTEM ADMINISTRATION manual for

information about loading and unloading LAN drivers.

IBMTRDBO-xx-072: A resource tag is unavailable.

Cause:  The NetWare operating system was unable to allocate

memory for a resource tag.  The IBMTRDBO LAN driver

did not load because required internal resources

were not available.  There may be too many NetWare

Loadable Modules (NLMs) running in the NetWare Server.

Action: Unload the NLMs that are loaded but are not being used,

and then load the IBMTRDBO LAN driver.  If this error

persists, down, exit and then restart the server.  Load

the IBMTRDBO LAN driver before loading NLMs.  This may

require you to edit the AUTOEXEC.NCF file before restarting

the server.  Refer to NOVELL NETWARE SYSTEM ADMINISTRATION

for information about the AUTOEXEC.NCF file.

IBMTRDBO-xx-073: Unable to allocate memory.

Cause:  The NetWare operating system was unable to allocate

memory for the IBMTRDBO LAN driver.  The IBMTRDBO

LAN driver did not load.

Action: Unload the NetWare Loadable Modules (NLMs) that are

loaded but are not being used, and then load the IBMTRDBO

LAN driver.  If this error persists, down, exit and then

restart the server.  Load the IBMTRDBO LAN driver before

loading NLMs.  This may require you to edit the

AUTOEXEC.NCF file before restarting the server.  Refer to

NOVELL NETWARE SYSTEM ADMINISTRATION for information

about the AUTOEXEC.NCF file.

IBMTRDBO-xx-074: The hardware interrupt cannot be set.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  12

Cause:  The NetWare operating system was unable to set the

hardware interrupt level requested by the IBMTRDBO

LAN driver.  The IBMTRDBO LAN driver did not load.

Action: Try to load the IBMTRDBO LAN driver again.  If the error

persists, contact your network administrator or IBM

support center.

IBMTRDBO-xx-075: The MLID cannot be registered with the LSL.

Cause:  The IBMTRDBO LAN driver could not be registered with the

Link Support Layer (LSL) module.  The maximum number of

LAN drivers have already been loaded or there was not

enough memory to load the IBMTRDBO LAN driver.  The IBMTRDBO

LAN driver did not load.

Action: Unload LAN drivers that are not being used and then

load the IBMTRDBO LAN driver.  If this error persists,

down, exit and then restart the server.  Load the IBMTRDBO

LAN driver before loading other LAN drivers.  This may

require you to edit the AUTOEXEC.NCF file before restarting

the server.  Refer to NOVELL NETWARE SYSTEM ADMINISTRATION

for information about the AUTOEXEC.NCF file.

IBMTRDBO-xx-076: The polling procedure cannot be added.

Cause:  Too many polling processes are registered with

the NetWare operating system.  The IBMTRDBO LAN

driver did not load.

Action: Unload LAN drivers or NetWare Loadable Modules that

are loaded but are not being used.  Try to load the

IBMTRDBO LAN driver again.  If this error persists,

contact your network administrator or IBM support

center.

IBMTRDBO-xx-077: The event notification routine cannot be registered.

Cause:  The NetWare operating system was unable to allocate

memory to register the IBMTRDBO LAN driver event

notification routine.  The IBMTRDBO LAN driver did

not load.

Action: Unload LAN drivers or NetWare Loadable Modules that

are loaded but are not being used.  Try to load the

IBMTRDBO LAN driver again.  If this error persists,

contact your network administrator or IBM support

center.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  13

IBMTRDBO-xx-078: The firmware file cannot be read.

Cause:  The Netware operating system was unable to load the

IBMTRDBO LAN driver.  The IBMTRDBO LAN driver was not

loaded.

Action: Obtain another copy of the IBMTRDBO LAN driver from

the original distribution diskette.  Reload the IBMTRDBO

LAN driver.  If this error persists, contact your network

administrator or IBM support center.

IBMTRDBO-xx-084: Unable to allocate memory below 16 Megabyte boundary.

Cause:  The Netware operating system ran out of memory buffers

below 16 megabytes.  The IBMTRDBO LAN driver was not

loaded.

Action: Set the Reserved Buffers Below 16 Meg parameter in the

STARTUP.NCF to a larger number.

IBMTRDBO-xx-086: The driver parameter block is too small.

Cause:  The IBMTRDBO.LAN file is designed to run with an older

version of the MSM.NLM.  The IBMTRDBO LAN driver was

not loaded.

Action: Obtain an older version of the MSM.NLM or a newer

version of the IBMTRDBO.LAN file.

IBMTRDBO-xx-087: The media parameter block is too small.

Cause:  The TOKENTSM.NLM NetWare Loadable Module (NLM) is

designed to run with an older version of the MSM.NLM

or MSM31X.NLM if running NetWare Version 3.1x.

Action: Obtain an older version of the MSM.NLM (MSM31X.NLM

if running NetWare Version 3.1x) or a new version of

the TOKENTSM.NLM.

IBMTRDBO-xx-091: The hardware configuration conflicts.

Cause:  The IBMTRDBO LAN driver requested use of a hardware

resource that was already being used by another

hardware device.  The IBMTRDBO LAN driver did not load.

Action: Change the hardware configuration of the IBM LANStreamer

Adapter or the hardware configuration of the conflicting

hardware device.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  14

IBMTRDBO-xx-092: Cannot schedule AES without an HSM routine.

Cause:  The IBMTRDBO.LAN file is damaged.

Action: Obtain another copy of the IBMTRDBO LAN driver from

the original distribution diskette.  Reload the IBMTRDBO

LAN driver.  If this error persists, contact your network

administrator or IBM support center.

IBMTRDBO-xx-093: Cannot schedule interrupt time call back without an

HSM routine.

Cause:  The IBMTRDBO.LAN file is damaged.

Action: Obtain another copy of the IBMTRDBO LAN driver from

the original distribution diskette.  Reload the IBMTRDBO

LAN driver.  If this error persists, contact your network

administrator or IBM support center.

IBMTRDBO-xx-094: Cannot set hardware interrupt without an HSM routine.

Cause:  The IBMTRDBO.LAN file is damaged.

Action: Obtain another copy of the IBMTRDBO LAN driver from

the original distribution diskette.  Reload the IBMTRDBO

LAN driver.  If this error persists, contact you network

network administrator or IBM support center.

IBMTRDBO-xx-095: Cannot add polling without an HSM routine.

Cause:  The IBMTRDBO.LAN file is damaged.

Action: Obtain another copy of the IBMTRDBO LAN driver from

the original distribution diskette.  Reload the IBMTRDBO

LAN driver.  If this error persists, contact your network

administrator or IBM support center.

IBMTRDBO-xx-126: The group bit in the node address override was cleared.

Cause:  The address specified for the node address parameter

is an incorrect adapter node address.  The group

address bit for the node address specified had been

set.  The group address bit was cleared by the IBMTRDBO

LAN driver.  The IBMTRDBO LAN driver was loaded.

Action: Correct the node address parameter to specify a locally

administered address according to the IBM TOKEN-RING

NETWORK ARCHITECTURE REFERENCE.  On a DOS or

OS/2 workstation, change the node address parameter in

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  15

the NET.CFG file.  On a NetWare Server, change the node

address parameter specified on the command line when the

IBMTRDBO LAN driver was loaded, or change the node address

specified in the IBMTRDBO.CFG configuration file.

IBMTRDBO-xx-127: The local bit in the node address override was set.

Cause:  The address specified for the node address parameter

is an incorrect adapter node address.  The node address

specified does not have the locally administered address

bit set.  The locally administered address bit was set

by the IBMTRDBO LAN driver.  The IBMTRDBO LAN driver was

loaded.

Action: Correct the node address parameter to specify a locally

administered address according to the IBM TOKEN-RING

ARCHITECTURE REFERENCE.  On a DOS or OS/2

workstation, change the node address parameter in the

NET.CFG file.  On a NetWare server change the node

address parameter specified on the command line when the

IBMTRDBO LAN driver was loaded, or change the node address

specified in the IBMTRDBO.CFG configuration file.

IBMTRDBO-xx-200: Initializing the IBM LANStreamer Adapter ... please wait.

Cause:  The IBM LANStreamer Adapter initialization process has

started.

Action: None.

IBMTRDBO-xx-202: The IBM LANStreamer Adapter is closed.

Cause:  The IBMTRDBO LAN driver could not insert the IBM

LANStreamer Adapter into the ring due to previosly

reported hardware problems.  The IBMTRDBO LAN driver

has closed the adapter and has generated an alert

after trying to insert the adapter into the ring

several times.

Action: Follow the recommended action for correcting any error

messages displayed by the IBMTRDBO LAN driver before

this one.

IBMTRDBO-xx-204: The IBM LANStreamer Adapter requires a computer with a

Micro Channel bus.

Cause:  The IBM LANStreamer Adapter will work only in a computer

that supports the IBM Micro Channel Architecture.

Action: Install the IBM LANStreamer Adapter and IBMTRDBO LAN

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  16

driver in a computer that supports the IBM Micro Channel

Architecture.

IBMTRDBO-xx-205: The IBMTRDBO LAN driver was not loaded because of an

incorrect keyword value.

Cause:  The value for one of the keywords specified on the

command line or in the IBMTRDBO.CFG file was not within

the allowable range.  The IBMTRDBO LAN driver did not

load.

Action: Correct the value of the keyword displayed in the

message that was displayed above this error message,

and reload the IBMTRDBO LAN driver.

IBMTRDBO-xx-207: The IBM LANStreamer Adapter could not be reset.

Cause:  The IBMTRDBO LAN driver timed out while waiting for the

IBM LANStreamer Adapter to complete a hardware reset.  The

IBMTRDBO LAN driver did not load.

Action: Restart your computer and try to load the IBMTRDBO LAN

driver again.  If this error persists, contact your

network administrator or IBM support center.

IBMTRDBO-xx-209: The IBMTRDBO LAN driver failed to initialize the

IBM LANStreamer Adapter.

Cause:  There was a problem while initializing the IBM LANStreamer

Adapter.  There may be previous error messages above this

one or the IBM LANStreamer Adapter may not be responding.

Action: Correct any problems with error messages displayed

above this error message, and then reload the IBMTRDBO

LAN driver.

If this is the only error message that is displayed,

restart your computer and check the adapter by running

the adapter diagnostic tests located on the Option

Diskette that was shipped with the IBM LANStreamer Adapter.

Then, reload the IBMTRDBO LAN driver.  If this error

persists, contact your network administrator or IBM

support center.

IBMTRDBO-xx-210: Could not find an IBM LANStreamer Adapter in this computer.

Cause:  There is no IBM LANStreamer Adapter installed in this

computer.  The IBMTRDBO LAN driver did not load.

Action: Install an IBM LANStreamer Adapter in this computer if it

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  17

has a 80386 or higher microprocessor, and then reload the

IBMTRDBO LAN driver.

IBMTRDBO-xx-211: The IBM LANStreamer Adapter was not found in the specified

slot.

Cause:  There is no IBM LANStreamer Adapter in the slot specified

by the SLOT keyword.

Action: Correct the SLOT keyword so that it indicates a slot

number in which an IBM LANStreamer Adapter is located, and

then reload the IBMTRDBO LAN driver.

If there is an IBM LANStreamer Adapter in the specified slot,

contact your network administrator or IBM support center.

IBMTRDBO-xx-212: Insufficient memory available for configured transmit and

receive buffers.

Cause:  During initialization of the IBMTRDBO LAN driver, there

was insufficient memory available for the configured

number of transmit and receive buffers.

Action: Increase the number of buffers that the server operating

system can allocate by editing the STARTUP.NCF file on

the server.  Increase the Minimum Packet Receive Buffers

server parameter in the STARTUP.NCF file by the total

number of transmit and receive buffers that the IBMTRDBO

LAN driver is using.  Down, exit, and then restart the

NetWare Server.

If the IBM LANStreamer Adapter is installed in a 16-bit

slot in your computer, add or increase the following

parameter in the STARTUP.NCF file:

Set the Reserved Buffers Below 16 = total transmit and

receive buffers used by the IBMTRDBO LAN driver.

If this error still persists, unload the LAN drivers and

NetWare Loadable Modules (NLMs) that are loaded but are

not being used at the NetWare Server.

If this error still persists, decrease the number of

buffers that are being used by the IBMTRDBO LAN driver

by specifying the transmit and receive keywords for the

IBMTRDBO LAN driver.

See "Configurable IBMTRDBO LAN Driver Parameters"

for information about specifying keywords for the

operating system that you are using.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  18

IBMTRDBO-xx-213: The slot to use for IBM LANStreamer Adapter was not specified.

Cause:  The IBMTRDBO LAN driver prompted the user to select a

slot number that has an IBM LANStreamer Adapter installed

but no choice was made.

Action: None.  The IBMTRDBO LAN driver was not loaded.

IBMTRDBO-xx-215: The value specified for RXBUFFERS is not valid.  The

value has been changed to the next supported value.

Cause:  An incorrect value was specified for RXBUFFERS either on

the command line or in the IBMTRDBO.CFG file for the

server.  The IBMTRDBO LAN driver used the next lower

supported value from the one specified.

Action: None.  However, to keep this message from being displayed

when the IBMTRDBO LAN driver is loaded, change the value

specified by the RXBUFFERS keyword to one of the supported

values for your operating system.

See "Configurable IBMTRDBO LAN Driver Parameters"

for the supported values.

IBMTRDBO-xx-216: The value specified for TXBUFFERS is not valid.  The

value has been changed to the next supported value.

Cause:  An incorrect value was specified for TXBUFFERS either on

the command line or in the IBMTRDBO.CFG file for the

server.  The IBMTRDBO LAN driver used the next lower

supported value from the one specified.

Action: None.  However, to keep this message from being displayed

when the IBMTRDBO LAN driver is loaded, change the value

specified by the TXBUFFERS keyword to one of the supported

values for your operating system.

See "Configurable IBMTRDBO LAN Driver Parameters"

for the supported values.

IBMTRDBO-xx-219: An internal error was reported when the IBM LANStreamer

Adapter tried to insert into the ring.

Cause:  The open adapter command issued by the IBMTRDBO LAN

driver to insert into the ring failed.  An alert was

generated by the IBMTRDBO LAN driver.

Action: Restart the computer and try to load the IBMTRDBO LAN

driver again.  If this message continues to occur,

check the adapter by running the adapter diagnostic tests

located on the Option Diskette that was shipped with the

IBM LANStreamer Adapter.  If there is a problem with the

adapter hardware, contact your network administrator or

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  19

IBM support center.  If there are no problems with the

adapter hardware, report this problem to your IBM support

center.

IBMTRDBO-xx-220: The LANStreamer Adapter could not insert into the ring.

Cause:  The adapter performs a number of tests when it

attempts to insert into the ring.  This message

indicates that one of the tests failed and the

adapter could not insert into the ring.

Action: Restart the computer and try to load the IBMTRDBO LAN

driver again.  If this message continues to occur,

check the adapter by running the adapter diagnostic tests

located on the Option Diskette that was shipped with the

IBM LANStreamer Adapter.  If there is a problem with the

adapter hardware, contact your network administrator or

IBM support center.

IBMTRDBO-xx-221: The IBM LANStreamer Adapter received a disconnect frame

from another token-ring Adapter.

Cause:  The IBM LANStreamer Adapter received a disconnect frame

request that was sent by another token-ring adapter at

a remote station.  The IBM LANStreamer Adapter installed

in this computer disconnected from the ring.

Action: To insert the IBM LANStreamer Adapter back into the ring,

unload the IBMTRDBO LAN driver and then reload it.  If

this error persists, contact your network administrator

or IBM support center.

IBMTRDBO-xx-222: The IBM LANStreamer Adapter could not insert into the

ring because the ring is beaconing.

Cause:  The token-ring LAN segment to which the IBM LANStreamer

adapter is connected is beaconing.  The adapter could

not insert into the ring.  An alert was generated by

the IBMTRDBO LAN driver.

Action: The IBMTRDBO LAN driver will try to insert into the

ring three more times.  This alert condition is corrected

when the IBMTRDBO-NW-231 message is displayed.  If

this condition does not correct itself, contact

your network administrator or IBM support center.

IBMTRDBO-xx-223: The IBM LANStreamer Adapter failed to insert into the ring;

another adapter is using the same node address.

Cause:  The IBM LANStreamer Adapter could not insert into the ring

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  20

because another station is using the same value that was

specified for the node address parameter at your station.

Action: Change the node address parameter value specified for

your station.  This parameter is specified as NODE=

at the NetWare Server.

See "Configurable IBMTRDBO LAN Driver Parameters"

for more information about configuration parameters.

IBMTRDBO-xx-224: The IBM LANStreamer Adapter detected a wire fault error

and closed the adapter.

Cause:  The connection between the IBM LANStreamer Adapter and

the LAN has been broken.  The adapter or the adapter

cable may have developed a problem.

Action: Check the connection between the adapter and cable

by running the adapter diagnostic tests located

on the Option Diskette that was shipped with the

IBM LANStreamer Adapter.

IBMTRDBO-xx-225: The token-ring network is beaconing.

Cause:  The token-ring LAN segment to which the IBM LANStreamer

adapter is connected is beaconing.  The ring is

experiencing an error condition detected by the adapter

when there is either a signal loss (possibly caused by a

broken line) or no token circulating the ring within the

predefined time limit.

Action: Contact your network administrator.

IBMTRDBO-xx-228: A Micro Channel error occurred; the adapter was reset.

Cause:  An error was detected with the Micro Channel bus while

data was being transferred between the IBM LANStreamer

Adapter and the computer.  This condition is reported as

an alert.

Action: The adapter will retry the data transfer, and the

IBMTRDBO LAN driver Micro Channel Error statistics counter

will be incremented.

If this error persists, check the adapter by running the

adapter diagnostic tests located on the Option Diskette

that was shipped with the IBM LANStreamer Adapter, and check

your computer by running the hardware diagnostic tests.  If

this error still persists, contact your IBM support center

or place of purchase of the IBM LANStreamer Adapter.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  21

IBMTRDBO-xx-229: The IBM LANStreamer Adapter reported a hardware error.

Cause:  The IBM LANStreamer Adapter reported an adapter hardware

error.  The adapter was closed.

Action: The IBMTRDBO LAN driver will increment the Adapter

Hardware Error counter, issue an alert, and try to reset

the adapter.  The IBM LANStreamer Adapter and IBMTRDBO

LAN driver may recover from this error.  If not, run

the adapter diagnostic tests located on the Option

Diskette that was shipped with the IBM LANStreamer Adapter.

If there are no problems with the adapter, reload the

IBMTRDBO LAN driver.  If a problem was found, follow

the instructions supplied with the diagnostic tests to

correct the problem.

IBMTRDBO-xx-231: The alert condition was corrected.

Cause:  The IBM LANStreamer Adapter, IBMTRDBO LAN driver, or

token-ring network have recovered from a previously

reported alert condition.

Action: None.

IBMTRDBO-xx-232: The IBM LANStreamer Adapter detected a parity error.

Cause:  The IBM LANStreamer Adapter detected a parity error

while data was being transferred between the IBM

LANStreamer Adapter and the computer.

Action: The IBMTRDBO LAN driver will increment the Adapter

Detected Parity Error counter, and this condition

is reported as an alert.  The IBM LANStreamer Adapter

and IBMTRDBO LAN driver may recover from this error.

If not, run the adapter diagnostic tests located on

the Option Diskette that was shipped with the IBM

LANStreamer Adapter.  If there are no problems with

the adapter, reload the IBMTRDBO LAN driver.  If a

problem was found, follow the instructions supplied

with the diagnostic tests to correct the problem.

IBMTRDBO-xx-233: The IBM LANStreamer Adapter reported a RAM access violation.

Cause:  The IBM LANStreamer Adapter reported a RAM access

violation.  The IBMTRDBO LAN driver may have tried to

access RAM on the LANStreamer adapter that was not

accessible.

Action: The IBMTRDBO LAN driver will increment the Adapter

RAM Access Error counter, and this condition is reported

as an alert.  The IBM LANStreamer Adapter and IBMTRDBO

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  22

LAN driver may recover from this error.  If not, run the

adapter diagnostic tests located on the Option Diskette that

was shipped with the IBM LANStreamer Adapter.  If

there are no problems with the adapter, reload the

IBMTRDBO LAN driver.  If a problem was found, follow

the instructions supplied with the diagnostic tests to

correct the problem.

ODI for IBM LANStreamer Adapter, Version 1.0   March 26, 1993  23

+-------------------------------------------------------+

|                       HEADINGS                        |

+-------------------------------------------------------+

__      ____                ____    __________          ID      FILE                PAGE    REFERENCES

ODITI   ODIREAD SCRIPT

1        ODI for IBM

LANStreamer Adapter,

Version 1.0   March

26, 1993

INST    ODIINST

2       IBMTRDBO LAN Driver

Installation

Procedure

MIB     ODIMIB2

5       Configurable IBMTRDBO

LAN Driver

Parameters

8, 18, 19, 19,

21

IBMCFG  ODIMIB2

7       Sample IBMTRDBO.CFG

Configuration File

for NetWare Server

4, 5, 5

PERF    ODIPERF

8       IBMTRDBO LAN Driver

Performance Tuning

COUNT   ODICOUNT

9       IBM LANStreamer

Adapter Custom

Statistic Counters

MSGS    ODIMSGS

12      Messages for the

IBMTRDBO LAN Driver

+-------------------------------------------------------+

|                      LIST ITEMS                       |

+-------------------------------------------------------+

__      ____                ____    __________          ID      FILE                PAGE    REFERENCES

DIR3    ODIINST

3       1

CPY3    ODIINST

3       2

3

+-------------------------------------------------------+

|                       REVISIONS                       |

+-------------------------------------------------------+

__      ____                ____    __________          ID      FILE                PAGE    REFERENCES

R1      ODIREAD SCRIPT

1

5, 5, 5

R2      ODIREAD SCRIPT

1

7

R3      ODIREAD SCRIPT

1

R4      ODIREAD SCRIPT

1

+-------------------------------------------------------+

|                  PROCESSING OPTIONS                   |

+-------------------------------------------------------+

Runtime values:

Document fileid ........................ ODIREAD SCRIPT

Document type .......................... USERDOC

Document style ......................... IBMXAGD

Profile ................................ EDFPRF30

Service Level .......................... 0029

SCRIPT/VS Release ...................... 4.0.0

Date ................................... 93.03.08

Time ................................... 01:50:09

Device ................................. 3270

Number of Passes ....................... 3

Index .................................. NO

Formatting values used:

Annotation ............................. NO

Cross reference listing ................ YES

Cross reference head prefix only ....... NO

Dialog ................................. LABEL

Duplex ................................. NO

DVCF conditions file ................... (none)

DVCF value 1 ........................... (none)

DVCF value 2 ........................... (none)

DVCF value 3 ........................... (none)

DVCF value 4 ........................... (none)

DVCF value 5 ........................... (none)

DVCF value 6 ........................... (none)

DVCF value 7 ........................... (none)

DVCF value 8 ........................... (none)

DVCF value 9 ........................... (none)

Explode ................................ NO

Figure list on new page ................ YES

Figure/table number separation ......... YES

Folio-by-chapter ....................... NO

Head 0 body text ....................... (none)

Head 1 body text ....................... (none)

Head 1 appendix text ................... Appendix

Hyphenation ............................ NO

Justification .......................... NO

Language ............................... ENGL

Layout ................................. 1

Leader dots ............................ YES

Master index ........................... (none)

Partial TOC (maximum level) ............ 4

Partial TOC (new page after) ...........  PAGE

Print example id's ..................... NO

Print cross reference page numbers ..... NO

Process value .......................... (none)

Punctuation move characters ............ .,

Read cross-reference file .............. (none)

Running heading/footing rule ........... NONE

Show index entries ..................... NO

Table of Contents (maximum level) ...... 3

Table list on new page ................. YES

Title page (draft) alignment ........... RIGHT

Write cross-reference file ............. (none)

+-------------------------------------------------------+

|                      Imbed Trace                      |

+-------------------------------------------------------+

Page 2        ODIPLAT

Page 2        ODIINST

Page 4        ODIMIB2

Page 7        ODIPERF

Page 8        ODICOUNT

Page 11       ODIMSGS