This README file of IDE286.ZIP accompanies IDE.OBJ and IDE.DSK for

NetWare v2.X. The following files are included for this revision:

IDE      OBJ     5705   8-09-91  11:39a

IDE      DSK      963   8-12-91   9:24a

Installation Instructions:

1) (NetWare v2.2) Copy the IDE.DSK to the SYSTEM-1 diskette

(NetWare v2.1x) Copy the IDE.DSK to the AUXGEN diskette

2) (NetWare v2.x) Copy the IDE.OBJ to the DSK_DRV_001 diskette.

3) (NetWare v2.2)  Run  INSTALL

(NetWare v2.1x) Run  NETGEN

Notes:

The IDE.OBJ file is a NetWare 2.x disk driver which makes it

possible to use IDE/ATA type disk drives with file servers using

the IDE/ATA interface.  The new IDE disk driver is very similar to

the old Netware ISA disk driver.  However, one major difference

must be noted.

The ISA disk driver gets the drive descriptions from a table

pointed to by Interrupt 41.  This table contains a list of disk

drives used with the old AT type machines.  However, not all disk

drives are listed in the table and these tables vary with different

BIOS applications.

The IDE disk driver was designed to eliminate this problem.  The

IDE disk driver does not get the drive description from a table.

It gets the drive descriptions directly from the disk drives at

initialization time.

When using the IDE disk driver, the following limitations must be

observed:

*    Non-Dedicated NetWare will not work properly on IBM

Model 35 or IBM Model 40 machines.   Data read

from, or written to floppy disks will be

corrupted.  This problem will be corrected in the

future.

*    The drive type number reported by the INSTALL

program is generated by the IDE disk driver at

initialization time and is useful only to the

NetWare operating system.

*    The IDE disk driver will not work with ISA bus

controllers (i.e. ST506 and ESDI).  The IDE/ATA,

ST506, and ESDI interfaces look similar to the

system software, but are handled differently by the

drivers.

*    IDE/ATA, ST506, and ESDI hardware is not

interchangeable.

*    The IDE disk driver supports multiple disk drives

of the same or different types.

Note:  Although the ISA disk driver appears to work with IDE/ATA

disk drives, it will not handle errors on these drives correctly.