Installing NDIS Drivers for Microsoft (tm) LAN Manager (tm) 2.0

===============================================================

General

The Kodiak~8 and Kodiak~16 software package includes LAN

Manager-compatible NDIS software drivers for DOS-based

workstations and OS/2 based servers and workstations.

NDIS Driver Installation (Both DOS and OS/2)

1.   Install LAN Manager (version 2.0 or later)

Follow the procedure in the Microsoft LAN Manager

documentation

2.   Change to the LANMAN directory on the C:drive

3.   Run Setup by Typing

Setup <enter>

4.   Load the Kodiak~8 or Kodiak~16 driver as follows:

a)   Choose the Configuration menu

b)   Choose the Network Driver menu

c)   Select "Other"

d)   Insert the Kodiak Distribution Diskette in the A

Drive.

e)   Select the Kodiak Driver that matches your adapter

type.

f)   Complete the procedure in the LAN Manager

documentation.

Changing the Base Address and IRQ configuration

Kodiak adapters are shipped with a factory default setting of

Address 300, Interrupt 3.  To change this, the following

procedure must be followed:

1.   Change to the LANMAN directory and locate the

Protocol.INI file

2.   Edit the Protocol.INI file for the address and interrupt

settings you wish.  A list of supported base addresses

and interrupts for the Kodiak adapters is contained in

the Installation Guide.

3.   Reboot your system

NOTE:     Because of the multi-tasking nature of OS/2, make

sure that you have shut down all applications and

closed all files before rebooting your system.