Using Kodiak's NetBIOS Drivers

==============================

Before using the NetBIOS drivers, refer to the installation guide

for your particular network operating system.

The Kodiak NetBIOS v1.10 consists of 2 files. They are:

netk08.exe for Kodiak~8 adapters

netk16.exe for Kodiak~16 adapters

The command format for NetBIOS v1.10 is:

NETk08 [/Ixx] [/Pxxx] [/Ax] [/Nxx] [/Sxx] [/Exxxx] [/K]

or

NETk16 [/Ixx] [/Pxxx] [/Ax] [/Nxx] [/Sxx] [/Exxxx] [/K]

where

/Ixx      This option specifies the interrupt signal used by

the adapter.

For the Kodiak~8, the default is 3. Valid options

are:  2, 3, 5.

For the Kodiak~16, the default value is 3. Valid

options are: 3, 5, 9, 10, 11, 12, and 15.

/Pxxx     This parameter specifies the base I/O port used by

the adapter.

The default is 300.  Valid options are 200, 210,

300, 310.

/Ax       This option specifies the adapter number to which

this NetBIOS is configured.  This specification

allows installation of multiple Kodiak cards in one

system, when supported by the Network Operating

System (NOS).  Some NOS allow multiple adapters in

one computer (e.g.,Artisoft's LANtastic NOS).

Consult your NOS manual for more information on

multiple network cards.

The default is 0 (the first card installed).  Valid

options are 0, 1, 2, and 3.

/Nxx      This parameter specifies the maximum number of NCBs

supported by this NetBIOS.  NCBs are command blocks

used by applications or NOS software to pass

commands to NetBIOS.  Settingthe maximum number of

NCBs limits the number of commands that an

application or NOS can issue to NetBIOS.  The

program occupies less memory when configured for

fewer NCBs. The default value is 32.  Valid options

are    1 - 32.

/Sxx      This option specifies the maximum number of

sessions supported by this NetBIOS.  Sessions are

connections over which data can be sent.  Most NOS

require that at least one session per user be

supported (i.e., the more users supported by the

NOS, the more sessions required).  NetBIOS occupies

less memory when configured for fewer sessions.The

default is 32.  Valid options are 1 - 32.

/Exxxx    This parameter specifies the Ethernet protocol type

to use when transmitting and receiving frames.  It

is used by NetBIOS to differentiate its own frames

from other protocol frames when several protocols

are running on the same network.  All stations must

use the same Ethernet type in order to communicate.

No Ethernet types are needed if the entire network

uses only NetBIOS. The default is No Ethernet Type.

Valid options are 0 - FFFF.

/K        This is a special option used for IBM PC LAN NOS.

If you are running IBM PC LAN with Kodiak NetBIOS

v1.10, this switch is recommended.

Where options are not specified default values are used.

Examples :

NETK08              ; Load NetBIOS with default

or                  ; interrupt signal 3, base I/O port

NETK16              ; 300, adapter 0 (the first adapter

; installed), 32 sessions, 32 NCBs,

; and no Ethernet type.

NETk08 /I2 /P310    ; Load NetBIOS for one adapter with

; interrupt signal 2 and I/O port

; 310.

NETK16 /I5 /P200 /A0 ; Load NetBIOS for two adapters

NETK16 /I15 /P210 /A1; with the first adapter using

; interrupt signal 5 and base I/O

; port 200, and the second adapter

; using interrupt signal 15 and

; base I/O port 210.

NETK08 /N12 /S12    ; Load NetBIOS with fewer sessions

or             ; and NCBs.  This lowers the amount

NETK16 /N12 /S12    ; of memory required by NetBIOS.

; This configuration is good for

; workstations since NOS require

; fewer sessions for workstations.

NETk08 /E3875 /K    ; Load NetBIOS with Ethernet

or             ; protocol type 3875 and running

NETK08 /E3875 /K    ; under IBM PC LAN NOS.

NOTE: Version 1.10 of the Kodiak NetBIOS works with LanTastic

versions prior to Version 4.0. A LanTastic-specific driver is

required for Versions 4.0 and above. Contact Kodiak Customer

Service for the availability of this driver.