TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

TNDIS PROTOCOL Parameter Descriptions

---------------------------------------------------------------------------

The following parameters are generic to any model of the EtherStar-based

LanCard/E.

KeyWord       Minimum Maximum Default   Description / Comments

------------  ------- ------- -------   -----------------------------------

DriverName                     TNDIS$   REQUIRED Parameter.

Up to nine copies of this driver

may be loaded into memory at once,

one for each LanCard/E supported.

The default name specified will

apply to the first copy of the

driver.  Additional copies should

use the following names in the

order specified:

TNDIS2$, TNDIS3$, TNDIS4$, TNDIS5$,

TNDIS6$, TNDIS7$, TNDIS8$, TNDIS9$

MaxMultiCast        1    1000      10   Maximum number of entries allowed

in the MultiCast Address Table.

MaxRequests         1     400       6   Maximum number of General Requests

which can be queued by the driver.

MaxTransmits        1     142       6   Maximum number of Transmit Requests

which can be queued by the driver.

MaxReceives         1     400       6   Maximum number of Received Frames

which can be queued by the driver.

ReceiveBuffers      1     400       6   Maximum number of Receive Buffers

which are available for temporary

storage of incoming packets.

ReceiveBufSize    256    1514     256   Size of an individual Receive

Buffer.

___________________________________________________________________________

Document:  NDIS.DOC          December 7, 1990                        Page 1

TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

TNDIS PROTOCOL Parameter Descriptions

---------------------------------------------------------------------------

The following parameters are specific to the particular version of

LanCard/E.

ISA-bus:  Industry Standard Architecture.

MCA-bus:  Micro Channel Architecture.

KeyWord       Minimum Maximum Default   Description / Comments

------------  ------- ------- -------   -----------------------------------

Interrupt           2      15       3   Interrupt Request Level for ISA-bus

machines.

For allowable values, consult the

appropriate owner's manual.

IOAddress           0 0x0FFE0   0x300   Input/Output Base Address for ISA-

bus machines

For allowable values, consult the

appropriate owner's manual.

Slot                0       8       0   Slot Number for MCA-bus machines.

If the Slot Number is set to zero,

or unspecified on a MCA-bus

machine, the driver will execute a

search algorithm scanning from slot

1 to 8 in an attempt to identify a

slot containing a LanCard/E*MC.

If the above search is

unsuccessful, or the specified slot

does not contain a LanCard/E*MC,

the driver will use the IOAddress

and Interrupt parameters just as

though it were operating in an ISA-

bus machine.

___________________________________________________________________________

Document:  NDIS.DOC          December 7, 1990                        Page 2

TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

TNDIS PROTOCOL Parameter Descriptions

---------------------------------------------------------------------------

For those familiar with the 3Com/Microsoft NDIS v2.01 specification:

This MAC-level driver uses "ReceiveChain" calls exclusively.

In terms of the above PROTOCOL.INI specifications, this means that

driver performance will depend upon the timely release of receive

resources by the upper-level Protocol.

Receive resources are divided into:

ReceiveChainBlocks (allocated by the MaxReceives parameter) which

determine the maximum number of received packets which may be

queued simultaneously by the MAC driver.

ReceiveBuffers (allocated by ReceiveBuffers times ReceiveBufSize)

which also places a limit on the maximum number of received

packets which may be queued simultaneously by the MAC driver.

For each incoming packet, the MAC (TNDIS) allocates receive resources

is as follows:

1.   Attach a ReceiveChainBlock, if this fails the MAC is out of

resources, and it will wait until a ReceiveChainBlock

becomes available before continuing to the next step.

2.   Attach a ReceiveBuffer, if this fails, the MAC is out of

resources, and it will wait until a ReceiveBuffer becomes

available.

3.   Transfer up to ReceiveBufSize bytes from the LanCard/E

receive buffer to the attached buffer.

4.   If more data remains to be copied, go back to step 3.

The impact of the above procedure is that if the upper-level Protocol

fails to release attached ReceiveChainBlocks and ReceiveBuffers, the

MAC will stop transferring received packet data from the LanCard/E

until the necessary resources become available.

This stoppage will be reflected in various ways; in severe cases, all

communications with the Server and other Work-Stations will cease; in

milder cases, network performance will slow due to the constant re-

transmission of packets which will seem to have been lost by the

receiver.

Therefore, the user should assure that sufficient receive resources

are allocated so as to reduce the possibility of receive resource

overrun.

___________________________________________________________________________

Document:  NDIS.DOC          December 7, 1990                        Page 3