TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

NDIS Device Driver Installation

- for -

Microsoft Lan Manager Versions 1.10 and 2.00

---------------------------------------------------------------------------

Please note that these instructions assume that the user is already

familiar with the requirements of the particular environment in terms of

workstation setup and software generation.  That is, these instructions

only deal with the basic issue of merging the TIARA device driver into the

particular environment so that workstation generation may then be performed

by the user.

Copyright/Trademark Information:--

Microsoft is a registered trademark of Microsoft Corporation.

OS/2 is a registered trademark of International Business Machines

Corporation.

General Instructions:--

In the instructions which follow, "S:" is used to designate the

(SOURCE) drive containing the TIARA driver distribution diskette,

and "T:" is used to designate the (TARGET) drive to which the

particular item is to be installed.

For example, if the TIARA diskette is in drive "A:" and the user

is installing to drive "B:", then the user should substitute "A:"

for "S:" and "B:" for "T:" as follows:

Original command:   "COPY S:FILENAME T:"

Modified command:   "COPY A:FILENAME B:"

Installation instructions for version 2.00

The MSLANMAN.DOS and MSLANMAN.OS2 directories have been constructed in

accordance with the requirements of the Lan Manager SETUP utility

Action menu Import function.

___________________________________________________________________________

Document:  LANMAN.DOC        December 7, 1990                        Page 1

TIARA Computer Systems, Inc.

(C) Copyright 1990.  All Rights Reserved.

---------------------------------------------------------------------------

NDIS Device Driver Installation

- for -

Microsoft Lan Manager Versions 1.10 and 2.00

---------------------------------------------------------------------------

Installation instructions for version 1.10

OS/2 diskette preparation.

1.   Use the DISKCOPY command to make a duplicate of the OS/2 NET

TRANSPORTS & NDIS DRIVERS diskette for steps 2 and 3.

2.   Create the TIARA device driver directory:

MD T:\LANMAN\DRIVERS\TNDIS

3.   Copy the TIARA driver files to the above directory:

COPY S:\MSLANMAN.OS2\DRIVERS\ETHERNET\TNDIS\TNDIS.OS2

T:\LANMAN\DRIVERS\TNDIS

COPY S:\MSLANMAN.110\OS2\TNDIS.NIF T:\LANMAN\DRIVERS\TNDIS

COPY S:\MSLANMAN.110\OS2\PROTOCOL.INI

T:\LANMAN\DRIVERS\TNDIS

DOS diskette preparation.

1.   Use the DISKCOPY command to make a duplicate of the DOS NET

TRANSPORTS & NDIS DRIVERS diskette #2 for steps 2 and 3.

2.   Create the TIARA device driver directory:

MD T:\LANMAN\DRIVERS\TNDIS

3.   Copy the TIARA driver files to the above directory:

COPY S:\MSLANMAN.DOS\DRIVERS\ETHERNET\TNDIS\TNDIS.DOS

T:\LANMAN\DRIVERS\TNDIS

COPY S:\MSLANMAN.110\DOS\TNDIS.NIF T:\LANMAN\DRIVERS\TNDIS

COPY S:\MSLANMAN.110\DOS\PROTOCOL.INI

T:\LANMAN\DRIVERS\TNDIS

___________________________________________________________________________

Document:  LANMAN.DOC        December 7, 1990                        Page 2