Readme file for LDREMOTE.NLM

============================

LDREMOTE is an NetWare Loadable Module to automatically load REMOTE.NLM

with a password. Normally REMOTE.NLM prompts the user for a password

when loaded, or the password should be part of the NCF file that loads

REMOTE (e.g. autoexec.ncf).

Putting the password in the autoexec.ncf file in readable form is a

possible security problem if users see the file when the system manager

maintains the server.

LDREMOTE stores the password for REMOTE in the bindery and retrieves the

password from there, when REMOTE must be loaded. The password is stored

in the bindery as RCONSOLE property of the user SUPERVISOR. So only the

supervisor and supervisor equivalent users can see the remote password.

(You will need a bindery scanner for that). Any users knowing the password

can still use rconsole. LDREMOTE is not a replacement for REMOTE.NLM,

LDREMOTE will only run to retrieve the password from the bindery and then

load REMOTE.NLM.

Instructions:

=============

1. Copy LDREMOTE.NLM to the SYS:SYSTEM directory

2. Edit your AUTOEXEC.NCF file and replace the "LOAD REMOTE" command

with the command "LOAD LDREMOTE". Remove the optional password from the

load command.

Also remove the LOAD RSPX command from the AUTOEXEC.NCF file. LDREMOTE

will load RSPX for you.

3. To store the password for remote console in the bindery, type

"LOAD LDREMOTE <password>" at the server console.

Repeat step 3 when you want to change the remote password.

DISCLAIMER

LDREMOTE is given to the Public Domain by Bart Mellink of Cyco Automation in

the Netherlands. Cyco Automation is a systems house in the Netherlands

that implements CAD-systems, Networks and DocumentManagement systems.

Also the maker of WHO386.EXE, WHO_NLM.NLM, SHOWEVNT.NLM and ELRDCHK.NLM.

Although it has been tested on a number of different servers and uses

only standard Novell published API's, it carries no warrantee whatsoever.

For any questions: Bart Mellink compuserve ID: 75170,1156

Cyco disclaims and excludes any and all implied warranties of merchantabili-

ty, title and fitness for a particular purpose. Cyco does not warrant that

the software will satisfy your requirements or that the software is without

defect or error or that operation of the software will be uninterrupted.

You are using the software at your risk.

Revision history:

=================

1.0  march 26, 1993

Initial version

1.1  april 9, 1993

Fixed a problem when loading LDREMOTE and RSPX in the same NCF file.

Since NetWare, while executing NCF files, does not wait for a NLM to

terminate before the next command in the NCF file is executed, loading

LDREMOTE and RSPX in the same NCF file failed, because LDREMOTE was

not yet ready loading REMOTE at the moment the NCF file wanted to

load RSPX.

The current version of LDREMOTE will load both REMOTE.NLM and RSPX.NLM

and make sure REMOTE.NLM is loaded first.