NAVYTIME calls the Naval Observatory in Washington, DC for the

correct time, and sets your computer's time accordingly.  NAVYTIME

sets your system clock, the real-time clock in an AT, and optionally

it can set the clock of a Novell file server if run from a workstation

signed on as the supervisor.

NAVYTIME was written by Zedak Corp of New York City, and may be used

and copied freely.

syntax:  NAVYTIME [parmfilename] [/w]

or NAVYTIME ? or NAVYTIME /help  for a help message

parmfilename is the name of a parameter file.

if a parmfile is not specified, NAVYTIME looks for one

called NAVYTIME.CFG in the current directory.

/w means write a default parameter file.

it is written to parmfilename or to NAVYTIME.CFG

if no parmfilename was specified.

The parameters file is an ASCII text file with one line

per parameter. Each line consists of a keyword followed

by a blank followed by a parameter.  Keywords all start

with a period.  The keywords are:

1)  .DialString  is a command string to cause the modem to

dial the Naval Observatory.  The string can be whatever

it takes to make your modem do its thing.  The default

string is suitable for a Hayes 2400 baud modem or

compatible.  The following command sequences are recognized

by NAVYTIME when embedded in the dial string, and are handled

specially during dial command transmission:

\d  quit if the modem Data Set Ready signal is not active

\c  quit if the modem has not sent back any chars yet

\1 thru \9  delay n timer ticks (18ms each)

\n emit linefeed (hex 0A)

\r emit carriage return (hex 0D)

\b emit blank (trailing blanks are trimmed in dial string)

2) .HangUp  a control string to cause the modem to hang up.

This is not really needed for a Hayes because the default dial

string will cause the Hayes to hang up when NAVYTIME drops the

Data Terminal Ready signal, but some compatibles don't have that

feature.

3) .ConnectMsgs  a set of messages to be recognized as an indication

that the modem has connected.  The messages are started, ended,

and delimited by slashes.  For a Hayes or compatible, CONNECT is

the standard message, so /CONNECT/ is a good parameter.

3) .FailMsgs  a set of messages to be recognized as an indication

that the modem has failed to connect.  The messages are started,

ended, and delimited by slashes.  For a Hayes or compatible,

/NO CARRIER/BUSY/NO DIALTONE/ERROR/ all indicate we are not

going to get through.

4) .CharDelay  A number of ticks to delay between characters of

dial command string.  Some modems cannot accept command strings

at full speed (like MicroCom), so CharDelay should be set to 1

for them.  For a Hayes, 0 is fine.

5) .ComPort may be 1 or 2 for COM1 or COM2.

6) .TimeZone  The number of hours to add to a Zulu time to

compute local time.  For Eastern Standard Time, -5 is correct.

for Pacific Standard, -8 is correct.

7) .Daylight  y if Daylight Savings is in effect.

8) SetServer  y if the computer is a workstation on a Novell

Network and NAVYTIME should set the file server's time as

well.  This feature requires that the workstation be signed

on with supervisor authority.

Default Parameters

.DialString \dATQ0V1S11=50S12=3&C1&D2L1 DT 1 202 653 0351\r\6

.HangUp \6+++\6ATH\r

.ConnectMsgs /CONNECT/

.FailMsgs /NO CARRIER/BUSY/NO DIALTONE/ERROR/

.CharDelay 0          1 = slow modem commands (microcom

.ComPort 1            Com 1 or Com 2 are supported

.TimeZone -5          Hours to add to Zulu for local

.Daylight n           Daylight Savings time

.SetServer y          y means set Novell Server clock

(requires supervisor privilege)