This README file of NDSCSI.ZIP accompanies NDSCSI.COM v1.01.  The

following files are included for this revision:

NDSCSI.COM  v1.01    655   8-14-91     8:33a

PS2SCSI.OBJ v1.11    7405  8-22-91     2:02p

Purpose:

This patch addresses a problem found when a NetWare and a non-Netware device

are on the same SCSI bus.

This TSR is intended for use in the non-dedicated process of NetWare 286

v2.15c and v2.2 (5,10, 50, and 100 user versions).

NSCSI.COM is a TSR that fixes a bug that results in the 25th line Broadcast

message:  "Fatal dir error on vol SYS:," and data corruption. This bug is a

result of a conflict between non-Netware devices (such as a CDROM) and a

NetWare drive when they are both on the same SCSI bus. This TSR NDSCSI.COM

allows DOS and NetWare to access devices on the SCSI adapter.

Installation:

1)    Make a backup copy of the existing PS2SCSI.OBJ.

2)    Copy the new PS2SCSI.OBJ driver over the existing file.

3)    Run NETGEN (v2.15) or INSTALL (v2.2) to configure and

link the PS2SCSI.OBJ.

4)    While the server is up, run NDSCSI.COM at the DOS

prompt.  (NDSCSI may be invoked from the autoexec.bat.)

******************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to any

NetWare software, and specifically disclaims any express or implied

warranties of merchantability, title, or fitness for a particular purpose.

Distribution of any NetWare software is forbidden without the express

written consent of Novell, Inc.  Further, Novell reserves the right to

discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss if use of the

software, loss of data, costs of re-creating lost data, the cost of any

substitute equipment or program, or claims by any party other than you.

Novell strongly recommends a backup be made before any software is

installed.   Technical support for this software may be provided at the

discretion of Novell.

******************************************************************

*****************************************************************************

CAUTION

ALWAYS backup your system before implementing any program/utility revision

involving the low-level functions of NetWare including re-linking of

operating system .OBJ files, Bindery utilities, drive and volume operation,

etc.

*****************************************************************************