ODI for DOS

User's Guide

June 1991 Edition

Novell, Incorporated

122 East 1700 South

P.O. Box 5900

Provo, Utah  84606  USA

�   Copyright 1991 Novell, Inc.  All rights reserved.  No part

of this publication may be reproduced, photocopied, stored on

a retrieval system, or transmitted without the express prior

written consent of the publisher.

Novell Part # 100-000871-002

Disclaimer

Novell, Inc. makes no representations or warranties with respect to the contents or use of this

manual, and specifically disclaims any express or implied warranties of merchantability or

fitness for any particular purpose.  Further, Novell, Inc. reserves the right to revise this

publication and to make changes to its content, at any time, without obligation to notify any

person or entity of such revisions or changes.

Further, Novell, Inc. makes no representations or warranties with respect to any NetWare

software, and specifically disclaims any express or implied warranties of merchantability or

fitness for any particular purpose.  Further, Novell, Inc. reserves the right to make changes to

any and all parts of NetWare software, at any time, without obligation to notify any person or

entity of such changes.

Trademarks

Novell, Inc., has made every effort to supply trademark information about company names,

products, and services mentioned in this book.  Trademarks indicated below were derived

from various sources.

3Com, 3Com EtherLink, and EtherLink Plus are trademarks of 3Com Corporation.

AppleTalk is a registered trademark of Apple Computer, Inc.

IBM is a registered trademark of International Business Machines Corporation.

IBM PC AT/XT are trademarks of International Business Machines Corporation.

IBM Personal System/2 is a registered trademark of International Business Machines

Corporation.

Internetwork Packet Exchange (IPX) is a trademark of Novell, Inc.

LANtern is a trademark of Novell, Inc.

Micro Channel is a trademark of International Business Machines Corporation.

MS-DOS is a trademark of Microsoft Corporation.

NetWare and Novell are registered trademarks of Novell, Inc.

NetWire is a trademark of Novell, Inc.

Novell RX-Net is a trademark of Novell, Inc.

OS/2 is a registered trademark of International Business Machines Corporation.

Open Data-Link and ODI are trademarks of Novell, Inc.

PC-DOS is a trademark of International Business Machines Corporation.

Personal Computer AT and Personal Computer XT are trademarks of International Business

Machines Corporation.

Personal System/2 is a trademark of International Business Machines Corporation.

Token-Ring is a trademark of International Business Machines Corporation.

Windows is a trademark of Microsoft Corporation.

� Copyright 1991 Novell, Inc.  All rights reserved.  No part of this

publication may be reproduced, photocopied, stored on a retrieval system, or

transmitted without the express prior written consent of the publisher.

Novell, Incorporated

122 East 1700 South

Provo, Utah  84606  USA

June 1991 Edition

Novell Part # 100-000871-002

User Comments

We are continually looking for ways to make our products and our manuals as easy

to use as possible.  You can help us by sharing your comments and suggestions

about how our manuals could be made more useful to you and about any

inaccuracies or information gaps they may contain.

You can submit your comments either by filling out the �User Comments� form at

the end of this manual or by writing to us directly at the following address:

Novell, Inc.

Technical Publications

122 East 1700 South MS C-24-1

Provo, UT 84606 USA

We sincerely appreciate your comments about our products.

Table of Contents

How to Use This Manual                      iii

About DOS ODI                                 1

Installing ODI for DOS                        3

Before you begin                           3

Create the master workstation diskette     4

Maintenance tips                           6

NET.CFG Options                               7

Link Driver drivername                     9

Link Support                              20

Protocol protocol_name                    21

Appendix A:  Using the NetWare IPX Shell with ODI23

Installation                              23

Using NET.CFG                             26

Appendix B:  Using the Token-Ring Source Routing Driver29

Installing the source routing driver with DOS ODI30

Setting alternate parameters              31

Appendix C:  System Messages                 35

Index                                        61

How to Use This Manual

This manual explains how to install Novell's Open Data-Link

Interface (ODI) for DOS.  You may also need to consult the

documentation accompanying the protocols and drivers you are

using for product-specific information.

�About DOS ODI� offers an overview of ODI's function.

�Installing ODI for DOS� provides detailed information for

installing DOS ODI.

�NET.CFG Options� contains configuration information for the

NET.CFG file.

�Appendix A:  Using the NetWare IPX Shell with ODI� explains

how to configure DOS ODI to connect to a NetWare server using

IPX and the NetWare shell.

�Appendix B:  Using the Token-Ring Source Routing Driver�

explains how to install the Token-Ring Source Routing Driver

with DOS ODI.

�Appendix C:  System Messages� provides possible explanations

and corrections for system messages that you may encounter.

About DOS ODI

Open Data-Link Interface (ODI) adds functionality to network

computing environments by supporting multiple protocols and

multiple LAN adapters in a single workstation.

ODI creates a �logical network board� to allow multiple frame

formats over one network board and wire.

You can benefit from ODI in the following ways:

�  You can expand your network by using multiple protocols

(such as IPX/SPX, AppleTalk, LAT, or TCP/IP) without adding

network boards to the workstation.

�  You can communicate with a variety of workstations, file

servers, and mini and mainframe computers via different

protocol stacks without rebooting your workstation.

�  You can protect your LAN adapter investment, because all

protocols written to ODI specification can communicate

through any LAN adapter written to ODI specification.

�  You can spend less time and money on support.  With one

LAN driver supporting multiple protocols, you have fewer

hardware components to support.

�  You can use the NET.CFG file to configure the LAN driver for

any possible hardware configuration.

You convert a standalone personal computer into a DOS ODI

network workstation by adding one or more network boards,

cabling, the DOS ODI workstation software, and protocol suite

software.

Three sets of files allow workstations to �talk� to each other, to the

file server, and to other hosts.

LSL.COM

LAN drivers

(MLIDs)

Protocol stacks

The Link Support Layer file enables

the workstation to communicate using

several protocols.

Driver files such as NE2000.COM

and TOKEN.COM communicate

directly with the LAN boards.

Drivers are also called MLIDs

(Multiple Link Interface Drivers).

Files such as IPXODI.COM and

TCPIP.EXE manage communications

among network stations.

You may need additional files specific to the networking product

you are using.

The following figure illustrates one configuration that would

benefit from using ODI.  Because the LSL directs information to

the appropriate protocol stack, a user can use TCP/IP services as

well as IPX and AppleTalk services.

Installing ODI for DOS

This section explains how to install DOS ODI.  You may also need

to consult the documentation accompanying the protocols and

drivers you are using for additional information.

IPX users should see �Appendix A:  Using the NetWare IPX Shell

with ODI� of this manual.  TCP/IP users should see the LAN

WorkPlace for DOS Administrator's Guide.

Before you begin

What you need

�  A working copy of the DOS ODI Workstation Services diskette

�  A �disk� (formatted diskette or hard disk) for each workstation

to boot from

1.   Check the hardware and requirements.

Use any of the following computers as workstations:

�  IBM PC or compatible

�  IBM PC XT or compatible

�  IBM PC AT or compatible

�  IBM Personal System/2 (any model) or compatible

2.   Make sure that the network board conforms to your cabling topology (such as

ARCnet, Token-Ring, or Ethernet) and that you have a DOS ODI LAN driver for

the board.

For a listing of drivers supplied by Novell, see the DOS ODI

Workstation Services diskette or the driver list on page 10.  If an

appropriate driver is not provided by Novell, you must obtain one

from the board's vendor.

Create the master workstation diskette

The master workstation diskette contains copies of the files

necessary to boot one type of workstation.  In this section, you will

copy files from the DOS ODI Workstation Services diskette to the

master workstation diskette.

Then you will either make a workstation boot diskette for each

workstation or copy the files from the master workstation diskette

to the hard disk from which a workstation boots.

1.   Copy the workstation files from the DOS ODI Workstation Services diskette to

the master workstation diskette.

The following files are required to communicate with a server or

host:

�  LSL.COM

�  The LAN drivers for your network boards (NE2000.COM, for

example)

The following files are optional:

Note:The LANSUP.COM driver should be used on workstations using

the IBM LAN Support Program.

2.   Put the filenames you need into a batch file on the master workstation diskette.

Load the LSL and then the LAN drivers.  Your batch file should

look similar to the following:

LSL

NE1000

Note:If you are using a 3COM 3C503 board and want to use DIX

cabling, you must specify the DIX connector setting in the

NET.CFG file (see page 19) or at the command line when you load

the driver by entering a D, as follows:

3C503 D

BNC/TP, the default connector setting, does not require command

line information.

3.   Create a NET.CFG file on the master workstation diskette (optional).

You must create a NET.CFG file if, for example, you have changed

hardware options on your network board or you are using multiple

protocols.

See the following section, �NET.CFG Options,� (on page 7) for

more information.  You should also see the documentation specific

to your protocol.

4.   Copy protocol suite files to the master workstation diskette or run the

necessary install utilities that came with your protocol suite.

Refer to the documentation specific to your protocol.

5.   Use the master diskette to connect to the file server or host.

6.   Create the workstation boot disk.

Copy the master workstation diskette files to each workstation

boot diskette or hard disk.  From each workstation, connect to the

file server or host.

Maintenance tips

1.   If you cannot communicate on the network, check to see if

�  The network board is seated properly.

�  Board settings match the values displayed when you loaded

the LAN driver.

�  Board settings do not conflict with settings for other devices in

the machine.

�  The cable is connected properly.

�  All cabling is terminated properly.

�  The workstation node address does not conflict with any other

node address on the same physical network.

2.   If you want information about a DOS ODI executable, type its name followed by

a question mark.

For example

NE2000 ?  <Enter>

or

LSL.COM ?  <Enter>

3.   If you unload ODI drivers from memory, unload them in reverse order.

For example, if you work with two protocols that conflict when

both are running concurrently on the workstation or if you need to

free memory for large applications, unload the first set of files (in

reverse order) before you load the next set.

Unload the workstation files as follows:

PROTOCOL u  <Enter>(for example, IPXODI u)

DRIVER u  <Enter>(for example, NE2000 u)

LSL u  <Enter>

If you do not unload the files in reverse order, you get an error

message similar to this: �FATAL: There is a TSR above the loaded

NE1000.�

NET.CFG Options

The NET.CFG file is a configuration file that contains section

headings and options that deviate from the established defaults of

the ODI software.  If you changed the default hardware settings

on the LAN adapter or if you are using multiple protocols, you

probably need a NET.CFG file.

You may also need to see the documentation specific to your

protocol for additional NET.CFG information.

Use any DOS text editor to create the file.  Specify only options

that will change from the defaults.

Conventions

Main section headings must be left-justified and are not case

sensitive.  The heading must precede the options you want to

include in that section.  Options are not case sensitive and must

be preceded by a tab or hard spaces.

Precede comments with a semicolon (;).  End each line with a hard

return.  Write all numbers in decimal notation except where noted

otherwise.

The following are common main section headings in the NET.CFG

file:

�  Link driver

�  Link support

�  Protocol

Sample NET.CFG file

LINK DRIVER NE1000

; Change the interrupt (IRQ) to 5.

INT 5

; Change the port to 320 (hex)

PORT 320

The following sections contain an explanation of each option's

function and some possible reasons for changing the setting.  In

the explanations, these conventions are used:

[ ]         An optional element

number      A decimal number

hex         A hexadecimal number

The following chart lists the options defined by the DOS ODI

software.  Other options may be available for the LAN driver or

protocol stack you are using.  Refer to their documentation for

more information.

Main section headings are in white print and flush with the left

margin.  Options are listed under each heading and indented.

Link Driver drivername

The following table outlines the NET.CFG options available to

LAN drivers on the DOS ODI Workstation Services diskette.

Square bullets (�) indicate options available to both ISA (industry

standard architecture) and MCA (microchannel architecture).

Options marked with an I are available to ISA only; options

marked with an M are available to MCA only.

Note:If you are using a LAN driver not listed above, refer to its

documentation for parameter applicability.

To use this heading, replace drivername with the name of the

driver you are using; the name is typically the LAN driver's

filename.

Replace drivername with one of the following driver names:

�  3C501 (for 3COM EtherLink)

�  3C503 (for 3Com EtherLink Series II)

�  3C505 (for 3COM EtherLink Plus)

�  3C523 (for 3Com EtherLink/MC)

�  EXOS (for Novell EXOS205 and EXOS215)

�  LANSUP (for the IBM LAN Support program only)

�  NE2 (for Novell Ethernet NE/2)

�  NE2-32 (for Novell Ethernet NE/2-32)

�  NE1000 (for Novell Ethernet NE1000)

�  NE2000 (for Novell Ethernet NE2000)

�  NE2100 (for Novell Ethernet NE2100)

�  PCN2L (for IBM PC Network II and II/A)

�  TOKEN (for IBM Token-Ring)

�  TRXNET (for Novell RX-Net and Turbo RX-Net)

After specifying a drivername, place both hardware commands

and software commands under the Link Driver heading.

You can specify options for each network board you are using, but

you must have a separate �Link Driver drivername� heading for

each network board.

Hardware commands

DMA [#1 | #2] channel_number

This option specifies the hardware setting of the network board

used in the workstation.  It allows DMA channels to be

configured.

Enter the channel number to be used.  The default is the first

configurable channel, (#1).

You do not need to include the #1 option if you are using only the

default option.  For example, if the first configurable DMA

channel on your board uses DMA channel 3, place the following

lines in your NET.CFG file:

Link Driver name

DMA 3

If, however, you are using the first and second DMA channels, you

do need to include the # sign for the second channel.  For example,

if the first configurable DMA channel on your board will use

channel 3 and the second configurable channel will use channel 4,

place the following lines in your NET.CFG file:

Link Driver name

DMA 3

DMA #2 4

INT [#1 | #2] interrupt_request_number

This option specifies which interrupt the network board uses.  Use

the interrupt request number set on the board.

The default is the first configurable interrupt line, (#1).  Do not

include the #1 option if you are using only the default option.

For example, if the first configurable interrupt line on your board

will use IRQ 2, place the following lines in your NET.CFG file:

Link Driver name

Int 2

If your board can use two configurable interrupt lines and you

want to use both of them or only the second one, you must specify

the interrupt line (#1 or #2) to configure.

For example, if the first configurable interrupt line on your board

will use IRQ 2 and the second will use IRQ 3, place the following

lines in your NET.CFG file:

Link Driver name

Int 2

Int #2 3

The second INT channel requires you to use the pound sign (#).

MEM [#1 | #2] hex_starting_address [hex_length]

This option specifies a memory range to be used by the network

board.

Use the hexadecimal physical (absolute) address of the memory

used by the board.  (This starting address must match the

starting address configured on the board.)

Enter the length (in hexadecimal paragraphs) of the memory

address range used by the board.  A paragraph is 16 bytes.

For example, if you address a board from D0000 to D4000 (bytes),

the starting address is D0000 and the range is 400 (paragraphs).

In this case, place the following lines in your NET.CFG file:

Link Driver name

Mem D0000

Usually, the length is not needed.

PORT [#1 | #2] hex_starting_address [hex_number_of_ports]

Use this option to specify the starting port and number of ports in

the range.  All values must be written in hexadecimal notation.

Use the starting hexadecimal I/O port number.

Use the hexadecimal number of ports in the range.  If your

network board can use two ranges and you want to use either the

second range or both ranges, you must specify which range (#1 or

#2) to use.

For example, suppose you want to specify the starting port and

the number of ports in the first range on your board.  The starting

I/O port is 300; 16 ports are in the first range.  Place the following

lines in the NET.CFG file:

Link Driver name

Port 300

The number of ports is optional.

NODE ADDRESS hex_address

This option overrides any hard-coded node address in the network

board, if the hardware allows it.  Use the hexadecimal address

number.

Changing the node address on a board can create conflicts

with other network boards.  Use the hard-coded node

address whenever possible.

The example below shows how to change the node address on a

3C523 board:

Link Driver 3C523

Node Address 12D43

SLOT number

In slot-based machines, the driver usually locates the board by

scanning through the slots in order from lowest to highest.  The

SLOT option directs the driver to eliminate the scan and locate

the board in the specified slot.

Use the number of the slot into which you inserted the board.

The slot number is usually found on the back of the computer.

The LAN driver will use the board in this slot.

For example, if you are using two NE/2 boards in the same

workstation and you insert one board into slot 1 and the other

into slot 2, place the following lines in your NET.CFG file:

Link Driver NE2

Link Driver NE2

Slot 2

Then place the following lines in your batch file:

LSL

NE2

NE2

The first NE/2 driver scans for an NE/2 board, finding the board

in slot 1.  The second NE/2 driver, as directed by the SLOT option,

uses the NE/2 board in slot 2.

Software commands

FRAME frame_type

This option enables the frame types used by the network board.

Use this option with boards that support multiple frame types.

For example, to enable the Ethernet II frame type on an NE1000

board, place the following lines in the NET.CFG file:

Link Driver NE1000

Frame ETHERNET_II

Usually, Ethernet LAN drivers default to the Ethernet_802.3

frame type; Token-Ring LAN drivers default to the Token-Ring

frame type.

The following chart lists the supported frame types for each LAN

driver supplied by Novell.

PROTOCOL name hex_protocol_ID frame_type

This option allows new network protocols to be handled by

existing LAN drivers.

Replace name with the name of the new protocol.

Replace hex_protocol_ID with the assigned hexadecimal protocol

ID that the protocol is assigned.

Replace frame_type with the frame type that the new protocol ID

applies to.

For example, if you want to use a new protocol XYZ with an NE2-

32 network board, the NET.CFG file would look similar to the

following:

Link Driver NE2-32

Frame ETHERNET_SNAP

Protocol XYZ 904A ETHERNET_SNAP

SAPS number

If you use the LANSUP driver, you may specify the number of

Service Access Points (SAPs) needed.  Set the option to allow for

all applications using the IBM LAN Support Program.  The

maximum value depends on the type of board used.

Default:  1

Note:This option is ignored if another application has already opened

the adapter before the LANSUP.COM driver was loaded.

LINK STATIONS number

If you use the LANSUP driver, you may specify the number of

link stations needed.  Set the option to allow for all applications

using the IBM LAN Support Program.  The maximum value

depends on the type of board used.

Default:  1

Note:This option is ignored if another application has already opened

the adapter before the LANSUP.COM driver was loaded.

ALTERNATE

Normally the LANSUP, Token, and PCN2L drivers use the

primary adapter.  Use this option if you want the driver to use the

alternate adapter.

Specify ALTERNATE under the appropriate driver section

heading, as shown below:

Link Driver LANSUP

ALTERNATE

MAX FRAME SIZE number

This option sets the maximum number of bytes that can be put on

the wire by the LAN driver.

The TOKEN.COM driver's default size is 4216 bytes.  But if your

board has 8 KB of shared RAM available, the default size is 2168

bytes.

The LANSUP.COM driver's default is normally 1144 bytes.

However, if you are using the IBM LAN Support Program with an

Ethernet device driver, the maximum size is 1496.

For TOKEN.COM and LANSUP.COM, the value for number must

be a multiple of 8.  It must include the number of bytes for the

data packet, for adapter overhead (6 bytes), and for the largest

possible header (currently, 35 bytes LAN header + 5 bytes SNAP

header + 74 bytes protocol header = 114 bytes).

If the line speed is 16 Mbps, the value for number may be between

632 and 17,960.  If the line speed is 4 Mbps, the value for number

must be between 632 and 4464.

If you wanted to use 2 KB data packets, the number would be

calculated as

2048 + 6 + 35 + 5 + 74 = 2168

then rounded up to the next multiple of 8:  2168.

The NET.CFG file would look similar to the following:

Link Driver TOKEN

MAX FRAME SIZE 2168

Note:When the driver loads, the sign-on message does not reflect the 6

bytes of overhead.  Therefore, the MAX FRAME for the above

example would be displayed as 2162.

CONNECTOR DIX

This option configures the 3C503 LAN driver to use the DIX

(thick) connector.  BNC/TP (thin) is the default.

DIX may also be selected at the command line, as in the following

example:

LSL

3C503 D

Link Support

BUFFERS communication_number [size]

This option configures the number and size of receive buffers that

will be maintained by the LSL.

The number of communication buffers must be large enough to

hold all media headers and the maximum data size.

Default:  0

Refer to the documentation for the third-party protocol stacks for

possible settings.

Buffer size is optional.  The minimum size is 618.  The total buffer

space must fit into approximately 59 KB (number times size).

Default:  1130

MEMPOOL number[k]

Some protocols use this option to configure the size of the memory

pool buffers that the LSL will maintain.

The k notation means multiply by 1024.

Refer to the protocol documentation for settings.

Protocol protocol_name

BIND #board_number

Usually a protocol binds to the first network board it finds.  The

BIND option forces the protocol to bind to the boards you specify.

Board numbers are displayed when you load the LAN drivers.

Replace board_number with logical boards you want the protocol

to bind to.  For example, if you wanted protocol XYZ to bind to the

third logical board, the NET.CFG would look like this:

Protocol XYZ

Bind #3

To bind protocol XYZ to the second and third logical boards, the

NET.CFG would look like this:

Protocol XYZ

Bind #2, #3

Note:Some protocols do not support binding to multiple boards.  Refer

to protocol documentation for binding information.

Notes:

Appendix A:  Using the NetWare IPX Shell

with ODI

This chapter explains how to configure DOS ODI to connect to a

NetWare server using IPX and the NetWare shell.  It highlights

the IPX-specific differences, exceptions, and additons to the

information in the first two sections of this manual.  You need to

focus on the following areas:

�  Installation

�  Using NET.CFG

Installation

Follow the installation instructions in the �Installing ODI for

DOS,� section (page 3) using IPXODI as the protocol suite and

using the NetWare shell file.

1.   Copy IPXODI.COM from the DOS ODI Workstation Services diskette to the

master workstation diskette.

2.   Copy the shell file you will use from your NetWare release diskette to the

master workstation diskette.

(The x in NETx.COM refers to the DOS version that runs on your

workstations.)

3.   Place IPXODI and then the shell file after the LAN drivers in the batch file you

created.

Your batch file should look similar to the following:

LSL

NE1000

IPXODI

NET3.COM

You can remove parts of IPXODI.COM to save memory.

The IPXODI.COM file actually contains three protocols:

�  IPX

�  SPX

�  Remote Diagnostics Responder

Since many applications do not need SPX or the Remote

Diagnostics Responder (required for third-party applications that

gather diagnostics information), workstations can save memory by

not loading these parts of the IPXODI.COM file.

Some NetWare utilities, such as RCONSOLE and NVER,

require SPX and the Remote Diagnostics Responder to be

loaded.

The chart below shows how to load IPXODI without the other two

parts of the file.

Note:If you will use remote boot with a DOS ODI workstation, follow

the remote boot instructions in your NetWare manual, with one

addition: include RPLODI.COM in the AUTOEXEC.BAT.  The

RPLODI.COM file is located on the DOS ODI Workstation Services

diskette.

Use the following chart to locate remote boot instructions:

When you create the batch file, place RPLODI.COM after the LSL

but before the driver files, as shown below:

LSL.COM

RPLODI.COM

driver.COM

IPXODI

NET3

Using NET.CFG

When using the NET.CFG options with IPX and the NetWare

shell, you need the information contained in this section for the

following parameters:

�  Link Driver, PROTOCOL

�  Link Support, BUFFERS

�  Link Support, MEMPOOL

�  Protocol, BIND

Note:The SHELL.CFG file also contains NET.CFG network

configuration information.  Any options from the SHELL.CFG file

can be specified in the more versatile NET.CFG file.

Use the chart below to determine whether to use both .CFG files

or to combine them into NET.CFG.

If a SHELL.CFG file exists and you place SHELL.CFG options in

the NET.CFG file, they will be ignored.

See your NetWare installation manual for SHELL.CFG options.

Link Driver, PROTOCOL

The PROTOCOL parameter assigns a board and frametype

combination.  If you use a protocol in addition to protocol IPX, you

must explicitly include the protocol ID for IPX and for the

additional protocol.  The NET.CFG would look like the following:

Link Driver NE2-32

Frame Ethernet_II

Frame Ethernet_802.3

Protocol IPX 0 Ethernet_802.3

Protocol ADD 1234 Ethernet_II

The following table lists the protocol IDs for frametypes available

with IPX.

Link Support, BUFFERS

The IPXODI protocol stack does not require the Link Support

Layer communication buffers.

Link Support, MEMPOOL

The IPXODI protocol stack does not require MEMPOOL buffers.

Protocol, BIND

If you do not specify a BIND parameter, IPXODI binds to the first

board it finds.  If you are using multiple boards and frametypes,

you may want to specify a BIND parameter.

For example, if you wanted protocol IPXODI to bind to the third

logical board, the NET.CFG would look like this:

Protocol IPX

Bind #3

In a DOS environment, you can bind IPXODI.COM to only one

network board/frametype combination.

Appendix B:  Using the Token-Ring Source

Routing Driver

This appendix explains the use of the IBM Token-Ring Source

Routing Driver with DOS ODI.

The IBM Token-Ring Source Routing Driver enables

communication across IBM Token-Ring network bridges.  Any type

of DOS ODI protocol stack can use this source routing feature to

communicate across IBM Token-Ring network bridges.

All nodes that need to communicate across a source route bridge

must be running the IBM Token-Ring Source Routing Driver.

The following figure shows an example network configuration

using IBM source routing bridges.

Installing the source routing driver with DOS ODI

To install source routing on workstations, complete the following

steps.

1.   Copy the ROUTE.COM file from the DOS ODI Workstation Services diskette to

the boot disk.

The command to load ROUTE.COM should be added after

LANSUP.COM or TOKEN.COM, but before the protocol stack you

are using.

Note:If you are are using ROUTE.COM with remote boot, you must

load ROUTE.COM before the LAN driver in the AUTOEXEC.BAT.

See Appendix A for more information.

2.   Set the parameters for ROUTE.COM.

The default parameters for ROUTE.COM can be used with most

network communications.

For additional ROUTE.COM parameters see �Setting alternate

parameters� on the next page.  Add the parameters at the

command line.

Note:The ROUTE.COM module can be removed from memory by

specifying the u command line switch, as in the following example:

ROUTE U

Setting alternate parameters

The following parameters can be entered when ROUTE.COM is

first loaded.

BOARD = number               MBR

CLEAR                        NODES = number

DEF                          REMOVE = number

GBR

The parameters can also be changed by loading ROUTE.COM a

second time.  For online help, type

ROUTE ?  <Enter>

Most of the parameters have default values that should work with

simple configurations for IBM bridges.  If you have installed

parallel IBM bridges, you can change some of the parameters to

reduce traffic on some of the paths.  Each parameter is described

below.

A sample ROUTE file is included on page 34.

BOARD = number

Use this parameter to specify a board number for the network

board.

�  If this parameter is not specified, the default of 1 is used.

�  If you have loaded more than one LAN driver in the

workstation, the board number is determined by the order in

which the LAN drivers are loaded (the first board is 1).  See

your batch file for the order you have specified.

�  Load ROUTE for each logical board number (frame type)

enabled by the Token-Ring LAN driver.

CLEAR

Use this parameter to manually clear the Source Routing table

and force a dynamic rebuilding of the table by sending a default

frame to each node in the network.  Use this parameter when an

IBM bridge on the network has gone down and an alternate route

is available.

DEF

Use this parameter to prevent frames that have unknown

(DEFault) destination addresses from being sent across Single

Route IBM bridges.

�  If this parameter is specified, all frames that have addresses

not in the workstation's Source Routing table are forwarded as

All Routes Broadcast frames.

�  If this parameter is not specified, all frames that have

addresses not in the workstation's Source Routing table are

forwarded as Single Route Broadcast frames.

�  If ROUTE.COM has already been loaded with the DEF

parameter, reloading ROUTE.COM with this parameter

broadcasts all subsequent Single Route Broadcast frames as

All Routes Broadcast frames.

GBR

Use this parameter to specify that all General BRoadcast frames

are to be sent as All Routes Broadcast frames.

�  If this parameter is not specified when ROUTE is loaded, all

General Broadcast frames are broadcast as Single Route

Broadcast frames.

�  If ROUTE has already been loaded with the GBR parameter,

reloading ROUTE with this parameter broadcasts all

subsequent General Broadcast frames as All Routes Broadcast

frames.

MBR

Use this parameter to specify that all Multicast BRoadcast frames

are to be sent as All Routes Broadcast frames.

�  If the parameter is not specified when ROUTE is loaded, all

Multicast Broadcast frames are broadcast as Single Route

Broadcast frames.

�  If ROUTE has already been loaded with the MBR parameter,

reloading ROUTE with this parameter broadcasts all

subsequent Multicast Broadcast frames as All Routes

Broadcast frames.

NODES = number

Use this parameter to specify the number of table entries in the

Source Routing table.  This parameter must be entered the first

time ROUTE.COM is loaded.  The parameter cannot be changed

by reloading ROUTE.COM.

Replace number with a value from 8 to 255.  The default is 16.

REMOVE = number

Use this parameter to remove a specified node address from the

workstation's Source Routing table.  Use the parameter when a

bridge has gone down.  Removing the node from the Source

Routing table forces the workstation to determine an alternate

path.

Replace number with a 12-digit (6-byte) hexadecimal number.  If

fewer than 9 digits are entered, ROUTE.COM prefixes the address

with 4000h.  For example, REMOVE=2 is interpreted as

REMOVE=400000000002.

Sample ROUTE.COM

The following command shows one possible ROUTE setting:

ROUTE BOARD=3, DEF, GBR, MBR

If you set these parameters, ROUTE.COM would do the following:

�  Send packets through logical board 3

�  Send all frames that are not addressed in the workstation's

source routing table as All Routes Broadcast

�  Send General Broadcast frames as All Routes Broadcast

�  Send Multicast Broadcast frames as All Routes Broadcast

When the NODES parameter is not set, ROUTE assumes the

default of 16.

The CLEAR and REMOVE parameters are not needed for the

initial load of ROUTE.COM.  However, if a bridge goes down, you

can reload ROUTE with these parameters to reconfigure the

Source Routing table.

Note:For more information on using source routing, see the IBM Token-

Ring Network Architecture Reference manual.

Appendix C:  System Messages

Connector is BNC/TP (Thin).

Source:  3C503.COM

Explanation:  The connector selected for the board is for Thin Ethernet

cabling.

Action:  None.

Connector is DIX (Thick).

Source:  3C503.COM

Explanation:  The connector selected for the board is for Thick Ethernet

cabling.

Action:  None.

FATAL:  3C501 -Card not found.

Source:  3C501.COM

Explanation:  The 3C501 card did not respond to a reset command by

the 3C501.COM driver at the specified I/O port settings.

Action:  Make sure a 3C501 board is installed.  Verify the hardware

jumper settings.  If the settings do not match the defaults, specify the

settings in the NET.CFG file.

FATAL:  3C503 -Card not found.

Source:  3C503.COM

Explanation:  The 3C503.COM driver could not locate a 3C503 board at

the specified hardware settings.

Action:  Make sure a 3C503 board is installed.  Verify the hardware

jumper settings.  If the settings do not match the defaults, specify the

settings in the NET.CFG file.

FATAL:  3C503 -Card will not initialize.

Source:  3C503.COM

Explanation:  A hardware failure occurred.

Action:  Check for hardware settings that conflict with other installed

boards.

FATAL:  3C503 -Interrupt is invalid (2, 3, 4, 5, and 9 valid).

Source:  3C503.COM

Explanation:  An invalid interrupt was selected in the NET.CFG file.

Action:  Correct the NET.CFG entry with a valid interrupt.

FATAL:  80186 Failed to Reset Properly.

Source:  EXOS.COM

Explanation:  The 80186 on the EXOS 205/215 board failed to function

properly.

Action:  Try a different slot.  If the error persists, replace the board.

FATAL:  82586 Failed in IA Setup.

Source:  EXOS.COM

Explanation:  The 82586 on the EXOS 205/215 board could not set the

individual node address.

Action:  Try a different slot.  If the error persists, replace the board.

FATAL:  82586 Failed to Configure Properly.

Source:  EXOS.COM

Explanation:  The 82586 on the EXOS 205/215 board failed to configure

itself properly.

Action:  Try a different slot.  If the error persists, replace the board.

FATAL:  82586 Failed to Diagnose Command.

Source:  EXOS.COM

Explanation:  The 82586 on the EXOS 205/215 board failed its self-

diagnostics routines.

Action:  Try a different slot.  If the error persists, replace the board.

FATAL:  82586 Failed to Reset Properly.

Source:  EXOS.COM

Explanation:  The 82586 on the EXOS 205/215 board failed to initialize.

Action:  Try a different slot.  If the error persists, replace the board.

FATAL:  82586 Failed to Start RU Properly.

Source:  EXOS.COM

Explanation:  The 82586 on the EXOS 205/215 board was unsuccessful

in getting the receive unit to start.

Action:  Try a different slot.  If the error persists, replace the board.

FATAL:  An Interrupt Failed to Occur During Dir.Open.Adapter.

Source:  TOKEN.COM

Explanation:  After the Dir.Open.Adapter command was given, the

adapter failed to respond.

Action:  Try a different slot.  If the error persists, replace the adapter.

FATAL:  Board failed to initialize correctly.

Source:  MLID

Explanation:  The MLID could not initialize its board correctly.  A

hardware failure usually prevents initialization.

Action:  Refer to the MLID documentation for a description of the error.

FATAL:  Bring Up Error After Call to DIR.Initialize = xx.

Source:  TOKEN.COM

Explanation:  DIR.Initialize has returned a value for the bring-up

diagnostics result code of xx.

Action:  For the description of the actual error, refer to the interface

chapter of the IBM Local Area Network Technical Reference.

FATAL:  Could not find a board that supports IPX (See PROTOCOL

keyword).

Source:  IPXODI

Explanation:  IPX could not find a board to bind with.  For IPX to bind,

it must have a protocol ID registered with the Link Support Layer.  Most

MLIDs will register an IPX Protocol ID by default.  However, if you have

specified the PROTOCOL keyword under an MLID's section heading in

the NET.CFG file, this error may be generated.  When a protocol ID has

been specified for another protocol stack, the MLID does not register a

default protocol ID for IPX.

Action:  Register a protocol ID for IPX along with the other Protocol IDs

you are registering.  Refer the chart on page 27 for a listing of IPX

protocol ID numbers.

FATAL:  Could not find an enabled 3C523 adapter in any slot.

Source:  3C523.COM

Explanation:  By default, the 3C523.COM driver scans the PS/2 slots to

find the 3C523 board it should use.  The driver starts scanning at slot 1.

The driver could not find a 3C523 in any slot.

Action:  Make sure that the 3C523 board is firmly seated in the slot.

FATAL:  Could not find an NE2 adapter in any slot.

Source:  NE2.COM

Explanation:  By default, the NE2.COM driver scans the PS/2 slots to

find the NE2 board it should use.  The driver starts scanning at slot 1.

The driver could not find an NE2 in any slot.

Action:  Make sure the NE2 board is firmly seated in the machine's slot.

FATAL:  Could not find an NE2-32 adapter in any slot.

Source:  NE2-32.COM

Explanation:  By default, the NE2-32.COM driver scans the PS/2 slots to

find the NE2-32 board it should use.  The driver starts scanning at slot 1.

The driver could not find an NE2-32 in any slot.

Action:  Make sure the NE2-32 board is firmly seated in the slot.

FATAL:  Could not find <name> MLID to unload.

Source:  MLID

Explanation:  A request was made to unload the MLID, but the MLID is

not loaded.

Action:  None.

FATAL:  Different IPX or an IPX interrupt has been hooked.

Source:  IPXODI

Explanation:  You tried to unload the IPX from memory, but IPX

detected a condition that would not allow it to be removed safely.  This

can occur for two reasons:

� The resident IPXODI and the IPXODI used to unload the resident

IPXODI are not the same version.

� Another program has been loaded that has hooked one of IPXODI's

interrupt vectors.  IPXODI uses the following interrupt vectors:  INT

08h, INT 2Fh, INT 64h, and INT 7Ah.

Action:  Complete one of the following:

� Use the same version of IPXODI to unload IPX as you used to load

it.

� Unload the program that has hooked to one or more IPX interrupt

vectors and then unload IPXODI.

FATAL:  Different LSL or a LSL interrupt has been hooked.

Source:  LSL

Explanation:  You tried to unload the LSL from memory, but it detected

a condition that would not allow it to be removed safely. This can occur

for two reasons:

� The resident LSL and the LSL used to unload the resident LSL are

not the same version.

� Another program has been loaded that has hooked one of the LSL's

interrupt vectors.  The LSL uses the following interrupt vectors:  INT

08h and INT 2Fh.

Action:  Complete one of the following:

� Use the same version to unload LSL as you used to load it.

� Unload the program that has hooked to the LSL's interrupt vectors

before unloading the LSL.

FATAL:  Direct Station 0 already in use by another application.

Source:  LANSUP.COM

Explanation:  The LANSUP.COM driver uses Direct Station 0.  Only

one application running on the IBM LAN Support program can use Direct

Station 0.

Action:  Unload the other application.

FATAL:  Dir.Open.Adapter Failed to Respond Properly.

Source:  TOKEN.COM

Explanation:  After receiving false completions and trying several times

to get the card to respond to the Dir.Open.Adapter command, the driver

has given up.

Action:  Try a different slot.  If the error persists, replace the card.

FATAL:  Dir.Open.Adapter:-Lobe Media...

-Physical Insertion...

-Addr. Verification...

-Ring Poll...

-Request Parameters...

Action:  Refer to the �Token-Ring Network Adapter Open errors for all

CCBS� section in Appendix B of the IBM Local Area Network Technical

Reference.

FATAL:  Dir.Open.Adapter:  Returned Error Code = xx.

Source:  TOKEN.COM

Explanation:  The Dir.Open.Adapter call has returned an error code.

Action:  The driver will try again.  The definition of the error code is in

the interface chapter of the IBM Local Area Network Technical Reference.

FATAL:  Error initializing board.

Source:  LANSUP.COM

Explanation:  The LAN Support Program could not initialize the

network board.

Action:  Check the installation of the LAN Support software.

FATAL:  Error opening board.

Source:  LANSUP.COM

Explanation:  The LAN Support Program could not open the network

board.

Action:  Check the installation of the LAN Support software.

FATAL:  Error shutting down <name> MLID, unload operation aborted.

Source:  MLID

Explanation:  The MLID was trying to remove the resident MLID from

memory.  The MLID could not successfully shut down the resident MLID.

A hardware failure has probably occurred.

Action:  Run diagnostics on the network board and workstation.  Replace

the faulty hardware.

FATAL:  EXOS 215 Card Not Found.

Source:  EXOS.COM

Explanation:  The EXOS.COM driver scans by default through all the

slots starting at slot 1.  Either the EXOS 215 board was not present in

the slot designated by the NET.CFG file, or the driver could not find the

board at all.

Action:  Verify the EXOS 215 card is installed and the slot number is

described correctly by the NET.CFG if the file is present.

FATAL:  EXOS Card Memory Failure.

Source:  EXOS.COM

Explanation:  The EXOS 205/215 has not passed the memory test.

Action:  If the card's jumper settings are not the defaults, verify that

they match the NET.CFG file's settings.  If the settings match, try a

different slot.  If the error persists, replace the board.

FATAL:  Failed to locate MLID Section Heading in NET.CFG file.

Source:  MLID

Explanation:  You tried to load the MLID again.  Normally you would do

this so that you could use two or more boards in the workstation.  When

two or more of the same type of network board are installed in the

workstation, you must specify an associated MLID section heading in the

NET.CFG file. An entry in the NET.CFG file for two boards would look

similar to the following:

;Allow the first one to use default values.

LINK DRIVER NE1000

;This section is for the second board.

LINK DRIVER NE1000

INT 4

PORT 320

Action:  Add the commands for both MLID boards to the NET.CFG file.

Then reboot the workstation.

FATAL:  IBM LAN Support Program has not been loaded.

Source:  LANSUP.COM

Explanation:  The LANSUP.COM driver requires that the IBM LAN

Support Program be loaded.

Action:  Install the LAN Support Software using the IBM-supplied

DXMAID utility and retry the operation.

FATAL:  Init586:  Configure command failed.

Source:  3C523.COM

Explanation:  A hardware failure has occurred.

Action:  Install the 3C523 board in a different slot.  If the driver still

fails to load, the 3C523 board is probably bad and should be replaced.

FATAL:  Init586:  IA_Setup command failed.

Source:  3C523.COM

Explanation:  A 3C523 hardware failure has occurred.

Action:  Install the 3C523 in a different slot.  If the driver still fails to

load, the 3C523 board is probably bad and should be replaced.

FATAL:  Init586:  Initial communication with 82586 failed.

Source:  3C523.COM

Explanation:  A hardware failure has occurred.

Action:  Install the 3C523 board in a different slot.  If the driver still

fails to load, the 3C523 board is probably bad and should be replaced.

FATAL:  Invalid base memory address - Must be C0000 or D0000.

Source:  NE2-32.COM

Explanation:  Because the NE2-32 board is a true 32-bit board, it can

have its base memory located above the 1MB real mode memory limit.

Since DOS ODI is executing in real mode, the NE2-32.COM driver cannot

access the board's RAM when it is located above the 1MB address range.

Action:  Use the IBM Reference diskette to change the board's base

memory setting to either C0000 or D0000.

FATAL:  Invalid Ethernet node address specified.

Source:  MLID

Explanation:  You used the NODE ADDRESS option in the NET.CFG

file to override the node address on the network board.  The number

specified was not a valid Ethernet node address.  An Ethernet address is

6 bytes long.

This error occurs if Bit 0 of the first address byte is a 1.  This bit must

always be 0.  For example, if the first byte has the following address, an

invalid Ethernet address is generated.

First Byte

7 6 5 4 3 2 1 0

���������������Ŀ

�0�0�0�0�0�0�0�1�

�����������������

Action:  Change the NET.CFG file so that a valid node address is

specified.

FATAL:  Invalid parameter.

Source:  IPXODI

Explanation:  When you tried to load IPXODI, you specified an invalid

parameter on the command line.  IPXODI was not loaded.  The only valid

parameters are ? for information, U for unload, D to load IPX and SPX

only, and A to load IPX only.

Action:  Specify a valid parameter.

or

Source:  LSL

Explanation:  When you tried to load the LSL, you specified an invalid

parameter on the command line.  The LSL was not loaded.  The only

valid parameters are ? for help and U for unload.

Action:  Specify a valid parameter.

FATAL:  Invalid Token-Ring node address specified.

Source:  MLID

Explanation:  You used the NODE ADDRESS option in the NET.CFG

file to override the node address on the network board.  The number

specified was not a valid Token-Ring node address.  A Token-Ring

address is 6 bytes long.  This error occurs if Bit 7 of the first address byte

is a 1.  This bit should always be a 0.  For example, if the first byte has

the following address, an invalid Token-Ring address is generated.

First Byte

7 6 5 4 3 2 1 0

���������������Ŀ

�1�0�0�0�0�0�0�0�

�����������������

Action:  Change the NET.CFG file so that a valid node address is

specified.

FATAL:  IPX already loaded.

Source:  IPXODI

Explanation:  IPX has already been loaded.  It needs to be loaded only

once.

Action:  None.

FATAL:  IPX is already registered with the LSL.

Source:  IPXODI

Explanation:  IPX was not removed from the LSL's register when it was

unloaded.  The system may be corrupted.

Action:  Reboot the workstation.

FATAL:  IPX is not loaded.

Source:  IPXODI

Explanation:  You tried to unload IPX, but it has not been loaded.

Action:  None.

FATAL:  Loading MLID again requires configuration information in

NET.CFG.

Source:  MLID

Explanation:  You tried to load the MLID a second time.  Normally you

would do this so that you could use two or more boards in the

workstation.  When two or more of the same type of network board are

installed in the workstation, you must specify an associated MLID section

heading in the NET.CFG file.

An entry in the NET.CFG file for two boards would look similar to the

following:

;Allow the first one to use default values.

LINK DRIVER NE1000

;This section is for the second board.

LINK DRIVER NE1000

INT 4

PORT 320

Action:  Create a NET.CFG file and add the commands for both MLID

boards to the file.  Then reboot the workstation.

FATAL:  LSL already loaded.

Source:  LSL

Explanation:  The LSL has already been loaded.  It can be loaded only

once.

Action:  None.

FATAL:  LSL is not loaded.

Source:  LSL

Explanation:  You tried to unload the LSL, but it is not loaded.

Action:  None.

FATAL:  Multiplex interrupt 2Fh has no free slots.

Source:  LSL

Explanation:  The LSL could not find a free slot for use with INT 2F

(Hex).  When your computer is first booted, approximately 63 multiplex

slots are available.  All available slots are already being used by

applications.

Action:  Unload an application that is using an INT 2F multiplex

interrupt; then load the LSL.

FATAL:  NE2 NIC command port failed to respond.

Source:  NE2.COM

Explanation:  The installed NE2 board has malfunctioned.

Action:  Replace the board.

FATAL:  NE2 NIC RAM failure.

Source:  NE2.COM

Explanation:  The NE2 board's installed on-board memory has

malfunctioned.

Action:  Replace the board.

FATAL:  NE2-32 NIC command port failed to respond.

Source:  NE2-32.COM

Explanation:  The installed NE2-32 board has malfunctioned.

Action:  Replace the board.

FATAL:  NE2-32 NIC RAM failure.

Source:  NE2-32.COM

Explanation:  The installed NE2-32 board's on-board memory has

malfunctioned.

Action:  Replace the board.

FATAL:  NE1000 NIC command port failed to respond.

Source:  NE1000.COM

Explanation:  The NE1000 board has malfunctioned, is not installed, or

is not configured to the same port as selected for the driver.

Action:  Complete one or more of the following:

� Make sure that an NE1000 board has been installed and configured.

� Make sure that the configuration for the NE1000 board matches the

configuration for the NE1000 driver.  Configuration options other

than the defaults must be entered in the NET.CFG file.

� Replace the board.

FATAL:  NE1000 NIC RAM failure.

Source:  NE1000.COM

Explanation:  The NE1000 board's on-board memory has malfunctioned.

Action:  Replace the board.

FATAL:  NE2000 NIC command port failed to respond.

Source:  NE2000.COM

Explanation:  The NE2000 board has malfunctioned, is not installed, or

is not configured to the same port as selected for the driver.

Action:  Complete one or more of the following:

� Make sure that an NE2000 board has been installed and configured.

� Make sure that the configuration for the NE2000 board matches the

configuration for the NE2000 driver.  Configuration options other

than the defaults must be entered in the NET.CFG file.

� Replace the board.

FATAL:  NE2000 NIC is NOT supported in a 8-bit slot.

Source:  NE2000.COM

Explanation:  The NE2000 board is located in an 8-bit slot in the

machine.  The NE2000.COM driver supports the NE2000 only in a 16-bit

AT bus slot.

Action:  Place the NE2000 board in a 16-bit slot and try again.

FATAL:  NE2000 NIC RAM failure.

Source:  NE2000.COM

Explanation:  The NE2000 board's on-board memory has malfunctioned.

Action:  Replace the board.

FATAL:  NE2000 NIC was not found at specified hardware settings.

Source:  NE2000.COM

Explanation:  The NE2000.COM driver found a board at the specified

hardware settings, but the board is not an NE2000 board.

Action:  Check the installed hardware.

FATAL:  No more room in the LSL for another protocol stack.

Source:  IPXODI

Explanation:  The maximum number of protocol stacks has been

registered with the LSL.  The DOS ODI LSL can support up to eight

protocol stacks.

Action:  Remove an existing protocol stack.

FATAL:  No room in LSL for board using FRAME <string>.

Source:  MLID

Explanation:  The maximum number of boards, whether virtual or

physical, has been registered with the LSL.  The DOS ODI LSL can

support up to eight boards.

Action:  Reduce the number of active boards in the system by unloading

a board or by decreasing the number of frame types activated by the

MLID.

FATAL:  RXNet command port failed to respond.

Source:  TRXNET.COM

Explanation:  The TRXNET.COM driver could not find an RX-Net board

in the workstation.

Action:  Make sure an RX-Net board is installed in the workstation and

is firmly seated.  Make sure that the values for the hardware settings in

the driver (the default or the values set in the NET.CFG file) match the

values set on the board.

FATAL:  RXNet RAM failure.

Source:  TRXNET.COM

Explanation:  The RX-Net board's RAM has failed the RAM test.

Action:  Replace the board.

FATAL:  Specified slot contains an NE2 but it is not enabled.

Source:  NE2.COM

Explanation:  The located NE2 board was not enabled.  It has probably

malfunctioned.

Action:  Replace the board.

FATAL:  Specified slot contains an NE2-32 but it is not enabled.

Source:  NE2-32.COM

Explanation:  The located NE2-32 board was not enabled.  It has

probably malfunctioned.

Action:  Replace the board.

FATAL:  Specified slot does not contain an enabled 3C523 adapter.

Source:  3C523.COM

Explanation:  A SLOT number option was specified in the NET.CFG file.

The specified slot does not contain a 3C523 board.  Note that slot

numbers are 1 based and correspond to the slot numbers on the back of

the computer.

Action:  Modify the NET.CFG to specify the correct number.

FATAL:  Specified slot does not contain an NE2 adapter.

Source:  NE2.COM

Explanation:  A SLOT number option was specified in the NET.CFG file.

The specified slot does not contain an NE2 board.  Note that slot numbers

are 1 based and correspond to the slot numbers on the back of the

computer.

Action:  Modify the NET.CFG file to specify the correct slot number.

FATAL:  Specified slot does not contain an NE2-32 adapter.

Source:  NE2-32.COM

Explanation:  A SLOT number option was specified in the NET.CFG,

but the specified slot does not contain an NE2-32 board.  Slot numbers

are 1 based and correspond to the slot numbers on the back of the

computer.

Action:  Modify the NET.CFG file to specify the correct slot number.

FATAL:  The LSL is not loaded.

Source:  IPXODI

Explanation:  IPX requires that the LSL be loaded first.

Action:  Load LSL.COM; then load IPXODI.COM.

or

Source:  MLID

Explanation:  The LSL must be loaded before an MLID can be loaded.

Action:  Load LSL.COM; then load the MLID.

FATAL:  There is a TSR above the loaded IPX.

Source:  IPXODI

Explanation:  You tried to unload IPX from memory, but IPX detected

another terminate-and-stay-resident program loaded above IPX.  For IPX

to unload safely, TSRs that were loaded after it must be unloaded first.

Action:  Either load the TSR before loading IPX or unload the TSR

before trying this operation.

FATAL:  There is a TSR above the loaded LSL.

Source:  LSL

Explanation:  You tried to unload the LSL from memory, but the LSL

detected another terminate-and-stay-resident program loaded above the

LSL.  For the LSL to unload safely, TSRs that have been loaded after it

must be unloaded first.

Action:  Either load the other TSR before loading the LSL or unload the

TSR before trying this operation.

FATAL:  There is a TSR above the loaded <name> MLID.

Source:  MLID

Explanation:  You tried to unload the MLID from memory, but the

MLID detected another terminate-and-stay-resident program loaded

above the MLID.  For the MLID to safely unload, TSRs that were loaded

after it must be unloaded first.

Action:  Either load the other TSR before loading the MLID or unload

the TSR before trying this operation.

FATAL:  This old LSL is not supported.

Source:  IPXODI

Explanation:  IPXODI cannot run correctly using this version of the

LSL.

Action:  Update your LSL.COM to a newer version.

or

Source:  MLID

Explanation:  The MLID cannot run correctly using this version of the

LSL.

Action:  Update your LSL.COM to a newer version.

FATAL:  Token-Ring Adapter Not Found at Designated Slot/IO Port.

Source:  TOKEN.COM

Explanation:  The TOKEN.COM driver by default scans each slot on a

microchannel machine.  If a Token-Ring card is located, the I/O port

address is checked to determine if the correct board has been found.

Either the Token-Ring card is not present or the NET.CFG does not

reflect the POS register settings.

Action:  Verify that the Token-Ring card is installed.  Verify that the

NET.CFG and the POS registers correctly describe the desired

configuration.

FATAL:  Token-Ring Buffer Memory Failure.

Source:  TOKEN.COM

Explanation:  The TOKEN.COM card has failed the shared RAM

memory test by the driver.

Action:  Try a different slot.  If the error persists, replace the board.

FATAL:  Token-Ring Hardware Failed to Respond After Retry.

Source:  TOKEN.COM

Explanation:  The TOKEN.COM driver has tried twice to initialize the

Token-Ring card and failed.

Action:  Try a different slot.  If the error persists, replace the board.

FATAL:  Token-Ring Hardware failure during card initialize.

Source:  TOKEN.COM

Explanation:  The TOKEN.COM driver waited for the card to complete

its internal diagnostics and produce an interrupt, but the card has not

responded.

Action:  Try a different slot.  If the error persists, replace the board.

FATAL:  Token-Ring Shared RAM is on Incorrect Boundary.

Source:  TOKEN.COM

Explanation:  The TOKEN.COM driver has found the base address for

shared RAM is not on a 16K boundary.

Action:  Verify that the addresses given in the NET.CFG file do not

conflict and that they match the jumper (ISA) or POS (MCA) settings.

FATAL:  Token-Ring Signature Hardware Failure.

Source:  TOKEN.COM

Explanation:  The TOKEN.COM card failed the signature test.

Action:  Verify that the Token-Ring card is installed and that the

settings in the NET.CFG match the jumper settings (ISA) or that POS

register values (MCA) are correct.

FATAL:  Token-Ring ROM/MMIO Domain and Shared RAM Overlap.

Source:  TOKEN.COM

Explanation:  The base address settings for the shared RAM and the

ROM/MMIO are too close and overlap.

Action:  Change the base address settings for the shared RAM or the

ROM/MMIO.

FATAL:  Work Area Exceeded, reduce number of SAPs and/or Link

Stations.

Source:  LANSUP.COM

Explanation:  The number of SAPs or Link Stations specified in the

NET.CFG file exceed the number of SAPs or Link Stations that the

network board can handle.

Action:  Modify the number of SAPs or Link Stations specified in the

NET.CFG file.

Inserting into ring...Please wait

Source:  TOKEN.COM

Explanation:  The Token-Ring card is trying to insert into the Token-

Ring.

Action:  None.

IPX protocol bound to <name> MLID Board # <number>.

Source:  IPXODI

Explanation:  This message is displayed after IPX has loaded

successfully.  It is not a error message but simply tells you which logical

MLID IPX is using.

<name>      The MLID's short name (NE1000, for example).

<number>    The logical board number of the MLID.  This number

is 1based (for example, the first MLID is assigned

board #1).

Action:  None.

IPX protocol successfully removed.

Source:  IPXODI

Explanation:  A request was made to unload IPXODI, and IPXODI was

removed from memory.

Action:  None.

LSL successfully removed.

Source:  LSL

Explanation:  A request was made to unload the LSL, and the LSL was

removed from memory.

Action:  None.

<name> MLID successfully removed.

Source:  MLID

Explanation:  A request was made to unload an MLID, and the MLID

was removed from memory.

Action:  None.

Number of buffers <number1>, Buffer size <size> bytes, Memory pool

<number2> bytes

Source:  LSL

Explanation:  This information appears when the LSL has read

configuration information from the NET.CFG file.  (The LSL does not

display default values when it loads.)

<number1>   The number of communication buffers allocated by

the LSL.  Normally, the LSL does not need buffers;

therefore, none should be allocated unless directed by

a protocol's documentation.  This number may be less

than requested due to memory limits in the LSL.

<size>      The size of the LSL's communications buffers (106

bytes are reserved for protocol and media headers).

This value cannot be smaller than 618 bytes.

<number2>   The amount of memory allocated to the LSL's free

memory pool.  This free memory pool is used by

protocols.  This value may be smaller than requested

due to memory limits within the LSL.  The LSL first

allocates the requested number of communications

buffers and then allocates the free memory pool from

the remaining memory.  Normally, a free memory

pool is not needed and should not be allocated, unless

directed by a protocol's documentation.

Action:  None.

Programmed I/O Mode Selected.

Source:  3C503.COM

Explanation:  The adapter is jumpered with the shared RAM disabled.

The driver and adapter support either shared RAM or programmed I/O.

Action:  To configure the adapter for shared RAM mode, change the

jumpers to the appropriate settings for the base-shared RAM address.

Trying again...Dir.Open.Adapter Return Code = xx.

Source:  TOKEN.COM

Explanation:  The Dir.Open.Adapter call has returned an error code.

Action:  The driver will try again.  The definition of the error code is in

the interface chapter of the IBM Local Area Network Technical Reference.

WARNING:  3C503  DMA Mode not Supported

Source:  3C503.COM

Explanation:  The 3C503.COM supports only programmed I/O or shared

RAM modes.  The NET.CFG contains a DMA entry.  The driver issues

this warning and ignores the setting.

Action:  DMA is not supported with this driver.  Use either programmed

I/O or shared RAM mode.  Shared RAM mode is faster.

WARNING:  3C503  NET.CFG Specified Shared RAM Address--NIC

Configured PIO.

Source:  3C503.COM

Explanation:  The 3C503 adapter is shipped with the shared RAM

memory disabled.  The NET.CFG file has specified a shared RAM

address, and the adapter is jumpered with memory disabled.  This is not

fatal, however; the driver supports both shared RAM and programmed

I/O (PIO) modes.  The driver will use PIO mode without modification.

Action:  Set the jumper on the adapter to the appropriate address if you

want shared RAM mode.  Shared RAM mode is faster than PIO or DMA

modes.  The 3C503.COM driver supports only shared RAM and PIO

modes.

WARNING:  Byte value greater than 255 was truncated

Source:  IPXODI

Explanation:  An IPX or SPX parameter specified in the NET.CFG or

SHELL.CFG file was set to a value greater than 255.  The value used will

be 255 (the maximum configurable value).

Action:  Change the NET.CFG file so that the IPX or SPX parameter is

valid.

WARNING:  Error registering Protocol=<name>, PID=<number>,

Frame=<type>

Source:  MLID

Explanation:  The MLID could not register the specified Protocol ID.

<name>      The name of the protocol.

<number>    The value of the Protocol ID used to register the

Protocol ID.

<type>      The frame type for which the Protocol ID was

registered.

Action:  Verify the protocol information in the NET.CFG file.

WARNING:  Frame type is already activated--duplicate entry ignored.

Source:  MLID

Explanation:  Two FRAME keywords under the same main section

heading specified the same frame type.  A specific frame type can be

specified only once per driver.

Action:  Remove the duplicate entry.

WARNING:  Max Frame Size is too Large in NET.CFG.  Max = 17960.

Source:  TOKEN.COM

Explanation:  The NET.CFG file has defined the maximum frame size to

be larger than the maximum allowed value.

Action:  Adjust the NET.CFG entry.  The driver will use the maximum

value until the correction is made.

WARNING:  Max Frame Size is too Small in NET.CFG.  Min = 632.

Source:  TOKEN.COM

Explanation:  The NET.CFG file has defined the minimum frame size to

be smaller than the minimum allowed value.

Action:  Adjust the NET.CFG entry.  The driver will use the minumum

value until the correction is made.

WARNING:  MLID does not support Frame <type> - Protocol keyword

ignored.

Source:  MLID

Explanation:  The PROTOCOL option was specified in the NET.CFG for

an MLID.  The specified frame type is not supported by the MLID.

Action:  Check the PROTOCOL line in the NET.CFG file for omissions of

required dashes and underscores or misspellings.  Check the MLID

documentation for supported frame types.

WARNING:  NET.CFG ignored - MLID name cannot be more than 8

characters long.

Source:  IPXODI

Explanation:  The Bind MLID option in the NET.CFG file specified an

MLID with a name longer than 8 characters.

Action:  Refer to the MLID's documentation for more information on the

name.

WARNING:  Node Address override not supported. Specify override

when loading IBM LAN Support Software.

Source:  LANSUP.COM

Explanation:  A node address was specified in the NET.CFG file for the

LANSUP.COM driver.  The node override must be specified when loading

the IBM LAN Support software.

Action:  Refer to the IBM LAN Support documentation for instructions.

WARNING:  Protocol keyword must have a frame type - entry ignored.

Source:  MLID

Explanation:  The PROTOCOL option was specified in the NET.CFG for

an MLID.  The entry failed to specify the associated frame type for the

protocol ID addition.  An entry in the NET.CFG file for the PROTOCOL

option should look similar to the following:

LINK DRIVER NE1000

PROTOCOL IPX 8137 ETHERNET_II

Action:  Specify a frame type with the PROTOCOL option.

Index

A

ALTERNATE (NET.CFG)  17

B

Batch file, creating for master

workstation diskette  5

BIND (NET.CFG)  21

BOARD (ROUTE.COM)  31

Boot disk, creating master  4

Bridges, IBM source routing  29

BUFFERS (NET.CFG)  20

C

CLEAR (ROUTE.COM)  32

CONNECTOR (NET.CFG)  19

Connector setting, 3C503 board  5

D

DEF (ROUTE.COM)  32

DMA (NET.CFG)  11

E

EMSNETx.EXE, expanded memory

shell file  23

Expanded memory shell  23

Extended memory shell  23

F

FRAME (NET.CFG)  15

G

GBR (ROUTE.COM)  32

H

Hardware, workstations supported

3

I

IBM Token-Ring network bridge  29

IBM Token-Ring Source Routing

Driver

guidelines for using  29

installing with DOS ODI  30

using with ROUTE.COM  30

INT (NET.CFG)  12

IPXODI

copying to master workstation

diskette  23

loading and unloading  23

removing parts of  24

L

LAN drivers

accessing information about  6

configuration options  9

drivernames  10

loading  4

NET.CFG options  9

supporting multiple protocols  1

unloading  6

LAN SUPPORT, adding device

drivers for  4

LAN WorkPlace for DOS  3

LANSUP.COM driver  4

LINK STATIONS (NET.CFG)  17

Link support options (NET.CFG)

20

LSL.COM  2, 4, 6

M

MAX FRAME SIZE (NET.CFG)  18

MBR (ROUTE.COM)  33

MEM (NET.CFG)  13

Memory

saving  24

setting for NET.CFG  13

MEMPOOL (NET.CFG)  20

N

NET.CFG

conventions  7

creating  5

example  7

options  8, 9

using with SHELL.CFG  26

NETx.COM on master workstation

diskette  23

NODE ADDRESS (NET.CFG)  14

NODES (ROUTE.COM)  33

P

PORT (NET.CFG)  13

PROTOCOL (NET.CFG)  16

Protocol, options in NET.CFG  21

protocol, unloading  6

R

Remote boot  25

Remote Diagnostics Responder,

unloading  24

REMOVE (ROUTE.COM)  33

ROUTE.COM  30

S

SAPS (NET.CFG)  17

Shell files  23

SHELL.CFG  26

SLOT (NET.CFG)  14

Source routing driver  29

SPX, unloading  24

T

Troubleshooting, installation  6

X

XMSNETx.EXE, extended memory

shell file  23

User Comments

Novell manuals and Novell products

We at Novell would like to hear from you if you have comments about our

manuals and products, or have suggestions for improving them.  Please

address responses to

Novell, Inc.

NPD Technical Publications

122 East 1700 South  MS C-24-1

Provo, UT  84606 USA

Please indicate the relevant chapters and page numbers and other

pertinent information as requested below.

Product Name:

Manual Name: ODI for DOS User's Guide

Part Number: 100-000871-002

Your Name:

Company Name:

Address:

City:                                        State/Province:

Country:                                    Zip/Postal Code:

Phone Number:    (     )

FAX Number:    (     )

Comments or suggestions