DOSWS.DOC

***************************************************************************

NOTE: We strongly recommend that ODI drivers be used instead of Drivers

below.  See either DOSODIWS.ZIP or DOSUP5.ZIP DOSUP6.ZIP etc. for latest

drivers. (separate on netwire).

***************************************************************************

DOSWS.DOC accompanies DOSWS.ZIP containing IBM Token-Ring V2.63 workstation

drivers.  The following drivers are included for this revision:

STOKEN   OBJ     6683   8-23-91   9:12a  (certified v2.63)

STOKEN   LAN      105  10-15-91  12:16p  (certified v2.63)

SLANSUP  OBJ     6080  10-18-91   3:33p  (certified v2.63)

SLANSUP  LAN      122   2-03-92   6:24a  (certified v2.63)

DOSWS    DOC          (this file)

Two DIFFERENT IPX.COM files can be generated with the above drivers.

These two IPX.COM files are described below as CASE #1 and CASE #2:

CASE #1:  IPX.COM.......(IPX.OBJ generated/linked with SLANSUP.OBJ)

This is the Lan Support driver.   To work properly, this driver REQUIRES

the IBM Lan Support drivers and it will not work without them. This IPX.COM

requires that the following IBM Lan Support files MUST BE SPECIFIED in the

workstation's CONFIG.SYS:

device = DXMA0MOD.SYS   (5C arbitrator)

device = DXMC0MOD.SYS   (adapter interface)

device = DXMT0MOD.SYS   (optional IBM Netbios driver)

The above IPX.COM driver also supports Remote Program Loading.

**************************** HISTORY **************************************

NOTE: Version 2.63 of SLANSUP.OBJ is the latest version that has been through

software test.  The most common problem with SLANSUP.OBJ version 2.62 is

the workstation will occasionally hang when loading IPX.COM especially when

broadcast traffic exists on the ring.

***************************************************************************

CASE #2:  IPX.COM......(IPX.OBJ generated/linked with STOKEN.OBJ)

This is the NATIVE Token-Ring driver.  It talks directly to the

Token-Ring adapter.  This IPX.COM driver works exactly opposite of the

IPX.COM mentioned in CASE #1 above.  This IPX.COM works WITH-OUT IBM Lan

Support drivers in the work-station's CONFIG.SYS.  When using this IPX.COM,

the workstation's CONFIG.SYS MUST NOT include IBM Lan Support driver

specifications.  This IPX.COM supports Remote Program Loading.

***************************************************************************

IMPLEMENTATION of SLANSUP.OBJ & STOKEN.OBJ

***************************************************************************

1.   Determine which IPX.COM to create, LAN Support Token-Ring or Native

Token-Ring.

2.   Copy the corresponding .lan files to the SHGEN or WSGEN directory or

diskette and the    corresponding .obj files to the LAN_DRV_001

diskette.

3.   Generate the new IPX.COM with SHGEN or WSGEN.

4.   Configure the IPX.COM at the command line based on your needs.  See

below for instructions.

LAN SUPPORT IPX.COM  (IPX.OBJ generated/linked with SLANSUP.OBJ)

===========================================================================

IPX.COM - This is a Client IPX Driver for the IBM Token-Ring generated with

the IBM LAN Support Program.  It works with ROUTE.com if Source Routing is

REQUIRED.  This version of IPX.COM can be configured by typing in the

following parameters at the command line (or batch file) when it is loaded:

CONFIGURING the DOS Workstation's IPX.com (Lan Support):

IPX.com - (IPX.obj linked with SLANSUP.obj) can be configured by typing

in the following parameters when it is loaded:

IPX o,ALT/PRI,ls=nn,mem=xx00,node=xxxxxxxx,ntb=nn,saps=nn,tbz=nnnn

where:

o - (lower case letter "o") REQUIRED to override the SHELL.cfg.

ALT - Configure the Driver as the ALTERNATE Adapter

PRI - Configure the Driver as the PRIMARY Adapter -- this is the DEFAULT

mem=xx00  xx00 is a HEXADECIMAL address of where Shared Ram is located.

The DEFAULT is D800 for PRIMARY, D400 for ALTERNATE.  This setting

ALWAYS gets overridden by a Microchannel Reference mem setting;

microchannel machines have this ability in the machine reference.

But, ISA bus machines MUST use this driver's setting because there

is no other way to specify it except with this driver's

"mem=xx00" setting.

ls=nn nn is a DECIMAL number specifying the number of LINK Stations that

the driver should allocated when initializing.  The DEFAULT is 00.

node=xx  xx is up to a 4 byte (8 hex digits) HEXADECIMAL Number

specifying a LAA (locally administered node address) which

overrides the hardware's default Node Address.  The hardware's node

address is known as a universally administered address.  4000h will

be PREFIXED to this number to form a 6 byte NODE Address: 4000 xxxx

xxxx.  If a LAA is specified in the CONFIG.SYS as shown:

device=dxmc0mod.sys 4000xxxxxxxx

device=dxmt0mod.sys o=n (o=n means don't open adapter) and IPX.COM opens

the Adapter card, then this node ID must also be specified with

"IPX o,pri,node=xxxxxxxx" when IPX.COM loads.

ntb=nn nn is a DECIMAL number specifying the NUMBER of TRANSMIT BUFFERS

to allocate.  The DEFAULT is ONE.  Specifying TWO MAY help

performance on some of the NEW IBM Token-Ring Cards.

saps=nn nn is a DECIMAL number specifying the number of SAP Stations

that the driver should allocated when initializing.  The driver

will increment nn by ONE (1) because NetWare REQUIRES 1 SAP.  The

DEFAULT is ONE.

tbz=nnnn nnnn is a DECIMAL number specifying the SIZE of the Transmit

buffer.  It can be ANY number between 512 and 4202.  The driver

will ADJUST it to the NEAREST power of TWO PLUS 106 bytes of

overhead.  The DEFAULT is 1024 (1130).

ALL of the above parameters are separated by COMMAs or BLANKS, are OPTIONAL,

are NOT case sensitive, and may be entered in ANY order.

NATIVE TOKEN-RING  IPX.COM  (IPX.OBJ generated/linked with STOKEN.OBJ)

========================================================================

IPX.COM - This is a NATIVE Token-Ring Driver for the DOS Workstation.  IT

DOES NOT REQUIRE the IBM LAN Support Program.  It is FASTER and SMALLER

than LAN Support IPX.COM.  It works with ROUTE.COM if Source Routing is

REQUIRED. Its DEFAULT Transmit Buffer Size is 4096 (4k) for the 16/4 MBps

Adapter Cards, 2048 for the 8 bit 4 MBps long card.   This version of IPX.COM

can be configured by typing in the following parameters at the command line

(or batch file) when it is loaded:

IPX.com - (ipx.obj generated/linked with stoken.obj) can be configured by

typing in the following parameters when it is loaded:

IPX o,ALT/PRI,mem=xx00,node=xxxxxxxx,slot=n,tbz=nnnn

where:

o - REQUIRED to override SHELL.cfg.

ALT - Configure the Driver as the ALTERNATE Adapter

PRI - Configure the Driver as the PRIMARY Adapter -- PRI is the DEFAULT

mem=xx00 xx00 is a HEXADECIMAL address of where Shared Ram is located.

The DEFAULT is D800 for PRIMARY, D400 for ALTERNATE.  Microchannel

machines already set this mem address in the MACHINE REFERENCE, and

the machine reference setting ALWAYS overrides ANY "IPX

o,PRI/ALT,mem=xx00" driver setting.  ISA bus adapters MUST be set

with THIS driver's "mem=xx00" setting because there is no other

way.

node=xx  xx is up to an 4 byte HEXADECIMAL Number specifying the Node

Address.  4000h will be PREFIXED to this number to form a 6 byte

NODE Address: 4000xxxxxxxx.

slot=n   n is a DECIMAL number specifying the MICROCHANNEL slot number

where the board is located.  ONLY valid if on a MICROCHANNEL

Machine.

tbz=nnnn  nnnn is a DECIMAL number specifying the SIZE of the Transmit

buffer.  It can be ANY number between 512 and 4202.  The driver

will ADJUST it to the NEAREST power of TWO PLUS 106 bytes of

overhead.  The DEFAULT is 4096 (4202).

ALL Parameters are separated by COMMAs or BLANKS, are OPTIONAL, are NOT case

sensitive, and may be entered in ANY order.

*****************   DOSWOC/DOSWS.ZIP MODIFICATION HISTORY  ****************

4-29-91   Initial release of DOSWS.DOC & DOSWS.ZIP.

11-11-91  Updated the STOKEN.OBJ to version 2.63.  Added note about

SLANSUP.OBJ.

05-27-92  Updated the SLANSUP.OBJ from version 2.62 to version 2.63. (revised

note about SLANSUP.OBJ and explained a common problem with 2.62

SLANSUP.OBJ.

************************  END OF MODIFICATION HISTORY************************

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************