DOSODIWS.DOC

This DOSODIWS.DOC accompanies DOSODIWS.ZIP which includes the ODI Token-Ring

workstation drivers for Netware 2.1x, 2.15c, 2.2, 3.1,and 3.11.  The

following files accompany this revision:

LANSUP   COM    14094   4-30-91  10:32a   (certified)

TOKEN    COM    15663   6-14-91   4:10p   (certified)

LSL      COM     7662   1-30-92   4:05p	  (certified)

IPXODI   COM    20903  11-20-91   4:57p   (certified)

DOSODIWS DOC           (this file)

The DOS ODI shells provide functionality to Netware and network

computing environments by supporting multiple protocols and multiple LAN

adapters in a single workstation.

ODI (Open Data-Link Interface) creates a "logical network board" to

allow multiple frame formats over one network board and wire.

Both LAN drivers support RPL, Source Routing, and a 4K packet size.

These LAN drivers are part of the ODI shell, they require no shell

generation.  They load with the following drivers in the following order.

LSL.COM, LAN Driver, IPXODI.COM, NETx.COM. (NOTE: Other ODI drivers (latest)

are in ODIUP3.ZIP, The latest version 3.22 NETX.COM, EMSNETX.EXE, and

XMSNETX.EXE are located in DOSUP3.ZIP on the NDD area.  These are the shells

that were tested as a group with the ODI drivers in TOKENB.ZIP.)

The ODI shell will load any legally configured parameters from the

SHELL.CFG or NET.CFG files.  NET.CFG file is an enhanced SHELL.CFG file and

supports additional parameters.  Any options available in the SHELL.CFG file

can be incorporated in the NET.CFG file.  Options from both files will be

loaded by the shell.  If you wish to incorporate the SHELL.CFG file into the

NET.CFG file, place the appropriate SHELL.CFG commands LEFT JUSTIFIED at the

beginning of the NET.CFG file.  (See the sample NET.CFG file, located later

in this document, as a guide.)

Token.com -    This is the Native Token-Ring driver for the DOS workstation.

This driver MUST be used WITHOUT the IBM Lan Support program.

Token.com defaults to a 4K packet size unless your lan board

has 8KB of shared RAM then it will default to a 2K packet

size.  Token.com will utilize the following parameters in the

NET.CFG: Node Address, Slot (microchannel only), Frame,

Protocol, Alternate, and Max Frame Size.

Lansup.com -   This is the client Token-Ring driver for the DOS workstation.

This driver MUST be used with the IBM LAN Support program.

Lansup.com defaults to a 1K packet size.  Lansup.com will

utilize the following parameters in the NET.CFG:  Frame,

Protocol, Saps, Link Stations, Alternate, and Max Frame Size.

The following NET.CFG file is a SAMPLE only to show the basic format of

the file.  Most settings are at default.  Please refer to the Novell ODI

shell for DOS booklet part #100-000871-001 for further information on

individual parameters.  The Novell ODI shell booklet is also available on

NETWIRE in 311ODI.ZIP.

Sample NET.CFG

IPX SOCKETS = 20

IPX RETRY COUNT = 20

CACHE BUFFERS = 5

LONG MACHINE TYPE = IBM_PC

PROTOCOL ipx

BIND Token

LINK DRIVER Token

Node Address 12D43

Slot 2

Frame Token-Ring

SAPS 1

Link Stations 1

ALTERNATE

Max Frame Size = 2168

When calculating the Max Frame Size the value must be a multiple of 8.

It must include the number of bytes for the data packet, for adapter overhead

(6 bytes), and for the largest possible header (currently, 35 bytes LAN

header + 5 bytes SNAP header + 74 bytes protocol header = 114 bytes).

If the line speed is 16 Mbps, the value of Max Frame Size must be

between 632 and 17960. If the line speed is 4Mbps, the value of Max Frame

Size must be between 632 and 4464.

If you want to use a 2KB packet size, the Max Frame Size will be

calculated as follows:

2048 + 6 + 114 = 2168

Then round up to the nearest multiple of 8: 2168

Although Max Frame Size can be configured for frame sizes larger than

4KB tests show no significant improvement in performance when a frame size

is larger than 4216 bytes.

************* DOSODIWS.DOC/DOSODIWS.ZIP MODIFICATION HISTORY **************

4-29-91   Initial release of DOSODIWS.DOC & DOSODIWS.ZIP.

6-25-91   Updated TOKEN.COM & LANSUP.COM to certified files.

11-11-91  Added IPXODI.COM and LSL.COM.

05-27-92  Updated IPXODI.COM and LSL.COM.

************* END OF MODIFICATION HISTORY *********************************

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************