286OS.DOC

This 286OS.DOC file accompanies 286OS.ZIP which contains IBM Token-Ring

drivers for all IBM Token-Ring Adapters (excluding the 24-bit Busmaster

card).  These drivers are for Novell v2.15c/v2.2 Operating Systems and

external Routers and Bridges.  The following files accompany 286OS.ZIP:

286OS    DOC        (This File)

ATOKENA2 OBJ     5150   7-31-91   4:49p  (certified v2.63)

BTOKENA2 OBJ     5150   7-31-91   4:50p  (certified v2.63)

CTOKENA2 OBJ     5150   7-31-91   4:51p  (certified v2.63)

DTOKENA2 OBJ     5150   7-31-91   4:52p  (certified v2.63)

TOKENA2  LAN      206   9-24-91   2:20p  (certified v2.63)

TRCONFIG COM     3826   5-14-91   4:54p  (certified by Software Test)

Each of the above files is described in detail below:

ATOKENA2.OBJ, BTOKENA2.OBJ, CTOKENA2.OBJ, DTOKENA2.OBJ - These are 286 OS

Token-Ring Drivers v2.63.  Copy these files onto a newly formatted diskette

labeled LAN_DRV_001.  They can be configured with TRConfig.COM and will be

linked into NET$OS with NETGEN/INSTALL or ROUTE.EXE/BRIDGE.EXE with

ROUTEGEN/BRGEN.  These drivers work with RPL.VP1 to support Remote Program

Load.

TRCONFIG.COM - TRCONFIG.COM (06-14-90) replaces TRCONFIG.COM (06-08-90).

TRCONFIG.COM (06-14-90) will work with the following token ring drivers:

2.50, 2.60, 2.61, 2.62, and 2.63.  TRCONFIG.COM (06-08-90) will only work

with 2.50 token ring drivers,  If TRCONFIG.COM (06-08-90) is used on 2.60,

2.61, and 2.62, it will cause driver corruption.  TRCONFIG.COM (05/14/91) has

been enhanced to modify NET$OS as well as the ?TOKENA2.OBJ.

TRCONFIG.COM is a configuration utility that will modify ATOKENA2.OBJ,

BTOKENA2.OBJ, CTOKENA2.OBJ, DTOKENA2.OBJ, and NET$OS. After running

TRCONFIG.COM, a message will be displayed on the screen saying "?TOKENA2.OBJ

Configuration SUCCESSFUL; date on file changed".  The object file

(?TOKENA2.OBJ) will be modified to have the current date and time.  If the

date and time of the file has not changed, TRCONFIG.COM did not run

successfully.  (?=A,B,C,D)

TRCONFIG.COM can be copied to the diskette labeled LAN_DRV_001 to modify

?TOKENA2.OBJ, OSEXE-1, OSEXE-2.  If using v2.1x NetWare or OSEXE if using

v2.2 NetWare to modify NET$OS.

To use TRCONFIG.COM, type trconfig <ENTER>, it will help you along.

TRCONFIG.com is used to configure the following options:

MEMA=xx00 xx00 is a HEXADECIMAL number specifying the Shared RAM Segment

of the ALTERNATE Adapter.  The DEFAULT is D400.  This value is IGNORED

if the Driver is run on a MICRO CHANNEL Machine and is defined by the

Reference diskette.

MEMP=xx00 xx00 is a HEXADECIMAL number specifying the shared RAM Segment

of the PRIMARY Adapter.  The DEFAULT is D800.  This value is IGNORED

if the driver is run on a MICRO CHANNEL Machine and is defined by the

Reference diskette.

NODE=xxxxxxxx xxxxxxxx is a 04 Byte HEXADECIMAL Number specifying the

NODE Address.  The PREFIX 4000h will be added to this number to form a

6 Byte Locally Administered Address:4000 xxxx xxxx.

NOTE: NODE=xxxxxxxx will not work with v2.62 286 O/S drivers.  It is

necessary to use the TRCONFIG.com for this option after NET$OS,

BRIDGE.exe, or ROUTE.exe have been linked and configured.  This will be

addressed in another release of 2.1x/v2.2 drivers.

TBZ=zzzz zzzz is a DECIMAL number from 512 to 65535 specifying the

Transmit Buffer Size.  The Driver WILL adjust this number to the NEAREST

Power of 2.  The Driver may adjust this number to the LARGEST packet

size the board WILL accept.  If zzzz = 0000 then 4096 will be used.  If

zzzz is between 1 and 512, then 512 will be used. The DEFAULT Transmit

Buffer Size is set to 2048 to allow MORE SERVICE Processes.

TOKENA2.LAN - This file is the options file, it will call the desired

?TOKENA2.OBJ file when selecting the Token Ring lan driver option in

Netgen.  This file can also be copied to the LAN_DRV_001.

******************* 286OS.DOC MODIFICATION HISTORY ************************

4-29-91   Initial release of 286OS.DOC.

5-30-91   Updated the following file:

TRCONFIG.COM     3422   6-14-91   4:10P

to the following certified file:

TRCONFIG COM     3826   5-14-91   4:54p

5-30-91   Removed the following file:

DCONFIG.EXE     22589   1-18-91  12:03p

11-11-91  Updated from the ?TOKENA2.OBJ v2.62 to the ?TOKENA2.OBJ v2.63

drivers. No major enhancements.  Made minor readme changes.

******************* END OF MODIFICATION HISTORY ***************************

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************