WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARD DRIVERS

FOR USE WITH NOVELL ADVANCED NetWare AND SFT NetWare

v2.1 AND v2.11

INSTALLATION GUIDE

This directory provides drivers for the "PLUS" (WD8003) family of

Western Digital LAN adapter boards:

DRIVERS         WESTERN DIGITAL ADAPTER BOARD TYPES

WD8003          StarLAN:

StarCard PLUS  (WD8003S)

StarLink PLUS  (WD8003SH)

StarCard PLUS/A  (WD8003ST/A)

Ethernet:

EtherCard PLUS   (WD8003E, WD8003EBT)

EtherCard PLUS/A  (WD8003ET/A)

EtherCard PLUS For Unshielded Twisted

Pair Wiring  (WD8003WT)

Included on this directory are the following files:

INSTALL.DOC                  This File.

RELEASE.DOC                  Version number and specific

release information.

WD8003.LAN                   The LAN driver descriptor file.

WD8003AD.OBJ                 The Server A-Driver object file.

WD8003BD.OBJ                 The Server B-Driver object file.

WD8003CD.OBJ                 The Server C-Driver object file.

WD8003DD.OBJ                 The Server D-Driver object file.

WD8003SH.OBJ                 The Shell Driver object file.

To install the drivers on the Novell Advanced NetWare v2.1

installation diskettes, copy the following files onto the

specified Novell diskettes:

WD FILE      Copy To    Novell Diskette

-------                 ---------------

WD8003.LAN -----------> AUXGEN & SHGEN-1

WD8003AD.OBJ\

WD8003BD.OBJ \

WD8003CD.OBJ  >-------> LAN_DRV_002

WD8003DD.OBJ /

WD8003SH.OBJ/

Once the drivers are installed on the Advanced NetWare diskettes,

proceed with the normal NETGEN and SHGEN procedures.  At the

point in the procedures where you are directed to select the

LAN drivers, the WD8003 drivers will appear in the list of

choices as follows:

WD Star/EtherCard PLUS

CONFIGURATION INFORMATION

For XT and AT bus boards, there are various configurations of

the drivers that you can select.  Some are designated for

adapter boards with 8 Kbyte RAM buffers, and some for boards

with 32 Kbyte RAM buffers.  The drivers configured for 32 Kbyte

RAM buffers will automatically revert to using 8 Kbyte RAM if

they find that the adapter board only contains 8 Kbyte RAM or

is configured to only use 8 Kbyte RAM.

For Micro Channel bus boards, there is only one configuration

in NetWare for you to select.  This is because the usual

configuration parameters such as interrupt, i/o base address

and RAM base address have already been established by the

system configuration procedure when you installed the adapter

board in your system.

If your NetWare server contains more than one Micro Channel LAN

adapter board, be sure that you select a different Interrupt

Request (IRQ) channel for each adapter.  To do this, you must use

the manual configuration option of the Set Configuration program

on your system Reference Diskette when you install the boards.

AUTOMATIC ADAPTATION

These Western Digital drivers automatically adjust to either

XT/AT or Micro Channel bus operation.  Each driver makes this

adjustment based on the type of adapter board that is actually

installed, regardless of the configuration you selected during the

NetWare shell generation.

Therefore, if you need a shell driver for both XT/AT and Micro

Channel bus workstations, you may generate one for the XT/AT bus

configuration that you need and use the same driver in your

Micro Channel bus workstations as well.

This automatic adaptation between XT/AT and Micro Channel bus

adapter boards is a unique feature of Western Digital drivers.

####################################################################################