WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARD SHELL DRIVER

FOR USE WITH

NOVELL ADVANCED NetWare, SFT NetWare, AND ELS NetWare LEVEL II

v2.1 AND LATER VERSIONS

RELEASE INFORMATION

Version 3.06a         6 March 1990

This Western Digital driver for Novell Advanced NetWare, SFT

NetWare and ELS NetWare Level II (v2.1 and later versions)

has been tested by Western Digital.

VERSION HISTORY

-- Version 3.06a (Current Version, March 1990) --

IBM PS/2-Models 25 and 30 are supported in this version.

The Netware 386 MAP ROOT command is supported in this version.

This version is compatible with the Novell utility JUMPERS. This

utility may be used to reconfigure the IPX.COM file and create

configurations not contained in the driver configuration table.

JUMPERS map also be used to set the StarLAN packet protocol.

Refer to the INSTALL.DOC file for more information on StarLAN

packet protocols.

-- Version 3.05 (January 1990) --

This version supports the IBM PS/2 Model 55.

-- Version 3.04 (December 1989) --

With this release of the driver new functions have been added to

increase the drivers flexibility and some old problems have been

fixed.  This driver supports the entire Star/EtherCard PLUS and

EtherCard PLUS16 family of LAN adapters.

In this release the driver will read the configuration informa-

tion directly from registers in software configured XT/AT bus

adapters (currently WD8003EB & WD8003W) in the same fashion that

Micro Channel adapters have been configured in the past. This

"Auto Configuration" feature will eliminate the need to recon-

figure the driver to match different adapter configurations.

When using StarLAN adapters this driver can be configured to use

either the Novell StarLAN driver compatible packet format (AT&T

StarLAN compatible) or the standard IEEE 802.3 packet format.

More information about these features as well as tips on how to

best utilize them can be found in the file INSTALL.DOC.

This driver contains fixes for two problems which existed in

previous drivers, including the Novell certified v2.11.3 driver.

Problem 1; A workstation problem, had to do with the IPX call to

CancelRequest and occurred only in certain NetBIOS applications

which call this IPX function. Problem 2; Occurred only on certain

adapter revisions when under high load situations.  This problem

would cause the server to "lock up" or get a GPI interrupt.

-- Version 3.02 (September 1989) --

This release provides support for the EtherCard PLUS16 adapter.

It contains a completely new driver configuration table to

support adapters with 8, 16, and 32 Kbytes of RAM.  The default

configuration has changed to IRQ 3, Base I/O 280, RAM at CA00:0

for 8K.

There is one known problem with this driver, Configuration 12

(IRQ=2, I/O Base=220, RAM at C000:0 for 32K) will NOT function

with Ethercard PLUS16 in the file server.

NOTE: This driver has been tested by Western Digital and

distrubuted on a limited basis.

-- Version 2.11.3 (February 1989) --

The driver files were moved to the root directory to comply

with new NetWare v2.15 installation procedures, and the

WD8003.LAN file was changed to specify floppy disk volume label

LAN_DRV_200 instead of LAN_DRV_002.  No software changes to the

actual driver were made.

Two additional versions of 2.11.3 are available.  These are not

Novell certified drivers.  They are slight alterations of the

certified driver designed to meet specific Novell network

installation requirements.

2.11.3b is designed to allow intermixing of Western Digital

Ethernet and StarLAN boards in a Novell bridge.  Without 2.11.3

the two segments of the network will not see each other.

2.11.3c is designed to operate with Tandy 1000SL series

computers.  If you do not use the c version with Tandy 1000SL

the driver does not determine the bus type (Micro Channel or

XT/AT) and the driver will not operate.

NOTE:  Versions b and c work with no apparant problems, however

they are not certified by Novell and cannot be distributed on

the SuperDisk.  If you require these versions contact LAN

Technical Support.

-- Version 2.11.3 (November 1988) --

Changes were made to meet Novell certification requirements.

There were no problems reported by any users with the previous

version.

-- Version 2.11.2 (August 1988) --

Problems were fixed that

o prevented interoperation with Ethernet adapters

from certain other manufacturers

o caused workstations to stop when receiving certain

queries from Novell Diagnostic Utilities

-- Version 2.11.1 (July 1988) --

The following problems having to do with differences between

NetWare v2.1 and v2.11 were corrected:

o StarLAN adapters would not operate on any NetWare v2.11

o No adapters would operate on Non-Dedicated NetWare v2.11

-- Version 2.11.0 (June 1988) --

Operation with Ethernet and StarLAN Micro Channel adapter boards

has been added to both the shell and server drivers.  The same

drivers also operate with XT/AT bus boards.

-- Version 2.10.0  (May 1988) --

Two problems that were present in previous versions of this

driver have been corrected:

o A server or workstation would occasionally stop

functioning when the network was under extremely

heavy load.

o Certain application programs that used the Novell

NETBIOS interface would not operate properly.

This version and subsequent versions have the following

characteristics:

o accommodates LAN adapter boards with 32 Kbyte RAM buffers

as well as those with the original 8 Kbyte RAM buffers

o has improved workstation transmit speeds

o the RAM buffer base address in the workstation shell is

changed from C4000 (hex) to D0000 to minimize conflicts

with other adapter boards (especially EGA, VGA)