These are MHS drivers for Zoom v.32/v.42/v.42bis 9600 baud modems.

They have been tested successfully at four sites that I know of with

no problems.  To install the drivers, please follow the instructions

below.

1)  Copy ZOOM96.* to your \MHS\SYS directory.

2)  Start MHS and select "Directory Manager", then "Setup This Host".

3)  Enter ZOOM96 for the modem driver, and 9600 as the speed.

IMPORTANT:

You *must* set the speed to 9600 for proper operation.  The Zoom

modem operates best with a 38400 baud line speed, but MHS will not

accept a rate that high.  So, the driver is set up to interpret

9600 as 38400 in order to work around MHS limitations.

4)  Select "Done" to save your changes, and "Done" again to return

to the main menu.

5)  Select "Setup Transport Server", and again enter ZOOM96 as the

modem driver and 9600 as the modem speed.

6)  Select "Done" to save your changes, and you're all set to run the

Communications Manager and dial out.

Please note that I have no connection with Novell (other than

being a member of the fan club), and these drivers are not

supported by Novell.  My ability to support them is limited to

spare time, but I am interested to hear where they're used and

how well they work for you.  Please contact me by electronic

mail ONLY - I tend to be very rude to uninvited telephone

contacts.

Scott Wertz

CompuServe - 71541,3051

MHS - SW@RRC (via CSERVE or NHUB)