From CADENCE Magazine May, 1991

Head: Make Your Own

Sub: Create Fonts with  MAKEFONT.LSP

by Brad Halls

MAKEFONT.LSP  (Listing 1) is an AutoLISP program designed to  aid

in  creating  and customizing fonts for AutoCAD. The  program  is

designed  so  that anyone can customize fonts  without  having  a

thorough  knowledge  of AutoCAD shape  files.  MAKEFONT.LSP  also

handles  the frustrating process of generating the  actual  shape

code  from  the  creator's shoulders. To  do  this,  MAKEFONT.LSP

prompts the user through the description of a character and keeps

track  of pen movements, drawing modes, and the number  of  bytes

needed  to describe the character in AutoCAD's SHP  file  format.

There  are  five  steps needed to run the program  and  create  a

custom font.

CREATING THE DRAWING

(Note: see included makefont.dxf.)

Grid  size  is the first thing to consider when  creating  a  new

font.  I found that a 50 x 50 grid works well, but the base  size

is really arbitrary. In general, the more detailed the font,  the

more accuracy needed to define it and hence the larger the  grid.

Once  the  size is decided, the drawing limits should be  set  to

match the grid.

Next, the units and snap need to be set so that the program  will

write  integer values to the SHP file. Call up the Units  command

and set the units to "decimal" (option 2), with zero places after

the decimal point. The snap should be on and set to 1. The screen

coordinates should be on and displaying integer values.

It  is  a good idea to set up a view slightly  larger  than  your

grid, so the entire character can still be seen if you decide  to

go  outside the limits (zoom .8x should do it). Now, turn on  the

grid, save the view, and end the drawing.

DESIGNING THE CHARACTER SET

Now  that the drawing grid is set up, you should be ready to  lay

out  individual  characters in the font.  Fortunately,  it  isn't

necessary  to  complete  the entire character  set  in  order  to

compile  the  font,  so it is possible to start  with  the  less-

complicated characters and compile them first.

Since  the program works by digitizing each character  to  create

the SHP file, no true arcs are allowed. You can, however, achieve

pretty good results by using a series of short line segments as I

have  shown  here  for the number 2 (Figure 1).  Of  course,  the

characters  don't  need  to  be drawn  out  at  all_they  can  be

digitized  from scratch. This, however, would be more  difficult.

Remember, the more vectors (points) it takes to describe a  font,

the  more  time  will be needed to display it, so  don't  get  to

detailed  unless you are prepared to wait for many of long,  slow

regens.

When  creating  a character set, it is also a good idea  to  take

advantage  of AutoCAD's subshape feature whenever  possible.  Any

character can be used as a subshape for another character, so  it

is  possible  to  save time and file  space  by  using  subshapes

creatively. One example is using the existing digits 0 through  9

as  subshapes  to create the fractional characters  1/16  through

15/16. The keys that the fractions are assigned to are up to you.

The lowercase keys "qwerty" and "asdfg" work well if you  usually

use caps on your drawings.

Another  advantage of creating a custom font is  that  frequently

used  characters  that are are not standard ASCII,  such  as  the

"plate"  and "center line" symbols used by many AE firms, can  be

easily  added  and  assigned to any  key  available.  After  each

character  is completed to your satisfaction, make a block of  it

to be digitized later.

BEGINNING A PROTOTYPE

FONT FILE

Before  a  font can be compiled, a special shape  number  0  that

contains  information  about  the  font  name,  size,  and   text

orientation must be added. This should be the first entry in  the

SHP file, followed by the codes for a space (ASCII 32) and a line

feed  (ASCII  10). For a 50 x 50 grid, the  first  three  entries

should look something like this: (see accompanying template.shp).

Copy this file and rename it  to whatever  you want to call your

font, then adjust the sizes accordingly.

*0,4,fontname

50,0,2,0

*32,5,space

2,8,(50,0),0

*10,5,line_feed

2,8,(0,-75),0

These  lines  of code (or something similar) need to  be  written

into a file with an SHP extension and placed in the AutoCAD  root

directory  so that the character descriptions  from  MAKEFONT.LSP

can be appended to it later.

DEFINING A CHARACTER

Now  you  are ready to begin digitizing the characters  you  have

made and writing their descriptions to the SHP file. Call up  the

MAKEFONT.DWG  and  insert  the first character  block.  Load  the

MAKEFONT.LSP  program  and follow the  instructions  from  there.

Figure 1 shows the description process for a number 2. Start  out

by picking (0,0) for the origin and then typing "U" (for pen up).

Pick  point  number 2, and type "S" to start the  continuous  pen

down  (or  CPD)  mode.  Now  pick  continuous  points  along  the

character path through point number 3. Type "E" to exit CPD  mode

and  "U"  to raise the pen. Pick point number 4 and type  "D"  to

lower  the pen, then pick point number 5. Type "U" and  pick  the

point  (50,50). The character is now complete, so type a  "Q"  to

quit  the  description process. Note that the last point  of  one

character  is  the insertion point for the next, so  if  the  "2"

would  have  used  the  entire 50 x 50  limits,  the  last  point

digitized would have been about (60,0) instead of (50,0).

The program will now ask if you want to write the description  to

a file. Respond with a "y," and MAKEFONT.LSP will prompt you  for

the  name  of  the  SHP file created in step  3.  Type  the  name

(without  the SHP extension). Next you will be prompted  for  the

ASCII  code and a short description of your character. The  ASCII

codes  for  the keyboard are listed on page 387  of  the  AutoCAD

manual. The description (not the one provided by the program  but

by  you)  should  be short and contain  no  spaces  or  uppercase

letters. Autodesk uses "uca" for an uppercase "A," but  something

like "upper_case_a" will also work and may be more readable.

COMPILING AND TESTING THE FONT

After  the character is finished, end the MAKEFONT.DWG  and  type

"7"  at the AutoCAD main menu to compile your font. Now  go  back

into  that drawing or start a new one and create a  style  (test,

for  example)  that utilizes the new font. If all goes  well,  it

should  be possible to create a fairly large character set  in  a

short  amount  of time. Compilation errors for  shape  files  are

sometimes  difficult to find, so be patient and consult  appendix

B.7  of  the AutoCAD Reference Manual  for  further  information.

Compile  and test your font often as new letters are  created  so

that you will have a good idea of where to find any problems that

may arise. One hint is to watch for trailing blanks at the end of

lines  or a missing return at the end of the SHP file. Keep  your

favorite word processor handy for debugging and have fun with it.

Good Luck! @

Brad  Halls  is  a  CAD  designer  and  programmer  for  Ruby   &

Associates, P.C. in Southfield, Michigan.