#########################    PLEP.DOC     ################################

***************************************************************************

*                                                                         *

*                                                                         *

*              PROTECTED-MODE ADI(R) PLOTTER DRIVER                       *

*                                                                         *

*                           for the                                       *

*                                                                         *

*               EPSON LQ2550 24-PIN PRINTER/PLOTTER                       *

*                                                                         *

*                                                                         *

*                                                                         *

*              PLEP.EXP Version 1.00 for AutoCAD 386                      *

*                                                                         *

*                         Revision 1.0                                    *

*                        March 18, 1991                                   *

*                                                                         *

*                                                                         *

*                                                                         *

***************************************************************************

Copyright(C) 1990, 1991 Autodesk, Inc.

All Rights Reserved

===========================================================================

Permission to use, copy, modify, and distribute this software and its

documentation for the purpose of creating device drivers or applications

for use in conjunction with Autodesk products, is hereby granted in

accordance with the terms of the License Agreement accompanying this

product.

For government end-users, these materials are provided with RESTRICTED

RIGHTS. Use, duplication, or disclosure by the U.S. Government is subject

to restrictions set forth in FAR 52.227-19 (Commercial Computer Software -

Restricted Rights) and DFAR 252.227-7013(c)(1)(ii) (Rights in Technical

Data and Computer Software), as applicable.

AUTODESK, INC. MAKES NO WARRANTY, EITHER EXPRESSED OR IMPLIED, INCLUDING

BUT NOT LIMITED TO ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR

A PARTICULAR PURPOSE, REGARDING THESE MATERIALS AND MAKES SUCH MATERIALS

AVAILABLE SOLELY ON AN "AS-IS" BASIS.

IN NO EVENT SHALL AUTODESK, INC. BE LIABLE TO ANYONE FOR SPECIAL,

COLLATERAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES IN CONNECTION WITH OR

ARISING OUT OF PURCHASE OR USE OF THESE MATERIALS. THE SOLE AND EXCLUSIVE

LIABILITY TO AUTODESK, INC., REGARDLESS OF THE FORM OF ACTION, SHALL NOT

EXCEED THE PURCHASE PRICE OF THE MATERIALS DESCRIBED HEREIN.

For conditions of use and permission to use these materials for publication

in other than the English language, contact Autodesk, Inc.

Autodesk, Inc. reserves the right to revise and improve its products as it

sees fit. This publication describes the state of this product at the time

of its publication, and may not reflect the product at all times in the

future.

Autodesk Trademarks

-------------------

The following trademarks are registered in the U.S. Patent and Trademark

Office by Autodesk, Inc.: ADI, ATC, Autodesk, Autodesk Animator, the

Autodesk logo, AutoCAD, AutoCAD Training Center, AutoLISP, AutoShade,

AutoSketch, and AutoSolid.

The following are trademarks of Autodesk, Inc.: ACAD, Advanced Modeling

Extension, Advanced User Interface, AME, AUI, AutoCAD Development System,

Autodesk Animator Clips, Autodesk Animator Theatre, Autodesk Cyberspace,

Autodesk Device Interface, Autodesk Software Developer's Kit, Autodesk

Training Center, 3D Studio, AutoFlix, CA Lab, DXF, and James Gleick's

CHAOS: The Software.

Third-Party Trademarks

----------------------

RenderMan is a registered trademark of Pixar used by Autodesk under license

from Pixar.

All other brand and product names are trademarks or registered trademarks

of their respective companies.

GOVERNMENT USE

--------------

Use, duplication, or disclosure by the U.S. Government is subject to

restrictions as set forth in FAR 52.227-19 (Commercial Computer Software -

Restricted Rights) and DFAR 252.227-7013(c)(1)(ii) (Rights in Technical

Data and Computer Software), as applicable.

***************************************************************************

INTRODUCTION

============

This document explains the installation and use of PLEP.EXP, a protected-

mode Autodesk Device Interface(TM) driver for the Epson(R) LQ2550 24-pin

printer. This driver was developed by Autodesk(R) and does not represent

an endorsement of any particular product. (Note: this driver was tested

using AutoCAD Release 11.)

Although the Epson LQ2550 is physically a printer, which would normally

mean that it would be an AutoCAD(R) printer/plotter, protected-mode ADI

printer/plotter drivers are considered plotters to AutoCAD 386. Therefore,

configuration-time and execution-time commands for protected-mode ADI

plotters, not real-mode ADI printer/plotters, will apply. (The drawing

vectors sent by AutoCAD 386 are rasterized by this ADI driver before being

sent to your Epson printer.)

For more information concerning plotting from AutoCAD 386 to your

printer/plotter, you may refer to the plotting sections in the AutoCAD 386

"Installation and Performance Guide," and the "AutoCAD (Release 11)

Reference Manual."

COPYING THE DRIVER SOFTWARE

===========================

In order for AutoCAD 386 to use your ADI driver, AutoCAD must be able to

find the proper files. To do this, copy the ADI driver (PLEP.EXP) from

your distribution disk to the directory that AutoCAD 386 will know to look

in. The method AutoCAD 386 uses to find your ADI driver is explained below.

To copy the ADI driver from your distribution disk to where you have

AutoCAD 386 installed on your hard disk, use the DOS Copy command. The

following command (entered from the DOS prompt) demonstrates this, and

assumes that the ADI driver PLEP.EXP is in a directory named \PLEP on the

distribution disk and is in the A: drive; and the AutoCAD 386 executable

files are in a directory named \ACAD on the C: drive:

C:> COPY A:\PLEP\PLEP.EXP C:\ACAD /V

CONFIGURING AUTOCAD TO USE AN ADI DRIVER

=========================================

To configure AutoCAD 386 to use an ADI driver, AutoCAD must be able to

locate the driver file (i.e., PLEP.EXP). Before starting AutoCAD and

configuring to use the ADI driver, you should tell AutoCAD where to find

the ADI driver file. This is partially done when you copy the ADI driver

to the appropriate AutoCAD directory.

AutoCAD 386 goes through several steps to locate the ADI P386 plotter

driver. First, it checks for a system environment variable named PLPADI

which provides the location of the ADI driver. The PLPADI variable can be

set to provide either a fully qualified path name or just the file name.

You can create the environment variable using the DOS Set command (this is

normally done in a batch file). The following example sets the PLPADI system

environment variable to indicate the location of PLEP.EXP, and assumes that

the PLEP.EXP file was copied to the C:\ACAD directory. AutoCAD will

automatically assume the .exp file extension so it is not required.

C:>SET PLPADI=C:\ACAD\PLEP

See the AutoCAD 386 Release 11 "Installation and Performance Guide," page

125, for more information on setting system environment variables.

If AutoCAD 386 cannot find the ADI driver using the environment variable

(or if the variable doesn't exist), then it searches for a magic filename,

ADIPLOT.EXP. The filename is referred to as a magic filename because

AutoCAD will automatically search for an ADI driver with this filename and

use it as though it was found "magically." To do this, copy the ADI driver

into the appropriate AutoCAD directory and rename the file to ADIPLOT.EXP.

The following example command accomplishes this using the DOS Copy command

with the same assumptions as above in the "Copying the Driver Software"

section:

C:> COPY A:\PLEP\PLEP.EXP C:\ACAD\ADIPLOT.EXP /V

Note: the recommended method of informing AutoCAD of the location of the

ADI driver is through the use of the system environment variable.

Where AutoCAD Searches

----------------------

AutoCAD searches for the ADI driver first in the current DOS default

directory, then the AutoCAD configuration directory (ACADCFG environment

variable), the execution directory (ACAD environment variable). If it still

hasn't found the ADI driver, AutoCAD checks the command line execution

prefix for a location. Finally, if all of these automatic searches fail,

AutoCAD will prompt you for the name and location of the driver.

Selecting the ADI Driver

------------------------

AutoCAD Release 11 recognizes only one device driver at a time for each

device type, such as your display, digitizer, or plotter devices. As

mentioned above, AutoCAD 386 implements a protected-mode ADI

printer/plotter driver as though your printer was actually a plotter.

This means that when you configure AutoCAD 386 to use the PLEP ADI driver,

you must use the "Configure plotter" menu instead of the "Configure printer

plotter" menu.

From the "Configure plotter" menu, select the "ADI P386 plotter" choice.

The P386 indicates the driver operates in protected mode. AutoCAD 386 will

try to find your ADI driver (in this case, PLEP.EXP) when you make the

selection. This is why you must have copied the ADI driver to your AutoCAD

run time environment before configuring for the ADI driver.

CONFIGURING THE ADI DRIVER

==========================

The AutoCAD 386 "Installation and Performance Guide" provides a full

description of configuring an ADI plotter driver in appendix C. However,

specific to the PLEP ADI driver, the following configuration prompt will

be asked.

After you have selected the "ADI P386 plotter" from the AutoCAD

Configuration menu and AutoCAD has found the ADI driver (in this case

PLEP.EXP), you will be asked to select a resolution from the following

prompt:

Print density table (Horizontal dots/inch versus Vertical dots/inch)

0.  60 x 180

1.  90 x 180

2.  120 x 180

3.  180 x 180

4.  360 x 180

5.  360 x 360

Select print density, 0 to 5 <3>:

You will then go through the standard AutoCAD configuration dialogue as

described in the AutoCAD 386 "Installation and Performance Guide."

PLEP DRIVER OUTPUT

==================

Maximum Size

------------

The largest size that can be plotted on the Epson LQ2550 printer is

16.0 inches horizontally by 14.3 inches vertically.

Monochrome and Color

--------------------

Plots can be produced in either monochrome or color on the Epson LQ2550.

Monochrome or color output is determined by how the plotter pen

configuration is set up. These settings can be set during the execution-

time plot configuration (when you Plot from AutoCAD).

To produce plots in monochrome, you assign one of the Epson printer's

four primary colors (yellow, cyan, magenta, or black) to all of the pens.

The plot will be generated in the primary color selected. In order to

produce plots in color, you assign a pen to a particular color using the

same pen configuration that plotters use.

When you issue and complete the Plot command, AutoCAD displays the current

plotter configuration and before doing the plot, prompts:

Do you want to change anything? <N>

If you answer Yes, the print density table is displayed and you have a

chance to select a new print density if you wish.

Next the AutoCAD table showing entity color, pen number, and line type is

displayed. You may change the pen color assignments by answering Yes to the

prompt:

Do you want to change any of the above parameters? <N>

The process for changing the pen assignments is described in the "AutoCAD

Reference Manual," page 436.

After completing the pen color assignments, the remaining reconfiguration

sequence is identical to the plotter configuration you completed when the

driver was first configured. The plot is generated upon completion of this

sequence.

The following table is the palette used for multicolor printing:

No.  AutoCAD color  Printed color  Ribbon color(s)

-----------------------------------------------------------------

1    Red             Red             Yellow, magenta

2    Yellow          Yellow          Yellow

3    Green           Green           Yellow, cyan

4    Cyan            Cyan            Cyan

5    Blue            Blue            Cyan, magenta

6    Magenta         Magenta         Magenta

7    White           Black           Black

8                    Purple          Magenta, black

9                    Brown-black     Yellow, magenta, cyan

10                   Dark blue       Magenta, cyan, black

11                   Light olive     Yellow, black

12                   Dark olive      Yellow, cyan, black

13                   Brown           Yellow, magenta, black

14                   Steel blue      Cyan, black

15                   Dark black      Yellow, magenta, cyan, black

Memory requirements

-------------------

Multicolor plotting with the PLEP ADI driver uses four times more memory

than monochrome plotting (one of the printer's four primary colors)

requires. The print density of the plot also effects the amount of memory

used (the higher the resolution, the more memory required).

The PLEP ADI driver uses a monochrome rasterizing technique whenever

possible. However, if the selected color is not one of the printer's four

primary colors or if more than one color is selected, then the multicolor

rasterizing technique is used.

For example, the color red, which is composed of yellow and magenta, will

cause the multicolor rasterizing technique to be used. This technique uses

functions that make use of four monochrome arrays, one for each ribbon

color (thus using four times the memory). In the worst case, multicolor

printing may require four passes to print each band of pixels.

Warning: a multicolor plot at 360x360 dots per inch on the maximum paper

size will consume almost 15MB of memory, and the plot will fail if there is

insufficient virtual memory (i.e., hard disk space) on your system.

COMMUNICATION INTERFACE

=======================

During driver configuration, you will be asked to select either a serial or

parallel interface to connect the Epson LQ2550 to your computer. When both

types of interface are available, the parallel interface is recommended

because the speed of transmission is much faster than the serial interface.

If you are using the serial interface, you should set your printer for

9600 baud, even parity, 8 data bits, and 1 stop bit. The serial cable

configuration is shown below:

Computer         Epson

2---------------3

3---------------2

4---|

|

5---|

6---------------20

7---------------7

Your selected interface (parallel or serial) and the serial communication

parameters (if the serial interface was selected) should be selected from

the front panel of the Epson LQ2550 printer.

######################## END OF PLEP.DOC #################################