ADI Driver for IDS Prism P80 and P132 Printers

ADIIDS.COM  is a TSR (Terminate and Stay Resident)  program  that

uses  the AutoDesk Device Interface (ADI) specification to  drive

an  IDS Prism P80 or P132 printer.  It may also work  with  other

IDS printers.  This is a monochrome driver that does not make use

of  the printer's color capabilities.  To use this  program,  you

must  load it into memory before you use AutoCAD  or  AutoSketch.

To load it into memory, make the drive and/ or directory contain-

ing ADIIDS.COM the default drive/directory, and enter

ADIIDS

at the system prompt, and press return.  This should be done once

each  time you boot your computer.  If you want to unload  ADIIDS

from memory later, enter

ADIIDS U

and press return.

Before  you can use ADIIDS.COM for the first time, you must  con-

figure  AutoCAD  or AutoSketch.  If you use AutoCAD,  select  the

"Configure  printer plotter" option from the configuration  menu,

and  select  "ADI  printer plotter" from the  list  of  available

devices.   (You  will need your disk of  printer  plotter  device

drivers  unless you have copied them to your hard disk.)  If  you

use AutoSketch, start it by entering SKETCH /R, and when you  get

to  the  Select plotter menu, select item 3.  Then,  with  either

program, answer the questions asked as shown below.  (Note:  With

AutoCAD, The first two questions are asked later, after the "dots

per inch" questions.)

Select output format: 2

Interrupt number in hex <7B>: (press Return)

Specify plot size in millimeters <N> (press Return)

Maximum horizontal (X) plot size in inches? <11.0>

Answer  8 or less to the above question if you  have  narrow

paper, or 13.5 or less if you have wide paper.

Printer dots per inch in the horizontal (X) direction <100.0> 84

Maximum vertical (Y) plot size in inches? <8.5>

Answer  10.5  to the above question if you want  to  confine

your drawing to a single sheet of paper, or any larger  num-

ber  if  you  want to allow it to "spill  over"  onto  other

sheets.

Printer dots per inch in the vertical (Y) direction <100.0> 84

Does the printer/plotter support color? <N> (press Return)

Note: Some versions of AutoCAD will not ask the above  ques-

tion.

The  rest  of the configuration for AutoCAD is the  same  as  for

supported printers, and you can consult your AutoCAD installation

manual for more information.  When you are asked

Rotate plot 90 degrees? <N>

1

type Y if you are using narrow paper, and you have a drawing that

is wider than it is high and you want it to fill the page as much

as possible.  You may also want to answer Y if you are using wide

paper and you want the drawing to spill over more than one  sheet

of paper.  When you plot a drawing, you will have the opportunity

to change this and some of the other settings.

When  you  want to plot with AutoSketch, set up a plot  box,  and

fill  in the X and Y sizes yourself.  Be sure that they  are  not

greater  than the X and Y values you entered when you  configured

the  program.   If  you are using narrow paper, and  you  have  a

drawing  that is wider than it is high, you will have to turn  on

the  "Rotate plot 90 degrees" option if the plot is to  fill  the

paper.

2

#######################################################################################################