BLK_MOD.LSP  (includes DDMODBLK.DCL)

BLOCK MODIFY is a dialog driven program was written to satisfy the need to globally edit block definitions automatically vs. the time consuming, usually futile, method of editing scales and attributes. This routine will globally update or change a selected block’s layer, rotation angle, scale, and attributes.

INSTALLATION

To install the Block Mod routine, copy both the BLK_MOD.LSP and DDMODBLK.DCL files to the directory where AutoCAD is installed.  Installation is complete!!!!

To load the LISP file inside AutoCAD, at the command prompt type:

(LOAD “BLK_MOD”) and press return.  The file will load and you will see the message, “BLK_MOD loaded... BM or BLK_MOD to start”

EXECUTION

To execute the program, type BM or BLK_MOD and press enter.  The program will display the dialog box and load the list boxes with the block and layer names.  This operation, depending in the speed of your processor and number of blocks and layers will vary.  If you are using a 486 or a Pentium, most likely you will not notice it.

DIALOG BOX

The following is a description of the elements of the dialog box and how each effects the outcome for the program.

BLOCK - Click on the arrow to the right.  From the list choose the block name you want to modify.  The data from that block will be placed into the LAYER, ROTATION, and SCALE boxes.

LAYER - The layer displayed is the current layer of the selected.  Click on the arrow to the right.  From the list choose the layer name you want the block to be changed to.

ROTATION - The rotation angle displayed is the current angle of the selected block.  Edit this data to change the rotation angle of the block.

SCALE - The scale displayed is the current scale of the selected block.  Edit the “X” scale data to change the scale of the block (X, and Y), including the attributes.

UPDATE BLOCK SCALE - If this box is checked, the routine will globally change the selected blocks all to the same scale.  For instance, two of block “A” are at scale “1” and two are at scale “3”.  This matches the scales of all block “A’s” to the number in the SCALE box.

If the selected block has attributes, the lower box, “Block Attribute Box”, will become active.

ATTRIBUTES - Click on the arrow to the right and select the attribute or “ALL”, for all attributes, to be modified.  Data from the selected attribute will be placed into the JUSTIFICATION, HEIGHT, and COLOR boxes.

JUSTIFY - The justification displayed is the current justification of the attribute selected.  Click on the arrow to the right to select a new justification.

HEIGHT -  The height displayed is the current text height of the attribute selected.  Edit the data in this box to change attribute text heights.

COLOR - The color displayed, both number and picture, is the current color of the selected attribute.  Edit the data in the text box to change the color of the attribute.

Selecting “OK” will begin the edits to the blocks.  Selecting “CANCEL” will about the program without any edits.

All the people who have tested and used this routine during production work have expressed that it is very helpful.  If you have any comments or would like to see updates or changes to the routine, please contact me:

Dave Paul, CIS  - 70524,3042

c/o TDS TELECOM - Southeast Region

P.O. Box 22995

Knoxville, TN  37932

Daytime - (615) 671-4516