Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

v1.2

ANSI Flange/Nozzle Parametrics Program

By: CADsultants

PO Box 5364

Deptford, N.J. 08096

(609) 384-1378

(c) 1990-1991

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

Table of Contents ------------------------------------------------(1)

Copyright --------------------------------------------------------(2)

Shareware Concepts -----------------------------------------------(2)

Limited License --------------------------------------------------(2)

Registration -----------------------------------------------------(2)

Disclaimer -------------------------------------------------------(3)

Trademarks -------------------------------------------------------(3)

Introduction -----------------------------------------------------(4)

System Requirements ----------------------------------------------(5)

Installation -----------------------------------------------------(5)

Recommended DOS Settings -----------------------------------------(6)

Configuring AutoCAD Rel 10  (DOS) --------------------------------(6)

Configuring  AutoCAD/386 -----------------------------------------(7)

Required Layers --------------------------------------------------(7)

Adding FLG-PAK  to your Menu -------------------------------------(8)

How to Run GenNOZZ -------------------------------------------(9 -10)

How to Run FLGface ----------------------------------------------(10)

Registration Form -----------------------------------------------(11)

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 1 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

Copyright

Copyright 1990-1991 by CADsultant All rights reserved. No part of the

publication may be reproduced, transcribed, copied or reduced to any

electronic medium or machine readable form without the written

permission of CADsultants.

Shareware Concept

Flange-PAK is not public domain or free software. but is being

distributed under the shareware concept.  This enables the user to

first use the software before deciding whether or not to purchase it.

Limited License

NON-REGISTERED users of this software are hereby granted a limited

licence for a maximum of 21 days to make a evaluation copy for trial

needs.  Because this is an evaluation you can only create nozzles

that are 150#-LJ.  At the end of this trial period, Flange-PAK must

be registered, or use discontinued.

Registration

The registration fee for Flange-PAK is US $65.00 (*).  You must send

a US $65.00 to CADsultants for one license per CPU concurrently using

Flange-PAK.  You may register Flange-PAK only with CADsultants.  No

other registrations will be recognized.  If you are updating from a

previous version, registration will only cost US $10.00 (*).

Please submit all registration and updates using either a printed

copy of the REGISTER.FRM text file including Flange-PAK (shareware

version) or a photocopy of the registration form at the end of the

document.

If and when you register you will be given the latest version of

Flange-PAK including all the supported flange types and ratings.

You will also receive a printed manual and a serial-numbered copy.

It will also show ways to customize Flange-PAK, giving you the

ability to add your own flange sizes.

(*) New Jersey State residents please add 7% sales tax.

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 2 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

Disclaimer

CADsultants makes no representations or warranties with respect to

this publication or its contents and specifically disclaims any

implied warranties of merchantability or fitness for any particular

purpose. In addition, CADsultants reserves the right to revise this

publication and to make changes from time to time in its contents

without obligations of CADsultants to notify any person of such

revisions or changes.

USE THIS SOFTWARE AT YOUR OWN RISK!

CADsultants

PO Box 5364

Deptford, NJ 08096

USA

Trademarks

Flange-PAK is a trademark of CADsultants.

IBM is a trademark of International Business Machines, Inc.

AutoCAD is a trademark of AutoDESK Inc.

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 3 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

Introduction

Flange-PAK consists of two programs which enable you to create a

multitude of nozzles in a matter of minutes. Whether you are a novice

or an advanced AutoCAD operator, this user friendly program is easily

understood the first time through.  Since "TIME IS MONEY" speed and

accuracy will be the best reason to purchase this package.  A typical

nozzle configuration can be generated at full scale, per ANSI

Specifications,  and 100% accurate in less than 1 minute. Imagine

never having to look up the diameters, thickness's, bolt size, No. of

bolts, etc..  Because both programs are written in AutoLISP they work

from within AutoCAD.  All flange information is gathered from common

DataBases that you can modify and/or add to.

GenNOZZ.lsp is the first of the two programs.  It  is used to create

full outlines as well as internal and bolting features.  GenNOZZ

consists of a series of prompts, such as Flange Type, Rating, Nominal

Size, Wall Thickness, Projection Angle, an d Projection Distance.  It

will also give you the option to generate a Blind Flange, Show Flange

Internals, and Bolt Holes. Nozzle location can be entered as

coordinates from the keyboard or by digitizing the location. Multi-

Level HELP Screens are included in the program for User-Friendly

Interfacing. Nozzles can consist of any of the following

configurations.

*) Flange Type = Slip-On, Socket Weld, Lap Joint, Weld Neck

*) Rating = 150#, 300#, 400#, 600#, 900#, 1500#, 2500#

*) Size = All Standard Sizes 1/2" to 24"

(With the ability to create custom sizes)

*) Thickness = Schd 40, 80, STD, 160, X-HVY, XXHVY or User Specify

*) Projection Distance = Any Distance

*) Blind Flange = With or Without Blind Flange

*) Flange I.D. = Show Inside Wall of Flange and Pipe

*) Bolt Holes = With or without Bolt Holes

The second program is FLGface.lsp. This program will generate the

plan view or face of the flange, showing all the necessary bolting as

well as nozzle wall and raised face on the flange.

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 4 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

System Requirements

IBM-AT  286/386 or 100% Compatible

1 Meg Recommended

(1) 5-1/4" 1.2 mb or 360k Floppy Drive

1 Meg of Hard Disk Storage

AutoCAD R10 w/ExtLisp or AutoCAD/386

Installation

Insert Flange-PAK  Program Disk in Drive A and enter the following:

A:

A:\> INSTALL

Installation program will prompt you for a directory to place

Flange-PAK's files.  You will need to use the dos "SET" command to

tell Flange-PAK where you have put the files. The install program

will automatically setup your autoexec.bat file if you choose. If you

would like to do it manually simply add the following to your

autoexec.bat or acad.bat file :

SET FLG-PAK=C:\FLG-PAK

Where "C:\FLG-PAK" is the PATH showing the location of Flange-PAK

Also Print-out the READ.ME file supplied on your Program Disk for

last minute items not covered in this document.

NOTE: If you are NOT running AutoCAD/386 be sure to read the section

"Configuring AutoCAD Rel 10 (DOS)".  There is important information

concerning Extended Lisp Setup.

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 5 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

Recommended DOS Setting

In the Root Directory of your Hard Drive there should be two files

called CONFIG.SYS and AUTOEXEC.BAT. If these files do not exist, they

must be created. If they do exist,  compare them to the settings

listed below and make the appropriate changes.

<CONFIG.SYS>

FILES=20

BUFFERS=10

<AUTOEXEC.BAT>

SET  LISPHEAP=42000

SET  LISPSTACK=3000

SET  ACADFREERAM=28

SET  LISPXMEM=1024k,128k -------For Systems w/ 1 Meg RAM or Greater

[PATH]\EXTLISP -----------------For Systems w/ 1 Meg RAM or Greater

Where [PATH] equals the location of the program EXTLISP.EXE.  This

is usually wherever ACAD.EXE and other main AutoCAD files exist.

NOTE: For optimum system performance some of the above  settings may

have to be adjusted to suit your particular system. Refer to your

AutoCAD "Installation/Performance Guide"  for further details.  Also

if your system is running AutoCAD/386, it is not necessary to add

anything listed in the Autoexec.bat sample.

Configuring AutoCAD  REL 10  (DOS)

AutoCAD may already be configured to run AutoLISP programs. If you

are not sure,  go through the following steps. Execute Autocad and

stop when you reach the "Main Menu". Type 5 (ENTER) for Configure

AutoCAD, this will display a listing of AutoCAD's current

configuration. Type (ENTER) to continue to the next menu. After the

"Configuration Menu" is displayed type 8 (ENTER) for configure

operating parameters, then 7 (ENTER) for AutoLISP Feature. When

prompted with "Do you want to enable AutoLISP" , type YES (ENTER).

The next prompt will be,  "Do you want Extended AutoLISP",  Type NO

(ENTER), unless you have Extended Memory, and have allocated memory

for Extend AutoLISP by placing SET LISPXMEM=1024K,256K and EXTLISP in

your Autoexec.bat file. Whatever the case may be, enter your chose,

and press (ENTER), then 0 <ZERO> (ENTER) to exit "Configuration

Menu", then Type YES (ENTER) to keep changes.

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 6 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

Configuring  AutoCAD/386

AutoCAD may already be configured to run AutoLISP programs.  If you

are not sure, go through the following steps.  Execute AutoCAD/386

and stop when you reach the "Main Menu". Type 5  (ENTER)  for

Configure AutoCAD,  this will display a listing of AutoCAD's current

configuration.  Type  (ENTER) to continue to the next menu.  After

the "Configuration Menu" is displayed, Type  8 (ENTER) for configure

operating parameters,  then Type 7 (ENTER) for AutoLISP Features.

When prompted with "Do you want to enable AutoLISP", Type  YES

(ENTER).  Now Type 0 <ZERO> (ENTER) to exist "Configuration Menu",

then Type  YES (ENTER) to keep changes.

Required Layers

GenNOZZ and FLGface require several existing layers in order for them

to function.  These layers can be setup in your Acad default drawing

prototype.  The prototype drawing contains all commonly used

settings, such as layers, fonts, Ltypes etc..  I f you do not have an

existing drawing prototype, Ref to your AutoCAD

"Installation/Performance Guide" for further details.  If you do have

a prototype defined, check to make sure all the layers listed below

exist.  Color and Ltype is not critical, b ut it is needed.

LAYER            COLOR           LTYPE

Main             White         Continuous

Center           Yellow          Center

Hidden           Blue            Hidden

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 7 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

Adding FLG-PAK to your Menu

For convenience, a sample menu file called "FLG-PAK.MNU" has been

provided, it allows you to run both GenNOZZ.lsp and FLGface.lsp by

picking them off the side bar menu in AutoCAD. To load menu, type the

following at the command prompt in AutoCAD :

Command: MENU (ENTER)    [path]\FLG-PAK (ENTER)

If modifying an existing menu is better for your needs, simply add

the lines show below in Italic to your current menu file.  A word

processor with a search feature is recommended do to the large size

of AutoCAD's menu files. Remember to save your menu in a non-document

form, also know as ASCII.

***SCREEN

**S

[AutoCAD]^C^C$S=X $S=S $P1=POP1 $P3=POP3

[* * * *]$S=OSNAPB

[Setup]^C^C^P(progn(prompt "Loading setup...   ")(load "setup"))

^P$S=X $S=UNITS

[FLG-PAK]$S=X $S=FP

[BLOCKS]$S=X $S=BL

[DIM:]$S=X $S=DIM ^C^CDIM

.

.

.

[__LAST__]$S= $S=

[  DRAW  ]^C^C$S=X $S=DR

[  EDIT  ]^C^C$S=X $S=ED

**FP  3

[FLG-PAK]

[-------]                                         | NOTE:

[GenNOZZ:]^c^c^P(cond ((null c:GN)+               | replace

(prompt "Please Wait....Loading Data.   ")+       | [path]

(load"[path]\\Gennozz")) (T (princ)))  ^FGN       | with the

[FlgFACE:]^c^c^P(cond ((null c:FLGface)+          | proper

(prompt "Please Wait....Loading Data.   ")+       | location

(load"[path]\\FLGface")) (T (princ)))  ^FLGface   | of FLG-PAK

**BL 3

[ATTDEF:]$S=X $S=ATTDEF ^C^CATTDEF

[BASE:]^C^CBASE

[BLOCK:]$S=X $S=BLOCK ^C^CBLOCK

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 8 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

How to Run GenNOZZ

GenNOZZ is easily learned after using it a couple of times.  This is

because of the On-Line Help, as well as the descriptive command

prompts. The first thing to do is to load GenNOZZ,  this is done by

selecting FLG-PAK from the side bar menu, then pick GenNOZZ. An

alternative is to load it manually by typing (load "[path]\\gennozz")

at the "Command:"  prompt.  The first prompt you will see is "Do You

Want HELP Screens <N>: " . If you answer with anything besides "N" ,

the program will display HELP Screens throughout its execution. Next

you will be asked "Type of Flange=LJ/SO/SW/WN: " , enter the desired

flange type.  It is important to note, you can enter uppercase,

lowercase or both.  As long as the characters entered correspond to

one of the options at the prompt.  When the next prompt displays

"Flange Rating=150/300/400/600/900/1500/2500: <150> " ,enter the

flange rating (number only). Press (ENTER) to except the default

<150> or enter one of the other values.  The next prompt is "Nominal

Size of Pipe [Inches]: " , size of pipes range from 1/2" to 24".  The

break down is as follows:

(1/2" to 1-1/2") ---------  1/4" Increments

(1-1/2" to 4") -----------  1/2" Increments

(4" to 24") --------------  1" Increments

After selecting one of the available pipe sizes (number only),

GenNOZZ will display this prompt,

"Wall Thk'ness=STD/40/80/160/XHVY/XXHVY/SPecify: <STD>. You must

select one of the available pipe schedules. or Type SP to specify.

If you selected "SP"  an additional prompt will appear, "Wall

Thk'ness [Decimal of Inch]: ".  This will allow you to specify the

exact decimal value for the Nozzle wall. The next prompt is

"Projection Angle: <0> " , now enter any desired angle.  Projection

angles must be in decimals of degrees, such as 30d30'0" would be 30.5

(ENTER).  Angles, and positive direction is as follows:

(North)

0

|

(West)  270--+--90 (East)

|

180

(South)

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 9 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

How to Run GenNOZZ

(continued)

After entering the projection angle you will see "Projection Distance

[Inches]: <6> ", this value represents the distance from the Nozzle

Base Point (Base Pt. will be selected later) to the flange face.

Type the projection distance desired in inches , and press (ENTER).

The next three  prompts allows you to "Show Bolting Holes",  "Show

I.D. of Flange" (Inside Edges) and "With Blind Flange".  All three

prompts are defaulted to <N>.  Finally you will be asked to "Pick

Nozzle Base Location: ", this can be coordinates typed in from the

keyboard,  picked with a mouse or Digitizer, utilizing all available

tools within AutoCAD, such as ENDpoint, INTsection, MIDpoint, etc...

Now just sit back and watch GenNOZZ  GO..............

How to Run FLGFace

The data entry is basically the same as GenNOZZ, with the exception

that there is much less data to enter.  Simply answer prompts as you

did in GN then select a center point for the Plan View.

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 10 of 11

Flange-PAK  v1.2  Evaluation Copy                     March 13, 1991

To register ownership of Flange-PAK v1.2, print out and complete the

following form.  Include a check or money order for $65 per copy

(US dollars,plus $5.00 within US, and $10.00 Outside of US for Shipping

Handling, New Jersey State residents add 7% sales tax) and mail

to:

CADsultants

PO Box 5364

Deptford NJ, 08096

Name: ______________________________________________________________

Title: _____________________________________________________________

Company: ___________________________________________________________

Address: ___________________________________________________________

City: __________________________ State: _____ Zip: ________________

Phone: (______) _____________________ Date: ________________________

AutoCAD version/machine type: ______________________________________

Number of copies: _______   Serial Number: __________________

How did you receive Flange-PAK v1.2

__ Order / Upgrade             __ Friend / User Group

__ Autodesk - Which division: ____________________________________

__ Software BBS - BBS Name ____________________ Phone: ___________

__ CompuServe GO ADESK Forum

Comments: __________________________________________________________

____________________________________________________________________

____________________________________________________________________

____________________________________________________________________

____________________________________________________________________

(c) 1990-1991  CADsultants - CAD/Computer Consultant      Pg 11 of 11