���������������������������������Ŀ

�                                 �

�   Latitude-Longitude Converter  �

�                                 �

�         Versions 3.0            �

�                                 �

�         TDM Software            �

�                                 �

�����������������������������������

�������������������������������Ŀ

� ABOUT THE CONVERSION SOFTWARE �

���������������������������������

This software is designed for those people who would like to

convert known latitude and longitude coordinates into X and Y

coordinates for plotting in AutoCAD or any other CAD program that

will accept DXF input files.

The user has the ability to define a Central Meridian and base

parallel as an initial starting point from which all X and Y

coordinates will be set. The user may also add an easting and

northing value to the calculated X and Y coordinates so that

negative X and Y values can be eliminated.

It is suggested that the Central Meridian and the base parallel

be set to the approximate center of the area you are mapping. The

further the input latitude and longitude values are from the

central meridian and base parallel the more distortion there will

be in the mapping of the points. The software reports a scale or

distortion factor for each converted point so that the user can

monitor.

The software converts the latitude and longitude to X and Y

coordinates by using a Transverse Mercator projection Algorithm

developed by the U.S. Geological Survey and printed in Geological

Survey Bulletin 1532.

Transverse Mercator projections are made by placing a transverse

cylinder around the earth. In effect, a light source at the

center of the earth is used to project locations on the earth

onto a map much like a slide projector projects an image onto a

screen.

The program is set up to use a Clarke 1866 Spheroid as an earth

model for developing constant values in the transverse Mercator

projection. This spheroid model is normally used for projections

centered on North America.

��������������������������������������Ŀ

� PROGRAM CONFIGURATION AND DATA INPUT �

����������������������������������������

ALL INPUT OF LATITUDE AND LONGITUDE MUST BE IN THE FOLLOWING

FORMAT.

xxxDxxx'xxx"

degrees value must be followed by the letter D or d or +

minutes value must be followed by the '

seconds value must be followed by the "

example input would be:

100D15'24"

88d10'

15D25'.5"

-80d5'25"     *

50+10'25"

15D10.5'

* YOU ONLY ENTER A NEGATIVE LATITUDE OR LONGITUDE VALUE IN THE

SETUP CONFIGURATION FILE. Input of latitude and longitude values

for conversion to X and Y values are ALWAYS entered without the

positive or negative sign. The negative sign in the configuration

file would tell the software to consider entered values for

conversion to be negative.

To start the program enter the command LATLON.

You have four choices in the main menu.

1) Input ASCI data file

2) Convert Latitude and Longitude to X-Y

* 3) Convert X-Y to Latitude and Longitude

4) Change setup configuration values

5) Quit

* Available on registered  version 3.0 software only.

�����������������������������������������������������Ŀ

� CONVERTING LATITUDE AND LONGITUDE TO X AND Y VALUES �

�������������������������������������������������������

To convert latitude and longitude to X and Y values press the 'L'

key at the main menu.

You will first be prompted for a note input for the point. This

text will be printed next to the point when it is input into a

CAD drawing. You may just press <<return>> if you no do wish to

include a note.

You will then be asked for the Latitude and longitude for the

point.

The conversion software will calculate the coordinates, apply any

easting or northing values and then display the X and Y

coordinates. A calculated scale factor will also be displayed.

The scale value is for your information only. The closer the

value is to 1 the more accurate you know the X and Y coordinate

values are. The further the input latitude and longitude are from

your central meridian, the greater the distortion/scale factor

will be.

Menu item 1 allows the user to read and ASCI format data file

of point descriptions and corresponding latitude and longitude

coordinates.

The format for the ASCI data file must be a note, latitutude and

longitude on a single line seperated by commas.

A typical data file might look like:

Plattsmouth, 41d15'25",94d10'12"

Mouth of Creek, 41d10'5",94d9'5"

Des Moines, 39d10.5',93d5.3'

*                ,38d12'5",91d9'15"

* If no note is to be included, you must still put a blank space and

comma on the data line for program execution.

Use any text editor that can read/write in an ASCI format to create

the data file.

If your are using a registered software version 3.0 you may also input an

ASCI text file of X and Y values to be converted to Latitude and Longitude.

The X and Y data file must include a note, X value, Y value on a single

line seperated by commas.

A typical X,Y data file might look like:

Lincoln,75,50

Chicago,1024,40

Cozad, -100, 40

,200,150

����������������������������Ŀ

� CREATING A DXF OUTPUT FILE �

������������������������������

The software will automatically create a DXF output file of the

converted points and notes if you have selected that option in

the setup configuration file.

If you have chosen that option in your setup, a DXF output file

will be created with the name LATLON.DXF

All latitude and longitude data converted during a session with

the software will be input into the same DXF file. Once you exit

or Quit from the main menu, the file is saved.

WARNING!!

If you wish to enter the conversion program again the file

LATLON.DXF will be REPLACED by new data. If you wish to keep the

DXF file you just created, use your DOS commands to rename the

file to a new file name with the .DXF extension to prevent

erasing it.

The DXF file can be input into AutoCAD and many other CAD

programs to generate an accurate drawing or map of the latitude

and longitude coordinates.

Consult the CAD program instructions for input of a DXF file.

�����������������������������������������������������������������Ŀ

� CONVERTING X AND Y VALUES TO LATITUDE AND LONGITUDE COORDINATES �

�������������������������������������������������������������������

Users may alternatively convert X and Y values to Latitude and Longitude

coordinates. To use this feature, simply enter X and Y values when prompted

on the menu. X values will be converted to Latitude. Y values will be

converted to Longitude. The X and Y values will be converted to Lat-Lon

based on the coordinates in your setup configuration file. The coordinates

in your setup configuration file represent 0,0.  X and Y value greater or

less than 0,0 will be offset from your setup coordinates.

���������������������������Ŀ

� SETUP CONFIGURATION FILE  �

�����������������������������

The software reads an ASCI text file name "LATLON.CFG"

The file LATLON.CFG contains basic setup information for the

software to use in calculating X and Y coordinates and for

creating the DXF output file.

A sample LATLON.CFG file is as follows.

Each line contains one setup value.

-100D  *

40D

FEET

0

0

Y

2

The first line in the file is the Central meridian (longitude) to

use as an initial X coordinate value of 0.  If the central

meridian you select is West of the Prime Meridian (which runs

thru England) you must enter then longitude as a negative value.

If the central meridian is East of the Prime meridian, you enter

the longitude as positive.

* YOU ONLY ENTER A NEGATIVE LATITUDE OR LONGITUDE VALUE IN THE

SETUP CONFIGURATION FILE. Input of latitude and longitude values

for conversion to X and Y values are ALWAYS entered without the

positive or negative sign. The negative sign in the configuration

file would tell the software to consider entered values for

conversion to be negative.

The second line is the base parallel (latitude) to use as an

initial Y coordinate value of 0. If the base parallel is north of

the equator the latitude is positive. If the base parallel is

south of the equator then the latitude must be entered as a

negative value.

The third line in the configuration file is the default UNITS to

use in conversion. Units may be in METERS, FEET or MILES. A

conversion factor is applied to all calculated X and Y

coordinates based on the units you select.

The fourth line is an easting or 'X' value to add to the

calculated X coordinate.

Line 5 is a northing or 'Y' value to add to the calculated Y

coordinate.

By adding an easting or northing value to the calculated X and Y

coordinates, you can avoid getting negative coordinates. This may

be desirable once the points are input into a CAD program. The

location of points relative to each other is not affected by the

easting and northing values.

Line 6 tells the software whether you want to create a DXF output

file of the calculated X and Y coordinates. A simple Y or N is

required.

Line 7 is the height for notes in the DXF file. This value must

be entered even if you say No to creation of a DXF output file.

To change any of these setup configuration values choose that

option from the main menu by pressing the letter C. You will be

asked for input on each of the options and a new configuration

file will be created to reflect your choices. A default

configuration file equal to the sample in this documentation file

is included with this package. The configuration file must be

present on the disk or in the same sub-directory as the main

program.

��������������Ŀ

� USER SUPPORT �

����������������

Registered users who need technical help or have questions may contact

TDM Software via E-Mail over Compuserve via user ID # 73727,2602 or surface

mail by writing to:

TDM Software

Terry Majewski

3620 L Street

Lincoln, NE. 68510

Comments and suggestions are always welcome.