PrintCAD 1.18

printer driver for IBM/DOS CAD packages

including AutoCAD and Microstation

(C) Copyright Ravitz Software Inc. 1991,1992

Ravitz Software Inc.                              BBS/FAX 606-268-0577

P.O. Box 25068

Lexington, KY  40524-5068

USA                                  Compuserve Cary Ravitz [70431,32]

Introduction ....................................................... 2

License and Registration                                             3

What Is New ........................................................ 3

Operation                                                            4

Installation ....................................................... 5

Suggested Startup .BAT Files                                         5

Syntax ............................................................. 6

Overview of Options                                                  7

/1, /2, /3, /4, /5 - Alternate Configurations ...................... 7

/A - Location Option                                                 7

/B - Tiling Option ................................................. 8

/C - Pen Color Option                                                8

/D - Output Destination Option ..................................... 9

/F - Output Format Option                                           10

/I - Scaling Point Location Option ................................ 12

/J, /K - Printer Code Prefix, Suffix Options                        12

/L - Page Layout Option ........................................... 13

/M - Magnification Option                                           14

/O - Origin and Orientation Option ................................ 14

/P - Intercept Mask Option                                          14

/Q - Number of Copies Option ...................................... 15

/R - Internal Resolution Option                                     15

/S - Pen Shading Option ........................................... 15

/T - Temporary File Option                                          15

/W - Pen Width Option ............................................. 16

/X - Control Option                                                 16

/Y - D/Y and S/E Switch Settings Option ........................... 17

/Z - Chord Angle and Enhanced Font Option                           17

Large Paper ....................................................... 18

Color Processing                                                    19

DESQview and Windows .............................................. 19

Answers                                                             20

Useful Prefix and Suffix Codes .................................... 21

Output Devices                                                      22

Compatibility and Technical Information ........................... 27

User Support                                                        27

p-2

Introduction ---------------------------------------------------------

PrintCAD is a pen plotter emulator that acts as a printer driver for

most IBM/DOS based CAD packages. It lets you print drawings directly

from CAD programs, via the CAD program's pen plotter driver - either

AutoCAD's ADI plotter driver or an HP-GL driver.

PrintCAD  works  with  AutoCAD,  AutoSketch,  Microstation,  Cadkey,

Generic CADD, Draft Choice, PC-Draft-CAD, and other CAD programs. It

will  print on Epson and  IBM 9  pin  printers, Epson, Fujitsu, IBM,

NEC,  and  Toshiba 24 pin  printers, HP LaserJet, DeskJet, PaintJet,

QuietJet, HP-GL/2  devices, and  HP-GL plotters, Canon LBP,  BJ, and

BJC printers, IBM LaserPrinter,  ExecJet, and Quietwriter 2  and  3,

Kodak Color 4,  PDP ProTracer, Tektronix ColorQuick,  and PostScript

printers, and it will generate PCX files.

PrintCAD  prints  faster than  many native  CAD printer  drivers and

provides  additional  flexibility, such as pen  width,  shading, and

color   (with  color   mixing),   and   paper   size,   orientation,

magnification, and  positioning.  PrintCAD  uses  the  best graphics

modes available for each printer that it supports.

PrintCAD  is a swapping resident program that uses 5K of base memory

and  400K of EMS or XMS memory.  Either 400K of EMS or XMS memory is

required.  EMS and  XMS are expanded and extended  memory management

protocols that are typically set  up with memory management software

such as HIMEM/EMM386, QEMM386, or  386MAX.  If you have  a  computer

with extended  or expanded memory hardware, you need to install such

a driver to use it as EMS or XMS memory.

PrintCAD  automatically loads high if a 5K DOS  or XMS  upper memory

block is available. Do not use LOADHI or LOADHIGH.

For  use with DOS extender based  programs such as  AutoCAD  386 and

Microstation  4, a VCPI handler  needs to be present.  VCPI handlers

are built into MS-DOS  5's  EMM386.EXE and  recent  versions of QEMM

386, 386MAX, and other 386 memory managers.  If PrintCAD notes "VCPI

not  present"  then you  need  to  upgrade  your  memory  management

software.

PrintCAD  has  been tested in  a variety of  environments.  However,

because it  is a  swapping resident program, unusual  situations may

cause  it to crash the system.  Please  test  it  thoroughly on your

system before using it with unsaved data.  PrintCAD  will  generally

work  with multitasking  systems  such as DESQview and  Windows, but

this is not guaranteed. See DESQview and Windows for information.

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to  make  sure  that  the  shareware

principle  works   for  you.   If  you   are  unable  to  resolve  a

shareware-related  problem  with  an ASP  member  by  contacting the

member directly, ASP may be able to help. The ASP Ombudsman can help

you resolve a  dispute or problem  with an  ASP member, but does not

provide technical support for members' products. Please write to the

ASP  Ombudsman at  545  Grover Road,  Muskegon, MI 49442  or send  a

CompuServe message via CompuServe Mail to ASP Ombudsman 70007,3536.

p-3

License and Registration ---------------------------------------------

PrintCAD is  provided  as is.  There are  no warranties expressed or

implied.  PrintCAD  is  a  swapping resident  program that may  have

problems with unusual environments.

PrintCAD  is  copyrighted  by  Ravitz Software  Inc.  You  may  copy

PrintCAD only for use under this license agreement.

PrintCAD  is distributed as shareware.  You may use PrintCAD without

charge on a trial basis to determine its suitability for you. If you

continue to  use  it  after  your  evaluation,  you must  purchase a

registered copy for $49.  This registration  covers use  by a single

person (on multiple computers) or installation on  a single computer

(may be used by a group of people).  There is no cost to use a later

1.** version after registering any 1.** version.

You may distribute PrintCAD as a stand alone product if you keep the

entire   package  together,  unchanged,   including   this   license

explanation, and do not charge more than $10. You may not distribute

PrintCAD as part of another product.

To purchase a registered copy of PrintCAD, send your name,  address,

and $49 check to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

You will  be sent  a  printed manual  and  disk (always  the  latest

version).  Please specify 3.5  or 5.25 inch disk.  If you previously

registered  PrintCAD,  you may send $15 to  get a current  disk  and

manual.

The price for multiple licenses is $49 for the first copy (with disk

and  manual) and thereafter $40 per copy with disk and manual or $25

per copy without disk or manual.

For users outside the USA, please make checks payable in US dollars,

and except  for  Canada,  please  use  a  check  from  a US bank  if

possible. Don't forget to include your country in your address.

If you register through a business, be sure to include your name.

What Is New ----------------------------------------------------------

V 1.18 - Supports HP DeskJet 500C, PaintJet XL 300, LaserJet 4. /XN

inhibits swapping for improved compatibility with DOS

extender based programs.

V 1.16 - HP-GL/2 output (/F0), you can set the nominal resolution

of PostScript or HP-GL/2, /X+ and /X- let you increase or

shrink PrintCAD's memory space, PS XP macro selects a paper

tray, /FG drives the PDP ProTracer, XMS is the default

for swapping memory.

p-4

Operation ------------------------------------------------------------

When PrintCAD  is run  for the first  time,  it installs itself  and

swaps most  of  its  code  to EMS (expanded memory) or XMS (extended

memory). Subsequent runs look for the resident copy and send control

information to it, updating the parameters.

When resident, PrintCAD intercepts calls to DOS that the CAD package

makes to output  HP-GL.  This data is used to drive the printer.  To

make this work, you must tell  PrintCAD how  to recognize that HP-GL

data is being output from your CAD package.  Choose  a filename mask

that you can match with your  CAD  package, such as *.PLT.  Set this

with the /P option (/P*.PLT).  Inside your CAD package, output HP-GL

(HP 7475, HP 7440/17440, or HP 7550) to a file name that matches the

mask. PrintCAD intercepts the data and prints the picture.

For  AutoCAD and  AutoSketch, you can use  ADI for the filename mask

(/PADI).  Configure AutoCAD for an ADI plotter on interrupt hex 78 -

up to 40 pens, 1016 dots/inch.  Then all output  to this driver goes

to  PrintCAD  via interrupt  78.  You can  also  use AutoCAD's HP-GL

driver - setup is the same as noted in the previous paragraph.

After making PrintCAD resident, you can verify that it is functional

by copying an HP-GL file to "pcad.plt" with the DOS COPY command.

For color output, the CAD package should be configured for a plotter

with multiple pens  (up to 40),  the CAD  package's  pen  assignment

table  should be  set  up match  screen colors to  pen  numbers, and

PrintCAD's /C option should set up to match.

PrintCAD beeps  the first time  that  it is sent  data  after a file

open.  If you turn on sound (/XS), PrintCAD causes a high beep after

swapping in its code and a low  beep before swapping it out,  to let

you know that it is processing.

When PrintCAD  is running,  Esc gives  stops processing  at the next

good stopping  point.  Ctrl-B stops processing immediately and  will

likely leave the printer in need of a reset.

Internally PrintCAD uses two  byte coordinates  on a 1016  dots/inch

grid. This allows for plots with coordinates of over 30 inches - far

larger  than  an HP  7475  can handle.  However, if  you  are  using

PrintCAD to emulate  a large plotter, you may  overflow the internal

coordinate system.  This condition is not checked, and it will cause

garbage output.  You can change the  internal resolution to 508 dots

per  inch (giving  60 inch maximum  plot sizes) with the  /R  option

(/R508).  This only helps  store the original  plot internally.  The

next  step in PrintCAD converts these  coordinates  to the requested

size in the resolution  of the output device,  and these coordinates

must also fit in the two byte coordinate list.

p-5

Installation ---------------------------------------------------------

Copy  the  files PCAD.EXE  and  PCADDV.EXE to  your  working disk or

directory. They are all that you need to run PrintCAD.

PCAD.EXE is the PrintCAD executable file. PCADDV.EXE is a loader for

setting up PrintCAD underneath DESQview.

In addition to the above files, these files are part in the package,

and must be included if you distribute PrintCAD as shareware:

PCAD.DOC   documentation

ADD.DOC    additional documentation since last release

ORDER.FRM  order form

README.*   quick information

Suggested Startup .BAT Files -----------------------------------------

Below  are some sample .BAT files for various situations.  Such .BAT

files may be used  to start PrintCAD or  change its parameters.  You

should not start PrintCAD from under a DOS shell, but you can change

parameters in this situation.

AutoCAD or AutoSketch, DeskJet or LaserJet III, 1/1 prints

PCAD /padi/fi

AutoCAD or AutoSketch, DeskJet 500C, 1/1 prints

PCAD /padi/fa/crgbcmyk

In AutoCAD configure  a  seven pen ADI  plotter and set up the pen

assignment table so that pens  1..7  have colors red, green, blue,

cyan, magenta, yellow, black.

AutoCAD, DeskJet (printer 1), PaintJet (LPT2), plotter (COM1)

PCAD /p#prn.plt/fi

PCAD /2/fp/ckrgbcmyk/dlpt2

PCAD /3/f8/dcom1

From AutoCAD plot to 1prn, 2prn, or 3prn.

Microstation, Canon BJ330, 1/1 print, 11x17 paper

PCAD /p*.000/f7/l10,16,.25,.25/j27 67 0 17

PC-Draft-CAD, LaserJet II, legal paper, best fit

PCAD /xa/p*.plt/mf1/fl/j27 38 108 51 65/l7.97,13.25,0,0

(/XA and/or /XL may be needed for use with PC-Draft-CAD.)

p-6

Syntax ---------------------------------------------------------------

The calling syntax of PrintCAD is:

PCAD /option1 /option2 ...

Spaces between parameters are not required.

Except for the /X and  /P options, if a given option is entered more

than once, only the last entry is used.  And if an option is invalid

(but not incorrect), it is ignored. Only one /X and one /P option is

allowed.

If you do  not enter any  parameters, PrintCAD  displays  its option

reminder screen.

Below, [  and ] enclose optional suboptions and | separates mutually

exclusive suboptions. The options and suboptions may be in any order

except  that  multiple  numeric suboptions are  ordered  and numeric

suboptions must follow character suboptions where the characters may

be digits.  "c"  refers to  a suboption character,  "x" and  "y"  to

suboption  numbers,  "i"  and "j" to suboption  integers, "b"  to  a

suboption byte, and "f" to a DOS file or device name.

For numbers (x, y), a decimal point  is allowed but is not required,

and scientific notation is not allowed.  "-" is allowed but not "+".

For  integer  input (i, j),  only  base ten  integers from -32768 to

32767 are allowed. "+" is not allowed. A byte (b) must be a base ten

integer from 0 to  255 or a hexadecimal  number prefixed with $ from

$00 to $FF.  Further range checking is done for most options to flag

unreasonable input.  If an option  has multiple  numeric suboptions,

these may be separated with commas or blanks.

********************************************************************

It  is very important to choose the right output format (/F option).

It is likely  that  the  default  output  format will  work on  your

printer, but it is equally likely  that it  will not produce optimal

results.  Please look carefully at the entire list of output formats

before  choosing one, and do not assume that because a  format works

that it is the best one to use.

********************************************************************

p-7

Overview of Options --------------------------------------------------

PrintCAD's options fall into several categories.  The first is input

and output  choices.  The input option  (/Pmask) lets you  choose an

intercept filename mask, or you may select ADI interrupt 78 by using

"adi". The output options are /F, the output format (printer model),

and /D, the destination port, device, or file.

The second category is  the page layout.  /L  controls the size  and

location of  the print  window (hard  clip limits) and the option to

box the window. Tiling is set with /B and copies with /Q.

The third category defines  how the  plot  looks  within  the  print

window.  /M,  the  magnification,  sets  the size  of  the plot.  /A

determines the location of the plot within the print window. /O sets

the  origin  and  orientation, and  is used to rotate or  mirror the

plot. /I sets the plotter's initial scaling points.

The  fourth category defines the pens, /W for the width,  /C for the

color, and /S for the shade.

The fifth category determines how PrintCAD processes the plot.  This

includes the temporary file  /T, the default and minimum chord angle

/Z,  the plotter  Y/D  and  S/E  switch position  /Y,  the  internal

resolution /R, and prefix and suffix printer codes /J and /K.

The  last category is PrintCAD setup, /P sets the intercept mask, /X

sets  up  the initial configuration, /1,  /2, /3, /4, and /5 set  up

multiple configurations.

/1, /2, /3, /4, /5 - Alternate Configurations ------------------------

These options select up to 5 alternate configurations, labeled 1..5.

/1 is  the  default.  The configuration used  for printing is chosen

based on the file name and mask (/P option).

/A - Location Option -------------------------------------------------

/A[O][x,y] - location - default /A

The A option determines the location of the plot in the print window

via a point on  the plot that is  aligned with a point in  the print

window.  You can specify the plot alignment point with x,y in inches

from the  plot's origin (before  PrintCAD applies magnification), or

let  it default to  the plot center.  For the print window alignment

point, you can default to the center or specify the origin with O.

/A      puts the center of the plot at the print window center

/AO0,0  puts the plot 0,0 at the print window origin

/A0,0   puts the plot 0,0 at the print window center

p-8

/B - Tiling Option ---------------------------------------------------

/B[i,j,x] - tiling parameters - default /B1,1,0

/B  lets  you  specify tiling options that break a  large  plot into

pages. Each page is defined by the page layout option (/L).  I and j

specify  the number of tiles (pages) horizontally and vertically.  X

specifies the overlap in inches.

/B2,2,.5  breaks the plot into 4 pages with .5 inch overlap

/B        is the same as /B1,1,0 and produces no tiling

/C - Pen Color Option ------------------------------------------------

/C[O|T]c.. - color - default /CK

/C specifies the color of each pen. The colors are B for blue, C for

cyan,  G for green, K for black,  M  for magenta,  R  for red, W for

white, and Y  for  yellow.  Unspecified  pens use the  last selected

color  (/CRGB  is the same as /CRGBBBBBB).  For  black/white  output

formats, all nonwhite colors become black.

Additional colors can be created with shade mixing. These colors use

shading patterns  to  mix  the  eight pure colors.  The format for a

mixed color is  .??  where each ?  represents any of  the eight pure

colors.  The first  color is printed  with the pen's shading pattern

(/S option)  and the  second  is printed  with  the  inverse shading

pattern.  If  the shade value is  0 (solid) it is replaced by 2 (one

dot checkerboard) which produces the most uniform mixed colors.

Mixed colors need line  widths  of at least  2 (4  for 2 dot shading

patterns) to look good, and they may have  visual artifacts on edges

near a 45 degree angle.  Multipass dot matrix formats may not handle

color mixing well.

By default, colors are  overwritten into the print,  so they  appear

opaque.  You can  switch to translucent colors (ored into the print)

with the  T suboption, and then back to opaque with the O suboption.

Make  this  mode switch in front of the pen that you want to change.

For PostScript devices and plotters, the color processing is done by

the device, not PrintCAD.  PostScript  devices  use opaque lines and

plotters use translucent lines.

" means repeat the previous color.

/CKOR.MB""T.KY  sets pen 1     to black

pen 2     to opaque red

pen 3..5  to opaque magenta on blue

pen 6..40 to translucent black on yellow

p-9

/D - Output Destination Option ---------------------------------------

/D[+]f - destination - default /D1

The D option specifies the output printer port, device, or file. The

+ suboption causes files to  be appended rather than overwritten (it

has no effect on device output). 1, 2, and 3 select the BIOS printer

routines.  X1, X2, X3, and X4 select xon/xoff controlled serial port

drive.  H1,  H2,  H3, and H4 select hardware controlled serial  port

drive. You can also use the DOS devices LPT1, COM1, etc.

PrintCAD cannot set up the serial ports. This is usually done in the

AUTOEXEC.BAT with MODE (probably MODE COM1:9600,N,8,1,P).

If you  specify a three digit number  for a file name extension (for

example .000), and multiple pages are output via the multiple copies

option or tiling, then rather than  appending the destination  file,

additional files  are  created  with sequential extensions.  This is

useful for PCX output.

/D2          sends output to the second parallel port via BIOS

/DLPT3       sends output to the DOS device LPT3

/D+TEMP.PRN  appends output to file TEMP.PRN

/DOUTPUT.000 sends output to OUTPUT.000, OUTPUT.001, ...

p-10

/F - Output Format Option --------------------------------------------

/Fc[-|+|*][!][~][`][x,y] - output format - default /FN~

The F option specifies the output format. -, +, and * are resolution

modifiers, !  selects a compression  mode driver, ~ turns  off color

processing,  and  `  turns off  four plane  color processing.  These

suboptions are allowed even if they have no effect.

For  HP-GL (/F8), HP-GL/2 (/F0), and PostScript (/FS) you can append

the nominal device resolution.  Matching the  nominal  resolution to

the printer may yield improvements in print quality.

You can append the horizontal and vertical  resolution of the output

device  in  dots/inch.  This  overrides  the  normal  output  device

resolution, but does  not affect the structure of the output.  These

numbers should follow  all other suboptions.  In addition to letting

you choose any resolution  for a  bit map,  this  feature  lets  you

compensate for print size error.  For example, a 360x360 dpi printer

consistently under indexes so that  a 6 inch high box comes out 5.94

inches.  Using 360,363.6 (6/5.94*360 = 363.6) removes the error. For

print size correction, you also  specify a number from .5 to 2 which

is  interpreted as  the  nominal  resolution  time this number.  For

example on a 360 dpi printer, 1.0,1.01 is the same as 360,363.6.

Drivers marked below with ! have a compression mode driver, selected

with "!".  This may be  very useful, but it depends on your printer.

These  drivers compress the  output data by breaking  bit map strips

with printhead positioning commands.  This  saves data  transmission

time, but some  printers spend more  time positioning the  printhead

than is saved by not sending the data.

A  number of  printers use  compression  mode by default.  It can be

turned off with � (ASCII 173).

Drivers marked below with ~  are color  devices.  Using ~ suppresses

color processing, substituting  black for  all colors  except white.

The ` suboption switches  between  4 and 3 plane  color  (see  Color

Processing).

/F0     ~ HP-GL/2 ......................................... 600x600

/F0i    ~ HP-GL/2                                             ixi

/F1     ~ IBM 9 pin ....................................... 120x72

/F1+    ~ IBM 9 pin (2 pass)                                120x144

/F1*    ~ IBM 9 pin (4 pass) .............................. 240x144

/F2       IBM Quietwriter 2                                 240x240

/F3       IBM Quietwriter 3 ............................... 240x240

/F4-      IBM LaserPrinter (PPDS mode)                      150x150

/F4       IBM LaserPrinter (PPDS mode) .................... 300x300

/F5    !~ IBM 24 pin alternate 1                            180x180

/F5+   !~ IBM 24 pin alternate 1 (2 pass) ................. 360x180

/F5*   !~ IBM 24 pin alternate 1 (4 pass)                   360x360

/F6    !  IBM Proprinter X24 .............................. 180x182

/F6+   !  IBM Proprinter X24 (2 pass)                       360x182

/F6*   !  IBM Proprinter X24 (4 pass) ..................... 360x364

/F7       Canon BJ (Proprinter mode), IBM ExecJet           360x360

/F7+      Canon BJ (Proprinter mode), IBM ExecJet (2 pass)  360x360

p-11

/F8       HP-GL .......................................... 1016x1016

/F8i      HP-GL enhanced (7550 enhanced mode)                 ixi

/F9    !~ NEC 24 pin ...................................... 180x180

/F9+   !~ NEC 24 pin (2 pass)                               360x180

/F9*   !~ NEC 24 pin (4 pass) ............................. 360x360

/FA-    ~ HP DeskJet 500C, Mannesmann Tally MT92C           150x150

/FA    !~ HP DeskJet 500C, Mannesmann Tally MT92C ......... 300x300

/FB     ~ Canon BJC 800                                     360x360

/FB+    ~ Canon BJC 800 (line overlap) .................... 360x360

/FD-      Canon LBP (ISO mode)                              150x150

/FD       Canon LBP (ISO mode) ............................ 300x300

/FF    !~ Fujitsu 24 pin                                    180x180

/FF+   !~ Fujitsu 24 pin (2 pass) ......................... 360x180

/FF*   !~ Fujitsu 24 pin (4 pass)                           360x360

/FG       PDP Protracer (Proprinter mode) ................. 360x360

/FI-      HP LaserJet 3, DeskJet                            150x150

/FI       HP LaserJet 3, DeskJet .......................... 300x300

/FI+      HP LaserJet 4                                     600x600

/FJ       Canon BJ (BJ130 mode) ........................... 360x360

/FK     ~ Kodak Diconix Color 4                             192x192

/FL-      HP LaserJet ..................................... 150x150

/FL       HP LaserJet                                       300x300

/FN     ~ Epson 9 pin ..................................... 120x72

/FN+    ~ Epson 9 pin (3 pass)                              120x216

/FN*    ~ Epson 9 pin (6 pass) ............................ 240x216

/FO    !~ Toshiba 24 pin                                    180x180

/FO+   !~ Toshiba 24 pin (2 pass) ......................... 360x180

/FO*   !~ Toshiba 24 pin (4 pass)                           360x360

/FP    !~ HP PaintJet ..................................... 180x180

/FQ       HP QuietJet (PCL mode)                            192x192

/FR     ~ Tektronix ColorQuick ............................ 216x216

/FS     ~ PostScript                                        600x600

/FSi    ~ PostScript ........................................ ixi

/FT    !~ Epson 24 pin                                      180x180

/FT+   !~ Epson 24 pin (2 pass) ........................... 360x180

/FT*   !~ Epson 24 pin (4 pass)                             360x360

/FU-      HP LaserJet 2 ................................... 150x150

/FU       HP LaserJet 2                                     300x300

/FW     ~ Epson 9 pin alternate ........................... 120x72

/FW+    ~ Epson 9 pin alternate (3 pass)                    120x216

/FW*    ~ Epson 9 pin alternate (6 pass) .................. 240x216

/FX    !~ IBM 24 pin alternate 2                            180x180

/FX+   !~ IBM 24 pin alternate 2 (2 pass) ................. 360x180

/FX*   !~ IBM 24 pin alternate 2 (4 pass)                   360x360

/FZ     ~ ZSoft PCX ....................................... 100x100

/FZi,j  ~ ZSoft PCX                                           ixj

p-12

/I - Scaling Point Location Option -----------------------------------

/IA|R|W[N] - IP point location - default /IA

This  option is not useful with AutoCAD or any software that outputs

HP-GL that  does not use  scaling commands  or that initializes  the

plotter or sets the scaling points.

The  I option  sets the initial scaling points (P1, P2 - the scaling

points  may  be set  in  the HP-GL  file  with  the IP  command).  A

(absolute) sets the points  to the default  for  the paper size (see

below).  R fits P1 and P2 to the print window with at least .25 inch

margins  and an x/y ratio of 10/7.2 (the same ratio as  the HP  7475

with small paper).  W fits P1 and P2  to the print window with a .25

inch margin. N makes the margin 0 for R or W and is ignored with A.

The  default scaling points depend on the print window  (/L  option)

with the implied paper size and the plot orientation (/O option).

paper    options            default scaling pts  with RO90 command

7440 A   /O1 /L 7.54,10.14  250,279 10250, 7479  279,250  7479,10250

7440 A4  /O1 /L 7.54,10.74  250,279 10250, 7479  279,250  7479,10250

7475 A4  /O1 /L 7.84,10.20  250,596 10250, 7796  154,244  7354,10244

7475 A4  /O1 /L 7.60,10.88  603,521 10603, 7721    0,610  7200,10610

7475 B   /O1 /L10.20,16.38  522,259 15722,10259  283,934 10283,16134

7475 A3  /O1 /L10.88,15.90  170,602 15370,10602  607,797 10607,15997

7440 A   /O2 /L10.14, 7.54  250,279 10250, 7479  279,250  7479,10250

7440 A4  /O2 /L10.74, 7.54  250,279 10250, 7479  279,250  7479,10250

7475 A   /O2 /L10.20, 7.84  250,596 10250, 7796  154,244  7354,10244

7475 A4  /O2 /L10.88, 7.60  603,521 10603, 7721    0,610  7200,10610

7475 B   /O2 /L16.38,10.20  522,259 15722,10259  283,934 10283,16134

7475 A3  /O2 /L15.90,10.88  170,602 15370,10602  607,797 10607,15997

anything else               250,279 10250, 7479  279,250  7479,10250

/O3 and /O4 gives the same default scaling points as /O1 and /O2.

/IA   uses the default scaling points

/IWN  sets the scaling points to fit the print window

/J, /K - Printer Code Prefix, Suffix Options -------------------------

/J[b,b,..] - printer code prefix - default /J

/K[b,b,..] - printer code suffix - default /K

/J lets you send codes to the  printer before  PrintCAD's usual data

is  sent and /K lets you send codes  after PrintCAD's usual data  is

sent.  This lets  you  set  up  and reset the  printer  for  special

situations. For PostScript and HP-GL output, the /J codes follow the

initialization commands.

The  codes  are  specified  with  byte  values  (0..255 or  $0..$FF)

separated by blanks or commas. There is a limit of 80 characters for

any option, so long sequences must be handled outside of PrintCAD.

/J27 79   sets skip perforation off  to allow plots  larger than one

page on most continuous form printers

p-13

/L - Page Layout Option ----------------------------------------------

/L[B][F|N][R][x,y[,x,y]] - page layout - default depends on printer

The L option defines the page layout.

B boxes the plot at the print window edge using pen 8.

F and N set form feed or no form feed.  Form feed is the default and

does not need to be specified. This is ignored for PCX output.

R reverses the colors on bit maps and output formats 4, D, I, L, and

U (Canon, HP, and IBM laser printers) and is otherwise ignored. This

gives the equivalent of a photographic negative.

The first optional x and y are  the print window width and height in

inches.  These define the hard  clip limits.  These values  are  not

checked against your printer's capabilities.  If you specify a print

window that exceeds  the  output  device  limits,  the  results  are

unknown.  You may need to reset your printer's margins, maximum line

length, or page length  to get  large plots  (see Large Paper).  The

size needed for a given  width and height exceeds x and y  by 8 dots

to allow for line widths.

The second optional x and  y are the left  and top margins, measured

from the printer's left margin and current  vertical position to the

print  window.  The margins for HP-GL, HP-GL/2, and  PostScript  are

measured from 0,0 (left, bottom).

THE PRINT  WINDOW PLUS MARGINS MUST FIT WITHIN THE PRINTABLE AREA.

Each printer has a default page layout.  The default print window is

7.54x10.14  for  printers   and   10.14x7.54  for   plotters.   This

corresponds  to  HP  7440  A  size paper.  The margins  are  printer

specific to compensate for different paper handling.  In  evaluating

user  options, PrintCAD processes  each  option  and  when  finished

reprocesses the default page layout for the chosen  printer and then

reprocesses  the last  user /L  option.  This  procedure gives  good

default values  so  that  your  /L  option  can  change  the  letter

suboptions while keeping the default print window.

/L7.5,3,.5,0   print window 7.5x3, left margin .5, top margin 0

/L7.54,10.14   sets window equivalent to HP 7440 with A  paper

/L7.54,10.74   sets window equivalent to HP 7440 with A4 paper

/L7.84,10.20   sets window equivalent to HP 7475 with A  paper

/L7.60,10.88   sets window equivalent to HP 7475 with A4 paper

/L10.20,16.38  sets window equivalent to HP 7475 with B  paper

/L10.88,15.90  sets window equivalent to HP 7475 with A3 paper

p-14

/M - Magnification Option --------------------------------------------

/M[F]x - magnification - default /M1

The M option specifies  the magnification.  F fits the  plot to  the

print window size. The default /M1 gives the same size plot as an HP

7475 plotter.

/MF.9  fits the plot to the print window and shrinks it 10%

/M1    prints at the same size as an HP 7475

/O - Origin and Orientation Option -----------------------------------

/O[1|2|3|4[L]] - origin, orientation - default /O

The O  option  sets  the print  window origin  and orientation.  The

origin  is  one of four corners numbered 1..4  for upper left, lower

left, lower right, and upper right, and implies a plot rotation. The

L suboption specifies a left handed coordinate system, which mirrors

the plot.  If no origin is specified then it is chosen so that the x

axis is the longer dimension.

Here are pictures of PrintCAD's view of the output media compared to

an HP 7475's view. O marks the default origin.

HP 7475                               PrintCAD

A size           B size               printer        plotter

y��������Ŀ    0,0��������Ŀy       ul o����Ŀur   ul ��������Ŀur

�        �       �        �         1 �     �4     1 �        �4

�        �       �        �           �     �        �        �

0,0����������x      �        �           �     �     ll o���������lr

�        �           �     �      2           3

�        �        ll �������lr

�        �         2        3

x����������

/O1  puts the print window's origin in the upper left corner

/O2  puts the print window's origin in the lower left corner

/P - Intercept Mask Option -------------------------------------------

/PADI|mask - intercept mask

/PADI sets up PrintCAD as an AutoCAD ADI plotter driver. /Pmask lets

you  choose  a DOS file mask as PrintCAD's intercept mask.  The mask

should not include disk  or directory information - PrintCAD ignores

this information when looking at  file names.  The mask may  contain

the character #.  This is the same as ?, but when a file matches the

mask   the  character  that  matches  the  #  selects  the  PrintCAD

configuration - 2..5  selects configurations 2..5  and anything else

selects configuration 1.

/Padi       sets up PrintCAD to take data from AutoCAD interrupt 78

/P*.plt     intercepts data written to *.plt

/P#prn.plt  intercepts data written to 1prn.plt, 2prn.plt, etc.

p-15

/Q - Number of Copies Option -----------------------------------------

/Qi - number of copies - default /Q1

/Q  sets the number of copies of  each printed page.  The default is

one.  Page printers  produce multiple  copies  faster using  the  "N

copies" prefix code (see Useful Prefix and Suffix Codes).

/R - Internal Resolution Option --------------------------------------

/Ri - internal resolution - default /R1016

/R sets the internal grid resolution  in dots/inch.  It is  normally

1016 and rarely needs  to be changed.  To fit very large plots  into

the   -32768..32767  coordinate  limit,  use   /R762.   Setting  the

resolution to a multiple  of the output device  resolution times the

magnification may yield a small improvement in print quality.

/S - Pen Shading Option ----------------------------------------------

/Sc.. - pen shading patterns - default /S0

/S  assigns a shading pattern to each pen.  Unspecified pens use the

last specified shade.  The shade values are 0..8.  0  is the default

solid line.  1..4 are 1 dot patterns (75%, 50%, 25%, 12.5%) and need

line widths of at least three to give good looking  lines.  5..8 are

2x2 dot patterns  (75%, 50%, 25%,  12.5%) and need line widths of at

least five to give good looking lines. Some printers, especially dot

matrix printers  in  multipass modes,  may wash  out shading because

they have a dot size that is large relative to the dot spacing.

1     2     3     4       5         6         7         8

����  ����  ��       �    ������    ��  ��    ��              ��

����  ����    ��   �      ��  ����    ��  ��    ��

����  ����  ��       �    ����  ��  ��  ��          ��    ��

����  ����    ��   �        ������    ��  ��      ��

/S024  sets pen 1 to solid, pen 2 to 50%, pen 3..40 to 12.5%

/T - Temporary File Option -------------------------------------------

/T[+]f - temporary file - default /TPLOT.TMP

PrintCAD  uses  a temporary file when it does not have enough memory

to handle  a plotfile.  You can use /T to put it on a RAM  disk  for

extra speed.  You can force the use of a temporary file, if the plot

has over  1024 data points, with +.  If you use a RAM  disk this may

speed printing.

p-16

/W - Pen Width Option ------------------------------------------------

/Wc.. - pen widths - default /WA

The W option assigns pen line widths. Each width may be 0..9, .10 ..

.24 dots, A..I for 2..10 units of 4/1016 inch (.1  mm), or N to turn

off the  pen.  A..I widths are  approximations, based  on the device

technology. Odd widths of 9 or above are modified to the next higher

(even) width. Unassigned pens use the last assigned width.

For nonsquare  matrix printers, PrintCAD  uses pen points  optimized

for  an  h/v  ratio of .67,  1, or  1.5.  Widths  are figured in the

direction of higher resolution, and are  not accurate  in  the other

direction.

Line widths  of over eight dots are  distorted at  the print  window

edge to fit within the four dot border.

/WA      sets pen 1..40 to 0.2 mm

/WB2.16  sets pen 1 to .3 mm, pen 2 to 2 dots, pens 3..40 to 16 dots

/X - Control Option --------------------------------------------------

/XQ - queries PrintCAD installation

/XR - removes PrintCAD if allowed

/X[E|N|X][L][A][M][S][-|+] - controls PrintCAD's setup

/XQ  checks to see  if  PrintCAD is installed.  /XR removes PrintCAD

from memory, if allowed. /XQ and /XR disallow all other options.

The other  suboptions  control setup,  and  are  effective only  for

initial setup. X specifies XMS for swapping - this is the default. E

forces the use  of  EMS  for  swapping instead  of  XMS.  N inhibits

swapping.  This forces the  entire program to load in low memory and

it will  take a large chunk of  DOS  memory.  In return,  all of the

compatibility problems of  swapping TSRs  are removed.  This  can be

very useful with DOS extender based CAD programs.

L forces PrintCAD to load low.  A sets an alternate handle (use with

PC-Draft-CAD and Drafix). M sets an alternate mouse handler. Each of

these suboptions may or may not improve compatibility.

+ or  -  increases  or  decreases  the PrintCAD  memory  space,  for

improved speed or reduced resource usage.

S adds progress beeps. This is not advised with the N suboption.

/XL  sets up PrintCAD in low memory

p-17

/Y - D/Y and S/E Switch Settings Option ------------------------------

/Y[D|Y][S|E] - D/Y and S/E switch setting - default /YDS

The Y option sets the plotter's D/Y  switch to  D  or Y and the  S/E

switch to S or  E.  Y starts  with the  plotter off and responds  to

esc.(, esc.Y, esc.), and esc.Z commands.  D  starts with the plotter

on and ignores these commands. S sets the standard interpretation of

IW (clipping window) commands.  E sets 7550 enhanced  interpretation

of IW commands, so that they use scaled coordinates.

/YD  puts the YD switch in the usual D position

/Z - Chord Angle Option ----------------------------------------------

/Z[E]i,j - default, min chord angle, enhanced font - default /Z10,5

This option is  not useful with AutoCAD or any software that outputs

HP-GL  that draws circles,  arcs, and  text as line segments  rather

than using HP-GL circle, arc, and label commands.

The  Z  option  specifies the  default and minimum chord angles  for

circles and arcs. The HP 7475 defaults to a chord angle of 5 with no

minimum.  Using chord angles of less than 10  returns  little  print

quality  improvement  and adds to processing  time.  The default  is

/Z10,5. For better emulation use /Z5,2.

The  E suboption  says to use the enhanced font,  which  doubles the

number of chords on curves in text, improving the  curve smoothness.

This is only effective on character heights above .5 cm.

/Z5,2  sets the default chord angle to 5 degrees, the minimum to 2

p-18

Large Paper ----------------------------------------------------------

PrintCAD can handle printing on  paper larger than the usual 8.5x11,

but  this  takes  some  extra  work.  First  you  need  to  increase

PrintCAD's print window to cover the large paper.  This is done with

the /L option.  The  print  window plus the  margins  (/L)  must fit

within the printer's printable area.

And  then you must make sure that your printer recognizes the larger

paper  height.  For  continuous forms  paper  this is best  done  by

setting skip perforation to  off,  either by printer switch or  with

PrintCAD prefix codes.  For Canon, Epson, Fujitsu, IBM, and NEC line

printers (dot matrix, ink jet, thermal) use /J27 79.

For  cut sheet paper, on  Canon,  Epson, Fujitsu,  IBM, and NEC line

printers use /J27 67 0 n, where n is the page length in inches.

For page  printers, legal size  paper  may  be specified  with these

printer code prefixes: HP-PCL  printers  /J27 38 108 51 65  (/K27 38

108 50 65 to  reset to letter), Canon  LBP  /J27 91  51 50 59 59 112

(/K27 91 51 48  59 59 112 to reset), IBM LaserPrinter /J27 91 70 5 0

3 1 2 1 (/J27 91 70 5 0 3 1 1 1 to reset).

To  get a 10x16  print  window on  11x17  paper with a Canon, Epson,

Fujitsu, IBM, or NEC line printer, use these options: /L10 16 .4 .25

/J27 67 0 17.  You will probably need to adjust the margins (.4 .25)

for best centering.

The  maximum  width  of  the print  window on  most  narrow carriage

printers is 8  - 8/(horizontal dpi).  For 300 dpi printers,  this is

7.97.  At 120 dpi it is 7.93.  The maximum width of the print window

on most wide carriage printers is 13.6 - 8/(horizontal dpi).  At 180

dpi this is 13.55.  At 360 dpi it is 13.57.  The  horizontal  margin

should be set to 0 to get this width.

p-19

Color Processing -----------------------------------------------------

Color graphics devices use a variety of color technologies. PrintCAD

supports these basic types.

CMY or  cyan/magenta/yellow is used by inkjet printers with no black

ink. This uses three color planes and give eight pure colors.

CMYK or  cyan/magenta/yellow/black  is used by inkjet and dot matrix

printers.  This uses  four color planes  and give eight pure colors.

Using black ink instead  of  a cyan/magenta/yellow mix gives a  more

neutral black color.

RGB or red/green/blue is used by  bit  maps.  This uses  three color

planes and give eight pure colors.

RGBI  or red/green/blue/intensity is  used and bit  maps.  This uses

four color planes. All simple colors except black are intense.  When

you  specify any color mixed with black at  shade 0, then instead of

mixing the  colors at  50%, the intensity is turned off.  This gives

fifteen pure colors.

CMYK and RGBI are used by default when they are appropriate. You can

switch to CMY or RGB with  the  ` suboption of /F.  For  dot  matrix

printers this  may speed  printing  by  reducing  memory  usage  and

eliminating the use of the black ribbon.

/FA!  (300  dpi color  HP-PCL) uses  CMY  color  by  default.  /FA!`

switches to CMYK color to let you  access true  black on the DeskJet

550C.

DESQview and Windows -------------------------------------------------

PrintCAD can generally be run inside a session/window of DESQview or

Windows. No special steps are needed for this.

PrintCAD  can  generally be run under  DESQview or  Windows but this

requires special consideration.  Run PCADDV instead of PCAD  to load

PrintCAD  (only to load,  not  to reset parameters).  This loads the

swapping area at the top  of lower memory,  avoiding  protected mode

interrupt code.  You must  have  a free 5K upper memory block to use

PCADDV. With Windows, do not use EMS memory for swapping.

p-20

Answers --------------------------------------------------------------

To access PrintCAD from a CAD package you must PLOT. Do NOT print.

If you are getting system crashes or nothing  from PrintCAD, try the

options /XA, /XM,  /XE, /XL, and all  combinations  of  these (/XAM,

/XAE, /XAL, ...). Try /XN for DOS extender based software.

If  your plot is messed up, with wraparound lines covering the page,

you have probably  exceeded the PrintCAD  internal coordinate limit.

This happens when the HP-GL coordinates exceed 32 inches. Reduce the

internal resolution  with /R762 for  coordinates up to 42  inches or

/R508 for coordinates up to 64 inches.

If  you  are  having  trouble getting  the  print  size or  location

correct, check your  /L  option.  The print window  plus  the  print

margins  must define an area that fits in the printable area  of the

paper. Printers cannot print over the entire area of the paper. They

usually require .25 to .5 inch margins.

To get color output from PrintCAD, you must:

Define a multipen plotter in your CAD package.

Set up the pens in your CAD package so each color is assigned to a

pen.

Use the /C option to assign the same colors to PrintCAD's pens.

Plot spoolers are  not generally compatible with PrintCAD, and would

be  of little value if they were.  Do  not  bother  with one.  Print

buffers/caches are generally compatible. You must install the buffer

first. A print buffer may be very useful.

If your prints are coming out not quite to scale:

For ADI plotters,  check  that your  plotter  is  defined  as 1016

steps/inch.

Check that no calibration is being done by the CAD package.

See the /F Output Format Option for print size correction.

For the  Canon BJC-800,  the right margin defaults to 8 inches.  Use

/J27 64 27 81 114 to set it to 11.4 inches.

For the  Canon BJ-330 and most wide  carriage printers,  the maximum

print window  width is 13.57  inches.  This is a printer  limitation

that you cannot get around.

If your  printer ejects the paper  before the plot is  finished, you

probably needed to specify a paper length prefix code. For most line

printers use /J27 67 0 n where n is the page length in inches.

p-21

Useful Prefix and Suffix Codes ---------------------------------------

Below,  n  represents any number 0..255, nA represents any number in

ASCII format (1 becomes 49, 2 becomes 50, 43 becomes 52 51, etc.).

Canon, Epson, Fujitsu, IBM, NEC dot matrix and inkjet printers

Reset                    27 64

N line paper             27 67 n

N inch paper             27 67 0 n

17 inch paper            27 67 0 17

Skip perforation off     27 79

Right margin n columns   27 81 n  (not compatible with IBM)

Unidirectional printing  27 85 1

HP-PCL printers

Reset                 27 69

Portrait mode         27 38 108 48 79

Top margin 0          27 38 108 48 69

Skip perforation off  27 38 108 48 76

11 inch paper         27 38 108 50 65

14 inch paper         27 38 108 51 65

11x17 inch paper      27 38 108 54 65

manual feed           27 38 108 50 72

N copies              27 38 108 nA 88  (page printers only)

HP DeskJet 500C

High quality, slow  27 42 111 49 81

No ink depletion    27 42 111 49 68

HP PaintJet XL300

Low quality, fast  27 42 111 45 49 81

PostScript

N copies      nA 32 88 67

Paper tray n  nA 32 88 80

IBM LaserPrinter

11 inch paper, tray 1, n copies  27 91 70 5 0 3 1 1 n

14 inch paper, tray 1, n copies  27 91 70 5 0 3 1 2 n

14 inch paper, manual, n copies  27 91 70 5 0 1 1 2 n

N copies                         27 91 70 5 0 0 0 0 0 n

Canon LBP

Reset                  27 99

11 inch paper          27 91 51 48 59 59 112

14 inch paper          27 91 51 50 59 59 112

Manual feed            27 91 49 113

Automatic feed         27 91 48 113

Full page memory mode  27 59 27 91 50 38 122

N copies               27 91 nA 118

Cursor to top of page  27 91 48 100

Diablo mode            27 58

ISO mode               27 59

HP QuietJet

HP-PCL mode      27 37 65

DIP switch mode  27 37 64

p-22

Output Devices -------------------------------------------------------

Canon BJ and BJC Printers, IBM ExecJet

For the BJ130e,  BJ5, BJ10,  BJ20,  BJ300,  BJ330,  and ExecJet in

Proprinter  mode, use /F7.  This is the fastest  driver for  these

printers. /F7+ gives dark print with line to line overlap.

/FJ gives 360 dpi graphics for any Canon BJ printer in BJ130 mode.

The printer is set to 10 pitch text.

You can use /FT*! with the BJ or ExecJet printers in Epson LQ mode

to get 360 dpi print, but it is very slow.

These printers  have an image  density option (switch  1-9 on  the

BJ130,  1-4  on  the  BJ300,  BJ330,  and  ExecJet).  Low  density

eliminates  every second dot.  To get the full 360 dpi resolution,

use high density.  For the BJ300, BJ330, and ExecJet, also be sure

that the front panel FONT HQ (high quality) light is on.

Use /FB for Canon BJC printers. The prefix code 27 81 114 sets the

right margin to 11.4 inches. /FB+ gives a sparse line overlap.

Canon LBP Printers

/FD  works  with any Canon LBP printer in ISO mode.  At 300 dpi it

takes up to one  meg  of  printer memory to print a full  page  of

graphics. At 150 dpi (/FD-) 512K handles a full page.

It  may help to set the printer memory mode with /J27 59 27 91  50

38 122.

If the printer  is in Diablo mode, PrintCAD switches to  ISO mode.

To switch back to Diablo mode on completion, use /K27 58.

Epson, IBM 9 Pin Printers

/FN, /FW, and /F1 cover a wide range of Epson/IBM compatible 9 pin

dot matrix printers.  You need a  cyan/magenta/yellow/black ribbon

to get color prints. The * drivers use 1/240 graphics. /FN+, /FN*,

/FW+,  and  /FW*  use  1/216  indexing  and  do  three  vertically

interleaved passes.  /F1+  and /F1* use  1/144 indexing and do two

vertically interleaved passes. /FW sets line feeds to 1/6 inch.

For Epson printers,  the IBM Graphics  Printer, and other printers

with 1/216 indexing hardware, /FN is preferred. For some not quite

compatible  printers, /FW works.  For  IBM  Proprinters  and other

printers with 1/144  indexing hardware, /F1  gives better results.

Some Proprinters do not handle 1/240 graphics correctly.

These drivers use the following escape codes:

esc 3  set n/216 line feeds for all /FW

esc J  index n/216 (n=2 for 1/144 index) for all /FN and /F1

esc L  graphics command for /FN, /FN+, /FW, /FW+, /F1, and /F1+

esc Z  graphics command for /FN*, /FW*, and /F1*

esc r  set ribbon color (only if color is specified)

p-23

Epson, Fujitsu, NEC, Toshiba 24 Pin Printers

/FT  covers  all  Epson LQ printers and  many  compatible printers

(Panasonic in LQ mode and others).  /FF works with  Fujitsu 24 pin

printers in Fujitsu mode.  /F9 works with NEC 24 pin printers. /FO

works  with Toshiba 24  pin printers.  /FO  resets the  line  feed

distance to 1/6 inch on completion.

For all of  these  drivers, you  need  a cyan/magenta/yellow/black

ribbon to get color prints. The + and * drivers use 1/360 graphics

mode,  and  the * drivers use 1/360  (1/120 for /FO*) indexing and

reset the line  feed distance to 1/6 inch on completion.  A  clean

paper path with  equal tension on  each side  is  needed  for good

360x360 graphics.

Many 24 pin printers are not capable of 360x360 graphics and a few

cannot handle 360x180 graphics.

The compression mode drivers  (! suboption)  may or  may not be an

improvement with these printers.

The /FT drivers use the following printer escape codes:

esc $    absolute tab

esc * '  graphics command for /FT

esc * (  graphics command for /FT+, /FT*

esc +    set n/360 indexing for /FT*

esc 2    set 1/6 indexing for /FT*

esc J    index n/180 for /FT, /FT+

esc r    set ribbon color (only if color is specified)

HP DeskJet, HP PaintJet XL300, Mannesmann Tally MT92C

/FI works with any black/white DeskJet in portrait mode.

/FA drives the  DeskJet 500C, PaintJet XL300, MT92C, and any color

300 dpi HP-PCL printer that supports data transfer mode 2.

/FA  has three modes and covers different  levels of 300 dpi color

HP-PCL printers.  /FA uses RGB color  and  works with the  DeskJet

500C and Mannesmann  Tally MT92C.  /FA!  uses  CMY color which can

speed printing  in some  cases.  This works with the DeskJet 500C,

DeskJet 550C, and PaintJet XL300.  /FA!` uses  CMYK  color to give

give better blacks on the DeskJet 550C.

HP LaserJet

/FL works with any LaserJet compatible printer.  /FU works with LJ

2 (PCL 4) printers.  /FI works with LJ 3 (PCL 5) printers. /FI+ is

a 600 dpi driver for the LaserJet 4.

All  of these drivers work in portrait mode, which may be set with

/J 27 38 108 48 79 or you can reset the printer with /J 27 69.

LaserJets with under one meg of memory may not be able to handle a

full page of graphics.  /FU uses less printer  memory than /FI, so

it may be preferred on LaserJet 3s with limited memory.

p-24

HP PaintJet

/FP drives 180 dpi PaintJet printers.  The compression mode driver

(/FP!)  uses  data  transfer  mode 2 (instead of 1) and works only

with a PaintJet XL.

/FA! (the DeskJet 500C CMY driver) works with the PaintJet XL300.

HP-GL, HP-GL/2, HP and IBM Pen Plotters

/F8  outputs low level  HP-GL, converting text,  arcs,  fill,  and

clips to  vectors.  The output  is HP 7220 and HP-GL/2 pen plotter

compatible, containing  IN,  IP, SC, SP, IW,  LT,  PA, PU,  and PD

commands.  PG is used for form feeds.  If you specify no form feed

then multiple plots may be appended to the same page.

/F0  drives  HP-GL/2  raster  devices.  Use  /F8  for HP-GL/2  pen

plotters.

You  can specify the nominal resolution  of  HP-GL  or  HP-GL/2 by

appending it to the /F option (for example /F8 300).  When you set

the nominal  resolution of HP-GL to other  than 1016,  enhanced IW

command interpretation (HP 7550 enhanced mode) is needed to handle

dashed lines correctly.

For HP-GL output  the  width parameter  (/W)  is  used  as  a  pen

selector, so for each pen in the original HP-GL, you can specify a

new  pen  in  the  output.  The  default  /W  option  for  /F8  is

/W12345678. Shading and color are ignored.

HP QuietJet

/FQ works with QuietJets in HP-PCL mode.

IBM LaserPrinter

/F4 drives any LaserPrinter in PPDS (native)  mode.  /FL will work

if the LaserPrinter is in HP LaserJet mode, but this may be slower

than  PPDS mode.  At  300 dpi it takes up to  one meg  of  printer

memory  to print a full page of graphics.  At  150 dpi (/F4-) 512K

handles a full page.

p-25

IBM 24 Pin Printers

PrintCAD  has  three drivers that  cover Proprinter X24 compatible

printers.  Use native mode, not AGM. The first driver, /F6, is for

all IBM Proprinter X24s and the 24P.  These printers do 1/144 inch

indexing,  and using them at 180 dpi results  in horizontal  white

streaks  every 2/3 inch.  The /F6 drivers compensate for  this  by

indexing 19/144 inch per line,  giving 182 dpi.  To work  properly

the printer  must  start on a 1/72 boundary.  If you keep the line

feeds  at a multiple of 1/72 (1/6, 1/8, 1/9) then this will always

be the case.

The second X24 compatible driver is /F5 and is  referred to as X24

alternate 1.  This  is for printers that are X24 compatible except

that  they do 1/180 inch indexing.  This is probably the case with

most X24  compatible  printers, including  the IBM Quickwriter and

IBM  2390 and 2391.  /F5* works only  with printers  that do 1/360

indexing, including the 2390 and 2391 but not the Quickwriter.

The third X24 compatible driver  is /FX and is referred  to as X24

alternate 2.  This  is the  same as /F5  except that  the vertical

units command. This driver works with Panasonic 24 pin printers in

X24 mode.

The compression  mode drivers (!  suboption)  may or may not be an

IBM Quietwriter 2 and 3

/F2 and /F3 drive  the Quietwriter 2 and 3.  Do not use  1 for pen

widths because  Quietwriters do not  print consistent 1 dot lines.

These  drivers  reset  the  line  feed distance to  1/6  inch upon

completion.

Kodak Diconix Color 4

/FK drives the Color 4.

Pacific Data Products ProTracer

/FG drives the ProTracer (base model) in IBM Proprinter mode.  The

maximum print window width is 14.97.

PostScript Printers

/FS outputs PostScript.  All characters, arcs, fill, and clips are

converted to simple vectors.

PrintCAD's pen  color and shading parameters are  mapped into gray

levels or RGB color levels.

You can specify  the  nominal  resolution  of  the  PostScript  by

appending it to the /F option (for example /FS 300).

You can  specify  the  number  of copies with a  prefix code  that

includes the number of copies followed by ' XC'. For example /J 50

32 88 67  makes 2 (ASCII  050) copies.  You  can specify the paper

tray with the XP macro.

p-26

Tektronix ColorQuick

/FR drives the ColorQuick.

ZSoft PCX and Fax

Use /FZ to output a ZSoft PCX bit map. The horizontal and vertical

dots/inch may  be appended  to override  the default 100x100,  for

example /FZ80,80.  Such output  should generally be directed to  a

file with the /D option.

Since  most  fax  software can  take PCX files  as  input, this is

useful for  faxing  drawings.  Use /FZ200,196  to  match  the  fax

resolution.

p-27

Compatibility and Technical Information ------------------------------

PrintCAD will run on any IBM compatible  DOS computer  with 400K  of

EMS or XMS memory and a base memory region of 304K.  Because it is a

swapping   resident  program,  it   may   have  problems   in   some

environments.  PrintCAD can handle  VCPI  (Virtual  Control  Program

Interface) based  DOS extenders,  but DPMI based programs  and stand

alone DOS extenders may cause problems.

When  using a DOS extender  based program  such  as  AutoCAD  386 or

Microstation   4,  use  an  extended/expanded  memory  manager  that

provides VCPI, such as MS-DOS 5 EMM386/HIMEM, QEMM 386, or 386MAX.

PrintCAD is a  TSR that takes over interrupt  hex 21 and watches for

DOS 2.0+ file management open functions.  If the filename on an open

command  matches  either "pcad.plt" or  the  mask specified with /P,

then  file  handle  dec 99 (dec 15 if /XA is used) is  returned  and

PrintCAD is awakened. It then responds to write, device information,

close functions, seek, and set date functions.

When awake, PrintCAD stores HP-GL commands (from  the write command)

until its 16K buffer is filled  and then it swaps in its 304K memory

space, processes the data, and swaps the original memory back.

PrintCAD  was written  by Cary Ravitz and  compiled  with  Borland's

Turbo Pascal 6.0. PrintGL Printer Driver is the graphics engine.

User Support ---------------------------------------------------------

Correspondence about PrintCAD may be sent via the  support  bulletin

board, fax, Compuserve, or mail.

BBS:        606-268-0577  1200..9600,N,8,1  24 hours/day

To ask a question, prepare an ASCII text file with the details  of

your question.  Then call the bulletin board and choose the upload

question option.  XMODEM  file transfer  protocol  is required for

this.  Your  question will  be assigned a  number (such as 1015) -

remember  this number.  An answer file (with the  assigned number)

will be posted, usually within  24 hours.  To get an answer,  call

the  bulletin board, choose the  download option,  then the answer

option, and then enter the answer number.

Compuserve: Cary Ravitz [70431,32]

Fax:        606-268-0577

If  you hand dial,  the BBS  will answer  and  hang up  (about  20

seconds), then the call is routed  to the fax.  Faxes from outside

the  USA may be answered via  letter.  The  fax may be unavailable

sometimes.

Mail:       Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA