*********************

***    P L O T    ***

*********************

Serial plotter queuing utility

Version  2.0a

January 6, 1991

(c) Copyright 1990-91  Kourosh Derakhshani

All rights reserved

10-173 Dell Place

Stanhope, NJ  07874

PLOT 2.0a                                                           Page 1

--------------------------------------------------------------------------

Specifications subject to change without notice.

PLOT 2.0a  (c) Copyright 1990-91 Kourosh Derakhshani,

all rights reserved

This document  (c) Copyright 1990-91 Kourosh Derakhshani,

all rights reserved

All brand/product names are trademarks or registered trade

names of their respective holders.

Comments can be addressed to:

K. Derakhshani, 10-173 Dell Place, Stanhope, NJ 07874

-- or --

Compuserve ID 71311,1644

PLOT 2.0a                                                           Page 2

--------------------------------------------------------------------------

LICENSE AGREEMENT

1- COPYRIGHT. This software is not and never has been free or public

domain software; it is copyrighted material, published as Shareware.

2- DISTRIBUTION. You are free to copy and distribute this software

provided that:

a) no part of it is modified or disassembled in any way

b) no fee is charged for it's use or distribution

c) it is distributed in whole, including this license agreement

3- USE. You are granted use of the software for a limited trial period, on

an "evaluation basis" only. If, after this period, the software is

found to be useful, you are asked to register and pay a $20.00 license

fee (single machine).

4- WARRANTY. THIS SOFTWARE AND ACCOMPANYING DOCUMENTATION ARE PROVIDED "AS

IS",  AND THE AUTHOR, K. DERAKHSHANI, HEREBY DISCLAIMS ALL  WARRANTIES,

EITHER  IMPLIED  OR  EXPRESS,  INCLUDING  ANY  IMPLIED  WARRANTIES   OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. IN NO EVENT  WILL

THE AUTHOR BE HELD LIABLE FOR ANY SPECIAL, INCIDENTAL, CONSEQUENTIAL OR

INDIRECT  DAMAGES WHATSOEVER, (INCLUDING, WITHOUT  LIMITATION,  DAMAGES

FOR ANY LOST PROFITS, SAVINGS, BUSINESS INFORMATION, OR OTHER PECUNIARY

LOSS), RESULTING FROM THE USE OR INABILITY TO USE THIS SOFTWARE AND ANY

OTHER  ACCOMPANYING DOCUMENTATION, EVEN IF THE AUTHOR HAS BEEN  ADVISED

OF  THE  POSSIBILITY  OF SUCH DAMAGES. SOME STATES  DO  NOT  ALLOW  THE

EXCLUSION OR LIMITATION OF INCIDENTAL OR CONSEQUENTIAL DAMAGES, SO  THE

ABOVE LIMITATIONS MIGHT NOT APPLY TO YOU.

In no event will the author's liability for ANY damages ever exceed the

price paid for the license to use this software.

The use of this software constitutes your assumed acceptance of it's

license agreement.

PLOT 2.0a                                                           Page 3

--------------------------------------------------------------------------

INTRODUCTION

The PLOT utility's purpose is to automate the task of plotting

CAD files from disk. Using PLOT, one plotter plus one PC can be

shared between several CAD stations, with individual stations

plotting their work to a disk file and carrying it on a floppy to the

"plotter server". This can be an alternative to multiple RS-232

switch boxes and networks.

Once the plot file (or collection of plot files) has been made

and copied onto a floppy, the user inserts it into the server's "data

drive" and logs PLOT to this drive. PLOT reads and displays the plot

files' directory, and at this point the files to be plotted can be

tagged, and plotted one at a time. After the files have been plotted,

the data floppy can be removed from the data drive, until either

another user is ready to make more plots, or the same user has more

disks to plot from. PLOT is logged onto the data drive containing the

new floppy, and the process is repeated.

Optionally, PLOT can automatically delete plot files, once they have

been plotted, to open space on the floppy. PLOT will also blank the

screen if the PC has not been used for a set period of time. This can

help reduce monitor wear.

REQUIREMENTS

- MS-DOS 2.1 or later

- RS-232 serial plotter

- At least 5K free space on program disk

- CAD software capable of plotting drawings to file

- COMMAND.COM available on the boot drive

PLOT 2.0a                                                           Page 4

--------------------------------------------------------------------------

INSTALLATION

PLOT consists of three files, PLOT.EXE (the program), PLOT.INI

(the initialization file), and PLOT.DOC (this file). PLOT.INI is

created through the configuration of PLOT.

Hard disk setup:

Both PLOT.EXE and PLOT.INI can be placed in the working

directory on the hard drive, and drive A: (or B:) will be assigned as

the data drive.

Two floppy only setup:

Best results are obtained with drive B: as the data drive, and

A: as both the boot and program drive. Floppy A: should be bootable,

should contain PLOT.EXE and PLOT.INI, MUST NOT be write protected,

and MUST remain in drive A: as long as PLOT is being used. PLOT uses

this drive to write temporary files and access COMMAND.COM.

One floppy only setup:

With this configuration, a single floppy is used to contain the

plot files, PLUS all the floppy A: requirements for a two floppy

system (see above). This configuration does not leave much free space

on the disk, and is therefore not highly recommended.

NOTE:

PLOT requires that the file PLOT.INI be available in the working

drive and directory, however PLOT.EXE can be placed and run from any

other accessible drive and directory.

CONFIGURATION

Before PLOT can be run for the first time, or each time a

configuration change is needed, type PLOT /C to invoke the

configuration screen. Configurable items are displayed one at a time,

along with a list of valid options, and for most items, the current

or default setting.

To select from an option list, hit the number corresponding to that

option. For all other items, enter a response which is within the

displayed limits. At any time within the configuration, ESCAPE can be

used to abort configuration without saving the changes.

Once configuration has been successfully made, PLOT is ready to run.

PLOT 2.0a                                                           Page 5

--------------------------------------------------------------------------

Configurable items:

COM port:

Select whether the plotter is connected to the PC's COM1 or COM2;

make new selection or hit ENTER to keep present setting.

Communications settings:

Check the plotter's RS-232 settings, and set the following 4 items to

match the plotter; make new selection or hit ENTER to keep present

setting.

Baud rate

Parity

Data bits

Stop bits

Data drive:

One of the server's drives is assigned as the data drive. It is

through this drive that plot files are delivered to PLOT.

If the data drive is a floppy drive other than the program/boot

drive, then each time PLOT is first started from DOS, it looks to

this drive for directory information; therefore this drive must

contain the floppy with the plot files, BEFORE the program is

started. To get around this requirement, either use ANY formatted

floppy in the data drive, or configure the data drive to be the same

as the boot/program drive. This way, PLOT can be successfully

started, and once the actual plot floppy is available, it can be

inserted in the data drive and logged onto.

Select the floppy drive that will be used as the primary data drive.

This is the drive where PLOT will always look for plot files when

first started from DOS, and must therefore contain a disk at start

up. All valid floppy and hard drives recognized will be listed; hit

drive letter for new selection, or ENTER to keep present setting.

(Note: Any valid drive can be logger onto from within PLOT).

Plot files:

PLOT can be configured to recognize up to 5 different file extension

names as plot files. The two default extensions are PLT and HP.

Hit ENTER to keep present settings. To change settings, first select

the number of different extension names to be configured by hitting

the appropriate keys 1,2,3,4 or 5. Then for each entry, type a valid

DOS extension name, up to 3 characters each.

Blanking time-out:

PLOT will blank the screen after a period of inactivity to reduce

monitor wear. This period can be set between 1 and 255 minutes. Type

in the desired time-out period.

PLOT 2.0a                                                           Page 6

--------------------------------------------------------------------------

RUNNING PLOT

Once configuration is successfully completed, PLOT is ready to use.

Insert a plot (or dummy) disk in the data drive, and type PLOT to

start.

SCREEN LAYOUT

PLOT's screen layout consists of 5 individual windows, numbered 1-5,

as shown below.

+----------------------------+

|                  | P L O T |

|                  |         |

|                  |---------|

|                  |   (2)   |

|                  |---------|

|       (1)        |   (3)   |

|                  |---------|

|                  |   (4)   |

|                  |---------|

|                  |   (5)   |

+----------------------------+

(1) Main window

The main window is used to display and select plot files. The plot

file directory is displayed as shown below :

FILE_1   PLT    88082   9-24-90   6:00p

FILE_2   PLT    97481   9-24-90   6:00p

FILE_3   PLT    27136   9-24-90   6:00p

FILE_4   PLT    62475   9-24-90   6:00p

|        |      |       |         |

|        |      |       +------------- file's time/date

|        |      +------------------ file size

|        +---------------------- file extension

+---------------------------- file name

The highlight bar is the cursor, and can be moved up and down using

the up/down arrow keys or page up/down keys.

Use the cursor keys to highlight the desired plot file(s) and tag

them.

PLOT 2.0a                                                           Page 7

--------------------------------------------------------------------------

(2) "Gas gauge"

+---------------------------+

E             2             F

The "gas gauge" is a simple indication of plot progress. When

plotting, It indicates using an arrow the relative position in the

current plot job, that is, how much of the file has been plotted so

far. It starts from empty (E) and moves toward full (F).

(3) File statistics

This window is used to indicate the following :

- the number of plot files found on the data drive,

- the number of plot files tagged to be plotted,

- total byte count of tagged plot files.

(4) Commands menu

This window contains a menu of the 7 available commands. A command is

selected by pressing it's corresponding highlight letter.

( ^ indicates the CTRL key)

Tag  Untag  ^TagAll  ^UntagAll

Log drive       Start plot

Quit

Tag :

Hit (T) to tag the highlighted file and add it to the plot list.

Untag :

Hit (U) to untag the highlighted file and remove it from the plot

list.

^TagAll :

Hit (CTRL-T) to tag all displayed files for plotting.

^UntagAll :

Hit (CTRL-U) to untag all displayed files.

PLOT 2.0a                                                           Page 8

--------------------------------------------------------------------------

Log drive :

Hit (L) to log onto another (or same) data drive. A list of valid

drive letters will be displayed in the prompt window; select the

drive to be logged by hitting the corresponding letter.

The disk will be logged onto, it's plot file directory will be

read and displayed, and PLOT is now ready.

Start plot :

Once the desired file(s) are tagged, hit (S) to start the plot

process. PLOT will prompt whether the plotted files are to be

deleted once plotted; reply (Y) to delete and (N) the keep them on

the data disk.

Starting with the first tagged file, and for each tagged file,

PLOT will announce that the file is ready to plot. Make whatever

preparations the plotter requires, then hit RETURN to start

plotting. At any point, ESCAPE can be used to abort.

Quit :

Hit (Q) to quit PLOT and exit to DOS.

(5) Prompt/entry line

This window is used for prompts or user inputs.

#############################################################################################################################