NMI  Version 1.03

by Dennis Lee

Have you ever seen the message 'PARITY CHECK' on your

display?  If you have, you've probably noticed that it

always seems to happen at the most inopportune time.  You're

cruising along putting the finishing touches on something

that you've worked long and hard, just to look up from the

keyboard to a blank display with only the words 'PARITY

CHECK' in the upper left corner.  Your heart sinks into your

shoes, your face gets flush, and palms begin to sweat, as

you try franticly to figure a way out of this one... but you

KNOW that you've lost everything!  Your only option is to

power off and start over.

NMI is a resident program to replace the non-maskable

interrupt handler that the PC uses to handle parity errors.

In the event of a parity error, NMI will interrupt the

currently executing program(s) and begin testing memory,

looking for the error.  It will continue testing until

either it locates a failing address, or you interrupt it by

pressing a key.  If a failure is located, NMI will display

the failing 64K bank, exact memory address, and if possible,

the failing bit(s). You have the option to continue or

terminate testing.  If you interrupt the memory test, or

elect not to continue testing after an error is detected,

you are given the options to return to the interrupted

program(s) with or without parity checking enabled, or to

reboot the system.

The memory test is of a nondestructive type, in that it

will preserve the current contents of memory.  It can be

executed any time without regard for concurrently executing

processes.  It's also a very thorough test, reading and

writing each byte of memory with several different bit

patterns and checking for parity errors following each read

or write operation.  Every memory location is tested,

including the locations used by NMI.

This all sounds a bit like insurance doesn't it?  It

doesn't do you a bit of good unless you have a tragedy on

your hands.  Well, I like programs that you can play with,

and since you've got this neat little memory test just

sitting there in memory, why not be able to use it? NMI will

execute the memory test any time you like, as many times as

you like!

If you're like me (a memory miser), the question in your

mind right now is... 'Just how much memory does this thing

eat?'.  The answer, approximately 1.8K including the Program

Segment Prefix and a normal size environment.  In English,

that means about 1.8K TOTAL!  Even I can afford that.

Page 1

NMI  Version 1.03

by Dennis Lee

PROGRAM LOADING

---------------

NMI /Q/I/T####

| | |

| | Test memory #### times.

| |

| |   If the number of test repetitions is 0 or

| |   blank, then the memory test loops indefinitely

| |   until an error is detected or you interrupt it

| |   by pressing a key.  This option may be used to

| |   invoke the memory test from the command line

| |   or a batch file.  NMI need not be installed to

| |   run the memory test.

| |

| Install NMI.

|

|   This option must be selected the first time NMI

|   is executed if you desire to replace the current

|   non-maskable interrupt handler.  It need not,

|   and should not be specified again.

|

Quiet

This option is selected to suppress display of the

program logo when NMI is installed.

*  If no parameters are specified, or the specified

parameters are invalid, NMI will display the program logo

followed by a summary of valid parameters.

*  Parameters need not be separated, however, they may be

separated by one or more spaces, tabs, commas, slashes,

or semicolons.

*  Alphabetic parameters may be upper or lower case.

*  Parameters may be entered in any order or combination so

long as the number of test repetitions follows the 'T'

parameter and they are not separated by another

parameter.

Page 2

NMI  Version 1.03

by Dennis Lee

+--------------------------+

|   TESTING ####K MEMORY   |

| Bank    Bit     Address  |

|  00  P76543210 0000:0000 |

|        0000 Passes       |

+--------------------------+

When the memory test is invoked, either as the result of

a parity error, or from the command line, the above window

will be displayed in the upper left corner of the screen.

This window indicates the size of the memory being tested,

as sensed by NMI, the 64K bank currently being tested, and

the number of successful test repetitions.  In the interest

of speed, the 'Address' field is updated only for each 64K

bank in this window.  The 'Bit' field is used to indicate

failing bits and will not change unless an error is

detected.

If the 'Esc' key is pressed, the memory test will be

terminated with parity checking enabled, and control passed

to the interrupted process. Any other key will exit the

memory test and display the termination options.

Pass one of the memory test is always executed in a read

only mode.  This increases the possibility of detecting the

failing address.  If an error is detected in the first pass,

the address will will be written to in hopes of detecting

the failing bit.  The memory test takes approximately 35

seconds per 64K bank, longer depending upon other system

activity.

+--------------------------+

|   ** ERROR DETECTED **   |

| Bank    Bit     Address  |

|  00  P76543210 0000:0000 |

| Continue testing? _      |

+--------------------------+

In the event that an error is detected during the memory

test, the above window will be displayed.  This window

indicates the failing 64K bank, exact address, and bit(s).

Depending upon the display type and video mode active at the

time of the failure, the failing bit(s) will be indicated

either by blinking, or non-display.  These bit(s) directly

correspond to the failing chip(s) in the indicated 64K bank.

In the event that the failing bit cannot be isolated, all

bits in the failing address will be indicated.

Pressing 'Y' (or ENTER) will continue the memory test

starting with the next byte.  The number of successful

passes will be reset to zero. Pressing 'N' will exit the

memory test and display the termination options.

Page 3

NMI  Version 1.03

by Dennis Lee

+--------------------------+

| 1) Continue with parity  |

| 2) Continue w/o  parity  |

| 3) Reboot system         |

| Enter selection: _       |

+--------------------------+

The termination option window is displayed as a result of

any key other than 'Esc' being pressed during the memory

test, or electing not to continue testing following an error

detection.  Selecting option 1 will return to the

interrupted process with parity checking enabled. Selecting

option 2 will resume the interrupted process without parity

checking.  Option 3 will reboot the system.  Pressing ENTER

will restart the memory test.

RETURN CODES

------------

If NMI is invoked from within a batch file, or by another

process, a return code may be retrieved indicating any

errors.  For batch files, the return code may be

interrogated using the 'IF ERRORLEVEL' batch subcommand.

The following table indicates possible return codes for the

current release.

0 - Successful, no errors detected.

1 - Error detected in parameters, program aborted.

2 - NMI was invoked with the 'I' parameter, but is

already installed.  Installation aborted.

4 - Memory test already active, no memory test

performed.  This error can occur in multi-partition

systems if memory test is called while active in

another partition.

It is possible for more than one error to be returned in

the return code.  For example, a return code of 6 indicates

both errors 2 and 4 occurred.

Page 4

NMI  Version 1.03

by Dennis Lee

PROGRAM DESIGN CONSIDERATIONS

-----------------------------

Certain performance and function trade-offs are made in

the design of every program.  These trade-offs are usually

made in an attempt to provide the most function within the

constraints imposed by the operating environment in which

the program must run.  In this case, the following criteria

were taken into consideration.

1) All resident programs must use the smallest possible

amount of memory.

2) During the memory test, it is assumed that a memory

failure has occurred.  It is possible that the memory

failure occurred in the resident portion of DOS.  For

this reason, DOS function calls must not be used within

the resident NMI code.

No attempt is made to save and restore the contents of

graphics screens.  This would have required additional

code and buffer space within the resident portion of

the program.  If a disk file had been used to save the

video buffer, DOS function calls would have been

required, increasing the possibility of program check

during the memory test.

3) The memory test should first attempt to find the failure

using a read operation, so that the original failure

remains undisturbed.  If the read operation fails to

produce the error, only then should write operations be

used.  Furthermore, the failure should be located in a

single pass through memory, if possible.  This method

will provide the best odds of locating an intermittent

failure.

Page 5

####################################################################################################################