Documentation for Loci ver 1.1

Summary:

Loci  is a public domain program by Greg Jackson for  doing  Root

Locus  design.   Loci runs on IBM and compatible.  It  does  not

require graphics capability but will make use of it if available.

All  major graphics systems are supported including Hercules  and

AT&T monochrome graphics.

Commentary:

Root locus design is fundamental to control theory and is used by

Chemical,  Electrical,  and Mechanical engineers.   Basically  it

solves for the roots of the characteristic equation in a transfer

function  and  plots them in the complex plane.  If  this  sounds

like absolute nonsense then you might as well quit right here.

Features:

* Transfer function descriptions can be stored to disk,

read from disk, and edited.

* Menu driven, very simple for first time use.

* Solutions in the form of calculated poles, zeros, and

loci can be stored in ASCII format for use with other

programs.

* Hardcopy of root locus plots is available for Epson or

IBM dot matrix printers.

* Plotter interface through TPLOT (another public

domain program).

* It's free!

Limitations:

* Maximum order of the polynomial is 20.

* Maximum number of loci is 100 x order (70 for 8087).

Legal Stuff:

This  program cannot be sold for profit other than  a  reasonable

reproduction  cost.  The author is not responsible for damage  to

property, profit, or reputation which may result from use of this

program.  If your high performance controlled system  experiences

`crash and burn' don't come crying to me.

Technical Stuff:

Root  Locus  analysis was developed by W.R. Evens in  1950.   The

program  uses  Laguerre's method and linear  deflation  for  root

calculations.   The root calculation code is taken directly  from

Borland's Numerical Method toolbox.

The  program  is written in Pascal using Borland's  Turbo  Pascal

v4.0  and  makes  use of Borland units  CRT,  PRINT,  and  GRAPH.

Source code is available to anyone less the numerical code  which

is copyrighted to Borland.  For source code contact....

mail: Greg Jackson            modem: CIME-ISE BBS

1616 Capital Ave.              (608) 233-3378

Madison, WI 53705              8/N/1  24 hrs

Comments, questions, and suggestions are welcome.  This is public

domain, not  shareware. Money  is not requested.  However, if you

want to ship me a 12-pak, I  like  Special Export.  Please report

any  problems  with  the  program.   Program  output  of transfer

function description and loci should  be sent along with  problem

description.

History

Ver 1.1  (2/13/88)

- Speed increase of 400% by better implementation of 8087.

- Unit circle added and transfer function error checking is

relaxed for use with Z-transforms & discrete systems.

- A bug in the printout was fixed.

General operation:

The  menu driven program is generally self apparent.  This  brief

documentation  should  be read in full because it  describes  the

details of the program which are not self apparent.

Main Menu:

Rather  than  selecting the "Quit" command you can  quit  at  any

place by pressing the ESC key.

The  default  command is automatically  incremented  through  the

logical  progression  of "Enter Function", "Calc  &  Plot",  then

"Plot only".

Entering a transfer function:

The values displayed in dim video are default values.  To  accept

defaults simply press <ENTER> at the prompt.  If you already have

a transfer function entered or loaded from disk you can review or

edit it by accepting or changing the defaults.

Hardcopy:

When the plot is on the screen you can initiate a graphic printout

by  pressing 'p'. Pressing any other key will return you  to  the

main menu.  The printout is pretty slow.  If you decide you  want

to cancel in the middle just press ESC.

File Menu:

File  operations  are limited.  There is  no  default  extension.

Directory listings are not available from the menu.  Remember the

file names you want before you start the program.

The  files  output are ASCII.  You can create  transfer  function

description  files  with  any text editor if  you  like  but  the

easiest  way is to enter the function in LOCI and then  save  it.

The numbers must be in the same sequence but the comments are not

necessary.    The  pole/loci/zero  files  are  the   results   of

calculations  and can be used for plotting with  other  programs.

Each pair of numbers is a point on the complex plane (ie: a+bs is

an  x,y   pair).   The  poles are  noted in the file.  In general

the  pairs that  come after a pole  correspond to the same  root.

If you use  this  file with a pen plotter  do NOT take this as  a

rule.   The  numerical  estimation technique  will sometimes find

alternate roots in  conjugate  pairs.   This  switching  back and

forth will make  a mess if  you try  to connect points  with  the

plotter, so plot points, not lines. The file will also reveal the

location  of far  zeros.   The scale of the plot is determined by

the loci.  If a zero is  outside of the scale it will not be seen

on the screen  but it  will appear  in the file as a point on the

complex plane.

Using the 8087:

This  is  a numerically intensive program.   The  program  cannot

recognize  the  presence  of a math  chip  and  act  accordingly.

Rather  you  should  use the right version.   LOCI.EXE  does  all

floating point calculations in software and LOCI87.EXE makes  use

of  a  math chip. Using a math chip will make  things  very  much

faster.   The  difference is particularly  noticeable  on  higher

order  equations.  Both  versions  should  be  included  in   the

distribution package.

There are six adjustable defaults:

Gain exponent:  The loci calculated tend to pile up near poles or

zeros.   To  get them spread out somewhat the gains used  in  the

closed  loop  calculations  are stepped  exponentially.   A  gain

exponent  of 1.0 indicates a linear gain distribution.   Using  a

gain  exponent less than 1.0 will make the gain  increments  much

smaller  at the higher gain values.  This will tend to  make  the

loci pile up toward zeros.  Using a gain exponent larger than 1.0

will  make  the gain increments much smaller at  the  lower  gain

values.  The result is a distribution of loci shifted toward  the

poles.  A gain exponent of 2.0 provides a parabolic  distribution

of gain.  To get a feeling for this vary the exponent between 0.1

and 5.0 for the same transfer function.

Axis  line type:  This determines the line type for the real  and

imaginary axis.  A solid line generally looks best.

Number  of  Loci:  The total number of  characteristic  equations

calculated.  With n roots to each polynomial the total number  of

points is  n x NumLoci.  Larger numbers give nicer plots but take

longer for calculations.

Print  Mode:   There  are  4 different  print  modes.   The  main

difference  in  each  one is the number of dots  per  inch  on  a

horizontal  pass.   Each  printer pin is a  representation  of  a

screen pixel.  Since a number of screen types are supported, each

with  a  different aspect ratio, the print mode that  works  best

will vary with both the video type and hardcopy orientation.

Orientation:   This  refers to the orientation  of  the  hardcopy

printout.   Horizontal is as it appears on the screen.   Vertical

is turned +90 degrees.

Loci Size:  The size of the point plotted in pixel units.

Unit  Circle:   The  unit  circle  is  put  on  the  Z-plane  for

determination  of stability of discrete systems.  This  will  not

show up on the Tplot plotter output.

Problems with LOCI:

The  calculation part of LOCI has been very dependable.   If  you

appear  to  get garbage on the graphic display don't  assume  you

have faulty numerical values.  LOCI graphic output was tested  on

a  number of typical systems.  Very unstable systems,  very  fast

systems, and very slow systems may not display well.  If you  get

problems  like that use the file output function to get the  data

onto  disk.  You can then verify the data in any of a  number  of

ways.

**************** The following details plotter interface *******************

If you don't use a plotter just quit here.

Plotter Interface:

While  LOCI will not run a plotter directly, it will  create  data

files  and  command  files that are directly  usable  by  another

public  domain  program,  TPLOT  by  Dr.  Dudley  Benton.   TPLOT

supports  a number of graphics devices including Hewlett  Packard

graphics terminals, Tektronix terminals, IBM color graphics,  and

the  7470, 7475, 7550, & 9872 HP pen plotters.  I use it with  an

Epson plotter that emulates the HP graphic language (HPGL) and it

works fine.

LOCI  does this by creating data files and a TPLOT command  file.

The TPLOT command file is necessary since TPLOT will only receive

instructions  from an instruction file specified on  the  command

line  (ie:  the  DOS prompt A>).  The  TPLOT  command  file  will

include scaling commands for the plot which are calculated on the

basis of the calculated data.  It also contains the names of  the

data  files.  It is critical that you provide the  proper  format

for  command  file by making available a  working  TPLOT  command

file.  LOCI will read your working command file, find the  places

to  make  changes,  and  then write out a  new  command  file  of

whatever  name  you  specify.  The reason for  the  original,  or

template,   is  that there needs to be much more in  the  command

file than scaling and name commands.  The command file also needs

to have titles, pen colors, type of plotter, baud rate, and  lots

of other things specified.  The command file template must be  in

the  default  directory and must have the name  ROOTLOCI.PLT.

A  typical  template should be included among the  files  in  the

distribution package of LOCI.  The distribution package DOES  NOT

include   TPLOT.   TPLOT  is  almost  300K  and  is   distributed

separately.   You  should be able to find it on  bulletin  boards

which have an engineering/scientific slant.

The  command template included is for an HP 7475 plotter  set  to

run at 9600 baud through COM1.  It uses pen 1 for the axis, pen 2

for  poles,  pen  3 for loci, and pen 4 for zeros.   The  strings

@XAXIS,  @YAXIS, @POLE, @ZERO, and @LOCI which are in the  sample

command file are not seen by TPLOT which only looks at the  first

few  characters  of  each line.  The @ strings  must  be  in  the

ROOTLOCI.PLT  file  for successful creation of  the  new  command

file.   These  key words must be upper case.   The  LOCI  program

looks for those strings to know what lines to replace in the  new

command  file.  When you select TPLOT output in the LOCI  program

the following things happen....

1)  LOCI  will  look  for the file ROOTLOCI.PLT  in  the  default

directory.

2)  LOCI  will  ask  you for a filename for  your  output.   Lets

assume you call it `servo'.

3)  LOCI  will copy ROOTLOCI.PLT to a new file SERVO.PLT.  As  it

does  this it modifies certain lines marked in  the  original

file with the @ signifier.

4)  LOCI will create SERVO.POL, SERVO.ZRO, and SERVO.LOC.   These

are the datafiles that are used in plotting.

What Happens next ?

If  the original ROOTLOCI.PLT template was  correctly  configured

for  your  system (baud rate, plotter type, etc.), then  the  new

file will work with the newly created datafiles.  After you  exit

the LOCI program a simple call on the command line...

A>TPLOT SERVO.PLT  <cr>

will call up TPLOT and your plotter should start chugging away.

In Summary :

When calling the TPLOT output from the FILE function in the LOCI

program, LOCI will...

read in   : ROOTLOCI.PLT

write out : *.PLT

*.POL

*.LOC

*.ZRO

Then the  TPLOT  program can read the  instructions  in *.PLT to

plot the data found  in  *.POL,  *.LOC, and *.ZRO where the * is

specified by the user.

Problems in plotting

Most of the problems involve configuration of the original  TPLOT

command  file (ROOTLOCI.PLT).  TPLOT can be a bit tricky to  use.

The  best way to approach this is to modify the  sample  included

rather  than  creating a new command template.   Don't  let  this

discourage  you, once you get it working each run  usually  works

perfectly.

TPLOT will only work with integer scale factors.  This means that

if  your root locus plot goes from -1.2e-2 to 0 on the real  axis

you  will get a plot with a scale from -1 to 0.  This limits  the

output when doing analysis of very slow systems.  LOCI will  take

care to provide only integer values in the command file but  this

does  not do much good  since  the minimum  integer scale of  0-1

fails to display very small values correctly.

You  will often get error messages from zeros out of range.   The

scaling  is done on the basis of the loci and poles.   Often  you

will get zeros that are a long way off the scale.

If  you have problems getting the TPLOT interface going don't  be

shy to contact me or to contact Dudley Benton.  I can be  reached

through  the BBS  listed near  the  top of this file.  Dr. Benton

often calls the BBS as well.

#####################################################################

#################################################################

########################################################################################################################################################