��������������������������������������������������������������������

X10DC X10 Direct Control

��������������������������������������������������������������������

Version 1.4

August 1993

Copyright 1993 � D.Huras � All Rights Reserved

�����������������������������������������������X10DC.DOC v1.41������

CONTENTS

��������������������������������������������������������������������

1. DESCRIPTION

2. WARRANTY

3. FEATURE OVERVIEW

4. REQUIREMENTS

5. INSTALLATION INSTRUCTIONS

6. COMMAND LINE ARGUMENTS

7. COMMUNICATION PORTS

8. OPERATING MODES

9. DEBUG MODE

10. PROGRAM OVERVIEW

11. KEYBOARD USAGE

12. MENU BAR

13. CP290 SELF TEST

14. SIGNAL TEST GENERATOR

15. CLOCK SYNCHRONIZATION

16. BASE HOUSECODE

17. PROBLEM RESOLUTION

18. REGISTRATION INFORMATION

19. OTHER X-10 BASED SOFTWARE

20. RELEASE HISTORY

1. DESCRIPTION

��������������������������������������������������������������������

X10DC is a program which sends immediate commands to the X10 CP290

(*1) Computer Interface.  The CP290 transmits these commands over the

power line to control X10 devices.  X10DC does not currently modify

the EVENT DATA stored in the CP290 Interface (*2).  X10DC is a DOS

program that uses a simple point & click user interface.  (A Mouse

though not required is highly recommended).  X10DC is designed to be

robust software that is processor speed independant.  This program is

shareware and you are invited to evaluate it for 30 days. If you

continue to use the program after the 30 day evaluation period you are

asked to register it.  The author retains all rights to the program.

(*1) The CP290 Computer Interface is a device manufactured by X-10

that can be connected to a micro-computer serial port.  The CP290

also plugs into an electrical outlet and it is through this

connection that the CP290 imposes X-10 commands onto the powerline.

X-10 commands use the power line as a carrier; other X-10 devices

connected to the power line will respond to these commands (ie.

switch ON/OFF lights, appliances).

2. WARRANTY

��������������������������������������������������������������������

X10DC is supplied as is.  The author disclaims all warranties,

expressed or implied, including, without limitation, the warranties

of merchantability and of fitness for any purpose.  The author

assumes no liability for damages, direct or consequential, which may

result from the use of X10DC.

3. FEATURE OVERVIEW

��������������������������������������������������������������������

� User-Friendly point & click DOS character-based Interface

� All program functions can be controlled using a mouse; no keyboard

intervention is required.

� Supports Standard CommPorts 1 thru 4 and a User Defined CommPort

which can support any IRQ between 1 & 15 (inclusive) and any

Base I/O Address.

� The Program supports 3 operating modes; each mode uses a different

combination of on-screen controls.  Mode 1 is suggested for users

not using a mouse.

� When running in Mode 3, the program reads in an X10.DAT file

[this file format is used by the X10 Program supplied with

the CP290 Interface] and presents a list box of device

descriptions/locations which can be targetted for commands.

(The Unregistered version will only send commands to the 1st

8 Devices found; SEE REGISTRATION BENEFITS)

� Commands can be sent to all 256 X10 device addresses.

� Commands can be targetted for any number of units within an X10

House Code (effectively supporting for ALL ON and ALL OFF).

� Supports 16 discrete dimming levels.

� Acknowledgements sent from the CP290 (after receiving commands from

the program) are monitored and checksums are verifed.

� A scrollable Command History is available for browsing while the

program is running.

� A software-based Signal Test Generator is available to assist in

identifying and isolating X10 transmission/reception problems.

� A Debug Mode is available which reports all data sent from the

CP290 as well as timing information.

� A CP290 Self Test with event preservation is supported.

(*2) NOTE, the self test overwrites the EVENT DATA.  X10DC

will preserve the EVENT DATA by downloading it from the

CP290, saving it in memory, running the Self Test, and

finally uploading the saved EVENT DATA back to the CP290.

� Clock Synchronization function for the CP290 and the PC

� Change Base HouseCode stored in the CP290 function with

or without Event Preservation.

� A comprehensive on-line Help sub-system that can be disabled

at the user's option to decrease program load time.

� The program WILL load without a CP290 Interface attached, however

all attempts at serial communications will result in errors; the

program will attempt to trap all such errors.

4. REQUIREMENTS

��������������������������������������������������������������������

� An IBM-Compatible PC with at least 1 serial port.

� An X10 CP290 Computer Interface attached to a serial port.

� A Microsoft-Compatible Mouse is highly recommended

� A Colour adapter/display is recommended although the program

should run with a monochrome display/adapter.  If you

experience problems with the default colors, then try the /BW

command line argument which will set everything to Black &

White.

The program requires approximately 415K available conventional memory

to run.  The Command History will consume up to an additional 32K

of memory however this translates to approximately 1000 commands; its

unlikely that this many commands will be sent during one invokation of

the program.  However, if Debug Mode is on, the Command History will

quickly fill up since all CP290 responses are reported in the

Command History list box.

6. INSTALLATION INSTRUCTIONS

��������������������������������������������������������������������

There are no special installation instructions; simply copy the

program to a sub-directory of your choice and run it.  The program

will look for 3 files in the directory it is run from.  However,

the program will run with or without the following 3 files:

1) X10.DAT

If you want to use Mode 3 then you may want to copy an

X10.DAT file to the sub-directory where you have installed

X10DC.EXE.  Alternatively, you can load an X10.DAT file from

any sub-directory by using the 'Load X10 File' menu option.

An X10.DAT file is a file format used by the X10 program

supplied by X10 with the CP290 interface.  The unit

addresses, locations and descriptions are read from this

file and presented in a list box by X10DC.  X10DC DOES NOT

MODIFY THE X10.DAT FILE.  A sample X10.DAT is provided

with X10DC.

2) X10DC.HLP

X10DC retrieves the on-line help text from this file. If the

file is not found then X10DC will display a message during

startup and then continue.  The help feature will be

unavailable in this case.  If you decide that you no longer

need the help sub-system, then you should run the program

with the /NH command line argument; this will suppress the

'Help not found' message and decrease program load time.

3) X10DC.INI

The registered version of the program will look for a

X10DC.INI file in the current directory.  The X10DC.INI file

is a text file containing all program parameters; there is no

need to create this file yourself since the program will

generate one whenever you select the 'Save Options' menu item.

A sample X10DC.INI file (generated by X10DC.EXE) is shown

below:

;

;X10DC.INI File Created: 08-06-1993 01:40:00

;

;All Lines prefixed with ; are ignored

; Program Options

; ---------------

; CommPort=1,2,3 OR 4            DEFAULT=1

; Mode=1,2 OR 3                  DEFAULT=1

; Debug=ON/OFF                   DEFAULT=OFF

; X10File=[d:]\FILENAME.EXT      DEFAULT=X10.DAT

; LoadHelp=YES/NO                DEFAULT=YES

; Sounds=ON/OFF                  DEFAULT=ON

;

Mode=1

X10File=X10.DAT

;

; User Defined CommPort

; ---------------------

; IRQ and Base_IO_Address apply to User Defined CommPort ONLY

;

; IRQ=1-15                 DEFAULT=4; IF Base_IO_Address is set

;                                     --

; Base_IO_Address=nnnn(n)  DEFAULT=1016; IF IRQ is set

;                                        --

IRQ=5

Base_IO_Address=1016

;

; Colour Values

; -------------

; 0=Black  4=Red       8=Gray          12=Bright Red

; 1=Blue   5=Magenta   9=Bright Blue   13=Pink

; 2=Green  6=Brown    10=Bright Green  14=Yellow

; 3=Cyan   7=White    11=Bright Cyan   15=Bright White

;

BackColor=7

ForeColor=0

HiLiteKey=14

DisabledItem=8

MenuBackColor=1

MenuForeColor=7

MenuSelectedBackColor=0

MenuSelectedForeColor=7

LabelBackColor=9

LabelForeColor=7

DebugForeColor=4

HelpTitleBackColor=5

HelpTitleForeColor=15

StatusTitleBackColor=4

StatusTitleForeColor=15

WarningTitleBackColor=12

WarningTitleForeColor=15

;

; Other Screen Settings

; ----------------------

; ThreeD=ON/OFF*                 DEFAULT=ON

; Shadow=ON/OFF*                 DEFAULT=ON

;  * OFF Recommended for Monochrome Displays

;

ThreeD=ON

Shadow=ON

;

; End of X10DC.INI (13:59:37)

This X10DC.FILE was created using the 'Save Options' menu item.

6. COMMAND LINE ARGUMENTS

��������������������������������������������������������������������

DEFAULTS:  If the program is run without any command line arguments

it will default to COMMPORT 1,OPERATING MODE 1,DEBUG OFF,

F:X10.DAT.

The following command line arguments can be entered in any order:

/Annnn(n) � Base I/O Address applicable to User Defined CommPort

(Must be specified as a decimal number)

If /I agrument is present, but /A is not; then Base

I/O Address will default to 1016 (3F8 Hex).

/BW  � Use Monochrome colours (Black & White)

/Cn  � COMMPORT where n = 1,2,3 & 4

ALWAYS overrides User Defined CommPort during Program Startup

/D   � TURNS ON DEBUG MODE

When DEBUG MODE is ON, the CP290 responses & checksums are

displayed in the Command History List Box.

/F:[d]\filename � X10.DAT FILE SPECIFICATION for MODE 3

where [d] = drivespec eg. /F:C:\X10\X10.DAT

/In  � IRQ applicable to User Defined CommPort where n = 1 - 15

If /A argument is present, but /I is not; then IRQ

will default to 4.

/Mn  � MODE where n = 1,2 or 3

MODE 1 Provides a List Box of Unit Codes; only 1 Unit Code

can be selected.  However, this mode will be easier to

use if you don't have a mouse since you'd have to TAB

through all the unit code check boxes used in MODE 2.

MODE 2 Provides a Check Box for each Unit Code plus ALL and

NONE buttons to quickly Check/Uncheck all check boxes.

In this mode, multiple Unit Codes can be targetted for

the X10 Command.

MODE 3 Provides a List Box of Devices which are read from

an X10.DAT file; X10DC will look for "X10.DAT" in the

current directory UNLESS the /F: command line argument

is supplied.  If X10.DAT cannot be found, the program

will switch to Mode 1.  Select 'Load X10 File' under

the File menu to load an X10.DAT file from another

sub-directory. (Once the file is loaded, its location

can be saved [REGISTERED VERSION ONLY] by selecting

'Save Options' under the Options menu.

/NH  � Do Not Load HELP Sub-System

/NH- � Load HELP Sub-System (Override INI directive)

/NS  � No Sounds during Signal Test

/NS- � Sounds during Signal Test (Override INI directive)

NOTE: ONLY THE 1st 8 DEVICES CAN BE TARGETTED FOR

COMMANDS WITH THE UNREGISTERED VERSION.

1) Most Command Line directives (with the exception

exception of /A,/I & /NH) can be modified once the

program loads by selecting the appropriate menu items

under the Options Menu.  Registered Users can save all

these settings to the X10DC.INI file by using the 'Save

Options' menu item.

2) COMMAND LINE OPTIONS OVERRIDE ALL X10DC.INI SETTINGS

7. COMMUNICATION PORTS

��������������������������������������������������������������������

X10DC supports "pseudo standard" CommPorts 1 thru 4 using the

following settings:

������������������������������������Ŀ

�          �      � Base I/O Address �

� CommPort � IRQ  ������������������Ĵ

�          �      � Decimal �  HEX.  �

������������������������������������Ĵ

�    1     �   4  �  1016   �  3F8   �

������������������������������������Ĵ

�    2     �   3  �   760   �  2F8   �

������������������������������������Ĵ

�    3     �   4  �  1000   �  3E8   �

������������������������������������Ĵ

�    4     �   3  �   744   �  2E8   �

��������������������������������������

If one of these CommPort settings matches your hardware configuration

simply use the /C command line argument with the appropriate

CommPort.  The CommPort= X10DC.INI directive can also be used.  Note

that if X10DC is run without the  /I (IRQ) or /A (Base I/O Address)

Command Line arguments then the 'User Defined CommPort' is

"undefined" and not selectable from the menu bar.

If your hardware configuration differs from the table above, then

you can define your own CommPort by using the /I and /A command

line arguments (or IRQ= & Base_IO_Address= X10DC.INI file directives).

IRQ can be set to any value between 1 and 15 (inclusive).  Base I/O

Address can be set to any value greater than zero.  CAUTION: your

hardware may "lock up" if incorrect settings are used.  You should

supply both arguments (/A & /I), however, if you only supply one of

the arguments then the defaults applicable to CommPort 1 will apply

to the other argument.  (eg. if only the /I5 argument is supplied,

then the /A argument defaults to 1016).  If either the /I or /A

argument is supplied, then the 'User Defined Commport' under the

'Options' menu becomes enabled.

NOTES: 1) The /C command line argument AND the CommPort= X10DC.INI

directive ALWAYS override the the User Defined CommPort

when the program first loads.  However, the 'User Defined

CommPort' will remain enabled under the 'Options' menu.

To start the program with the User Defined CommPort active,

ensure that there is no /C command line argument AND no

CommPort= X10DC.INI directive.

THE BASE I/O ADDRESS MUST BE SPECIFIED IN DECIMAL NOTATION.

IBM PS/2 CommPort Parameters are shown below:

������������������������������������Ŀ

�   PS/2   �      � Base I/O Address �

� CommPort � IRQ  ������������������Ĵ

�          �      � Decimal �  HEX.  �

������������������������������������Ĵ

�    1     �   4  �  1016   �  3F8   �

�    2     �   3  �   760   �  2F8   �

�    3     �   3  � 12832   � 3220   �

�    4     �   3  � 12840   � 3228   �

�    5     �   3  � 16928   � 4220   �

�    6     �   3  � 16936   � 4228   �

�    7     �   3  � 21024   � 5220   �

�    8     �   3  � 21032   � 5228   �

��������������������������������������

A User Defined CommPort must be used for

PS/2 CommPorts 3-8.

8. OPERATING MODES

��������������������������������������������������������������������

Operating Mode can be set using the 'Options' menu, the MODE command

button (cycles through the modes), or the /M command line argument.

��������������������������������������������������������������������

MODE 1                    Unit

Code            NOTE: Mode can be toggled

In this mode only        �����Ŀ                at runtime by clicking

one Unit Code can be     �   1 #                the MODE button.

selected from the        �   2 �                (Keyboard users press

Unit Code List Box.      �   3 �                 ALT-m).

�   4 �

�   5 �                Can also be set from

�   6 #                the Options Menu.

�������

RECOMMENDED IF YOU DON'T HAVE A MOUSE.

��������������������������������������������������������������������

MODE 2                             Unit Code(s)

����������������������������Ŀ

In this mode multiple    �[X] 1  [ ] 5  [ ] 9   [ ] 13�

Unit Codes can be        �[ ] 2  [ ] 6  [ ] 10  [ ] 14�

checked; in addition     �[ ] 3  [ ] 7  [ ] 11  [ ] 15�

the ALL button can be    �[ ] 4  [ ] 8  [ ] 12  [ ] 16�

clicked to check all     ������������Ŀ  �����������Ŀ�

the Unit Codes. The      ��    ALL    �  �    NONE   ��

NONE button unchecks     ��������������  ������������ٳ

all the unit codes.      ������������������������������

��������������������������������������������������������������������

MODE 3          ** Example 1 **

Unit Location               Description

An X10.DAT      ����������������������������������������������������Ŀ

is read and     � A  1�DINING ROOM           �TABLE LAMP             #

a List Box      � A  2�FAMILY ROOM           �TV                     �

replaces the    � A  3�GARAGE                �CEILING LIGHT          �

House/Unit      � A  4�GUEST ROOM            �DE-HUMIDIFIER          �

Controls used   � G  7�HALL                  �CEILING LIGHT          #

in Modes 1 & 2. ������������������������������������������������������

Also note that   {11 Device(s) Found}       Brightness[             ]#

due to a                                    [1 = OFF]

shortage of

screen real

estate, a Combo

Box replaces                                          ��������������Ŀ

the Brightness  ** Example 2 **                       �  1 �  0.0 %  #

List Box. When    Unit Location               Descript�  2 �  6.7 %  �

the down-arrow  �������������������������������������ĳ  3 � 13.3 %  �

is clicked (or  � A  1�DINING ROOM           �TABLE LA�  4 � 20.0 %  �

F4 is pressed)  � A  2�FAMILY ROOM           �TV      �  5 � 26.7 %  �

a List Box will � A  3�GARAGE                �CEILING �  6 � 33.3 %  �

pop up as shown � A  4�GUEST ROOM            �DE-HUMID�  7 � 40.0 %  �

in Example 2.   � G  7�HALL                  �CEILING �  8 � 46.7 %  #

������������������������������������������������������

{11 Device(s) Found}       Brightness[ 8 � 46.7 %  ]#

[1 = OFF]

��������������������������������������������������������������������

9. DEBUG MODE

��������������������������������������������������������������������Ŀ

� Using COMMPORT: 2        X10 DIRECT CONTROL     CP290 Day: Tuesday �

�Base House Code: P           Version 1.2       CP290 Time: 11:51    �

�Mode: 1   Debug ON        (c)1993 � D.Huras       PC Time: 11:52    �

����������������������������������������������������������������������

����Ŀ

��������

When enabled, the DEBUG ON message appears in the screen Header and all

responses from the CP290 are displayed in the Command History List Box,

as well as Checksums and Timing Information.

Debug Mode can be toggled ON/OFF from the Options Menu.

NOTE: Debug Mode increases program overhead; memory requirements will

be greater and program performance will be somewhat degraded.  It is

recommended that you only toggle Debug Mode ON when necessary.

10. USING THE PROGRAM

��������������������������������������������������������������������

When the program first loads, it requests the base house code and

time stored in the CP290 via serial I/O using the selected COMMPORT.

This information will be reported in the Screen Header if found.

Note:  the CP290 Time and PC Time reported in the Screen are static

and are only updated when the Base House Code is changed, a

Self-Test is run or the Time is set.  You can use the 'Refresh' menu

item under the 'Clock' menu to force an update to the on-screen Time

displays.  The House Code List Box defaults to the Base House Code

if the CP290 is found.  Once the program is loaded the SEND command

button is disabled until you have selected a Unit Code(s). If the

DIM command is selected a Brightness Level must be be selected

before the SEND button becomes enabled.  The Command History list

box is scrollable and will store up to 1000 commands after which it

becomes a circular FIFO buffer.

11. KEYBOARD USAGE

��������������������������������������������������������������������

Use the TAB/Shift-TAB keys to move among the various controls.(*3)

The Up/Down arrows can be used to move up and down List Boxes, and

the Spacebar and Enter key can be used to select the current List

Box item, check/uncheck a check box, or invoke a command button.

The ESCAPE key will exit the current function or the program if no

other windows are visible.  All of the Command Buttons and controls

have quick access keys:

House Code � Alt-o [Mode 1 & 2]

Unit Code(s) � Alt-u [Mode 1 & 2]

Device List � Alt-u [Mode 3]

ALL � Alt-l [Mode 2]

NONE � Alt-n [Mode 2]

Action � Alt-a

Brightness � Alt-b [when enabled]

MODE � Alt-m (*3)

SEND � Alt-s

EXIT � Alt-x

HELP � Alt-h

Command History � Alt-c

(*3) The MODE Command Button is NOT in the TAB STOP sequence;

Keyboard users must use ALT+character to access this

button. (This eliminates extraneous TAB keystrokes)

12. MENU BAR

��������������������������������������������������������������������

Press & release the ALT key to activate the Menu Bar.  Menu items

can be selected by pressing the highlighted key (when applicable),

or by positioning the highlight bar using the up/down arrow keys and

pressing ENTER.  Some screen button functions are replicated within

the menus (eg. Exit, Help).  However, there are some functions which

are only available from within menus.  CommPort, Operating Mode,

Debug, & Sounds can all be set from the Options Menu; the current

state of each of these parameters is indicated on the menu itself

with a '�' character to the left of the menu item.

The following short-cut keys can also be used at any time to

select menu items:

*FILE* Menu

Load X10 File � Ctrl-L

Exit      � Ctrl-X (Same as Exit Button, Alt-X)

*CLOCK* Menu

Set PC Clock � Ctrl-T (Enter Time)

Set CP290 Clock � Ctrl-K (Synchronize w/PC) - Registered Version

Refresh Clocks � Ctrl-R

Set Base HouseCode � Ctrl-B (Preserve Events) - Registered Version

*TEST* Menu

CP290 Self Test � Ctrl-E

Signal Test Generator � Ctrl-G

*OPTIONS* Menu

CommPort 1 � Ctrl-F1

CommPort 2 � Ctrl-F2

CommPort 3 � Ctrl-F2

CommPort 4 � Ctrl-F2

User Defined CommPort � Ctrl-F5 (when applicable)

Mode 1 � Shift-F1

Mode 2 � Shift-F2

Mode 3 � Shift-F3

Sounds Toggle � Ctrl-N

Debug Mode Toggle � Ctrl-D

Screen Colours � Ctrl-C

Save Options � Ctrl-S  [Not Available in Unregistered Version]

*HELP* Menu

Help � F1 (Same as Help Button, Alt-H)

About � Ctrl-A

13. CP290 SELF TEST

��������������������������������������������������������������������

The CP290 can perform a Self-Test to determine if there are any

detectable problems within the device.  The Self-Test will return

either a pass or fail status when it is run.  X10DC will report the

results of the test in the CP290 Status window.  Whenever a

Self-Test is run the memory contents of the CP290 are overwritten.

X10DC can preserve the CP290 Event Data by downloading it from the

CP290 prior to the Self-Test and uploading it back to the CP290

after the Test.  (X10DC also allows a Self-Test to be invoked

without event preservation, but this leaves the device in a state

which may cause problems with other programs that communicate with

the CP290. It is recommended that the Self-Test always be invoked

with Event Preservation regardless of whether or not there is any

stored Event Data.)

14. SIGNAL TEST GENERATOR

��������������������������������������������������������������������

The Signal Test Generator is a powerful feature that enables the

user to track down and isolate X-10 transmission/reception problems

and faulty modules.  This feature will send X-10 ON/OFF commands

repeatedly until the test is ended by the user.  (Similar to the

Leviton 6385 device).  The address to be targetted for commands can

be a fixed House Code and Unit Code; alternatively, either the House

Code or Unit Code (or both) can be varied. This allows you to send

commands repeatedly to:

� A 'Fixed' House Code & 'Fixed' Unit Code (eg. A1)

� A 'Fixed' House Code & 'Variable' Unit Code (eg. A1 - A16)

� A 'Variable' House Code & 'Variable' Unit Code (ie. A1 - P16;

All 256 addresses would be tested in this case)

� A 'Variable' House Code & 'Fixed' Unit Code (eg. A1 - P1)

When the Signal Test Generator is selected, a window is

displayed that allows you to set the House Code & Unit Code to

either 'Fixed' or 'Variable' mode; if 'Fixed' is selected then

a List Box is enabled so that the fixed code can be selected.

The Delay between commands can be set from 0 to 60 seconds;

this delay applies to the pause between each pair of ON/OFF

commands.  Once all required settings have been made the OK

button becomes enabled.  When the OK button is clicked the

test will begin; the test can be terminated by pressing [Esc].

Note, if the Delay is set to 0, then the [Esc] key may have to

be hit several times to stop the Signal Test.  X10DC will emit

a brief beep before each ON command is sent, and another lower

pitched beep before each OFF command is sent.  These beeps can

be disabled by toggling the Sounds item on the 'Options'

Menu.

'Fixed' Mode Testing

��������������������

Generally speaking, you would want to use the 'Fixed' option

to test the range of X-10 signal reception by starting the

Signal Test using address P1 for example (the "only" address

that the Leviton 6385 will test) and then walking around to a

variety of outlet locations within your home and plugging in a

module set to P1 to see if the ON & OFF commands are being

received at all locations.  The program attempts to let you

know when the commands are sent by beeping although you may

not be able to hear the beep from all outlet locations.

'Variable' Mode Testing

�����������������������

The 'Variable'(*4) option can be used when you suspect that an

X-10 module has trouble receiving commands targetted for

specific House and/or Unit Codes.  In this case, instead of

walking around to locations, you would adjust the House and/or

Unit Code dials on the module as the Signal Test cycles

through each combination.  You may want to toggle the Sounds

off in this case.

����

(*4) Not available in UNREGISTERED Version

15. CLOCK SYNCHRONIZATION

��������������������������������������������������������������������

Clocks tend to run either fast or slow and therefore should be

synchronized periodically.  X10DC allows you to set the time in both

the PC and CP290 manually by specifying hh:mm and also allows you to

synchronize the PC and CP290 by using one of the clocks to set the

other.  When the PC clock is used to synchronize the CP290 clock,

X10DC will wait until the seconds are at zero since the resolution

of the CP290 clock is limited to minutes.  (This wait can be

cancelled by pressing the [Esc] key).  When the CP290 is used to

synchronize the PC, the hours and minutes from the CP290 are used

immediately to set the PC Clock; since the CP290 cannot provide

'seconds', it is recommended that a CP290->PC Clock Sync be followed

by a PC->CP290 Clock Sync.  This will ensure that both Clocks are

synchronized.

16. BASE HOUSECODE

��������������������������������������������������������������������

The Base HouseCode stored in the CP290 is only applicable to the

rocker keys on the top of the CP290.  These keys are labelled 1 to 8

and correspond to whatever House Code is stored as the Base

HouseCode in the CP290.  The CP290 re-initializes itself whenever

the Base HouseCode is changed; as a result, all of the CP290 Event

Data is erased.  X10DC gives you the option of either Preserving or

Erasing the Event Data whenever the Base HouseCode is changed.  If

the user elects to preserve the Event Data, it is downloaded from

the CP290 before the Base HouseCode is changed and then uploaded

back to the CP290 after the Base HouseCode has been changed.

17. PROBLEM RESOLUTION

��������������������������������������������������������������������

Generally, if your CP290 works with the X-10 supplied software then

you should have no problems running X10DC.  The program will load if

you run it without any arguments (thereby defaulting to COMMPORT 1)

and your CP290 is actually attached to COMMPORT 2.  If this is the

case X10DC will report an error when it attempts to retrieve the

Time and Base HouseCode.  You can toggle to the correct COMMPORT

once the program is running by using the Options menu.  In future

you should include the Command Line argument /C2 if your CP290 is

connected to COMMPORT 2.  The Save Options menu option will save all

settings to the X10DC.INI which obviates the need for command line

arguments.

If you run X10DC with a modem attached to the COMMPORT, you may get

unpredictable results since the modem will echo everything back to

X10DC.

If you are sure that the X10DC is using the correct COMMPORT and

problems persist, try using Debug mode and then scroll through the

Command History List Box; see if the calculated checksums match;

look at the TIMER and MaxTimer Values. If the MaxTimer Value exceeds

the TIMER value then the program is timing out while waiting for

X-10 responses. PLEASE REPORT ANY TIMING PROBLEMS TO ME; adjustments

can be made to the program to improve its compatibility if this

turns out to be a problem.  As a last resort, run the CP290 Self

Test, but only do so if you have backed up your EVENT DATA using a

program you are sure works within your operating environment.

(X10DC will preserve the CP290 Event Data but if you are

experiencing problems with X10DC then this function may fail).

In some cases X10DC may report that a command failed, or a checksum

did not match, but in fact the command was accepted by the CP290 and

transmitted on the powerline.  This could happen if spurious

responses are received from the CP290, ie.  if you were to press the

rocker buttons on the CP290 just after sending a command.  If this

happens, exit the program and re-invoke it to resynchronize,

alternatively, if you have 2 serial ports you could try toggling the

CommPort. (ie. if your CP290 is on CommPort 1 then switch to

CommPort 2, then switch back to CommPort 1).

The open architecture of the PC compatible operating environment and

the near infinite combinations of hardware and software complicate

the compatibility of applications software.  While every effort has

been made to ensure that X10DC is compatible with all

hardware/software combinations, there may be operating environments

in which X10DC will not run properly.

Problems can be reported to:

David Huras

37 Ruddell Crescent

Georgetown, Ont. L7G 5N4

CANADA

FAX: 416-873-6429

CIS: 72767,472

Internet: david.huras@canrem.com

18. REGISTRATION INFORMATION

��������������������������������������������������������������������

X10DC is currently supplied as fully functioning software without

built-in delays or "nag" screens prompting you to register.  The

ABOUT screen contains registration information in case this

documentation file gets misplaced.  A significant amount of time and

effort went into this program; please register the program if you

intend to use it beyond the 30 day evaluation period.  Registered

Users will be shipped a printed manual and a 3�" diskette (if you

would prefer a 5�" diskette please advise) containing the Registered

Version of the program which includes the following features:

� Full support for the Signal Test Generator which allows you to

cycle through all House & Unit Codes and send an ON/OFF command

to each address.

� Full support for operating mode 3 which reads in an X10.DAT file

and builds a "list box" of device names which can be targetted

for X10 commands.  (Unregistered Version will only send commands

to the first 8 Devices).

� Support for an INI file which provides complete program

customization capabilities including Screen Colours.

� BONUS PROGRAM: Registered Users will also receive a bonus

program; X10DAT reads an X10.DAT file and prepares a report

listing all installed devices and scheduled events including

"frozen" events and multiple unit events. See X10DAT.RPT for

a report sample.

Register via CompuServe Shareware Registration;

GO SWREG (Registration ID# 620)

-=OR=-

Charge the registration fee to your Visa, Master Card, American

Express by ordering X10DC (Item #10924) from Public (software)

Library, call 800-242-4PsL (713-524-6394 overseas) or FAX your

order to 713-524-6398.  These numbers are for ordering only.

ALL OTHER INQUIRIES (eg. technical support, volume discounts

site licensing) should be directed to the author: see PROBLEM

RESOLUTION section.

-=OR=-

Register by sending $20 (cheque/money order) to:

David Huras

37 Ruddell Crescent

Georgetown, Ont. L7G 5N4

CANADA

��������������������������������������������������������������ͻ

�REGISTERED USERS of prior versions of X10DC can upgrade to the�

�current version at ANY TIME by simply sending a diskette      �

�containing their registered version of the software to the    �

�address above.                                                �

��������������������������������������������������������������ͼ

19. OTHER X10-BASED SOFTWARE by David Huras

��������������������������������������������������������������������

Program   Description                                     Status

-------   ------------------------------------------   --------------

X10DC     X10 Direct Control - CP290 Point & Click     Version 1.41

Initial Release: February '93                August '93

X10EC     X10 Event Control - CP290 Point & Click      Version 0.95

Interface Program to manage Event Data.      Beta Testing

Includes Sunrise/Sunset Time Offsets.

Expected Release Date: Sept '93

X10UC     X10 Ultimate Control                         Development

A program which combines the features of

X10DC & X10EC and provides multiple modeless,

screen windows.

��������������������������������������������������������������������

Please register this software if you use it regularly; by registering

you reinforce my commitment to ongoing development.

��������������������������������������������������������������������

20. RELEASE HISTORY � X10DC Shareware/Registered Version

��������������������������������������������������������������������

Date      Version     Changes

------   ----------   ----------------------------------------------

AUG'93    1.41        �Public (software) Library registration method

added to documentation.

�X10DAT Bonus Program added as registration

benefit.

�X10DC.INI File Comment re: X10File argument

corrected in documentation.

AUG'93    1.40        �Corrected 'Load X10 File' problem when X10.DAT

located in Root Directory.

�Corrected problem whereby X10DC.INI was not

processed when X10DC was started from a Root

Directory (impacted Registered Version only).

AUG'93    1.40 �      �COMMPORT 3 & 4 Support Added.

�User Defined CommPort support Added.

�Minor changes to default colours (LABELS).

JUL'93    1.32        �Corrected problem with DEBUG MODE colour setting

not 'sticking'; setting from INI is workaround.

(Not released).

JUL'93    1.31        �Fixed mouse cursor reset problem if Signal Test

is run without a CP290 device. (Not released).

JUL'93    1.30        �ENABLED SEND Command under MODE 3 in Unregistered

version for 1st 8 Devices in X10.DAT File

�ENABLED ALL Clock Sychronization options in

Unregistered version

�ENABLED Change Base HouseCode with Event

Preservation in Unregistered version

�ENABLED Screen Colours dialogue in Unregistered

version

�Test menu item added to Menu Bar;

� CP290 Self Test disassociated with Debug Mode

and Self Test button removed from Screen

Header; Self-Test feature now a sub-menu

item under Test.

� Signal Test Generator feature added; send

ON/OFF commands repeatedly to a fixed/variable

House Code/Unit Code address.  Unregistered

version supports the fixed option only.

�Help Sub-system added; contains additional

X10 background information as well as extracts

from the documentation file.

�/NH command line argument added to optionally

not load the help system for faster program

load times.

�/NS command line argument added to disable

sounds generated during the Signal Test.

�Sounds Toggle added as a sub-menu item under

Options.

�INI Overwrite warning now uses the WARNING

screen title colour(*5)

�'Save Options' displays a message box whenever

INI options are saved(*5)

�The access key for the 'Exit' option under the

'File' menu changed from 'E' to 'x'.

�[Esc] key will now back out any screen

colour changes while in the Screen Colours

dialogue

�[Esc] key will now skip the wait while

SYNC'ing the CP290 clock based on the PC

Clock

MAR'93    1.20        �Merged Shareware & Registered Versions;

Registered Version features are disabled in

the Unregistered version, specifically:

� Complete Mode 3 Support (X10.DAT File)

� Complete Screen Color Customization

Dialogue

� INI File Support to save/modify

Program settings/colors

� Preserve/Restore Event Data when

Base House Code changed

� PC � CP290 Clock Synchronization

� CP290 � PC Clock Synchronization

�ENABLED Self Test EVENT Preservation

in Unregistered version

�ENABLED MODE 3 Switch & X10.DAT File Load

in Unregistered version (SEND-Disabled)

�ENABLED some Clock Functions, & Base House

Code Setting in Unregistered Version

�CP290 Self Test changed to reset interface

upon completion (The self test overwrites

the Event Data and this data may cause

problems with some programs)

�Added /BW command line argument to startup

with monochrome color settings

�Added 'Refresh' menu item to refresh on-screen

Clock Displays

�Error handling improved

�Retry logic added for all CP290 communications

with automatic wait time adjustment (if program

times out waiting for a response, it will

increase the wait time if the command is retryed)

�Command History caption now indicates which

entry is highlighted (ie. entry x of n)

MAR'93    1.10        �MENU BAR Added - CommPort,Mode, & Debug can

now be changed using the Options Menu

�Mode Indicator added to Screen Header

�Minor Screen Colour modifications

FEB'93    1.00        �Mode Switch Command Button Added (Alt-M)

�Debug Command Button TAB Stop disabled-keyboard

users must use ALT-T

�CIS Software Registration # added to ABOUT

FEB'93    1.00 �.02   �Operating Mode 2 Added (FIRST PUBLIC RELEASE)

�Operating Mode 3 Added to Registered Version

�Clock/Base House Code manipulation added to

Registered Version

JAN'93    1.00 �.01   �Debug Mode Added

JAN'93    1.00 �.00   �Initial Beta (not publicly released)

������

(*5) Applicable to REGISTERED VERSION Only.