AUTOMATE

Home Automation Using a Personal Computer

by    Boyd W. Penn

91 Navarre Street

Hyde Park, Ma. 02136

(617) 364-3731

May 1, 1992

Version 3.0

Shareware Version

(c) Copyright 1991, 1992 Boyd W. Penn

All Rights Reserved

Permission is granted to distribute

exact copies of this document with its

source referenced

#

AUTOMATE - Home Monitoring and Control Shareware Version 3.0

AUTOMATE INDEX

Page No.

1.  INTRODUCTION  .............................................    1

2.  OBJECTIVES OF THE AUTOMATE PROJECT  .......................    3

3.  ABOUT THE AUTHOR  .........................................    4

4.  LICENSE  ..................................................    6

5.  APPLICATIONS  .............................................    8

6.  HOW TO DO AUTOMATION AND CONTROL   ........................    11

USING A PERSONAL COMPUTER

7.  SIMULATIONS AND ILLUSTRATIONS OF COMPUTER..................    15

MONITORING AND CONTROL

8.  SENSORS  ..................................................    16

9.  FILES OF ACTUAL MONITORING  ...............................    19

10. X-10 POWER LINE INTERFACE SYSTEM  .........................    20

11. GAME CARD   ...............................................    23

12. COMMUNICATIONS WITH YOUR PC USING  ........................    27

INFRARED OR RADIO

13. ROBOTICS  .................................................    28

14. CALLER ID AND OTHER TELEPHONE APPLICATIONS  ...............    29

15. PROGRAMMING  ..............................................    30

16. THE SOLAR ENERGY PROJECT  .................................    33

17. THE WEATHER STATION PROJECT ...............................    35

18. GENERAL AUTOMATION AND CONTROL THEORY  ....................    36

19. AUTOMATION AND CONTROLS LAB  ..............................    40

20. MURPHY'S LAW ACCORDING TO CONTROL THEORY  ................     41

21. PROBLEM AREAS AND LIMITATIONS  ............................    44

22. WHAT DO I GET IF I REGISTER  ..............................    45

23. THE AUTOMATE TOOL-BOX .....................................    48

24. QUESTIONS  ................................................    49

REGISTRATION AND ORDER FORM

INTRODUCTION

This software offering  is probably a  little different than most you have

seen.  Its about doing things with your computer. Not just databases, word

processing,  games and all  that sort of  thing but doing  real stuff like

turning  on  lights,  making the  coffee,  monitoring the  house  in short

getting out  of the  computer world and  bringing the real  world into the

computer.

Home automation,  monitoring and control  has been  the dream and  goal of

many folks but until recently it has  not been practical for many reasons.

Number one,  the cost of  real computer  automation was out  the financial

reach of most people in their home environment.  Number two, you needed to

be a  programmer,  electrical engineer,  control engineer  and  just plain

lucky to get your brilliant buzzing  computer box to do anything practical

as far as  the rest  of the world  was concerned.  Number three,  the real

world runs on high voltage,  like 115 VAC for lights, dishwashers, etc and

220 VAC for larger appliances like dryers and air conditioners.

High voltage  and  computers don't mix,  put high voltage  (except for the

wall plug to power it)  anywhere near your computer and you have one fried

machine that is in bad need of a repair shop.

That has all changed. The hardware is now available on the market to allow

the average computer owner  to have automation,  monitoring and control of

the home  environment.  This program  explains it  in simple  terminology,

shows what is possible,  tells you how to get the information into and out

of the  computer  and for  registered  users provides details  on building

equipment,  design of control programs,  lists of equipment manufacturers,

references to all sorts of information,  more sample programs,  how to get

analog signals  into  the computer  and  write programs  to  manipulate or

control from  the data.  And as the  sales persons  like to say  and much,

much, more.

The offering is divided  into two issues.  This the shareware version is a

discussion of what is possible and simulations.  The registered version is

an completely different  set of files,  devoted to the  actual control and

monitoring applications.

Welcome to the  exciting world  of computer monitoring  and control.  This

version  introduces  the  concepts,  provides  background  information and

education in the overall  field and allows  for a "test drive" to show and

demonstrate what monitoring and control  using your personal computer will

look like.

Imagine  pressing  a  key on  your  computer keyboard  and  turning on/off

lights, appliances, dimming/brightening lights, setting the lighting mood,

monitoring  temperature,   alarming  when   the  temperature  drops  below

freezing,  total automated control and monitoring  of the home environment

and all those other  things folks have  promised for years but where never

really available to the average personal computer user.

AUTOMATE is written  and developed  by an experienced  instrumentation and

PAGE 2

controls  engineer  with  extensive  background  in  automation  of  large

projects.  AUTOMATE  is  a tutor,  equipment selection  guide,  provides a

complete engineering working method for achieving your goals.  In addition

it is fun,  maybe the best computer fun ever, with that elusive quality of

vastly increasing your  computer skills  while enjoying the experience and

having a ball.

More important AUTOMATE'S  instruction manual is  written to the  level of

the average computer user and assumes no engineering,  electronics or math

background.   The   registered   version  contains   a  huge   manual  and

documentation written to  be very  detailed instructions to accomplish the

goals  of  home  monitoring  and  control.  The  instructions  provide the

background and  step  by step instructions  to allow the  average computer

user to develop their dream home automation systems.

The AUTOMATE menuing system  is provided to  allow for access of all areas

and functions provided by this offering.

PAGE 3

OBJECTIVES OF THE AUTOMATE PROJECT

The primary  objective  of the  AUTOMATE  project was  first  and foremost

information.  This  shareware  program  is designed  to be  an information

program,  a one stop,  essentially free look  at what  is available to the

home computer hobbyist.

Another very high  objective was to  develop an integrated home automation

system totally under control of the user  to develop in any direction that

met that users needs.  The one big  thing that stands  out in the field of

automation systems there  is no  "one size  fits all".  Everyones desires,

needs and wants  are different.  AUTOMATE was  developed to  allow for the

user to customize their systems to their tastes.

Integrated is just  that integrated.  The users  has total  control of all

functions in the automation system.  All functions can be in one big fully

coordinated program written by the user.

AUTOMATE is modular in construction. Each function is developed as a stand

alone  fully  functional  application.  All  functions  are  developed and

presented with a  text descussion,  source code applications  programs and

complete bills of materials if hardware is required.

AUTOMATE is  developed  with the home  computer hobbyist's pocket  book in

mind.  All  functions  are  as  low cost  as possible.  In many  cases the

function is  available as  a commercial  product.  The AUTOMATE  developed

function will be a  fraction of the  commercial products cost  and in most

cases will  perform as  good if not better.  Using the junkbox  as much as

possible with a common sense approach to problem solving and costs.

AUTOMATE was developed as a tutor.  The short time on the market has shown

the vast difference in user's skills.  Some are very accomplished and some

are new or novice computer users. One of the goals was to accommodate all.

This shareware  program  serves the  function of  very  basic introductory

information.

AUTOMATE was  designed to  never be  complete.  As long  as  there are new

ideas, there will always be something else to do with AUTOMATE. In keeping

with that concept,  AUTOMATE can never be  out of date,  constant upgrades

are prossible  as  technology changes  or new functions  become available.

None of this scrap the system and buy some new whiz bang system.

The overall goal of the AUTOMATE project  is to become the premeir program

for the home computer hobbyist and experimenter.  A one source, constantly

improving  development  incorporating affordable,  understandable,  doable

home automation projects.

The AUTOMATE project is a very time hungry, resources gobbling beast. I am

now on my fifth year of the project.

PAGE 4

ABOUT THE AUTHOR

Most software I  have used  brings to mind the question,  who is the idiot

who did this monstrosity?  If you buy  a book,  there is usually  a little

section on the  the back cover  that tells you about the author,  then you

can decide to believe what they say based on what they have done. Software

isn't  like  that,  something about  computer stuff  that says  you better

believe it and  if it don't  work it must be your fault.  I said if I ever

wrote anything for the  general public I  would at least  include a sketch

about the author.

I spent  about  twenty years  working  as an  instrumentation  and control

engineer building big things like power plants,  industrial plants, energy

projects, etc.  Basically I did the engineering for all sensors to collect

data,  the control room, computer systems, monitoring systems, controls of

all types, programming for industrial type computers and the like. This is

my first software ever released to the general public,  mostly because the

bulk of  what I  write is very  custom and  I wanted  something general in

nature.  Plus something useful  and that could  be modified by the user to

their  needs.  So  no  fancy  flashing  graphics,  windows,  gimmicks  but

something you can use and develop further.

Also  had  factory jobs,  electronics technician,  The Army,  Navy nuclear

power program on a submarine as a reactor operator/technician, worked in a

power plant and in general kind of knocked  around the world and saw a lot

of different situations.  I had  one interesting  project of  developing a

monitoring system based on a personal  computer to monitor a small factory

operations,  compute and log the data and  transmit it to remote locations

over the phone lines.  Also worked in  a calibration lab  and did a lot of

field cal  work.  Will give some  hints on  low tech/low  cost calibration

methods.

Any  programming  examples  will  be  done  in  modules  with  each module

explained.  The shareware version does not have any real world application

programming examples.  All programs in  this version  use the  joystick to

simulate  computer  monitoring  inputs.  The  registered  version includes

program examples for monitoring using real world inputs.

My software philosophy is simple. It is:

a. Software should be of use to the user.

b. Be tunable to their needs.

c. Be understandable

d. Be fully disclosed, with no "hidden secrets".

PAGE 5

e. Rights to use the results generated from use of that

software should be included in the purchase.

f. Folks should be willing to pay for what they get.

g. And finally it should be enjoyable, fun and hopefully

have some humor.  Enough of this white knuckle approach to

things.

PAGE 6

LICENSE AGREEMENT

AUTOMATE is developed and published by Boyd W.  Penn of 91 Navarre Street,

Hyde Park, Ma. 02136.

The information  presented therein  is for  educational,  experimental and

hobby  purposes  only.  NO  direct  applications for  home automation  and

control have been provided as part of this program, in whole or in part.

Although extra  efforts  have been  made  to ensure  this  program and its

documentation are accurate and precise, they are provided to you on an "as

is"  basis.  No warranty  of any  kind (direct  or implied)  is made  with

respect to this program and its related files.  The author of this program

is expressly not liable for any damages to computer hardware,  software or

any other equipment.  Also there shall  be no  liability to the  author of

this  program  for  lost  profits,  lost revenue  or  other incidental  or

consequential  damages  arising  out of  the  use or  information  of this

program.

The  user  assumes  ALL  risks  with  respect  to  the accuracy,  quality,

reliability,  performance  or  otherwise  use  of  this  program  and  its

information.

A limited license is  granted to all  users to this program to make copies

of the program files and its related documentation, and to distribute them

to other users subject to the following conditions:

1. The program, its related files and documentation are

not modified in any way and all files are distributed

together in their original form.

2. No fee is charged for the software and documentation,

except for a small distribution and media fee.

3. The program, its related files and documentation may not

be packaged with any other software (except as part of a

shareware software distribution program as stated in items

1 and 2) or hardware without written agreement from the author.

4. Any for profit, business or commercial use of this program,

its files and information is expressly prohibited.

5. The software contained herein will be used in a "shareware try

before you buy mode", without continuous or useful daily use.

Additional restrictions  and  terms are  contained  in chapter 22  of this

version for the right to purchase this software. In signing the order form

agreement,  the purchaser is acknowledging reading and understanding those

additional restrictions and terms in addition  to an understanding of what

is being offered in the registered version.

PAGE 7

Note: X-10 is registered trademark of X-10 (USA) INC.

PAGE 8

WHAT CAN I DO - APPLICATIONS USING YOUR COMPUTER

What kind of things  can a home automation,  monitoring and control system

based on using a personal computer do?

These applications come to mind and I'm sure its far from complete:

A. Burglar alarm

B. Security systems

1. Control lights in responds to motion

2. Control surveillance video systems

3. Calls if fire/break in

4. Extra smart garage door control

5. Door entry control

6. Night light control

7. Fire alarms

8. Outside parameter monitor

9. Counting events, people passing, etc.

10. Smart doorbell(s) (rings where you are)

11. Basement/cellar flooding monitor

12. Crib/area/room monitor

13. Oil tank level low monitor

14. Pool area monitor

15. Event logging

16. Panic/Help buttons

17. Sump pump control

C. Energy systems

1. Solar energy (a big one, more to come)

2. Attic fan

3. Thermostat control

4. Air conditioning

5. Heating

6. Hot water

7. Jacuzzi

8. Greenhouse temperature control

9. Snow melting

10. Auto warming

11. Environment control, air cleaners, humidity

12. Exhaust fans

13. Swimming pool temperature/level/pump control

D. Appliance control

1. Make coffee in morning

2. Dishwasher

3. Wake up service

4. Stereo

5. TV

6. Ceiling fans

7. Window fans

8. Light bright/dim control

PAGE 9

E. Communications

1. Phone answering machine

2. Remote control of home over the phone

3. Home status over the phone

4. Home based switchboard/paging/call forwarding

5. Telemarketing dialing

6. Voice mail

7. Remote door answer over the phone

8. Security calling features

9. PBX features through out house using one phone line

10. Communications/TV dish/antennas

11. FAX monitor

12. Phone/communications charges/time/logging monitor

F. Convenience/Monitoring

1. Lawn watering

2. Freezer thaw alarm

3. Automatic plant watering

4. Pet entry/exit control

5. Outdoor/Indoor temperature monitoring

6. Snowfall/rainfall monitoring

7. Mailbox monitor (did the mailman cometh yet)

8. Gate monitor (Is it open/closed)

9. Dawn/dusk light control

10. hallway/bathroom nighttime light control

11. Drapery control

12. Voice/motion activated control

13. Aquarium monitor

14. Freeze Alarm

15. Automatic timed computer operations

16. Automatic weighting and data entry to weight control

program

17. Fitness program monitoring

As you can see security,  energy systems and communications  are big areas

for home automation.  But there are already systems on  the market to do a

lot of those things, why mess around with a computer?

Number 1 -  Most stand  alone systems  are "dumb".  They blindly  do their

thing  no  matter what  is happening  around them.  Basically you  want an

integrated system.  Most stand alone systems  work on time.  Why water the

lawn just because  its 6 O'clock  in the morning and raining.  Or why turn

off the  window  fan at  three  in morning when  its still hot.  Using the

information developed by  one system and  then shared by all other systems

is very powerful, cost effective and provides systems that are very useful

and easy to live with.

Number 2 -  Most don't work  very well at  their best.  Murphy's Law talks

about the hidden flaw  and most stand  alone systems manage  to find them.

Some little  gismo breaks  and the  entire  system no longer  works or the

set-up is complicated with instructions that make  no sense and can not be

remembered plus you lost  the written instructions.  Another big flaw many

require  running  wires   for  power   or  control  through   an  existing

PAGE 10

ceilings/walls or where ever, lots of luck.  The systems I will talk about

in later  sections  avoid new  wiring  completely or  let  you decide  the

applications where it is possible.

Number  3  - Most  don't tell  what they  did or  if they  did it  at all.

Computer based systems provide data logging  with the event time and date.

Plus its at no real cost and can be very complete and detailed.

PAGE 11

HOW TO BUILD AN AUTOMATION SYSTEM

Ok so how do we actually build one of these automated systems.  The answer

is the the X-10  "power line carrier"  technology that has been around for

awhile.  Basically its little modules that plug  into the standard 115 VAC

wall power receptacles.  They are capable of sending signals to each other

over the same wiring that powers the house. The modules talk to each other

based on codes set on small dials on each module.

Many are  familiar  with the  motion  sensing light  systems  that turn on

lights when someone walks  in an area.  These sensors are  also capable of

turning on  remote  lights inside  the  house via  other  modules tuned to

listen for their signal.  Many manufacturers make these systems,  some are

Powerhouse,  Stanley and  Heathkit to  name a few.  Many stores  carry the

basic X-10 modules in the security section.

What type of modules are there?

1. Lamp modules to turn on/off and dim/brighten lights

2. Appliance modules to control small appliances

(15A or 1/3HP motor limits)

3. Motion sensing modules

4. Annunciator modules

5. High voltage appliance modules (220 VAC at 20 amps)

6. Contact sensing/input modules

7. Transmitter units to control the modules

(usually by a pushbutton)

8. Communication modules

9. Timer modules

All this has been around awhile,  the system is programmed by setting dial

codes and  control is  by a  fixed  base station  or  remote hand  carried

transmitter.

What is now  different is that  hardware gateway modules are now available

to allow your personal computer to talk to the X-10 modules over the power

line that powers the house.  The power line interface  plugs into the wall

receptacle and another  cable connects to  the computer serial or parallel

port.  Two way communication is now possible  between the computer and the

X-10 modules.  In short the computer  can now control lights,  appliances,

etc.  Also the computer can read the power line for communications between

two modules or  a transmitter/control unit.  An integrated  automation and

control system is now possible.

So   what's   the    difference,   the    computer   just   replaced   the

transmitter/control unit and is  doing the same  thing only it costs a lot

more.  Doesn't sound  like a  big gain to me.  What is needed is  a way of

using the power of computer software  to react to the changing environment

PAGE 12

and modify a fixed rigid program (that put in the X-10 transmitter/control

units - push a button turn something on) to a flexible program reacting to

changes like what time of the day is it, what day of the week is it, is it

raining, is it hot/cold outside, is the sun shining. In short the computer

replaces the human for monitoring and some decision making.

AUTOMATE is  written with  a common  sense approach.  The why's,  what and

concepts are based on proven engineering techniques.  AUTOMATE was written

around a  concept  of what was  needed to having  a working system.  Those

difficult or impossible  areas are worked  out and presented  as "plug in"

files  for  developing  your systems.  There is  no  experiment with  this

concept and see  if you  can get a  working application.  All applications

presented in  AUTOMATE  have been  built,  debugged and tested  as working

applications by the author.

AUTOMATE  was  developed  based  on  a  scaled  down  model  of industrial

monitoring and control concepts.  Safety was and is a prime consideration.

Safety is  discussed  in detail  and  is a  common  thread throughout  the

offering.  The  user  is  guided  in  the  areas  of  equipment selection,

construction and in the overall use concepts.

Lets stop  and see  what we  know and  what is  missing to  allow use of a

personal computer as a good control device.

A. The X-10 technology can solve the problem of connecting

the computer to the high voltage real world.

B. The X-10 technology is "Dumb". It just does what its told.

Therefore we need information from another source. Using this

other source of information we can modify the system operation

to meet the changing environment.

C. The status of the X-10 technology devices is not continuously

reported. The system only reports status when commands are

given. Therefore in certain situations we may need to verify

an operation actually happened.

D. Data logging and event reporting really become a useful possible

option at little or no cost.

E. Software design can become a problem if many different pieces of

software from different sources are used for separate functions.

F. We are kind of limited to on/off type control output.

G. The X-10 technology is modular in nature so you can try a little

project and then expand as the needs change.

H. The X-10 technology is not totally "Fail Safe", especially in a

computer control environment. The local control switches to the

devices being controlled are still operational but good software

design is still required.

I. The X-10 technology can not perform direct operation of many

systems.

PAGE 13

AUTOMATE is basically  the missing link  to allow the  home monitoring and

control system  to operate  as an integrated,  smart system.  AUTOMATE can

measure parameters in the real world, be programmed with the user's wishes

and exercise the control via a  communications gateway to operate the X-10

technology.  Those X-10  control/transmitters  are a  very poor  method of

achieving control  using  the X-10  technology.  First they  are  not very

powerful in capabilities. Second they make no sense to program and you are

forced to do it the  way someone else wanted.  Third they do not keep time

very well, your computer keeps excellent time. Fourth, they cannot measure

the outside world.  Fifth, the system size is very limited, using AUTOMATE

it is possible to build huge systems.

Where do we get those other sources of  information and how do we get them

into the computer?  Plus why who you really want to do it? A good place to

start is the game port.  You can buy input/output boards but my experience

with them has  been dismal.  Manufacturers like  to sell  these boards and

make  all  sorts  of claims.  I found  few  to pan  out.  They tend  to be

expensive and  the software  included (if any)  is very cramped  and short

sighted. Usually your into some sort of development program to make it all

work.

There are input/output  boards for  personal computers ranging from around

several hundred dollars to  over a thousand  but when you stop and look at

what is  required to  actually get them  to perform real  world control or

monitoring functions,  its many  of  the same things  that you have  to do

using my methods.  Only my methods don't cost even one hundred dollars, is

totally under your control, uses very low cost sensors and input equipment

and in the end, if properly understood and implemented is just as accurate

or maybe even more accurate.

I have used some  of these other  input systems and found little problems,

like no ability to make corrections for sensor errors or inaccuracy in the

software,  hardware and software compatibility  problems and stupid things

like you could not  unplug a sensor  without causing the  entire system to

crash.  Another big drawback,  you pay them  for the  input board but they

forget to mention the amount of calibration equipment required to ever get

a working system.  The board winds up being only  a very small fraction of

the cost of a working system.

The game port is good because usually  its already there at no cost.  Both

analog and contact status  data is possible.  The software routine to read

the data is  very straightforward.  Game ports  can be  added at  very low

cost.  Many sensors can be fashioned that work quite well.  As long as you

don't try  a  great complex  system  or require  blazing  accuracy it will

probably serve well.

The reason for doing it is simple. It is what is known as a hybrid control

system,  both  analog  and  digital  (contact  type)  are available.  When

combined with the  X-10 technology you  have all the necessary ingredients

for a hybrid  control system.  That allows all the  really good stuff like

direct  control/monitoring  of  systems  such  as solar  energy collecting

systems to  be possible.  Plus meaningful data  logging is available.  Lot

more on all this in the other chapters.

PAGE 14

What emerges  from all  this is the  ability to build  control systems for

home automation where equipment, appliances, lights, etc.  can be directly

controlled  via  a  personal computer.  The control  can  be initiated  by

signals developed by AUTOMATE, the computer keyboard, time/date as kept by

the  computer,  computed set  points  by a  program,  signals from  remote

control stations or  combinations thereof.  AUTOMATE carries  this concept

through the signal developments, integration into a monitoring and control

system and  the ability  to control  the  X-10 modules  to  make a working

system.  AUTOMATE includes no hardware as  part of the purchase.  All X-10

hardware and some software is by others.  A complete reference is given in

the registered  version for  purchase of those parts.  The instructions to

integrate all components is given in AUTOMATE.

AUTOMATE approximates the  capabilities of using  a commercial 8 bit input

card.  The lowest cost card  is in the  general range of $500 for just the

card.  This method has  no software  development problems and  unlike most

commercial cards the input have been found  to be extremely stable with no

drifting of values.

The systems  built using  AUTOMATE are  very flexible.  They can be easily

modified or adjusted to compensate for changes. The systems using AUTOMATE

do not become out of date as the technology changes.  The AUTOMATE systems

can  be  upgraded  without  scrapping  the  original system.  The AUTOMATE

systems are very low  cost and powerful  when compared to any other method

available for home automation, monitoring or control.

PAGE 15

SIMULATIONS AND ILLUSTRATIONS OF COMPUTER MONITORING AND CONTROL

Simulation files have  been included to  show what computer monitoring and

control using X-10 modules and the gamecard would look like.

All simulations  use  the joystick  or  keyboard cursor  keys  to simulate

analog or contact signals into the computer via the gamecard.  Programming

of the simulations is actually more difficult than programming of the real

control or monitoring scheme.

The simulations feature some animation and have a written text explanation

that is offered before each simulation.  The program automatically selects

the proper mode for running the simulations based on your equipment.

The simulations and illustrations are selected by choosing Item C from the

Main Menu.

The operative sections  of the simulations,  graphic displays  for analog,

etc.  source code has  been snipped out,  packaged and is  included in the

AUTOMATE TOOL-BOX.

PAGE 16

********** WARNING ************ WARNING ************** WARNING *******

DO NOT TRY TO USE ANY SENSOR THAT GENERATES ITS OWN POWER OR VOLTAGE

AS PART OF ITS SIGNAL INTO YOUR COMPUTER. ALL SENSORS AS REPRESENTED

IN THIS DOCUMENTATION ARE RESISTIVE IN NATURE. IN SHORT THE COMPUTER

SUPPLIES THE POWER. EXTREME CARE MUST BE TAKEN IN ANY DESIGN THAT NO

OTHER VOLTAGE SOURCE OTHER THAN THAT OF THE COMPUTER GAMECARD IS PRESENT

IN ANY WAY, SHAPE OR MANNER AT ALL TIMES!!!!!!!!!   THERMOCOUPLES FOR

MONITORING TEMPERATURES SHOULD NEVER BE USED AS PART OF THIS MONITORING

SCHEME!!!!!!!  THIS SHAREWARE VERSION IS NOT AN INSTRUCTION MANUAL FOR

THE SELECTING, CONSTRUCTION OR ACTUAL MONITORING OF ANY SIGNAL BY ANY

COMPUTER. IF YOU DAMAGE YOUR HARDWARE I AM NOT RESPONSIBLE IN ANY MANNER,

FASHION OR DEGREE!!!!!!!!!!!!!!!

******** WARNING ************* WARNING *************** WARNING *********

SENSORS

First  of  all  what  is  a  sensor.  A sensor  is a  device that  detects

something, or measures something. Some display directly the parameter they

are measuring,  like a wall thermometer.  It measures the  air temperature

and displays  it by  means of a pointer  on a dial or  a column of colored

liquid.  Other sensors measure  something but  do not directly display the

results.

Instrumentation and controls  engineers spend a  lot of time selecting the

right device to measure or monitor a particular control application. There

are literally thousands  of different sensors.  The following  factors are

usually involved:

a. A knowledge of the function to be monitored.

b. A understanding of the scientific principles involved.

c. How does the sensor work.

d. What sensor is best for this particular application.

e. How much does it cost.

f. Who makes it and how do I get one.

g. What is involved in getting it to work in a complete system.

I will  try to  boil all that  down into something the  average person can

understand,  can use  to  monitor things  with a  personnel computer,  can

afford,  can readily obtain,  can calibrate, can build and finally can put

into a home automation system as part of a complete package.

PAGE 17

The devices we  will be talking  about are called  analog sensors.  Analog

means it  is  a continuous  measurement.  The speedometer  in a  car is an

analog device. It shows your speed at all times and changes its display as

your speed changes.  Remember,  we can put four such measurements per game

connector into the computer.  (good stuff all right) What might we like to

monitor around the house? The following comes to mind:

a. Temperature

b. Levels  (of tubs, tanks, etc)

c. The amount of light (is light or dark)

d. Motion

e. Weight

f. Pressure

g. Time

h. Humidity

i. All those things I haven't thought of yet.

Remember that is  for analog measurements,  those where you  could look at

the computer and see a continuous display of a parameter.  Like looking at

a wall  thermometer  only this  time  the temperature  would  be displayed

continuous on the computer screen. A temperature sensor would be connected

to the gamecard and  use one of  the four available analog channels on the

gamecard.

There are other types of sensors than the analog ones we have been talking

about for use to  a home control system.  Many sensors sense  something is

either on or off,  is high or low, is hot or not hot, etc.  In short these

sensors detect one of  two possible conditions.  Usually they are referred

to as switches.  The temperature switch in an  attic fan detects the attic

is too  hot  or not  hot.  If too  hot  it switches  on (closes  a  set of

contacts)  and starts the attic fan.  Their output to the  real world is a

set of contacts that usually complete  an electrical circuit to make some-

thing happen.  Contacts look like this --[ ]-- when open, and are referred

to as  N.O.  or normally open  contacts contacts  when closed  --[/]-- are

represented like  this  and are  referred to  as N.C.  or normally  closed

contacts.  Another way to think about it  is contacts are the working part

of switches.  Examples of switches  around the  house are  light switches,

level switches to stop filling the clothes  washer when the water level is

correct and many others.

Contacts are also referred to as wet or dry.  A wet contact is like in the

thermostat  to  start  your  heating  furnace.  Take off  the  cover of  a

thermostat and you will see a little  glass tube pinched at both ends with

wires going into  the tube.  The tube has a little  ball of mercury inside

and is tilted as the temperature changes.  With the thermostat not calling

PAGE 18

for heat the mercury  is down in one  end of the tube and the contacts are

open. As the room temperature cools a bimetal wound spring tilts the glass

tube and  the mercury  covers the  bottom  of the tube  its entire length.

Mercury conducts electricity so an electrical circuit is completed through

the ball of mercury.  So the term wet contact. A dry contact is just that,

dry,  no mercury.  Pinch your forefinger and thumb together,  two contacts

close,  a good illustration of  a dry contact.  We can monitor  both types

with our computer.

So where are we?  We now know what an analog sensor is? What contacts are?

We also know we can  monitor both with the computer.  Four measurements of

analog and four of contacts. (In the registered version, I show you how to

put  about  all the  contacts you  could  want into  the  computer so  the

restriction on four contacts isn't really what it seems.)

Remember what analog and contacts are.  I use those terms through out this

documentation.  Analog is the continuous display  of a measured value.  If

you want to see a reading of something displayed and constantly updated on

the computer  screen  that will  probably  require an analog  input to the

computer.

Contacts  are  the  on or  off things  in life.  Contacts are  actuated by

switches.  Contacts are about position or status.  Things like is the gate

open or  closed.  It could be  halfway open,  a computer reading  a set of

contacts would just see the gate is open.

The registered version includes the data files,part No.s, curve generators

and  instructions  for  four  temperature  sensors and  one  visible light

sensor.

Industrial type sensors can get very complex.  Most include the ability to

measure the  parameter  intended and then  "condition or smooth"  the data

before sending it to a computer or other monitoring device. Just because a

sensor sees something doesn't mean that what it is seeing is correct.

PAGE 19

FILES OF ACTUAL MONITORING

Several files  of actual  monitoring using  the gamecard,  a visible light

sensor  and  a  calibrated  temperature   sensor  have  been  included  to

demonstrate  this  monitoring  capability.  A  sample  mailbox  monitoring

program using a contact  input with a  microswitch to monitor and log when

the mail arrives is included.

These files were collected by sensors mounted on my connector box and left

running in an unattended mode for several days.

The visible light sensor  logged the light  out the dining room window and

the temperature sensor logged a couple of steamy Boston day's temperatures

inside the dining room.

The registered  version  includes all instructions,  files and  methods to

implement monitoring to this level.

The information in  these files can  be further developed  for control via

X-10 Power Line Carrier technology.  AUTOMATE Version No.1 (the registered

version)  does carry the monitoring through control development with X-10.

X-10 is a separate set of software and the hardware is developed, patented

and distributed by others.

The  registered  version   provides  a   discussion  of  X-10  technology,

communication  file  for  controlling  X-10  modules from  within AUTOMATE

derived programs  and  additional information  for purchase of  the better

X-10 technology,  software for X-10 control by others and full details for

integration of all aspects.

Control can be via real time information as developed by AUTOMATE, through

keyboard inputs,  time generated setpoints by the computer or via computed

or  logged  information  based  on AUTOMATE  inputs.  Combinations of  the

methods are also possible.

To print out the daily log,  select data logging feature from the menu and

follow the prompts.

PAGE 20

X-10 POWER LINE INTERFACE SYSTEM

Many people have called me with questions about the X-10 technology. There

seems to be a lot of interest, some confusion and misunderstanding.

First a little history.  First X-10 didn't invent the power line method of

communication.  The method of impressing information  on a power source at

one location and  reading that information  back from the power source has

been around for quite awhile.  A number of industrial  processes use these

methods.

In the  late 1970's  the X-10  concept  was introduced  through  a line of

modules to be controlled  from base transmitters.  Sears,  Radio Shack and

others marketed  these systems.  Basically it  was light  control and  few

feeble attempts at  burglar alarms  systems and the like.  The systems had

problems, were buggy, lot of false triggering.

Over the next twenty years the bugs got  worked out of the modules and the

general communications method.  What emerged was a  kind of standard.  The

code format became a kind of defacto standard because so many modules were

produced built on that format.

Basically the technology is  to set a  system of codes kind  of like Morse

Code. In general there is 256 separate codes possible in the basic set and

something called extended codes. To sent the codes the hardware detects as

accurately as possible when the power sine wave (the house 220/110 VAC) is

alternating current)  is at the  Zero Crossing point.  120 KHz signals are

then impressed on the power referenced  to the zero crossing point.  These

signals are shaped and have a 1 millisecond duration and are placed on the

power in a certain sequence.

Kind of like with  a teletype communications  there is a  start portion of

the code and then the data is in blocks. It takes a number of power cycles

to send one complete X-10 command. The zero crossing point is the same for

all power through out the house. So the X-10 modules also have a reference

with which to know when a signal starts and can decode the signal strings,

validate the data and filter for noise.

I have found no problems in the basic X-10 module technology. None seem to

produce  large  amounts  of heat  that was  common  in dimmer,  timer type

controllers.

A lot of  the problems,  folks are  attributing to  poor X-10  modules are

actually caused  by  their electrical  systems.  Poorly installed systems,

incorrect  grounding,  noise  generating  appliances.  This  explains  the

complain, I have all kinds of problems, the guy next door does not. In the

registered version section on X-10 I  go into many of these areas,  how to

tell  what  is  probably the  cause  of your  problems  and some  possible

solutions.

The most likely causes of problems are the cheap transmitter/control units

and poorly installed or badly retrofitted electrical systems.  A word here

about this  offering.  AUTOMATE Shareware  Version 3.0  and its  preceding

issues is  basically  an information  program.  I do  not  offer technical

PAGE 21

support to the shareware version.  This offering is to inform,  put issues

in focus and see  who is really  interested in the  subject of automation,

monitoring and control.

Registered owners  of AUTOMATE  get full support.  I cannot design systems

over the phone.  I can answer questions concerning what you are getting in

the registered version.  Registered owners get the full support up to that

I can legally do.  If I don't see  the actual installation,  I cannot give

specific guidance.

What registered owners  do get is  full support in the monitoring area.  I

will analysis the  AUTOMATE generated  page prints for problems.  Not just

phone  support,  but  real  support  of  looking  at your  problem  on the

individual level.  The X-10 support level in more general,  I will look at

programs for general faults in the AUTOMATE/X-10 interface. I will not and

cannot comment on programs for the  actual control features to any control

project.

Back to the discussion of X-10. After going through a computer development

to  control  X-10,  as I  said  the problems  appear  to be  in  the cheap

transmitters/controllers. The computer based systems have none of the time

keeping problems. The quality of the transmitted signals is superior and I

have never found any of the type problems folks talk about. The quality of

the transmitted signal is important, if you put a junky signal out, do you

really expect a  perfect reception  on the other end??  The quality of the

computer  generated  signals  appears  to  be  very  high  grade.  I  have

experienced  no  communication  problems  what  so  ever  between AUTOMATE

generated programs and X- 10.

So  AUTOMATE's  basic  contribution is  in  the area  of  data collection,

ability to have total control of all programming and software development,

identify the best hardware available  for monitoring and control including

X-10  and  finally   to  put  it   all  together  in   one  fully  usable,

understandable  system.  The  X-10  communications  are  in  ASCII  (plain

english) and can be accomplished from the keyboard, generated as setpoints

in AUTOMATE derived programs or as computed values from AUTOMATE programs.

The  X-10  thing  isn't  really a  big  deal when  you  get into  it.  The

communications to actually send commands  takes quite awhile when compared

to anything in the industrial world. A command sequence is in the order of

seconds not milliseconds. Most X-10 systems on the market use only time as

the controlling parameter or try to  sell you fancy expensive do-dads that

you then jury rig to make some kind of control system. AUTOMATE allows you

to do it the right way, with the proper information and system control and

at a very good price.

I talk a  lot about safety  in both issues  of AUTOMATE.  It is not  to be

taken  lightly.  Safety and  the subjects  above are  tied together.  Good

safety practices can prevent problems before they occur.

The  general  drift  of  the  presentation  through  out  AUTOMATE  is  be

responsible,  research your projects carefully,  know what you  are doing.

The registered version is that, a careful step by step approach. The level

of detail is very high, cautions and warnings are given at the appropriate

times and the general  idea is to  progress in an orderly and safe fashion

both from  an  equipment and  personnel safety  point  of view.  I welcome

PAGE 22

comments from registered  owners of AUTOMATE  when they have their systems

up and running.

PAGE 23

********** WARNING ************ WARNING ************ WARNING ***************

WHO SHOULD NOT USE THIS APPROACH TO LIFE ON THE COMPUTER HIGHWAY OF

HAPPINESS?

ANSWER - ANYONE WHO HAS A COMPUTER THAT THEY JUST CAN'T AFFORD TO LOSE !!!

IF YOU HAVE A HIGH PRICED, HIGHLY DEVELOPED, MUST HAVE COMPUTER SYSTEM

THAT IS CRITICAL TO YOUR LIFE. TAKE MY ADVISE, DON'T GET ANY MONITORING

AND CONTROL, X-10 MODULE ON/OFF APPLICATIONS, HARDWARE OR ANY OTHER "OUT

OF THE COMPUTER" STUFF WITHIN TEN YARDS OF YOUR VALUABLE MACHINE. THINGS

DO HAVE A WAY OF GOING "BUMP IN THE NIGHT".

I HAVE A HARD DISK SYSTEM USED IN MY BUSINESS, WITH ALL SORTS OF

DEVELOPMENT TIME IN VARIOUS THINGS. UNDER NO CIRCUMSTANCES WOULD I EVEN

CONSIDER IT.

GET A ONE DISK DRIVE "BANG AROUND MACHINE" OR IF IT GETS "HURT" TYPE

MACHINE THAT YOU CAN STILL GET TO SLEEP. GOOD TIPS ON WHERE TO GET THESE

IN THE REGISTERED VERSION.

BESIDES IT MORE FUN WHEN YOU CAN GET IT "DOWN IN THE DITCH AND NOT WORRY"

*********** WARNING ************ WARNING ************ WARNING ****************

GAMECARD

Why  use  the  game  card  for  automation and  control  using a  personal

computer.  First  its  probably there,  most personal  computers  have one

installed.  Second,  if you don't  they are cheap  to purchase and easy to

install.  Sears sells a game card with  two game port connectors for about

$20.

Another big feature is you can easily put both analog and digital (contact

closures) type information into the computer.  Four channels of analog and

four  contact  closures  are  available  at  each  port connector.  In the

registered version I show you how to  increase the number of contacts into

the computer  to just  about any  number  you could  want  with no changes

required to the gamecard and its very easy to do.

Using BASIC or about another programming  language this information can be

addressed  and  collected  directly.  Very  nice,  write a  little control

program in BASIC and the intelligence  needed to monitor conditions in the

outside environment can  be brought in  though the game port and addressed

directly  in  the  same BASIC  program.  The commands  to  X-10 technology

modules to turn ON/OFF high voltage equipment can also be initiated by the

same program. Just what the doctor ordered, we can get into and out of the

computer and have total control of all processes.  No archaic gateway card

to buy with software that nobody could ever understand let alone ever make

it work in an integrated monitoring and control system.

PAGE 24

The gamecard can also "trap events".  For contact inputs the gamecard will

act like a information  monitor while your  software is off taking care of

other business.

If you  have  an IBM,  or IBM  clone,  or IBM  compatible  computer with a

gamecard the port connector should be a 15-pin,  miniature D-shell, female

connector.  Sometimes its a  separate card and  sometimes part of a mother

board  but  it  should  be  marked  "GAME".  Some  systems  will  have two

connectors, one for joystick #1 and another for joystick #2.

The connector should look something like this:

____________________________________________

[                                           ]

[  8     7    6    5    4    3    2    1    ]

[ ________________________________________  ]

[ \ .    .    .    .    .    .    .    . /  ]

[  \                                    /   ]

[   \ .    .    .    .     .    .    . /    ]

[    \________________________________/     ]

[                                           ]

[    15   14   13   12    11   10   9       ]

[___________________________________________]

Look up the gamecard in the  manual that came with the computer.  The pins

should be identified like so:

Pin 1   +5V                  Pin 9   +5V

Pin 2   Button               Pin 10  Button

Pin 3   Position             Pin 11  Position

Pin 4   Ground               Pin 12  Ground

Pin 5   Ground               Pin 13  Position

Pin 6   Position             Pin 14  Button

Pin 7   Button               Pin 15  +5V

Pin 8   +5V

If the  manual is  missing or  doesn't  say you  can check  by using  your

joystick.  Read pins  2 &  4 with an  ohmmeter while pushing  the #1 push-

button on the joystick. Sometimes its wired 2 & 5, but it should be one of

the two.  Check the #2 pushbutton (if you have one)  by reading pins 7 & 4

(again sometimes its wired 7 & 5).

********** WARNING  ******   WARNING ******   WARNING ******

***** DO NOT TRY TO READ THE PINS ON THE CONNECTOR ATTACHED TO THE ****

COMPUTER.  YOU CAN CAUSE DAMAGE.  BE SAFE.  USE THE JOYSTICK.

Read pins 1 & 3 for resistance.  you should get a reading of between about

20 to 70K ohms by varying the calibration dial on the joystick. Check pins

PAGE 25

6 to 8 for resistance and you should get the same readings.

If all this checks out, then all the information, programming examples and

hookups I give in this offering should work for you.

If you  take  your joystick  apart (no  big deal,  isn't very complicated,

there isn't much  in there why  do they cost so much?)  you will find very

few use all  of the pins  usually only 6 or 7.  Also a good  way to really

check things out.  How do gamecards really work? A little more information

can be helpful  at this point.  The gamecard generates  a staircase shaped

voltage pattern. something like this:

]__                       5 v

]__                    4.5 v

]__                 4 v

]__              3.5 v

]__           3 v

]__        2.5 v

]__     2 v

This is not  the actual pattern,  just something to  illustrate the point,

the actual  pattern is  much finer between steps.  Just great for  what we

want  to  do.  This staircase  voltage  pattern is  compared to  the input

voltage developed  across  the resistor  value in  the joystick.  The card

selects the staircase voltage value that  closest matches the input signal

from the  joystick and  generates a  binary  value for use  by the program

software.  So different  resistor  values in  the joystick  wind  up being

different binary  values  in the  computer.  Easy to see  how the computer

knows  where  the  joy-  stick position  is.  By playing  around  with the

joystick demo programs  included you can  see better how  this works.  The

pushbuttons do  not use  this staircase  voltage pattern,  just the analog

joystick position readings.

PAGE 26

********************* WARNING *************** WARNING ********************

NEVER PUT ANY SIGNAL INTO THE GAME CONNECTOR THAT HAS ITS OWN VOLTAGE

SOURCE.YOUR JOYSTICK HAS NO POWER.  NO BATTERIES.  NO POWER FROM A 115

VAC WALL RECEPTACLE. THE COMPUTER SUPPLIES ALL THE POWER SOURCE FOR ALL

MONITOR AND CONTROL FUNCTIONS THROUGH THE GAMECARD.  ALL!!!!  THAT IS ALL

!!!!!  SENSORS THAT I WILL TALK ABOUT ARE PURELY RESISTIVE IN NATURE AND

GENERATE NO VOLTAGE OR POWER BY THEMSELVES. IF YOU DON'T KNOW WHAT YOU ARE

DOING ASK SOMEONE WHO DOES. ALSO GREAT CARE MUST BE TAKEN IN CONNECTING

SIGNALS INTO THE GAMECARD. DO NOT USE ALLIGATOR CLIPS OR JURY RIGGED

CONNECTIONS. GET A GOOD CONNECTOR BOX. I GIVE DETAILED INSTRUCTIONS IN THE

REGISTERED VERSION ON A CONNECTOR BOX CONSTRUCTION AND CHECK-OUT. A MISTAKE

OR ERROR IN DESIGN, HOOKUP OR ACCIDENTAL FAULT CAN DAMAGE YOUR GAMECARD,

COMPUTER POWER SUPPLY, CONNECTOR CABLE AND BOX AND SENSORS. KNOW WHAT YOU

ARE DOING.

******************** WARNING ***************** WARNING ********************

By using  this  staircase voltage  pattern  we can  measure  and calibrate

analog measurements  like temperature,  amount of light  (how bright,dim),

level,  pressure and  many others.  I give  detailed  instructions in  the

registered version  on sensors,  calibration and how to  use the developed

signals in a program.

The AUTOMATE method of using the gamecard approximates the capabilities of

using  a  commercial  8  bit  input  card.  That  is  a  signal resolution

capability of one  part in 256.  I have found this  method far more stable

than most commercial input cards. The commercial input cards all exhibited

problems with the measurement values drifting, especially temperature. The

AUTOMATE methods have proven to be remarkably stable and repeatable.

A final big reason!!  MONEY !!.  The X-10 technology  does somethings well

but sensing and reacting to a dry contact, with no power involved, telling

the control system something is happening is not one them.

PAGE 27

COMMUNICATIONS WITH YOUR PC USING INFRARED OR RADIO COMMUNICATIONS

Imagine setting in the  old easy chair  and control the house  with one of

those TV channel changing clicker controls.  Or being out in  the yard and

have  the  personal  computer  automation  system  do  things  around  the

homestead via use  of radio signals.  Kind of like carrying  around one of

those garage door opener type transmitter.

This was always one  of the applications  to design for use with AUTOMATE.

At present  I  have several  designs  for both applications  in the works.

There is commercial equipment on the market in both areas. All of it is in

the hundreds of dollars range,  doesn't appear to be  all that capable and

they brush over the description of the software included.

So  I  began  working  up  some  very low  cost  alternative applications.

Probably  can  get  most  of  the  hardware  complete  and  with a  little

modifications have  working systems.  Both applications  built  by me were

under $20.  The infrared transitter and receiver  cost me the princely sum

of $10.50.

With the other few  parts needed this  application is going to cost around

$14  when  completed.  Talk about  a bargain.  The unit  will  have twelve

different control channels.  That is twelve different  commands capable of

being sent  to turn  some on/off  or otherwise.  Control engineers like to

talk about how much something cost in  terms of cost per measurement point

or control channel. Folks that's about a buck a control channel.

Like all AUTOMATE applications the methods will be fully described.  Bills

of material furnished  by model  number and  suppliers referenced.  Sample

programs for the application will be included.  A picture will be included

of any complete prototype hardware.

This application will be included as part of the AUTOMATE TOOL-BOX.

PAGE 28

ROBOTICS

The shareware version includes a questionnaire attached to the order form.

That questionnaire allows me to determine areas of potential user interest

in which to  develop marketable products.  The two most asked for products

are something  to do  with the telephone,  either control external  to the

house via  the telephone,  message logging/numbers dialed  out or incoming

caller ID and the other big requested area is robotics capabilities.

I am in the process of developing a robotics capability for AUTOMATE. This

will be a separate commercial program with no shareware information.  This

is a fairly time hungry project.

Brief details. Just getting into the project.

The project will include software,  software drivers,  instruction manual,

hardware to connect to the parallel port.

The robotics capability requires a  rather fast acting control loop.  This

is not  possible  the X-10  technology  the delay  in  sending one  signal

command is in the order of seconds.

Plus this project opens many other interesting possibilities. Things like:

1. Motor speed control

2. Quick acting control loops

3. Monitoring of many more parameters, capacitance, voltage, current, etc

4. Measuring light color and other properties of light

5. Positioning applications, to include sensing and measurement

6. Digital storage techniques for audio, analog data, etc.

7. Speech techniques

8. Oscilloscope type displays

9. All those analog to digital or digital to analog applications.

A preliminary look  shows the  present developed capabilities  of AUTOMATE

with  inputs  through  the  connection  box complement  this  project very

nicely. The connector box provides usable very cheap input ability.

No time  frame for  having a marketable product.  The hardware required is

probably going to have to be  pre-manufactured on maybe in kit form.  Just

have to play it by ear.

PAGE 29

CALLER ID AND OTHER TELEPHONE APPLICATIONS

The telephone applications  of automation are  an area of  great interest.

The telephone companies are installing  a Caller ID capability.  What this

allows is for the receiving party of  the telephone call to see the number

of the calling party, including the area code.  I guess the rational is to

see who  is really  making all those  calls with just  the heavy breathing

sounds.

There are  a number  of systems on  the market which will  monitor and log

your out going calls,  length of the call in minutes, seconds, time of day

the call was placed and the number called.

A number  of  AUTOMATE users  indicated  they presently  use  some type of

telephone automation equipment.  Apparently it is very useful for checking

the accuracy to the telephone bill or  determining who in the household is

running up the phone bill. A number of systems have the ability to log the

information on a personal computer.

There is  a new  integrated circuit chip  on the market for  the caller ID

function.  A brief look  and talking to  one of the makers representatives

seems to be fairly simple circuits.  A handful of components. The computer

serial port would power  the circuit and  there is some sort of hook-up to

the telephone.

The only rub I've heard of, the person or company building the circuit has

to certify  that  the operating  device meets  some standard.  There is  a

standard  set  by  Ma Bell  and  all connected  equipment  must meet  than

standard.  Apparently not that tough to do, but if requires test equipment

and some skills in that area.

The whole idea works around the principle  of when a call comes in,  after

so many  rings  the calling ID  information is available  on certain phone

connections.  A signal generates an interrupt and the computer software is

able to read and process the information from the serial port.

Haven't done that  much work  on the application yet.  Am getting a IC set

for that purpose,  after a little more study maybe  will include it in the

AUTOMATE stable. No promises at this point.

I'm sure  the equipment  is going  to  be available  very  shortly on  the

commercial market. That may prove to be the way to go. Buy a pre-certified

hunk of hardware  and it  would probably include some software.  Like most

things what will be  the price and  will the phone  company want something

extra?

PAGE 30

PROGRAMMING

Groan, don't tell me programming is involved.

Yes,  Virginia programming is part of the deal to have really capable home

automation custom to your application.

Programming has gotten a  very bad rap.  AUTOMATE 1.0 to 2.0 shareware and

its  attendant  registered   version  were   developed  around  the  BASIC

programming language.  All program lines  had line  numbers and  would run

under GWBASIC,  BASICA in a  interpreted mode.  All executable files where

prepared with  the  IBM BASIC compiler.  The compiler  was buggy,  lots of

teeth gashing, muttering and sometimes worse.

The good thing about sticking with the BASIC language is most folks have a

foggy idea of  the language,  it is  upwards and  downward compatible with

about all versions ever  produced and is  still the language of choice for

many  data   acquisition   systems,  large   data  file   manipulation  or

control/automation work.

AUTOMATE  3.0  and  up  will   use  the  Microsoft  QuickBasic  integrated

programming environmnent. All prior AUTOMATE BASIC programs will run under

this  environment.  The  QuickBasic  4.5  is  totally compatible  with the

AUTOMATE concepts and objectives.  First is it very affordable.  Microsoft

QuickBasic version 4.5 can be purchased from:

Computer Discount Warehouse

2840 Maria Avenue

Northbrook, Il. 60062

1-800-729-4239

1-708-498-1426 (in Illinois)

1-708-291-1737 (FAX)

For a price around $73 including all charges.  This is the lowest price, I

seen advertised.

QuickBasic  allows  many  of the  more powerful  programming  features and

program design methods available in some of the more erubescent languages.

Its downward compatible  feature coupled  with the advanced  features uses

what you already  know and builds  toward the more advanced features.  The

commonsense approach.  The QuickBasic makes .EXE files smooth as silk, any

problems it runs them down, puts the cursor right on the offending critter

and has a help section to explain what is wrong.

QuickBasic has  an  excellant tutoral,  is menu driven,  can be used  in a

object module library mode.  In addition users can  very easily turn their

BASIC control  and  monitoring programs  into DOS executable  files (those

with the .EXE extentsions).  I highly recommend  the product.  In addition

the compiled programs run  much faster and  are much easier to  use in the

everyday environment.

QuickBasic consists of  four 360K disks.  It requires a hard  disk or high

capacity floppy  drives to  use the product.  I use it on  a hard disk and

PAGE 31

just love  the system.  Microsoft has a winner,  the novice programmer has

been well considered.  With a BASIC manual, a little practice, you will be

able to handle anything in AUTOMATE in very short order.  QuickBasic has a

458 page manual and the documentation is excellant.

QuickBasic is self installing and can be customized to the users taste.  A

very nicely  done product.  QuickBasic is  far more  particular about  its

syntax than is plain vanilla BASIC.  The other big advantage is QuickBasic

does not have the 64  K limit to program size.  Getting into large control

programs with  lots  of graphic displays  can really produce  the lines of

program. QuickBasic can handle a 640 K program.

Each module is  still restricted to  a 64 K  block size.  The way to write

huge programs is lay  out the organization  first to ensure each module is

less than 64K.  Modules are then  compiled to  make object files and those

can be  linked to  make the huge programs.  The code is  pretty compact in

QuickBasic so 64 K is a fairly capable chunk of code.

The whole  programming thing  is very  over done.  I constantly  see folks

referring to "SOFTWARE" with all sorts of strange notions. All software is

essentially only two things.

1. Program steps to execute instructions

2. Data to be manipulated by those instructions

With  QuickBasic  and  a simple  instruction  book like  the  old Sinclair

Manuals or other introductory manual anyone  can master the skills in very

short order.  Your childern who  have taken  the most rudimentary computer

course can walk you through the process.

The QuickBasic programs can also be run  under QBasic which is included as

part of MircoSoft's DOS 5.0 and 6.0 releases.

I have Borland's Turbo  C and Turbo PASCAL  4.0 and I can chug along but I

really love  the QuickBasic.  I am  not the  most dazzling programmer,  so

nothing  is  going to  get very  complicated.  Where required  all program

source code will always have an  accompanying written text description and

the source code.

A number of folks  call and a  very disappointed that everything is not in

the C language. If its not in C it must not be very capable.

The simple answer is, it just isn't required.  That fine detail of control

and capability  of  C or  Pascal,  just isn't required.  In some  cases it

actually gets in the way. Blinding speed isn't that important either. When

moving into robotics,  I can see  some assembly  language routines will be

required but  QuickBasic  is very capable  of bolting those  on piggy back

style.

C and Pascal don't give you any faster access to I/O than does QuickBasic.

The compatibility problems between different  versions of Pascal can drive

you up a wall.  The data handling  and control  routines are actually more

compact  in  QuickBasic.  Why  do  some  folks  like  life  complicated as

possible? Beats me.

A good understanding of the control concepts and application techniques is

PAGE 32

far,  far more important than all of the rigamarole about langauges or the

fine hair splitting some folks like to engage in.

Just using the program modules provided in the registered versions, making

minor alterations and  using QuickBasic  can develop very capable,  custom

home automation systems.  The AUTOMATE  approach will  get slicker as time

goes  by.  Modules  will  be  provided in  vehicles like  TOOL-BOX  so the

programming can become more an editing function than writting of a control

program.

The TOOL-BOX has  been developed to  have each application compiled to run

as a stand alone application.  There is a manual for each application.  In

addition the source code is included for each application. That way if you

don't want to get into the source  code you can run a standard application

program.

All AUTOMATE programs will eventually get redone in QuickBasic 4.5.  Those

will be compiled and  be menu driven.  I highly recommend  the QuickBasic.

The programs are much smaller, much easier to write and understand and any

complex situation it's QuickBasic hands down.

The QuickBasic also  includes a  very slick text editing feature.  Kind of

like a word processor in many ways.  You can copy,  move, delete and other

wise manipulate large blocks of source code very efficently.  This feature

is available within  the programming envirnoment.  It really proved useful

in the development of the TOOL-BOX.

Some applications will  be complete stand  alone programs that also can be

linked together to make large integated control programs.  There will also

be very basic text instructions in most areas.

More functions  will  get bundled,  compiled and have  set-up routines via

menus. The source code will always be provided for the daring folks.

PAGE 33

THE SOLAR ENERGY PROJECT

Included in this shareware  version is a  bit of information about a drain

down solar energy collection system being designed by the author.

I have always been very interested in solar energy. Over the years, I made

accommodations for adding it  to my house.  I dug a pit  for the hot water

storage area under the front porch.  I installed part of the piping system

to interconnect the system.  I researched, researched, did costs analysis,

more on paper dreaming.

The same project impediments  would always pop up.  One was the commercial

available control systems. Expensive, poorly designed, just not what I was

looking for in  a control system.  I talked to  a number  of folks who had

sprung for a commercial system in the days of the big energy shortages. Do

you remember those days?

Many of  the  systems didn't  last.  Operated poorly,  the warranties were

worthless.  The vendors had all gone bankrupt,  gotten out of the business

or just plain run off.

Another impediment was  total system cost.  The collectors were expensive.

None performed up to specifications,  were easily damaged,  sufferred from

corrosion, had problems with leaking and deteriorated over time.  For cold

climates  (Boston  does  get  cold)  most  systems  have  multiple  pumps,

antifreeze loops and other Rube Goldberg craptrap.

Ta-daa, enter SUPER AUTOMATE to save the day. AUTOMATE could do the system

control  in  real  bang-up  style.  I  could  design an  all steel  welded

collector from scrap  parts laying around.  Use some tempered glass sheets

(saved from going to increase our dump capacity)  and have the lowest cost

collector around, like almost free.

Use some door length ordinary mirrors to have system gain. Why not have at

it.  Drag out ye olde welder, get some 1" square channel, use the old chop

saw,  weld the channel up  in a serpentine configuration,  weld on backing

plates. Presto, one super energy collecting device and this baby will take

full home heating  system pressure.  (Up to  30 psi).  One instant plug in

module that can heat the house direct if required.

AUTOMATE  to  the rescue.  AUTOMATE can  do  very complex  logic,  measure

temperature,  control  115  Vac  pumps  and solenoid  control valves,  run

different modes depending on the season, do data collection, be controlled

from  the  keyboard  of  my control  computer.  Winter approaches  and the

project grinds to slow pace as the weather gets worse.

Playing around one night  designing the system.  Why not do a recreational

programming exercise to  see what  this system will  look like.  The solar

demo  program  is  born.  There is  not  room on  this disk  to  include a

simulation of the solar project.

The drain down feature removes the water from the system collector and any

PAGE 34

areas subject to freezing.  The attic does not go below freezing. How do I

know?  Temporary AUTOMATE temperature measuring and data logging over last

winter.  Great for  those  little projects  like what is  really happening

there all the time.

The solar energy  program will probably  be fully developed as a shareware

stand  alone  program.  A control  program  will get  written and  will be

available through one of the AUTOMATE commercial offerings.

PAGE 35

WEATHER STATION

Again a number of ideas to develop for AUTOMATE come from the users of the

registered version.  A couple of comments on  the order form questionnaire

are responsible for this application.

They said you already have ambient temperature and daylight why not go all

the way  and develop  the bulk  of  measurements found  in  a home weather

station. Good idea.

This project is in  the very preliminary stages.  The shareware version of

AUTOMATE gets updated about  once a year.  The TOOL-BOX and the Registered

Version  are  worked on  a continuous  basis.  So this  is  an attempt  to

preguess how the weather station will finally work out.

Probably fit into  the TOOL-BOX because  it is a  stand alone application,

will get the full graphics treatment.

What can be added.  Wind Speed and Wind  Direction for sure,  maybe a rain

and snow fall detector and gauge.  Prototypes will get built and a picture

included. The calibration procedures will have to be developed. Wind speed

and direction seem very plausible on paper.

Other possible  parameters  might be  a barometer  and humidity.  This all

brings up the point won't I run  out of analog channels pretty quick.  No,

that requirement to  expand the analog  channel capability has  been under

consideration for some time.  So another application would be to develop a

way to pack the entire weather station into one analog channel.

There  are  examples  of  this  requirements  from  things  I  did in  the

industrial world.  So I'm  fairly confident  it can be done.  Also a small

drop in commercial data acquisition board is being looked at to expand the

capabilitites.  Some of this  runs into the  things that will  be required

when the robotics part gets tackled.

The bottom  line is  it will  all  have to  meet  the AUTOMATE objectives.

Everything has  to  be capable of  being integrated together  and be under

direct control of the user.  That probably means source code compatible to

the overall ojectives from each application.

I'm always a little amused when  researching a new project like this.  The

prices and promises for some of the commercial versions. Plus when you ask

that question, can this thing run in the background, can I use my personal

computer to monitor other things at the same time.  Usually either silence

or mumblings about yes,  sure,  maybe but it's up to you to figure out how

its done.

Could get pretty wild, couple X-10 to the weather. Wind speed over 20 MPH,

maybe make the coffee early.

PAGE 36

GENERAL AUTOMATION AND CONTROL THEORY

Control theory brings to mind,  lots of math,  complicated theory,  arcane

stuff and just  so much mumbo-jumbo.  In truth it is far  from that and in

the everyday home setting can be applied with great results,  all you have

to know is  a few  of the principles involved.  Have some common sense and

control theory can be  applied to many situations.  I will try to show how

some of those principles are applied in the everyday world.

First a  little bit  about control  engineering.  Basically it is  hard to

master from schooling,  the best control engineers learned the trade in an

apprentice fashion.  When applied to the real world its about knowing your

equipment,  knowing the application, knowing what not to do, understanding

how the design  process works,  organization,  experience,  history of the

trade,  knowing a  little  about what everyone  does to make  a successful

project,  a  little  programming  skill and  finally a  lot  about SAFETY,

SAFETY, SAFETY and more SAFETY.

When applied  to  home applications  it  can be  broken  down into  simple

principles. Some of those principles to be aware of are:

FAIL SAFE CONCEPT  - Fail  safe is just that,  when something fails  it is

inherently safe in  its failed state.  Example:  When a light burns out it

doesn't present much  of a threat,  you just don't get any light.  This is

true even though  the power to  the light is  still on.  Control engineers

spend a lot of time asking,  what happens when this device fails.  Another

example of fail safe:  The power to the burner and fuel oil solenoid valve

of a  home fuel  oil fired  furnace  comes through  the  contacts of  high

temperature sensing  probe and  other safety  sensors.  These sensors  are

designed to  fail  or actuate  with  their control  contacts  in the  open

position. With the contacts open, the burner and fuel oil cannot get power

to operate and the entire system shuts down.

Most  appliances  have  their  fail  safe  concept  built in.  Example:  A

dishwasher must have the door closed and latched to get power to the timer

to start the washing cycle.  If there is a problem  and the door is opened

to check on the problem the appliance is inherently fail safe. Opening the

door removes power from  all operative functions.  The water fill solenoid

valve cannot get power to open and flood the kitchen floor.

Most computer control is only to the level of operating a unit.  Like: The

computer might turn power  on to the  dishwasher but would not perform the

functions to  control  the washing  cycle.  Only a  fool tries  to put all

control functions into a computer.  Computers are not inherently fail safe

devices. In industry many systems rely on hard wired contacts with sensors

on the actual  equipment to  back up the  computer control.  If there is a

problem the hard wired sensor win any argument between it and the computer

as to what is safe.

With home automation  becoming a  real possibility many manufacturers will

be  hawking  all  sorts of  equipment  and gizmos  to  be controlled  from

computers.  Be careful. A good question to ask yourself is "would I really

do this if no computer was involved"? Computer control can actually make a

safe condition into  an unsafe one.  Example:  Put a electric  room heater

PAGE 37

near a pile of papers in the off condition,  walk away and have a computer

turn it on a later time.

PERMISSIVES - A  permissive is a  device that allows  something to happen.

Our above example of the oil fired furnace, the safety sensor contacts are

permissives. They must be closed to get power to the fuel valve and burner

in order for the system to operate.  Computer control is very good because

it allows for very extensive development of permissives in complex control

situations.  Usually a combination  of contacts  sensing something  in the

real world  and  software permissives in  order for an  action to proceed.

Home control can  benefit from this  power by bringing contact information

in thru the  gamecard and  developing software information  through analog

information  allows  for  very  powerful  design  concepts.  Solar  energy

collecting systems for example.  The general idea of  permissives and fail

safe operation are closely tied together in most applications.  In general

if you  are trying  to get a personal  computer to do something  in a home

environment where the concept is unsafe by itself, using a computer is not

going to  make  it safe.  Think out  any application.  Understand what  is

involved.  It is very easy to think I'll write a program to take care of a

problem.

ADAPTIVE CONTROL - Adaptive control is often touted as control that learns

from its own actions.  In practice it is really  a number of predetermined

courses of action to a given situation. It is usually the concept to which

people  refer  when  raving  about  the power  of  computer control.  Some

examples:

Say you are trying to  measure a range of  32 to 600 /F but can't find one

sensor to do it  in a satisfactory manner.  But you do find  a sensor that

can measure  32 to  350 /F  and one  for the  range of  300 to 650 /F.  By

connecting  both  to  the computer  and  then having  the  software switch

sensors at the appropriate time a satisfactory  solution could be a had to

your problem.

Industrial applications use a lot of adaptive control routines.  It is the

basis which  allows  many complicated processes  to be controlled  to very

tight and efficient setpoints.  Many processes will measure the same thing

with more than one sensor and then have  the computer switch on the fly if

it detects a failure in a sensor. Great concept to understand.

A good understanding of adaptive  control concept and computer programming

is the dynamite combination.  This combination allows solving many complex

problems or just getting you out the sticky I don't have the right sensors

bind.

FEEDBACK - Feedback  is a complicated  sounding concept but in practice is

quite  simple.  In general  is information  returned to  tune,  correct or

adjust  an  on  going  event.  It  is  probably  the  one  concept  that a

misunderstanding of can get a would-be home automator into big trouble.

Example:  You are controlling greenhouse grow  lights to maintain adequate

light for the plants at all times.  Just turning them on  based on time of

day may not  be energy efficient.  So banks of grow  lights are installed.

The overall light  level is  sensed in  the greenhouse,  and banks of grow

PAGE 38

lights are switched  on or off  to maintain that  certain light level.  On

most days all lights  would be off and  then come on as the light fades at

sunset.  On bright moonlight  nights less lights  would be on than on dark

nights.  The information provided by the sensing of the amount of light is

feedback. That information allows adjusting or correction of the amount of

light by varying the number of banks of grow lights.

So how can feedback get you into trouble.?  Basically by causing something

called cycling or sometimes called "pumping".  Take our above example: Say

a bank of grow  lights just turned  on to maintain a  certain light level.

The overall light sensor detects the  increased light level after the bank

of lights  turns on.  The software program  in the  computer reads the new

light level reported by  the sensor and  thinks the light level  is now to

high and  turns off  a bank of lights.  Its now too dark,  so the computer

turns on a bank  of lights and  the cycle repeats and  repeats because the

computer soft- ware  cannot find a  level which is  just right.  This is a

common problem that  must be guarded  against in all control applications.

Microprocessor  based  control  is particular  akin to  this problem.  The

process itself can be the problem, the setpoint in the software can be the

problem or the way  the sensor is mounted  can be the problem or finally a

combination of all of the situations. Cycling can be a problem even in the

simple program just based on time with no analog inputs.

The connector box described in the  registered version comes in very handy

in detecting these problems as it can act as a checkout lab or development

board.  This  allows  checkout  of the  software and  its  sensors without

actually  installing   the   equipment.  X-10   technology  can   also  be

incorporated in the lab checkout scheme.

It is not something to be afraid or excited about, instead something to be

aware is a common problem.  It is something to be detected during software

checkout in a lab setting  before installation of an application.  Cycling

can still  occur after  lab checkout  if  the problem  is  related to  the

process  feeding  back  information that  is causing  the cycling.  Sensor

mounting (the  way and  manner something is detected)  can be the problem.

The other  common cause  is too narrow  a control band in  the software to

turn something on and then off.  Some cycling problems may  be inherent in

the application.  These can  be  controlled by  having the  software leave

something for a minimum  amount of time  as long as no unsafe or operating

parameters are exceeded.

Cycling can and  does destroy equipment.  Trying cycling an ordinary light

wall switch and  see how  long the light  bulb lasts.  Cycling causes most

electrical equipment to increase in operating temperature. Again something

to be watched for carefully in designing projects,  programming,  software

checkout and finally in installation and checkout of that installation.

The purpose of this  discussion has not  been to alarm anyone  or to deter

anyone from  using home  computer automation type control.  Instead it has

been to point out there are a logical principals involved and well planned

projects that is thoroughly understood can be very beneficial.  Be wary of

manufacturer claims of easy, foolproof gizmos. Just take this software and

plug it in and lots  of magic will immediately happen.  Understand what is

being offered  and does  it fit  your application.  No industrial  control

engineer will take  precanned solutions  and just plug  them in.  No major

manufacturer of industrial control  equipment will offer any.  Examples of

PAGE 39

how  to  program  their  equipment  and  software  that  demonstrates  the

capabilities  are  usually all  you will  get.  Somewhere out  there  is a

programmer who is  going to write  the worlds perfect  menu driven control

program to solve  all home applications.  He is a fool  and the person who

tries to  use it  is a fool.  There is liability involved,  probability no

matter the number or manner of disclaimers offered.

In the registered  version are example  programs for use in monitoring and

control of the  real world environment.  Just as important are discussions

of  how  to  "trim"  your  programs  and  sensors to  adjust  them to  the

particular application.  Pre-canned programs WILL NOT work  in all or even

most situations.  Each computer,  program,  sensor and application has its

own set  of characteristics.  The registered  version  provides the  basic

knowledge  required  to  fit the  measurement to  the application.  A high

degree  of   programming   knowledge  is   not  required,  just   a  small

understanding of BASIC.

The registered version programs are working examples.  More important they

are  taken   though   all  steps,   design  of   equipment,  construction,

calibration, curve generation, data fitting and actual coding of a routine

to use with that information.  Also explained are some conditioning,  data

validation   and   smoothing   techniques  required   to  get   a  working

installation.  All  required  information is  provided with  instructions.

Where programs  are complied,  appropriate source  code  is furnished  for

those wanting to modify for their use.

PAGE 40

AUTOMATION AND CONTROL LAB

It won't  take long  after reading  and  playing with the  demos that most

people who are into computers will say "I've  just got to try some of this

control stuff".  Maybe you don't have any home projects on the menu. Maybe

you don't  even  own the place.  How to do  some projects.  You got it,  a

automation and  control lab.  It doesn't  have  to be  fancy.  A few  X-10

modules,  a gateway and a few lights,  the gamecard/connector box and some

analog and contact inputs.

The  lab  has  many functions.  If you  design home  projects to  actually

install,  the software has to  be tested and  a lab situation  is the best

way.  Plus labs are great fun, maybe the best computer fun ever. I used to

design  small  control  projects using  programmable controllers  (kind of

industrial computer dedicated  to control)  and I would  take a controller

home set it up in the den and check out software at night.

Some of the  industrial machines have  some of the  lab functions designed

in.  Like most consist of a control unit (computer), a power unit, a track

unit with input/output signal modules and  a box to program the unit.  The

track unit with its modules had LED's (lights) to show when output modules

were  turned  on/off  and  the  contact  input modules  had  little toggle

switches built in to simulate  contacts closing in the factory.  I built a

analog generator box and presto, one small factory application in the den.

I also built a  wooden mounting rack so  it was mounted very  similar to a

factory mounting and was safe.

The gamecard input box I designed can be used as a lab. The design is such

that inputs can be simulated even if no actual automation project is being

produced.  The demos  and  the joystick  is a kind  of lab.  The design is

simple but it does the job.

Automation labs  are  fun but  you can  get  into trouble.  There is great

temptation to do unsafe things. Like having 115 VAC on exposed wiring with

alligator clip jury rigs, etc. Using the gamecard as an input, having jury

rigged inputs.  If high  voltage  is involved  and exposed  a  mistake can

result in short circuits, smoke, equipment damage and maybe even injury or

death.  The US  Navy  has about  1000  sailors killed  each  year even  in

peacetime. Looking at who gets killed, it falls into three categories.

Sailors being  killed  in accidents  on  the dangerous  flight  deck of an

aircraft carrier, sailors being killed by electrocution and others.  Those

being  killed  by   electrocution  are   usually  sailors  who  are  radar

technician, electrician or electronic technician.  All had formal training

in their jobs, safety training and experienced supervision. The Navy has a

safety program  where  films called "your  deadly shipmate -  115 VAC" are

shown. Even with all this they still are killed, sometimes several hundred

each year.  Most are very young,  just out of training school.  There is a

moral to all  this "Rookies  and the careless  get hurt".  BE VERY CAREFUL

WHERE CHILDREN ARE INVOLVED IN THE HOME ENVIRONMENT.

The analog  and  contact inputs to  the computer through  the gamecard are

inherently safe.  The highest voltage involved is 5 VDC.  The danger comes

when combining  that  ability with X-10  technology to control  115 or 220

VAC.

PAGE 41

Why did I include this chapter?  Schools, colleges, the formal educational

system doesn't even recognize that  good olde Murphy even exits.  Murphy's

laws are not written by some fat assed politician, nor are they the fruits

of  very  deep  and  learned intellectual  experience,  instead they  have

evolved based on the  experiences of those  poor souls who get dirty doing

the actual work.  Play with personal computer automation very much and you

will get to know Murphy on a first name basis and probably get to meet all

his brothers, nephews and cousins.

MURPHY'S LAWS ACCORDING TO CONTROL THEORY

IN EVERY BIG PROGRAM THERE ARE THREE LITTLE ONES TRYING TO GET OUT. ONE OF

THEM IS FLAWED

CONTROL PROJECTS TAKE ALL THE PARTS YOU HAVE, PLUS ONE MORE.

ALL CONTROL PROJECTS TAKE TWICE THE TIME ALLOCATED.

THE FIRST INSTRUCTION DELETED WAS THE CORRECT ONE.

ALL MECHANICAL DESIGNS ARE FLAWED.

ALL BIG FLAWS ARE FOUND AFTER MIDNIGHT.

MOTHER NATURE REALLY IS A BITCH.

ALL MASTER PLANS ARE DOOMED TO FAIL

IF ITS FIXED, YOU HAVE TO PLAY WITH IT UNTIL, YOU BREAK IT.  IF ITS BROKE,

YOU HAVE TO PLAY WITH IT UNTIL ITS FIXED.

IF THE FLAW IS IN A SENSOR, IT WILL BE FOUND IN SOFTWARE.

THE ONLY GOOD CONTROL SYSTEM IS THE ONE THAT WAS JUST REPLACED.

MOTHER NATURE REALLY DOES SIDE WITH THE HIDDEN FLAW.

THE LAST PART CHECKED IS THE BAD ONE.

PAGE 42

THE WORK OF ALL JURY RIG EXPERTS LASTS TILL ONE DAY AFTER THEY QUIT.

ALL BREAKS OCCUR IN THE WRONG END OF WIRE.

NO PROGRAM  IS  ANY GOOD  UNTIL CHANGED  TWENTY TIMES.  A PROGRAM  CHANGED

TWENTY TIMES IS USELESS.

IF A PROGRAM IS USELESS IT WILL HAVE TO BE DOCUMENTED.

THE INSTRUCTIONS ARE NEVER READ. UNTIL THE EQUIPMENT IS DAMAGED.

NO FLAW IS EVER FOUND BY LOOKING FOR IT DIRECTLY.

DATA USEFULNESS IS INVERSELY PROPORTIONAL TO THE AMOUNT OF IT.

ALL BAD DESIGNS CAN BE FIXED BY MORE LINES OF PROGRAM

THE OBSCURE ROUTINE NOT THOROUGHLY CHECKED IS FLAWED.

ALL FLAWS ARE CAUSED BY OBSCURE ROUTINES.

IF IT DON'T WORK, MORE DATA WILL HAVE TO BE COLLECTED.

WHEN ANY CONTROL SYSTEM IS FULLY UNDERSTOOD IT IS OBSOLETE.

ALL CONTROL PROGRAM SCAN PROBLEMS ARE CAUSED BY GLITCHES

THINGS DO GO BUMP IN THE NIGHT

BUMPS IN THE NIGHT ARE CAUSED BY THINGS

A PERSON DOING KNOWS NOTHING. A PERSON WATCHING KNOWS ALL.

ALL SIMPLE PROBLEMS ARE COMPLEX. ALL COMPLEX PROBLEMS ARE SIMPLE.

COMPUTER MALFUNCTIONS ARE DIRECTLY PROPORTIONAL TO  THE AMOUNT OF PAPER IN

THE PRINTER BOX.

PAGE 43

THE WORTH OF ANYTHING IS DIRECTLY INVERSELY PROPORTIONAL TO ITS COST.

BIG TALKERS, DO SMALL, SMALL TALKERS, DO BIG

ALL PARTS THAT CAN NOT BE LOCATED ARE WORTHLESS.

ALL PLANS OR PROGRAMS ARE FLAWED, BE THEY OF MICE OR MEM.

IF THE COMPUTER IS BIG ENOUGH, ALL PROBLEMS CAN BE SOLVED.

THE AMOUNT OF  FLAWS DISCOVERED IS  DIRECTLY PROPORTIONAL TO THE AMOUNT OF

EQUIPMENT DAMAGE.

ALL CHECKS IN THE MAIL AND ALL PARTS SHIPPED TODAY TAKE SIX TO EIGHT WEEKS

TO ARRIVE.

THE PART NOT ORDERED, IS THE CRITICAL ONE.

PAGE 44

PROBLEM AREAS AND LIMITATIONS

X-10 technology  has some  bugs in it.  Basically the  modules communicate

over the wires that power the house,  depending on how your house is wired

will determine if you have problems or not.

Some of these problems can be very annoying.  In the registered version is

a much  more  detail discussion  with  the problem areas  fully explained,

solutions offered and  various situations  that different user  might face

are covered.

In  addition  all  problems with  equipment,  the best  equipment  for the

application and all problem areas/equipment  difference and other areas of

difficulty encountered during  the development  of AUTOMATE are discussed.

These  discussions  and  information  alone  are  worth the  price  of the

registered version of AUTOMATE. Some of these areas can rapidly exceed the

ability of the average user to find or understand/correct the problem.

PAGE 45

WHAT DO I GET IF I REGISTER ?????????

1. First of all my humble gratitude and thanks for rewarding my efforts.

Second a completely  different program and  instructions for doing all the

things discussed and demonstrated in the introductory shareware version.

2.  Construction details on how  to build sensors,  calibration techniques

and curve generators.

A look at the  in/outs of designing  sensors and trimming  programs to use

those sensors.

Discussions  on  methods  of  using  graphic  displays  to  show  off your

control/monitoring handiwork.

A complete example of how to build,  test,  calibrate and program a system

to monitor analog real world parameters (visible light and temperature).

The data sets  for four temperature  probes referenced to  their part no.s

and information on suppliers and design of those sensors.

Example of how to  use that system  for parameter display  on the computer

screen, data logging and setpoints generation for control applications.

A complete discussion on inputing contact information via the gamecard. In

addition a input expansion method to increase the number allowable.

The only  complete  engineering based  program  on the market  for using a

personal computer for monitoring, automation and control.

3.  Construction details on  building a connection  box for the game port.

The box  design  takes into  consideration the identification  of gamecard

channels, programming considerations, acts as prototyping testbed to allow

development  and  calibration  of  sensors  and allows  for  connection of

sensors to the gamecard for monitoring or control applications.  Walks you

through the calibration and testing of the gamecard for monitoring/control

applications and has automatic characteristic curve generator.

4.  A photo  of the  completed connection box,  other useful equipment and

several prototype sensors.

5.  Information  on  sensors  useful  in  home  applications.  Model No.s,

approximate price, where to buy and how to use them.

6. List of suppliers for X-10 technology equipment.

7. List of suppliers for electronic and other hardware.

8. Calibration details and examples of how to do it.

9.  Sample  programs  for monitoring  and data  logging.  (source code  is

PAGE 46

included for all program examples.).  Also included is a  mail box monitor

program. No more that mail person sneaking up on you.

The  lastest  additions  to  the   registered  version.  That  program  is

constantly being revised  and additions made.  The total program is 700 K+

and growing.

10. Tips and hints on designing control programs.

11.  A discussion  on  X-10 technology  and  how to communicate  with X-10

modules.  Included  is  a  communication  program  for  use  with AUTOMATE

monitoring and control programs.  A sample program using AUTOMATE and X-10

modules  together.  Information  on  buying  the  best X-10  interface and

communications software.  Discussions on the problem areas  and how to fix

them.

12.  A completely different program with more  in depth discussion in many

areas.  The  registered  version  IS  NOT  a  warmed over  version  of the

shareware version.  All redundant files of the  shareware version has been

deleted.

13.  Full disclosure  of all  methods and files.  All compiled  files have

their source  code included.  As time  goes by  the Registered  Version is

being updated and compiled more with menu driven instructions.  Where this

occurs source  code  will be furnished  of the working  parts and methods.

Nothing is "hidden".

14.  The only limitations to the purchaser are: The purchaser may not seek

to duplicate the intent, methods or manner of AUTOMATE so as to supersede,

or directly compete with  AUTOMATE in a  commercial manner or in any other

manner render  AUTOMATE obsolete.  The purchaser  shall have  the right to

develop any other application not contained in AUTOMATE for his own use or

sale. Registration is NOT a license for exclusive use.

No  restrictions  shall  be recognized  due to  the  purchasers copyright,

patent or  any  other legal protection,  where the source  is derived from

AUTOMATE. A reference to the source material is required. You CANNOT COPY,

USE OR INCLUDE AUTOMATE to complete your program. No bundling or attaching

of files,  whole or in part.  The use permission is  intended to allow for

new product development. It is not a use through attachment permission.

These restrictions are  basically for use  of AUTOMATE in  producing other

software for  sale or  distribution to others.  For registered users using

for their own use there are no restrictions.

A  bit  of explanation.  If you  buy a  software programming  package like

MicroSoft's QuickBasic, you can use that product to produce other software

to sell to  the general public.  But you  can not  use that product source

code, if you could get your hands on it, to produce a competing product to

QuickBasic.

I do something virtually no one else does. I give you the source code with

explanation to  all working  parts of AUTOMATE.  The reason is simple,  it

allows that  registered  user to modify  the programs to  make that custom

application exactly  like  they wanted.  It also helps  in producing other

related software.

PAGE 47

The results  of using  AUTOMATE are  usable  in your programs  for sale or

further distribution to others. Bundling, copying and wholesale direct use

of my source  code is not.  I have the ability to  break into any program,

compiled or not  to determine  the source  code used.  If I find  you have

reused my  source code  in a  wholesale  direct manner  and  then sold  or

redistributed  those  results,  you  have  violated  your  license to  use

AUTOMATE or the TOOL-BOX. None of my software is in the public domain. All

of the software is Copyrighted.

15.  Finally full  rights  to chop it  up and use  any part,  whole files,

develop  any  application   (software  and   hardware)  and   include  any

information,  in whole or  in part in  your application without any fee or

liability.  All ideas expressed in the registered version may be developed

free of any liability  or fee in so  far as they are  not in conflict with

paragraph 14.

This right for use is intended to allow the user to develop new, novel and

useful  products  where  the  results  of  development of  the  results of

AUTOMATE are integrated into  that product development.  Simple attachment

or copying of methods, techniques, files whole or in part as to add value,

enhance or  complete  a user's  application  for sale  shall  be viewed an

infringement of the right to use permission.

These two paragraphs are  not in conflict.  Basically they are  saying you

can use  it for  other honest development  work but  you cannot  use it to

build a  program for  distribution or  sale  that is  like or  of the same

purpose  or  competes  in  a  commercial  manner.  For  your  own  use  no

restrictions.

16.  All registered  users  MUST acknowledge  in writing (included  in the

order form)  their liability  for further  distribution of  the registered

version without financial remuneration to the author.  No purchase will be

accepted without the acknowledgement.

Purchasers shall fully  accept responsibility  for ensuring the registered

version is not placed on electronic bulletin  boards or any other means of

mass duplication.  An understanding shall exist that registered copies may

be "marked" by  an identification means  to identify the registered owner.

Registered users are free to make an unlimited number of copies for use by

the registered owner on as many systems as he owns or controls.

If the owner is not an individual, a site registration is required and the

number of users is restricted to 30 individuals per site.  Each individual

owner is restricted to his exclusive use only without a site registration.

Transfer of ownership of  AUTOMATE through sale,  gift or any  other means

shall not be recognized by the author.  Transfer of ownership by any means

shall be viewed as illegal further distribution of AUTOMATE.

PAGE 48

THE AUTOMATE TOOL-BOX

First what is the AUTOMATE TOOL-BOX? The TOOL-BOX is a software collection

of modular control routines, graphic displays and stand alone applications

programs for use with the AUTOMATE development package.

The TOOL-BOX starts to explore where the limits of the AUTOMATE techniques

are. Any application requiring very high speed reading of the inputs start

to test the limits of this type application.

These modular programs  can be plugged  in or quickly edited to accomplish

the features desired. Working examples of the techniques are presented and

a tutor section is included to aid in learning the techniques.

The AUTOMATE  TOOL-BOX  is a commercial  program and is  only available to

registered users  of AUTOMATE.  The TOOL-BOX is written  in QuickBasic 4.5

modules.

All program modules have the source code  included and have a written text

description. An overall instruction manual is included.

The modules are of two general varieties:

1.  Graphic  displays  -  Kind  of like  clip art.  On screen  displays of

measured parameters,  designs for meters,  digital displays, alarm panels,

panel  layouts  with  indicator  status  lights,  analog displays,  system

status,  etc.  A general  text discussion  on producing these displays,  a

sample menuing program and a manual  section with very basic discussion on

programming for the graphics mode.

2.  Small stand alone complete programs for particular applications. These

programs may  be  combined with control  routines and graphic  displays to

produce  very  large  custom  home  automation  systems.  Some application

programs are  infrared  and radio communications,  remote pushbutton input

stations,  clock and calendar display,  strip chart plotting program, etc.

Each program has a manual text section,  the source code is included and a

bill of material and construction details if hardware is involved.

The TOOL-BOX  is  designed for use  by the programming  and nonprogramming

user alike.

PAGE 49

QUESTIONS

All software should have a "QUESTIONS" chapter.  O K, I've read it all but

I still have  these questions!!  So,  here is my questions  chapter of all

those things I didn't get around to answering.

QUESTION:

Do you sell the hardware, I don't want to build my own?

ANSWER

Maybe,  if there is  enough demand.  I'll probably stock  the sensor parts

that  are  the  more  difficult ones  to obtain.  As for  the  other small

construction projects required  at this time,  I don't want  to build them

but if someone out there does I'll try to cooperate.  Nothing I present in

the registered version is difficult to build.  Plus everything has a parts

list and a supplier referenced  to obtain those parts.  Most come from one

source and I  even give  you the catalog numbers.  To get you  through the

project,  I do a lot  of "hand holding".  The general tone does not assume

that you are  an experienced in  electronics or computers and explains all

steps as they are accomplished. Where I know small differences exists from

computer to  computer,  the differences  are  pointed out,  with suggested

solutions.  I've also picked  over the suppliers  and only list those that

give results, plus I kind of rate them and point out some of the flaws.

QUESTION

Yeah,  but I don't know  anything about electronics,  I'm just the average

computer wizard and I  always thought an  engineer was the guy who drove a

train?  Plus I failed differential inverted  calculus and can't even spell

lapasal transform or was it laplase transform, anyhow you get my drift?

ANSWER

Great,  you  don't  have  to  know  electronics.  The electronics  part is

presented for dummies.  I "lead you  through the  woods" in the electronic

and math areas.  You can get  working applications  by just "following the

lead". In the areas of programing or math the examples are fully explained

in a  prior  discussion to  doing  that part  but  the actual  application

example to produce a working application is fully worked out.  The results

are used  "cookbook style".  For the  "rocket scientists"  and math expert

enough back ground is presented to let  you wander the woods for all sorts

of goodies. The required math is on the level of balancing your checkbook.

The more your knowledge base, the more you can do.

QUESTION

So,  if I can do automation  with the X-10 technology equipment,  why do I

need AUTOMATE,  why not  just  buy off the  shelf equipment and  do really

great systems  that will  really impress  all  my friends and  about to be

PAGE 50

friends?

ANSWER

Thats just it friend,  with the equipment bought off the shelf, you aren't

even going to impress yourself.  The "off the  shelf" automation equipment

is basically based on time.  All really good automation  type systems will

require a good source of independent information for decision making. Time

is  only  one  of  those  good  sources  of  information.  A  lot  can  be

accomplished  based  on  time.  But  the  limits  appear  rather  quickly.

Information like visible light level,  temperature and others  are what is

really required to automate  most desired functions.  AUTOMATE is the link

to allow  development of  that desired  information.  AUTOMATE allows  for

complete data logging,  automation systems based on information other than

time and the ability to monitor and display the results of automation, not

just  another  "dumb"  system  blindly doing  its thing.  AUTOMATE is  NOT

ANOTHER PRECANNED solution to the world's problems.

AUTOMATE is NOT another magic solution.  It requires thought and some work

on the users part. The flip side of the coin is it also allows the user to

develop  powerful  automation  systems for  their use  or  for sale.  Plus

AUTOMATE is the only really low cost, effective and workable system on the

market for analog  and contact  control/monitoring that offers  a complete

and fully implemented method for achieving results.  AUTOMATE is not about

selling you  hardware  and a  "pig-in-the-poke"  but is  about  a planned,

method based  approach  to control/monitoring  using a  personal computer.

Plus it  gets you  involved with the application,  don't believe any other

promises, your application with differ from the next guys, the trick is to

understand  the  basics  and  background  to  handle  your  own particular

application.  AUTOMATE is not an experiment type based program.  The basis

is engineering methods.  The sensor data is  developed from manufacturer's

data.

QUESTION:

Is there any good reference book for a beginner programming type who would

like to try but finds all those IBM manuals makes mush out of the brain?

ANSWER

Get the  ACORN  instruction manual  (lot  of grade school  and junior high

schools used that computer) very good entry level introduction.  For color

and graphics beginning reference I highly recommend Carl Shipman's "How to

program your IBM PC - Color & Graphics". Ask for it at any good bookstore.

If  they  don't  have  it,  ask  them  to  order  it.  The fancy  standard

identification number  (most  USA books  now  have a unique  standard type

number that identifies that book), kind of like "dog tags for books".  The

number is "ISBN 0-89586-256-4".  Using those references  and an occasional

foray into the IBM  BASIC manual you  can under stand anything in the demo

programs.  Using precanned programs to do things  you do not understand is

asking for trouble.  If you get stuck,  ask your kids,  if your a kid, ask

your parents.  Anyway how are you ever really  going to be able to explain

it to the insurance company if you burn the house down.

QUESTION

PAGE 51

Do you want to go to work for me?

ANSWER - No

QUESTION

I have a brilliant idea or a problem, can I call you.

ANSWER

Yes,  but please  not till  after 12 noon  EST and not after  8 PM EST.  I

prefer most things in writing. You have to think it out better.

QUESTION

Is this computer control and automation dangerous?

ANSWER

Yes,  if  you  are careless.  The X-10  technology  control involves  high

voltage,  such as  110 VAC  and 220 VAC.  If you stick your  tongue into a

regular wall  power receptacle  you will get hurt.  Computer control is an

extension of  the  safety and  good housekeeping  practices  that everyone

should follow around the home without any computers involved. There is "NO

MAGIC", if you do stupid things, no computer is going to protect you.  The

analog and control  monitoring that I  talk about is inherently safe.  The

highest voltage present if you do things correctly is 5 VDC.  If I make no

other impression or get  no other point across,  KNOW WHAT YOU DOING - the

home is probably  the most unsafe  place we are exposed too.  USE THE SAME

CARE WITH COMPUTER AUTOMATION THAT YOU  USE WITH ANY HIGH VOLTAGE,  MOVING

PARTS OR OTHER HOME HAZARDS.

Can money be made from the automation/control/monitoring applications?

ANSWER

Lord,  Yes,  The home market is probably  saturated with data bases,  word

processors, games, etc.  Automation labs are fun.  It requires a different

kind of computer thinking.  The world is full of one function,  don't hand

me that type screwdriver (I only do Phillip head screws) type folks.  Good

automation  requires  general  practitioners,  those with  skills  in many

different areas and the ability to see  the solution and then stay with it

through the final implementation.  Those folks who can cut the mustard and

are organized and can count  on a ready market.  It is there,  you have to

find it and then serve a niche.

QUESTION

What if I try it but can't understand or am just turned off?

ANSWER

The general field of  automation/control/monitoring is huge.  I have found

you start in one  area and your interests  just takes you from one area to

PAGE 52

another. It is easy to get "hooked". Unlike most computer stuff, where its

all pre-  canned and  you feel like a  fool for not being  able to see the

solution. Computer control and automation is not precanned and you have to

get involved with your particular application.  Once you acquire the basic

working  knowledge  its  very  difficult  not  to  like  it.  There is  no

right/wrong type thinking.  There are many different solutions.  I found I

will start with a preconceived idea about  something and find I'm all wet,

halfway through the problem.

QUESTION

I have seen all sorts of magazine articles, ads and that sort of thing, is

this just another come on,  long on selling,  long on promises  and then I

won't be able to get it to work?

ANSWER

There is  going  to be  a  certain amount of  that hype.  Anyone promising

"magic solutions"  with  no basic  computer knowledge  or  "miracles" with

precanned software is probably leading you down the primrose path.  A good

solid knowledge of computer basics,  a little programming knowledge,  some

curiosity,  good  organization,  common  sense  and  some  stick-to-it  is

required for  success.  If your  the  type person  who can be  found going

sixteen ways all at  once or expects  instant success based on a very thin

knowledge base your  in for trouble.  In addition you can  get hurt or get

others hurt.

PAGE 53

QUESTION

Isn't all this very expensive?

ANSWER

No, just the opposite. You can build most hardware for sensors and control

devices. There are new small, cheap computers (just buy the box), get used

monitors, printers, keyboards and hang a system together. You don't need a

hard disk, the number of ports (com, printer, game, external devices), the

amount of  RAM,  things like  that  are more important.  A one  disk drive

machine is fine.  Most can be  put together  by going to  salvage computer

companies. I bought a new keyboard and a very expensive tilt monitor (with

small burnt areas on  the screen - which  has no effect on  what I want to

do)  for  $50.  Used  all  the  extra  equipment  that  was  laying around

collecting dust.  In all for a 1 Meg RAM,  286 AT, one disk drive, w/2 com

ports,  parallel printer port,  external drive port, game port, monochrome

monitor,  joystick,  connector box, C-Itoh 8510 printer, electronic parts,

new digital  auto  ranging multimeter  final  cost around  $600.00  and my

regular make-the-money computer is not  at risk at all.  I have used parts

out of  my "junk  box" that I have  never expected to ever  have a use for

again.  You don't need super "high tech" to be effective. Moreover its the

one area of  personal computers  where if you want,  you can have complete

control over developing YOUR SYSTEM.  It may not be the best, may not even

work very  well,  but it  is YOURS.  With a little  skill and time,  it is

possible to build a very nice system.

Plus the  X-10 stuff  is reasonable and  can be built  up in phases.  Also

using the contact  input scheme given  in the registered  version bypasses

the most expensive modules.

QUESTION

Yeah,  but  is  it  fun?  I'm up  to HERE  with computer  stuff.  All that

databases, inputs demands, cross-inverted double whammy eat the dots games

until I could just bust.  It has left me badly  burnt-out on computers and

just plain  jaded.  I just  think I've  seen it all.  Nothing turns  me on

anymore !!!

ANSWER

Pal,  the only thing more fun is the monkey cage at the zoo around feeding

time when they get real frisky.  As for the jaded part,  you're gonna have

to see some sort of a therapist or shrink for the rest of your problem.