RS232 Communications Driver

Original Author: Greg Haley, Texas Instruments

Version 3.00 by

Joe McDaniel

Fein-Marquart Associates, Inc.

7215 York Road

Baltimore, MD 21212

(301) 821-5980

History:

The original work on this package was done by Greg Haley

of Texas Instruments. The system I took and modified had severe problems

in that it could support input, but output didn't work. Also it did

not support interrupt driven output, only input. The original version

also had support for (I assume) debugging aids which allowed for echoing

of characters received and transmitted and for direct keyboard input.

None of these features is currently present - I felt they were of no

value and just kludged up the code. Xon/Xoff now works. RS232.COM now reports

on the status of the control lines. I eliminated the TI specific stuff as I had

no way to test it. Also, the original distribution had a terminal emulator

that took advantage of the echoing features - it is now gone, too. I fixed

the 19,200 baud stuff so it will work.

Introduction

The purpose of this package is to provide RS232 communications support

in device driver form for the IBM PC. All machine dependent code is confined

to the device drivers. The device drivers in this package use the

MS-DOS I/O control channel to configure the driver and to check or change the

modem RS232 signals. Some of the features of the drivers used here that make

them useful for communications are:

o Interrupt driven communications

o Optional flow control

o Access to the modem signals

o Ability to change communication settings without disconnecting

o Ability to force a disconnect by dropping RTS and DTR

o 110 - 19200 baud rates

o EVEN, ODD, or no parity using 7 or 8 data bits

o Device driver is configured using standard MS-DOS function calls

o Access the number of characters in the receive queue

o Ability to purge the receive queue

These drivers support all the major needs of any communications program,

and are a substantial improvement over most of the default device drivers

that come standard with each MS-DOS computer.

Using these drivers, any language that can write to the device "AUX" can

now do interrupt driven RS232 I/O. If the language is not capable of issuing

the MS-DOS functions call 44 hex (I/O control) to configure the driver, the

assembly program RS232 is provided to set up the port, baud rate, parity,

etc. for the device driver. The source code for this program is provided as

an example of using the MS-DOS I/O control function call.

Here's a list of the files in this package with a brief description of each

file:

IBMRS232.ASM    Source for IBM PC machine dependent code

RS232.ASM       Source for RS232 configuration program

RS232.DOC       Documentation for this package

RS232.INC       Source for general device driver routines

Assembling the routines

Edit IBMRS232.ASM ans RS232.ASM to set the name of the device being created.

It is currently set to AUX. Assemble as follows:

masm ibmrs232;

masm rs232;

Link:

link ibmrs232;

exe2bin ibmrs232.exe auxdvr.sys

link rs232;

exe2bin rs232.exe rs232.com

Installing the Device Driver

Installing the device driver of your choice is done by editing your

CONFIG.SYS file and putting a copy of the driver in your root directory.

As an example, to install the IBM driver on drive C:, first copy the file

AUXDVR.SYS to someplace where you can access it during boot such as

C:\AUXDVR.SYS. Next, add the following statement to your CONFIG.SYS file:

DEVICE = AUXDVR.SYS

The next time you reboot, the driver will be installed and will take the

device name "AUX". The driver will not respond until you configure it by

writing the Device Configuration Word (DCW) to the I/O control channel.

This is accomplished using the MS-DOS function call 44h (hex) and is covered

in the next section.

Using the I/O Control Function Call

All I/O control to the device driver is done in one or two byte forms.

The two byte form is used to set up the communications paramenters. The

single byte form is used to select all other options such as checking the

modem signals. We'll cover the single byte form first. Reading one byte

from the I/O control channel causes the driver return the current modem

signals and I/O configuration. (See function 44h, AL=2 in your MS-DOS manual)

The modem signals and options are encoded as follows:

Bit number

+-------------------------------+

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

|-------------------------------|

|   |   |   | RI|SCF|DSR|CTS|DCD|

+-------------------------------+

Bit   Description

4    0 = RI (Ring Indicator) is down.

1 = RI (Ring Indicator) is up.

3    0 = SCF (High speed line) is down.

1 = SCF (High speed line) is up.

* The signal SCF is not supported by the IBM PC.

For the IBM PC, this signal will always be down (0).

2    0 = DSR (Data Set Ready) is down.

1 = DSR (Data Set Ready) is up.

1    0 = CTS (Clear to Send) is down.

1 = CTS (Clear to Send) is up.

0    0 = DCD (Data Carrier Detect) is down.

1 = DCD (Data Carrier Detect) is up.

Writing a single byte to the I/O control channel (function 44h, AL=3)

is used to set the driver options, flush the receive buffer, and disconnect

(by dropping RTS and DTR). Below is a chart showing the various values for

the byte code and the action taken:

Bit number

+-------------------------------+

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

|-------------------------------|

|   |   |   |   |   |LEN|FIB|DTR|

+-------------------------------+

Name    Bit     Description

LEN      2      0 = Output DCW on 2 byte I/O channel reads. (Default)

1 = Output receive queue length on 2 byte I/O channel reads.

* Note that this field need only be set once. The current

state is used until LEN is changed again.

FIB      1      0 = Don't flush receive buffer

1 = Flush receive buffer

DTR      0      0 = Disconnect (drop DTR and RTS) and reset driver.

1 = DTR and RTS up.

N.B.: The read byte and write byte have no relation to each other! If you

read, do not write the read value. Set your own parameters!

The two byte form of I/O control can be used to read or write the

communications settings. Writing a 2 byte value to the control channel

will set the Device Configuration Word (DCW). Reading 2 bytes from the

control channel will do one of two things depending on the state of the

LEN bit. (The LEN bit is set by the user in the single byte control channel

write.) If LEN = 0, the DCW will be read. If LEN = 1, the receive buffer

length is read. The default mode is LEN = 0, read the DCW. Below is a bit

map showing the various fields within the Device Control Word:

+-------------------------------+-------------------------------+

|           Upper Byte          |           Lower Byte          |

+-------------------------------+-------------------------------+

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

+-------------------------------+-------------------------------+

| 1 | 2 | 0 | 3 | 3 | 0 | 4 | 4 | 5 | 5 | 5 | 5 | 6 | 7 | 8 | 8 |

+-------------------------------+-------------------------------+

|                           Bit Codes                           |

+-------------------------------+-------------------------------+

Bit Code        Description

2            Number of data bits

0 = 7 data bits

1 = 8 data bits

3            Port Number

00 = Port 1

01 = Port 2

10 = Port 3

11 = Port 4

4            Flow Control

00 = none

01 = SCF     (NONE for IBM PC)

10 = DSR

11 = XON-XOFF

5            Baud Rate

0000 = 110

0001 = 150

0010 = 300

0011 = 600

0100 = 1200

0101 = 2400

0110 = 4800

0111 = 9600

1000 = 19200

6            Number of Stop Bits

0 = 1

1 = 2

8            Parity

00 = none

01 = ODD

10 = none

11 = EVEN

Writing a New Driver

Additional RS232 drivers may be written for other MS-DOS (2.xx and greater)

machines by changing the machine dependent portion of the driver. The two

driver provided has machine dependent code in IBMRS232.ASM. The file RS232.INC

is an include file and provides the general device driver functions. Any general

changes to the driver should be made here. To make a new driver, only the

machine dependent portion needs to be written, and any new driver should also

use the INCLUDE directive to add the code from RS232.INC. Following this form

will make maintaining the various drivers much easier.