Data Sync Engineering

40 Trinity Street

Newton, NJ  07860

USA

TEL: (201) 383-1355

FAX: (201) 383-9382

Internet: http://datasynceng.com

Demonstration Version

for the XDASM

Cross-Disassembler

XDEMO

The following tutorial will guide you through the disassembly

procedure of an 8051 type program. Certain restrictions apply

to this demo version;

o  Limited file size disassembly, (0000H to 01FFH).

o  Reads Intel hex records only.

o  Does not support the HEX Offset TAG command.

o  Does not support the alternate HEX format ($00).

o  Contains only two CPU tables, 8051 and 6805.

o  The 8051 table source (8051.XTS) is provided only as an

example to see how tables are written. A table compiler

is supplied with XDASM for creating the .CPU image.

-----------------------------------------------------------------

-----------------------------------------------------------------

!! Order XDASM Today !!

You may purchase the full version XDASM directly from us.

We also have a companion Universal Macro-Assembler, Cross-32.

This universal assembler readily accept XDASM's assembly source

file. The ORDER.DOC file lists the features for each product.

XDEMO  CROSS-DISASSEMBLER User's Guide

Table of Contents

GENERAL

General Description . . . . . . . . . . . . . . . . . 1

System Requirements . . . . . . . . . . . . . . . . . 1

USING THE XDEMO DISASSEMBLER

Starting XDEMO  . . . . . . . . . . . . . . . . . . . 2

XDEMO Options . . . . . . . . . . . . . . . . . . . . 3

Tag Files . . . . . . . . . . . . . . . . . . . . . . 4

Operand Text Files  . . . . . . . . . . . . . . . . . 5

Conventional Assembler Format . . . . . . . . . . . . 6

Output File Description . . . . . . . . . . . . . . . 7

Unresolved References . . . . . . . . . . . . . . . . 8

Cross Reference Lists . . . . . . . . . . . . . . . . 8

Altering The Disassembler Format  . . . . . . . . . . 9

A Typical Disassembly Procedure . . . . . . . . . . . 10

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 1

GENERAL DESCRIPTION

The XDEMO is a table based cross-disassembly tool. "Cross"

meaning that while running on one type of a computer, it

disassembles code for another.  In many ways, the XDEMO is

similar to a debugger with the exception that it produces a

source program code that is suitable for an assembler.

Features, such as, Instruction line HEXDUMP, Automatic Label

assignment, and various Cross-reference lists greatly aid in the

complicated task of reconstructing or debugging a program.

A separate control file, called a TAG file, allows the user to

mark (or tag) specific program areas for a particular type of

disassembly.

Another file, called a Format file, configures XDEMO's Source

Output and can be modified to match the requirements of your

favorite assembler.

Some CPU tables utilize external text files which are used to

substitute specific addresses with symbolic names. These text

files are user modifiable and enable XDEMO to be "tailored" or

"adapted" to other members of that processor's family.

SYSTEM REQUIREMENTS

XDEMO operates under MS-DOS 2.0 or later and uses about 20K of

memory. An additional 390K of memory is allocated for cross-

references and table storage. A typical system requirement would

be about 512K bytes of memory.

XDEMO produces a disassembled program with cross-references

in three passes. Pass #1 reads the input file and tabulates

labels and references. Pass #2 rereads the input file and

begins writing each line of program code. Pass #3, which is

optionally the reference generation pass, writes out the

sorted cross-reference lists.

An 8K Binary file, typically 23K in HEX format, may produce

a disassembled source file of approximately 173K. While it is

possible to run XDEMO on a floppy disk, a hard disk would be

recommended for speed considerations.

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 2

STARTING XDEMO

Starting XDEMO requires specifying the input file, the CPU

type and any of the disassembly options. The command form is:

XDEMO filename,type [/options]

The FILENAME is the name of the input file that is to be dis-

assembled. If the input file resides on a different drive or

path, it must be specified as part of the filename.

The cpu TYPE, preceded by a comma, usually resides on the same

drive and path as XDEMO. There are two control files for each cpu

type; the Disassembly Table, identified as "type.CPU", and the

Disassembly Format, identified as "type.FMT. The Disassembly Format

file is used to configure XDEMO's output for the Assembler being

used.

The disassembly OPTIONS are single character commands that are used

to select various listing formats and disassembler functions. The

option characters may be grouped together and must be preceded by

the forward slash (/).

For example, the user wants to disassemble an 8051 type program,

called TEST.HEX, and would like to include all listing features...

The command line would then be:

XDEMO TEST.HEX,8051 /CR

When XDEMO is started, the following messages would appear:

......................................................................

.                                                                    .

. XDEMO - Table Based Cross-Disassembler    Demo Version   PC/MS-DOS .

. Copyright (C) 1990-1996 Data Sync Engineering   All rights reserved.

.                                                                    .

. No Tag File                                                        .

. Starting PASS Number 1  -  Processing: xxxx                        .

. Starting PASS Number 2  -  Processing: xxxx                        .

. Starting PASS Number 3  -  Xref lists: xxxx                        .

.                                                                    .

.      x Unassigned Opcodes                                          .

.      x Unresolved References                                       .

.                                                                    .

. Disassembly Complete  --  Source File Created.                     .

......................................................................

A new file called TEST.SRC has now been created.

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 3

XDEMO OPTIONS

The XDEMO contains powerful disassembly options that provide a more

complete detail on the program that is being disassembled. Option

characters are preceded by a slash (/) and several characters may be

grouped together  (ie. XDEMO TEST.HEX,8051 /LCTR).

Option Summary:

Option

letter                        Result

------    --------------------------------------------------------

C       Append Comment Field Hexdump to each instruction line.

L       Converts output assembly code to lower case characters.

M       Mask 7-bits for ASCII Hexdump within comment fields.

R       Append Cross-reference lists to end of source file.

T       Include Tag File disassembly parameters.

X       Write Cross-Reference lists only, no assembly code.

COMMENT FIELD HEXDUMP

The Comment Field Hexdump is appended to each line containing a

disassembled instruction. It shows the Program Counters value, the

HEX bytes utilized by the current instruction and the displayable

ASCII character equivalent of the Hex bytes. Non-displayable

characters, such as control characters, are shown as periods ".".

CROSS-REFERENCE LISTS

The Cross-reference lists produced by the XDEMO, map out all

Program Code and Memory usages. The listing shows the Label or

Memory address followed by all locations that refer to that

address. The referencing locations are identified by the Program

Counter value as shown in the comment field hexdump.

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 4

TAG FILES

The Tag file is used to add a more direct control over the disassembly

process. It instructs XDEMO as to what type of disassembly to switch

to at what specified address. This file can be created with any ASCII

text editor and is identified with the same filename, as the input

file, but has the file extension of ".TAG".

Entries into the tag file consist of a four character hexadecimal

start address followed by the "=" character then the command

character. Only one command character is permitted on each line for

each designated address. The carat (^) character is used to terminate

the Tag list.

Tag File Command Summary:

Command                          Result

-------       ------------------------------------------------------

B           Define BYTE disassembly  (DB  000H).

G           GENERATE Label Equate for specified address.

I           INSTRUCTION disassembly.

S           SKIP disassembly (ignores marked bytes).

T           TEXT disassembly  (DB  'text').

>           Define WORD, first byte = LSB of word.

<           Define WORD, first byte = MSB of word.

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 5

OPERAND TEXT FILES

Some processor types, such as the 8051 and 6805, contain Special

Function Registers mapped into the Page 0 address range (00H to FFH).

They usually include parallel ports, serial ports, counter/timers,

A/D converters, PWM's and so on. In this case, XDEMO uses external

text files to substitute the Hex addresses (000H or $00) with the

symbolic name for that register. This provides an output listing

more easily readable than if just Hex addresses were shown. These

text files are identified with the same name as the processor type,

but have the file extensions of ".XR1", ".XR2" and ".XR3".

Not all text files may be used for a particular processor type. For

example, the 8051 uses two, one for its BYTE operands (8051.XR1) and

the other for its BIT operands (8051.XR2), whereas, the 6805 only

uses one for its Page 0 registers (6805.XR2).

These text files are easily modified using an ASCII text editor and

allow the symbolic names to be matched to a member of that processor's

family.

Entries within the text files begin with address "00H" and end with

address "FFH" making a total of 256 lines. Each line contains ten

characters, which includes the carriage-return and line-feed. A

symbolic name, which can have up to seven characters, must be

terminated by an equal "=" character. Any character(s) after the

equal character are not inserted into the assembly file.

Whenever the text files are being edited, it is important that each

line contains eight characters plus the carriage-return and line-feed.

It is also recommended that upper-case characters be used since XDEMO

does have a lower-case conversion option. Unassigned entries should

contain its Hex address text, such as 003H or $03.

The following are some examples of text file entries:

6805.XR2           8051.XR1            8051.XR2

--------           --------            --------

00 -      PORTA===           000H====            000H====

01 -      PORTB===           001H====            001H====

02 -      PORTC===           002H====            002H====

03 -      $03=====           003H====            003H====

04 -      DDRA====           004H====            004H====

05 -      DDRB====           005H====            005H====

06 -      DDRC====           006H====            006H====

:           :                  :                   :

:           :                  :                   :

80 -      $80=====           P0======            P0.0====

81 -      $81=====           SP======            P0.1====

82 -      $82=====           DPL=====            P0.2====

83 -      $83=====           DPH=====            P0.3====

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 6

CONVENTIONAL ASSEMBLER FORMAT

All program code is written out using the conventional assembler

format. This format divides each line in the program into four

fields: label, op code, operand and comments.

The LABEL field is used to assign a symbolic name or label to

the location of an instruction, so that it can be referenced by

other instructions in the program. For example, the instruction

JMP L0100 will cause the program counter to be unconditionally

loaded with the memory address 0100H which was assigned to the

label L0100. The instruction at label L0100 will be the next

instruction to be executed after the JUMP (JMP) instruction is

executed. Most instructions will not be labeled, however, if an

instruction is referenced, the label will begin in the leftmost

column of the line and will begin with the character "L". The

body of the label will contain the target address value. The label

is ended with colon ":" and followed by a tab. If no label was

assigned, the label field is skipped using a tab character.

The OP CODE field is mandatory for every line in the program

that contains an instruction. The op code begins in field 2 and is

separated from the label field by a tab character.

The OPERAND field is used to specify data or an address for

instructions that require an operand. The operand begins in field 3

and is separated from the op code by a tab character. The XDEMO

provides three types of operand forms:

000H - 0FFH or $00 - $FF      Hexadecimal notation.

L0000 - LFFFF                 Label names.

ACC, DPL, SBUF                Symbolic names.

The COMMENT field is used to add an explanatory note to a statement

The contents of the comment field are ignored by the assembler. The

text of the comment field will be preceded by the semicolon character

";". Comments may be used alone without being appended to a line

that contains an instruction. XDEMO uses the comment field in one of

three forms:

1.  To identify the address and contents of the current

instruction line.

2.  To deblock program segments for easier interpretation.

3.  To append the various cross-reference lists.

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 7

OUTPUT FILE DESCRIPTION

The initial ORG assembly directive is inserted into the output file

and always starts with address 0000 unless a LOAD ADDRESS offset was

specified in the Tag file. Further ORG directives are automatically

inserted whenever the Hex Load Address becomes non-sequential.

ORG     00000H

;

LJMP    L0100              ;0000  02 01 00        ...

;

ORG     00100H

:03 0000 00 020100 FA

:10 0100 00 90011812010880FEE493A3B400012230 8C

----

|

+---> HEX file Load Address information

When the "C" option is specified in the command line, XDEMO appends

a Comment Field Hexdump to each line that contains a disassembled

instruction. The description of that field is shown below:

L0100:  MOV     DPTR,#00118H       ;0100  90 01 18        ...

LCALL   L0108              ;0103  12 01 08        ...

;

L0106:  SJMP    L0106              ;0106  80 FE           ..

;

L0108:  CLR     A                  ;0108  E4              .

MOVC    A,@A+DPTR          ;0109  93              .

INC     DPTR               ;010A  A3              .

CJNE    A,#000H,L010F      ;010B  B4 00 01        ...

RET                        ;010E  22              "

;

;

L010F:  JNB     TI,L010F           ;010F  30 99 FD        0..

CLR     TI                 ;0112  C2 99           ..

MOV     SBUF,A             ;0114  F5 99           ..

AJMP    L0108              ;0116  21 08           !.

;

ORL     A,058H             ;0118  45 58           EX

AJMP    L024D              ;011A  41 4D           AM

;

JNC     L016A              ;011C  50 4C           PL

ORL     A,000H             ;011E  45 00           E.

-----  --------        ---

|      |             |

Program Counter  <--------------+      |             |

|             |

HEX bytes utilized  <---------+             |

|

ASCII equivalent of HEX bytes  <------+

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 8

UNRESOLVED REFERENCES

If a memory location reference was not found as an instruction address

then XDEMO automatically EQUATES the value to that label and shows it

in the Unresolved Reference list.

Unresolved References are generally caused by the disassembly of ASCII

text characters, Program Tables or by references made to undefined I/O

or memory addresses. These Undefined References can be eliminated by

using the Tag file to mark these Text or Program Tables or to force

XDEMO to Generate an Equate for that Label Address.

;

;       Unresolved Address Reference list

;

;

L016A:          EQU        0016AH

L024D:          EQU        0024DH

CROSS REFERENCE LISTS

When the "R" or "X" option is specified in the command line, XDEMO

appends up to four Cross-reference lists to the end of the source

file. The first list are references to Labels (or memory addresses),

the other three lists will vary according to the CPU type being

disassembled. They may represent 8 or 16 bit constants, memory, I/O

addresses or some other special registers. The LIST TITLE identifies

the type of cross-reference.

;

;

;       Cross-references to  . . .  <-- List Title

;

;

;  0000=  011E

;  0058=  0118

;  0099=  0114

----   ----

|     |

|     +--------> Program addresses generating the reference.

|

+---------> Referenced Address or Value.

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 9

ALTERING THE DISASSEMBLER FORMAT

Many types of assemblers require different variations of the Source

file syntax. For example, one asssembler may use the directive "DB"

to define a byte and the other may use "DFB". To overcome these

variations, XDEMO utilizes a Format file to configure the Directives

and Delimiters used for its output source file. The format file also

allows up to two lines of general text or assembly statements, to be

inserted into the source file. Each line in the format file configures

a different parameter or directive and may be altered using an ASCII

text editor. Format files have the same name as the cpu type but have

the extension of (.FMT). This enables XDEMO to have different formats

for each microprocessor. Listed below are the line descriptions and

their default settings:

Line  Max                                                 Current

#    Size         Description                            Default

--------------------------------------------------------------------

1    128     Assembler Directive Line #1                 ;

2    128     Assembler Directive Line #2                 ;

3    8       Define Byte Directive for Hex bytes         DB

4    8       Define Byte Directive for ASCII Text        DB

5    8       Define Word Directive (first byte = MSB)    DWH

6    8       Define Word Directive (first byte = LSB)    DWL

7    8       Origin Directive                            ORG

8    8       Equate Directive                            EQU

9    8       End Directive                               END

10   3       Output Source File Extension                SRC

11   1       Start Of Label character                    L

12   1       End Of Label character                      :

13   1       End Of Equate Label character               :

14   1       Comment Field Delimiter character           ;

15   1       Text Delimiter characters                   "

16   1       Hexadecimal format; 0 = 0xxH, 1 = $xx       0

17  The Carat (^) signifies the end of the format list   ^

--------------------------------------------------------------------

Each line entry is terminated with a carriage-return character. If

the carriage-return were the only character on a line, then that

directive or delimiter will not be written. For example, some

assemblers do not require colon (:) characters at the end of label

names, so line # 12 and possibly # 13 will contain only the carriage-

return character.

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 10

A TYPICAL DISASSEMBLY PROCEDURE

STEP 1 -  First we will disassemble TEST.HEX using the Hexdump

and Cross-reference options.

XDEMO TEST.HEX,8051 /RC

......................................................................

.                                                                    .

. XDEMO - Table Based Cross-Disassembler    Demo Version   PC/MS-DOS .

. Copyright (C) 1990-1996 Data Sync Engineering   All rights reserved.

.                                                                    .

. No Tag File                                                        .

. Starting PASS Number 1  -  Processing: 0110                        .

. Starting PASS Number 2  -  Processing: 0110                        .

. Starting PASS Number 3  -  Xref lists: 0118                        .

.                                                                    .

.      0 Unassigned Opcodes                                          .

.      2 Unresolved References                                       .

.                                                                    .

. Disassembly Complete  --  Source File Created.                     .

......................................................................

Now examine the disassembled listing (TEST.SRC).

;===============================================================;

; Disassembled Using XDEMO  -  (C)1990-96 Data Sync Engineering ;

; Newton, NJ 07860    Tel (201) 383-1355  ! Order XDASM Today ! ;

;===============================================================;

;

;

;

;       Unresolved Address Reference list

;

;

L016A:          EQU        0016AH

L024D:          EQU        0024DH

;

;

ORG     00000H

;

LJMP    L0100             ;0000  02 01 00        ...

;

;

ORG     00100H

;

;

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 11

A TYPICAL DISASSEMBLY PROCEDURE (cont.)

L0100:  MOV     DPTR,#00118H      ;0100  90 01 18        ...

LCALL   L0108             ;0103  12 01 08        ...

;

L0106:  SJMP    L0106             ;0106  80 FE           ..

;

L0108:  CLR     A                 ;0108  E4              .

MOVC    A,@A+DPTR         ;0109  93              .

INC     DPTR              ;010A  A3              .

CJNE    A,#000H,L010F     ;010B  B4 00 01        ...

RET                       ;010E  22              "

;

;

L010F:  JNB     TI,L010F          ;010F  30 99 FD        0..

CLR     TI                ;0112  C2 99           ..

MOV     SBUF,A            ;0114  F5 99           ..

AJMP    L0108             ;0116  21 08           !.

;

ORL     A,058H            ;0118  45 58           EX

AJMP    L024D             ;011A  41 4D           AM

;

JNC     L016A             ;011C  50 4C           PL

ORL     A,000H            ;011E  45 00           E.

;

;       Cross-references to LABELS

;

; L0100=  0000

; L0106=  0106

; L0108=  0103  0116

; L010F=  010B  010F

; L016A=  011C

; L024D=  011A

;

;       Cross-references to Data Memory locations

;

;  0000=  011E

;  0058=  0118

;  0099=  0114

;

;       Cross-references to BIT addressable locations

;

;  0099=  010F  0112

;

;       Immediate Byte references

;

;  0000=  010B

;  0118=  0100

;

END

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 12

A TYPICAL DISASSEMBLY PROCEDURE (cont.)

As you can see, XDEMO automatically inserted Labels for memory

location references and Directive statements for Assembler control.

A code segmentation method is also performed by inserting blank

comment lines after certain instructions, such as RET and JMP, and

before a line that contains a Label.

When we examine the listing, we notice that the area between addresses

0118 to 011E seem to contain ASCII text characters. It is also the

same area that is generating the Unresolved Address References.

STEP 2 -  Using a text editor, we will create a Tag file called

TEST.TAG which will contain the following lines:

0118=T

011F=B

^

Our Tag commands above simply specify, that, starting at address 0118

switch to "Define Text" disassembly. Then at address 011F switch to

"Define Byte" disassembly. The carat (^) character signifies the end

of the Tag file command list.

STEP 3 - The final step is to again disassemble the TEST.HEX

program using the "T" (Tag File) option. For clarity, we

will also use the "C" (Hexdump) and "L" (Lower-case)

options.

XDEMO TEST.HEX,8051 /TCL <RETURN>

......................................................................

.                                                                    .

. XDEMO - Table Based Cross-Disassembler    Demo Version   PC/MS-DOS .

. Copyright (C) 1990-1996 Data Sync Engineering   All rights reserved.

.                                                                    .

. Tag File Processed                                                 .

. Starting PASS Number 1  -  Processing: 0110                        .

. Starting PASS Number 2  -  Processing: 0110                        .

.                                                                    .

.      0 Unassigned Opcodes                                          .

.      0 Unresolved References                                       .

.                                                                    .

. Disassembly Complete  --  Source File Created.                     .

......................................................................

XDEMO  CROSS-DISASSEMBLER User's Guide               Page 13

A TYPICAL DISASSEMBLY PROCEDURE (cont.)

The final TEST.SRC result should now look like:

;===============================================================;

; Disassembled Using XDEMO  -  (C)1990-96 Data Sync Engineering ;

; Newton, NJ 07860    Tel (201) 383-1355  ! Order XDASM Today ! ;

;===============================================================;

;

;

;

;        Unresolved Address Reference list

;

;

;

org     00000h

;

ljmp    L0100              ;0000  02 01 00        ...

;

;

org     00100h

;

;

L0100:  mov     dptr,#00118h       ;0100  90 01 18        ...

lcall   L0108              ;0103  12 01 08        ...

;

L0106:  sjmp    L0106              ;0106  80 FE           ..

;

L0108:  clr     a                  ;0108  E4              .

movc    a,@a+dptr          ;0109  93              .

inc     dptr               ;010A  A3              .

cjne    a,#000h,L010F      ;010B  B4 00 01        ...

ret                        ;010E  22              "

;

;

L010F:  jnb     ti,L010F           ;010F  30 99 FD        0..

clr     ti                 ;0112  C2 99           ..

mov     sbuf,a             ;0114  F5 99           ..

ajmp    L0108              ;0116  21 08           !.

;

;

db      'EXAMPLE'

db      000h               ;011F  00              .

;

;

end